3GPP TSG-RAN WG2 #116-e	R2-21xxxxx

Electronic meeting 1st - 12th November, 2021

Agenda Item:	8.13.2.2

Source:	CATT

Title:	[Post115-e][898][SON/MDT] 2-step RA related SON aspects(CATT)

Document for:	Discussion and Decision

1	Introduction

This document captures the outcome of the following email discussion [1]

[POST 115e][898][SON/MDT] 2-step RA related SON aspects (CATT)

Scope:

Technical discussion rather than voting yes/no on open issues in 8.13.2.2 2-step RA related SON aspects.

How to capture all the related agreements we got so far.

Intended outcome: Report

Deadline: until next meeting

Please provide your comments for phase I before 9/27/2021 23:59 UTC and for phase II before 10/19/2021 23:59 UTC.

Phase I: progress on FFS

- Expected outcome: agreeable proposals

Phase II: progress on FFS of phase I and ASN.1 structure for all agreements

- Expected outcome: potential ASN.1 structure

If the convergence can be achieved in phase I, the corresponding conclusion can be captured in ASN.1 structure in phase II.

This document is organized as the following. For phase I, the discussions are in section 2, and the proposals are in section 3. For phase II, the content in section 4 and 5 are FFS, it will include the draft CR for all the agreements, and the possible ASN.1 structure depending on the progress in phase I.

2	Discussion

Rapporteur encourages the participating delegates to provide their contact information in this table.

| Company | Contact: Name (E-mail) |
| --- | --- |
| Samsung | Sb07.kim@samsung.com |

2.1 Switching information related

There are two options for switching information summarized in [1]:

Proposal 2: FFS which option should be made for RACH type switch indication in the RACH report: - Option 1: including an explicit switch indication in the IE related to the last/first RA attempt before/after the 2-step to 4-step RA switch. - Option 2: including the parameter MsgA-Transmax in each RA-InformationCommon IE.

Besides the 2 options above, one company points out that using stage-3 signaling design, i.e. to introduce a new field for reporting whether the DL beam quality, associated to the used 2 step RA resource, is above or below the msgA-RSRP-ThresholdSSB, could implicitly indicate the switching RA attempt in [1]. Therefore rapporteur lists this method as option 3, and there are three options for indicating switching point:

- Option 1: including an explicit switch indication in the PerRAAttemptInfo IE related to the last/first RA attempt before/after the 2-step to 4-step RA switch;

- Option 2: including the field msgA-Transmax in RA-InformationCommon IE;

- Option 3: switching indication from 2-step RA to 4-step RA can be implicitly indicated by introducing a new field, i.e. whether the DL beam quality, associated to the used 2 step RA resource, is above or below the msgA-RSRP-ThresholdSSB.

Since there is no consensus on bits consumption of the above three options in offline discussion [1], rapporteur analyzes the ASN.1 structure and bits consumption of the above options in the following.

- Option 1: including an explicit switch indication in the PerRAAttemptInfo IE related to the last/first RA attempt before/after the 2-step to 4-step RA switch

The ASN.1 structure could be:

PerRAAttemptInfoList-r16 ::=         SEQUENCE (SIZE (1..200)) OF PerRAAttemptInfo-r16

PerRAAttemptInfo-r16 ::=             SEQUENCE {

contentionDetected-r16               BOOLEAN                OPTIONAL,

dlRSRPAboveThreshold-r16             BOOLEAN                OPTIONAL,

...,

[[

lastRAAttemptOf2sRA-r17             ENUMERATED {true}                OPTIONAL

]]

}

The bits consumption:

Since the structure of “ENUMERATED {true}” will not cost bit, the consumption of option 1 is introduced by the “OPTIONAL”, it will cost 1 bit in each RA attempt to indicate whether the lastRAAttemptOf2sRA occurs or not. If there are N numbers of RA attempts, as many as 1*N bits are needed for option 1, the possible maximum number of bits could be 200.

The ASN.1 structure of indication related to first RA attempt after the 2-step to 4-step RA switch is similar as the above example.

If the option 1 is applied, the legacy field dlRSRPAboveThreshold can be reused with small description modification for 2-step RA report.

For RA procedure initiated for beam failure recovery, only rsrp-ThresholdSSB associated with 4-step RA can be configured in beamFailureRecoveryConfig. The msgA-RSRP-ThresholdSSB can be only configured in RACH-ConfigCommonTwoStepRA. Therefore, this field description can be modified as following to cover 2-step RA and 4-step RA cases:

dlRSRPAboveThreshold This field is used to indicate whether the DL beam (SSB) quality associated to the random access attempt was above or below the threshold rsrp-ThresholdSSB(for 4-step random access) in beamFailureRecoveryConfig in UL BWP configuration of UL BWP selected for random access procedure initiated for beam failure recovery; Otherwise, rsrp-ThresholdSSB(for 4-step random access) in rach-ConfigCommon or msgA-RSRP-ThresholdSSB(for 2-step random access) in msgA-ConfigCommon in UL BWP configuration of UL BWP selected for random access procedure.

- Option 2: including the field msgA-Transmax in RA-InformationCommon IE

The ASN.1 structure could be:

RA-InformationCommon-r16 ::=         SEQUENCE {

absoluteFrequencyPointA-r16          ARFCN-ValueNR,

locationAndBandwidth-r16             INTEGER (0..37949),

subcarrierSpacing-r16                SubcarrierSpacing,

msg1-FrequencyStart-r16              INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msg1-FrequencyStartCFRA-r16          INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msg1-SubcarrierSpacing-r16           SubcarrierSpacing                                OPTIONAL,

msg1-SubcarrierSpacingCFRA-r16       SubcarrierSpacing                                OPTIONAL,

msg1-FDM-r16                         ENUMERATED {one, two, four, eight}               OPTIONAL,

msg1-FDMCFRA-r16                     ENUMERATED {one, two, four, eight}               OPTIONAL,

perRAInfoList-r16                    PerRAInfoList-r16,

...,

[[

msgA-TransMax-r16                    ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}     OPTIONAL

]]

}

The bits consumption:

The field is indicated per RA procedure. The structure of “ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}” will cost 4 bits to indicate, and the consumption of the “OPTIONAL” is 1 bit. Therefore a mandatory size of 4+1=5 bits are needed for option 2.

If the option 2 is applied, similar as for the option1, the legacy field dlRSRPAboveThreshold can be reused with the small description modification for 2-step RA report.

- Option 3: switching indication from 2-step RA to 4-step RA can be implicitly indicated by introducing a new field, i.e. whether the DL beam quality, associated to the used 2 step RA resource, is above or below the msgA-RSRP-ThresholdSSB.

For option 3, there is a need to introduce a new field to indicate whether the DL beam quality, associated to the used 2 step RA resource, is above or below the msgA-RSRP-ThresholdSSB. Rapporteur gives the ASN.1 structure of the new field, e.g. dlRSRPAboveThreshold2sRA.

The ASN.1 structure could be:

PerRAAttemptInfoList-r16 ::=         SEQUENCE (SIZE (1..200)) OF PerRAAttemptInfo-r16

PerRAAttemptInfo-r16 ::=             SEQUENCE {

contentionDetected-r16               BOOLEAN                OPTIONAL,

dlRSRPAboveThreshold-r16             BOOLEAN                OPTIONAL,

...,

[[

dlRSRPAboveThreshold2sRA-r17         BOOLEAN                OPTIONAL

]]

}

The bits consumption:

The structure of “BOOLEAN” will cost 1 bit to indicate, and the “OPTIONAL” is also 1 bit cost. A size of 1+1=2 bits for each RA attempt is needed for the option 3. If there are N numbers of RA attempts, as many as 2*N bits are needed for option 3, the possible maximum number of bits could be 2*200.

Q1: Do you agree the rapporteur’s analysis on the ASN.1 structure and bits consumption of three options? If No, please give your analysis.

| Company | Yes/No | comments if any |
| --- | --- | --- |
| Samsung | No | We have assumed that the analysis of bits consumption is correct. However, we are not sure if the option 2 can fully cover the original intention indicating whether to switch to 4RA. For instance, when UE has received no RAR corresponding to the UE’s preamble until the expiry of window, UE may perform the “Random Access Resource Selection” described in TS38.321 clause 5.1.2 and 5.1.4. In other words, UE can switch 2RA to 4RA, due to the RA resource Selection process, as well as reaching to msgA-TransMax. Thus, we don’t think that the option 2 is a valid solution. |

Rapporteur would like the companies give the most suitable option based on the above analysis.

Q2: Which option do you prefer based on the analysis of the options?

- Option 1: including an explicit switch indication in the PerRAAttemptInfo IE related to the last/first RA attempt before/after the 2-step to 4-step RA switch;

- Option 2: including the field msgA-Transmax in RA-InformationCommon IE;

- Option 3: switching indication from 2-step RA to 4-step RA can be implicitly indicated by introducing a new field, i.e. whether the DL beam quality, associated to the used 2 step RA resource, is above or below the msgA-RSRP-ThresholdSSB.

| Company | Option 1/Option 2/Option 3 | comments if any |
| --- | --- | --- |
| Samsung | Option 1 | The option 1 is a clear solution covering all cases. Alternatively, a RA type indicator can be intuitively introduced. |

[Summary]

2.2 MSGA PUSCH related information

The companies as below suggest to include the MSGA PUSCH related information in RA report to optimize MSGA PUSCH transmission, the information suggested can be divided into two aspects: one is preamble group related, the other is MSGA PUSCH resource related.

- preamble group related

As the options summarized in [1], rapporteur lists the preamble group related information in the table below:

- A: the payload size transmitted in MSGA for a 2-step RACH attempt (from [2] Nokia) - B: indication of whether the payload size is above or below the ra-MsgA-SizeGroupA threshold (Samsung[1]) [Samsung] We has then suggested a new indicator to indicate whether MSGA PUSCH was transmitted or not during this RA attempt - C: the group type of a preamble i.e., group type A or B (from [2] Nokia, [4] Ericsson) - D:PUSCH group information(from [3] ZTE, [4] Ericsson) - E: indication of pathloss above or below the pathloss threshold for groupA/B (from [4] Ericsson)

For the RACH optimization in Rel-16, the RACH preamble split (among dedicated, group A, group B) aspects was included in stage 1, i.e. TR37.816. However, it was not discussed in the subsequent WI stage. Since the corresponding optimization was not introduced for 4-step RA report, rapporteur thinks we should firstly discuss the necessity to include the preamble group related information into RA report which is out of the scope of 2-step RA report optimization.

In TS 38.321, for the contention-based Random Access preamble selection of 4-step RA type and 2-step RA type, there are two kinds of condition to select the RA preambles group B, one is contention-free Random Access Resources have not been configured, another is contention-free Random Access Resources have been configured.

For the first condition, the preamble group selection is based on the threshold ra-Msg3SizeGroupA (ra-MsgA-SizeGroupA) and/or pathloss. For the second condition, the preamble group selection based on whether the transport block size of the MSGA payload configured in the rach-ConfigDedicated corresponds to the transport block size of the MSGA payload associated with Random Access Preambles group B.

Since the preamble group selection procedure is similar for 4-step RA and for 2-step RA, if there is a need to optimize the preamble group, the optimization needs to cover both 2-step RA and 4-step RA cases.

Please take above information into account when considering whether there is a need to optimize preamble group for RACH optimization. Therefore, the rapporteur suggests: RAN2 first discusses whether there is a need to optimize preamble group for RACH optimization and the corresponding conclusion is applied to both 2-step RA report and 4-step RA report.

If companies consider the preamble group optimization is necessary, the information may be needed to achieve an acceptable proposal.

Q3: Do you agree to optimize preamble group for RACH optimization, and the corresponding conclusion is applied to 2-step RA report and 4-step RA report?

| Company | Yes/No | comments if any (Reason or Benefit) |
| --- | --- | --- |
| Samsung | No |  |

Since the option “D” is general information and covered in Q3, rapporteur excludes this option in Q4.

Q4: If you agree with Q3, which option(s) do you prefer?

- A: the payload size transmitted in MSGA for a 2-step RACH attempt

- B: indication of whether the payload size is above or below the ra-MsgA-SizeGroupA threshold

- C: the group type of a preamble i.e., group type A or B

- E: indication of pathloss above or below the pathloss threshold for groupA/B

| Company | A/B/C/E/others | Comments if any |
| --- | --- | --- |

[Summary]

- MSGA PUSCH resource related

- F: the MCS index(from [3] ZTE) - G: the number of PRB per PO of the PUSCH resource(from [3] ZTE) - H: the combination of start symbol and length and PUSCH mapping type(from [3] ZTE) - I: offset of lowest PUSCH occasion in frequency domain with respect to PRB 0(from [3] ZTE) - J: the number of msgA PUSCH occasions FDMed in one time instance(from [3] ZTE) - K:MSGA PUSCH resource information (from [4] Errcsson and [5] CMCC) - M: whether MSGA PUSCH was transmitted or not during this RA attempt (from Samsung)

For the MSGA PUSCH resource related, the rapporteur lists the detail parameters related the above information that configured in MsgA-PUSCH-Resource.

F: the MCS index

msgA-MCS Indicates the MCS index for msgA PUSCH from the Table 6.1.4.1-1 for DFT-s-OFDM and Table 5.1.3.1-1 for CP-OFDM in TS 38.214.

G: the number of PRB per PO of the PUSCH resource

nrofPRBs-PerMsgA-PO Number of PRBs per PUSCH occasion (see TS 38.213, clause 8.1A).

H: the combination of start symbol and length and PUSCH mapping type

msgA-PUSCH-TimeDomainAllocation Indicates a combination of start symbol and length and PUSCH mapping type from the TDRA table (PUSCH-TimeDomainResourceAllocationList if provided in PUSCH-ConfigCommon, or else the default Table 6.1.2.1.1-2 in 38.214 is used if pusch-TimeDomainAllocationList is not provided in PUSCH-ConfigCommon). The parameter K2 in the table is not used for msgA PUSCH. The network configures one of msgA-PUSCH-TimeDomainAllocation and startSymbolAndLengthMsgA-PO, but not both. If the field is absent, the UE shall use the value of startSymbolAndLenghtMsgA-PO.

I: Offset of lowest PUSCH occasion in frequency domain with respect to PRB 0

frequencyStartMsgA-PUSCH Offset of lowest PUSCH occasion in frequency domain with respect to PRB 0 (see TS 38.213, clause 8.1A).

J: The number of msgA PUSCH occasions FDMed in one time instance

nrofMsgA-PO-FDM The number of msgA PUSCH occasions FDMed in one time instance (see TS 38.213, clause 8.1A).

The detail information and configuration of the above parameters can be checked in TS 38.214 and TS 38.213.

In the previous meeting, a fallback indication is agreed to be included into 2-step RA report per RA attempt. This indication can be used for network to optimize the frequency and time domain allocation or MCS of MSGA PUSCH, e.g. if the fallback occurs frequently, the network can adjust the above mentioned parameters for MSGA PUSCH payload transmission. It is a coarse granularity but efficient way to optimize MSGA PUSCH resource.

If a finer granularity method is needed, as in the above table, a lot of information may be needed to optimize MSGA PUSCH resource. Please take the complexity and the signalling overhead into account when considering this finer granularity optimization of MSGA PUSCH resource.

M: whether MSGA PUSCH was transmitted or not during this RA attempt.

In NR-U, the LBT failure may happen in the MSGA PUSCH occasion, or SSB/PRACH occasion corresponding to MSGA PUSCH occasion may be invalid. Then, MSGA PUSCH cannot be transmitted. Hence, M is helpful to identify any problem in MSGA transmission.

Q5: Do you agree that there is a need to introduce the MSGA resource related information in 2-step RA report?

| Company | Yes/No | Comments if any(Reason or benefit) |
| --- | --- | --- |
| Samsung | Yes | See the new description above |

Since the option “K” is general information and covered in Q5, rapporteur excludes this option in Q6.

Q6: If you agree with Q5, which option(s) do you prefer?

- F: the MCS index

- G: the number of PRB per PO of the PUSCH resource

- H: the combination of start symbol and length and PUSCH mapping type

- I: offset of lowest PUSCH occasion in frequency domain with respect to PRB 0

- J: the number of msgA PUSCH occasions FDMed in one time instance

- M: whether MSGA PUSCH was transmitted or not during this RA attempt

| Company | F/G/H/I/J/M/others | Comments if any |
| --- | --- | --- |
| Samsung | M | See the new description above |

[Summary]

3 Phase I Conclusion

Based on the discussion, we firstly have a set of proposals for RAN2 agreements:

4 Phase II

FFS.

5 Phase II Conclusion

FFS.

6 References

- R2-2108963, Report of [AT115e][821][SON/MDT] 2-Step RA related SON (OPPO)

- R2-2107507, Remaining Issues and New Aspects in 2-step NR UE RACH Report, Nokia

- R2-2108354, 2-step RA related enhancements, ZTE

- R2-2108418, 2-step RA information for SON purposes, Ericsson

- R2-2108542, SON Enhancement for 2-step RA, CMCC