3GPP TSG-RAN2 Meeting #116 electronic	Draft R2-2111626

# Online, November 01 – November 12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2858 | rev | 2 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on pucch-SpatialRelationInfoId-v1610 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the presence condition of pucch-SpatialRelationInfoId-v1610 in PUCCH-SpatialRelationInfoExt-r16, it must be: a) present upon creation of PUCCH-SpatialRelationInfo b) absent otherwise, Need M Both a) and b) are have severe consequences: Problem 1 Due to b) , it is impossible to modify any spatial relation info with ID >=9 Problem 2 In order to create a PUCCH spatial relation info with ID >= 9 or set pucch-PathlossReferenceRS-Id >=4 for a PUCCH spatial relation, it is necessary to include spatialRelationInfoToAddModListExt-v1610 in pucch-Config of the corresponding UL BWP. According to the field description of PUCCH-SpatialRelationInfoExt-r16, there must be entries in this list for all PUCCH spatial relation infos in the same pucch-Config. Then, for any creation of a PUCCH spatial relation info in the same message in the same UL BWP, according to a) there must be an ID >=9, i.e. it is impossible to create any spatial relation info with ID <=8 in this UL BWP in this message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the presence condition by Need S (as in the example in A.4.3.6) and indicate behaviour upon absence. Impact Analysis Impacted 5G architecture options: SA, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: PUCCH power control The CR is considered mandatory to support more than 4 PUCCH pathloss RSs and/or more than 9 spatial relation info configured in an UL BWP (when maxNumberPathlossRS-Update-r16 is greater than 4). Inter-operability: - If the UE is implemented according to the CR and the NW is not, it is impossible to modify any spatial relation info with ID >=9 or to create any spatial relation info with ID from 1 to 8 in a message that creates a spatial relation info with ID >=9 or includes a spatial relation with a pathloss reference RS with ID >=4 in the same UL BWP. - If the NW is implemented according to the CR and the UE is not, the UE may consider a reconfiguration to modify a spatial relation info with ID>=9 as invalid and trigger RRC re-establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is impossible to modify any spatial relation info with ID >=9 or to create any spatial relation info with ID from 1 to 8 in a message that creates a spatial relation info with ID >=9 or includes a spatial relation with a pathloss reference RS with ID >=4 in the same UL BWP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 6.3.2	Radio resource control information elements

<Omit unrelated text>

#### –	PUCCH-SpatialRelationInfo

The IE PUCCH-SpatialRelationInfo is used to configure the spatial setting for PUCCH transmission and the parameters for PUCCH power control, see TS 38.213, [13], clause 9.2.2.

PUCCH-SpatialRelationInfo information element

-- ASN1START

-- TAG-PUCCH-SPATIALRELATIONINFO-START

PUCCH-SpatialRelationInfo ::=           SEQUENCE {

pucch-SpatialRelationInfoId         PUCCH-SpatialRelationInfoId,

servingCellId                           ServCellIndex                                                    OPTIONAL,   -- Need S

referenceSignal                         CHOICE {

ssb-Index                               SSB-Index,

csi-RS-Index                            NZP-CSI-RS-ResourceId,

srs                                     PUCCH-SRS

},

pucch-PathlossReferenceRS-Id            PUCCH-PathlossReferenceRS-Id,

p0-PUCCH-Id                             P0-PUCCH-Id,

closedLoopIndex                         ENUMERATED { i0, i1 }

}

PUCCH-SpatialRelationInfoExt-r16 ::=       SEQUENCE {

pucch-SpatialRelationInfoId-v1610         PUCCH-SpatialRelationInfoId-v1610                              OPTIONAL,   -- Need S

pucch-PathlossReferenceRS-Id-v1610        PUCCH-PathlossReferenceRS-Id-v1610                             OPTIONAL,    --Need R

...

}

PUCCH-SRS ::=                       SEQUENCE {

resource                            SRS-ResourceId,

uplinkBWP                           BWP-Id

}

-- TAG-PUCCH-SPATIALRELATIONINFO-STOP

-- ASN1STOP

PUCCH-SpatialRelationInfo field descriptions

pucch-PathLossReferenceRS-Id When pucch-PathLossReferenceRS-Id-v1610 is configured, the UE shall ignore pucch-PathLossReferenceRS-Id (without suffix).

pucch-SpatialRelationInfoId When pucch-SpatialRelationInfoId-v1610 is configured, the UE shall ignore pucch-SpatialRelationInfoId (without suffix). If pucch-SpatialRelationInfo-v1610 is absent, the UE shall use the pucch-SpatialRelationInfoId (without suffix).

servingCellId If the field is absent, the UE applies the ServCellId of the serving cell in which this PUCCH-SpatialRelationInfo is configured

### <End of modification>