3GPP TSG-RAN WG2 Meeting #117-e		R2-210xxxx

Electronic, 17th Jan. – 25th Jan. 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | - | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.304 Running CR for ePowSav |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2021-11-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To capture agreements for ePowSav in NR into TS 38.304. This is a draft of 38.304 running CR for ePowSav. To be updated based on the progress on ePowSav in both RAN1 and RAN2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of ePowSav features in NR. This CR captures the idle/inactive aspects of enhanced power saving and it is based on the RAN2/RAN1 agreements made so far, which could be found in Annex at the end of this document. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ePowSav is not supported in NR in TS 38.304. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | TBD |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.331 CR TBD TS/TR 38.306 CR TBD TS/TR 38.321 CR TBD TS/TR 38.300 CR TBD |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be lifted to the latest version of the specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the initial version of running CR for TS 38.304 for ePowSav WI. |  |  |  |  |  |  |  |  |

Start of change

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AS	Access Stratum

CAG	Closed Access Group

CAG-ID	Closed Access Group Identifier

CMAS	Commercial Mobile Alert System

CN	Core Network

CSI-RS	Channel state information reference signal

DCI	Downlink Control Information

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

IMSI	International Mobile Subscriber Identity

MCC	Mobile Country Code

MICO	Mobile Initiated Connection Only

NAS	Non-Access Stratum

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

PEI	Paging Early Indication [FFS]

PLMN	Public Land Mobile Network

RAT	Radio Access Technology

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RRC	Radio Resource Control

SNPN	Stand-alone Non-Public Network

UAC	Unified Access Control

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

V2X	Vehicle to Everything

Next change

# 7	Paging

## 7.1	Discontinuous Reception for paging

The UE may use Discontinuous Reception (DRX) in RRC_IDLE and RRC_INACTIVE state in order to reduce power consumption. The UE monitors one paging occasion (PO) per DRX cycle. A PO is a set of PDCCH monitoring occasions and can consist of multiple time slots (e.g. subframe or OFDM symbol) where paging DCI can be sent (TS 38.213 [4]). One Paging Frame (PF) is one Radio Frame and may contain one or multiple PO(s) or starting point of a PO.

In multi-beam operations, the UE assumes that the same paging message and the same Short Message are repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the paging message and Short Message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.

The UE initiates RRC Connection Resume procedure upon receiving RAN initiated paging. If the UE receives a CN initiated paging in RRC_INACTIVE state, the UE moves to RRC_IDLE and informs NAS.

The PF and PO for paging are determined by the following formulae:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

The PDCCH monitoring occasions for paging are determined according to pagingSearchSpace as specified in TS 38.213 [4] and firstPDCCH-MonitoringOccasionOfPO and nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured as specified in TS 38.331 [3]. When SearchSpaceId = 0 is configured for pagingSearchSpace, the PDCCH monitoring occasions for paging are same as for RMSI as defined in clause 13 in TS 38.213 [4].

When SearchSpaceId = 0 is configured for pagingSearchSpace, Ns is either 1 or 2. For Ns = 1, there is only one PO which starts from the first PDCCH monitoring occasion for paging in the PF. For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF.

When SearchSpaceId other than 0 is configured for pagingSearchSpace, the UE monitors the (i_s + 1)th PO. A PO is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH monitoring occasions for paging which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF. When firstPDCCH-MonitoringOccasionOfPO is present, the starting PDCCH monitoring occasion number of (i_s + 1)th PO is the (i_s + 1)th value of the firstPDCCH-MonitoringOccasionOfPO parameter; otherwise, it is equal to i_s * S*X. If X > 1, when the UE detects a PDCCH transmission addressed to P-RNTI within its PO, the UE is not required to monitor the subsequent PDCCH monitoring occasions for this PO.

NOTE 1:	A PO associated with a PF may start in the PF or after the PF.

NOTE 2:	The PDCCH monitoring occasions for a PO can span multiple radio frames. When SearchSpaceId other than 0 is configured for paging-SearchSpace the PDCCH monitoring occasions for a PO can span multiple periods of the paging search space.

The following parameters are used for the calculation of PF and i_s above:

T: DRX cycle of the UE (T is determined by the shortest of the UE specific DRX value(s), if configured by RRC and/or upper layers, and a default DRX value broadcast in system information. In RRC_IDLE state, if UE specific DRX is not configured by upper layers, the default value is applied).

N: number of total paging frames in T

Ns: number of paging occasions for a PF

PF_offset: offset used for PF determination

UE_ID: 5G-S-TMSI mod 1024

Parameters Ns, nAndPagingFrameOffset, nrofPDCCH-MonitoringOccasionPerSSB-InPO, and the length of default DRX Cycle are signaled in SIB1. The values of N and PF_offset are derived from the parameter nAndPagingFrameOffset as defined in TS 38.331 [3]. The parameter first-PDCCH-MonitoringOccasionOfPO is signalled in SIB1 for paging in initial DL BWP. For paging in a DL BWP other than the initial DL BWP, the parameter first-PDCCH-MonitoringOccasionOfPO is signaled in the corresponding BWP configuration.

If the UE has no 5G-S-TMSI, for instance when the UE has not yet registered onto the network, the UE shall use as default identity UE_ID = 0 in the PF and i_s formulas above.

5G-S-TMSI is a 48 bit long bit string as defined in TS 23.501 [10]. 5G-S-TMSI shall in the formulae above be interpreted as a binary number where the left most bit represents the most significant bit.

## 7.x	PEI

### 7.x.1	PEI reception

If PEI configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state [FFS: supporting PEI] can monitor PEI through PEI parameters in system information according to below procedure.

Editor’s NOTE:	The detailed procedure may be updated based on RAN1 agreements.

Editor’s NOTE:	Whether to have UE capability on PEI would be further decided.

Editor’s NOTE:	Whether the PEI is used for only last used cell/TA area/some specific area would be further discussed.

The UE monitors one PEI occasion per DRX cycle [FFS]. A PEI occasion is a set of PDCCH monitoring occasions and can consist of multiple time slots (e.g. subframe or OFDM symbol) where PEI DCI can be sent (TS 38.213 [4]).

Editor’s NOTE:	FFS: 1 PEI for POs across multiple PFs.

The PEI occasion is determined by: TBD

The PDCCH monitoring occasions for PEI are determined according to peiSearchSpace as specified in TS 38.213 [4], [TBD firstPDCCH-MonitoringOccasionOfPEI] and nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured as specified in TS 38.331 [3].

When SearchSpaceId [TBD other than 0] is configured for peiSearchSpace, the UE monitors the PEI occasion according to SearchSpaceId. TBD

A PEI occasions is a set of S consecutive PDCCH monitoring occasions when nrofPDCCH-MonitoringOccasionPerSSB-InPO is not configured, where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1. The K-th PDCCH monitoring occasion for PEI in the PEI occasion has the same QCL assumption as that of the K-th PDCCH monitoring occasion for paging in the PO, where K=0,1,…,S.

[TBD]The following parameters are used for the calculation of PEI occasion above:

[TBD] subgroupsNumPerPO: Number of subgroups per PO for UE to read subgroups indication from physical-layer singling

Editor’s NOTE:	The detailed calculation for PEI reception would be further discussed and decided in RAN1.

Editor’s NOTE:	The detailed parameters for PEI reception will be aligned with RRC specification.

When the UE detects PEI, and the PEI indicates the UE (or the subgroup the UE belongs to, as specified in clause 7.y) to monitor the associated PO, the UE monitors paging DCI in the associated PO, including scheduling information for paging PDSCH (if included) as specified in clause 7.1. If the UE does not detect PEI on the monitored PEI occasion or the PEI does not indicate the UE (or the subgroup the UE belongs to, as specified in clause 7.y) to monitor the associated PO, the UE is not required to monitor paging DCI in the associated PO, including scheduling information for paging PDSCH (if included) as specified in clause 7.1.

Editor’s NOTE:	As a baseline RAN2 has a preference to support PEI with both DRX and eDRX, but potential issues (e.g. PEI and PTW) are FFS.

If the UE is not able to monitor the PEI occasion (i.e. all monitoring occasion for PEI) corresponding to its PO, e.g. during cell re-selection, the UE will monitor the paging DCI in the associated PO, including scheduling information for paging PDSCH (if included) according to clause 7.1.

Editor’s NOTE:	Companies are invited to provide suggestions on the wording of this behaviour.

## 7.y	Subgrouping [FFS]

Editor’s NOTE:	FFS: Whether to have a separate clause for subgrouping or merge it into the previous clause for PEI in 7.x as a subclause (e.g. 7.x.y) would be further decided after the decision on UE capability and detailed design for PEI and subgrouping.

### 7.y.0	General

UEs monitoring the same PO can be divided into multiple subgroups. With subgrouping, the UE monitors paging if the subgroup the UE belongs to is indicated by PEI to monitor the associated PO. Two subgrouping approaches can be supported by the UE: CN controlled subgrouping as specified in clause 7.y.1 and UE_ID based subgrouping as specified in clause 7.y.2.

[TBD] If a cell supports both CN controlled subgrouping and UE_ID based subgrouping simultaneously:

Editor’s NOTE:	FFS on how to determine the subgroup index of CN controlled subgrouping and UE_ID based subgrouping. Assume that one subgroup indication refer to either CN assigned subgroups or UE-ID based subgroup (no overlapping)

Editor’s NOTE:	FFS on where to capture this case depends on how to determine the subgroup index.

If the configuration of subgrouping is absent or nullified (e.g. subgroupsNumPerPO is either absent or set to zero), which means that the cell does not support any type of subgrouping. Then, no subgrouping is used by the UE, i.e. the UE monitors PEI or paging as specified in clause 7.x or clause 7.1.

Editor’s NOTE:	FFS for the signalling details.

### 7.y.1	CN controlled subgrouping

Editor’s NOTE:	FFS on the terminology for CN controlled subgrouping. Companies are invited to provide your suggestion.

Paging with CN controlled subgrouping is only used in the cell which supports CN controlled subgrouping by the indication provided in system information through [TBD]. A UE supporting CN controlled subgrouping in RRC_IDLE or RRC_INACTIVE state can be assigned a subgroup index by AMF through NAS signalling. The UE belonging to the assigned subgroup index monitors PEI with the paged subgroup(s) as specified in clause 7.x.

Editor’s NOTE:	RAN capability is known based on broadcast information. FFS with explicit indication or implicitly based configuration.

Editor’s NOTE:	FFS on the detailed NAS signalling between AMF and UE. The design and procedure are up to SA2/CT1.

Editor’s NOTE:	R2 assumes that all the cells within the registration area supports the same number of CN assigned subgroups, i.e. no remapping of CN assigned group ID to RAN subgroup ID (will revisit only if serious issues are found).

### 7.y.2	UE_ID based subgrouping

Editor’s NOTE:	FFS on the terminology for UE_ID based subgrouping. Companies are invited to provide your suggestion.

Paging with UE_ID based subgrouping is only used in the cell which supports UE_ID based subgrouping by the indication of [Nsg-UEID TBD] provided in system information.

Editor’s NOTE:	RAN capability is known based on broadcast information. FFS with explicit indication or implicitly based configuration.

If the UE supporting UE_ID based subgrouping is not configured with a CN assigned subgroup index, the subgroup of the UE is determined by below formula:

SubgroupID = UE_ID mod Nsg_UEID,

where:

UE_ID: 5G-S-TMSI mod X, where X is 8192

Nsg-UEID: number of subgroups for UE_ID based subgrouping, which is broadcasted in system information

Editor’s NOTE:	The detailed parameters for UE_ID based subgrouping will be aligned with RRC specification.

The UE belonging to the SubgroupID monitors PEI with the paged subgroup(s) as specified in clause 7.x.

Next change

# Z	TRS/CSI-RS

## Z.1	General

The UE in RRC_IDLE and RRC_INACTIVE state may use TRS/CSI-RS in order to reduce power consumption. The corresponding configuration of TRS/CSI-RS may be provided to the UE in the system information. For a cell with TRS/CSI-RS occasions configured for RRC_IDLE and RRC_INACTIVE state UEs, the availability of TRS/CSI-RS at the configured occasion(s) is informed to the RRC_IDLE and RRC_INATIVE state UEs based on explicit L1 based signaling defined in TS 38.213 [4].

Editor’s NOTE:	R2 assumes that additional TRS/CSI-RS configuration by dedicated signalling is not supported. Can revisit e.g. based on R1 provided info if needed.

Editor’s NOTE:	FFS on whether/how it should be possible to enable / disable the TRS/CSI-RS L1 based availability mechanism by broadcast signalling.

## Z.2	TRS/CSI-RS reception

Editor’s NOTE:	Companies are invited to provide comments on whether this part needs to be captured here or only captured in PHY specification.

When the UE receives the configuration of TRS/CSI-RS in SIBx,

-	If PEI is detected as specified in clause 7.x.1, and the availability indication(s) in PEI indicates that the TRS/CSI-RS at the configured occasion(s) is available, as specified in TS 38.213 [4]; or,

-	If the availability indication(s) in paging PDCCH (according to clause 7) indicates that the TRS/CSI-RS at the configured occasion(s) is available, as specified in TS 38.213 [4]:

-	the UE considers the corresponding TRS/CSI-RS configuration associated with the availability indication(s) is available for a time duration of [TBD];

Editor’s NOTE:	It is “working assumption” in RAN1 to support both paging PDCCH based and PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs, which would be further updated based on RAN1 progress.

Editor’s NOTE:	Postpone further discussion on TRS/CSI-RS applicability for eDRX UEs, which can be considered later.

Editor’s NOTE:	Whether to have UE capability on TRS/CSI-RS would be further decided.

Editor’s NOTE:	FFS on supporting SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured.

Editor’s NOTE:	FFS on the valid period for the availability of TRS/CSI-RS.

End of change

# Annex A– RAN2 agreements

Agreements on paging enhancement

| RAN2 agreements | Impacted specification | Comments, if any |
| --- | --- | --- |
| RAN2#111e |  |  |
| For PowSav solutions for Idle/Inactive (for smart phones) that can easily also be applied to redcap, R2 assume they may be applied. Details FFS and to be discuss case by case when the maturity is high (might in the end just be a question of UE caps). Dual DRX not in the scope of current WID. | No impact |  |
| RAN2#112e |  |  |
| Confirm that UE grouping is considered a candidate of paging enhancement for UE power saving | Not yet captured to wait for further progress. |  |
| RAN2 have discussed and considered “paging indication for UE subgroups using paging DCI”, “paging early indication or wake-up signal (WUS) for UE subgroups”, “cross-slot scheduling of paging for UE subgroups”. RAN2 understands that RAN1 have started to evaluate performance and complexity. RAN2 assumes that RAN1 continues with this evaluation, in order that decisions can be made regarding the paging indication/scheduling solution. As R2 is the leading group for this WI objective it is expected that final decisions are made by R2. Will send an LS to R1 (action to be discussed offline). The solution of PRNTI based group discrimination is deprioritized from RAN2 perspective The solution of “paging for UE subgroups using different time/frequency resources” is de-prioritized from RAN2 perspective. | No impact |  |
| RAN2#113e |  |  |
| There is support to have UE ID based enhancement There is still significant interest to have other additional methods (but also some concerns). The approach to have a single mechanism that can take several aspects into account can be a way forward. There are still questions on the details, e.g. whether CN or RAN would provide a parameter. | No impact |  |
| RAN2#113bis-e |  |  |
| If we go for network controlled subgrouping, If the network chooses to not provide specific subgrouping information, there will be configuration option where subgrouping can be supported by randomization (by UE-ID). | Captured in 7.y.2 |  |
| We adopt Network controlled subgrouping (based on individual UE characteristics, not specified or limited to paging prob as EUTRA, possibly with additional randomization) | Not yet captured to wait for further progress. |  |
| RAN2#114e |  |  |
| CN is responsible for allocating UEs to UE paging subgroups based on UE characteristics | Captured in 7.y.1 |  |
| Use same UE subgroups when in RRC_IDLE and RRC_INACTIVE | Captured in 7.y |  |
| RAN2#115e |  |  |
| When AMF has assigned a UE with a Paging subgroup, some NAS signaling should be supported between AMF and UE to convey the related information to the UE. Exact information is FFS. The design and procedure are up to SA2/CT1. | Captured in 7.y.1 |  |
| When AMF has assigned a UE with a Paging subgroup, some signaling should be supported between AMF and gNB(s) to inform gNB(s) about the related subgroup information for paging a UE in RRC_IDLE/RRC_INACTIVE. Exact information is FFS. The message(s) and associated design are up to RAN3. | Captured in 7.y.1 |  |
| It is FFS when a UE in RRC_INACTIVE has been assigned by CN a Paging subgroup, whether some signaling should be introduced between gNBs to inform each other about the UE’s subgroup for RAN paging. | No impact by now |  |
| If RAN2 agrees to support UE assistance information to CN in support of Paging subgroup assignment, RAN2 will focus on the paging probability and power profile attributes. | No impact by now |  |
| UEID-based subgroup method requires, in addition to the already available information for legacy UEID-based grouping in PO, the total number of supported UEID-based subgroups by the network. | Captured in 7.y.2 |  |
| At least for UEID-based subgroup method the total number, Nsg, of supported subgroups by the network is decided by RAN and broadcasted in System Information. | Captured in 7.y.2 |  |
| At least for UEID-based subgroup method the total number, Nsg, of supported subgroups is controlled on a cell basis and can be different in different cells. | Captured in 7.y.2 |  |
| Option 2 is excluded We go with Option 1 Editor’s NOTE: option 1 and option 2 come from R2-2109094, which are: - Option 1: CN assigns subgroup ID - possible with or without remapping to RAN subgroup ID depends on the sub-options - Option 2: CN assigns a set of subgroup IDs - Similar to option 1 but with multiple subgroup IDs assigned from CN and the UE needs to choose the corresponding subgroup ID based on RAN configuration | Captured in 7.y.1 |  |
| R2 assumes that All the cells within the registration area supports the same number of CN assigned subgroups, i.e. no remapping of CN assigned group ID to RAN subgroup ID (will revisit only if serious issues are found). | Captured in 7.y.1 |  |
| For the purpose of continued discussions, R2 assumes that UE has separate UE caps for CN assigned and UEID based subgrouping, the actual decision to be taken later. | Not yet captured to wait for further progress. |  |
| RAN capability is known based on broadcast information. FFS with explicit indication or implicitly based configuration. | Captured in 7.y |  |
| RAN2#116e |  |  |
| Assume that one subgroup indication refer to either CN assigned subgroups or UE-ID based subgroup (no overlapping) Both UE ID based and CN based subgrouping can be supported simultaneously in a cell, it is allowed to just support one of them. | Captured in 7.y.0 |  |
| FFS if the total number of CN-assigned subgroups is OAM configured. Max would be 8 as this is what RAN support. | No impact |  |
| The total number of CN-assigned subgroups that is used is not fixed can be configured up to 8 (e.g. by OAM). No impact on signalling is assumed. | No impact |  |
| RAN introduces a new parameter Nsg-UEID to indicate its support of UE-ID based subgrouping. | Captured in 7.y.2 |  |
| RAN does not support any type of subgrouping if its configuration for subgrouping is either absent or nullified (e.g. subgroupsNumPerPO is either absent or set to zero). FFS for the signalling details. | Captured in 7.y.0 |  |
| We assume separate indications for UE capability of CN based subgrouping and UEID based subgrouping. UE’s capability of supporting the UE ID based subgrouping is reported to RAN by AS UE capability signalling while R2 assumes that UE’s capability of supporting the CN-assigned subgrouping is reported to CN by NAS signalling. | Captured in 7.y.1 and 7.y.2 |  |
| RAN2 assumes that if PEI is detected, and the PEI indicates that the UE has to monitor the associated PO, then the UE monitors paging DCI in the associated PO, including scheduling information for paging PDSCH (if included) as in legacy. This assumption may be updated based on RAN1 agreements. | Captured in 7.x.1 |  |
| As a baseline RAN2 has a preference to support PEI with both DRX and eDRX, but potential issues (e.g. PEI and PTW) are FFS. | Captured in 7.x.1 |  |
| For UE-ID based subgroups the UE identity is UE_ID = 5G-S-TMSI mod X, where X is 8192 (1024*8). | Captured in 7.y.2 |  |
| Introduce a UERadioPagingInfo IE in the UECapabilityInformation message in NR in Rel-17. | No impact |  |
| If the UE was not able to monitor the PEI occasion corresponding to its PO the UE shall monitor the PO. | Captured in 7.x.1 |  |

Agreements on TRS/CSI-RS in idle/inactive

| RAN2 agreements | Impacted specification | Comments, if any |
| --- | --- | --- |
| RAN2#113e |  |  |
| [041] On signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s): SIB signalling is the baseline; Other dedicated high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) can be additionally considered with justification. It is assumed they do not work alone. | Not yet captured to wait for further progress. |  |
| [041] RAN2 will down select from the following options on SIB signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s): Option 2: Existing SIB, other than SIB1; Option 3: New SIB type, e.g. SIB-x; | Not yet captured to wait for further progress. |  |
| RAN2#114e |  |  |
| N/A |  |  |
| RAN2#115e |  |  |
| The TRS/CSI-RS configuration is provided in a new SIB. On demand SI should be possible for the SIB with TRS/CSI-RS information. | Captured in Z.2 |  |
| RAN2 assumes that TRS/CSI-RS configurations are broadcasted. Potential addition of dedicated signalling can be discussed in a later meeting based on company contributions. | Not yet captured to wait for further progress. |  |
| The legacy SI update procedure is used for changing TRS/CSI-RS configurations. | No impact by now. |  |
| Postpone the topic about TRS/CSI-RS availability until a later meeting when RAN1 also has progressed. Postpone the discussion on segmentation of the new SIB until RAN1 has sent the list of the parameters and a potential structure. Postpone the discussion on splitting the TRS/CSI-RS information to a common and RS-specific part until RAN1 has sent the list of the parameters and a potential structure. | Not yet captured to wait for further progress. |  |
| RAN2#116e |  |  |
| The scope of the new SIB-X is configurable (either cell or area scope) based on NW implementation. RAN2 to wait for additional RAN1 feedback, before finalizing aspects on SIB-X sizing, segmentation etc. RAN2 to wait for further RAN1 input on whether TRS/CSI-RS configuration can be split as common and TRS specific part. The new SIB-X can be made on demand, and it is up to NW configuration. There are no UE side impacts due to any additional NW side restriction on on-demand SIB-X. IDLE/INACTIVE UEs do NOT have to report any feedback on its TRS/CSI-RS resource usage. | No impact |  |
| RAN2 assumes to support current RAN1 working agreement of L1 based signalling for TRS/CSI-RS availability indication. FFS whether it should be possible to enable / disable the TRS/CSI-RS L1 based availability mechanism by broadcast signalling. | Captured in Z.1 |  |
| R2 assumes that additional TRS/CSI-RS configuration by dedicated signalling is not supported. Can revisit e.g. based on R1 provided info if needed. Postpone further discussion on TRS/CSI-RS applicability for eDRX UEs. Can consider later | Not yet captured to wait for further progress. |  |
| RAN2#117e |  |  |

# Annex B– RAN1 agreements (only RAN2 relevant part)

Agreements on paging enhancement

| RAN1 agreements | Impacted specification | Comments, if any |
| --- | --- | --- |
| RAN1#102e |  |  |
| N/A |  |  |
| RAN1#103e |  |  |
| N/A |  |  |
| RAN1#104e and RAN#93e |  |  |
| Agreements: For the evaluation and comparison of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, the following are assumed: - Behv-A: - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO - Behv-B: - PEI indicates whether or not UE should monitor a PO - UE is required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO Rapporteur: Further conclusion in RAN#93 below should be considered together. - Support PDCCH-based PEI as the only option • Only essential function for PEI is support • New DCI format • Higher layer configuration, including SS • Details of the procedures of PEI monitoring, and identification of MOs before PO • Only Behv-A (per RAN1#104e agreement) is supported • If TRS availability indication is agreed to be supported in both paging DCI and the DCI format for PEI, same mechanism/principle for TRS availability indication is adopted for the two DCI formats • Supporting TRS availability indication in DCI format for PEI shall not delay the completion of essential functionality of PEI | Captured in 7.x.1 |  |
| RAN1#104bis-e |  |  |
| N/A |  |  |
| RAN1#105e |  |  |
| For UE subgroups indication in physical layer, maximum of 8 subgroups per PO is supported. | Not yet captured to wait for further progress. |  |
| RAN1#106e |  |  |
| N/A |  |  |
| RAN1#106bis-e |  |  |
| For NR Rel-17, paging indications to UE subgroups are carried only in PEI. | Captured in 7.y.0 |  |
| For PEI, a new DCI format is supported to include at least paging indications to UE group(s)/subgroups of the associated PO(s) - One bit in the DCI payload indicating one UE subgroup of a PO or one UE group/PO - The maximum number of total bits for paging indication field in PEI DCI format is x - One PEI can be configured to indicate up to 4 PO(s) in a PF - FFS whether to supporting map PEI to 3 POs in a PF - FFS: 1 PEI for POs across multiple PFs - FFS: value of x | Captured in 7.x.1 |  |
| A PEI occasion (PEI-O) is a set of S consecutive PDCCH monitoring occasions when nrofPDCCH-MonitoringOccasionPerSSB-InPO is not configured - S is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 - The K-th PDCCH monitoring occasion for PEI in the PEI-O has the same QCL assumption as that of the K-th PDCCH monitoring occasion for paging in the PO. - Note: QCL reference is SSB - FFS: Determination of the PEI-O location - FFS: Support of unlicensed spectrum operation with nrofPDCCH-MonitoringOccasionPerSSB-InPO configured | Captured in 7.x.1 |  |
| Agreement CORESET # 0 or commonControlResourceSet in SIB1 can be used for PEI - Note: The number of CORESETs configured for a UE follows the requirement of UE feature 3-1 Agreement Support configuration of a dedicated search space (‘peiSearchSpace’) for PEI - FFS: Configuration details and whether and how to reuse legacy search space sets, including pagingSearchSpace and searchSpaceSetZero | Captured in 7.x.1 |  |
| RAN1#107e |  |  |

Agreements on TRS/CSI-RS in idle/inactive

| RAN1 agreements | Impacted specification | Comments, if any |
| --- | --- | --- |
| RAN1#104e |  |  |
| N/A |  |  |
| RAN1#104bis-e |  |  |
| N/A |  |  |
| RAN1#105e |  |  |
| Agreement: Confirm the following working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling Agreement: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint - e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources - e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources - FFS maximum number of configured RS resources per physical layer availability indication to support. - FFS whether availability/unavailability information is for all or some of configured RS resources | Captured in Z.1 |  |
| Working assumption: Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected. - FFS whether and how to enable/disable L1 based availability indication configurable by SIB | Captured in Z.2 |  |
| RAN1#106e |  |  |
| N/A |  |  |
| RAN1#106bis-e |  |  |
| Agreement At least for paging PDCCH based L1 availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, the L1 availability indication is valid for a time duration starting from a reference point, where - the time duration is a validity duration configured by higher layer, - FFS applicable values, e.g. # of DRX cycles, or multiple of default paging cycle duration (i.e. modification period) - FFS UE doesn’t expect inconsistent L1 based indication during the time duration - the reference point for start of the validity duration is one of the following alternatives: - Alt1: SFN of the first PF from the next DRX cycle - Alt2: SFN of the first PF from the current DRX cycle where UE receives the indication - Alt3: based on SFN configured by higher layer, i.e. modification period configured as multiple of default paging cycle duration - Alt4: start of the PF for the PO where UE receives the indication - Note: the DRX cycle in Alt1 and Alt2 is the default paging cycle broadcast in SIB - Note: The SFN for the first PF is for (UE mod N) = 0, and can be calculated by (SFN + PF_offset) mod T = 0 - the time duration can be optionally configured by gNB - when the time duration is not configured, one of the following alternatives can be considered: - Alt1: the availability indication is valid until when the UE receives another availability indication. - Alt2: the availability indication is valid until L1 availability indication is changed by network - Alt3: default time duration e.g. default paging cycle - FFS whether and how to handle the miss detection issue of L1 signaling | Partially captured in Z.2 |  |
| RAN1#107e |  |  |