3GPP TSG-RAN WG2 Meeting #116e	Tdoc R2-21xxxxx

November 1st - 12th 2021

Agenda:	8.14

Source:	Ericsson

Title:	Summary of e-mail discussion [080] Mobility

Document for:	Discussion, Decision

# 1	Introduction

In this document the following e-mail discussion is handled:

[Post116-e][080][eQoE] Mobility (Ericsson)

Scope: Discuss whether RAN2 intends to fulfil the SA4 requirements related to mobility, what those requirements are (e.g. based on different case). Determine whether we need further clarifications by LS, and if so LS approval. In case there is need (in order to converge on mobility in general), the non-LS part of this discussion can continue in a long email discussion (and then the report is then for next meeting).

Intended outcome: Approved LS out, Report

Deadline: Short 2 (not for RP)

Contact information:

| Company | Contact Name, Email |
| --- | --- |
| Ericsson | cecilia.eklof@ericsson.com |
| Samsung | s90.jeong@samsung.com |
| vivo | panxiang@vivo.com |
| ZTE | Liu.yansheng@zte.com.cn |
| Huawei | dawid.koziol@huawei.com |
| Intel | ziyi.li@intel.com |
| Qualcomm | jianhua@qti.qualcomm.com |
| Apple | pnuggehalli@apple.com |
| OPPO | liuyangbj@oppo.com |
| China Unicom | gaos30@chinaunicom.cn |

# 2	Discussion

## 2.1	Possible LS to SA4

In RAN2#116e RAN2 discussed the SA4 requirements related to mobility, described in TS 26.247 clause 10.1. RAN3 previously sent an LS to SA4 (cc: RAN2) in R2-2106945 asking for clarification of the requirement and SA4 replied in R2-2109384. There were different views in RAN2 of what the reply means. An LS to SA4 for clarification could be one option to be able to proceed in RAN2. The requirement:

“The QoE configuration shall only be checked by the client when each session starts, and thus all logging and reporting criterias for an ongoing session shall be unaffected by any QoE configuration changes received during that session. This also includes evaluation of any filtering criterias, such as geographical filtering, which shall only be done when the session starts. Thus changes to the QoE configuration will only affect sessions started after these configuration changes have been received.”

- Question 1: If RAN2 would send an LS to SA4 for clarification of their requirement in TS 26.114, what do you think the question(s) should be?

| Company | Answer |
| --- | --- |
| Ericsson | We could ask SA4 to describe the wanted network and UE behaviour related to mobility and release, e.g. when the UE moves out of the area and there is an ongoing session. Also, we could ask about the expected behaviour if the network needs to release the measurements for some other reason. |
| Samsung | In RAN2#116e, RAN2 showed different views on whether or not ongoing session is affected in case of release of QoE configuration. In our understanding, in LS (i.e., R2-2109384), SA4 clarified "the logging and reporting criteria for ongoing session should be affected when the client receives a release of the QoE configuration.". However, there were some companies who have different understanding (i.e., ongoing session is NOT affected in case of release of QoE configuration). Therefor we are fine to ask SA4 to clarify this issue. |
| vivo | We would ask SA4 the following: - What is the intention to continue the QoE measurement when the UE moves out of the target Area Scope? - How to utilize the QoE measurement outside the target Area Scope? - If the intention is to collect the QoE at the edge of specific cells, is it feasible that just extend the Area Scope, i.e., adding the neighbour cells into the target Area Scope? - Whether SA4 has any concern about the current Area Scope checking solution of RAN side (without session start/end indication from UE). Worth noting that RAN side solution will not introduce spec impact on the current client behaviour, that is, the target Area Scope is checked by RAN node. - If the RAN node needs to check the QoE session state when UE moves out of the target Area Scope, the QoE session state may always be exposed to RAN node, which will introduce user privacy concerns, is it acceptable from SA4 perspective? |
| ZTE | How to handle a QMC session in the following scenario and explain their understanding: - UE moves out of the areascope with an ongoing QMC session - UE moves out of the areascope with a not ongoing QMC session(e.g. not activated QoE session, QoE session with paused QoE reporting flag). |
| Huawei, HiSilicon | There are two different aspects that need to be considered, related to two different questions from RAN3 posed in R2-2106945. It seems the confusion comes from the fact that these two aspects are mixed. First question from RAN3 was: Q1: Will the requirement for configuration changes of ongoing QMC sessions be applicable also for NR QMC?. This requirement comes from TS 26.247, clause 16.3 and it is quoted directly in the RAN3 LS, as follows: RAN3 is discussing, in the context of NR QMC, the following SA4 requirement described in TS 26.114 clause 10.1, TS 26.247 clause 16.3, which was defined for QMC in UMTS and LTE. “The QoE configuration shall only be checked by the client when each session starts, and thus all logging and reporting criterias for an ongoing session shall be unaffected by any QoE configuration changes received during that session. This also includes evaluation of any filtering criterias, such as geographical filtering, which shall only be done when the session starts. Thus changes to the QoE configuration will only affect sessions started after these configuration changes have been received.”. The yellow part refers to geographical filtering, i.e. checking area scope, and it says it should only be done when the session starts. Since the answer to this question from SA4 was simply: “Yes.”, it is clear that this requirement should be met and the gNB should not release an ongoing QoE measurement even if the UE moves out of the area scope. Then, the second question was about the UE behaviour when the release is received. RAN2 already agreed that the network may release QoE configuration any time and SA4 just confirms this. It does not mean that the gNB should release ongoing QoE configurations when the UE moves out of area scope as suggested by some companies. To us, the situation is clear based on the current reply: - In normal circumstances, gNB should not release the QoE configuration for ongoing QoE session, no matter whether UE is moving out of area scope or not (see reply to Q1 from SA4). - If the gNB sends release for QoE configuration, the QoE configuration is released by the UE, no matter whether it is ongoing or not (see reply to Q2 from SA4). We would prefer not to unnecessarily bother SA4 with another LS, but in case companies interpret this differently, then we are OK to ask to confirm bullet 1 above (bullet 2 seems clear). We should just focus on the SA4 preferred behaviour, no need to ask for motivations etc. |
| Intel | Agree with Ericsson’s comment and we are fine to check with SA4 for further clarification. |
| Qualcomm | We are fine to ask SA4, with the following questions 1) What is the intention to continue the QoE measurement when the UE moves out of the Area Scope? 2) It is noticed that SA4 specification already defines the application layer QoE configuration, including the location filter information will be provided to UE client, and UE client will check whether to initiate QoE session based on the location filter information. RAN2 would like to ask whether such application layer mechanism already satisfies the requirement “This also includes evaluation of any filtering criterias, such as geographical filtering, which shall only be done when the session starts.”; if no, what ‘s the different of RAN level geographical filtering and application layer level geographical filtering? |
| Ericsson | We also think we can ask the difference/relation between the area that the gNB has and the area defined inside the container. |
| CATT | About the area scope controlled in the container or out the container, in 26.247 and 26.114, SA4 mentioned as below: Note that if geographical filtering is handled on the network side (i.e. QoE reporting is turned on/off by the network depending on the UE location), no LocationFilter should be specified in the QoE Configuration, as this would mean two consecutive filterings. From the above quote, the area scope in container will be omitted if the NW handles the area scope. So we don’t need to ask question related to this issue. But one issue like as Huawei said as below: 1.In normal circumstances, gNB should not release the QoE configuration for ongoing QoE session, no matter whether UE is moving out of area scope or not (see reply to Q1 from SA4). 2.If the gNB sends release for QoE configuration, the QoE configuration is released by the UE, no matter whether it is ongoing or not (see reply to Q2 from SA4). These two behaviors may be conflicting when we use the NW handle the area scope solution. i.e. when UE moves out the area scope, the NW will send release to UE. If we follow the item2, the ongoing session will be released but the behavior does not obey the item1. So we may have two approach, one is RAN2 to specify the different release type. Another approach is that we state our solution about he area scope and check how to handle this issue from their specification with SA4 |

- Summary: Several questions were listed above, so it seems to be relevant to send an LS. An LS is drafted separately.

## 2.2	Phase 2

### 2.2.1	Phase 2 preparation

- In RAN2#116e RAN2 discussed the SA4 requirements

- There is also a second part of the e-mail discussion related to whether to fulfil the SA4 requirement and how to converge on mobility in general. Please provide feedback related to this part in question 2.

- Question 2: Is there something related to the SA4 requirement that could be discussed already now? Is there some other mobility issue that needs to be discussed?

| Company | Answer |
| --- | --- |
| Ericsson | We think we can discuss different solutions related to the SA4 requirements and the pros and cons of the different solutions. When the reply is received, we can decide what to do. We would like to discuss different options related to fullConfig from a technical point of view. |
| Intel | We suggest discussing the full solution of mobility issue, not limited to fullConfig. This includes procedure of normal handover, RRC Resume, RRC Setup including to legacy gNBs, Rel-17 gNBs not supporting the feature, e.g. what to be released/setup and how, forward compatibility, RRC ID handling at AS layer, whether is AS layer or application layer to be responsible for the release of application layer measurement report, etc. |
| CATT | Contribution driven for the mobility issue discussing. Forward compatibility should be the imported fact since the RAN-visible QoE will be discussed in RAN2 soon. |
| Ericsson | Agree to discuss various technical issues related to Mobility, that is also the part of the e-mail discussion. |

### 2.2.2	Phase 2 discussion

- In case the SA4 requirement needs to be fulfilled, the measurements may need to continue until the end of the session, even if the UE moves outside the configured area. The following solutions to achieve that behavior have been proposed:

- 1) Session start/stop indication. The UE sends an indication to the network when the session in the application layer starts and stops. The network then has information about ongoing sessions and can release the measurements at the right time.

- 2) The network sends release to the UE when the UE exits the area, but the UE doesn’t release the measurements until the session is completed. The UE needs to indicate to the network when the session is finally released, so that the network can release the measurement configuration also.

- 3) The network sends on indication to the UE when it exists the area, and presumably when it re-enters the area. Maybe needs to be combined with 2).

- 4) The LocationFilter inside the QoE configuration file is used by the application for area handling.

- 5) Network sends area scope to UE AS layer, and the UE AS layer checks whether UE moves outside of the area scope, and then indicates to application layer whenever UE moves outside or inside of the area scope.

- 6) Option 4)+Network-based release (as current RAN3 agreed). For those QoE configurations which require QoE session continuity, option 4) is applied; for those QoE configurations which don’t requires QoE session continuity, network-based release can be applied.

- Question 3: Please give technical comments to the solutions above (or propose new solution), e.g. issues, benefits, unclear things.

| Company | Answer |
| --- | --- |
| Intel | As defined in TS26.114 section 16.5.1, “If the MTSI client is informed that it is not inside the area, it shall not start any new QoE measurements even if it has received a valid QoE configuration container, but shall continue measuring for already started sessions.” However, it is not clear that whether the UE still needs to send the measurement report back to the network when it moves outside the configured area. From this aspect, we would like to wait for more information replied from SA4. If UE still needs to send the QoE measurement reports back to network even it moves outside of the area, the solution depends on where the geographical filtering is handled, e.g. network side or UE side. Case 1: When the geographical filtering is handled at network side, no LocationFilter will be specified in the QoE Configuration container, which will be sent to the UE. The QoE reporting is turned on/off by the network depending on the UE location. Hence, Solution 3) is preferred when area scope is handled by the network. Case 2: When the geographical filtering is handled at the UE side, that is, LocationFilter is included in the QoE configuration container and sent to the UE. The details of this needs confirmation from SA4. For example, can it be handled entirely within the application layer (option 4)? What is the UE expected to do with the results if it is outside of the area? When QoE measurement in application layer is finished when UE is outside of the area scope, should the UE send the session stop indication to the network and let the network decide whether and when to send QoE measurement report and when to release the corresponding QoE session? Should the result be retrieved when the UE is back in the area? However, there’s no need for the UE to send session start indication if UE moves outside of the area, considering new QoE session should not start when the UE moves outside of the configured area. |
| Apple | The solutions described above do not include the solution we proposed in R2-2110073 (submitted to R2-116e); namely, that the “the gNB provides explicit out-of-area indication to the UE while releasing QoE measurement configuration.” This can be thought of as extension of Alternative 2. We are opposed to the UE sending start/stop indications (Alternative 1) to the gNB because we are concerned about the leakage of unintentional and/or unauthorized information and the significant complexity at both UE and gNB. We are not sure why Alternative 3 needs to be considered since the same objective can be reached by releasing and activating QoE measurements based on area scope by the network. We need to wait for SA4 response before evaluating the feasibility of Alternative 4. |
| OPPO | Network based solution is preferred by us, especially considering that simultaneously multiple QoE measurement configuration and reporting are supported in 5G NR. UE based solution requires large UE memory space for storing different area scope for different QoE measurement configuration and more processing load. When UE leaves a certain area, it should be the network responsibility to check if UE enters an area outside the configured area scope and decides whether to transmit a out-of-area indication towards the UE. Upon reception of the indication, the UE should not trigger measurement for the corresponding QoE configuration. Whether or not to continue the measurement if already triggered depends on the SA4’s decision |
| Huawei, HiSilicon | It seems the companies forgot about the following RAN3 agreement: - Option 1 is agreed by RAN3 on area handling for QoE i.e. the network is responsible for keeping track of whether the UE is inside or outside the area and the network configures/releases configuration accordingly. The other two options were excluded already by RAN3 and these correspond to solution 3) (option 2) and solution 5) (option 3). Since only solution 1) is compatible with the previous agreement, we should follow it unless we identify a blocking issue or a major drawback which we currently do not see. When it comes to solutions reusing location filter, we should note that there is the following clarification in TS 26.247, Annex L.2: “Note that if geographical filtering is handled on the network side (i.e. QoE reporting is turned on/off by the network depending on the UE location), no LocationFilter should be specified in the QoE Configuration, as this would mean two consecutive filterings.” This means that it is up to the network choice/deployment whether to use location filter or rely on network based filtering (i.e. NG-RAN based filtering in this case). Solution 4) is already specified by SA4 while for RAN based filtering, solution 1) is needed. What we can clarify is that it should be possible to turn on/off session start/stop indications, e.g. in the case the network relies on location filter in application layer, they may not be needed. Solution 2) seems to have the same final result as solution 1). However, we do not think it is acceptable to allow the UE to ignore the release command from the network. |
| China Unicom | We support option 1. For UE-handling geographic filtering solutions, the disadvantages can be shown clearly as following: Firstly, for management-based QoE, UE-handling solutions will increase extra signalling interactions due to that network will send the configurations to the UEs which are not in the Area Scope. Secondly, UE-handling solutions will also increase extra signalling interactions and the complexity at handover. For example, network cannot directly release QoE sessions which are not ongoing at handover preparation phase, until UE accesses to the target gNB. And more specs impacts are required. So UE-handling geographic location solutions can be excluded. And according to the latest RAN3 agreements as below, UE won’t stop the measurement unless it received the release indication from the network. So it’s clearly enough that UE needs to send the measurement report back to the network when it moves outside the configured area. So RAN2 can decide this without RAN4’s reply. “A UE should continue an ongoing measurement once it leaves the Area, unless the network indicates to the UE to release the QoE configuration. The network is responsible for keeping track of whether the UE is inside or outside the area. ” According to RAN3 agreements above, the network (which means NG-RAN) is responsible for keeping track of whether the UE is inside or outside the area, thus the NG-RAN side is responsible to handle the geographical filtering. However, LocationFilter indication cannot be handled by the NG-RAN, which will not be aligned with RAN3 agreements. Besides, it is noted in TS 26.247 that LocationFilter should not be specified if geographical filtering is handled on the network side. Thus we support option 1 to use session start/stop indication instead of LocationFilter indication. # L.2 XML configuration When QoE reporting is configured via the QMC functionality, the configuration is done according to the XML schema below. The interpretation of the different elements and attributes are the same as described in clauses 10.4, 10.5 and Annex F in the current specification. Note that if geographical filtering is handled on the network side (i.e. QoE reporting is turned on/off by the network depending on the UE location), no LocationFilter should be specified in the QoE Configuration, as this would mean two consecutive filterings.. And I want to clarify that session start/stop indication means whether the session is ongoing or not, it doesn’t means new QoE session should start when the UE moves outside of the configured area. So UE need to send session start indication to the NG-RAN if UE moves outside of the area, which can ensure the continuity of mobility. |
| Ericsson | We agree RAN3 has agreed a network based solution and also further agreements in the direction of session start/stop indication. We are concerned about how alignment with MDT can work if the session start/stop indication is not standardized. The UE could potentially start (pre-configured) MDT measurements when a session starts, but there are also MDT measurements in the network side and the network will not know when to start these if no session start indication is received. We don’t think there can be any privacy issue with the UE indicating session start/stop, as there are time stamps for the sessions in the QoE report, so the same information is anyhow available to the network in the QoE report. We don’t think there could be any issue with high signalling load either, as the UE is anyhow starting/stopping a session, which means much more data being transferred, so an indication will not make any difference. The user data is being transferred in the user plane, but the session start/stop indication is very small and will not have much of impact to the amount of signalling. We thought option 3 above was the option proposed by Apple, that the network sends an out-of-area indication. We think this anyhow needs to be combined with option 2, or are we missing something? Just an out-of-area indication doesn’t give the UE enough information, the network needs to indicate when the measurements should be released also. And we think there needs to be two types of release in both option 2 and 3, one for immediate release and one for pending release when the UE first should finalize the ongoing measurement. Not sure what the out-of-area adds in addition, but maybe Apple or somebody else can explain. We think option 2 is network based and could work to fulfil the SA4 requirement, but the network needs to have the option to send immediate release also. However, the alignment with MDT will not work with this option as we understand it. |
| CATT | option 2 plus option1 is the better complete solution based on RAN3 agreements. For area scope handling, RAN3 already agree use the below network based solution on area handling for QoE i.e. the network is responsible for keeping track of whether the UE is inside or outside the area and the network configures/releases configuration accordingly. Send LS to RAN2 and SA4 informing RAN3 agreements. Also RAN3 agreed the UE should continue the ongoing session when it leaves area. A UE should continue an ongoing measurement once it leaves the Area, unless the network indicates to the UE to release the QoE configuration. From these agreements, we can derive the network behaviour for the area handling. When the UE handover from source node to target node. The source node will carry the area scope of the QoE configuration. If the target node doesn’t support QoE, RAN3 agree that In case of mobility to a target node not supporting QoE, the target node can release the QoE configuration If the target node support QoE, the target node will check whether itself in the area or not. Then configures or releases the QoE configuration. In general, the target node will release the configuration if target node is out of area or not support QoE. When UE receive the release, the configuration cannot be released immediately if the QoE is on-going. For the on-going status handling, we may have several solutions, - UE does not send the start indication to network. - UE sends the start indication to network as SA5 specification. Then the network has the information about if any QoE session is on-going. If we select a), the network can send the release command based on the network required. The UE itself decides how to handle the on-going session. It should be informed whether it is released for area handling. If we select b), the network have two alterative, one is same as we select the a). Another is like as option1 release the session when the on-going session is ended. In SA5 specification, the start indication is sending via SRB4 report message. There is no stop indication but use the QoE report. either use stop indication or QoE report to inform network the on-going session ended, the SRB4 configuration is needed.. So the option 2 should include option 1 for the QoE report sending from UE Compare the two solutions (option1 and option2), sending the release to UE in one message when it leaves area is better than sending several messages for the different sessions at different time. So we think option 2 plus option1 is the complete solution. |
| Apple_v2 | We would like to clarify our thinking (in response to Ericsson’s question). We think the network releases the QoE configuration after it determines that the UE is out of area scope. The network indicates to the UE that the QoE configuration is being released because of area scope considerations. The UE will then release the QoE configuration immediately (if no ongoing session) or release the QoE configuration when the current session ends. We don’t think the network needs to be informed of when the QoE session is actually released. As far as the network is concerned, it should consider the QoE configuration released. Of course, we can do all kinds of optimization but it is not clear to us that they are warranted. |

- A question raised is what needs to be maintained in the UE RRC layer and what is just forwarded to/from the application layer. In particular a question is whether the measConfigAppLayerId needs to be maintained in the RRC layer.

- Question 4: What QoE information needs to be maintained in the RRC layer and for what reason?

| Company | Answer |
| --- | --- |
| Intel | Firstly, we think it is important to have a clear and common understanding on what is stored in the AS – it is essential as it forms the basis of the any solution for reconfiguration such as during HO, full configuration. As RRC segmentation is a configuration at AS layer, and “Need M” is captured in current version of RRC running CR, rrc-SegAllowed-r17 needs to be stored as part of UE AS configuration. As for measConfigAppLayerId, we think it would be good to store it in the RRC layer considering it is configured by RRC, which provides more flexibility in terms of additional AS configurations in future releases that can be linked to this measConfigAppLayerId. Further, if measConfigAppLayerId is stored in AS layer, it makes the modelling and configurations during mobility (HO, full configuration, Resume, Re-establishment) easier. For example, to indicate explicitly which measConfigAppLayerId of certain QoE measurement need to be released during handover to a non-QoE-supporting gNB which can still comprehend ASN.1 (full configuration is not required in this case). However, during full configuration, it should be noted that all AS configurations including measConfigAppLayerId in AS layer) should be released. |
| Apple | We don’t have a very strong view. But we tend to agree with Intel’s view to maintain measConfigAppLayerId in RRC. |
| OPPO | Agree with Intel. measConfigAppLayerId should be maintained in RRC. |
| Huawei, HiSilicon | The meaning of the question is a bit unclear to us, i.e. “maintained” when? During mobility, full configuration? In general, the UE needs to store its QoE configuration, perhaps except for QoE container which, once delivered to the application layer, is not needed in AS layer. |
| China Unicom | Instead of the measConfigAppLayerId, other QoE configuration for one UE except the container needs to be maintained in the UE RRC layer. |
| Ericsson | We think the measConfigAppLayerId needs to be stored in UE RRC layer. The service type and QoE configuration can be discarded after they have been forwarded to the application layer. RAN Visible parameters may need to be stored also, we can see once we know which they are. |
| CATT | The measConfigAppLayerId should be stored in AS for the report transmission and configuration release. Also the session status which we discussed in Q3 need to maintained in AS. But I don’t think the AS need maintain the RAN-visible parameter except the RRC ID. |

- If a UE resumes or re-establishes in a gNB which e.g. does not recognize the UE context, the network may initiate an RRCSetup in response. The following is currently captured related to higher layers in the RRCSetup procedure:

#### 5.3.3.4	Reception of the RRCSetup by the UE

The UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1:

2>	discard any stored UE Inactive AS context and suspendConfig;

2>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release radio resources for all established RBs except SRB0, including release of the RLC entities, of the associated PDCP entities and of SDAP;

2>	release the RRC configuration except for the default L1 parameter values, default MAC Cell Group configuration and CCCH configuration;

2>	indicate to upper layers fallback of the RRC connection;

- When QoE measurements are released when the UE enters Idle the following is stated:

1> inform upper layers about the release of all application layer measurement configurations;

- Question 5: Is there any technical reason to add/not add the same handling at the beginning of the RRCSetup procedure as when QoE measurements are released?

| Company | Answer |
| --- | --- |
| Intel | We agree the same handling of RRCSetup should be followed by QoE, i.e. all AS layer configuration (including measConfigAppLayerId) should be released (as though the UE received an RRC Release message). As with RRC Release, AS layer should also inform application layer to release all application layer measurement configurations. For measurement reports, if UE’s AS layer has received some QoE measurement reports which haven’t been sent to the network, all measurement reports in AS layer should also be released. |
| Apple | We do not think there is any technical reason not to have the same wording. |
| Huawei, HiSilicon | As the UE releases its entire RRC configuration during fallback to RRC Setup, we agree that the UE should also indicate release of all application layer measurement configurations to upper layers and discard all unsent QoE reports. |
| China Unicom | We also agree the same wording can be add at the beginning of the RRCSetup procedure as when QoE measurements are released. |
| Ericsson | We think it would be clearer to add the same text at RRCSetup as when the UE enters Idle. It could easily be missed in implementation otherwise. |
| CATT | Agree with above |

- RAN2 agreed in RAN2#115 that the network explicitly indicates the measConfigAppLayerId’s of the QoE configurations that should continue at Resume. Later, most companies agreed in RAN2#116 offline [042], that the network does not need to explicitly indicate the measConfigAppLayerId’s at Resume with delta configuration as the QoE configurations are anyhow stored in the UE context and the network then only needs to indicate possible differences to the UE context.

- Question 6: Is there any technical reason for the network to explicitly indicate the measConfigAppLayerId of configurations that continues (i.e. which have no change in the configurations) at Resume with delta configuration?

| Company | Answer |
| --- | --- |
| Intel | We don’t think there’s any difference between Resume procedure for QoE and other configurations. Hence, we prefer to follow existing Resume procedure, which is to explicitly indicate the measConfigAppLayerId which need to be released as delta configuration, rather than what QoE configurations should continue. The current exception we have in the specification for Scell configuration etc. had to be done that way because these configurations were released in Rel-15 and we needed to have a mechanism to continue that using explicit signalling in gNB is a Rel-16 gNB and supports the feature. For QoE, this is not the case and all Rel-17 and future gNBs should be able to comprehend the Rel-17 ASN.1 and signal the release of the QoE configuration as is done with any another RRC configuration today. |
| Apple | No technical reason. Of course the network has to indicate the measConfigAppLayerId that needs to be released. |
| Huawei, HiSilicon | In our understanding the reason to explicitly indicate QoE configurations that should be resumed is to limit the impact on gNBs not supporting QoE. Such gNBs could indicate nothing and the QoE configuration would be then released. But such gNBs could also just implement release of QoE configuration, which should not be a big burden, so perhaps there is no need to diverge from normal delta configuration in this case. |
| China Unicom | We agree with that the network to explicitly indicate the measConfigAppLayerId of configurations that continues (i.e. which have no change in the configurations) at Resume with delta configuration. So that QoE configurations not indicated by the network (including both gNBs supporting QoE or not supporting QoE) can be released. |
| Ericsson | We don’t see any reason for the network to explicitly indicate the measConfigAppLayerId at delta configuration. If the target supports QoE, nothing needs to be sent as the UE anyhow has the QoE configurations and only possible differences need to be sent. If the target does not support QoE and doesn’t understand the QoE configuration, the network cannot send a delta configuration. Instead, a fullConfig will be triggered and the measurements will be released if no measurement is indicated. If the target does not support QoE, but somehow understands the QoE configuration, it can release them, otherwise it could trigger a fullConfig. |
| CATT | We share with intel |

- In existing RRC specification, at fullConfig the radio configurations in the UE are released, but not higher layer configurations such as application layer measurements:

#### 5.3.5.11	Full configuration

The UE shall:

1>	release/ clear all current dedicated radio configurations except for the following:

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the SRB1/SRB2 configurations and DRB configurations as configured by radioBearerConfig or radioBearerConfig2.

- Currently the application layer measurements are not released, which means that the measurements will continue in the application layer until the UE enters Idle. The reports will be discarded by the RRC layer according to RAN2 agreement, as long as the network doesn’t configure the same measConfigAppLayerId again.

- The fullConfig may be triggered if the target node doesn’t recognize something in the UE context (not necessarily the QoE part of the context) or for other reason.

- Some alternatives for fullConfig:

- 1) No change to current specification. The measurements will be hanging until the UE enters Idle or until the network configures the same measConfigAppLayerId again. This is the case in LTE.

- 2) The measurements are always released at fullConfig. This solution was rejected for LTE in RAN2#116 (see R2-2111148 and R2-2111149) ) as the QoE measurements cannot continue at any resume/handover/re-establishment with fullConfig in such case. The following (or similar) would need to be added to the CR:

The UE shall:

1>	release/ clear all current dedicated radio configurations except for the following:

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the SRB1/SRB2 configurations and DRB configurations as configured by radioBearerConfig or radioBearerConfig2.

[..]

1>	else (full configuration after re-establishment or during RRC resume):

2>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

1>	inform upper layers about the release of all application layer measurement configurations;

1>	discard received application layer measurement report information from upper layers;

1>	consider itself not to be configured to send application layer measurement report.

- 3) The measurements are released if the network doesn’t indicate any measConfigAppLayerId, i.e. if the target node cannot understand the QoE configurations. If the target node understands QoE configurations, the measurements can continue. The following (or similar) would need to be added to the CR:

The UE shall:

1>	release/ clear all current dedicated radio configurations except for the following:

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the SRB1/SRB2 configurations and DRB configurations as configured by radioBearerConfig or radioBearerConfig2.

[..]

1>	else (full configuration after re-establishment or during RRC resume):

2>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

1>	if no measConfigAppLayerId is indicated in the measConfigAppLayerToAddModList:

2>	inform upper layers about the release of all application layer measurement configurations;

2>	discard received application layer measurement report information from upper layers;

2>	consider itself not to be configured to send application layer measurement report.

- Question 7: Please give technical comments to the options above (or additional options), e.g. technical issues, advantages or improvements.

| Company | Answer |
| --- | --- |
| Intel | First of all, we would like to clarify that full configuration is normally used when the target gNB cannot comprehend ASN.1 from the source gNB. For example, a legacy target gNB from previous release. For other gNBs which can comprehend ASN.1 but not supporting QoE, full configuration should not normally be triggered and the target gNB can simply indicate the measConfigAppLayerId to be released if QoE is not supported. We think Solution 2) should be supported as baseline in order to support mobility to a target Rel-15 gNB. During full configuration, all measConfigAppLayerId as part of UE AS context are released. It is also better to release the application layer configuration – otherwise there is a risk of collision if same measConfigAppLayerId value is re-used afterwards for another QoE measurement, as source gNB will also release everything during full configuration. This means the mapping table between QoE reference and measConfigAppLayerId cannot be forwarded to the target gNB. The target gNB may use the same measConfigAppLayerId to represent different QoE reference. For that scenario, we will need to discuss and agree solutions to avoid such collisions (hence we don’t think solution 1 is acceptable). Then, on the use Full configuration without releasing the application layer configuration: it could potentially be useful if the network can comprehend the ASN.1 but still decides to Full configuration for other implementation specific reasons while trying to preserve the application layer QoE configuration. But such configurations should not happen frequently (for reasons given above) and we are not convinced it is essential. If a solution is to be defined for this, then Solution 3) maintains the basic concepts of Full configuration in that the entire AS configuration is released by default unless explicitly indicated). In our understanding of solution 3, if the network wants to continue with certain QoE configuration in the application layer, it has to provide the list of measConfigAppLayerIds that is to be maintained and released in the message with Full Config indication. This seems to work. However, the normal principle we follow (for example with EPS bearers in LTE), is that the network only provides the list of IDs to continue (and not the list of IDs to release). Such an approach is more forward compatible, for example, if the ID list is extended in the future. Hence, if RAN2 is to define the optimisation to keep QoE configuration during Full configuration, while solution 3 is acceptable, we have a preference to follow the principle used for LTE EPS bearers where the target only provides the list of IDs to continue. |
| Huawei, HiSilicon | We agree with Intel that option 1) is prone to errors and leads to misalignment between application layer QoE configuration and AS layer QoE configuration which should be avoided. The release of all QoE configuration during full configuration should be then possible and we think option 3) addresses this in the optimal way, i.e. it also allows to keep QoE configurations (and not disturb QoE sessions unnecessarily) in case the target gNB supports QoE. We think we cannot directly reuse the way we handle EPS bearers during full config as the QoE configuration in the target gNB may differ from the one in the source gNB, e.g. the target gNB may need to release one QoE configuration while configuring another one in its place. Hence, what is proposed in solution 3) is a preferred way. |
| China Unicom | As we illustrated in Q6, QoE configurations not indicated by the network can be released, so we support option 3. |
| Ericsson | We think both option 1 and 2 have issues. We would prefer option 1 in such case, as option 2 has too large negative impact on mobility. In option 1 the measurements will be released eventually. But we think it is quite clear to take option 3 as that one doesn’t seem to have any issues connected with it. We think the option proposed by Intel also works, with the assumption that the network doesn’t reuse the same Id for a new configuration at handover (we think this is the case for EPS bearer also). But several UE vendors had concerns about storing the measConfigAppLayerId at fullConfig, as they prefer to clear the RRC configurations at fullConfig. The EPS bearers are exemptions from that and the measeConfigAppLayerId would be another exemption in such case. |
| CATT | We think the option3 is good approach |

- The transmission of QoE reports over SRB4 is slower than transmission of other RRC messages and a handover may occur during the transmission.

- Question 8: What should the UE do if a handover occurs during the transmission of a QoE report, e.g. restart the transmission in target, continue the transmission in target, discard the transmission etc?

| Company | Answer |
| --- | --- |
| Intel | Whether the UE should continue sending/restart QoE report should depend on whether the target node. If full configuration is considered, the UE should discard the transmission. If the target gNB also supports QoE and the corresponding measConfigAppLayerId is not released during reconfiguration, UE can restart the transmission in the target gNB. |
| Apple | We think this may not be such an important scenario, and prefer to just drop the report. |
| OPPO | If the UE resume the transmission of the QoE measurement report collected previously at source gNB when it finishes the handover towards the target gNB, the network may use the content for optimization of radio service at the target gNB. So we suggest UE to drop the report when it starts the handover procedure until successfully finishes the handover. |
| Huawei, HiSilicon | If the related QoE configuration still exists after the handover, the UE may resend the unacknowledged QoE report. This may lead to duplicate reports, but that is probably something that can be dealt with during post-processing in OAM system. Dropping the report means that the measurement session is incomplete and such sessions are less useful. |
| China Unicom | If a handover occurs during the transmission of a QoE report, we support UE to restart the transmission in target to guarantee the integrity of QoE reports. |
| Ericsson | We think the UE can restart the transmission in the target node, to ensure the network gets the report. Continuing (i.e. not restarting) the transmission in target would be very complex. |
| CATT | Restart the report transfer if the target node supporting |

# 3	Conclusion

# 4	References

R2-2109565, QoE configuration, reporting and mobility, Qualcomm Incorporated, RAN2#116e, e, November 2021

R2-2109662, QoE measurement configuration and general aspects, Intel Corporation, RAN2#116e, e, November 2021

R2-2109832, Further discussion on transmission of QoE reports, Lenovo, Motorola Mobility, RAN2#116e, e, November 2021

R2-2109866, Configuration and reporting of QoE measurements, Ericsson, RAN2#116e, e, November 2021

R2-2109867, QoE measurements at handover, resume and re-establishment, Ericsson, China Unicom, RAN2#116e, e, November 2021

R2-2109984, Discussion on QoE configuration, vivo, RAN2#116e, e, November 2021

R2-2110073, Supporting mobility for NR QoE, Apple, RAN2#116e, e, November 2021

R2-2110099, Discussion on QoE measurement collection in NR, OPPO, RAN2#116e, e, November 2021

R2-2110605, Discussion on QoE measurement configuration and reporting, Huawei, HiSilicon, RAN2#116e, e, November 2021

R2-2110720, QoE configuration handling, Nokia, Nokia Shanghai Bell, RAN2#116e, e, November 2021

R2-2110991, Discussion on NR QoE configuration, ZTE Corporation, Sanechips, RAN2#116e, e, November 2021

R2-2110993, Discussion on NR QoE configuration, CATT, RAN2#116e, e, November 2021

R2-2111062, Remaining issues on configuration and reporting, CMCC, RAN2#116e, e, November 2021

R2-2111132, QoE configuration in general aspects, Samsung, RAN2#116e, e, November 2021

R2-2111133, RRC segmentation for QoE configuration and report, Samsung, RAN2#116e, e, November 2021

R2-2111188, Discussion on NR QoE measurement and configurations, China Unicom, RAN2#116e, e, November 2021