3GPP TSG-RAN WG2#116bis-e	Draft R2-2200057

Online, 17 - 25 January 2022

Source: 	Huawei (rapporteur)

Title: 	[Post116-e][225][R17 DCCA] Remaining details for SCG deactivation

Agenda Item:	8.2.2.1

Document for:	Discussion and decision

# 1	Introduction

This document is the summary of the following email discussion:

[Post116-e][225][R17 DCCA] Remaining details for SCG deactivation (Huawei)

Scope: List and discuss any remaining FFSs for the SCG deactivation, including at least how to handle RLF/BFD and RRM while SCG is deactivated.

Intended outcome: discussion summary

Deadline:  December 17th, 0900 UTC

Companies are invited to indicate the person providing input in the table below:

| Company | Name / Email |
| --- | --- |
| Intel | xun.tang@intel.com |
| Ericsson | stefan.wager@ericsson.com |
| NEC | hisashi.futaki@ nec.com |
| Futurewei | Jialinzou88@yahoo.com |
| Qualcomm | punyaslo@qti.qualcomm.com |
| LGE | hassium.kim@lge.com |
| Spreadtrum | Lifeng.Han@unisoc.com |
| NTT DOCOMO | riki.ookawa.rp@nttdocomo.com |
| MediaTek | Chun-fan.tsai@mediatek.com |
| Sharp | kyosuke_inoue@ sharp.co.jp |
| Lenovo, Motorola Mobility | zhangcc16@lenovo.com |
| CATT | Chandrika@catt.cn |
| KDDI | ya-li@kddi.com |

# 2	Discussion

## 2.1	UE actions upon Radio Link Failure

RAN2 agreed that the network can configure the UE to perform RLM while the SCG is deactivated but did not agree the UE behaviour when RLF is detected.

There are the following proposals for the UE behaviour upon SCG RLF while the SCG is deactivated:

1)	the UE does not report anything

2)	the UE reports SCGFailureInformation (legacy procedure) and the network can reconfigure the UE to release the SCG, change the PSCell or keep the PSCell and reconfigure RLM RS

Q1: Which option do companies prefer for UE action upon RLF while the SCG is deactivated?

| Company | Option (1/2/other) | Comments |
| --- | --- | --- |
| Intel | Option 2 | It’s necessary to let network know the PSCell cannot be activated successfully due to SCG RLF. |
| Ericsson | 2 | Legacy procedure can be reused since SCGFailureInformation can be transmitted via MCG. |
| NEC | Option 2 | We see no reason not to report the SCG RLF, given that the UE has detected the SCG RLF and can report the information via the existing mechanism. |
| Futurewei | Option 2 | RAN2 agreed to maintain the existing RLF for deactivated SCG. There is no change expected w.r.p.t existing RLF procedure. |
| Qualcomm | Option 2 | We think UE should report the failure because it indicates to the network that the configured current and neighbor beams are no longer providing coverage. The network can then reconfigure the UE with an updated set of beams and RLM RSs to monitor if it decides to keep the PSCell. Providing updated set of beams implies providing updated TCI states for PDCCH/PDSCH reception upon SCG activation. |
| LGE | 2 | It would be beneficial for the UE to prevent the case that the network commands the UE to reactivate a failed SCG. Besides, because the failure report will be transmitted to active MCG, we think there is no reason to limit the transmission of SCG failure information. |
| Spreadtrum | Option 2 | Since the UE performs RLM for the PSCell when SCG is deactivated, the UE shall report the RLF to the network through legacy procedure. |
| NTT DOCOMO | 1 | In our understanding, RLM and BFD during SCG deactivated state is solely required for UE to evaluate whether TCI is valid upon SCG activation is indicated so that UE can properly determine to perform RACH-less SCG activation or not. Above target can be achieved without any reporting, so UE should avoid extra power consumption for reporting. |
| MediaTek | Option 2 |  |
| Sharp | 2 | We think it is efficient to report SCG failure information as legacy. |
| Lenovo, Motorola Mobility | Prefer option 1 | Since SCG is deactivated at that moment, even if RLF is detected, does not mean the channel quality will not recover when SCG is activated. In either case, upon SCG activation, UE needs to do RACH to a new PSCell reconfigured by the gNB or to the same PSCell after RLF is detected. Besides, SCG reconfiguration and PSCell change based on RRM measurement is supported, so it does not seem necessary to report SCG failure to NW when it is detected during SCG deactivation. |
| CATT | Option 2 | In order to activate a functional SCG to reduce the activation delay, the network would need to know the SCG failure. |
| KDDI | Op2 | The UE should report the SCG RLF to MN, but whether MN reconfig the UE immediately, or via SCG is deactivated, should be further discussed. |
| OPPO | Option 2 | Legacy behaviour should be not changed due to SCG deactivation. |
| Nokia | Option 2 with reservation | we agree with Docomo and Lenovo that it is not really critical to report SCG failure to MCG as anyway regular RRM measurements continue and those can be used for same purpose. But as preventing that would require extra work we are fine to report it but if there is need to do impact specification then it would be just simpler not to report it as it is unnecessary activity. |
| Huawei, HiSilicon | Option 2 |  |

Summary: 16 companies provided views. 14 companies prefer option 2, 2 companies prefer option 1.

The arguments for option 1 are that it would waste UE power, RRM reports could be sufficient and it is not necessary to determine whether RACH-less activation are possible.

The arguments for option 2 are that SCG failure information is an existing UE behaviour, it is necessary for the network to know that activation of the current PSCell is not possible (even with RACH, i.e. RLM is not specifically for RACH-less access) and reconfigure the UE.

As it is an existing procedure and there is a large majority, the rapporteurs suggests following the majority view.

[Easy] Proposal 1: upon SCG RLF while the SCG is deactivated, the UE reports SCGFailureInformation (legacy procedure) and the network can reconfigure the UE to release the SCG, change the PSCell or keep the PSCell and reconfigure RLM RS.

## 2.2	UE actions upon beam failure

RAN2 agreed that the network could configure the UE to perform BFD on the PSCell while the SCG is deactivated, but did not agree the UE behaviour when BFD is detected.

There are the following proposals for the UE behaviour upon BFD on the PSCell while the SCG is deactivated:

For BF, there are the following proposals:

1)	do nothing (i.e. no RACH, no report via MAC CE or RRC message)

2) initiate RACH towards SCG without activating the SCG

3)	send a BFR MAC CE to the MCG

4)	send an SCGFailureInformation message with a new cause and the network can reconfigure the UE to release the SCG, change the PSCell or keep the PSCell and reconfigure BFD RS

Q2: Which option do companies prefer for UE action upon PSCell beam failure while the SCG is deactivated?

| Company | Option (1/2/3/4/other) | Comments |
| --- | --- | --- |
| Intel | Option 1 with updates | Do nothing in case of beam failure while the SCG is deactivated. When SCG is activated later, UE can perform RACH for beam failure recovery. |
| Ericsson | 4 | It is good to inform the network about the BFD so that it can take necessary action. Initiating RA towards SCG is not necessary nor appropriate if SCG is deactivated and there is otherwise no data to transmit on SCG. The BFR MAC CE is currently only for SCell BFD and is cell group specific as all other MAC procedures. We should keep it that way and not mix MCG and SCG MAC procedures. Thus, 4 is the best alternative, with a new cause value included in SCGFailureInformation message. |
| NEC | Option 1 | Unlike the SCG RLF, there is no need to do any specific action for this one-shot beam failure. If it is consistent, then SCG RLF is anyway triggered. |
| Futurewei | Option 1 | RAN2 agreed to allow UE to monitor BF is for assisting UE to decide whether to perform RACH-less access at the activation. It is not intended to conduct beam management/beam recovery for deactivated SCG. Beam level reporting will un-necessarily increase the UE power consumption and signalling overhead during the deactivation of the SCG. |
| Qualcomm | Option 4 | We think UE should report PSCell beam failure for similar reasons as for PSCell RLF (please see our response to Q1). |
| LGE | 3 and 4 | We strongly oppose option 1. RAN2 has already agreed to perform RLM and BFD while SCG is deactivated. Checking BFD and not taking any action from the UE definitely wastes the UE power only w/o any gain. To prevent causing any delay in a situation where data transmission is required, i.e. SCG reactivation, the UE should quickly detect BFD and inform the network before SCG reactivation, or the UE try to solve BFD by self-recovery, i.e. RACH. For that reason, we believe that option 1 should not be chosen. Among the options, considering that the UE can frequently detect BFD in cell deployments such as FR2, we think that options 3 and 4 are most appropriate to maintain SCG deactivation and notify the network. |
| Spreadtrum | Option 1 | There is no need to perform BFR when SCG is deactivated. |
| NTT DOCOMO | 1 | UE can properly determine to perform RACH-less SCG activation or not without any reporting. |
| MediaTek | Option 1 | We have similar understanding as Futurewei that the BFD is for RACH-less operation and there is no need to have complicate beam management/recover procedure for deactivated SCG. If companies think NW should be informed, we suggest to consider BFD as RLF and sending SCG failure information to NW (i.e. option 4 without new cause value). |
| Sharp | 1 | In order to ensure power consumption reduction, it is desirable not to perform RACH/ send via MAC CE for BFR in deactivated SCG. |
| Lenovo, Motorola Mobility | option 1 | Similar comment as in Q1 |
| CATT | Option 3 or Option 4 | For option 3, NW can get the beam failure information of the deactivation SCG, based on these, NW can command UE to perform BFR for deactivation SCG to keep UE have one valid beam even on deactivation SCG, so upon SCG activation, UE can perform RACH-Less SCG activation. But whether the information is reported via MAC CE or other RRC signalling can be further discussed. For option 4, in order to activate a good DL beam, some assistance information could be reported to network in SCGFailureInformation message. |
| KDDI | Op1 | Share the view as NEC |
| OPPO | Option 1 |  |
| Nokia | Option 1 | There is no use to do BFR when SCG is deactivated. Once SCG is activated. in fact UE could even recover BFD while SCG is deactivated prior SCG activation thus it is even totally useless and harmful to do any BFR while SCG is deactivated. |
| Huawei, HiSilicon | Option 3 or Option 4 | We share the similar view as Ericsson, LGE and CATT that the declaration of beam failure can assist network to decide if RACHless SCG activation is possible, otherwise it would be better to provide RACH resources if possible for faster SCG activation. |

Summary: out of 16 companies, 11 companies prefer option 1, 3 companies prefer option 3 or 4, 2 companies prefer option 4 (so 5 companies would support option 4). Based on this, the rapporteur only considers option 1 and option 4.

The argument for option 1 is that companies think BFD is only used for the UE to decide if RACH-less is feasible or if RACH must to be performed upon SCG activation.

The argument for option 4 is that it allows the network to either provide RACH resources at SCG activation, or to change the BFD RS to allow RACH-less activation.

In the rapporteur understanding:

-	in option 1, if the TAT is still running, the network will not include reconfigurationWithSync in the SCG RRC message for SCG activation and will expect the UE to monitor PDCCH. So the network will start DL transmission and/or allocate uplink grant without knowing that the UE is actually not monitoring PDCCH. The network can only understand the situation when it gets msg3 (the UE ID in CBRA).

-	in option 4, on the network side; knowing that there was BF is sufficient to allocate RACH resources at activation, knowing the good beams (from SCG failure information) allows reconfiguring BFD RS (if possible) for later RACH-less activation.

-	in both option 1 and option 4, MAC specification must be changed not to trigger BFR

-	in option 4, RRC specification must also be changed to trigger the SCG failure information and set a new cause (no other change)

The rapporteur tries to clarify the options and invites companies to further think about this (and indicate if they have a different understanding).

[For discussion]Proposal 2: Select one of the following options upon BF while the SCG is deactivated:

1)	no report, the UE will do CBRA in case of SCG activation without reconfigurationWithSync

2)	send an SCGFailureInformation message with a new cause, the network can reconfigure the UE to keep the SCell and allow RACH-less activation (by changing BFD RS), change the PSCell or release the SCG. If the network does not reconfigure the UE and activates the SCG, RACH will be used.

## 2.3	Configuration of RLM/BFD for SCG deactivated state

In the latest running CR [1]:

-	an explicit indication is configured by the SN to indicate that the UE performs RLM while the SCG is deactivated, when this indication is not configured, the UE does not perform RLM while the SCG is deactivated

-	when the indication is configured, the resources for RLM are the same like the existing configuration of RLM/BFD for the SCG activated state as in RadioLinkMonitoringConfig.

In Rel-15/16:

-	RadioLinkMonitoringConfig can provide RS explicitly for RLM/BFD and if no RS are provided, the UE uses activated TCI states for PDCCH reception;

-	the UE always performs RLM on the PSCell (there is no way to indicate to the UE not to perform RLM on the PSCell).

In [6] it is proposed that if the SN does not configure RS for the purpose set to "rlf", the UE does not perform RLM while the SCG is deactivated. The proposal apparently implies that RLM based on activated TCI states is not supported while the SCG is deactivated.

The justification of such a proposal might be that it is not possible to change the activated TCI state while the SCG is deactivated. However, there is also a proposal in [8] that, tci-Info, which in [1] can be used at SCG activation to provide TCI state for PDCCH/PDSCH reception, can be used in any reconfiguration while the SCG is deactivated, making it possible to change the activated TCI sate for PDCCH reception.

Q3: Do companies think that, while the SCG is deactivated, RLM can be based on activated TCI state for PDCCH reception when RadioLinkMonitoringConfig does not provide any RS for "rlf" or "both", like currently for the activated SCG?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| Ericsson | Yes | Same principle should apply as for activated. As we mentioned in [8], allowing TCI state update (transmitted to the UE in RRCReconfiguration via MCG) at any point while the SCG is deactivated allows the UE to perform BFD on the updated TCI state information and report possible issues before the SCG activation. Since the round trip delay for sending the BFD via MCG to the SN and the SN updating the TCI state to the UE via MCG is increased compared to the activated SCG case, there is an increased risk that the TCI state is outdated. In this case providing the updated TCI state only in the SCG activation message, it will lead to increased SCG activation delay as the UE will first try the provided TCI state and when that fails will trigger RA. If the network gets the BFD already while the SCG is deactivated, it can decide to trigger RA from the start in the SCG activation command. |
| NEC |  | We think technically it seems possible, while for deactivated SCG, simpler approach should be sufficient, i.e. network should ensure RadioLinkMonitoringConfig provides target RS for “both” (or “rlf” at least). |
| Futurewei | Yes |  |
| Qualcomm | Yes |  |
| LGE | Yes | We support the legacy behavior even if the network command RLM in SCG deactivation w/o RS configuration. |
| Spreadtrum | Yes |  |
| NTT DOCOMO | Yes |  |
| MediaTek | Yes but no strong view | We think the basic behavior is that NW configure RLM RS in RadioLinkMonitoringConfig and the UE perform RLM based on it (no need for TCI state). It is not a must to have RLM based on TCI state in SCG deactivation, but we are fine to support it. If supported, it goes together with Q5 that TCI state should be updated by NW via MCG. |
| Sharp | Yes | Our understanding is that UE should perform RLM based on the activated TCI state of the previously activated BWP. |
| Lenovo, Motorola Mobility | Yes |  |
| CATT | Yes |  |
| KDDI | Yes |  |
| OPPO | Yes | Legacy behaviour should be not changed due to SCG deactivation. |
| Nokia | Yes with reservation | We would expect that one agree tci-Info addition as proposed in [8]. Otherwise existing behaviour (in the draft CR) is not really useful. |
| Huawei, HiSilicon | Yes |  |

Summary: 16 companies provided views. All companies agreed that while the SCG is deactivated, RLM can be based on activated TCI state for PDCCH reception when RadioLinkMonitoringConfig does not provide any RS for “rlf” or “both”, like currently for the activated SCG.

[Easy]Proposal 3: while the SCG is deactivated, RLM can be based on activated TCI state for PDCCH reception when RadioLinkMonitoringConfig does not provide any RS for “rlf” or “both”, like currently for the activated SCG.

Q4: Do companies agree that and explicit indication is needed in order to indicate that the UE shall perform RLF while the SCG is deactivated? (if no, please indicate how exactly the UE is configured to perform or not perform RLF)

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes | RAN2 already agreed the following: The UE performs RLM and BFD on PSCell while the SCG is deactivated if network configures it. |
| Ericsson | Yes | We agreed already in RAN2#115e that whether RLF is performed shall be based on network configuration. The field bfd-and-RLM in running 38.331 CR is included for this purpose. |
| NEC | Yes | This indication should be explicit, which is clear and simple. |
| Futurewei |  | We are open to this. If the UE can be informed whether to perform the RLM with existing mechanism, e.g. using the TGI states indication, the same purpose of explicit indication is achieved. It would be better than to introduce new standard change. |
| Qualcomm | Yes | To clarify, we think UE behavior should be: - If explicit indication, as in the running CR [1], is provided, UE should perform RLM as per RadioLinkMonitoringConfig or based on activated TCI states. The activated TCI states may be updated while the SCG is deactivated, as discussed above by the rapporteur. - If there is no explicit indication, UE does not perform RLM in the deactivated SCG. |
| LGE | Yes |  |
| Spreadtrum | Yes | The explicit indication can be used. |
| NTT DOCOMO | Yes |  |
| MediaTek | Yes | Explicit indication is needed to avoid ambiguity |
| Sharp | Yes | bfd-and-RLM indicates whether RLM is performed or not and this is valid even if RS for the purpose set to "rlf" is not configured. |
| Lenovo, Motorola Mobility | Yes |  |
| CATT | Yes |  |
| KDDI | Yes |  |
| OPPO | Yes |  |
| Nokia | Maybe | Fine with explicit indication |
| Huawei, HiSilicon | Yes |  |

Summary: 16 companies provided views. 15 companies agree that explicit indication is needed. And 1 company is open to discuss other alternatives. For simplicity, the rapporteur suggests to agree to introduce an explicit indication.

[Easy] Proposal 4: Keep the existing indication in the 38.331 running CR to indicate whether the UE shall perform RLM while the SCG is deactivated.

Q5: Do companies support that tci-Info, which can provide activated TCI states for PDCCH/PDSCH reception at SCG activation (i.e. transition from deactivated SCG to activated SCG), can also be used at any RRC reconfiguration while the SCG is deactivated?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| Ericsson | Yes | See our comment in Q3. |
| NEC |  | We’re not sure about the question.. Is the question intended to ask “tci-Info, …, can also be changed at any RRC reconfiguration while the SCG is deactivated“? If that is the intention, then we think yes. it can be changed while the SCG is deactivated. |
| Futurewei | Yes |  |
| Qualcomm | Yes | This flexibility seems needed so that network can indicate updated beams while in SCG deactivated. |
| LGE | Yes | We think tci-info can be provided after the reception of SCG failure information due to RLF or BFD while SCG is deactivated. The updated tci-info can be helpful for the UE to obtain more chances to perform RACH-less activation on SCG. |
| Spreadtrum | Yes |  |
| NTT DOCOMO | No, but | We wonder how the network “traces” valid TCI. We think providing tci-Info in deactivated state is reasonable only when the network knows which TCI UE uses. (But if it can be done, yes.) |
| MediaTek | Yes | See also our comment in Q3. This is needed if R2 decide to support RLM based on TCI state in SCG deactivation. |
| Sharp | No | There are some impacts on the procedure of BWP switching without BWP activation, so we prefer that tci-Info is included in SCG activation only. |
| Lenovo, Motorola Mobility | Yes |  |
| CATT | Yes | We think it doesn’t need to restrict the tci-Info can only be configured upon SCG activation, even the tci-Info is sent to the UE when deactivation SCG, no additional power is introduced. And if we can assume the NW can provide an valid tci to UE upon activated the SCG, we can also assume the NW can provide an valid tci to UE while it is in deactivation SCG. |
| KDDI |  | Share the same question as NEC. |
| OPPO | Yes |  |
| Nokia | Yes | If this is not agreed then probably we would need to revisit many other decisions regarding BFD/RLM. |
| Huawei, HiSilicon | Yes | In general, we agree that normal RRC reconfiguration is allowed on deactivated SCG. It seems no need to exclude TCI state reconfiguration. However, we also would like to clarify that even after TCI reconfiguration when SCG is deactivated, no new UE behaviour is introduced. |

Summary: 16 companies provided views. 12 companies support RRC reconfiguration when SCG is deactivated. 1 companies answered no considering the potential impact on BWP switch. 1 companies answered no based on the understanding that network is unable to trace the valid TCI state used by UE. 2 companies would like to clarify whether "can be indicated" means "can be changed".

The rapporteur understands that the use case for TCI state reconfiguration on deactivated SCG is for implicit RLM and/or BFD RS configuration, in which the UE monitors the RS of activated PDCCH TCI states, as is agreeable to all companies according to Q4 and Q6.

With respect to "can be changed", the intention of "can be indicated" is "can be included" and whenever some parameter is included, the network is free to set any value, so it can be changed.

With respect to BWP switching, the rapporteur understands that the concern expressed is related to the possibility of changing the active BWP while the SCG is deactivated, which affects RLM/BFD and possibly RRM. If there is any issue with switching the BWP, some restriction could be decided.

With respect to "how the network traces valid TCI", the rapporteur understands that this question is applicable while the SCG is deactivated, regardless whether or not the SCG is to be activated.

Based on the wide majority, the rapporteur would like to make the following proposal:

[Hopefully easy]Proposal 5: tci-Info, which can provide activated TCI states for PDCCH/PDSCH reception at SCG activation (i.e. transition from deactivated SCG to activated SCG), can be included at any RRC reconfiguration while the SCG is deactivated and, if SCG remains deactivated and the UE performs BFD and/or RLM based on activated TCI states for PDCCH reception, the UE uses the newly activated TCI states for PDCCH reception.

In [6] it is also proposed that if the SN does not configure RS for the purpose set to "beamFailure", the UE does not perform BFD while the SCG is deactivated. The proposal apparently implies that BFD based on activated TCI states is not supported while the SCG is deactivated.

Q6: Do companies think that, while the SCG is deactivated, BFD can be based on activated TCI state for PDCCH reception when RadioLinkMonitoringConfig does not provide any RS for "beamFailure" or "both", like currently for the activated SCG?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| Ericsson | Yes |  |
| NEC | Maybe | This depends on the conclusion for RLM in Q3. If the conclusion is, for example, the network ensure RadioLinkMonitoringConfig provides target RS for “both” (i.e. No for Q3), then no need to consider this approach. Otherwise (i.e. Yes for Q3), same approach as RLM can be applied. |
| Futurewei | Yes |  |
| Qualcomm | Yes |  |
| LGE | Yes | We support the legacy behavior even if the network command BFD in SCG deactivation w/o RS configuration. |
| Spreadtrum | Yes |  |
| NTT DOCOMO | Yes |  |
| MediaTek | No strong view | We can follow the conclusion from RLM part. |
| Sharp | Yes | Our understanding is that UE should perform BFD based on the activated TCI state of the previously activated BWP. |
| Lenovo, Motorola Mobility | Yes |  |
| CATT | Yes |  |
| KDDI | Yes |  |
| OPPO | Yes |  |
| Nokia | No strong view |  |
| Huawei, HiSilicon | Yes |  |

Summary: 16 companies provided views. 13 companies support the BFD based on TCI state of PDCCH as legacy. 2 companies think the same conclusion for RLM part can apply to BFD as well. 1 company havs no strong view. In this case, the rapporteur understands all companies agree to support BFD based on TCI state of PDCCH, which is the same proposal for RLM.

[Easy]Proposal 6: while the SCG is deactivated, BFD can be based on activated TCI state for PDCCH reception when RadioLinkMonitoringConfig does not provide any RS for "beamFailure" or "both", like currently for the activated SCG

Q7: Do companies agree that and explicit indication is needed in order to indicate that the UE shall perform BFD while the SCG is deactivated? (if no, please indicate how exactly the UE is configured to perform or not perform BFD)

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| Ericsson | Yes | See our comment to Q4. |
| NEC | Yes but | We assume common explicit indication for RLM and BFD is sufficient. |
| Futurewei |  | We are open to this as for Q4 |
| Qualcomm | Yes | Similar remarks as for Q4 for explicit indication of RLM. |
| LGE | Yes |  |
| Spreadtrum | Yes |  |
| NTT DOCOMO | Yes |  |
| MediaTek | Yes |  |
| Sharp | Yes | bfd-and-RLM indicates whether BFD is performed or not and this is valid even if RS for the purpose set to "beamFailure" is not configured. |
| Lenovo, Motorola Mobility | Yes |  |
| CATT | Yes |  |
| KDDI | Yes |  |
| OPPO | Yes |  |
| Nokia | Yes | this is fine to us |
| Huawei, HiSilicon | Yes |  |

Summary: 16 companies provided views. 15 companies answered yes. 1 company is open to discuss. One company suggest using common explicit indication for both RLM and BFD, which is like current draft RRC CR.

[Easy] Proposal 7: Keep the existing indication in the 38.331 running CR to indicate whether the UE shall perform BFD while the SCG is deactivated.

In [3], it is considered that in some scenarios, while RLM is useful when the SCG is deactivated, it could be configured to be more power-efficient (and probably then less efficient to detect RLF).

According to [1], such a configuration could be provided in an RRC message to deactivate the SCG and the configuration for SCG activated state could be provided in an RRC message to activate the SCG. However, it somehow increases the size of RRC signalling at SCG activation and deactivation, and makes it not possible to use a MAC CE for SCG activation.

Therefore, it is proposed in [3] to provide a separate RLM configuration, only applicable for RLM while the SCG is deactivated, which could be included in deactivated-SCG-Config in [1], i.e. it can be configured at any RRC reconfiguration before SCG deactivation and it is stored by the UE.

Q8: Do companies agree to support a separate RLM configuration, applicable only while the SCG is deactivated, which can be provided to the UE in any RRC reconfiguration?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | It’s not necessary to define separate RLM configuration in RAN2, since RAN4 can specify relaxed measurement requirement for deactivated SCG. |
| Ericsson | No | Not clear that a separate RLM configuration is needed for SCG deactivation, and what it should contain. RLM relaxation for deactivated SCG should be discussed in RAN4. |
| NEC | Yes | We see some benefit for this approach. Network can omit this separate RLM configuration, if the network does not need to configure RACH-less activation. |
| Futurewei |  | We can have a working assumption to maintain the RLM configuration when transiting into the deactivated state at mean time, and wait for RAN4 input. |
| Qualcomm | No, please see comments | While the idea to provide a more power efficient RLM configuration is relevant and appealing, to determine the parameters of such a configuration will need RAN4 involvement. The R17 power savings WI is looking at RLM and BFD measurement relaxation mechanisms. However, the conclusions and solutions from that study may not be directly applicable to the scenario when SCG is deactivated. It seems that if we are to avoid having delays in finishing the WI, it is better not to support this feature. |
| LGE | Yes | As the proponent company, we agree for power-saving gain. If the UE is a type of stationary in the environment with stable radio quality, performing RLM with the same level of SCG activation state may just cause constant UE power consumption unnecessarily. This is because the UE barely performs PSCell change or barely detect radio link problem from the point that there wouldn’t be much different on the radio quality of PSCell. |
| Spreadtrum | No | The necessity of separate RLM configuration is not clear. |
| NTT DOCOMO | No | Need to involve RAN4 when discussing details on relaxation of RLM. We are afraid that not enough time is left for Rel-17. |
| MediaTek | No | The RLM requirement in SCG deactivation is up to RAN4 design. If needed, RAN4 could have relaxed requirement for that. There is no need to discuss this now. |
| Sharp | No | The current RLM configuration can be reconfigured for SCG activation/deactivation if needed. |
| Lenovo, Motorola Mobility | No | If NW wants, NW can configure the RLM configuration (especially if Q5 is agreed) when deactivating SCG. We don’t see the necessity to provide such configuration before SCG deactivation. |
| CATT | No | The measurement requirement for deactivation SCG depends on RAN4. |
| KDDI |  | Depends on RAN4 |
| OPPO | No | It is up to RAN4. |
| Nokia | No | up to ran4 and no time in this WI anymore |
| Huawei, HiSilicon | No | From RAN2 perspective, we do not see the need of one separate configuration specific to deactivated SCG. Whether RAN4 can/will discuss and decide on related RLM requirement when SCG is deactivated is totally up to RAN4. |

Summary: 16 companies provided views. 12 companies answered no. 2 companies provided no direct answer but also commented the decision is up to RAN4. 2 companies supported the separate RLM configuration for further power saving in case the radio condition surrounding the UE is stable.

Several companies think that in order to achieve power saving, as is the intention of this proposal, it would be necessary to involve RAN4. The rapporteur observes that it is possible to change the RLM configuration explicitly at SCG deactivation and at SCG activation and it is up to the network to determine the suitable configuration for RLM.

Since there is a rather large majority against this proposal, the rapporteur suggests that it is not further considered.

[Easy]Proposal 8: RAN2 does not consider introducing a separate RLM configuration specific for deactivated SCG. If RAN4 feel the necessity RAN4 can discuss/decide if it is needed or not.

## 2.4	RRM

In [3], [6] and [8], it is considered that some RRM measurements done while the SCG is activated may not be useful when the SCG is deactivated. As discussed in [4], it would be possible to remove the corresponding measIds when deactivating the SCG and configure them again when activating the SCG, or at a later RRC reconfiguration.

However, this certainly increases RRC signalling size, hence why [3], [6] and [8] propose other solutions, i.e. to indicate, prior to SCG deactivation, the frequencies (i.e. measObject) or the measIds to be measured after SCG deactivation.

Q9: Do companies agree to support configuring, prior to SCG deactivation, the measIds or the measObject to be measured after the SCG is deactivated? Please indicate if a preference for measIds or measObject.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | Similar view as [4], it would be possible to remove the corresponding measIds when deactivating the SCG and configure them again when activating the SCG, or at a later RRC reconfiguration. |
| Ericsson | Yes | We see some benefits as this can avoid SN involvement during SCG activation and thus it speeds it up and reduces signalling. It would be most natural to indicate per measObject whether it is measured on when SCG is deactivated. But it is perhaps not critical to immediately start new measurements immediately after the SCG activation. |
| NEC | Yes | Slightly prefer to go with approach by using measObject. |
| Futurewei | No | We have similar view as [4]. There is no need to add restriction/requirement on the measurement specifically to differentiate activated and deactivated SCG. The signalling overhead should not be of concern. We don’t expect a very frequent SCG activation/deactivation. Even if activation/deactivation occurs often, it is still questionable to have frequent measurement configuration change accordingly. |
| Qualcomm | Yes | Prefer to indicate the measIDs. |
| LGE | Yes | We think there are non-essential neighbour frequencies in SCG deactivation because the network may command measurement of some neighbour frequencies not for SCG connectivity, e.g. PSCell mobility robustness. The UE doesn’t need to perform these non-essential neighbour frequencies while SCG is deactivated. Therefore, we prefer measObject not to measure non-essential neighbour frequencies. Also, to avoid bulky signalling whenever SCG is activated/deactivated, non-essential measurement objects should be pre-configured, and UE should be allowed to suspend the measurements on nonessential measurement objects when SCG is deactivated, and resume it when SCG is activated. |
| Spreadtrum | No | There is no need to configure the measurements prior to SCG deactivation. The signalling overhead is not a critical problem. |
| NTT DOCOMO | Yes |  |
| MediaTek | Prefer No | We think that the baseline behavior in [4] is good enough. Signaling optimization is not a must especially at this late stage. |
| Sharp | Yes | We prefer measObject. |
| Lenovo, Motorola Mobility | No | Instead of RRC signalling size, it seems more about when to send the configuration, before SCG deactivation or when SCG deactivation happens. To us, we don’t see the necessity of providing it in advance. |
| CATT | No | RRM relax measurement could be achieved by releasing and adding measId or the measObject as current mechanism. |
| OPPO | No | It is up to RAN4. |
| Nokia | No | In fact deactivation by RRC allows any change to RRM configuration – no need to do anything special here. |
| Huawei, HiSilicon | No | We understand RAN2 already agreed that reconfiguration is supported when SCG is deactivated, then this kind of signalling optimization. We prefer not to go for such optimization at the late stage. |

Summary: 15 companies provided views. 9 companies prefer not to support the signalling optimization. 6 companies prefer to support it. Among the companies preferring to support it, 4 companies prefer measObject, 1 company prefers measID. Thus the rapporteur suggests to further discuss this aspect in the meeting.

[For discussion]Proposal 9: RAN2 to discuss whether to support configuring, prior to SCG deactivation, the measIds or the measObject to be measured after the SCG is deactivated.

In [4], it is observed that for the deactivated SCG, the UE does not monitor PDCCH so there is no valid DRX. It is proposed to signal explicitly a measCycle for the PSCell to be used for RRM measurement requirements when the SCG is deactivated.

Q10: Do companies agree to support configuring a measCycle for the PSCell to be used for RRM measurement requirements when the SCG is deactivated?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | It’s not necessary to define measCycle for the deactivated PSCell in RAN2, since RAN4 can specify relaxed measurement requirement for deactivated SCG. |
| Ericsson | Not yet | This should be discussed first in RAN4. RAN2#113bis agreed already that RRM requirements are FFS pending RAN4 work. There are different ways to apply a relaxed measurement cycle when the SCG is deactivated, including configuring a measCyclePSCell (similar to the measCycleSCell for deactivated SCell). Note that RAN4 has previously agreed to relax L3 RRM measurements on deactivated PSCell only if RAN2 agree to relax. |
| NEC | Not yet | It’s good to wait for RAN4 work. If this is to be introduced, the value range should be carefully decided in order not to be too much relaxed. |
| Futurewei | Maybe | We can wait for guidance from RAN4, and at mean time, maintain the same at the state transition to the deactivated. |
| Qualcomm | Yes | The range of values for the measCycle for the PSCell should be decided by RAN4. |
| LGE | Not yet | To support relaxed measCycle on PSCell, RAN4 work is absolutely necessary, so we think that RAN2 cannot first decide. Thus, at this moment, it seems that not performing non-essential frequencies (supporting Q9) is more effective on power saving for the UE than performing measurement with relaxed measCyle on PSCell. |
| Spreadtrum | No | RAN4 can make the decision whether to introduce it or not. |
| NTT DOCOMO | Not yet | This can be discussed after guidance from RAN4, otherwise we can put some assumptions. We think that Ericsson [8] well describes there is a deadlock between RAN2 and RAN4, waiting for each other’s decision. We should at least agree to relax RRM measurement to make a progress. |
| MediaTek | Yes, but | Similar to measCycle for deactivated SCell. We believed it is also needed for deactivated PSCell. However, we should of course wait for RAN4 guidance. |
| Sharp | Not yet | As other companies mention, we are fine with waiting for RAN4 discussion. |
| Lenovo, Motorola Mobility | No | RAN4 involvement and guidance is needed. |
| CATT | No | It depends on RAN4 for RRM measurement requirements when the SCG is deactivated. |
| OPPO | May be yes | We can wait for RAN4 inputs. |
| Nokia | Yes – something like this | As UE does not monitor PDCCH if we do not have something like this then UE does not measure PSCell at all as the requirements are based on DRX. Thus we are quite concerned about companies thinking we would not need something like used for deactivated SCelsl i.e. measCycleSCell. We can have separate parameter (maybe different value range but we assume there is no time to discuss details now) |
| Huawei,HiSilicon | Yes | We feel the similar configuration of measCycle for deactivated SCell can be reused for deactivation SCG. However, if companies prefer to let RAN4 discuss and decide it we are fine with it. |

Summary: 15 companies provided views. 4 companies showed clear support of the configuration of measCycle for deactivated SCG. Other companies preferred to let RAN4 decide. Considering this is related to RRM requirements which fall into RAN4 scope, the rapporteur suggest to stop discussion in RAN2 and leave this issue to RAN4.

[Easy]Proposal 10: Whether to support the configuration of measCycle for deactivated SCG is up to RAN4.

## 2.5	SCG-related MCG capability limitation

In [5], it is proposed that, while the SCG is deactivated, the UE ignores the UE MCG capability limitation due to SCG configuration, at least for UE power limitation and PDCCH blind decoding limitation.

Q11: Do companies agree that, while the SCG is deactivated, the UE ignores the UE MCG capability limitation due to SCG configuration, at least for UE power limitation and PDCCH blind decoding limitation? Do companies see other MCG limitations that could be lifted while the SCG is deactivated?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes | It’s beneficial to ignore the UE MCG capability limitation while the SCG is deactivated, but which MCG capability limitation can be ignored needs more discussion. |
| Ericsson | Maybe in specific cases | In general, whatever is configured for the MCG configuration shall apply also when SCG is deactivated. The purpose for SCG deactivation is power saving, and then it may seem counter intuitive to increase MCG power consumption when SCG is deactivated. What can be considered is to remove the MCG UE power limitation while SCG is deactivated to improve MCG coverage, as long as the UE is not allowed to initiate SCG transmission while the SCG is deactivated. |
| NEC |  | Logically speaking, both capability limitations can be lifted while the SCG is deactivated. However, it is not simple to take different handling among UE capabilities. On one hand some UE capabilities (limitations) are kept unchanged, on the other hand some other UE capabilities (limitations) are lifted. Even though the lifting gives some benefit, we prefer not to change (i.e. lift) the UE capabilities while the SCG is deactivated. |
| Futurewei | Maybe | Further study on the details is required. May have some benefit but not essential. |
| Qualcomm | Yes | Agree for at least UE power limitation. |
| LGE | Maybe No | We think MCG doesn’t have problems even if RAN2 doesn’t support this. Also, since MCG is still in the activation, the network likely provides RRC reconfiguration according to the coordination of MN and SN before/when SCG is deactivated if needed. |
| Spreadtrum | Maybe | It is beneficial to ignore the UE MCG capability limitation. But further study is needed. |
| NTT DOCOMO | Yes | Details needs more discussion but at least we can agree on ignore some limitation. We agree with Ericsson that UE power relaxation contributes to MCG coverage. |
| MediaTek | No. And clarification is really needed on what is proposed | This needed to be discussed case by case with clear proposal on what to do. It is entirely unclear what does UE ignore “MCG capability limitation” means. For power limitation, it is more like UE ignore the NW configuration on MCG power limitation while SCG is deactivated. We think it is not necessary as anyway MN could remove the power limitation while SCG is deactivated. For PDCCH blind decoding limitation, it is more like MN use the capability limitation from UE side as SCG is not configured. No “capability” is really ignored by UE here. |
| Sharp | Yes | We agree that it is beneficial to ignore the limitation of MCG capability, but it might be needed to consider some deactivation caused (overheat, etc.). |
| Lenovo, Motorola Mobility | Yes |  |
| CATT | Yes | Due to UE doesn’t perform UL transmission and doesn’t monitor SCG PDCCH, so at least the UE power limitation and PDCCH blind decoding limitation can be ignored. |
| OPPO | Yes | In MR-DC, the UE capability will be split for MN and SN and MN will send SCG configuration restriction to SN. There are lots of restriction for SCG, e.g. power split, PDCCH blind decoding number, measurement configuration, ROHC/EHC context number and so on. If the SCG is deactivated, the SN will not use this reserved capability. The MN can use this part capability if SCG is stay deactivation state to improve the performance in MN side. E.g. UE will use the UE power based on the limitation of “p-maxNR-FR1-MCG” in MN side. For PDCCH blind decoding limitation, the MN will decode PDCCH according to the search space id increase order until reach the PDCCH blind decoding limitation of MN. So some search space may omit due to limitation. If the UE can omit the UE capability restriction during SCG deactivation, the performance will be improved. |
| Nokia | No. | What is the proposal in fact? it is very vague and impossible to comment in detail. Anyway if there is nothing broken no changes should be introduced and we should focus on aspects that need resolving. |
| Huawei, HiSilicon | See comments | We understand the intention is to maximum UE capability usage in MCG side when SCG is deactivated. However, it needs case by case discussion. For instance, the UE power limitation may not be decided by RAN2 alone, RAN4 needs to be involved as well. |

Summary: 13 companies provided views. 6 companies support the proposal, especially for UE power limitation. Other companies think it is not a simple topic, more discussion is needed. The rapporteur shares the view that if RAN2 agree to go with this direction, then more analysis and case by case discussion is needed.

[For discussion]Proposal 11: RAN2 to discuss while the SCG is deactivated, how the UE applies the MCG power limitation and PDCCH blind decoding limitation (e.g. whether the UE does as if there would be no SCG, or as if the SCG would be activated).

## 2.6	MAC CE

The possibility to use an MCG MAC CE to deactivate or activate the SCG was mentioned several times but never concluded. The question whether this is supported was also asked by RAN4. As we are approaching the end of the WI, the rapporteur suggests focusing the discussion on the simplest solution: a MAC CE only indicating "SCG activation" or "SCG deactivation" without any additional information.

Such a MAC CE could be used to activate the SCG:

-	when the UE is configured to perform RACH-less activation and the network considers that SCG activation is feasible with the activated TCI states

-	when the UE is not configured to perform RACH-less activation or when the TAT has expired (and then the UE would use CBRA)

The processing time could be less than an RRC message, thus reducing the activation delay. The MN could use this MAC CE when the SN does not provide any RRC message at SCG activation.

Q12: Do companies agree to support a MAC CE to indicate "SCG activation", presumably with no additional information in the MAC CE?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | The main benefit to introduce SCG deactivation is power saving, and it’s not critical to support faster activation. |
| Ericsson | No | So far MAC signalling for MR-DC has been kept cell group specific. Introducing a MCG MAC CE to activate SCG would break this principle and increase complexity. There would be RAN3 impact, since CU-DU signalling would be needed to support the activation on MAC. With only two meetings left, there is risk of not finalising the WI on time, thus we should stick with the baseline in Rel-17, which is to use RRC. The latency concern should be addressed by defining a reduced processing time for RRCReconfiguration for SCG activation, with limited or no other reconfiguration content. |
| NEC | No, but | We see benefit for MAC CE, only if the MN can respond (i.e. activation) to the SCG activation request from the UE in MCG without waiting for the SN confirmation. Otherwise, no specific benefit is seen e.g. from latency point of view. IF RAN2 can agree with such MN behaviour, then we are fine to consider MAC CE. Otherwise, No. |
| Futurewei | Yes | Delay reduction is an important factor being considered in every aspect of (de)activation design that we conducted so far. Therefore, we think it is worth to support MAC CE based activation and the additional efforts are manageable. |
| Qualcomm | Yes | One thing that can perhaps be clarified is what is meant by “UE is configured/not configured to perform RACH-less activation”. It seems that if MAC CE is used to activate the SCG, the MAC CE should also perhaps contain an indication whether UE should perform RACH, i.e., CBRA, upon activation. If there is no such indication, then UE can perform RACH-less activation if certain conditions are met, e.g., TAT has not expired, UE has not experienced RLF or beam failure so far. If it is agreed by RAN2 that an RRC reconfiguration message can be transmitted while UE is in SCG deactivated, then the message can be used to provide a dedicated RACH configuration. Then, upon MAC CE SCG activation, UE can even perform CFRA upon activation. |
| LGE | Yes | As the rapporteur said, the benefit is clear to support lower layering signalling. |
| Spreadtrum | No | It is better to use RRC signalling to include other necessary configuration for SCG activation. |
| NTT DOCOMO | Yes | Agree with Futurewei’s comment. |
| MediaTek | No, but | We consider that MAC CE control is not a must for SCG deactivation operation. If there is clear majority to support this, the simple method provided by rapporteur is acceptable to us. |
| Sharp | No | We think there are few benefits for the latency reduction because the total processing time for SCG activation (including MN-SN negotiation) is long compared with the time for sending activation command. In addition, MAC signalling for MR-DC should be kept cell group specific as Ericsson's comment. |
| Lenovo, Motorola Mobility | Yes | We see some benefit in latency reduction. |
| CATT | No | Not support in Release 17, could be considered in later release. |
| OPPO | Yes | We are fine to have a MAC CE with only one indication for A/D SCG. If more detailed information is also included in this MAC CE, we are also fine. |
| Nokia | No | There is no time to do this as we would introduce huge amount of problems with MAC CE as can be seen in this email discussion and earlier that people assume that whenever SCG is deactivated/activated NW needs to configure/deconfigure some fields – this would be impossible with MAC CE thus leading to extremely complex autonomous UE actions. There is no time to do that in this WI. Additionally latency reduction would be minimal compared other aspects especially if we could reduce RRC processing delay of activation e.g. when only limited set of parameters are changed e.g. activation status/TCI states. – for deactivation we do not see any need to do faster. |
| Huawei, HiSilicon | Yes | We see clear benefit for latency reduction and simplicity of activation/deactivation signalling handling. And the design of such MAC CE is quite straightforward, so we prefer to support it. |

Summary: 15 companies provided views. 7 companies supported to introduce a MAC CE. 8 companies answered no, wherein 1 company would like to consider it in later release. 1 company think it is not a must but can accept it, and other companies think there is no clearly benefit.

There were comments that "So far MAC signalling for MR-DC has been kept cell group specific". The rapporteur observes that for PDCP duplication, the MAC CE can come from MCG or SCG. About CU-DU impact, the rapporteur understands that this is correct but rather straightforward (generate a MAC CE for SCG activation).

There was a suggestion to provide more information in the MAC CE but the rapporteur suggests keeping it simple in Rel-17.

[For discussion]Proposal 12: RAN2 to discuss whether to support a MAC CE to indicate "SCG activation", presumably with no additional information in the MAC CE.

# 3	Conclusion

Based on the input received, the rapporteur would like to make the following proposals:

[Easy] Proposal 1: upon SCG RLF while the SCG is deactivated, the UE reports SCGFailureInformation (legacy procedure) and the network can reconfigure the UE to release the SCG, change the PSCell or keep the PSCell and reconfigure RLM RS.

[For discussion]Proposal 2: Select one of the following options upon BF while the SCG is deactivated:

1)	no report, the UE will do CBRA in case of SCG activation without reconfigurationWithSync

2)	send an SCGFailureInformation message with a new cause, the network can reconfigure the UE to keep the SCell and allow RACH-less activation (by changing BFD RS), change the PSCell or release the SCG. If the network does not reconfigure the UE and activates the SCG, RACH will be used.

[Easy]Proposal 3: while the SCG is deactivated, RLM can be based on activated TCI state for PDCCH reception when RadioLinkMonitoringConfig does not provide any RS for “rlf” or “both”, like currently for the activated SCG.

[Easy] Proposal 4: Keep the existing indication in the 38.331 running CR to indicate whether the UE shall perform RLM while the SCG is deactivated.

[Hopefully easy]Proposal 5: tci-Info, which can provide activated TCI states for PDCCH/PDSCH reception at SCG activation (i.e. transition from deactivated SCG to activated SCG), can be included at any RRC reconfiguration while the SCG is deactivated and, if SCG remains deactivated and the UE performs BFD and/or RLM based on activated TCI states for PDCCH reception, the UE uses the newly activated TCI states for PDCCH reception.

[Easy]Proposal 6: while the SCG is deactivated, BFD can be based on activated TCI state for PDCCH reception when RadioLinkMonitoringConfig does not provide any RS for "beamFailure" or "both", like currently for the activated SCG

[Easy] Proposal 7: Keep the existing indication in the 38.331 running CR to indicate whether the UE shall perform BFD while the SCG is deactivated.

[Easy]Proposal 8: RAN2 does not consider introducing a separate RLM configuration specific for deactivated SCG. If RAN4 feel the necessity RAN4 can discuss/decide if it is needed or not.

[For discussion]Proposal 9: RAN2 to discuss whether to support configuring, prior to SCG deactivation, the measIds or the measObject to be measured after the SCG is deactivated.

[Easy]Proposal 10: Whether to support the configuration of measCycle for deactivated SCG is up to RAN4.

[For discussion]Proposal 11: RAN2 to discuss while the SCG is deactivated, how the UE applies the MCG power limitation and PDCCH blind decoding limitation (e.g. whether the UE does as if there would be no SCG, or as if the SCG would be activated).

[For discussion]Proposal 12: RAN2 to discuss whether to support a MAC CE to indicate "SCG activation", presumably with no additional information in the MAC CE.

# 4 References

- R2-2111638	Introduction of SCG deactivation	Huawei, HiSilicon	draftCR	38.331

- R2-2111639	Introduction of SCG deactivation	Huawei, HiSilicon	draftCR	36.331

- R2-2111017	UE Measurements in SCG Deactivation	LG Electronics

- R2-2110555	Measurements of deactivated SCG	Nokia, Nokia Shanghai Bell

- R2-2109471	UE measurements and reporting in SCG deactivation	OPPO

- R2-2109891	Discussion on UE behaviour when SCG is deactivated	ZTE Corporation, Sanechips

- R2-2110000	UE measurements and reporting in deactivated SCG	Qualcomm Incorporated

- R2-2110517	UE measurements, mobility and suspend/resume in deactivated SCG	Ericsson

- R2-2110660	Measurements when configured with RACH-less activation	NEC

- R2-2110894	Measurements while the SCG is deactivated	InterDigital

- R2-2110324	Discussion on RLF and BFD in deactivated SCG	Lenovo, Motorola Mobility

- R2-2110431	UE Behaviour in Deactivated SCG	CATT

- R2-2111014	Remaining issues for UE behaviour in deactivated SCG	SHARP Corporation