3GPP TSG-RAN2 Meeting #117-e	    	                											R2-211xxxx

e-Meeting, 17 – 26 Jan, 2022

Source: 		email discussion Rapporteur (ZTE Corporation)

Title: 	[Post116-e][515][RACH partitioning] MAC Procedure aspects (ZTE)

Agenda item: 	8.18.2

Document for: 	Discussion and Decision

# Introduction

This document contains summary of email discussion to related to MAC procedure aspects for common RACH partitioning:

[Post116-e][515][RACH partitioning] MAC Procedure aspects (ZTE)

-		General procedure for feature set selection

- 		General procedure for initialisation of RACH variables

-	 Overall RACH procedure in MAC

-  		Running CR

-	Deadline: Long

Discussion summary

- TBD

# Discussion

## Selection of RA resource partition

The following agreements were made for the configuration of the RACH resource partitions:

Agreements:

- No new feature and/ feature combination specific preambles are defined within the “not available” preambles defined at the end of a RO through the legacy  totalNumberOfRA-Preambles

- Specification allows for use of Separate time-frequency resources, not defined through legacy RRC signalling, within Contention free preamble defined through legacy RRC signaling and the combination of these (i.e. using the reserved preamble at the end of SSBs like 2-step RACH)

- RAN2 baseline is that preambles for a particular feature combination shall be present in all SSBs (e.g., a feature combination cannot only have preambles in SSB0 but not SSB1)

4	As a baseline, a feature combination shall have the same number of preambles in all SSBs

5	Signalling should allow that a particular feature/feature combination can be mapped only to a subset of the RACH occasions of a RACH configuration.

6	The legacy masking index approach is reused in Rel-17 RA partitioning

7	RAN2 adopts Approach A as baseline (an IE contains one field for each of the features) for indicating which feature/feature combination a partition applies to. Details are FFS, e.g. details around slicing.  FFS how to encode and design the signaling in a future compatible way (i.e. naming)

8	As a baseline, multiple "RA partitions" for one RA type which map to the same feature/feature combination is not supported on a given BWP.  FFS if there is any special use case that requires multiple RA partition configuration.

Based on the above, the general RACH resource configuration will look something like below:

Figure 1: Assumed RACH resource structure

Thus, based on the above structure, for the initiated RACH procedure, the MAC entity should determine which feature(s) are applicable to the RACH procedure. It seems that prior to the RACH being triggered, the applicability of REDCAP, Slice and SDT will be already known to the MAC entity. For CE, this is not clear. There are two options:

Option 1: CE will also be considered as part of the feature combination for each RACH partition and the use of CE will be determined in MAC and the RACH partition selection is performed considering CE to be similar to other features

Option 2: CE will be considered as a type of RA resource within each RACH partition and the use of CE will be determined after the selection of RACH partition (i.e. CE will not be considered during RACH partition selection and is only used in case the CE resource is configured within the RACH partition selected)

Option 3: Other option (pelase explain).

So, the first question is how to handle CE in the overall framework above.

| Q1: For handling the CE, which option is preferred? Option 1: CE will also be considered as part of the feature combination for each RACH partition and the use of CE will be determined before the RACH partition selection is performed Option 2: CE will be considered as a type of RA resource within each RACH partition and the use of CE will be determined after the selection of RACH partition (i.e. CE will not be considered during RACH partition selection and is only used in case the CE resource is configured within the RACH partition selected) Option 3: Other option (pelase explain). |  |  |
| --- | --- | --- |
| Company | Preferred option Option 1/2/3 | Comments (why?) |
| Qualcomm | Option 1 | If selection of RACH partition is up to UE implementation, then the question seems irrelevant because that decision will be up to UE implementation. If selection of RACH partition is based on some redefined rules, then we prefer Option 1 for the following two reasons: - We think UE can evaluate whether it meets the RSRP threshold for CE before it selects a RACH partition. This evaluation is not difficult to perform regardless of whether the RSRP threshold is configured per RACH paritition or the same for all partitions. It would simplify UE behavior if CE is considered one of the RACH features. - Option 2 may require extra tie breaking rule in selecting RACH partition. For example, suppose there are two RACH partitions. Partition A and Partition B have the same set of features, except that Partition B also supports CE. Then if Option 2 is applied, UE can’t determine whether it should select Partition A or Partition B. But there would be no ambiguity if Option 1 is applied. |
| Apple | Option 1 | When RAN2 agreed the feature/feature combination specific RACH partitioning, the features refer to the SDT, Slicing, RedCap and CE. Therefore, the CE feature should be treated in the same way as other features. |
| ZTE | Option 2 | In our view, CE should be like RACH-type. This is because, Msg1 based CE is only applicable if 4-step RA-type is selected and may be only used when the CE resource is configured for msg1 (note that network may also rely just on msg3 based CE indication instead). So, we think CE can be treated as a RA-type within each RACH partition. This simplifies the overall procedure (i.e. CE resource can be selected after selecting RACH partition and if the CE type RA resource for msg1 based CE indication) exists on the selected RA partition. @QC: In the example given QC, it seems the expectation is that there could be two partitions satisfying feature A and feature B (with and without CE resource). However, we don’t think such use case is really needed. i.e. for a given feature combination (i.e. A+B), only one RACH partition should exist. Within this RACH partition, there can be CE and non-CE resources. So, this is how option B works. |

Then, we need some general understanding on how the RACH resource partitions are used. As already agreed, each RACH partition will contain an IE indicating which feature/feature combination a partition applies to.

Hence, for each RACH partition configured, the understanding could be that the RACH partition will be considered as available for a triggered RACH procedure in case all the following conditions are satisfied:

- if REDCAP indication is configured for the partition, then the RACH partition is only applicable to the RACH procedure triggered for REDCAP UE where Msg1 identification is required. Otherwise, if REDCAP indication is not configured, then the RACH partition is applicable to non-REDCAP UE and REDCAP UE where Msg1 identification is not required. (FFS how to determine whether Msg1 identification is required or not)

- if slice info is configured for the partition,then the RACH partition is only applicable to the RACH procedure triggered for the slice. Otherwise, if the slice info is not configured, then the RACH partition is applicable to all slices.

- if SDT indication is configured, then the RACH partition is only applicable to the RACH procedure triggered for SDT. Otherwise, if SDT indication is not configured, then the RACH partition is applicable to the RACH procedure not triggered for SDT.

- if CE indication is configured, then the RACH partition is only applicable to the RACH procedure where CE is required. Otherwise, if CE indication is not configured, then the RACH partition is applicable to the RACH procedure where CE is not required. (if CE is considered as part of feature combination)

| Q2: Do companies agree with the general understanding below: for each RACH partition configured, the RACH partition will be considered as available for a triggered RACH procedure in case all the following conditions are satisfied: - if REDCAP indication is configured for the partition, then the RACH partition is only applicable to the RACH procedure triggered for REDCAP UE where Msg1 identification is required. Otherwise, if REDCAP indication is not configured, then the RACH partition is applicable to non-REDCAP UE and REDCAP UE where Msg1 identification is not required. (FFS how to determine whether Msg1 identification is required or not) - if slice info is configured for the partition,then the RACH partition is only applicable to the RACH procedure triggered for the slice. Otherwise, if the slice info is not configured, then the RACH partition is applicable to all slices. - if SDT indication is configured, then the RACH partition is only applicable to the RACH procedure triggered for SDT. Otherwise, if SDT indication is not configured, then the RACH partition is applicable to the RACH procedure not triggered for SDT. - if CE indication is configured, then the RACH partition is only applicable to the RACH procedure where CE is required. Otherwise, if CE indication is not configured, then the RACH partition is applicable to the RACH procedure where CE is not required. (if CE is considered as part of feature combination) If there are any specific comments or other understanding on how each feature is mapped to the partition, please explain in the comments section |  |  |
| --- | --- | --- |
| Company | Agree with general understanding? Y/N/comments | Comments (why?) |
| Qualcomm | Yes |  |
| Apple | See comments | The question is for the feature specific RACH partition, not for the feature combination specific case. If only we only consider the feature specific RACH partition, we agree with the general understanding. But for the feature combination specific RACH partition, NW may configure more than one feature indication per RACH partition, and further clarification is needed. |
| ZTE | Y in general | In general these look fine to us. For bullet point a) We should clarify whether REDCAP UE is allowed to select partition that donot indicate REDCAP indication (in which case, msg3 based REDCAP indication is assumed to be used in the cell – this understanding needs to be confirmed first). If this assumption is valid, then the bullet point a) could be revised as follows: a) if REDCAP indication is configured for the partition, then the RACH partition is only available to REDCAP UE. Otherwise (i.e. if no REDCAP indication is configured for a given partition), then, this RACH partition is available to both REDCAP UEs (which use msg3 based identification) and non-REDCAP UEs. Since the network knows that some REDCAP UEs may use non-RECAP partition assuming msg3 based identification, the network can configure the partitions in such a way that either msg1 based or msg3 based identification is possible in the network as required. For bullet point d) If CE indication is configured for a partition, it seems that there should also be a partition with the same feature set combination but without CE indication. Otherwise, the CE indication for the given feature set combination doesn’t sense. With this assumption then, we think bullet d) is okay. |

Then, we also agreed that not all feature combinations will need to be supported by the network:

Agreements

1	RAN2 assumes that the network may not provide all possible permutation.  FFS whether the selection in case of missing combination is specified or left to UE implementation

2	For slicing, unified partitioning framework should take priority

This means that we need to discuss what the UE behaviour shall be if only a subset of features are supported.

The main options to consider here are:

If only a subset of features have a matching RACH partition, and the triggered RACH doesn’t fit with any of the configured RACH partitions, then:

Option 1: it is up to UE implementation to select the RACH partition that matches UE’s preference based on implementation

- The consequence of this is that if there is no suitable Rel-17 RACH partition satisfying the triggered RACH feature combination, then the UE may choose any other RACH partition (this may even include other Rel-17 RACH partition that suits a subset of the features that triggered the RACH). How the UE chooses this subset is not specified (and left to UE implementation)

Option 2: the UE selects legacy RACH resource

- The consequence of this is that if there is no suitable Rel-17 RACH partition satisfying the triggered RACH feature combination, then UE will not select any other Rel-17 partition (even if that partition may indicate a subset of features that triggered the RACH procedure)

Option 3: we specify a set of rules based on which the UE shall select another RACH partition

- The consequence of this is that we need to specify clear priority rules that the UE shall follow in determining a fallback subset (if the feature set combination is not available)

Option 3: Would obviously need some further discussion. These can be discussed further down.

So, the first question is which of the broader options do companies prefer and why?

| Q3: If only a subset of features have a matching RACH partition, and the triggered RACH doesn’t fit with any of the configured RACH partitions, then which option do companies prefer and why?: Option 1: it is up to UE implementation to select the RACH partition that matches UE’s preference based on implementation - The consequence of this is that if there is no suitable Rel-17 RACH partition satisfying the triggered RACH feature combination, then the UE may choose any other RACH partition (this may include other Rel-17 RACH partition that suits a subset of the features that triggered the RACH). How the UE chooses this subset is not specified (and left to UE implementation) Option 2: the UE selects legacy RACH resource - The consequence of this is that if there is no suitable Rel-17 RACH partition satisfying the triggered RACH feature combination, then UE will not select any other Rel-17 partition (even if that partition may indicate a subset of features that triggered the RACH procedure) Option 3: we specify a set of rules based on which the UE shall select another RACH partition - The consequence of this is that we need to specify clear priority rules that the UE shall follow in determining a fallback subset (if the feature set combination is not available) |  |  |
| --- | --- | --- |
| Company | Preferred option Option 1/2/3/(Anything else?) | Comments (why?) |
| Qualcomm | Option 3 | We prefer Option 3 because the other two options have the following drawbacks: - Option 1 can lead to different (unpredictable) RACH performance by different UE implementations. In addition, Option 1 can make it difficult for network to estimate RACH load of different feature sets. That may lead to inefficient allocation of RACH resources; - Option 2 is unnecessarily restrictive and may result in inefficient use of RACH resources. For example, suppose network configures one RACH partition for RedCap and another for legacy. If a RedCap UE triggers RACH and it also satisfies the criteria for SDT, then with Option 2 this RedCap UE has to use legacy partition instead of RedCap partition. That’s clearly not efficient. |
| Apple | Option 1/3 | We are fine with Option 1 or Option 3. For Option 1, we can trust the reasonable UE implementation. Option 2 will degrade the intial access performance in some cases, e.g. when UE is in coverage enahcnement area, and NW doesnot provide the CE only RACH partition but provides CE+Slicing RACH partition. |
| ZTE | Option 2 is needed when there is no RACH resource available for the triggered feature combination Option 3 may be needed in case multiple RACH resources are available | We alredy agreed that the network is not obliged to configure RACH partitions for all features. So, if none of the RACH partitions is available for the triggered RACH procedure, then obviously, the UE has to select legacy RACH partition. So, option 2 is the baseline in this case. Then the question is whether option 3 is needed on top of option 2. We think option 3 comes in to picture only when there is one or more available RACH partitions (satisfying only a subset of the triggered features). For example: RACH procedure is triggered for REDCAP (R) and Slice 1(Sl-1): - RACH partion 1: R+Sl-2 - RACH partition 2: R+Sl-3 - RACH partition 3: R - RACH partition 4: Sl-1 Then, RACH partitions 1 and 2 are obviously not available because they are for slice 2 and slice 3 and the current RACH procedure is triggered for slice 1. RACH partition 3 is still available because it supports only a subset of features (i.e. REDCAP only). RACH partition 4 is also available because it supports only a subset of features (i.e Slice 1 only). We then need some predefined rule to determine whether partition 3 or 4 will be selected. For instance if we define some rule like REDCAP>Slice>SDT>CE then in the above example we would select RACH partition 3. If no such priority rule is defined (of if neither partitions 3 or 4 exist), then option 2 becomes the default option and legacy RACH resource will be selected. So, it seems both option 2 and option 3 may need to be supported in the end. So, we have to have some discussion on priorities. |

Then, the next question, is to discuss the details of option 3, should this be preferred by the majority of companies.

Firstly, if we want to specify a set of rules, there may be multiple sub-options:

Option a: Priority rules are static and will be defined in the specs (e.g. the available RACH partition with slice info will be prioritized  etc)

Option b: Priority rules are configurable (e.g. can be configured in SI)

Option c: Others (please explain)

| Q 4: If we agree to specify the priority rules, which option is preferred and why? Option a/b/c |  |  |
| --- | --- | --- |
| Company | Preferred option (option a/b/c/d/) | Comments (why?) |
| Qualcomm | Option a | We expect this priority rule to be fairly static. Hence we do not see a need to signal it in SI, which introduces unnecessary overhead in SIB1. |
| Apple | Option a/b |  |
| ZTE | a | If majority companies prefer to fully specify the priority rules, then we think option a is sufficient. |

If we choose to specify the detailed priority rules (i.e. fallback options), then we need to further discuss how to specify this.

For instance this may be based on some static priority rules (e.g. the available RACH partition with slice info will be prioritized etc etc). i.e. this means if both RACH partition with and without slice info are configured and be considered as available, then the UE should prioritize the RACH partition with slice info configured etc.

Since it is not straightforward to describe all such possible priories, we can start with some open discussion and companies can explain their views on how the priority and order can be specified

| Q 5: If there is no RACH partition that maps to all triggered features and if we want to define specific rules for fallback to a subset, how should UE determine the alternative Rel-17 partition to be selected (for a subset of features)? |  |
| --- | --- |
| Company | Comments (e.g. companies can explain how the priority order would look – etc) |
| Qualcomm | In our paper R2-2109452 ([2]), we have described the following steps for UE to apply to select a RACH partition, assuming there is a priority list among different RACH features predefined in the spec: - Start with all configured RACH partitions, and the RACH feature which has the highest priority; - Determine if the RACH feature selected for this step is one of the triggers for the RACH procedure or UE is eligible to use. If it is not, select the next RACH feature in the priority list and check again. Otherwise, among the RACH partitions selected at the start of this step, select those that include the selected RACH feature and then perform Step 3; - Among the RACH partitions selected for this step, select those partitions that UE meets the criteria of all its included RACH features. In addition, select the next RACH feature on the list. Then repeat Step 2. If UE does not meet criteria of any partitions selected for this step, or all RACH features in the priority list has been evaluated, UE stops this proccedure. |
| Apple | The fallback priority list provides the feature priotiy in the order for the RACH paritition selection, e.g. CE is the 1st priority, RedCap is the 2nd priority…and so on. If there is no RACH partition mapped to all triggered features, UE will follow the fallback priority list to select the feature specific RACH partition. - UE selects the RACH partition mapped to the feature with the 1st priority, and if not, UE selects the RACH partition mapped to the feature with 2nd priority… - If no RACH partition is selected finally, UE will perform legacy RACH procedure. |
| ZTE | Firstly, for SDT, before the resume procedure is triggered for RA-SDT, RRC will anyway check with MAC that there are RA-SDT specific RACH resources available. Thus, it can be ensured that the RACH partition for SDT will be available in this case. If RACH partition for SDT doesn’t exist, then RA-SDT will not be triggered by RRC. So, we don’t need to further consider SDT in priority rules. Then, we think that CE should be considered as a RACH type instead of as a RACH partition as mentioned above. So, CE also doesn’t need to be considered then in priority selection. This leaves us with REDCAP and slice indication. We think then we can specify slice indication has higher priority than REDCAP. This works because the network can always rely on msg3 based REDCAP indication. Hence, the UE can prioritise the slice resources. Since the network knows that it has to rely on msg3 based REDCAP indication in some fallback scenarios where slice indication takes precedence, it can always tailor the RACH partitions accordingly (i.e. either to provide explicit partitions with REDCAP indication and slice combinations where this is seen as important and then to rely on the msg3 based REDCAP indication where this is not seen as important). So, with this approach, we think the only priority rule to specify is that Slice indication has higher priority than REDCAP. |

## Initialization of RACH variables

Once the RACH partition is selected, the RACH variables can be initialised based on the selected partition. We can check if companies share this view.

| Q 6: Do companies agree that once the RACH resource partition for a given feature set combination is determined, RACH procedure related variables in sections 5.1.1 and 5.1.1a can be initialized based on the values signalled within the selected RACH partition? |  |  |
| --- | --- | --- |
| Company | Yes/No | Comments (why?) |
| Qualcomm | Yes |  |
| Apple | Yes |  |
| ZTE | Yes | Once the RA partition is selected, the MAC entity should know all the corresponding RA parameters for the partition and should proceed to initialise the RA variables in section 5.1.1 and 5.1.1a using the configured variables. |

Then, one further question is whether all the RACH parameters within a RACH partition are common to all features that use the corresponding RACH partition. This aspect is mentioned in [2] for instance (see P1). In general, it is possible for the network to configure the RACH parameters within a given RACH partition to take into account all features that are mapped to the specific RACH partition. For instance, if a given slice requires specific power control or other RACH prioritisation related parameters, the network can configure the corresponding parameters in this RACH partition accordingly. It seems that other features which may also use this partition will also use these parameters, but since this is under control and can be tailored to each partition, it seems there is sufficient flexibility to allow the RACH variables to be specific to the partition (i.e. not specific to the feature within the partition). So, the following question is asked.

| Q 7: Do companies agree that all features that are mapped to a given RACH partition use the same set of RACH parameters (signalled within this partition) – in other words the RACH parameters are per RACH partition rather than per feature within the partition? |  |  |
| --- | --- | --- |
| Company | Yes/No | Comments (why?) |
| Qualcomm | See comment | We do not fully understand the rapporteur’s question. The following comments are based on our best guess on what the rapporteur is asking. It seems that the second part of the question asks whether RACH parameters should be configured per RACH partition (e.g. two-step RACH in different RACH partitions can be configured with different values) or per feature globally (e.g. two-step RACH in all RACH partitions share the same values for two-step RACH parameters). In our view, the configuration should be per RACH partition, because different RACH features, when jointly configured, may require different values for the same parameter. For example, suppose Partition A includes only legacy RACH and Partition B includes slice-specific 4-step RACH. Then Partition B may be configured with, say, different preambleTransMax or powerRampingStep to meet the special requirement of the slice. Regarding the first part of the question, we first observe that all RACH parameters are unique in a RACH partition, for the following reasons. Since each feature included in a RACH partition is always unique, then one may conclude that RACH parameters specific to a feature (e.g. msgA-RSRP-Threshold is specific to only two-step RACH) is also unique within that partition. If there are RACH parameters used by all RACH features, then those parameters have to be associated with either RACH resources or PHY-layer transmission procedures, not RACH features. Since the mapping between RACH resources and RACH partition is one-to-one, then those parameters are also unique/specific to a RACH partition. |
| Apple | Yes | RACH parameters should be provided per RACH partition. In our understanding, the RACH configuraiton of one RACH partition has 3 cases: a) 4-step RACH only, b) 2-step RACH only, and c) 2-step RACH +4-step RACH. |
| ZTE | Yes | Although it is possible to configure feature specific RA parameters within the RA partition that uses multiple features, we think such flexibility is not really needed. If a RA partition is applicable to a high priority feature, then even if some low priority features are mapped to this partition, they can also use the high priority RA parameters applicable to this partition. If this is not desirable, then the network always has the choice to change the feature set mapping to a different partition. So, we don’t think that additional complexity should be introduced by further dividing the RA parameters per feature within the partition. Of course within the RACH partition, the RACH parameters are per RACH type (same as legacy – i.e. for 2-step and 4-step RACH the parameters will be different). |

## Carrier/BWP selection

Currently, in sections 5.1.1, UE performs the carrier and BWP selection based on the thresholds configured in the RACH configuration. We can check if the same procedure can be reused for the common RACH procedure.

| Q 8: Do companies agree that carrier selection and BWP selection can be performed based on the RACH parameters signalled in the selected RACH partition? |  |  |
| --- | --- | --- |
| Company | Yes/No | Comments (please explain any changes needed?) |
| Qualcomm | No | We think carrier and BWP selection should be performed before the selection of RACH partitions. Therefore, the thresholds for their selections should be configured separately from the configuration of RACH partitions. |
| Apple | No | Carrier selection and BWP selection should be performed before RACH partition selection. |
| ZTE | Yes | For Carrier selection: There is a bit of a chicken-and-egg situation here since the carrier selection happens based on the RSRP threshold configured and this will be configured per RACH partition (i.e. per feature as agreed for some of the features already). If as carrier selection has to happen before RACH partition selection, then we need to clarify which RSRP threshold is then used for the carrier selection and how this threshold is configured. It should be noted that since carrier selection thresholds are configured per partition anyway, the network can ensure that once the given partition is selected it can always be ensured that the correct carrier is selected by the UE. i.e. there will be no case where the carrier selection will result in a scenario where the feature combination specific RA resources donot exist. So, we think we can stick with the existing mechanism where the modelling in MAC assumes carrier selection to be performed after selecting RACH partition. For BWP selection: Similar to the carrier selection above, we can also leave it up to the network implementation to ensure that on the selected BWP, the feature specific RACH resources exist. Note that for now the only question here is for REDCAP. So, if the REDCAP UE selects the REDCAP BWP, then all RACH resources on this partition implicitly support REDCAP indication. So, then REDCAP indication can always be provided anyway. In otherwords on the REDCAP BWP, all RACH partitions are REDCAP partitions. For other features, we can leave it up to network implementation. |

## RACH type selection

After carrier and BWP are selected, UE performs the RA-type selection and initialise RA-type specific variables (see section 5.1.1a of the MAC spec). We can check whether this procedure can also be replicated (again based on the RACH parameters signalled in the selected RACH partition).

| Q 9: Do companies agree that the RA-type selection can happen like today (i.e. after the carrier and BWP selection) based on the RACH parameters signalled in the selected RACH partition? |  |  |
| --- | --- | --- |
| Company | Yes/No | Comments (please explain any changes needed to the current procedure?) |
| Qualcomm | No | In our view, RA-type selection should be a part of selection of RACH partitions, because the priority between 4-step or 2-step RACH may be depend on which other R17 feature(s) it is configured with. For example, slice-specific 4-step RACH may be prioritized over common 2-step RACH. |
| Apple | Yes | After RACH partition is selected, UE will perform the RACH type selection if both 4-step RACH and 2-step RACH are provided in the RACH configuration of the selected RACH partition. |
| ZTE | Yes | For this, we are not sure how it will work if RACH type has to be selected before the RACH partition since the 2-step and 4-step RACH resources are configured per RACH partition! |

## RNTI collision problem

In [1], [3] the RNTI collision issue for RACH partitioning is discussed. In [1] it is proposed to solve this issue by using a custom offset signalled through RRC and associated with each PRACH configuration to solve this problem. On the otherhand in [3], it is proposed to use a separate search space for RAR/MSGB monitoring. It should be noted that for some work items (e.g. SDT), separate search space has already been agreed to be configured. So, it seems we need to support this option anyway and it seems it may be possible then to extend this to all RACH partitions too. So, companies are asked to answer the following question.

| Q 10: To solve the RNTI collision issue, which option do companies prefer? Option 1: Do nothing (i.e. leave to network implementation) Option 2: A custom offset, signalled through RRC and associated to each PRACH configuration, is added in the formula for RA-RNTI and/or MSGB-RNTI. The legacy PRACH configuration it is assumed to have offset = 0 (see [1]) Option 3: the network should be able to (optionally) configure a specific search space for RAR/MSGB monitoring per RACH resource partition (see [3] – as was already agreed anyway for some features – e.g. SDT) |  |  |
| --- | --- | --- |
| Company | Option 1/2/3 | Comments (why?) |
| Qualcomm | Option 1 | Although there can be more RACH configurations in R17, network still has several options to handle possible RNTI collision, as discussed in the past. If a super-majority of companies (e.g. more than 2/3) want to introduce enhancements to handle RNTI collision, we are fine with option 2, or option 2 together with Option 3. |
| Apple | Option 2 | The RA-RNTI/MsgB-RNTI used for R15/R16 and R17 can be overlapped due to the multiple PRACH configurations for the same RACH type in one cell. The RNTI offset should be introduced to avoid the overlapping between between legacy and R17 usage and to avoid the impact to legacy UE. |
| ZTE | Option 3 (option 1 is also okay) | We think option 3 is anyway needed (since some features already agreed feature specific search space). We think the search space then can be made to be feature combination specific (and apply a given search space per RA-partition). With this, option 3 seems necessary and is also sufficient. |

# Conclusion and proposals

TBD

# References

- R2-2110560	RNTI collision problem for Rel-17 features	Ericsson	discussion

- R2-2109452	Selection and fallback between RACH partitions Qualcomm Incorporated	discussion

- R2-2110598 	MAC aspects for RACH partitioning	Huawei, HiSilicon	discussion

- R2-2110578	User plane aspects of RACH partitioning	ZTE Corporation, Sanechips	discussion	Rel-17

- R2-2109532	RA Procedure Aspects	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_cov_enh-Core

- R2-2109542	Considerations on the common aspects of RACH procedure	Beijing Xiaomi Software Tech	discussion	Rel-17

- R2-2109882	RACH resource/configuration selection and fallback mechanism	Intel Corporation	discussion	Rel-17

- R2-2110260	Discussion on RACH indication and partitioning	CMCC	discussion	Rel-17

- R2-2110665	Overview of RACH resource selection	NEC	discussion	Rel-17

- R2-2110813	Selection of RACH partition	Nokia, Nokia Shanghai Bell	discussion	Rel-17

- R2-2110917	RACH indication and partitioning	InterDigital	discussion	Rel-17

- R2-2110927	Discussion on RACH Partitioning in RA Procedure Aspect	vivo	discussion	Rel-17

- R2-2111164	Discussion on common RA procedure for RACH partitioning features	LG Electronics Inc.	discussion

# Annex (contact details for email discussions)

| Company | Contact name | Contact email |
| --- | --- | --- |
| Apple | Fangli XU | fangli_xu@apple.com |
| ZTE | Eswar Vutukuri | eswar.vutukuri@zte.com.cn |