3GPP TSG-RAN WG2 Meeting #116bis electronic	R2-21xxxx

E-Meeting, 17 – 25, Jan 2022

Agenda item:	8.15.1

Source: 	LG Electronics Inc. (Rapporteur)

Title: 		Summary of [POST116-e][716][SL] MAC open issues

Document for:	Discussion and decision

# Introduction

This is the summary of the following email discussion:

- [POST116-e][716][V2X/SL] MAC open issues (LG)

Scope: Address and solve further stage 3 open issues (including details of LCP, SL DRX command, need of further considerations on SL impacting on Uu, and selection of SL DRX start offset for GC between option1 and option5)

Intended outcome:  Discussion summary and updated 38.321 running CR (if needed)

Deadline: Long email discussion. Recommend to have short intermediate phase to check if you list all options/solutions companies mind when to discuss solution.

| Company | Name | E-mail |
| --- | --- | --- |
| Xiaomi | Xing Yang | Yangxing1@xiaomi.com |
| OPPO | Bingxue Leng | lengbingxue@oppo.com |
| Ericsson | Min Wang | min.w.wang@ericsson.com |
| Huawei, HiSilicon | Tao Cai | tao.cai@huawei.com |
| CATT | Jie Shi | shijie@catt.cn |

# Overall description

This email discussion discusses open issues that can be further discussed based on the RAN2 agreement so far. For RAN2 progress, among the contributions submitted by companies at the previous meeting, issues requiring further discussion based on RAN2 agreements were included in this email discussion.

# MAC open issues

#### Issue 1: Priority order of SL DRX Command MAC CE

In the [AT116-e][702][V2X/SL] 38.321 running CR [1] of #116-e meeting, the priority order of SL DRX Command MAC CE has been discussed, and the following companies view were gathered.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	data from SCCH;

-	Sidelink CSI Reporting MAC CE;

-	data from any STCH.

- Between data from SCCH and Sidelink CSI Reporting MAC CE

- Same priority order of Sidelink CSI Reporting MAC CE

- Between Sidelink CSI Reporting MAC CE and data from any STCH

- Lower than STCH

- Any other

Summary: Out of 16 companies

Option-a: 4

Option-b: 1

Option-c: 13

Option-d: 2

Rapporteur thinks similar results would be gathered even if the discussion is reconsidered. Therefore, to avoid duplicate discussion, rapporteur proposes to agree with the majority opinion, option-c.

Q1: Would your company agree that the priority order of Sidelink DRX Command MAC CE is between Sidelink CSI Reporting MAC CE and data from any STCH?

- Yes

- No

- Any other?

| Company | Yes or No | Comment |
| --- | --- | --- |
| Xiaomi | NO | Sidelink DRX Command MAC CE shall not be prioritized over data from STCH. Otherwise, it’s possible RX UE is commanded to asleep while there is available data from STCH for transmission. |
| Ericsson | Yes | Regarding xiaomi’s comments, we think TX UE will not trigger DRX command MAC CE for RX UE if there is any data pending for transmission. |
| OPPO | Yes | Agree with Ericsson. |

#### Issue 2: UE operation when receiving SL DRX Command MAC CE

RAN2 agreed to support SL DRX command MAC CE in Unicast. RAN2 should further discuss UE behavior when receiving SL DRX Command MAC CE. Same as Uu DRX, for unicast, when an Rx UE receives SL DRX command MAC CE from Tx UE, the UE can stop the running onduration timer and inactivity timer associated with the unicast link.

Q2: Would your company agree when an Rx UE receives SL DRX command MAC CE from a TX UE, the Rx UE can stop the running onduration timer and inactivity timer associated with a unicast link?

- Yes

- No

- Any other?

| Company | Yes or no | Comment |
| --- | --- | --- |
| Xiaomi | Yes |  |
| Ericsson | Yes |  |
| OPPO | Yes |  |

#### Issue 3: HARQ Feedback option for SL DRX Command MAC CE

RAN2 agreed to support SL DRX command MAC CE in Unicast. RAN2 should additionally discuss an HARQ feedback option for SL DRX Command MAC CE. According to current TS 38.321, HARQ feedback on PSFCH is not support for (re-)transmission of a MAC PDU only carrying CSI reporting MAC CE [3]. RAN2 should decide whether to set the HARQ feedback option to “HARQ Feedback is disabled” or to “HARQ Feedback is enabled” when a UE transmits a MAC PDU only carrying the SL DRX Command MAC CE.

Q3.1: Would your company agree to allow a UE to transmit a SL DRX command MAC CE alone in MAC PDU?

- Yes

- No

- Any other?

| Company | Yes or No | Comment |
| --- | --- | --- |
| Xiaomi | Yes |  |
| Ericsson | Yes | For a unicast PC5 link, in typical cases, when the UE triggers a DRX command CE, the UE has no data pending for transmission to the intended RX UE, in this case, it is reasonable to allow the UE to transmit the DRX command MAC CE alone. |
| OPPO | Yes |  |

Q3.2: If your company answered yes to Q3.1, which option would your company prefer of HARQ feedback option when a UE transmits a MAC PDU only carrying SL DRX Command MAC CE?

- HARQ Feedback enabled MAC PDU

- HARQ Feedback disabled MAC PDU

- Any other?

| Company | Option | Comment |
| --- | --- | --- |
| Xiaomi | B | We prefer to follow legacy handling of CSI reporting MAC CE. |
| Ericsson | B | Agree with xiaomi |
| OPPO | B |  |

#### Issue 4: SR configuration for SL DRX Command MAC CE

RAN2 agreed to support SL DRX command MAC CE in Unicast. RAN2 should additionally discuss an SR configuration option for SL DRX Command MAC CE [3].

Q4.1: Would your company agreed to configure an additional SR configuration for SL DRX command MAC CE?

- Yes

- No

- Any other?

| Company | Yes or No | Comment |
| --- | --- | --- |
| Xiaomi | b | We don't see the need to introduce new SR configuration for SL DRX command MAC CE. |
| Ericsson | b | It is sufficient for the UE to use either Mode 2 grant or Mode 1 CG grant to transmit a SL DRX command MAC CE |
| OPPO | b |  |

Q4.2: If your company answered yes to Q4.1, which option would your company prefer of SR configuration for SL DRX Command MAC CE?

- SR configuration of SL CSI Command MAC CE is reused for SL DRX command MAC CE.

- New SR configuration for SL DRX command MAC CE is supported.

- Any other?

| Company | Option | Comment |
| --- | --- | --- |
| Xiaomi | a | We don't see the need to introduce new SR configuration for SL DRX command MAC CE. |

#### Issue 5: drx-HARQ-RTT-TimerSL is supported (or not supported) when sl-PUCCH-Config is not configured

In the #115-e meeting [4], the following Uu DRX timer behaviors have been agreed assuming that SL-specific drx-HARQ-RTT-Timer is not supported.

- #115-e meeting’s agreements on Uu DRX timer impact

- SL-specific drx-RetransmissionTimer is started at the first symbol after the end of last PSSCH resource scheduled through one DCI (with the assumption RAN2 agrees not to support SL-specific drx-HARQ-RTT-Timer but to support SL-specific drx-RetransmissionTimer when sl-PUCCH-Config is not configured, when sl-PSFCH-Config is configured). FFS the SL-specific drx-RetransmissionTimer is started at the first slot after the end of last PSSCH resource scheduled through one DCI instead.

- SL-specific drx-RetransmissionTimer is started at the first symbol after the end of last PSSCH resource scheduled through one DCI (with the assumption RAN2 agrees not to support SL-specific drx-HARQ-RTT-Timer but to support SL-specific drx-RetransmissionTimer when sl-PUCCH-Config is not configured, when sl-PSFCH-Config is not configured). FFS the SL-specific drx-RetransmissionTimer is started at the first slot after the end of last PSSCH resource scheduled through one DCI instead.

RAN2 should decide this (support SL-specific drx-HARQ-RTT-Timer or not support SL-specific drx-HARQ-RTT-Timer) as there is no conclusion on whether to support the SL-specific drx-HARQ-RTT-Timer when sl-PUCCH-Config is not configured. Moreover, submitted related proposals are as follows:

Proposal 31. SL-specific drx-HARQ-RTT-Timer is not needed when sl-PUCCH-Config is not configured [5].

Proposal 2. For the case when PUCCH resource is not scheduled, the UE does not start the drx-HARQ-RTT-TimerSL for the corresponding SL HARQ process [6].

Proposal 7. RAN2 should support the SL specific DRX HARQ RTT timer, in case PUCCH is not configured and PSFCH is configured in the resource pool [7].

Q5: Which option of drx-HARQ-RTT-TimerSL would your company prefer if sl-PUCCH-Config is not configured?

- Not support drx-HARQ-RTT-TimerSL

- Support drx-HARQ-RTT-TimerSL in case PSFCH is configured in resource pool and sl-PUCCH-Config is not configured

- Any other?

| Company | Option | Comment |
| --- | --- | --- |
| Xiaomi | B | We understand gNB is allowed to send DCI scheduling retransmission resource at the first symbol after the end of last PSSCH resource scheduled through one DCI. However, we would like to keep RTT timer to achieve common design. RTT timer could be zero in this case. |
| Ericsson | a | HARQ RTT timer is not needed, it is sufficient to only have retransmission timer. |
| OPPO | B | Even if PUCCH is not configured, as long as PSFCH is configured (which is known by network), UE has to wait for a specific processing delay as agreed by RAN1 before re-transmission. (NOTE that even if one relies on the time gap field in DCI (gap between PDCCH and indicated PSSCH) to solve the issue (use a PDCCH before PSFCH to schedule a PSSCH after PSFCH by ensuring the gap between new PSSCH and PSFCH), it is anyway not sustainable and certainly a bad implementation considering there could be as many as 32 re-transmission.) Have configurable value (including zero) of RTT timer for different cases (W/ or W/O PUCCH/PSFCH) can achieve both power saving and the flexible scheduling mechanism since the value of RTT timer is configured by NW. |

#### Issue 6: When sl-drx-HARQ-RTT-Timer is used as a configured value

In the #116-e meeting [2], RAN2 agreed on the following sl-drx-HARQ-RTT-Timer behavior.

- #116-e meeting’s agreement on HARQ RTT:

- Regardless whether HARQ feedback is enabled or disabled, the HARQ RTT timer can be derived based on the resource assignment information for retransmission of the same TB in the SCI if the resource assignment information for retransmission of the same TB is present.

- When HARQ feedback is disabled, either zero value or non-zero value can be configured for the HARQ RTT timer if the resource assignment information is not present. FFS on details of configuration.

According to the above RAN2 agreements, if resource assignment information for the next transmission presents in the SCI, UE does not use the configured value for sl-drx-HARQ-RTT-Timer. Also, if the resource assignment information for the next transmission does not exist in the SCI and HARQ feedback is disabled, the UE uses the configured value for sl-drx-HARQ-RTT-Timer. Even if the behaviour for the HARQ RTT timer value for HARQ feedback enabled was not agreed, rapporteur thinks that most companies would think that the UE can use the configured value for sl-drx-HARQ-RTT-Timer when the HARQ Feedback is enabled. Thus, rapporteur understands that regardless of HARQ feedback option, if resource assignment information for the next transmission does not exist in the SCI, the UE uses the configured value of the sl-drx-HARQ-RTT-Timer.

Q6: Would your company agree that UE can use configured sl-drx-HARQ-RTT-Timer value when the resource assignment information for the next re-transmission does not exist in the SCI regardless of whether HARQ feedback is enabled or disabled?

- Yes

- No

- Any other?

| Company | Yes or No | Comment |
| --- | --- | --- |
| Xiaomi | Yes | The configured RTT timer could be zero or non-zero value. |
| Ericsson | Yes |  |
| OPPO | Yes |  |

#### Issue 7: sl-drx-HARQ-RTT-Timer when HARQ feedback is disabled

In the #116-e meeting [2], RAN2 agreed on the following sl-drx-HARQ-RTT-Timer behavior.

- #116-e meeting’s agreement on HARQ RTT:

- When HARQ feedback is disabled, either zero value or non-zero value can be configured for the HARQ RTT timer if the resource assignment information is not present. FFS on details of configuration.

It should be determined which sl-drx-HARQ-RTT-Timer value can be configured among the following options when HARQ feedback is disabled.

Option 1. Can be set to either Zero value or non-zero value

Option 2. Can be set to only Zero value

Option 3. Can be set to Non-zero value

Option 4. Can be set to both Zero value and non-zero value

Q7: Which option of sl-drx-HARQ-RTT-Timer value would your company prefer when the HARQ feedback option is “disabled”?

- Can be set to zero either zero value or non-zero value

- Can be set to only zero value

- Can be set to only non-zero value

- Can be set to both zero value and non-zero value

| Company | Option | Comment |
| --- | --- | --- |
| Xiaomi | C | It’s up to gNB or TX UE’s implementation to configure zero or non-zero value in unicast. It can be set to zero in groupcast to accommodate all possible cases. |
| Ericsson | none | Don’t understand intention of this question, it will be up to gNB implementation, right? Why do we need to discuss this issue? |
| OPPO | See comments | Agree with Ericsson that it should be gNB implementation. |

#### Issue 8: Further issues on SL DRX for a mode-1 SL grant

In the #116-e meeting [2], the following UE behaviors for mode-1 SL grant have been agreed.

- #116-e meeting’s agreements on SL DRX for mode 1:

- For the issue that a mode-1 SL grant being provided by network to Tx-UE yet it is not in SL active time of any destination that has data to be sent, for initial transmission, drop the grant. FFS if any spec change.

- For the issue that a mode-1 SL grant being provided by network to Tx-UE yet it is not in SL active time of any destination that has data to be sent, for retransmission, drop the grant.

Rapporteur thinks that based on the above RAN2 agreements, RAN2 can additionally discuss “how to handle the MAC PDU (re-)transmission/Mode-1 SL grant” and “what information (e.g., ACK or NACK) should be reported via PUCCH if configured”, when the scheduled Mode-1 resources do not overlap with the active time of RX UEs.

Q8.1: If the mode 1 grant for initial transmission does not overlap with the active time of the Rx UE, and the mode 1 grant is dropped, what information can be reported through the PUCCH if configured?

- ACK

- NACK

- Any others

| Company | Option | Comment |
| --- | --- | --- |
| Xiaomi | comment | Either a or b would make the mismatch worse. If UE send ACK, gNB would assume the transmission successful and the RX UE’s active time would be extended by inactivity timer. gNB may continue schedule the SL grant in the ‘active time’ assumed by gNB. However, these grants would be dropped since it’s actually not in the active time. If UE send NACK, gNB would schedule grant for retransmission. According to the agreements, UE would also drop the grant. Therefore, UE shall indicate the grant dropping to gNB to stop the mismatch and avoid continuous grant drop. |
| Ericsson | b | b) is simpler, we can assume gNB will send a valid grant next time. |
| OPPO | a | xiaomi has some points that either a or b will cause some mismatch, but the issues of NW/UE state mis-match (either ACK or NACK) was discussed previously already, and the R2 view is that NW would perfectly know the UE state and has concluded not to do the enhancement based the assumption. And if we follow the current MAC spec without any enhancement on this, the UE will report ACK since there is no MAC PDU obtained in LCP. We prefer to go with the current spec 1> else(no MAC PDU obtained): 2> instruct the physical layer to signal a positive acknowledgement on the PUCCH according to clause 16.5 of TS 38.213 [6].. |

Q8.2: If the mode 1 grant for retransmission does not overlap with the active time of the Rx UE, and the mode 1 grant is dropped, what information can be reported through the PUCCH if configured?

- ACK

- NACK

- Any others

| Company | Option | Comment |
| --- | --- | --- |
| Xiaomi | Comment | Either a or b would make the mismatch worse. If UE send ACK, gNB would assume the transmission successful and the RX UE’s active time would be extended by inactivity timer. gNB may continue schedule the SL grant in the ‘active time’ assumed by gNB. However, these grants would be dropped since it’s actually not in the active time. If UE send NACK, gNB would schedule grant for retransmission. According to the agreements, UE would also drop the grant. Therefore, UE shall indicate the grant dropping to gNB to stop the mismatch and avoid continuous grant drop. |
| Ericsson | b | b) is simpler, we can assume gNB will send a valid grant next time. |
| OPPO | a | Prefer to align with initial transmission case. |

#### Issue 9: Issue on active time for announced periodic transmission

Another issue related to SL DRX active time is whether the RX UE considers the announced periodic resources of the TX UE as part of the active time. In the #114-e meeting, although the following proposal was proposed from the email discussion on timers [8], this was not agreed upon.

- Proposal 14 – The SL active time of the RX UE includes the slots associated with announced periodic transmissions by the TX UE (as per SCI) [9/15].

And RAN2 agreed on the UE behavior to derive SL DRX HARQ RTT Timer based on resource assignment information for next transmission included in the SCI at #116-e meeting. This agreement means that the UE can perform active time operation based on the resource assignment information included in the SCI. Thus, rapporteur thinks that RAN2 can go with this slight majority view (proposal 14) as in the previous email discussion.

Q9: Would your company agree that the SL active time of the RX UE includes the slots associated with the announced periodic transmissions by the TX UE?

- Yes

- No

- Any others

| Company | Yes or No | Comment |
| --- | --- | --- |
| Xiaomi | No | RAN2 had sent LS to RAN1 to consider active time during resource selection. We understand RAN1 can ensure the resource is within RX UE’s active time defined by RAN2. There is no need to further introduce duplicated mechanism in RAN2 to extend active timer for announced periodic transmission. |
| Ericsson | Yes |  |
| OPPO | No | Firstly, on_duration timer + cycle is designed to cover the periodic transmission. We don’t need an additional timer; otherwise, proponent need to clarify why duplicated tools needed for the same reason. furthermore, more importantly, the active time derived from announced periodic transmission is not feasible and efficient. Unlike RTT timer derivation from retransmission, the destination of the MAC PDU to be transmitted in next period is uncertain, i.e. the Rx UE cannot know whether it needs to be active in next period. |

#### Issue 10: Down-selection of cycle and onduration

At #116-e meeting, the down-selection behaviour for cycle and onduration was agreed as follows [2].

- #116-e meeting’s agreements on down-selection of cycle/onduration:

- Working assumption: Option2 (Need of down-selection for DRX cycle and on-duration) for GC/BC when multiple QoS profiles are associated with the same DST L2 ID.

Rapporteur would like to further discuss the UE's down-selection behavior based on the above WA for the progress of RAN2. Moreover, below down-selection agreement of the inactivity timer can be applied to down-selection of cycle and onduration timers.

- #116-e meeting’s agreements on inactivity timer:

- For GC, when performing the down-selection of the inactivity timer, select the inactivity timer whose inactivity timer length is the largest one (among multiple ones for the corresponding L2 id) as the selected inactivity timer.

For example, it is possible to support down-selection of cycle and onduration timer like the following proposals suggested by many companies.

Proposal 7: 	The TX/RX UE determines the DRX cycle applied for groupcast/broadcast transmissions associated with a specific L2 destination ID as the minimum DRX cycle configured for any of the QoS profiles associated with that L2 destination ID [9].

Proposal 8: 	The TX/RX UE determines the on duration applied for groupcast/broadcast transmissions associated with a specific L2 destination ID as the maximum on duration configured for any of the QoS profiles associated with that L2 destination ID [9].

[Proposal 2]: For GC/BC, it is proposed to down-select to one DRX cycle with the shortest DRX cycle length [10].

Proposal 2: Select the length of the on-duration timer associated with the QoS profile, which is associated with the smallest DRX cycle [16].

Q10.1: Would your company agree that TX/RX UE determines the DRX cycle applied for groupcast/broadcast transmissions associated with a specific L2 destination ID as the minimum DRX cycle configured for any of the QoS profiles associated with that L2 destination ID?

- Yes

- No

- Any others

| Company | Yes or No | Comment |
| --- | --- | --- |
| Xiaomi | Yes |  |
| Ericsson | no | Actually, we don’t think the down-selection is needed. UE can just maintain multiple DRX configurations to be active at the same time. |
| OPPO | no | Agree with Ericsson. |

Q10.2: Which option would your company agree that TX/RX UE determines the on duration applied for groupcast/broadcast transmissions associated with a specific L2 destination ID?

- The maximum on duration configured for any of the QoS profiles associated with that L2 destination ID

- Select the length of the on-duration timer associated with the same QoS profile of selected DRX cycle.

- Any others

| Company | Yes or No | Comment |
| --- | --- | --- |
| Xiaomi | B | The key point is how to avoid UE to be always wakeup. If the longest on-duration timer is selected, the selected on-duration timer and selected DRX cycle may be associated to different QoS, which may result in selected on-duration timer may be longer than DRX cycle and UE would always wake up. Therefore, UE would always be active. With option c, the on-duration timer and DRX cycle are associated with the same QoS profile. UE would not always wake up. |
| Ericsson | None | We don’t think the down-selection is needed. |
| OPPO | None | - Is not feasible since in a worse case a longest on_duration timer and a shortest cycle might make the UE active all the time; - Is not feasible since there maybe multiple “minimum DRX cycle” being selected according to Q10.1, i.e. still multiple on_duration timers. that’s why we did not support the down-selection, i.e., there seems no feasible solution on the table. |

#### Issue 11: Down-selection of Inactivity timer

At #116-e meeting, the down-selection behaviour for inactivity timer was agreed as follows [2].

- #116-e meeting’s agreements on inactivity timer:

- For GC, when performing the down-selection of the inactivity timer, select the inactivity timer whose inactivity timer length is the largest one (among multiple ones for the corresponding L2 id) as the selected inactivity timer.

According to the above agreement, the UE selects the largest value among the inactivity timer values ​​of multiple QoS profiles related to the L2 DST ID during down-selection of the inactivity timer. However, if the UE succeeds in SCI decoding and fails in MAC PDU decoding, the intended 24-bit full L2 DST ID cannot be acquired. If only the above RAN2 agreement is considered, the down-selection behavior when the receiving UE acquires only the L1 DST ID through the received SCI is ambiguous. For example, if the RX UE fails to decode the MAC PDU and obtains only the L1 DST ID from the SCI, it is unclear whether the RX UE's behavior of selecting the largest activity timer among all L2 DSTs mapped to the partial L1 ID matches the above agreement. Thus, rapporteur believes that RAN2 should further discuss the down-selection behavior in the scenario where an Rx UE fails to decode the MAC PDU.

Q11.1: Would your company agree that RAN2 should further discuss the down-selection of inactivity timer when an Rx UE fails to decode MAC PDU (i.e., when only L1 DST ID is obtained through the received SCI)?

- Yes

- No

- Any others

| Company | Yes or No | Comment |
| --- | --- | --- |
| Xiaomi | No | In this case, UE can’t acknowledge whether the MAC PDU is addressed to itself. So, UE just ignore the MAC PDU and doesn’t start inactivity timer. |
| Ericsson | no | This is an optimization issue, RAN2 can focus on other more important issues. |
| OPPO | NO | Agree with Ericsson. |

Q11.2: If you answered yes to Q10.1, what UE behavior can RAN2 consider?

| Company | Option | Comment |
| --- | --- | --- |

#### Issue 12: LCP

At #113b-e meeting, the LCP enhancement considering SL DRX was agreed as following [11]:

- #113b-e meeting’s agreements on down-selection of cycle/onduration:

- RAN2 assumes LCP enhancements for ensuring a TX UE transmits data in the active time of an RX UE are needed. FFS on the resource (re)selection enhancements (e.g. limiting the resources to the active time for peer UE).

In addition, many companies proposed the LCP procedure of selecting a destination in consideration of the active time as follows:

Proposal 17: During LCP for broadcast/groupcast, only those logical channels should be considered for Destination selection and TB generation procedure that are in Active Time for a corresponding SL grant [12].

Proposal 18: For unicast transmissions, one additional condition is required for the destination selection, that Tx UE considers only those SL LCH(s) for the selection of the Destination whose corresponding DRX ActiveTime matches with the allocated SL resources, e.g. SL resources allocated by gNB are within the DRX ActiveTime of the SL LCH(s) [12].

Proposal 15	Upon reception of a SL grant, the MAC layer selects a suitable destination based on active time of each destination. FFS details of the selection [5].

Proposal 4: The LCP enhancement due to DRX is that the Destination(s), if the grant would not fall into its active time, will be exlcluded before Destination selection in current LCP procedure [3].

Proposal 3:	In SL LCP, the Tx UE should select a destination associated with a Rx UE that is in SL active time for the SL transmission occasion. FFS for the case when the SL grant includes more than one transmission occasions [13].

Proposal 8: When selecting a destination for a SL grant, among the total candidate destinations, only consider the destinations that have the active time overlapping with the SL grant [14].

The proposals are clear LCP procedure considering the RAN2 agreement. Thus, RAN2 can agree that the Tx UE should select a destination associated with an Rx UE that is in SL active time for the SL transmission occasion in SL LCP.

Q12: Would your company agree that the Tx UE should select a destination associated with an Rx UE that is in SL active time for the SL transmission occasion in SL LCP?

- Yes

- No

- Any others

| Company | Yes or No | Comment |
| --- | --- | --- |
| Xiaomi | Yes |  |
| Ericsson | Yes |  |
| OPPO | Yes |  |

#### Issue 13: Issues on resource (re)selection

At #116-e meeting, UE behaviour for resource selection were agreed as follows [2].

- #116-e meeting’s agreements on candidate resource selection:

- TX UE shall select initial transmission resource only in the RX UE’s active time where SL DRX timers are running now or will be running in future (at least on-duration timer). Further details of active time can be considered later. FFS on spec impact.

- If RAN 2 agrees that TX UE shall select initial transmission resource only in the RX UE’s active time, it is applied for all cast types.

- For each SL grant, the grant is used if it is in active time of at least one destination; otherwise the grant is skipped.

- MAC indicates the active time information to PHY.

- It is up to RAN1 to select an option.

- We will send LS to inform RAN1 of the related agreements from this offline discussion [706].

First, RAN2 should determine whether the MAC layer provides the RX UE’s active time where SL DRX timers are running now or will be running in future to the Physical layer when the UE triggers the resource (re)selection.

Q13.1: Which options of the RX UE’s active time that the MAC layer provides to the Physical layer would your company prefer when the Tx UE triggers the resource (re)selection?

- RX UE’s active time where SL DRX timers are running now.

- RX UE’s active time where on duration timer will be running in future.

- RX UE’s active time where inactivity timer will be running in future.

- RX UE’s active time where retransmission timer will be running in future.

- Any others

| Company | Option | Comment |
| --- | --- | --- |
| Xiaomi | e | We think it’s UE’s internal implementation, which can be unspecified. |
| Ericsson | e | In order to more RAN2 discussion forward, we share the same view as xiaomi, this can be just left to UE implementation. |
| OPPO | e | Agree with xiaomi and Ericsson that it should be UE implementation. Specifically, it is unclear how to define the “RX UE’s active time..” since the RX-UE selection is done at LCP afterwards, and no matter which Rx-UE selected before resource selection, it might end up with difference compared with the Rx-UE selected at LCP.. |

As shown below, RAN2 agreed on the UE behaviour considering RX UE’s active time where SL DRX timers (e.g., inactivity timer or retransmission timer) are running now.

- #115-e meeting’s agreements on resource selection [4]:

- For unicast, the TX UE selects the resources for the initial transmission associated with any active time (e.g. on duration timer or inactivity timer, or retransmission timer) at the RX UE. How to handle cases when a transmission may cause these timers to be running at the RX UE is FFS. FFS on groupcast. FFS on whether any spec impact.

- For unicast, the TX UE can select the resources for the retransmission associated with any active time (e.g. on duration timer or inactivity timer, or retransmission timer) at the RX UE.  How to handle cases when a transmission may cause these timers to be running at the RX UE is FFS. FFS on groupcast. FFS on whether any spec impact.

- For broadcast, the TX UE can select the resources for the initial transmission associated with any active time supported by broadcast (i.e. on duration timer) at the RX UE.

In addition to #115-e meeting’s agreements, as an open issue related to resource selection, it should be further discussed whether the TX UE can consider TX resources within the RX UE’s active time where SL DRX timers (e.g., inactivity timer or retransmission timer) will be running in future when the TX UE selects transmission resources.

Moreover, the resource (re)selection procedure of the TS 38.321 is specified separately for single MAC PDU transmission and multiple MAC PDU transmission. Thus, when considering the resource (re)selection procedure of the TX UE considering the SL DRX active time of the RX UE, not only single MAC PDU transmission but also multiple MAC PDU transmission should be considered.

Q13.2: Which option of the Tx resource would your company prefer when the Tx UE selects initial transmission resource for single MAC PDU transmission?

- For initial transmission for single MAC PDU, the TX UE can consider TX resource within RX UE’s active time where SL DRX timers are running now.

- For initial transmission for single MAC PDU, the TX UE can consider TX resource within RX UE’s active time where on duration timer will be running in future.

- For initial transmission for single MAC PDU, the TX UE can consider TX resource within RX UE’s active time where inactivity timer will be running in future.

- For initial transmission for single MAC PDU, the TX UE can consider TX resource within RX UE’s active time where retransmission timer will be running in future.

- select resources according to the existing procedure in the MAC

| Company | Option | Comment |
| --- | --- | --- |
| Xiaomi | E | This depends on RAN1 design, i.e. the candidate resources provided by PHY. We can wait for RAN1 progress. |
| Ericsson | e | We think MAC layer can just provide a list of active time to the PHY layer, how to select the resource is up to RAN1. As soon as PHY layer has provided resources to the MAC layer, the MAC layer can just follow the existing procedure in the MAC. |
| OPPO | e | Same view as Ericsson. |

Q13.3: Which option of the Tx resources would your company prefer when the Tx UE selects retransmission resources for single MAC PDU transmission?

- For retransmission for single MAC PDU, the TX UE can consider TX resources within RX UE’s active time where SL DRX timers are running now.

- For retransmission for single MAC PDU, the TX UE can consider TX resources within RX UE’s active time where on duration timer will be running in future.

- For retransmission for single MAC PDU, the TX UE can consider TX resources within RX UE’s active time where inactivity timer will be running in future.

- For retransmission for single MAC PDU, the TX UE can consider TX resources within RX UE’s active time where retransmission timer will be running in future.

- select resources according to the existing procedure in the MAC

| Company | Option | Comment |
| --- | --- | --- |
| Xiaomi | E | This depends on RAN1 design, i.e. the candidate resources provided by PHY. We can wait for RAN1 progress. |
| Ericsson | e | We think MAC layer can just provide a list of active time to the PHY layer, how to select the resource is up to RAN1. As soon as PHY layer has provided resources to the MAC layer, the MAC layer can just follow the existing procedure in the MAC. |
| OPPO | e | Same view as Ericsson. |

Q13.4: Which option of the Tx resource would your company prefer when the Tx UE selects initial transmission resource for transmissions of multiple MAC PDUs?

- For initial transmission for multiple MAC PDUs, the TX UE can consider TX resource within RX UE’s active time where SL DRX timers are running now.

- For initial transmission for multiple MAC PDUs, the TX UE can consider TX resource within RX UE’s active time where on duration timer will be running in future.

- For initial transmission for multiple MAC PDUs, the TX UE can consider TX resource within RX UE’s active time where inactivity timer will be running in future.

- For initial transmission for multiple MAC PDUs, the TX UE can consider TX resource within RX UE’s active time where retransmission timer will be running in future.

- select resources according to the existing procedure in the MAC

| Company | Option | Comment |
| --- | --- | --- |
| Xiaomi | E | This depends on RAN1 design, i.e. the candidate resources provided by PHY. We can wait for RAN1 progress. |
| Ericsson | e |  |
| OPPO | e |  |

Q13.5: Which option of the Tx resources would your company prefer when the Tx UE selects retransmission resources for transmissions of multiple MAC PDUs?

- For retransmission for multiple MAC PDUs, the TX UE can consider TX resources within RX UE’s active time where SL DRX timers are running now.

- For retransmission for multiple MAC PDUs, the TX UE can consider TX resources within RX UE’s active time where on duration timer will be running in future.

- For retransmission for multiple MAC PDUs, the TX UE can consider TX resources within RX UE’s active time where inactivity timer will be running in future.

- For retransmission for multiple MAC PDUs, the TX UE can consider TX resources within RX UE’s active time where retransmission timer will be running in future.

- select resources according to the existing procedure in the MAC

| Company | Option | Comment |
| --- | --- | --- |
| Xiaomi | E | This depends on RAN1 design, i.e. the candidate resources provided by PHY. We can wait for RAN1 progress. |
| Ericsson | e |  |
| OPPO | e |  |

#### Issue 14: Issue on drx-RetransmissionTimerSL

RAN2 agreed as following RX UE to start drx-HARQ-RTT-TimerSL even when PUCCH transmission is dropped by UL/SL prioritization.

- #115-e meeting’s agreement on Uu DRX timer impact [4]

- When sl-PUCCH-Config is configured but the PUCCH is not transmitted due to UL/SL prioritization, the TX UE should start the SL-specific drx-HARQ-RTT-Timer in Uu for the corresponding SL HARQ process in the first slot/symbol after the end of the corresponding PUCCH resource.

RAN2 can further discuss and determine whether the Tx UE starts the drx-RetransmissionTimerSL or not when PUCCH (ACK or NACK) is not transmitted due to UL/SL prioritization. According to Legacy Uu DRX, drx-RetransmissionTimerDL can be started when data was not successfully decoded.

Q14: Would your company think that drx-RetransmissionTimerSL is always started after expiring drx-HARQ-RTT-TimerSL regardless of whether the unsent PUCCH is ACK or NACK?

- Yes

- No (i.e., drx-RetransmissionTimerSL is started after expiring drx-HARQ-RTT-TimerSL when the unsent PUCCH is NACK)

- Any others

| Company | Yes or No | Comment |
| --- | --- | --- |
| Xiaomi | Yes | gNB would schedule retransmission if PUCCH is not sent. If UE doesn't wake up, gNB would continuously schedule retransmission due to no PUCCH received. |
| Ericsson | Yes |  |
| OPPO | No | We should follow the legacy Uu retransmission timer mechanism, i.e. only start the drx-RetransmissionTimerSL when the next retransmission is needed (unsent PUCCH is NACK). For the problem from xiaomi, we think nothing is broken, NW implementation absolutely can handle the case where no PUCCH is received. Please note that PUCCH dropping may happen in Uu DL reception, and there is no specification work on that, i.e. just rely on NW implementation. |

#### Issue 15: Issue on sl-drx-RetransmissionTimer

RAN2 agreed to start the RTT timer even when the Rx UE fails to transmit the PSFCH due to UL/SL prioritization.

- #113-e meeting’s agreement on SL DRX timer [11]

- If the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g. due to UL/SL prioritization) the RX UE still starts the HARQ RTT timer in the symbol/slot following the end of PSFCH resource.

For clarification, RAN2 needs to check whether the above agreement is applicable to GC NACK only. In other words, in the case of GC NACK only, if the unsent PSFCH of the RX UE is NACK, the TX UE determines that the unsent PSFCH is ACK, so it needs clarification whether a separate procedure should be defined according to whether the unsent PSFCH is ACK or NACK.

Q15.1: Would your company think that #113-e meeting’s agreement below also applies to GC NACK only?

“If the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g. due to UL/SL prioritization or ACK) the RX UE still starts the HARQ RTT timer in the symbol/slot following the end of PSFCH resource.”

- Yes

- No

- Any others

| Company | Yes or No | Comment |
| --- | --- | --- |
| Xiaomi | Yes | According to companies’ responses in AT discussion, this is the natural consequence. |
| Ericsson | No | For GC NACK only, there is no point to start the RTT timer and also retransmission timer since TX UE will anyway not perform retransmission |
| OPPO | No | Agree with Ericsson. |

Q15.2: Would your company think that sl-drx-RetransmissionTimer is always started after expiring sl-drx-HARQ-RTT-Timer if feedback is unsent in GC NACK only?

- Yes

- No

- Any others

| Company | Yes or No | Comment |
| --- | --- | --- |
| Xiaomi | No | In GC NACK only, TX UE assumes unsent PSFCH as ACK. Therefore, TX UE would not perform retransmission. It’s pointless for RX UE to start RTX timer. |
| Ericsson | No | For GC NACK only, there is no point to start the RTT timer and also retransmission timer since TX UE will anyway not perform retransmission |
| OPPO | No |  |

#### Issue 16: sl-drx-startoffset

RAN2 agreed following options to determine the sl-drx-startoffset.

- #116-e meeting’s agreement on sl-drx-startoffset [2]

- RAN2 to select one of the following options to determine the sl-drx-startoffset:

Option-1:

- n=DST L2 ID MOD N, where N is the total number of sl-drx-startoffset values, and n is an index in the N sl-drx-startoffset values.

Option-5:

- sl-drx-StartOffset (ms) = DST L2 ID MOD sl-drx-Cycle (ms)

In the email discussion [15] of the previous meeting, the voting results for each option were as follows, and similar results would be expected even if the discussion is reconsidered.

Option-1: 12

Option-5: 10

RAN2 can have a chance to hear companies’ opinions on the two options. Rather than sticking to the previous answer, I would appreciate it if you could flexibly think about other options and reply.

Q16: Which option of the equation to determine the sl-drx-startoffset would your company prefer?

- Option-1

- Option-5

- Any others

| Company | Yes or No | Comment |
| --- | --- | --- |
| Xiaomi |  | Don’t have strong view. Can follow majority. |
| Ericsson | a |  |
| OPPO | b | Option 5 is a cleaner solution. I.e., it is purely based on the formula, while option-1 is to combine with a index definition “n is an index in the N sl-drx-startoffset values. ” which is not clear to us on the mechanism/benefit, so not prefer.. |

# Conclusion and recommendation

# Reference

- R2-2111418		Summary of open issues for 38.321 running CR, LG

- R2-2111298		#116-e meeting report from session on LTE V2X and NR SL, Session Chair (Samsung)

- R2-2110650		Remaining issues for sidelink DRX, vivo

- R2-2108838		#115-e meeting report from session on LTE V2X and NR SL, Session Chair (Samsung)

- R2-2109907		Remaining aspects of SL DRX, Ericsson

- R2-2109610		Remaining issues of SL communication impact on Uu DRX	Huawei, HiSilicon	discussion

- R2-2110155		Discussion on remaining issues and further consideration on SL DRX, LG

- R2-2107268		 [POST114-e][706][V2X/SL] Discussion on remaining FFSs/open issues in SL DRX timer maintenance (InterDigital)

- R2-2109937		Remaining aspects on SL DRX Timers,	InterDigital

- R2-2110938	Open issues on SL DRX operation in groupcast,	Samsung Research America

- #113b-e meeting report from session on LTE V2X and NR SL, Session Chair (Samsung)

- R2-2109724	DRX Active time, Sensing and Configuration aspects	Lenovo, Motorola Mobility

- R2-2111008	Discussion on remaining issues on Sidelink DRX	ASUSTeK

- R2-2111204	Remaining issues of the sidelink DRX for unicast	Huawei, HiSilicon

- R2-2110680	Summary of [Post115-e][715][SL] Determination of DRX timer length and start time(vivo)

- R2-2110225	Discussion on Sidelink DRX for broadcast and groupcast	Xiaomi