3GPP TSG-RAN WG2 Meeting #116bis electronic	draft R2-220xxxx

Online Meeting, Jan 17th – 25th 2022

Agenda item:	8.23.2

Source:	CATT

Title:            Report of [Post116bis-e][053][UDC] CRs and LS out (CATT)

WID/SID:	NR_UDC-Core

Document for:	Discussion and Agreement

- 1	Introduction

This document is for summary of the following discussions:

- [Post116bis-e][053][UDC] CRs and LS out (CATT)

Scope: Take agreements into account. Review updated CRs. Endorse if possible (technical endorsement). LS out to RAN3 according to agreement.

Intended outcome: CRs (Endorsed if possible), Approved LS out

Deadline: Short

The participants are invited to leave their contact information in the following table.

| Company | Contact: Name (E-mail) |
| --- | --- |
| Samsung | Donggun Kim (s_dg.kim@samsung.com) |
| Huawei, HiSilicon | Jun Chen (jun.chen@huawei.com) |
| Apple | Ralf Rossbach (rrossbach@apple.com) |

- 2	Review of the updated draft CRs

The draft CRs [1]-[5] have been updated (available in the server folder), taking into account the AT-meeting discussions (see R2-2201914), as well as agreements as shown in the Appendix.

Please provide your comments if any in the following tables.

## 2.1 draft CR for TS 38.300

Rapporteur’s note: The only change of this version of 38.300 CR than [1] is addition of one more so-sourcing company.

Question 1	Please provide your comments if any to the updated draft CR of 38.300

| Company | Comments if any |
| --- | --- |
| Huawei, HiSilicon | No comments for now. |

## 2.2 draft CR for TS 38.331

Question 2	Please provide your comments if any to the updated draft CR of 38.331

| Company | Comments if any |
| --- | --- |
| Huawei, HiSilicon | We have two comments. (1) The following editorial correction may be left to TS 38.331 Rapporteur. Ehc-Uplink Indicates the configurations that apply for only uplink. If the field is configured, then Ethernet header compression is configured for uplink. Otherwise, it is not configured for uplink. (2) for the following change for inter-node message, we think it is better to use INTEGER (0..2) so that the MN can indicate at most 0, 1 or 2 UDC DRBs for the SN. If the MN indicates value 0 for the SN, it means SN should not configure UDC for the UE, and it is clear for the network side. maxNumberUDC-DRB-r17 INTEGER(1..2) OPTIONAL |
| Apple | 1) Since maxNumberUDC-DRB-r17 is an optional parameter the addition of value 0 (as proposed by Huawei) is not necessarily required, but no strong view. 2) For the following part it seems that the last OPTIONAL (in red) is not needed, we propose to delete it. udc-r17 SEQUENCE { standardDictionary-r17 ENUMERATED {supported} OPTIONAL, operatorDictionary-r17 SEQUENCE { versionOfDictionary-r17 INTEGER (0..15), associatedPLMN-ID-r17 PLMN-Identity } OPTIONAL, continueUDC-Context-r17 ENUMERATED {supported} OPTIONAL } OPTIONAL |

## 2.3 draft CR for TS 38.323

Question 3	Please provide your comments if any to the updated draft CR of 38.323

| Company | Comments if any |
| --- | --- |
| Samsung | Regarding PDCP re-establishment procedure, to avoid decompression failure resulted from mismatch, the UE should continue to apply UDC only to the PDCP SDU which has not been compressed before, as in normal scenario if UDC continuation is configured, i.e. UE should not apply UDC compression to the PDCP SDU which has been compressed before. Note that ciphering and integrity protection should be applied to UDC data block again with new security key. However, UE can apply UDC to the PDCP SDU as in LTE if UDC continuation is not configured, because the compression buffer is reset as follows: - for AM DRBs whose PDCP entities were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below: - perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4; - perform uplink data compression of the PDCP SDU if drb-ContinueUDC is not configured, as specified in the subclause 5.X.4; - perform uplink data compression of the PDCP SDU which has not been compressed before and if drb-ContinueUDC is configured, as specified in the subclause 5.X.4; NOTE: If drb-ContinueUDC is configured and if the PDCP SDU has been compressed before, UE performs integrity protection and ciphering of UDC data block using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8, . - perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8; - submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1. |
| Huawei, HiSilicon | We are basically fine with Samsung’s text, and we have further comments as below: (1) We think the note should be moved to normative text because it is a requirement for UE behaviour (2) We have a small suggestion on the wording for the Note, i.e. UE performs integrity protection and ciphering of UDC data block is changed into UE performs integrity protection and ciphering of the PDCP SDU (containing UDC header and UDC data block) |
| Qualcomm | We basically agree with Samsung’s proposal. But we are not sure whether the Note is needed. If go through the spec text line by line, after performing uplink data compression of the PDCP SDU, there is already one step to specify that - perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8; UE anyway needs to perform IP and ciphering again for the compressed data PDU |
| Apple | Regarding the update for drb-ContinueUDC proposed by Samsung, we support the principle of performing uplink data compression for the PDCP SDU with has not been compressed before if drb-ContinueUDC is configured. However, we think it can be specified in a slightly more direct way. - for AM DRBs whose PDCP entities were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below: - perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4; - if drb-ContinueUDC is not configured; or - if drb-ContinueUDC is configured and the PDCP SDU has not been compressed before: - perform uplink data compression of the PDCP SDU as specified in the subclause 5.X.4; - perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8; - submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1. This way no NOTE needs to be added. Remark: The very part of “drb-ContinueUDC is configured and” is not strictly required by the logic and could be taken out; it is preferred to aid readability. |

## 2.4 draft CR for TS 38.306

Question 4 	Please provide your comments if any to the updated draft CR of 38.306

| Company | Comments if any |
| --- | --- |
| Huawei, HiSilicon | The wording “continueUDC-Context” can be improved, i.e. change it into “continueUDC-Buffer” or “continueUDC”. The wording “context” has not been used for UDC functionality, and instead, we are using “UDC buffer” instead of “UDC context” in the latest TS 38.323 CR (see a text as below). UDC works on the condition that compression buffer and de-compression buffer are synchronized. UDC buffer reset mechanism is to resynchronize buffer when error is detected. For resynchronization, UE shall reset the compression buffer to all zeros. For other “context” (e.g. UDC context), we also suggest to replace “context” with “buffer”. For the following change, we have a small suggestion: If UE supports operator defined dictionary, the UE shall report versionofDictionary-17 and associatedPLMN-ID-r17 of the stored operator defined dictionary as defined in TS 38.331 [3]. This parameter is not required to be present if the UE is in VPLMN. The associatedPLMN-ID-r17 is only associated to the operator defined dictionary which has no relationship with UE’s HPLMN ID. |
| Qualcomm | Regarding the naming ‘continueUDC-Context’, it is indeed nowhere mentioning ‘context’ of UDC so far. However, we also not prefer to use ‘buffer’. Because in 38.331, the behaviour description of ‘drb-ContinueUDC’ only states whether the PDCP entity continues or resets the uplink data compression protocol during PDCP re-establishment. Thus, we think ‘continueUDC’ is already good enough for now. For the capability description, we have similar comments with Huawei. In addition, we are wondering why we have this sentence. ‘The associatedPLMN-ID-r17 is only associated to the operator defined dictionary which has no relationship with UE’s HPLMN ID.’ And should we just copy the similar from LTE UDC which is ‘In this release of specification, UE can only support one operator defined dictionary.’. |
| Apple | We are OK to keep ‘context’ in the name but have no strong view to remove it either. Potential update to the capability description: continueUDC-r17 Defines whether the UE supports continuation of uplink data compression protocol operation where the UE does not reset the current UDC buffer upon PDCP re-establishment, as specified in TS 38.323 [16]. |

## 2.5 draft CR for TS 37.340

Question 5 	Please provide your comments if any to the updated draft CR of 37.340

| Company | Comments if any |
| --- | --- |
| Samsung | The change seems OK but it may be misleading that UDC can be configured for all the bearer types, regardless of the association with RLC AM entities, given that the text proposal is almost the same as that of ROHC and EHC. As you know, ROHC and EHC can be configured for all bearer types, regardless of RLC modes. Even if 38.331 has a restriction that RLC AM is mandated when UDC is configured, the same text as that ROHC and EHC should be avoided. For example, we can clarify this as follows: In MR-DC, RoHC and EHC (as described in TS 36.323 [15] and TS 38.323 [16]) can be configured for all the bearer types. In MR-DC with 5GC, UDC (as described in TS 38.323 [16]) can be configured for all the bearer types using RLC AM. |
| Huawei, HiSilicon | We are Ok with Samsung’s suggestions. |
| Apple | The restriction on RLC AM suggested by Samsung may not be specific to MR-DC and is already captured in 38.331. But no strong view to keep it. |

Summary for section 2:

TBD

- 3	Discussions on LS draft to RAN3

Following the agreement below, a LS draft has been prepared, for which companies’ comments are invited.

- Send an LS to RAN3 to inform of NR UDC potential impact to CU-CP/UP splitting scenario. R2 understands that decisions as well as the required specification work are up to RAN3.

Question 6 	Please share your comments if any on the LS draft to RAN3

| Company | Yes or no | Comments if any |
| --- | --- | --- |

Summary for section 3

TBD

- 4	Conclusion

TBD

- 5	References

[1] R2-2201277		Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung	draftCR	Rel-17	38.300	16.8.0	B	NR_UDC-Core

[2] R2-2201278		Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung	draftCR	Rel-17	38.331	16.7.0	B	NR_UDC-Core

[3] R2-2201279		Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung	draftCR	Rel-17	38.323	16.6.0	B	NR_UDC-Core

[4] R2-2201280		Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung	draftCR	Rel-17	38.306	16.7.0	B	NR_UDC-Core

[5] R2-2201281		Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, Samsung	draftCR	Rel-17	37.340	16.8.0	B	NR_UDC-Core

- Appendix Agreements for NR UDC

- RAN#116bis-e

On CRs

R2-2201277	Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung	draftCR	Rel-17	38.300	16.8.0	B	NR_UDC-Core

R2-2201278	Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung	draftCR	Rel-17	38.331	16.7.0	B	NR_UDC-Core

R2-2201279	Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung	draftCR	Rel-17	38.323	16.6.0	B	NR_UDC-Core

R2-2201280	Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, ZTE, Samsung	draftCR	Rel-17	38.306	16.7.0	B	NR_UDC-Core

R2-2201281	Introduction of the support for UDC in NR	CATT, CMCC, Huawei, HiSilicon, MediaTek, Ericsson, China Unicom, China Telecom, OPPO, Samsung	draftCR	Rel-17	37.340	16.8.0	B	NR_UDC-Core

- Can Use these CRs as a baseline for further work (except 37340 CR which may not be needed dependent on further agreements)

On email summary in R2-2200039

- The parts without TBD in Table 1 are assumed to directly follow LTE UDC mechanism.

- UDC is not applied to the SDAP header and SDAP control PDU.

- The UDC header is located after SDAP header in the UDC PDU format.

- UDC is not applied to DAPS in NR.

- NR UDC is not applied to sidelink DRBs.

- With Figure 4.2.2-1, there is no need to further clarify UDC decompression being performed after PDCP re-ordering in the specification.

- UE shall support number of UDC DRBs 2. FFS if we need to support some additional UE capability.

- Continue by email, can include tech proposals from tdocs below (proponents are expected to request), continue on the non-agreed parts, review CRs.

On offline summary R2-2201914

[Change the UE cap FFS into: FFS whether UE data rate limitation with UDC need to be provided as a UE capability.] Chair: The FFS for the UE cap agreement above is removed, and the below is agreed instead.

- FFS whether UE data rate limitation with UDC need to be supported with a UE capability.

- UDC continuity can be configured for the same cases as ROHC continuity

- Assume that P2 and P5 can be supported, CRs for review to next meeting anyway. If issues are found R2 can revert this assumption (at next meeting).

- P2: UDC is supported for non-split bearer type in NR-DC. It is supported that MN sends to SN the maximum number of UDC DRBs that can be configured by SN. FFS if any other coordination is needed.

- P5: Support NR UDC for MR-DC and split bearer type, with the following restrictions

- - Only include NR-DC, NGEN-DC, and NE-DC (i.e., EN-DC is not supported)

- - No enhancements supported for potential data loss for split bearer case.

- Send an LS to RAN3 to inform of NR UDC potential impact to CU-CP/UP splitting scenario. R2 understands that decisions as well as the required specification work are up to RAN3.

- Update CRs taking into acct all agreements, review in an offline discussion, tech. endorse if possible.