3GPP TSG-RAN WG2 #116bis-e	R2-22xxxxx

E-meeting, 17 – 25 January 2022

Agenda item:	8.4.2.4

Source: 	Samsung (rapporteur)

Title: 	Summary of discussion [Post116bis-e][077][eIAB] 38321 (Samsung)

Document for:	Discussion and decision

# Introduction

This document captures the following discussion:

- [Post116bis-e][077][eIAB] 38321 (Samsung)

Scope: Updated running CR taking into account agreements of R2-116bis-e. Best effort review. Endorsement if possible. Capture TS related Open Issues, not captured elsewhere and suggest how to treat.

Intended outcome: Updated Running CR, reviewed, baseline for next meeting. TS related Open issue with suggestion how to treat.

Deadline: Short.

Section 2 captures individual companies’ comments on rapporteur’s draft running CR for the MAC spec, based on the baseline endorsed at RAN2#116bis-e and captured in R2-2201850, and the further agreements made at the same meeting (please see R2-2201876 for further info). The goal is to update the running MAC CR, and endorse the new iteration if possible.

A further goal is to capture any TS-related issues not captured elsewhere, and Section 3 captures this.

# Running MAC CR

Companies are invited to comment on the running CR updated by the rapporteur and available at

www.3gpp.org / ftp / email_discussions / RAN2 / [RAN2#116bis-e] / [Post116bis-e][077][eIAB] 38321 (Samsung),

using the following table:

COMMENTS ON CR VERSION (FILE NAME LABELLED v0):

| Company | Section comment relates to / General comment | Comment | Rapporteur’s response |
| --- | --- | --- | --- |
| Apple | 5.18.19 | For additional information on the timing modes, the additions for the timing case indications Case-1, Case-6, and Case-7 could be linked with a reference to TS 38.213, clause 14. The IAB-MT on the child node may inform the parent IAB-DU or IAB-donor-DU about the number of guard symbols desired via the Desired Guard Symbols MAC CE. IAB-MT transmission timing modes are specified in TS 38.213 [6], clause 14. |  |

# Open issues

If you have identified any TS38.321 related issues related to our eIAB work and not captured in the above running MAC CR discussion, please provide the details in the Table immediately below:

| Company | Issue | Suggestion on how to treat it |
| --- | --- | --- |

# Conclusions