3GPP TSG-RAN WG2 Meeting #116bis Electronic						R2-22xxxxx

Online, Jan. 17th – Jan. 25th, 2022

Title:	[Draft] LS on MBS SPS

Response to:	-

Release:	Release 17

Work Item:	NR_MBS

Source:	RAN2

To:	RAN1

Contact person:	Shukun Wang

wangshukun@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

- 1	Overall description

RAN2 has discussed MBS SPS related issues and made the following agreements:

- One-to-many mapping between G-CS-RNTI and MBS sessions is supported and it is assumed that this does not introduce additional specification work.

- Capture CS-RNTI usage in table for MBS in section 7.1 in MBS MAC running CR, i.e. for PTP for PTM retransmission via CS-RNTI and MBS SPS deactivationvia CS-RNTI when MBS SPS is configured.

- If MBS SPS is configured and CS-RNTI is not configured, the retransmission of SPS via PTP is not supported and MBS SPS deactivation via CS-RNTI is not supported.

- The sps-ConfigIndex should unique in UE no matter the SPS is for unicast or multicast.

Besides, RAN2 has the following understanding:

RAN2’s understanding: There can be multiple MBS SPS-configs and multiple G-CS-RNTIs. However, the association between G-CS-RNTIs and MBS SPS-configs will not be specified in RRC signalling. The DCI scrambled with G-CS-RNTI will indicate which MBS SPS-config will be activated for G-CS-RNTI via HARQ process ID field which equals to sps-ConfigIndex in an MBS SPS-config.

Based on this, RAN2 understands that one to one mapping or one to multiple mapping from G-CS-RNTI to MBS SPS-config is supported as legacy. However, if multiple G-CS-RNTIs were mapped to the same MBS SPS-config, it is not clear how that would work. Thus, RAN2 would like to ask the following question:

Q1:RAN2 respectfully asks RAN1 to confirm RAN2’s understanding and the maximal number of G-CS-RNTI configured for UE? Can multiple G-CS-RNTIs be mapped to same MBS SPS-config and if so how that would work?

In addition, RAN2 discussed the handling of retransmission and wonders if retransmission (i.e. via PTM or PTP) can be changed per TB or per TB per transmission, or configured in RRC signalling and whether a single CS-RNTI is used for PTP retransmissions of all G-CS-RNTIs?

Q2: Clarify the handling of PTM retransmissions.

- 2	Actions

To RAN1:

RAN2 respectfully asks RAN1 to take into account the above RAN2 agreements and respond to the Q1 and Q2. RAN2 expects the response LS in the first week in next meeting for progress.

- 3	Dates of next RAN2 meetings

TSG-RAN2 Meeting #117-e 	Feb. 21 – Mar. 03, 2022

TSG-RAN2 Meeting #118-e 	May 16 – May 27, 2022