3GPP TSG-RAN WG2 Meeting #116bis Electronic						R2-22xxxxx

Online, Jan. 17th – Jan. 25th, 2022

Title:	[Draft] LS on MBS SPS

Response to:	-

Release:	Release 17

Work Item:	NR_MBS

Source:	RAN2

To:	RAN1

Contact person:	Shukun Wang

wangshukun@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

- 1	Overall description

Based on RAN1 agreements, RAN2 discussed the MBS related issue and RAN2 made following agreements.

- One-to-many mapping between G-CS-RNTI and MBS sessions is supported and it is assumed that this does not introduce additional specification work.

- Capature CS-RNTI usage in table for MBS in section 7.1 in MBS MAC running CR, i.e. for PTP for PTM retransmission via CS-RNTI and MBS SPS deactivationvia CS-RNTI when MBS SPS is configured.

- If MBS SPS is configured and CS-RNTI is not configured, the retransmission of SPS via PTP is not supported and MBS SPS deactivation via CS-RNTI is not supported.

- The sps-ConfigIndex should unique in UE no matter the SPS is for unicast or multicast.

Agreement: (RAN1#104) For RRC_CONNECTED UEs, more than one SPS group-common PDSCH configuration for MBS can be configured per UE subject to UE capability - The total number of SPS configurations supported by a UE currently defined for unicast is not increased due to additionally supporting MBS. Agreement: (RAN1#106) If a SPS-config for MBS is configured in CFR, one G-CS-RNTI is associated with the SPS-config. - FFS: Multiple G-CS-RNTIs associated with one SPS-config Agreement: (RAN1#106bis) The association between a G-CS-RNTI and a SPS-Config-Multicast is indicated by the activation GC-PDCCH for SPS GC-PDSCH, i.e., a value of the HARQ process number field in a DCI format indicates an activation for a SPS GC-PDSCH configuration for multicast with a same value as provided by sps-ConfigIndex in a SPS-Config-Multicast.

Based on RAN1 agreements above, RAN2 made following understanding and confused whether multiple to one mapping between G-CS-RNTI and SPS is supported or not.

RAN2’s understanding: There will be multiple MBS SPS-config and there will also be multiple G-CS-RNTI. However, the association between G-CS-RNTI and MBS SPS-config will not be specified in RRC signalling. The DCI scrambled with G-CS-RNTI will indicated which MBS SPS-config will be activated via HARQ process id in this DCI and sps-ConfigIndex in a SPS-Config-Multicast. Then this G-CS-RNTI will be associated with the MBS SPS-config.

RAN2 can understand that one to one mapping or one to multiple mapping between G-CS-RNTI and MBS SPS config are supported as legacy today. From RAN2 point of view, multiple to one mapping between G-CS-RNTI and MBS SPS config is not supported due to the complexity. RAN2 would like to confirm the following question with RAN1.

Q1: Whether multiple to 1 mapping between G-CS-RNTI and SPS-configure are supported or not?

- 2	Actions

To RAN1:

RAN2 respectfully asks RAN1 to take into account the above RAN2 agreements and response the Q1.

- 3	Dates of next RAN2 meetings

TSG-RAN2 Meeting #117-e 	Feb. 21 – Mar. 03, 2022

TSG-RAN2 Meeting #118-e 	May 16 – May 27, 2022