3GPP TSG-RAN WG2 Meeting #117	R2-220xxxx

# eMeeting, 21st February - 3rd March, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2913 | rev | 2 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of RRC signaling for measurement gap enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-Core |  |  |  |  |  | Date: |  |  | 2022/03/06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The CR introduces the RRC signaling for the following measurement gap enhancement mechanism introduced in Rel-17. (1) Pre-configured MG pattern(s) (2) Multiple concurrent and independent MG patterns (3) Network Controlled Small Gap (NCSG) |  |  |  |  |  |  |  |  |
| Summary of change: |  | <1> For pre-confgiured MG, - Add 1 bit indication in GapConfig to indicate pre-configured measurement gap. - Add MG status indicator (activated or not) per BWP per CC - Add MG status indicator (activated or not) per SCell (for deactivated SCell) <2> For Concurrent MG, - introduce multiple gap configuration in IE MeasGapConfig (i.e. by configuring multiple GapConfig). - For association between concurrent MG and measured frequencies Indicate the associated gaps (via “gap ID”) in MO; (for PRS measurement, indicating in the association in MG configuration). <3> For NCSG - Re-use the Rel-16 NeedForGap reporting like procedure for NCSG reporting. NW configures the NCSG reporting in RRCReconfiguration and RRCResume message. UE reports the NCSG capabilities in RRCReconfigurationComplete and RRCResumeComplete messages - Configuration of NCSG gap pattern (per UE or per FR) - NCSG reporting for both intra-frequency, inter-frequency, and E-UTRA target bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI is not completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.3.5.3, 5.3.13.4, 5.5.1, 5.5.2.9, 5.5.2.11, 6.2.2, 6.3.2, 6.4, 11.2.1, 11.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R2-2111189, R2-2201903, R2-2202868, R2-2203881 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

BH RLC channel: An RLC channel between two nodes, which is used to transport backhaul packets.

CEIL: Mathematical function used to 'round up' i.e. to the nearest integer having a higher or equal value.

DAPS bearer: a bearer whose radio protocols are located in both the source gNB and the target gNB during DAPS handover to use both source gNB and target gNB resources.

Dedicated signalling: Signalling sent on DCCH logical channel between the network and a single UE.

Dormant BWP: The dormant BWP is one of downlink BWPs configured by the network via dedicated RRC signalling. In the dormant BWP, the UE stops monitoring PDCCH on/for the SCell, but continues performing CSI measurements, Automatic Gain Control (AGC) and beam management, if configured. For each serving cell other than the SpCell or PUCCH SCell, the network may configure one BWP as a dormant BWP.

Field: The individual contents of an information element are referred to as fields.

FLOOR: Mathematical function used to 'round down' i.e. to the nearest integer having a lower or equal value.

Global cell identity: An identity to uniquely identifying an NR cell. It is consisted of cellIdentity and plmn-Identity of the first PLMN-Identity in plmn-IdentityList in SIB1.

Information element: A structural element containing single or multiple fields is referred as information element.

NCSG: Network controlled small gap as defined in TS 38.133 [14].

NPN-only Cell: A cell that is only available for normal service for NPNs' subscriber. An NPN-capable UE determines that a cell is NPN-only Cell by detecting that the cellReservedForOtherUse IE is set to true while the npn-IdentityInfoList IE is present in CellAccessRelatedInfo.

NR sidelink communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [55], between two or more nearby UEs, using NR technology but not traversing any network node.

PNI-NPN identity: an identifier of a PNI-NPN comprising of a PLMN ID and a CAG -ID combination.

Primary Cell: The MCG cell, operating on the primary frequency, in which the UE either performs the initial connection establishment procedure or initiates the connection re-establishment procedure.

Primary SCG Cell: For dual connectivity operation, the SCG cell in which the UE performs random access when performing the Reconfiguration with Sync procedure.

Primary Timing Advance Group: Timing Advance Group containing the SpCell.

PUCCH SCell: An SCell configured with PUCCH.

PUSCH-Less SCell: An SCell configured without PUSCH.

RLC bearer configuration: The lower layer part of the radio bearer configuration comprising the RLC and logical channel configurations.

Secondary Cell: For a UE configured with CA, a cell providing additional radio resources on top of Special Cell.

Secondary Cell Group: For a UE configured with dual connectivity, the subset of serving cells comprising of the PSCell and zero or more secondary cells.

Serving Cell: For a UE in RRC_CONNECTED not configured with CA/DC there is only one serving cell comprising of the primary cell. For a UE in RRC_CONNECTED configured with CA/ DC the term 'serving cells' is used to denote the set of cells comprising of the Special Cell(s) and all secondary cells.

SNPN identity: an identifier of an SNPN comprising of a PLMN ID and an NID combination.

Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG, otherwise the term Special Cell refers to the PCell.

Split SRB: In MR-DC, an SRB that supports transmission via MCG and SCG as well as duplication of RRC PDUs as defined in TS 37.340 [41].

SSB Frequency: Frequency referring to the position of resource element RE=#0 (subcarrier #0) of resource block RB#10 of the SS block.

UE Inactive AS Context: UE Inactive AS Context is stored when the connection is suspended and restored when the connection is resumed. It includes information as defined in clause 5.3.8.3.

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [56], between nearby UEs, using E-UTRA technology but not traversing any network node.

<Skip>

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

1>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3:

2>	remove all the entries within VarConditionalReconfig, if any;

1>	if the RRCReconfiguration includes the daps-SourceRelease:

2>	reset the source MAC and release the source MAC configuration;

2>	for each DAPS bearer:

3>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

2>	for each SRB:

3>	release the PDCP entity for the source SpCell;

3>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

2>	release the physical channel configuration for the source SpCell;

2>	discard the keys used in the source SpCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

1>	if the RRCReconfiguration is received via other RAT (i.e., inter-RAT handover to NR):

2>	if the RRCReconfiguration does not include the fullConfig and the UE is connected to 5GC (i.e., delta signalling during intra 5GC handover):

3>	re-use the source RAT SDAP and PDCP configurations if available (i.e., current SDAP/PDCP configurations for all RBs from source E-UTRA RAT prior to the reception of the inter-RAT HO RRCReconfiguration message);

1>	else:

2>	if the RRCReconfiguration includes the fullConfig:

3>	perform the full configuration procedure as specified in 5.3.5.11;

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1>	if the RRCReconfiguration includes the masterKeyUpdate:

2>	perform AS security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the sk-Counter:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCReconfiguration includes the mrdc-SecondaryCellGroupConfig:

2>	if the mrdc-SecondaryCellGroupConfig is set to setup:

3>	if the mrdc-SecondaryCellGroupConfig includes mrdc-ReleaseAndAdd:

4>	perform MR-DC release as specified in clause 5.3.5.10;

3>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

4>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

3>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

4>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

2>	else (mrdc-SecondaryCellGroupConfig is set to release):

3>	perform MR-DC release as specified in clause 5.3.5.10;

1>	if the RRCReconfiguration message includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the radioBearerConfig2:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	if the RRCReconfiguration message includes the dedicatedNAS-MessageList:

2>	forward each element of the dedicatedNAS-MessageList to upper layers in the same order as listed;

1>	if the RRCReconfiguration message includes the dedicatedSIB1-Delivery:

2>	perform the action upon reception of SIB1 as specified in 5.2.2.4.2;

NOTE 0:	If this RRCReconfiguration is associated to the MCG and includes reconfigurationWithSync in spCellConfig and dedicatedSIB1-Delivery, the UE initiates (if needed) the request to acquire required SIBs, according to clause 5.2.2.3.5, only after the random access procedure towards the target SpCell is completed.

1>	if the RRCReconfiguration message includes the dedicatedSystemInformationDelivery:

2>	perform the action upon reception of System Information as specified in 5.2.2.4;

1>	if the RRCReconfiguration message includes the dedicatedPosSysInfoDelivery:

2>	perform the action upon reception of the contained posSIB(s), as specified in sub-clause 5.2.2.4.16;

1>	if the RRCReconfiguration message includes the otherConfig:

2>	perform the other configuration procedure as specified in 5.3.5.9;

1>	if the RRCReconfiguration message includes the bap-Config:

2>	perform the BAP configuration procedure as specified in 5.3.5.12;

1>	if the RRCReconfiguration message includes the iab-IP-AddressConfigurationList:

2>	if iab-IP-AddressToReleaseList is included:

3>	perform release of IP address as specified in 5.3.5.12a.1.1;

2>	if iab-IP-AddressToAddModList is included:

3>	perform IAB IP address addition/update as specified in 5.3.5.12a.1.2;

1>	if the RRCReconfiguration message includes the conditionalReconfiguration:

2>	perform conditional reconfiguration as specified in 5.3.5.13;

1>	if the RRCReconfiguration message includes the needForGapsConfigNR:

2>	if needForGapsConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap requirement information of NR target bands;

1>	if the RRCReconfiguration message includes the needForNCSG-ConfigNR:

2>	if needForNCSG-ConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

1>	if the RRCReconfiguration message includes the needForNCSG-ConfigEUTRA:

2>	if needForNCSG-ConfigEUTRA is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

1>	if the RRCReconfiguration message includes the sl-ConfigDedicatedNR:

2>	perform the sidelink dedicated configuration procedure as specified in 5.3.5.14;

NOTE 0a:	If the sl-ConfigDedicatedNR was received embedded within an E-UTRA RRCConnectionReconfiguration message, the UE does not build an NR RRCReconfigurationComplete message for the received sl-ConfigDedicatedNR.

1>	if the RRCReconfiguration message includes the sl-ConfigDedicatedEUTRA-Info:

2>	perform related procedures for V2X sidelink communication in accordance with TS 36.331 [10], clause 5.3.10 and clause 5.5.2;

1>	set the content of the RRCReconfigurationComplete message as follows:

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrentTwoCarrier:

3>	include in the uplinkTxDirectCurrentTwoCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the MCG;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each SCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each SCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrentTwoCarrier:

3>	include in the uplinkTxDirectCurrentTwoCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the SCG;

NOTE 0b:	It is expected that the reportUplinkTxDirectCurrentTwoCarrier is only received either in masterCellGroup or in secondaryCellGroup but not both.

2>	if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

2> if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to nr-SCG:

3>	include in the nr-SCG-Response the RRCReconfigurationComplete message;

2>	if the RRCReconfiguration includes the reconfigurationWithSync in spCellConfig of an MCG:

3>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	include the logMeasAvailable in the RRCReconfigurationComplete message;

4>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

5>	include the logMeasAvailableBT in the RRCReconfigurationComplete message;

4>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

5>	include the logMeasAvailableWLAN in the RRCReconfigurationComplete message;

3>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

4>	include connEstFailInfoAvailable in the RRCReconfigurationComplete message;

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

4>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

2>	if the RRCReconfiguration message was received via SRB1, but not within mrdc-SecondaryCellGroup or E-UTRA RRCConnectionReconfiguration or E-UTRA RRCConnectionResume:

3>	if the UE is configured to provide the measurement gap requirement information of NR target bands:

4>	if the RRCReconfiguration message includes the needForGapsConfigNR; or

4>	if the NeedForGapsInfoNR information is changed compared to last time the UE reported this information:

5>	include the NeedForGapsInfoNR and set the contents as follows:

6>	include intraFreq-needForGap and set the gap requirement information of intra-frequency measurement for each NR serving cell;

6>	if requestedTargetBandFilterNR is configured, for each supported NR band that is also included in requestedTargetBandFilterNR, include an entry in interFreq-needForGap and set the gap requirement information for that band; otherwise, include an entry in interFreq-needForGap and set the corresponding gap requirement information for each supported NR band;

3>	if the UE is configured to provide the measurement gap and NCSG requirement information of NR target bands:

4>	if the RRCReconfiguration message includes the needForNCSG-ConfigNR; or

4>	if the needForNCSG-InfoNR information is changed compared to last time the UE reported this information:

5>	include the NeedForNCSG-InfoNR and set the contents as follows:

6>	include intraFreq-needForNCSG and set the gap and NCSG requirement information of intra-frequency measurement for each NR serving cell;

6>	if requestedTargetBandFilterNCSG-NR is configured, for each supported NR band included in requestedTargetBandFilterNCSG-NR, include an entry in interFreq-needForNCSG and set the NCSG requirement information for that band; otherwise, include an entry for each supported NR band in interFreq-needForNCSG and set the corresponding NCSG requirement information;

3>	if the UE is configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands:

4>	if the RRCReconfiguration message includes the needForNCSG-ConfigEUTRA; or

4>	if the needForNCSG-InfoEUTRA information is changed compared to last time the UE reported this information:

5>	include the NeedForNCSG-InfoEUTRA and set the contents as follows:

6>	if requestedTargetBandFilterNCSG-EUTRA is configured, for each supported E-UTRA band included in requestedTargetBandFilterNCSG-EUTRA, include an entry in needForNCSG-EUTRA and set the NCSG requirement information for that band; otherwise, include an entry for each supported E-UTRA band in needForNCSG-EUTRA and set the corresponding NCSG requirement information;

1>	if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC):

2>	if the RRCReconfiguration message was received via E-UTRA SRB1 as specified in TS 36.331 [10]; or

2>	if the RRCReconfiguration message was received via E-UTRA RRC message RRCConnectionReconfiguration within MobilityFromNRCommand (handover from NR standalone to (NG)EN-DC);

3>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution for CPC:

4>	submit the RRCReconfigurationComplete message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10], clause 5.6.2a.

3>	else if the RRCReconfiguration message was included in E-UTRA RRCConnectionResume message:

4>	submit the RRCReconfigurationComplete message via E-UTRA embedded in E-UTRA RRC message RRCConnectionResumeComplete as specified in TS 36.331 [10], clause 5.3.3.4a;

3>	else:

4>	submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4/5.4.2.3;

3>	if reconfigurationWithSync was included in spCellConfig of an SCG:

4>	initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3];

3>	else:

4>	the procedure ends;

2>	if the RRCReconfiguration message was received within nr-SecondaryCellGroupConfig in RRCConnectionReconfiguration message received via SRB3 within DLInformationTransferMRDC:

3>	submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4;

3>	if reconfigurationWithSync was included in spCellConfig of an SCG:

4>	initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3];

3>	else:

4>	the procedure ends;

NOTE 1:	The order the UE sends the RRCConnectionReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

2>	else (RRCReconfiguration was received via SRB3) but not within DLInformationTransferMRDC:

3>	submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

NOTE 2:	In (NG)EN-DC and NR-DC, in the case RRCReconfiguration is received via SRB1 or within DLInformationTransferMRDC via SRB3, the random access is triggered by RRC layer itself as there is not necessarily other UL transmission. In the case RRCReconfiguration is received via SRB3 but not within DLInformationTransferMRDC, the random access is triggered by the MAC layer due to arrival of RRCReconfigurationComplete.

1>	else if the RRCReconfiguration message was received via SRB1 within the nr-SCG within mrdc-SecondaryCellGroup (UE in NR-DC, mrdc-SecondaryCellGroup was received in RRCReconfiguration or RRCResume via SRB1):

2>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution for CPC:

3>	submit the RRCReconfigurationComplete message via the NR MCG embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3.

2>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

3>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

2>	else

3>	the procedure ends;

NOTE 2a:	The order in which the UE sends the RRCReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

1>	else if the RRCReconfiguration message was received via SRB3 (UE in NR-DC):

2>	if the RRCReconfiguration message was received within DLInformationTransferMRDC:

3>	if the RRCReconfiguration message was received within the nr-SCG within mrdc-SecondaryCellGroup (NR SCG RRC Reconfiguration):

4>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

5>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

4>	else:

5>	the procedure ends;

3>	else:

4>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	else:

3>	submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	if this is the first RRCReconfiguration message after successful completion of the RRC re-establishment procedure:

3>	resume SRB2 and DRBs that are suspended;

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG, and when MAC of an NR cell group successfully completes a Random Access procedure triggered above:

2>	stop timer T304 for that cell group;

2>	stop timer T310 for source SpCell if running;

2>	apply the parts of the CSI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the respective target SpCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the respective target SpCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of that target SpCell;

2>	for each DRB configured as DAPS bearer, request uplink data switching to the PDCP entity, as specified in TS 38.323 [5];

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG:

3>	if T390 is running:

4>	stop timer T390 for all access categories;

4>	perform the actions as specified in 5.3.14.4.

3>	if T350 is running:

4>	stop timer T350;

3>	if RRCReconfiguration does not include dedicatedSIB1-Delivery and

3>	if the active downlink BWP, which is indicated by the firstActiveDownlinkBWP-Id for the target SpCell of the MCG, has a common search space configured by searchSpaceSIB1:

4>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13], of the target SpCell of the MCG;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG; or:

2>	if the reconfigurationWithSync was included in spCellConfig of an SCG and the CPC was configured

3>	remove all the entries within VarConditionalReconfig, if any;

3>	for each measId of the source SpCell configuration, if the associated reportConfig has a reportType set to condTriggerConfig:

4>	for the associated reportConfigId:

5>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

4>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

5>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

4>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup:

3>	if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2:

4>	initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information;

4>	start or restart the prohibit timer (if exists) associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration;

3>	if SIB12 is provided by the target PCell; and the UE initiated transmission of a SidelinkUEInformationNR message indicating a change of NR sidelink communication related parameters relevant in target PCell (i.e. change of sl-RxInterestedFreqList or sl-TxResourceReqList) during the last 1 second preceding reception of the RRCReconfiguration message including reconfigurationWithSync in spCellConfig of an MCG; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution and the UE is capable of NR sidelink communication and SIB12 is provided by the target PCell, and the UE has initiated transmission of a SidelinkUEInformationNR message since it was configured to do so in accordance with 5.8.3.2:

4>	initiate transmission of the SidelinkUEInformationNR message in accordance with 5.8.3.3;

2>	the procedure ends.

NOTE 3:	The UE is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

NOTE 4: The UE sets the content of UEAssistanceInformation according to latest configuration (i.e. the configuration after applying the RRCReconfiguration message) and latest UE preference. The UE may include more than the concerned UE assistance information within the UEAssistanceInformation according to 5.7.4.2. Therefore, the content of UEAssistanceInformation message might not be the same as the content of the previous UEAssistanceInformation message.

<Skip>

#### 5.3.13.4	Reception of the RRCResume by the UE

The UE shall:

1>	stop timer T319;

1>	stop timer T380, if running;

1>	if T331 is running:

2>	stop timer T331;

2>	perform the actions as specified in 5.7.8.3;

1>	if the RRCResume includes the fullConfig:

2>	perform the full configuration procedure as specified in 5.3.5.11;

1>	else:

2>	if the RRCResume does not include the restoreMCG-SCells:

3>	release the MCG SCell(s) from the UE Inactive AS context, if stored;

2>	if the RRCResume does not include the restoreSCG:

3>	release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored;

2>	restore the masterCellGroup, mrdc-SecondaryCellGroup, if stored, and pdcp-Config from the UE Inactive AS context;

2>	configure lower layers to consider the restored MCG and SCG SCell(s) (if any) to be in deactivated state;

1>	discard the UE Inactive AS context;

1>	release the suspendConfig except the ran-NotificationAreaInfo;

1>	if the RRCResume includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1>	if the RRCResume includes the mrdc-SecondaryCellGroup:

2>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

3>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

2>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

3>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

1>	if the RRCResume includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCResume message includes the sk-Counter:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCResume message includes the radioBearerConfig2:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCResume message includes the needForGapsConfigNR:

2>	if needForGapsConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap requirement information of NR target bands;

1>	if the RRCResume message includes the needForNCSG-ConfigNR:

2>	if needForNCSG-ConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

1>	if the RRCResume message includes the needForNCSG-ConfigEUTRA:

2>	if needForNCSG-ConfigEUTRA is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

1>	resume SRB2, SRB3 (if configured), and all DRBs;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	stop timer T320, if running;

1>	if the RRCResume message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	resume measurements if suspended;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if T302 is running:

2>	stop timer T302;

2>	perform the actions as specified in 5.3.14.4;

1>	enter RRC_CONNECTED;

1>	indicate to upper layers that the suspended RRC connection has been resumed;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of the of RRCResumeComplete message as follows:

2>	if the upper layer provides NAS PDU, set the dedicatedNAS-Message to include the information received from upper layers;

2>	if upper layers provides a PLMN and UE is either allowed or instructed to access the PLMN via a cell for which at least one CAG ID is broadcast:

3>	set the selectedPLMN-Identity from the npn-IdentityInfoList;

2>	else:

3>	set the selectedPLMN-Identity to the PLMN selected by upper layers from the plmn-IdentityInfoList;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrentTwoCarrier:

3>	include in the uplinkTxDirectCurrentTwoCarrierList the list of uplink Tx DC locations for the configured uplink carrier aggregation in the MCG;

2>	if the UE has idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport:

3>	if the idleModeMeasurementReq is included in the RRCResume message:

4>	set the measResultIdleEUTRA in the RRCResumeComplete message to the value of measReportIdleEUTRA in the VarMeasIdleReport, if available;

4>	set the measResultIdleNR in the RRCResumeComplete message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

4>	discard the VarMeasIdleReport upon successful delivery of the RRCResumeComplete message is confirmed by lower layers;

3>	else:

4>	if the SIB1 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or

4>	if the SIB1 contains idleModeMeasurementsEUTRA and the UE has E-UTRA idle/inactive measurement information available in VarMeasIdleReport:

5>	include the idleMeasAvailable;

2>	if the RRCResume message includes mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

2>	if the RRCResume message includes mrdc-SecondaryCellGroup set to nr-SCG:

3>	include in the nr-SCG-Response the SCG RRCReconfigurationComplete message;

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCResumeComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableBT in the RRCResumeComplete message;

3>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableWLAN in the RRCResumeComplete message;

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable in the RRCResumeComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCResumeComplete message;

2>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

3>	include the mobilityHistoryAvail in the RRCResumeComplete message;

2>	if speedStateReselectionPars is configured in the SIB2:

3>	include the mobilityState in the RRCResumeComplete message and set it to the mobility state (as specified in TS 38.304 [20]) of the UE just prior to entering RRC_CONNECTED state;

2>	if the UE is configured to provide the measurement gap requirement information of NR target bands:

3>	include the NeedForGapsInfoNR and set the contents as follows:

4> include intraFreq-needForGap and set the gap requirement information of intra-frequency measurement for each NR serving cell;

4>	if requestedTargetBandFilterNR is configured, for each supported NR band that is also included in requestedTargetBandFilterNR, include an entry in interFreq-needForGap and set the gap requirement information for that band; otherwise, include an entry in interFreq-needForGap and set the corresponding gap requirement information for each supported NR band;

2>	if the UE is configured to provide the measurement gap and NCSG requirement information of NR target bands:

3>	include the NeedForNCSG-InfoNR and set the contents as follows:

4> include intraFreq-needForNCSG and set the gap and NCSG requirement information of intra-frequency measurement for each NR serving cell;

4>	if requestedTargetBandFilterNCSG-NR is configured, for each supported NR band included in requestedTargetBandFilterNCSG-NR, include an entry in interFreq-needForNCSG and set the NCSG requirement information for that band; otherwise, include an entry for each supported NR band in interFreq-needForNCSG and set the corresponding NCSG requirement information;

2>	if the UE is configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands:

3>	include the NeedForNCSG-InfoEUTRA and set the contents as follows:

4>	if requestedTargetBandFilterNCSG-EUTRA is configured, for each supported E-UTRA band included in requestedTargetBandFilterNCSG-EUTRA, include an entry in needForNCSG-EUTRA and set the NCSG requirement information for that band; otherwise, include an entry for each supported E-UTRA band in needForNCSG-EUTRA and set the corresponding NCSG requirement information;

1>	submit the RRCResumeComplete message to lower layers for transmission;

1>	the procedure ends.

<Skip>

### 5.5.1	Introduction

The network may configure an RRC_CONNECTED UE to perform measurements. The network may configure the UE to report them in accordance with the measurement configuration or perform conditional reconfiguration evaluation in accordance with the conditional reconfiguration. The measurement configuration is provided by means of dedicated signalling i.e. using the RRCReconfiguration or RRCResume.

The network may configure the UE to perform the following types of measurements:

-	NR measurements;

-	Inter-RAT measurements of E-UTRA frequencies.

-	Inter-RAT measurements of UTRA-FDD frequencies.

The network may configure the UE to report the following measurement information based on SS/PBCH block(s):

-	Measurement results per SS/PBCH block;

-	Measurement results per cell based on SS/PBCH block(s);

-	SS/PBCH block(s) indexes.

The network may configure the UE to report the following measurement information based on CSI-RS resources:

-	Measurement results per CSI-RS resource;

-	Measurement results per cell based on CSI-RS resource(s);

-	CSI-RS resource measurement identifiers.

The network may configure the UE to perform the following types of measurements for NR sidelink and V2X sidelink:

-	CBR measurements.

The network may configure the UE to report the following CLI measurement information based on SRS resources:

-	Measurement results per SRS resource;

-	SRS resource(s) indexes.

The network may configure the UE to report the following CLI measurement information based on CLI-RSSI resources:

-	Measurement results per CLI-RSSI resource;

-	CLI-RSSI resource(s) indexes.

The measurement configuration includes the following parameters:

1.	Measurement objects: A list of objects on which the UE shall perform the measurements.

-	For intra-frequency and inter-frequency measurements a measurement object indicates the frequency/time location and subcarrier spacing of reference signals to be measured. Associated with this measurement object, the network may configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting. Whitelisted cells are the only ones applicable in event evaluation or measurement reporting.

-	The measObjectId of the MO which corresponds to each serving cell is indicated by servingCellMO within the serving cell configuration.

-	For inter-RAT E-UTRA measurements a measurement object is a single E-UTRA carrier frequency. Associated with this E-UTRA carrier frequency, the network can configure a list of cell specific offsets and a list of 'blacklisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting.

-	For inter-RAT UTRA-FDD measurements a measurement object is a set of cells on a single UTRA-FDD carrier frequency.

-	For CBR measurement of NR sidelink communication, a measurement object is a set of transmission resource pool(s) on a single carrier frequency for NR sidelink communication.

-	For CLI measurements a measurement object indicates the frequency/time location of SRS resources and/or CLI-RSSI resources, and subcarrier spacing of SRS resources to be measured.

2.	Reporting configurations: A list of reporting configurations where there can be one or multiple reporting configurations per measurement object. Each measurement reporting configuration consists of the following:

-	Reporting criterion: The criterion that triggers the UE to send a measurement report. This can either be periodical or a single event description.

-	RS type: The RS that the UE uses for beam and cell measurement results (SS/PBCH block or CSI-RS).

-	Reporting format: The quantities per cell and per beam that the UE includes in the measurement report (e.g. RSRP) and other associated information such as the maximum number of cells and the maximum number beams per cell to report.

In case of conditional reconfiguration, each configuration consists of the following:

-	Execution criteria: The criteria the UE uses for conditional reconfiguration execution.

-	RS type: The RS that the UE uses for obtaining beam and cell measurement results (SS/PBCH block-based or CSI-RS-based), used for evaluating conditional reconfiguration execution condition.

3.	Measurement identities: For measurement reporting, a list of measurement identities where each measurement identity links one measurement object with one reporting configuration. By configuring multiple measurement identities, it is possible to link more than one measurement object to the same reporting configuration, as well as to link more than one reporting configuration to the same measurement object. The measurement identity is also included in the measurement report that triggered the reporting, serving as a reference to the network. For conditional reconfiguration triggering, one measurement identity links to exactly one conditional reconfiguration trigger configuration. And up to 2 measurement identities can be linked to one conditional reconfiguration execution condition.

4.	Quantity configurations: The quantity configuration defines the measurement filtering configuration used for all event evaluation and related reporting, and for periodical reporting of that measurement. For NR measurements, the network may configure up to 2 quantity configurations with a reference in the NR measurement object to the configuration that is to be used. In each configuration, different filter coefficients can be configured for different measurement quantities, for different RS types, and for measurements per cell and per beam.

5.	Measurement gaps: Periods that the UE may use to perform measurements if the measurement gap is activated.

A UE in RRC_CONNECTED maintains a measurement object list, a reporting configuration list, and a measurement identities list according to signalling and procedures in this specification. The measurement object list possibly includes NR measurement object(s), CLI measurement object(s) and inter-RAT objects. Similarly, the reporting configuration list includes NR and inter-RAT reporting configurations. Any measurement object can be linked to any reporting configuration of the same RAT type. Some reporting configurations may not be linked to a measurement object. Likewise, some measurement objects may not be linked to a reporting configuration.

The measurement procedures distinguish the following types of cells:

1.	The NR serving cell(s) – these are the SpCell and one or more SCells.

2.	Listed cells – these are cells listed within the measurement object(s).

3.	Detected cells – these are cells that are not listed within the measurement object(s) but are detected by the UE on the SSB frequency(ies) and subcarrier spacing(s) indicated by the measurement object(s).

For NR measurement object(s), the UE measures and reports on the serving cell(s), listed cells and/or detected cells. For inter-RAT measurements object(s) of E-UTRA, the UE measures and reports on listed cells and detected cells and, for RSSI and channel occupancy measurements, the UE measures and reports on the configured resources on the indicated frequency. For inter-RAT measurements object(s) of UTRA-FDD, the UE measures and reports on listed cells. For CLI measurement object(s), the UE measures and reports on configured measurement resources (i.e. SRS resources and/or CLI-RSSI resources).

Whenever the procedural specification, other than contained in sub-clause 5.5.2, refers to a field it concerns a field included in the VarMeasConfig unless explicitly stated otherwise i.e. only the measurement configuration procedure covers the direct UE action related to the received measConfig.

In NR-DC, the UE may receive two independent measConfig:

-	a measConfig, associated with MCG, that is included in the RRCReconfiguration message received via SRB1; and

-	a measConfig, associated with SCG, that is included in the RRCReconfiguration message received via SRB3, or, alternatively, included within a RRCReconfiguration message embedded in a RRCReconfiguration message received via SRB1.

In this case, the UE maintains two independent VarMeasConfig and VarMeasReportList, one associated with each measConfig, and independently performs all the procedures in clause 5.5 for each measConfig and the associated VarMeasConfig and VarMeasReportList, unless explicitly stated otherwise.

The configurations related to CBR measurements are only included in the measConfig associated with MCG.

<Skip>

#### 5.5.2.9	Measurement gap configuration

The UE shall:

1>	if gapFR1 is set to setup:

2>	if an FR1 measurement gap configuration configured by gapFR1 is already setup, release the FR1 measurement gap configuration;

2>	setup the FR1 measurement gap configuration indicated by the gapFR1 in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

2>	associate the FR1 measurement gap with the measGapId indicated by the GapConfig (if present);

1>	else if gapFR1 is set to release:

2>	release the FR1 measurement gap configuration configured by gapFR1;

1>	if gapFR2 is set to setup:

2>	if an FR2 measurement gap configuration configured by gapFR2 is already setup, release the FR2 measurement gap configuration;

2>	setup the FR2 measurement gap configuration indicated by the gapFR2 in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

2>	associate the FR2 measurement gap with the measGapId indicated by the GapConfig (if present);

1>	else if gapFR2 is set to release:

2>	release the FR2 measurement gap configuration configured by gapFR2;

1>	if gapUE is set to setup:

2>	if a per UE measurement gap configuration configured by gapUE is already setup, release the per UE measurement gap configuration;

2>	setup the per UE measurement gap configuration indicated by the gapUE in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

2>	associate the per UE measurement gap with the measGapId indicated by the GapConfig (if present);

1>	else if gapUE is set to release:

2>	release the per UE measurement gap configuration configured by gapUE.

1>	for each measGapId included in the received gapFR1ToReleaseList:

2> release the FR1 measurement gap configuration associated with the measGapId;

1>	for each measGapId included in the received gapFR2ToReleaseList:

2> release the FR2 measurement gap configuration associated with the measGapId;

1>	for each measGapId included in the received gapUEToReleaseList:

2> release the per UE measurement gap configuration associated with the measGapId;

1>	for each GapConfig received in gapFR1ToAddModList:

2>	setup an FR1 measurement gap configuration indicated by the GapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

2>	associate the FR1 measurement gap with the measGapId indicated by the GapConfig;

2>	if gapSharing in the GapConfig is present:

3>	setup the gap sharing configuration for the FR1 measurement gap in accordance with the received gapSharing as defined in TS 38.133 [14];

2>	else:

3>	release the gap sharing configuration for the FR1 measurement gap;

1>	for each GapConfig received in gapFR2ToAddModList:

2>	setup an FR2 measurement gap configuration indicated by the GapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

2>	associate the FR2 measurement gap with the measGapId indicated by the GapConfig;

2>	if gapSharing in the GapConfig is present:

3>	setup the gap sharing configuration for the FR2 measurement gap in accordance with the received gapSharing as defined in TS 38.133 [14];

2>	else:

3>	release the gap sharing configuration for the FR2 measurement gap;

1>	for each GapConfig received in gapUEToAddModList:

2>	setup a per UE measurement gap configuration indicated by the GapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

2>	associate the per UE measurement gap with the measGapId indicated by the GapConfig;

2>	if gapSharing in the GapConfig is present:

3>	setup the gap sharing configuration for the per UE measurement gap in accordance with the received gapSharing as defined in TS 38.133 [14];

2>	else:

3>	release the gap sharing configuration for the per UE measurement gap;

1>	for each FR1, FR2, and per UE measurement gap that is setup:

2>	if preConfigInd-r17 in the corresponding GapConfig is present:

3>	determine whether the measurement gap is activated or not according to 38.133 FFS which section;

2>	else:

3>	consider the measurement gap to be activated;

NOTE 1:	For FR2 gap configuration with synchronous CA, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator is used in the gap calculation. Otherwise, the SFN and subframe of a serving cell on FR2 frequency is used in the gap calculation

NOTE 2:	For FR1 gap or per UE gap configuration, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator is used in the gap calculation. Otherwise, the SFN and subframe of the PCell is used in the gap calculation.

NOTE 3:	For FR2 gap configuration with asynchronous CA, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator and refFR2ServCellAsyncCA is used in the gap calculation. Otherwise, the SFN and subframe of a serving cell on FR2 frequency indicated by the refFR2ServCellAsyncCA is used in the gap calculation

<Skip>

#### 5.5.2.11	Measurement gap sharing configuration

The UE shall:

1>	if gapSharingFR1 is set to setup:

2>	if an FR1 measurement gap sharing configuration configured by gapSharingFR1 is already setup:

3>	release the FR1 measurement gap sharing configuration configured by gapSharingFR1;

2>	setup the FR1 measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received gapSharingFR1 as defined in TS 38.133 [14];

1>	else if gapSharingFR1 is set to release:

2>	release the FR1 measurement gap sharing configuration configured by gapSharingFR1;

1>	if gapSharingFR2 is set to setup:

2>	if an FR2 measurement gap sharing configuration configured by gapSharingFR2 is already setup:

3>	release the FR2 measurement gap sharing configuration configured by gapSharingFR2;

2>	setup the FR2 measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received gapSharingFR2 as defined in TS 38.133 [14];

1>	else if gapSharingFR2 is set to release:

2>	release the FR2 measurement gap sharing configuration configured by gapSharingFR2.

1>	if gapSharingUE is set to setup:

2>	if a per UE measurement gap sharing configuration configured by gapSharingUE is already setup:

3>	release the per UE measurement gap sharing configuration configured by gapSharingUE;

2>	setup the per UE measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received gapSharingUE as defined in TS 38.133 [14];

1>	else if gapSharingUE is set to release:

2>	release the per UE measurement gap sharing configuration configured by gapSharingUE.

# 6	Protocol data units, formats and parameters (ASN.1)

<Skip>

### 6.2.2	Message definitions

<Skip>

#### –	RRCReconfiguration

The RRCReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, radio resource configuration (including RBs, MAC main configuration and physical channel configuration) and AS security configuration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::=                  SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcReconfiguration                      RRCReconfiguration-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCReconfiguration-IEs ::=              SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

secondaryCellGroup                      OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Cond SCG

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-IEs                                           OPTIONAL

}

RRCReconfiguration-v1530-IEs ::=            SEQUENCE {

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

fullConfig                              ENUMERATED {true}                                                      OPTIONAL, -- Cond FullConfig

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message                     OPTIONAL, -- Cond nonHO

masterKeyUpdate                         MasterKeyUpdate                                                        OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery                  OCTET STRING (CONTAINING SIB1)                                         OPTIONAL, -- Need N

dedicatedSystemInformationDelivery      OCTET STRING (CONTAINING SystemInformation)                            OPTIONAL, -- Need N

otherConfig                             OtherConfig                                                            OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1540-IEs                                           OPTIONAL

}

RRCReconfiguration-v1540-IEs ::=        SEQUENCE {

otherConfig-v1540                       OtherConfig-v1540                                                      OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1560-IEs                                           OPTIONAL

}

RRCReconfiguration-v1560-IEs ::=         SEQUENCE {

mrdc-SecondaryCellGroupConfig            SetupRelease { MRDC-SecondaryCellGroupConfig }                        OPTIONAL,   -- Need M

radioBearerConfig2                       OCTET STRING (CONTAINING RadioBearerConfig)                           OPTIONAL,   -- Need M

sk-Counter                               SK-Counter                                                            OPTIONAL,   -- Need N

nonCriticalExtension                     RRCReconfiguration-v1610-IEs                                          OPTIONAL

}

RRCReconfiguration-v1610-IEs ::=        SEQUENCE {

otherConfig-v1610                       OtherConfig-v1610                                                    OPTIONAL, -- Need M

bap-Config-r16                          SetupRelease { BAP-Config-r16 }                                      OPTIONAL, -- Need M

iab-IP-AddressConfigurationList-r16     IAB-IP-AddressConfigurationList-r16                                  OPTIONAL, -- Need M

conditionalReconfiguration-r16          ConditionalReconfiguration-r16                                       OPTIONAL, -- Need M

daps-SourceRelease-r16                  ENUMERATED{true}                                                     OPTIONAL, -- Need N

t316-r16                                SetupRelease {T316-r16}                                              OPTIONAL, -- Need M

needForGapsConfigNR-r16                 SetupRelease {NeedForGapsConfigNR-r16}                               OPTIONAL, -- Need M

onDemandSIB-Request-r16                 SetupRelease { OnDemandSIB-Request-r16 }                             OPTIONAL, -- Need M

dedicatedPosSysInfoDelivery-r16         OCTET STRING (CONTAINING PosSystemInformation-r16-IEs)               OPTIONAL, -- Need N

sl-ConfigDedicatedNR-r16                SetupRelease {SL-ConfigDedicatedNR-r16}                              OPTIONAL, -- Need M

sl-ConfigDedicatedEUTRA-Info-r16        SetupRelease {SL-ConfigDedicatedEUTRA-Info-r16}                      OPTIONAL, -- Need M

targetCellSMTC-SCG-r16                  SSB-MTC                                                              OPTIONAL, -- Need S

nonCriticalExtension                    RRCReconfiguration-v17xx-IEs                                         OPTIONAL

}

RRCReconfiguration-v17xx-IEs ::=        SEQUENCE {

needForNCSG-ConfigNR-r17                SetupRelease {NeedForNCSG-ConfigNR-r17}                              OPTIONAL, -- Need M

needForNCSG-ConfigEUTRA-r17             SetupRelease {NeedForNCSG-ConfigEUTRA-r17}                           OPTIONAL, -- Need M

nonCriticalExtension                    SEQUENCE {}                                                          OPTIONAL

}

MRDC-SecondaryCellGroupConfig ::=       SEQUENCE {

mrdc-ReleaseAndAdd                      ENUMERATED {true}                                                     OPTIONAL,   -- Need N

mrdc-SecondaryCellGroup                 CHOICE {

nr-SCG                                  OCTET STRING  (CONTAINING RRCReconfiguration),

eutra-SCG                               OCTET STRING

}

}

BAP-Config-r16 ::=                      SEQUENCE {

bap-Address-r16                         BIT STRING (SIZE (10))                                    OPTIONAL, -- Need M

defaultUL-BAP-RoutingID-r16             BAP-RoutingID-r16                                         OPTIONAL, -- Need M

defaultUL-BH-RLC-Channel-r16            BH-RLC-ChannelID-r16                                      OPTIONAL, -- Need M

flowControlFeedbackType-r16             ENUMERATED {perBH-RLC-Channel, perRoutingID, both}        OPTIONAL, -- Need R

...

}

MasterKeyUpdate ::=                 SEQUENCE {

keySetChangeIndicator           BOOLEAN,

nextHopChainingCount            NextHopChainingCount,

nas-Container                   OCTET STRING                                                     OPTIONAL,    -- Cond securityNASC

...

}

OnDemandSIB-Request-r16 ::=                  SEQUENCE {

onDemandSIB-RequestProhibitTimer-r16         ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30}

}

T316-r16 ::=         ENUMERATED {ms50, ms100, ms200, ms300, ms400, ms500, ms600, ms1000, ms1500, ms2000}

IAB-IP-AddressConfigurationList-r16 ::= SEQUENCE {

iab-IP-AddressToAddModList-r16      SEQUENCE (SIZE(1..maxIAB-IP-Address-r16)) OF IAB-IP-AddressConfiguration-r16 OPTIONAL, -- Need N

iab-IP-AddressToReleaseList-r16     SEQUENCE (SIZE(1..maxIAB-IP-Address-r16)) OF IAB-IP-AddressIndex-r16         OPTIONAL, -- Need N

...

}

IAB-IP-AddressConfiguration-r16 ::=     SEQUENCE {

iab-IP-AddressIndex-r16                 IAB-IP-AddressIndex-r16,

iab-IP-Address-r16                      IAB-IP-Address-r16                                                OPTIONAL,  -- Need M

iab-IP-Usage-r16                        IAB-IP-Usage-r16                                                  OPTIONAL,  -- Need M

iab-donor-DU-BAP-Address-r16            BIT STRING (SIZE(10))                                             OPTIONAL,  -- Need M

...

}

SL-ConfigDedicatedEUTRA-Info-r16 ::=            SEQUENCE {

sl-ConfigDedicatedEUTRA-r16                    OCTET STRING                                              OPTIONAL,  -- Need M

sl-TimeOffsetEUTRA-List-r16                    SEQUENCE (SIZE (8)) OF SL-TimeOffsetEUTRA-r16             OPTIONAL    -- Need M

}

SL-TimeOffsetEUTRA-r16 ::=        ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot625, ms0dot75, ms1, ms1dot25, ms1dot5, ms1dot75,

ms2, ms2dot5, ms3, ms4, ms5, ms6, ms8, ms10, ms20}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

RRCReconfiguration-IEs field descriptions

bap-Config This field is used to configure the BAP entity for IAB nodes.

bap-Address Indicates the BAP address of an IAB-node. The BAP address of an IAB-node cannot be changed once configured to the BAP entity.

conditionalReconfiguration Configuration of candidate target SpCell(s) and execution condition(s) for conditional handover or conditional PSCell change. For conditional PSCell change, this field may only be present in an RRCReconfiguration message for intra-SN PSCell change. The network does not configure a UE with both conditional PCell change and conditional PSCell change simultaneously. The field is absent if any DAPS bearer is configured or if the masterCellGroup includes ReconfigurationWithSync. For conditional PSCell change, the field is absent if the secondaryCellGroup includes ReconfigurationWithSync. The RRCReconfiguration message contained in DLInformationTransferMRDC cannot contain the field conditionalReconfiguration for conditional PSCell change.

daps-SourceRelease Indicates to UE that the source cell part of DAPS operation is to be stopped and the source cell part of DAPS configuration is to be released.

dedicatedNAS-MessageList This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for each PDU in the list.

dedicatedPosSysInfoDelivery This field is used to transfer SIBPos to the UE in RRC_CONNECTED.

dedicatedSIB1-Delivery This field is used to transfer SIB1 to the UE. The field has the same values as the corresponding configuration in servingCellConfigCommon.

dedicatedSystemInformationDelivery This field is used to transfer SIB6, SIB7, SIB8 to the UE with an active BWP with no common serach space configured. For UEs in RRC_CONNECTED, this field is used to transfer the SIBs requested on-demand.

defaultUL-BAP-RoutingID This field is used for IAB-node to configure the default uplink Routing ID, which is used by IAB-node during IAB-node bootstrapping, migration, IAB-MT RRC resume and IAB-MT RRC re-establishment for F1-C and non-F1 traffic. The defaultUL-BAP-RoutingID can be (re-)configured when IAB-node IP address for F1-C related traffic changes. This field is mandatory only for IAB-node bootstrapping.

defaultUL-BH-RLC-Channel This field is used for IAB-nodes to configure the default uplink BH RLC channel, which is used by IAB-node during IAB-node bootstrapping, migration, IAB-MT RRC resume and IAB-MT RRC re-establishment for F1-C and non-F1 traffic. The defaultUL-BH-RLC-Channel can be (re-)configured when IAB-node IP address for F1-C related traffic changes, and the new IP address is anchored at a different IAB-donor-DU. This field is mandatory for IAB-node bootstrapping. If the IAB-MT is operating in EN-DC, the default uplink BH RLC channel is referring to an RLC channel on the SCG; Otherwise, it is referring to an RLC channel on the MCG.

flowControlFeedbackType This field is only used for IAB-node that support hop-by-hop flow control to configure the type of flow control feedback. Value perBH-RLC-Channel indicates that the IAB-node shall provide flow control feedback per BH RLC channel, value perRoutingID indicates that the IAB-node shall provide flow control feedback per routing ID, and value both indicates that the IAB-node shall provide flow control feedback both per BH RLC channel and per routing ID.

fullConfig Indicates that the full configuration option is applicable for the RRCReconfiguration message for intra-system intra-RAT HO. For inter-RAT HO from E-UTRA to NR, fullConfig indicates whether or not delta signalling of SDAP/PDCP from source RAT is applicable. This field is absent if any DAPS bearer is configured or when the RRCReconfiguration message is transmitted on SRB3, and in an RRCReconfiguration message for SCG contained in another RRCReconfiguration message (or RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1.

iab-IP-Address This field is used to provide the IP address information for IAB-node.

iab-IP-AddressIndex This field is used to identify a configuration of an IP address.

iab-IP-AddressToAddModList List of IP addresses allocated for IAB-node to be added and modified.

iab-IP-AddressToReleaseList List of IP address allocated for IAB-node to be released.

iab-IP-Usage This field is used to indicate the usage of the assigned IP address. If this field is not configured, the assigned IP address is used for all traffic.

iab-donor-DU-BAP-Address This field is used to indicate the BAP address of the IAB-donor-DU where the IP address is anchored.

keySetChangeIndicator Indicates whether UE shall derive a new KgNB. If reconfigurationWithSync is included, value true indicates that a KgNB key is derived from a KAMF key taken into use through the latest successful NAS SMC procedure, or N2 handover procedure with KAMF change, as described in TS 33.501 [11] for KgNB re-keying. Value false indicates that the new KgNB key is obtained from the current KgNB key or from the NH as described in TS 33.501 [11].

masterCellGroup Configuration of master cell group.

mrdc-ReleaseAndAdd This field indicates that the current SCG configuration is released and a new SCG is added at the same time.

mrdc-SecondaryCellGroup Includes an RRC message for SCG configuration in NR-DC or NE-DC. For NR-DC (nr-SCG), mrdc-SecondaryCellGroup contains the RRCReconfiguration message as generated (entirely) by SN gNB. In this version of the specification, the RRC message can only include fields secondaryCellGroup, otherConfig, conditionalReconfiguration and measConfig. For NE-DC (eutra-SCG), mrdc-SecondaryCellGroup includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration.

nas-Container This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although it affects activation of AS security after inter-system handover to NR. The content is defined in TS 24.501 [23].

needForGapsConfigNR Configuration for the UE to report measurement gap requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForNCSG-ConfigNR Configuration for the UE to report measurement gap and NCSG requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForNCSG-ConfigEUTRA Configuration for the UE to report measurement gap and NCSG requirement information of E-UTRA target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

nextHopChainingCount Parameter NCC: See TS 33.501 [11]

onDemandSIB-Request If the field is present, the UE is allowed to request SIB(s) on-demand while in RRC_CONNECTED according to clause 5.2.2.3.5.

onDemandSIB-RequestProhibitTimer Prohibit timer for requesting SIB(s) on-demand while in RRC_CONNECTED according to clause 5.2.2.3.5. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

otherConfig Contains configuration related to other configurations. When configured for the SCG, only fields drx-PreferenceConfig, maxBW-PreferenceConfig, maxCC-PreferenceConfig, maxMIMO-LayerPreferenceConfig, minSchedulingOffsetPreferenceConfig, btNameList, wlanNameList, sensorNameList and obtainCommonLocation can be included.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. In EN-DC this field may only be present if the RRCReconfiguration is transmitted over SRB3.

radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. This field can only be used if the UE supports NR-DC or NE-DC.

secondaryCellGroup Configuration of secondary cell group ((NG)EN-DC or NR-DC).

sk-Counter A counter used upon initial configuration of S-KgNB or S-KeNB, as well as upon refresh of S-KgNB or S-KeNB. This field is always included either upon initial configuration of an NR SCG or upon configuration of the first RB with keyToUse set to secondary, whichever happens first. This field is absent if there is neither any NR SCG nor any RB with keyToUse set to secondary.

sl-ConfigDedicatedNR This field is used to provide the dedicated configurations for NR sidelink communication.

sl-ConfigDedicatedEUTRA-Info This field includes the E-UTRA RRCConnectionReconfiguration as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRCConnectionReconfiguration can only includes sidelink related fields for V2X sidelink communication, i.e. sl-V2X-ConfigDedicated, sl-V2X-SPS-Config, measConfig and/or otherConfig.

sl-TimeOffsetEUTRA This field indicates the possible time offset to (de)activation of V2X sidelink transmission after receiving DCI format 3_1 used for scheduling V2X sidelink communication. Value ms0dpt75 corresponds to 0.75ms, ms1 corresponds to 1ms and so on. The network includes this field only when sl-ConfigDedicatedEUTRA is configured.

targetCellSMTC-SCG The SSB periodicity/offset/duration configuration of target cell for NR PSCell addition and SN change. When UE receives this field, UE applies the configuration based on the timing reference of NR PCell for PSCell addition and PSCell change for the case of no reconfiguration with sync of MCG, and UE applies the configuration based on the timing reference of target NR PCell for the case of reconfiguration with sync of MCG. If both this field and the smtc in secondaryCellGroup -> SpCellConfig -> reconfigurationWithSync are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

t316 Indicates the value for timer T316 as described in clause 7.1. Value ms50 corresponds to 50 ms, value ms100 corresponds to 100 ms and so on. This field can be configured only if the UE is configured with split SRB1 or SRB3.

| Conditional Presence | Explanation |
| --- | --- |
| nonHO | The field is absent in case of reconfiguration with sync within NR or to NR; otherwise it is optionally present, need N. |
| securityNASC | This field is mandatory present in case of inter system handover. Otherwise the field is optionally present, need N. |
| MasterKeyChange | This field is mandatory present in case masterCellGroup includes ReconfigurationWithSync and RadioBearerConfig includes SecurityConfig with SecurityAlgorithmConfig, indicating a change of the AS security algorithms associated to the master key. If ReconfigurationWithSync is included for other cases, this field is optionally present, need N. Otherwise the field is absent. |
| FullConfig | The field is mandatory present in case of inter-system handover from E-UTRA/EPC to NR. It is optionally present, Need N, during reconfiguration with sync and also in first reconfiguration after reestablishment; or for intra-system handover from E-UTRA/5GC to NR. It is absent otherwise. |
| SCG | The field is mandatory present in: - an RRCReconfiguration message contained in an RRCResume message (or in an RRCConnectionResume message, see TS 36.331 [10]), - an RRCReconfiguration message contained in an RRCConnectionReconfiguration message, see TS 36.331 [10], which is contained in DLInformationTransferMRDC transmitted on SRB3 (as a response to ULInformationTransferMRDC including an MCGFailureInformation). The field is optional present, Need M, in: - an RRCReconfiguration message transmitted on SRB3, - an RRCReconfiguration message contained in another RRCReconfiguration message (or in an RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1 - an RRCReconfiguration message contained in another RRCReconfiguration message which is contained in DLInformationTransferMRDC transmitted on SRB3 (as a response to ULInformationTransferMRDC including an MCGFailureInformation) Otherwise, the field is absent |

#### –	RRCReconfigurationComplete

The RRCReconfigurationComplete message is used to confirm the successful completion of an RRC connection reconfiguration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCReconfigurationComplete message

-- ASN1START

-- TAG-RRCRECONFIGURATIONCOMPLETE-START

RRCReconfigurationComplete ::=              SEQUENCE {

rrc-TransactionIdentifier                   RRC-TransactionIdentifier,

criticalExtensions                          CHOICE {

rrcReconfigurationComplete                  RRCReconfigurationComplete-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

RRCReconfigurationComplete-IEs ::=          SEQUENCE {

lateNonCriticalExtension                    OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1530-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1530-IEs ::=    SEQUENCE {

uplinkTxDirectCurrentList                   UplinkTxDirectCurrentList                                               OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1560-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1560-IEs ::=    SEQUENCE {

scg-Response                                CHOICE {

nr-SCG-Response                             OCTET STRING (CONTAINING RRCReconfigurationComplete),

eutra-SCG-Response                          OCTET STRING

}                                                                                                                       OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1610-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1610-IEs ::=    SEQUENCE {

ue-MeasurementsAvailable-r16                UE-MeasurementsAvailable-r16                                            OPTIONAL,

needForGapsInfoNR-r16                       NeedForGapsInfoNR-r16                                                   OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1640-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1640-IEs ::=    SEQUENCE {

uplinkTxDirectCurrentTwoCarrierList-r16     UplinkTxDirectCurrentTwoCarrierList-r16                                 OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v17xx-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v17xx-IEs ::=    SEQUENCE {

needForNCSG-InfoNR-r17                      NeedForNCSG-InfoNR-r17                                                  OPTIONAL,

needForNCSG-InfoEUTRA-r17                   NeedForNCSG-InfoEUTRA-r17                                               OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                                                             OPTIONAL

}

-- TAG-RRCRECONFIGURATIONCOMPLETE-STOP

-- ASN1STOP

RRCReconfigurationComplete-IEs field descriptions

needForGapsInfoNR This field is used to indicate the measurement gap requirement information of the UE for NR target bands.

needForNCSG-InfoNR This field is used to indicate the measurement gap and NCSG requirement information of the UE for NR target bands.

needForNCSG-InfoEUTRA This field is used to indicate the measurement gap and NCSG requirement information of the UE for E-UTRA target bands.

scg-Response In case of NR-DC (nr-SCG-Response), this field includes the RRCReconfigurationComplete message. In case of NE-DC (eutra-SCG-Response), this field includes the E-UTRA RRCConnectionReconfigurationComplete message as specified in TS 36.331 [10].

uplinkTxDirectCurrentList The Tx Direct Current locations for the configured serving cells and BWPs if requested by the NW (see reportUplinkTxDirectCurrent in CellGroupConfig).

uplinkTxDirectCurrentTwoCarrierList The Tx Direct Current locations for the configured uplink intra-band CA with two carriers if requested by the NW (see reportUplinkTxDirectCurrentTwoCarrier-r16 in CellGroupConfig).

<Skip>

#### –	RRCResume

The RRCResume message is used to resume the suspended RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCResume message

-- ASN1START

-- TAG-RRCRESUME-START

RRCResume ::=                       SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcResume                           RRCResume-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCResume-IEs ::=                   SEQUENCE {

radioBearerConfig                   RadioBearerConfig                                               OPTIONAL, -- Need M

masterCellGroup                     OCTET STRING (CONTAINING CellGroupConfig)                       OPTIONAL, -- Need M

measConfig                          MeasConfig                                                      OPTIONAL, -- Need M

fullConfig                          ENUMERATED {true}                                               OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                    OPTIONAL,

nonCriticalExtension                RRCResume-v1560-IEs                                             OPTIONAL

}

RRCResume-v1560-IEs ::=             SEQUENCE {

radioBearerConfig2                  OCTET STRING (CONTAINING RadioBearerConfig)                     OPTIONAL, -- Need M

sk-Counter                          SK-Counter                                                      OPTIONAL, -- Need N

nonCriticalExtension                RRCResume-v1610-IEs                                             OPTIONAL

}

RRCResume-v1610-IEs ::=             SEQUENCE {

idleModeMeasurementReq-r16          ENUMERATED {true}                                               OPTIONAL, -- Need N

restoreMCG-SCells-r16               ENUMERATED {true}                                               OPTIONAL, -- Need N

restoreSCG-r16                      ENUMERATED {true}                                               OPTIONAL, -- Need N

mrdc-SecondaryCellGroup-r16         CHOICE {

nr-SCG-r16                          OCTET STRING (CONTAINING RRCReconfiguration),

eutra-SCG-r16                       OCTET STRING

}                                                                                                   OPTIONAL, -- Cond RestoreSCG

needForGapsConfigNR-r16             SetupRelease {NeedForGapsConfigNR-r16}                          OPTIONAL, -- Need M

nonCriticalExtension                RRCResume-v17xx-IEs                                   OPTIONAL

}

RRCResume-v17xx-IEs ::=        SEQUENCE {

needForNCSG-ConfigNR-r17             SetupRelease {NeedForNCSG-ConfigNR-r17}                        OPTIONAL, -- Need M

needForNCSG-ConfigEUTRA-r17          SetupRelease {NeedForNCSG-ConfigEUTRA-r17}                     OPTIONAL, -- Need M

nonCriticalExtension                 SEQUENCE {}                                                    OPTIONAL

}

-- TAG-RRCRESUME-STOP

-- ASN1STOP

RRCResume-IEs field descriptions

idleModeMeasurementReq This field indicates that the UE shall report the idle/inactive measurements, if available, to the network in the RRCResumeComplete message

masterCellGroup Configuration of the master cell group.

mrdc-SecondaryCellGroup Includes an RRC message for SCG configuration in NR-DC or NE-DC. For NR-DC (nr-SCG), mrdc-SecondaryCellGroup contains the RRCReconfiguration message as generated (entirely) by SN gNB. In this version of the specification, the RRC message can only include fields secondaryCellGroup (with at least reconfigurationWithSync), otherConfig and measConfig. For NE-DC (eutra-SCG), mrdc-SecondaryCellGroup includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message only include the field scg-Configuration with at least mobilityControlInfoSCG.

needForGapsConfigNR Configuration for the UE to report measurement gap requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForNCSG-ConfigNR Configuration for the UE to report measurement gap and NCSG requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForNCSG-ConfigEUTRA Configuration for the UE to report measurement gap and NCSG requirement information of E-UTRA target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP.

radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. This field can only be used if the UE supports NR-DC or NE-DC.

restoreMCG-SCells Indicates that the UE shall restore the MCG SCells from the UE Inactive AS Context, if stored.

restoreSCG Indicates that the UE shall restore the SCG configurations from the UE Inactive AS Context, if stored.

sk-Counter A counter used to derive S-KgNB or S-KeNB based on the newly derived KgNB during RRC Resume. The field is only included when there is one or more RB with keyToUse set to secondary or mrdc-SecondaryCellGroup is included.

| Conditional Presence | Explanation |
| --- | --- |
| RestoreSCG | The field is mandatory present if restoreSCG is included. It is optionally present, Need M, otherwise. |

#### –	RRCResumeComplete

The RRCResumeComplete message is used to confirm the successful completion of an RRC connection resumption.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCResumeComplete message

-- ASN1START

-- TAG-RRCRESUMECOMPLETE-START

RRCResumeComplete ::=                   SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcResumeComplete                       RRCResumeComplete-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCResumeComplete-IEs ::=               SEQUENCE {

dedicatedNAS-Message                    DedicatedNAS-Message                                                    OPTIONAL,

selectedPLMN-Identity                   INTEGER (1..maxPLMN)                                                    OPTIONAL,

uplinkTxDirectCurrentList               UplinkTxDirectCurrentList                                               OPTIONAL,

lateNonCriticalExtension                OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1610-IEs                                             OPTIONAL

}

RRCResumeComplete-v1610-IEs ::=         SEQUENCE {

idleMeasAvailable-r16                   ENUMERATED {true}                                                       OPTIONAL,

measResultIdleEUTRA-r16                 MeasResultIdleEUTRA-r16                                                 OPTIONAL,

measResultIdleNR-r16                    MeasResultIdleNR-r16                                                    OPTIONAL,

scg-Response-r16                        CHOICE {

nr-SCG-Response                         OCTET STRING (CONTAINING RRCReconfigurationComplete),

eutra-SCG-Response                      OCTET STRING

}                                                                                                               OPTIONAL,

ue-MeasurementsAvailable-r16            UE-MeasurementsAvailable-r16                                            OPTIONAL,

mobilityHistoryAvail-r16                ENUMERATED {true}                                                       OPTIONAL,

mobilityState-r16                       ENUMERATED {normal, medium, high, spare}                                OPTIONAL,

needForGapsInfoNR-r16                   NeedForGapsInfoNR-r16                                                   OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1640-IEs                                             OPTIONAL

}

RRCResumeComplete-v1640-IEs ::=         SEQUENCE {

uplinkTxDirectCurrentTwoCarrierList-r16 UplinkTxDirectCurrentTwoCarrierList-r16                                 OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v17xx-IEs                                             OPTIONAL

}

RRCResumeComplete-v17xx-IEs ::=    SEQUENCE {

needForNCSG-InfoNR-r17                      NeedForNCSG-InfoNR-r17                                                  OPTIONAL,

needForNCSG-InfoEUTRA-r17                   NeedForNCSG-InfoEUTRA-r17                                               OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                                                             OPTIONAL

}

-- TAG-RRCRESUMECOMPLETE-STOP

-- ASN1STOP

RRCResumeComplete-IEs field descriptions

idleMeasAvailable Indication that the UE has idle/inactive measurement report available.

measResultIdleEUTRA EUTRA measurement results performed during RRC_INACTIVE.

measResultIdleNR NR measurement results performed during RRC_INACTIVE.

needForGapsInfoNR This field is used to indicate the measurement gap requirement information of the UE for NR target bands.

needForNCSG-InfoNR This field is used to indicate the measurement gap and NCSG requirement information of the UE for NR target bands

needForNCSG-InfoEUTRA This field is used to indicate the measurement gap and NCSG requirement information of the UE for E-UTRA target bands

selectedPLMN-Identity Index of the PLMN selected by the UE from the plmn-IdentityInfoList or npn-IdentityInfoList fields included in SIB1.

uplinkTxDirectCurrentList The Tx Direct Current locations for the configured serving cells and BWPs if requested by the NW (see reportUplinkTxDirectCurrent in CellGroupConfig).

uplinkTxDirectCurrentTwoCarrierList The Tx Direct Current locations for the configured uplink intra-band CA with two carriers if requested by the NW (see reportUplinkTxDirectCurrentTwoCarrier-r16 in CellGroupConfig).

<Skip>

### 6.3.2	Radio resource control information elements

<Skip>

#### –	BWP-DownlinkDedicated

The IE BWP-DownlinkDedicated is used to configure the dedicated (UE specific) parameters of a downlink BWP.

BWP-DownlinkDedicated information element

-- ASN1START

-- TAG-BWP-DOWNLINKDEDICATED-START

BWP-DownlinkDedicated ::=           SEQUENCE {

pdcch-Config                        SetupRelease { PDCCH-Config }                                     OPTIONAL,   -- Need M

pdsch-Config                        SetupRelease { PDSCH-Config }                                     OPTIONAL,   -- Need M

sps-Config                          SetupRelease { SPS-Config }                                       OPTIONAL,   -- Need M

radioLinkMonitoringConfig           SetupRelease { RadioLinkMonitoringConfig }                        OPTIONAL,   -- Need M

...,

[[

sps-ConfigToAddModList-r16          SPS-ConfigToAddModList-r16                                        OPTIONAL,   -- Need N

sps-ConfigToReleaseList-r16         SPS-ConfigToReleaseList-r16                                       OPTIONAL,   -- Need N

sps-ConfigDeactivationStateList-r16 SPS-ConfigDeactivationStateList-r16                               OPTIONAL,   -- Need R

beamFailureRecoverySCellConfig-r16  SetupRelease {BeamFailureRecoverySCellConfig-r16}                 OPTIONAL,   -- Cond SCellOnly

sl-PDCCH-Config-r16                 SetupRelease { PDCCH-Config }                                     OPTIONAL,   -- Need M

sl-V2X-PDCCH-Config-r16             SetupRelease { PDCCH-Config }                                     OPTIONAL    -- Need M

]],

[[

deactivatedMeasGapList-r17          SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF MeasGapId-r17            OPTIONAL    -- Cond PreConfigMG

]]

}

}

SPS-ConfigToAddModList-r16 ::=          SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-Config

SPS-ConfigToReleaseList-r16 ::=         SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-ConfigIndex-r16

SPS-ConfigDeactivationState-r16 ::=     SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-ConfigIndex-r16

SPS-ConfigDeactivationStateList-r16 ::= SEQUENCE (SIZE (1..maxNrofSPS-DeactivationState)) OF SPS-ConfigDeactivationState-r16

-- TAG-BWP-DOWNLINKDEDICATED-STOP

-- ASN1STOP

BWP-DownlinkDedicated field descriptions

beamFailureRecoverySCellConfig Configuration of candidate RS for beam failure recovery in SCells.

deactivatedMeasGapList Indicates a list of gap IDs where the corresponding pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) are deactivated upon the switch to this BWP.

pdcch-Config UE specific PDCCH configuration for one BWP.

pdsch-Config UE specific PDSCH configuration for one BWP.

sps-Config UE specific SPS (Semi-Persistent Scheduling) configuration for one BWP. Except for reconfiguration with sync, the NW does not reconfigure sps-Config when there is an active configured downlink assignment (see TS 38.321 [3]). However, the NW may release the sps-Config at any time. Network can only configure SPS in one BWP using either this field or sps-ConfigToAddModList.

sps-ConfigDeactivationStateList Indicates a list of the deactivation states in which each state can be mapped to a single or multiple SPS configurations to be deactivated, see clause 10.2 in TS 38.213 [13]. If a state is mapped to multiple SPS configurations, each of these SPS configurations is configured with the same harq-CodebookID.

sps-ConfigToAddModList Indicates a list of one or more DL SPS configurations to be added or modified in one BWP. Except for reconfiguration with sync, the NW does not reconfigure a SPS configuration when it is active (see TS 38.321 [3]).

sps-ConfigToReleaseList Indicates a list of one or more DL SPS configurations to be released. The NW may release a SPS configuration at any time.

radioLinkMonitoringConfig UE specific configuration of radio link monitoring for detecting cell- and beam radio link failure occasions. The maximum number of failure detection resources should be limited up to 8 for both cell and beam radio link failure detection. For SCells, only periodic 1-port CSI-RS can be configured in IE RadioLinkMonitoringConfig.

sl-PDCCH-Config Indicates the UE specific PDCCH configurations for receiving the SL grants (via SL-RNTI or SL-CS-RNTI) for NR sidelink communication.

sl-V2X-PDCCH-Config Indicates the UE specific PDCCH configurations for receiving SL grants (i.e. sidelink SPS) for V2X sidelink communication.

| Conditional Presence | Explanation |
| --- | --- |
| PreConfigMG | The field is optionally present, Need R, if there is at least one per UE gap configured with preConfigInd or there is at least one per FR gap of the same FR which the BWP belongs to and configured with preConfigInd. It is absent otherwise. |
| ScellOnly | The field is optionally present, Need M, in the BWP-DownlinkDedicated of an Scell. It is absent otherwise. |

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells).

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                        SEQUENCE {

cellGroupId                                CellGroupId,

rlc-BearerToAddModList                     SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig                        OPTIONAL,   -- Need N

rlc-BearerToReleaseList                    SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity                  OPTIONAL,   -- Need N

mac-CellGroupConfig                        MAC-CellGroupConfig                                                     OPTIONAL,   -- Need M

physicalCellGroupConfig                    PhysicalCellGroupConfig                                                 OPTIONAL,   -- Need M

spCellConfig                               SpCellConfig                                                            OPTIONAL,   -- Need M

sCellToAddModList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig                       OPTIONAL,   -- Need N

sCellToReleaseList                         SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex                        OPTIONAL,   -- Need N

...,

[[

reportUplinkTxDirectCurrent                ENUMERATED {true}                                                   OPTIONAL    -- Cond BWP-Reconfig

]],

[[

bap-Address-r16                            BIT STRING (SIZE (10))                                                  OPTIONAL,   -- Need M

bh-RLC-ChannelToAddModList-r16             SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelConfig-r16 OPTIONAL,   -- Need N

bh-RLC-ChannelToReleaseList-r16            SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelID-r16     OPTIONAL,   -- Need N

f1c-TransferPath-r16                       ENUMERATED {lte, nr, both}                                              OPTIONAL,   -- Need M

simultaneousTCI-UpdateList1-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousTCI-UpdateList2-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList1-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList2-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

uplinkTxSwitchingOption-r16                ENUMERATED {switchedUL, dualUL}                                         OPTIONAL,   -- Need R

uplinkTxSwitchingPowerBoosting-r16         ENUMERATED {enabled}                                                    OPTIONAL    -- Need R

]],

[[

reportUplinkTxDirectCurrentTwoCarrier-r16  ENUMERATED {true}                                                       OPTIONAL    -- Need N

]]

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=                        SEQUENCE {

servCellIndex                       ServCellIndex                                               OPTIONAL,   -- Cond SCG

reconfigurationWithSync             ReconfigurationWithSync                                     OPTIONAL,   -- Cond ReconfWithSync

rlf-TimersAndConstants              SetupRelease { RLF-TimersAndConstants }                     OPTIONAL,   -- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                                             OPTIONAL,   -- Need S

spCellConfigDedicated               ServingCellConfig                                           OPTIONAL,   -- Need M

...

}

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                     OPTIONAL,   -- Need M

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                                               OPTIONAL,   -- Need N

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

daps-UplinkPowerConfig-r16      DAPS-UplinkPowerConfig-r16                                      OPTIONAL    -- Need N

]]

}

DAPS-UplinkPowerConfig-r16 ::=      SEQUENCE {

p-DAPS-Source-r16                   P-Max,

p-DAPS-Target-r16                   P-Max,

uplinkPowerSharingDAPS-Mode-r16     ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic }

}

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Cond DRX-Config2

]],

[[

deactivatedMeasGapList-r17      SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF MeasGapId-r17          OPTIONAL    -- Cond PreConfigMG

]]

}

-- TAG-CELLGROUPCONFIG-STOP

-- ASN1STOP

CellGroupConfig field descriptions

bap-Address BAP address of the parent node in cell group.

bh-RLC-ChannelToAddModList Configuration of the backhaul RLC entities and the corresponding MAC Logical Channels to be added and modified.

bh-RLC-ChannelToReleaseList List of the backhaul RLC entities and the corresponding MAC Logical Channels to be released.

f1c-TransferPath The F1-C transfer path that an EN-DC IAB-MT should use for transferring F1-C packets to the IAB-donor-CU. If IAB-MT is configured with lte, IAB-MT can only use LTE leg for F1-C transfer. If IAB-MT is configured with nr, IAB-MT can only use NR leg for F1-C transfer. If IAB-MT is configured with both, it is up to IAB-MT to select an LTE leg or a NR leg for F1-C transfer. If the field is not configured, the IAB node uses the NR leg as the default one.

mac-CellGroupConfig MAC parameters applicable for the entire cell group.

rlc-BearerToAddModList Configuration of the MAC Logical Channel, the corresponding RLC entities and association with radio bearers.

reportUplinkTxDirectCurrent Enables reporting of uplink and supplementary uplink Direct Current location information upon BWP configuration and reconfiguration. This field is only present when the BWP configuration is modified or any serving cell is added or removed. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. If UE is configured with SUL carrier, UE reports both UL and SUL Direct Current locations.

reportUplinkTxDirectCurrentTwoCarrier Enables reporting of uplink Direct Current location information when the UE is configured with uplink intra-band CA with two carriers. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message.

rlmInSyncOutOfSyncThreshold BLER threshold pair index for IS/OOS indication generation, see TS 38.133 [14], table 8.1.1-1. n1 corresponds to the value 1. When the field is absent, the UE applies the value 0. Whenever this is reconfigured, UE resets N310 and N311, and stops T310, if running. Network does not include this field.

sCellState Indicates whether the SCell shall be considered to be in activated state upon SCell configuration.

sCellToAddModList List of secondary serving cells (SCells) to be added or modified.

sCellToReleaseList List of secondary serving cells (SCells) to be released.

secondaryDRX-GroupConfig The field is used to indicate whether the SCell belongs to the secondary DRX group. All serving cells in the secondary DRX group shall belong to one Frequency Range and all serving cells in the legacy DRX group shall belong to another Frequency Range.

simultaneousTCI-UpdateList1, simultaneousTCI-UpdateList2 List of serving cells which can be updated simultaneously for TCI relation with a MAC CE. The simultaneousTCI-UpdateList1 and simultaneousTCI-UpdateList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

simultaneousSpatial-UpdatedList1, simultaneousSpatial-UpdatedList2 List of serving cells which can be updated simultaneously for spatial relation with a MAC CE. The simultaneousSpatial-UpdatedList1 and simultaneousSpatial-UpdatedList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

spCellConfig Parameters for the SpCell of this cell group (PCell of MCG or PSCell of SCG).

uplinkTxSwitchingOption Indicates which option is configured for dynamic UL Tx switching for inter-band UL CA or (NG)EN-DC. The field is set to switchedUL if network configures option 1 as specified in TS 38.214 [19], or dualUL if network configures option 2 as specified in TS 38.214 [19]. Network always configures UE with a value for this field in inter-band UL CA case and (NG)EN-DC case where UE supports dynamic UL Tx switching.

uplinkTxSwitchingPowerBoosting Indicates whether the UE is allowed to enable 3dB boosting on the maximum output power for transmission on carrier2 under the operation state in which 2-port transmission can be supported on carrier2 for inter-band UL CA case with dynamic UL Tx switching as defined in TS 38.101-1 [15]. Network can only configure this field for dynamic UL Tx switching in inter-band UL CA case with power Class 3 as defined in TS 38.101-1 [15].

DAPS-UplinkPowerConfig field descriptions

p-DAPS-Source The maximum total transmit power to be used by the UE in the source cell group during DAPS handover.

p-DAPS-Target The maximum total transmit power to be used by the UE in the target cell group during DAPS handover.

uplinkPowerSharingDAPS-Mode Indicates the uplink power sharing mode that the UE uses in DAPS handover (see TS 38.213 [13]).

ReconfigurationWithSync field descriptions

rach-ConfigDedicated Random access configuration to be used for the reconfiguration with sync (e.g. handover). The UE performs the RA according to these parameters in the firstActiveUplinkBWP (see UplinkConfig).

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and NR PCell change. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon. For case of NR PCell change, the smtc is based on the timing reference of (source) PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If both this field and targetCellSMTC-SCG are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

SCellConfig field descriptions

deactivatedMeasGapList Indicates a list of gap IDs where the corresponding pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) are deactivated while this SCell is deactivated.

Smtc The SSB periodicity/offset/duration configuration of target cell for NR SCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in sCellConfigCommon. The smtc is based on the timing of the SpCell of associated cell group. In case of inter-RAT handover to NR, the timing reference is the NR PCell. In case of intra-NR PCell change (standalone NR) or NR PSCell change (EN-DC), the timing reference is the target SpCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

SpCellConfig field descriptions

reconfigurationWithSync Parameters for the synchronous reconfiguration to the target SpCell.

rlf-TimersAndConstants Timers and constants for detecting and triggering cell-level radio link failure. For the SCG, rlf-TimersAndConstants can only be set to setup and is always included at SCG addition.

servCellIndex Serving cell ID of a PSCell. The PCell of the Master Cell Group uses ID = 0.

| Conditional Presence | Explanation |
| --- | --- |
| BWP-Reconfig | The field is optionally present, Need N, if the BWPs are reconfigured or if serving cells are added or removed. Otherwise it is absent. |
| DRX-Config2 | The field is optionally present, Need N, if drx-ConfigSecondaryGroup is configured. It is absent otherwise. |
| PreConfigMG | The field is optionally present, Need R, if there is at least one per UE gap configured with preConfigInd or there is at least one per FR gap of the same FR which the SCell belongs to and configured with preConfigInd. It is absent otherwise. |
| ReconfWithSync | The field is mandatory present in the RRCReconfiguration message: - in each configured CellGroupConfig for which the SpCell changes, - in the masterCellGroup: - at change of AS security key derived from KgNB, - in an RRCReconfiguration message contained in a DLInformationTransferMRDC message, - in the secondaryCellGroup at: - PSCell addition, - SCG resume with NR-DC or (NG)EN-DC, - update of required SI for PSCell, - change of AS security key derived from S-KgNB in NR-DC while the UE is configured with at least one radio bearer with keyToUse set to secondary and that is not released by this RRCReconfiguration message, - MN handover in (NG)EN-DC. Otherwise, it is optionally present, need M. The field is absent in the masterCellGroup in RRCResume and RRCSetup messages and is absent in the masterCellGroup in RRCReconfiguration messages if source configuration is not released during DAPS handover. |
| SCellAdd | The field is mandatory present upon SCell addition; otherwise it is absent, Need M. |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |
| SCellAddSync | The field is optionally present, Need N, in case of SCell addition, reconfiguration with sync, and resuming an RRC connection. It is absent otherwise. |
| SCG | The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise. |

NOTE:	In case of change of AS security key derived from S-KgNB/S-KeNB, if reconfigurationWithSync is not included in the masterCellGroup, the network releases all existing MCG RLC bearers associated with a radio bearer with keyToUse set to secondary. In case of change of AS security key derived from KgNB/KeNB, if reconfigurationWithSync is not included in the secondaryCellGroup, the network releases all existing SCG RLC bearers associated with a radio bearer with keyToUse set to primary.

<Skip>

#### –	GapPriority

The IE GapPriority is used to identify the priority of a gap configuration.

GapPriority information element

-- ASN1START

-- TAG-GAPPRIORITY-START

GapPriority-r17 ::=                       INTEGER (1..maxNrOfGapPri-r17)

-- TAG-GAPPRIORITY-STOP

-- ASN1STOP

<Skip>

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEASCONFIG-START

MeasConfig ::=                      SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList                                              OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList                                              OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList                                            OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList                                            OPTIONAL,   -- Need N

measIdToRemoveList                  MeasIdToRemoveList                                                  OPTIONAL,   -- Need N

measIdToAddModList                  MeasIdToAddModList                                                  OPTIONAL,   -- Need N

s-MeasureConfig                     CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                                                                       OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig                                                      OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig                                                       OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig                                                OPTIONAL,   -- Need M

...,

[[

interFrequencyConfig-NoGap-r16      ENUMERATED {true}                                                   OPTIONAL    -- Need R

]]

}

MeasObjectToRemoveList ::=              SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=                  SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=            SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

-- TAG-MEASCONFIG-STOP

-- ASN1STOP

MeasConfig field descriptions

interFrequencyConfig-NoGap-r16 If the field is set to true, UE is configured to perform SSB based inter-frequency measurement without measurement gaps when the inter-frequency SSB is completely contained in the active DL BWP of the UE, as specified in TS 38.133 [14], clause 9.3. Otherwise, the SSB based inter-frequency measurement is performed within measurement gaps.

measGapConfig Used to setup and release measurement gaps in NR.

measIdToAddModList List of measurement identities to add and/or modify.

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

reportConfigToAddModList List of measurement reporting configurations to add and/or modify.

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS.

measGapSharingConfig Specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

#### –	MeasGapConfig

The IE MeasGapConfig specifies the measurement gap configuration and controls setup/release of measurement gaps.

MeasGapConfig information element

-- ASN1START

-- TAG-MEASGAPCONFIG-START

MeasGapConfig ::=                   SEQUENCE {

gapFR2                              SetupRelease { GapConfig }                                              OPTIONAL,   -- Need M

...,

[[

gapFR1                              SetupRelease { GapConfig }                                              OPTIONAL,   -- Need M

gapUE                               SetupRelease { GapConfig }                                              OPTIONAL    -- Need M

]],

[[

gapUEToAddModList-r17         SEQUENCE (SIZE (1..maxNrofGapId-1-r17)) OF GapConfig		OPTIONAL,   -- Need N

gapUEToReleaseList-r17        SEQUENCE (SIZE (1..maxNrofGapId-1-r17)) OF MeasGapId-r17	OPTIONAL,   -- Need N

gapFR1ToAddModList-r17        SEQUENCE (SIZE (1..maxNrofGapId-1-r17)) OF GapConfig		OPTIONAL,   -- Need N

gapFR1ToReleaseList-r17       SEQUENCE (SIZE (1..maxNrofGapId-1-r17)) OF MeasGapId-r17		                OPTIONAL,   -- Need N

gapFR2ToAddModList-r17        SEQUENCE (SIZE (1..maxNrofGapId-1-r17)) OF GapConfig		OPTIONAL,  -- Need N

gapFR2ToReleaseList-r17       SEQUENCE (SIZE (1..maxNrofGapId-1-r17)) OF MeasGapId-r17	OPTIONAL   -- Need N

]]

}

GapConfig ::=                       SEQUENCE {

gapOffset                           INTEGER (0..159),

mgl                                 ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp                                ENUMERATED {ms20, ms40, ms80, ms160},

mgta                                ENUMERATED {ms0, ms0dot25, ms0dot5},

...,

[[

refServCellIndicator                ENUMERATED {pCell, pSCell, mcg-FR2}                                 OPTIONAL   -- Cond NEDCorNRDC

]],

[[

refFR2ServCellAsyncCA-r16           ServCellIndex                                                       OPTIONAL,   -- Cond AsyncCA

mgl-r16                             ENUMERATED {ms10, ms20}                                             OPTIONAL    -- Cond PRS

]],

[[

measGapId-r17                       MeasGapId-r17                                                       OPTIONAL,   -- Cond GapID

preConfigInd-r17                    ENUMERATED {true}                                                   OPTIONAL,   -- Need R

nscgInd-r17                         ENUMERATED {true}                                                   OPTIONAL,   -- Need R

mgta-r17                            ENUMERATED {ms0dot75}                                               OPTIONAL,   -- Need R

mgl-r17                             ENUMERATED {ms1, ms2, ms5}                                          OPTIONAL,   -- Need R

gapAssociation-r17                  MeasGapAssociation-r17                                              OPTIONAL,   -- Need R

gapSharing-r17                      MeasGapSharingScheme                                                OPTIONAL,   -- Need R

gapPriority-r17                     GapPriority-r17                                                     OPTIONAL    -- Need R

]]

}

MeasGapAssociation-r17 ::=              SEQUENCE {

prsMeas-r17                             ENUMERATED {true}                                               OPTIONAL    -- Need R

}

-- TAG-MEASGAPCONFIG-STOP

-- ASN1STOP

MeasGapConfig field descriptions

gapFR1 Indicates measurement gap configuration that applies to FR1 only. In (NG)EN-DC, gapFR1 cannot be set up by NR RRC (i.e. only LTE RRC can configure FR1 measurement gap). In NE-DC, gapFR1 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR1 gap). In NR-DC, gapFR1 can only be set up in the measConfig associated with MCG. gapFR1 can not be configured together with gapUE unless concurrent MGs are supported. The applicability of the FR1 measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

gapFR1ToAddModList A list of of FR1 measurement gap configuartion to be added or modified. In this version of the specification, the network configures this field only in NR standalone.

gapFR1ToReleaseList A list of FR1 measurement gap configuartion to be released.

gapFR2 Indicates measurement gap configuration applies to FR2 only. In (NG)EN-DC or NE-DC, gapFR2 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR2 gap). In NR-DC, gapFR2 can only be set up in the measConfig associated with MCG. gapFR2 cannot be configured together with gapUE unless concurrent MGs are supported. The applicability of the FR2 measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

gapFR2ToAddModList A list of FR2 measurement gap configuartion to be added or modified. In this version of the specification, the network configures this field only in NR standalone.

gapFR2ToReleaseList A list of FR2 measurement gap configuartion to be released.

gapPriority Indicates the priority of this measurement gap. Value 1 indicates highest priority, value 2 indicates second level priority, and so on.

gapSharing Indicates the measurement gap sharing scheme that applies to this GapConfig. For applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on. The network does not include this field if this GapConfig is configured by gapFR1, gapFR2, or gapUE.

gapUE Indicates measurement gap configuration that applies to all frequencies (FR1 and FR2). In (NG)EN-DC, gapUE cannot be set up by NR RRC (i.e. only LTE RRC can configure per UE measurement gap). In NE-DC, gapUE can only be set up by NR RRC (i.e. LTE RRC cannot configure per UE gap). In NR-DC, gapUE can only be set up in the measConfig associated with MCG. The per UE measurement gap is configured with other FR1 gap or FR2 gap simultaneously only while this per UE gap is associated with PRS measurement. The applicability of the per UE measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

gapUEToAddModList A list of per UE measurement gap configuartion to be added or modified. A per UE measurement gap can be configured with other FR1 gap or FR2 gap simultaneously only while this per UE gap is associated with PRS measurement. In this version of the specification, the network configures this field only in NR standalone.

gapUEToReleaseList A list of per UE measurement gap configuartion to be released.

gapOffset Value gapOffset is the gap offset of the gap pattern with MGRP indicated in the field mgrp. The value range is from 0 to mgrp-1. If nscgInd-r17 is present, this offset value refers to the starting point of VIL1 (the visible interruption length before the ML).

measGapId The ID of this measurement gap configuration.

mgl Value mgl is the measurement gap length in ms of the measurement gap. If nscgInd-r17 is not present, the measurement gap length is according to in Table 9.1.2-1 in TS 38.133 [14]. If nscgInd-r17 is present, this field indicates the measurement length (ML) in NCSG pattern and is configured according to Table 9.1.2C-1 in TS 38.133 [14]. Value ms1dot5 corresponds to 1.5 ms, ms3 corresponds to 3 ms and so on. If mgl-r16 or mgl-r17 is present, UE shall ignore the mgl (without suffix).

mgrp Value mgrp is measurement gap repetition period in (ms) of the measurement gap. The measurement gap repetition period is according to Table 9.1.2-1 in TS 38.133 [14].

mgta Value mgta is the measurement gap timing advance in ms. The applicability of the measurement gap timing advance is according to clause 9.1.2 of TS 38.133 [14]. Value ms0 corresponds to 0 ms, ms0dot25 corresponds to 0.25 ms and ms0dot5 corresponds to 0.5 ms. For FR2, the network only configures 0 ms and 0.25 ms. If mgta-r17 is present, UE shall ignore the mgta (without suffix).

nscgInd Indicates that the measurement gap is a NCSG as specified in 38.133 [14].

preConfigInd Indicates whether the measurement gap is a pre-configured measurement gap.

refFR2ServCellAsyncCA Indicates the FR2 serving cell identifier whose SFN and subframe is used for FR2 gap calculation for this gap pattern with asynchronous CA involving FR2 carrier(s).

refServCellIndicator Indicates the serving cell whose SFN and subframe are used for gap calculation for this gap pattern. Value pCell corresponds to the PCell, pSCell corresponds to the PSCell, and mcg-FR2 corresponds to a serving cell on FR2 frequency in MCG.

MeasGapAssociation field descriptions

prsMeas Indicates that PRS measurement is associated with this measurement gap. The network only includes this field for one per UE gap.

| Conditional Presence | Explanation |
| --- | --- |
| AsyncCA | This field is mandatory present when configuring FR2 gap pattern to UE in: - (NG)EN-DC or NR SA with asynchronous CA involving FR2 carrier(s); - NE-DC or NR-DC with asynchronous CA involving FR2 carrier(s), if the field refServCellIndicator is set to mcg-FR2. In case the gap pattern to UE in NE-DC and NR-DC is already configured and the serving cell used for the gap calculation corresponds to a serving cell on FR2 frequency in MCG, then the field is optionally present, need M. Otherwise, it is absent, Need R. |
| GapID | This field is mandatory present when: - more than one per UE gap is configured; or - more than one FR1 gap is configured; or - more than one FR2 gap is configured; or - per UE gap is configured together with per FR gap. It is optional present, Need R, when: - one or more gap is configured as pre-configured measurement gap. Otherwise, this field is not present, Need R. Editor Note: It is FFS whether this gap Id is also needed for feature other than concurrent gap |
| NEDCorNRDC | This field is mandatory present when configuring gap pattern to UE in NE-DC or NR-DC. In case the gap pattern to UE in NE-DC and NR-DC is already configured, then the field is absent, need M. Otherwise, it is absent. |
| PRS | This field is optionally present, Need R, when configuring gap pattern to UE for measurements of DL-PRS configured via LPP (TS 37.355 [49]). Otherwise, it is absent. |

#### –	MeasGapId

The IE MeasGapId used to identify a per UE or per FR measurement gap configuration.

MeasGapId information element

-- ASN1START

-- TAG-MEASGAPID-START

MeasGapId-r17 ::=                       INTEGER (1..maxNrofGapId-r17)

-- TAG-MEASGAPID-STOP

-- ASN1STOP

#### –	MeasGapSharingConfig

The IE MeasGapSharingConfig specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

MeasGapSharingConfig information element

-- ASN1START

-- TAG-MEASGAPSHARINGCONFIG-START

MeasGapSharingConfig ::=        SEQUENCE {

gapSharingFR2                   SetupRelease { MeasGapSharingScheme }       OPTIONAL,   -- Need M

...,

[[

gapSharingFR1                   SetupRelease { MeasGapSharingScheme }       OPTIONAL,   --Need M

gapSharingUE                    SetupRelease { MeasGapSharingScheme }       OPTIONAL    --Need M

]]

}

MeasGapSharingScheme::=         ENUMERATED {scheme00, scheme01, scheme10, scheme11}

-- TAG-MEASGAPSHARINGCONFIG-STOP

-- ASN1STOP

MeasGapSharingConfig field descriptions

gapSharingFR1 Indicates the measurement gap sharing scheme that applies to the gap set via gapFR1. In (NG)EN-DC, gapSharingFR1 cannot be set up by NR RRC (i.e. only LTE RRC can configure FR1 gap sharing). In NE-DC, gapSharingFR1 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR1 gap sharing). In NR-DC, gapSharingFR1 can only be set up in the measConfig associated with MCG. gapSharingFR1 can not be configured together with gapSharingUE. For the applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

gapSharingFR2 Indicates the measurement gap sharing scheme that applies to the gap set via gapFR2. In (NG)EN-DC or NE-DC, gapSharingFR2 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR2 gap sharing). In NR-DC, gapSharingFR2 can only be set up by MCG in the measConfig associated with MCG. gapSharingFR2 cannot be configured together with gapSharingUE. For applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

gapSharingUE Indicates the measurement gap sharing scheme that applies to the gap set via gapUE. In (NG)EN-DC, gapSharingUE cannot be set up by NR RRC (i.e. only LTE RRC can configure per UE gap sharing). In NE-DC, gapSharingUE can only be set up by NR RRC (i.e. LTE RRC cannot configure per UE gap sharing). In NR-DC, gapSharingUE can only be set up in the measConfig associated with MCG. If gapSharingUE is configured, then neither gapSharingFR1 nor gapSharingFR2 can be configured. For the applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

#### –	MeasId

The IE MeasId is used to identify a measurement configuration, i.e., linking of a measurement object and a reporting configuration.

MeasId information element

-- ASN1START

-- TAG-MEASID-START

MeasId ::=                          INTEGER (1..maxNrofMeasId)

-- TAG-MEASID-STOP

-- ASN1STOP

<Skip>

#### –	MeasObjectEUTRA

The IE MeasObjectEUTRA specifies information applicable for E-UTRA cells.

MeasObjectEUTRA information element

-- ASN1START

-- TAG-MEASOBJECTEUTRA-START

MeasObjectEUTRA::=                          SEQUENCE {

carrierFreq                                 ARFCN-ValueEUTRA,

allowedMeasBandwidth                        EUTRA-AllowedMeasBandwidth,

cellsToRemoveListEUTRAN                     EUTRA-CellIndexList                                         OPTIONAL,    -- Need N

cellsToAddModListEUTRAN                     SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF EUTRA-Cell         OPTIONAL,    -- Need N

blackCellsToRemoveListEUTRAN                EUTRA-CellIndexList                                         OPTIONAL,    -- Need N

blackCellsToAddModListEUTRAN                SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF EUTRA-BlackCell    OPTIONAL,    -- Need N

eutra-PresenceAntennaPort1                  EUTRA-PresenceAntennaPort1,

eutra-Q-OffsetRange                         EUTRA-Q-OffsetRange                                         OPTIONAL,    -- Need R

widebandRSRQ-Meas                           BOOLEAN,

...,

[[

associatedMeasGap-r17                       MeasGapId-r17                                               OPTIONAL     -- Need R

]]

}

EUTRA-CellIndexList ::=                     SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF EUTRA-CellIndex

EUTRA-CellIndex ::=                         INTEGER (1..maxCellMeasEUTRA)

EUTRA-Cell ::=                              SEQUENCE {

cellIndexEUTRA                              EUTRA-CellIndex,

physCellId                                  EUTRA-PhysCellId,

cellIndividualOffset                        EUTRA-Q-OffsetRange

}

EUTRA-BlackCell ::=                         SEQUENCE {

cellIndexEUTRA                              EUTRA-CellIndex,

physCellIdRange                             EUTRA-PhysCellIdRange

}

-- TAG-MEASOBJECTEUTRA-STOP

-- ASN1STOP

EUTRAN-BlackCell field descriptions

cellIndexEUTRA Entry index in the cell list.

physicalCellIdRange Physical cell identity or a range of physical cell identities.

EUTRAN-Cell field descriptions

physicalCellId Physical cell identity of a cell in the cell list.

cellIndividualOffset Cell individual offset applicable to a specific cell. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on.

MeasObjectEUTRA field descriptions

allowedMeasBandwidth The maximum allowed measurement bandwidth on a carrier frequency as defined by the parameter Transmission Bandwidth Configuration "NRB" TS 36.104 [33].

associatedMeasGap Indicates the associated measurement gap for measuring this EUTRA frequency.

blackCellsToAddModListEUTRAN List of cells to add/ modify in the black list of cells.

blackCellsToRemoveListEUTRAN List of cells to remove from the black list of cells.

carrierFreq Identifies E-UTRA carrier frequency for which this configuration is valid. Network does not configure more than one MeasObjectEUTRA for the same physical frequency, regardless of the E-ARFCN used to indicate this.

cellsToAddModListEUTRAN List of cells to add/ modify in the cell list.

cellsToRemoveListEUTRAN List of cells to remove from the cell list.

eutra-PresenceAntennaPort1 When set to true, the UE may assume that at least two cell-specific antenna ports are used in all neighbouring cells.

eutra-Q-OffsetRange Used to indicate a cell, or frequency specific offset to be applied when evaluating triggering conditions for measurement reporting. The value is in dB. Value dB-24 corresponds to -24 dB, value dB-22 corresponds to -22 dB and so on.

widebandRSRQ-Meas If set to true, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [40]. The network may set the field to true if the measurement bandwidth indicated by allowedMeasBandwidth is 50 resource blocks or larger; otherwise the network sets this field to false.

#### –	MeasObjectId

The IE MeasObjectId used to identify a measurement object configuration.

MeasObjectId information element

-- ASN1START

-- TAG-MEASOBJECTID-START

MeasObjectId ::=                    INTEGER (1..maxNrofObjectId)

-- TAG-MEASOBJECTID-STOP

-- ASN1STOP

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for SS/PBCH block(s) intra/inter-frequency measurements and/or CSI-RS intra/inter-frequency measurements.

MeasObjectNR information element

-- ASN1START

-- TAG-MEASOBJECTNR-START

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                   OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                          OPTIONAL,   -- Need R

nrofCSI-RS-ResourcesToAverage       INTEGER (2..maxNrofCSI-RS-ResourcesToAverage)                   OPTIONAL,   -- Need R

quantityConfigIndex                 INTEGER (1..maxNrofQuantityConfig),

offsetMO                            Q-OffsetRangeList,

cellsToRemoveList                   PCI-List                                                        OPTIONAL,   -- Need N

cellsToAddModList                   CellsToAddModList                                               OPTIONAL,   -- Need N

blackCellsToRemoveList              PCI-RangeIndexList                                              OPTIONAL,   -- Need N

blackCellsToAddModList              SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

whiteCellsToRemoveList              PCI-RangeIndexList                                              OPTIONAL,   -- Need N

whiteCellsToAddModList              SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

...,

[[

freqBandIndicatorNR                 FreqBandIndicatorNR                                             OPTIONAL,   -- Need R

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}  OPTIONAL    -- Need R

]],

[[

smtc3list-r16                     SSB-MTC3List-r16                                                  OPTIONAL,   -- Need R

rmtc-Config-r16                     SetupRelease {RMTC-Config-r16}                                  OPTIONAL,   -- Need M

t312-r16                            SetupRelease { T312-r16 }                                       OPTIONAL    -- Need M

]],

[[

associatedMeasGapSSB-r17            MeasGapId-r17                                                   OPTIONAL,   -- Need R

associatedMeasGapCSIRS-r17          MeasGapId-r17                                                   OPTIONAL    -- Need R

]]

}

SSB-MTC3List-r16::=                 SEQUENCE (SIZE(1..4)) OF SSB-MTC3-r16

T312-r16 ::=                        ENUMERATED { ms0, ms50, ms100, ms200, ms300, ms400, ms500, ms1000}

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

SSB-ConfigMobility::=               SEQUENCE {

ssb-ToMeasure                           SetupRelease { SSB-ToMeasure }                              OPTIONAL,   -- Need M

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                         SS-RSSI-Measurement                                     OPTIONAL,   -- Need M

...,

[[

ssb-PositionQCL-Common-r16              SSB-PositionQCL-Relation-r16                                OPTIONAL,   -- Cond SharedSpectrum

ssb-PositionQCL-CellsToAddModList-r16   SSB-PositionQCL-CellsToAddModList-r16                       OPTIONAL,   -- Need N

ssb-PositionQCL-CellsToRemoveList-r16   PCI-List                                                    OPTIONAL    -- Need N

]],

[[

deriveSSB-IndexFromCell-Inter-r17       ServCellIndex                                               OPTIONAL    -- Need R

]]

}

Q-OffsetRangeList ::=               SEQUENCE {

rsrpOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrqOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

sinrOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrpOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

rsrqOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

sinrOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0

}

ThresholdNR ::=                     SEQUENCE{

thresholdRSRP                       RSRP-Range                                                      OPTIONAL,   -- Need R

thresholdRSRQ                       RSRQ-Range                                                      OPTIONAL,   -- Need R

thresholdSINR                       SINR-Range                                                      OPTIONAL    -- Need R

}

CellsToAddModList ::=               SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddMod

CellsToAddMod ::=                   SEQUENCE {

physCellId                          PhysCellId,

cellIndividualOffset                Q-OffsetRangeList

}

RMTC-Config-r16 ::=                 SEQUENCE {

rmtc-Periodicity-r16                ENUMERATED {ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffset-r16             INTEGER(0..639)                                                 OPTIONAL,   -- Need M

measDurationSymbols-r16             ENUMERATED {sym1, sym14or12, sym28or24, sym42or36, sym70or60},

rmtc-Frequency-r16                  ARFCN-ValueNR,

ref-SCS-CP-r16                      ENUMERATED {kHz15, kHz30, kHz60-NCP, kHz60-ECP},

...

}

SSB-PositionQCL-CellsToAddModList-r16 ::= SEQUENCE (SIZE (1..maxNrofCellMeas)) OF SSB-PositionQCL-CellsToAddMod-r16

SSB-PositionQCL-CellsToAddMod-r16 ::= SEQUENCE {

physCellId-r16                        PhysCellId,

ssb-PositionQCL-r16                   SSB-PositionQCL-Relation-r16

}

-- TAG-MEASOBJECTNR-STOP

-- ASN1STOP

CellsToAddMod field descriptions

cellIndividualOffset Cell individual offsets applicable to a specific cell.

physCellId Physical cell identity of a cell in the cell list.

MeasObjectNR field descriptions

absThreshCSI-RS-Consolidation Absolute threshold for the consolidation of measurement results per CSI-RS resource(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per CSI-RS resource as described in 5.5.5.2.

absThreshSS-BlocksConsolidation Absolute threshold for the consolidation of measurement results per SS/PBCH block(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per SS/PBCH block index as described in 5.5.5.2.

associatedMeasGapSSB Indicates the associated measurement gap for SSB measuring identified by ssb-ConfigMobility in this measurement object. When multiple MeasObjectNR with the same SSB frequency are configured, the network configures the same measurement gap ID in this field for each MeasObjectNR.

associatedMeasGapCSIRS Indicates the associated measurement gap for CSI-RS measuring identified by csi-rs-ResourceConfigMobility in this measurement object.

blackCellsToAddModList List of cells to add/modify in the black list of cells. It applies only to SSB resources.

blackCellsToRemoveList List of cells to remove from the black list of cells.

cellsToAddModList List of cells to add/modify in the cell list.

cellsToRemoveList List of cells to remove from the cell list.

freqBandIndicatorNR The frequency band in which the SSB and/or CSI-RS indicated in this MeasObjectNR are located and according to which the UE shall perform the RRM measurements. This field is always provided when the network configures measurements with this MeasObjectNR.

measCycleSCell The parameter is used only when an SCell is configured on the frequency indicated by the measObjectNR and is in deactivated state, see TS 38.133 [14]. gNB configures the parameter whenever an SCell is configured on the frequency indicated by the measObjectNR, but the field may also be signalled when an SCell is not configured. Value sf160 corresponds to 160 sub-frames, value sf256 corresponds to 256 sub-frames and so on.

nrofCSInrofCSI-RS-ResourcesToAverage Indicates the maximum number of measurement results per beam based on CSI-RS resources to be averaged. The same value applies for each detected cell associated with this MeasObjectNR.

nrofSS-BlocksToAverage Indicates the maximum number of measurement results per beam based on SS/PBCH blocks to be averaged. The same value applies for each detected cell associated with this MeasObject.

offsetMO Offset values applicable to all measured cells with reference signal(s) indicated in this MeasObjectNR.

quantityConfigIndex Indicates the n-th element of quantityConfigNR-List provided in MeasConfig.

referenceSignalConfig RS configuration for SS/PBCH block and CSI-RS.

refFreqCSI-RS Point A which is used for mapping of CSI-RS to physical resources according to TS 38.211 [16] clause 7.4.1.5.3.

smtc1 Primary measurement timing configuration. (see clause 5.5.2.10).

smtc2 Secondary measurement timing configuration for SS corresponding to this MeasObjectNR with PCI listed in pci-List. For these SS, the periodicity is indicated by periodicity in smtc2 and the timing offset is equal to the offset indicated in periodicityAndOffset modulo periodicity. periodicity in smtc2 can only be set to a value strictly shorter than the periodicity indicated by periodicityAndOffset in smtc1 (e.g. if periodicityAndOffset indicates sf10, periodicity can only be set of sf5, if periodicityAndOffset indicates sf5, smtc2 cannot be configured).

smtc3list Measurement timing configuration list for SS corresponding to IAB-MT. This is used for the IAB-node's discovery of other IAB-nodes and the IAB-Donor-DUs.

ssbFrequency Indicates the frequency of the SS associated to this MeasObjectNR. For operation with shared spectrum channel access, this field is a k*30 kHz shift from the sync raster where k = 0,1,2, and so on if the reportType within the corresponding ReportConfigNR is set to reportCGI (see TS 38.211 [16], clause 7.4.3.1). Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15]).

ssb-PositionQCL-Common Indicates the QCL relationship between SS/PBCH blocks for all measured cells as specified in TS 38.213 [13], clause 4.1.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the values 15 kHz or 30 kHz (FR1), and 120 kHz or 240 kHz (FR2) are applicable.

t312 The value of timer T312. Value ms0 represents 0 ms, ms50 represents 50 ms and so on.

whiteCellsToAddModList List of cells to add/modify in the white list of cells. It applies only to SSB resources.

whiteCellsToRemoveList List of cells to remove from the white list of cells.

RMTC-Config field descriptions

measDurationSymbols Number of consecutive symbols for which the Physical Layer reports samples of RSSI (see TS 38.215 [9], clause 5.1.21). Value sym1 corresponds to one symbol, sym14or12 corresponds to 14 symbols of the reference numerology for NCP and 12 symbols for ECP, and so on.

ref-SCS-CP Indicates a reference subcarrier spacing and cyclic prefix to be used for RSSI measurements (see TS 38.215 [9]). Value kHz15 corresponds to 15kHz, kHz30 corresponds to 30 kHz, value kHz60-NCP corresponds to 60 kHz using normal cyclic prefix (NCP), and kHz60-ECP corresponds to 60 kHz using extended cyclic prefix (ECP).

rmtc-Frequency Indicates the center frequency of the measured bandwidth (see TS 38. 215 [9], clause 5.1.21).

rmtc-Periodicity Indicates the RSSI measurement timing configuration (RMTC) periodicity (see TS 38.215 [9], clause 5.1.21).

rmtc-SubframeOffset Indicates the RSSI measurement timing configuration (RMTC) subframe offset for this frequency (see TS 38.215 [9], clause 5.1.21). For inter-frequency measurements, this field is optional present and if it is not configured, the UE chooses a random value as rmtc-SubframeOffset for measDurationSymbols which shall be selected to be between 0 and the configured rmtc-Periodicity with equal probability.

ReferenceSignalConfig field descriptions

csi-rs-ResourceConfigMobility CSI-RS resources to be used for CSI-RS based RRM measurements.

ssb-ConfigMobility SSB configuration for mobility (nominal SSBs, timing configuration).

SSB-ConfigMobility field descriptions

deriveSSB-IndexFromCell If this field is set to true, UE assumes SFN and frame boundary alignment across cells on the same frequency carrier as specified in TS 38.133 [14]. Hence, if the UE is configured with a serving cell for which (absoluteFrequencySSB, subcarrierSpacing) in ServingCellConfigCommon is equal to (ssbFrequency, ssbSubcarrierSpacing) in this MeasObjectNR, this field indicates whether the UE can utilize the timing of this serving cell to derive the index of SS block transmitted by neighbour cell. Otherwise, this field indicates whether the UE may use the timing of any detected cell on that target frequency to derive the SSB index of all neighbour cells on that frequency.

deriveSSB-IndexFromCell-Inter If this field is present, UE assumes SFN and frame boundary alignment between the reference serving cell indicated by ServCellIndex and all neighbour cells in this MeasObjectNR as specified in TS 38.133 [14]. This field also indicates that the UE can utilize the timing of the reference serving cell indicated by ServCellIndex to derive the index of SS block transmitted by all neighbour cells with same frequency as this MeasObjectNR.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not to be measured while value 1 indicates that the corresponding SS/PBCH block is to be measured (see TS 38.215 [9]). When the field is not configured the UE measures on all SS blocks. Regardless of the value of this field, SS/PBCH blocks outside of the applicable smtc are not to be measured. See TS 38.215 [9] clause 5.1.1.

SSB-PositionQCL-CellsToAddMod field descriptions

physCellId Physical cell identity of a cell in the cell list.

ssb-PositionQCL Indicates the QCL relation between SS/PBCH blocks for a specific cell as specified in TS 38.213 [13], clause 4.1. If provided, the cell specific value overwrites the value signalled by ssb-PositionQCL-Common.

| Conditional Presence | Explanation |
| --- | --- |
| CSI-RS | This field is mandatory present if csi-rs-ResourceConfigMobility is configured, otherwise, it is absent. |
| SSBorAssociatedSSB | This field is mandatory present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. Otherwise, it is absent, Need R. |
| IntraFreqConnected | This field is optionally present, Need R if the UE is configured with a serving cell for which (absoluteFrequencySSB, subcarrierSpacing) in ServingCellConfigCommon is equal to (ssbFrequency, ssbSubcarrierSpacing) in this MeasObjectNR, otherwise, it is absent. |
| SharedSpectrum | This field is mandatory present if this MeasObject is for a frequency which operates with shared spectrum channel access. Otherwise, it is absent, Need R. |

<Skip>

<Skip>

#### –	NeedForGapsConfigNR

The IE NeedForGapsConfigNR contains configuration related to the reporting of measurement gap requirement information.

NeedForGapsConfigNR information element

-- ASN1START

-- TAG-NeedForGapsConfigNR-START

NeedForGapsConfigNR-r16 ::=        SEQUENCE {

requestedTargetBandFilterNR-r16       SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR               OPTIONAL          -- Need R

}

-- TAG-NeedForGapsConfigNR-STOP

-- ASN1STOP

NeedForGapsConfigNR field descriptions

requestedTargetBandFilterNR Indicates the target NR bands that the UE is requested to report the gap requirement information.

#### –	NeedForGapsInfoNR

The IE NeedForGapsInfoNR indicates whether measurement gap is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured.

NeedForGapsInfoNR information element

-- ASN1START

-- TAG-NeedForGapsInfoNR-START

NeedForGapsInfoNR-r16 ::=        SEQUENCE {

intraFreq-needForGap-r16      NeedForGapsIntraFreqList-r16,

interFreq-needForGap-r16      NeedForGapsBandListNR-r16

}

NeedForGapsIntraFreqList-r16 ::=          SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForGapsIntraFreq-r16

NeedForGapsBandListNR-r16 ::=             SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16

NeedForGapsIntraFreq-r16  ::=                 SEQUENCE {

servCellId-r16                               ServCellIndex,

gapIndicationIntra-r16                       ENUMERATED {gap, no-gap}

}

NeedForGapsNR-r16  ::=                        SEQUENCE {

bandNR-r16                                   FreqBandIndicatorNR,

gapIndication-r16                            ENUMERATED {gap, no-gap}

}

-- TAG-NeedForGapsInfoNR-STOP

-- ASN1STOP

NeedForGapsInfoNR field descriptions

intraFreq-needForGap Indicates the measurement gap requirement information for NR intra-frequency measurement.

interFreq-needForGap Indicates the measurement gap requirement information for NR inter-frequency measurement.

NeedForGapsIntraFreq field descriptions

servCellId Indicates the serving cell which contains the target SSB (associated with the initial DL BWP) to be measured.

gapIndicationIntra Indicates whether measurement gap is required for the UE to perform intra-frequency SSB based measurements on the concerned serving cell. Value gap indicates that a measurement gap is needed if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP. Value no-gap indicates a measurement gap is not needed to measure the SSB associated to the initial DL BWP for all configured BWPs, no matter the SSB is within the configured BWP or not.

NeedForGapsNR field descriptions

bandNR Indicates the NR target band to be measured.

gapIndication Indicates whether measurement gap is required for the UE to perform SSB based measurements on the concerned NR target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value no-gap indicates a measurement gap is not needed.

#### –	NeedForNCSG-ConfigEUTRA

The IE NeedForNCSG-ConfigEUTRA contains configuration related to the reporting of measurement gap and NCSG requirement information.

NeedForNCSG-ConfigEUTRA information element

-- ASN1START

-- TAG-NeedForNCSG-ConfigEUTRA-START

NeedForNCSG-ConfigEUTRA-r17 ::=        SEQUENCE {

requestedTargetBandFilterNCSG-EUTRA-r17       SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA          OPTIONAL          -- Need R

}

-- TAG-NeedForNCSG-ConfigEUTRA-STOP

-- ASN1STOP

NeedForNCSG-ConfigEUTRA field descriptions

requestedTargetBandFilterNCSG-EUTRA Indicates the target E-UTRA bands that the UE is requested to report the measurement gap and NCSG requirement information.

#### –	NeedForNCSG-ConfigNR

The IE NeedForNCSG-ConfigNR contains configuration related to the reporting of measurement gap and NCSG requirement information.

NeedForNCSG-ConfigNR information element

-- ASN1START

-- TAG-NeedForNCSG-ConfigNR-START

NeedForNCSG-ConfigNR-r17 ::=        SEQUENCE {

requestedTargetBandFilterNCSG-NR-r17       SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR              OPTIONAL          -- Need R

}

-- TAG-NeedForNCSG-ConfigNR-STOP

-- ASN1STOP

NeedForNCSG-ConfigNR field descriptions

requestedTargetBandFilterNCSG-NR Indicates the target NR bands that the UE is requested to report the measurement gap and NCSG requirement information.

#### –	NeedForNCSG-InfoEUTRA

The IE NeedForNCSG-InfoEUTRA indicates whether measurement gap or NCSG is required for the UE to perform measurements on an E-UTRA target band while NR-DC or NE-DC is not configured.

NeedForNCSG-InfoEUTRA information element

-- ASN1START

-- TAG-NeedForNCSG-InfoEUTRA-START

NeedForNCSG-InfoEUTRA-r17 ::=        SEQUENCE {

needForNCSG-EUTRA-r17              NeedForNSCG-BandListEUTRA-r17

}

NeedForNSCG-BandListEUTRA-r17 ::=             SEQUENCE (SIZE (1..maxBandsEUTRA)) OF NeedForNCSG-EUTRA-r17

NeedForNCSG-EUTRA-r17  ::=                    SEQUENCE {

bandEUTRA-r17                               FreqBandIndicatorEUTRA,

gapIndication-r17                           ENUMERATED {gap, ncsg, nogap-noNcsg}

}

-- TAG-NeedForNCSG-InfoEUTRA-STOP

-- ASN1STOP

NeedForNCSG-InfoEUTRA field descriptions

needForNCSG-EUTRA Indicates the measurement gap and NCSG requirement information for E-UTRA measurement.

NeedForNCSG-EUTRA field descriptions

bandEUTRA Indicates the E-UTRA target band to be measured.

gapIndication Indicates whether measurement gap or NCSG is required for the UE to perform measurements on the concerned E-UTRA target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration message or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value ncsg indicates that NCSG is needed, value nogap-noNcsg indicates neither a measurement gap nor a NCSG is needed.

#### –	NeedForNCSG-InfoNR

The IE NeedForNCSG-InfoNR indicates whether measurement gap or NCSG is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured.

NeedForNCSG-InfoNR information element

-- ASN1START

-- TAG-NeedForNCSG-InfoNR-START

NeedForNCSG-InfoNR-r17 ::=        SEQUENCE {

intraFreq-needForNCSG-r17      NeedForNCSG-IntraFreqList-r17,

interFreq-needForNCSG-r17      NeedForNCSG-BandListNR-r17

}

NeedForNCSG-IntraFreqList-r17 ::=          SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForNCSG-IntraFreq-r17

NeedForNCSG-BandListNR-r17 ::=             SEQUENCE (SIZE (1..maxBands)) OF NeedForNCSG-NR-r17

NeedForNCSG-IntraFreq-r17  ::=                 SEQUENCE {

servCellId-r17                               ServCellIndex,

gapIndicationIntra-r17                       ENUMERATED {gap, ncsg, nogap-noNcsg}

}

NeedForNCSG-NR-r17  ::=                        SEQUENCE {

bandNR-r17                                   FreqBandIndicatorNR,

gapIndication-r17                            ENUMERATED {gap, ncsg, nogap-noNcsg}

}

-- TAG-NeedForNCSGInfoNR-STOP

-- ASN1STOP

NeedForNCSG-InfoNR field descriptions

intraFreq-needForNCSG Indicates the measurement gap and NCSG requirement information for NR intra-frequency measurement.

interFreq-needForNCSG Indicates the measurement gap and NCSG requirement information for NR inter-frequency measurement.

NeedForNCSG-IntraFreq field descriptions

servCellId Indicates the serving cell which contains the target SSB (associated with the initial DL BWP) to be measured.

gapIndicationIntra Indicates whether measurement gap or NCSG is required for the UE to perform intra-frequency SSB based measurements on the concerned serving cell. Value gap indicates that a measurement gap is needed if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP. Value ncsg indicates that a NCSG is needed if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP. Value nogap-noNcsg indicates that neither a measurement gap nor a NCSG is needed to measure the SSB associated to the initial DL BWP for all configured BWPs, no matter the SSB is within the configured BWP or not.

NeedForNCSG-NR field descriptions

bandNR Indicates the NR target band to be measured.

gapIndication Indicates whether measurement gap or NCSG is required for the UE to perform SSB based measurements on the concerned NR target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value ncsg indicates that a NCSG is needed, and value nogap-noNcsg indicates neither a measurement gap nor a NCSG is needed.

<Skip>

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxAI-DCI-PayloadSize-r16               INTEGER ::= 128      --Maximum size of the DCI payload scrambled with ai-RNTI

maxAI-DCI-PayloadSize-1-r16             INTEGER ::= 127      --Maximum size of the DCI payload scrambled with ai-RNTI minus 1

maxBandComb                             INTEGER ::= 65536   -- Maximum number of DL band combinations

maxBandsUTRA-FDD-r16                    INTEGER ::= 64      -- Maximum number of bands listed in UTRA-FDD UE caps

maxBH-RLC-ChannelID-r16                 INTEGER ::= 65536   -- Maximum value of BH RLC Channel ID

maxBT-IdReport-r16                      INTEGER ::= 32      -- Maximum number of Bluetooth IDs to report

maxBT-Name-r16                          INTEGER ::= 4       -- Maximum number of Bluetooth name

maxCAG-Cell-r16                         INTEGER ::= 16      -- Maximum number of NR CAG cell ranges in SIB3, SIB4

maxTwoPUCCH-Grp-ConfigList-r16          INTEGER ::= 32      -- Maximum number of supported configuration(s) of {primary PUCCH group

-- config, secondary PUCCH group config}

maxCBR-Config-r16                       INTEGER ::= 8       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control

maxCBR-Config-1-r16                     INTEGER ::= 7       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control minus 1

maxCBR-Level-r16                        INTEGER ::= 16      -- Maximum number of CBR levels

maxCBR-Level-1-r16                      INTEGER ::= 15      -- Maximum number of CBR levels minus 1

maxCellBlack                            INTEGER ::= 16      -- Maximum number of NR blacklisted cell ranges in SIB3, SIB4

maxCellGroupings-r16                    INTEGER ::= 32      -- Maximum number of cell groupings for NR-DC

maxCellHistory-r16                      INTEGER ::= 16      -- Maximum number of visited cells reported

maxCellInter                            INTEGER ::= 16      -- Maximum number of inter-Freq cells listed in SIB4

maxCellIntra                            INTEGER ::= 16      -- Maximum number of intra-Freq cells listed in SIB3

maxCellMeasEUTRA                        INTEGER ::= 32      -- Maximum number of cells in E-UTRAN

maxCellMeasIdle-r16                     INTEGER ::= 8       -- Maximum number of cells per carrier for idle/inactive measurements

maxCellMeasUTRA-FDD-r16                 INTEGER ::= 32      -- Maximum number of cells in FDD UTRAN

maxCellWhite                            INTEGER ::= 16      -- Maximum number of NR whitelisted cell ranges in SIB3, SIB4

maxEARFCN                               INTEGER ::= 262143  -- Maximum value of E-UTRA carrier frequency

maxEUTRA-CellBlack                      INTEGER ::= 16      -- Maximum number of E-UTRA blacklisted physical cell identity ranges

-- in SIB5

maxEUTRA-NS-Pmax                        INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxLogMeasReport-r16                    INTEGER ::= 520     -- Maximum number of entries for logged measurements

maxMultiBands                           INTEGER ::= 8       -- Maximum number of additional frequency bands that a cell belongs to

maxNARFCN                               INTEGER ::= 3279165 -- Maximum value of NR carrier frequency

maxNR-NS-Pmax                           INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxFreqIdle-r16                         INTEGER ::= 8       -- Maximum number of carrier frequencies for idle/inactive measurements

maxNrofServingCells                     INTEGER ::= 32      -- Max number of serving cells (SpCells + SCells)

maxNrofServingCells-1                   INTEGER ::= 31      -- Max number of serving cells (SpCells + SCells) minus 1

maxNrofAggregatedCellsPerCellGroup      INTEGER ::= 16

maxNrofAggregatedCellsPerCellGroupMinus4-r16   INTEGER ::= 12

maxNrofDUCells-r16                      INTEGER ::= 512     -- Max number of cells configured on the collocated IAB-DU

maxNrofAvailabilityCombinationsPerSet-r16   INTEGER ::= 512 -- Max number of AvailabilityCombinationId used in the DCI format 2_5

maxNrofAvailabilityCombinationsPerSet-1-r16 INTEGER ::= 511 -- Max number of AvailabilityCombinationId used in the DCI format 2_5 minus 1

maxNrofSCells                           INTEGER ::= 31      -- Max number of secondary serving cells per cell group

maxNrofCellMeas                         INTEGER ::= 32      -- Maximum number of entries in each of the cell lists in a measurement object

maxNrofCG-SL-r16                        INTEGER ::= 8       -- Max number of sidelink configured grant

maxNrofCG-SL-1-r16                      INTEGER ::= 7       -- Max number of sidelink configured grant minus 1

maxNrofSS-BlocksToAverage               INTEGER ::= 16      -- Max number for the (max) number of SS blocks to average to determine cell measurement

maxNrofCondCells-r16                    INTEGER ::= 8       -- Max number of conditional candidate SpCells

maxNrofCSI-RS-ResourcesToAverage        INTEGER ::= 16      -- Max number for the (max) number of CSI-RS to average to determine cell measurement

maxNrofDL-Allocations                   INTEGER ::= 16      -- Maximum number of PDSCH time domain resource allocations

maxNrofSR-ConfigPerCellGroup            INTEGER ::= 8       -- Maximum number of SR configurations per cell group

maxLCG-ID                               INTEGER ::= 7       -- Maximum value of LCG ID

maxLC-ID                                INTEGER ::= 32      -- Maximum value of Logical Channel ID

maxLC-ID-Iab-r16                        INTEGER ::= 65855   -- Maximum value of BH Logical Channel ID extension

maxLTE-CRS-Patterns-r16                 INTEGER ::= 3       -- Maximum number of additional LTE CRS rate matching patterns

maxNrofTAGs                             INTEGER ::= 4       -- Maximum number of Timing Advance Groups

maxNrofTAGs-1                           INTEGER ::= 3       -- Maximum number of Timing Advance Groups minus 1

maxNrofBWPs                             INTEGER ::= 4       -- Maximum number of BWPs per serving cell

maxNrofCombIDC                          INTEGER ::= 128     -- Maximum number of reported MR-DC combinations for IDC

maxNrofSymbols-1                        INTEGER ::= 13      -- Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13)

maxNrofSlots                            INTEGER ::= 320     -- Maximum number of slots in a 10 ms period

maxNrofSlots-1                          INTEGER ::= 319     -- Maximum number of slots in a 10 ms period minus 1

maxNrofPhysicalResourceBlocks           INTEGER ::= 275     -- Maximum number of PRBs

maxNrofPhysicalResourceBlocks-1         INTEGER ::= 274     -- Maximum number of PRBs minus 1

maxNrofPhysicalResourceBlocksPlus1      INTEGER ::= 276     -- Maximum number of PRBs plus 1

maxNrofControlResourceSets              INTEGER ::= 12      -- Max number of CoReSets configurable on a serving cell

maxNrofControlResourceSets-1            INTEGER ::= 11      -- Max number of CoReSets configurable on a serving cell minus 1

maxNrofControlResourceSets-1-r16        INTEGER ::= 15      -- Max number of CoReSets configurable on a serving cell extended in minus 1

maxNrofCoresetPools-r16                 INTEGER ::= 2       -- Maximum number of CORESET pools

maxCoReSetDuration                      INTEGER ::= 3       -- Max number of OFDM symbols in a control resource set

maxNrofSearchSpaces-1                   INTEGER ::= 39      -- Max number of Search Spaces minus 1

maxSFI-DCI-PayloadSize                  INTEGER ::= 128     -- Max number payload of a DCI scrambled with SFI-RNTI

maxSFI-DCI-PayloadSize-1                INTEGER ::= 127     -- Max number payload of a DCI scrambled with SFI-RNTI minus 1

maxIAB-IP-Address-r16                   INTEGER ::= 32      -- Max number of assigned IP addresses

maxINT-DCI-PayloadSize                  INTEGER ::= 126     -- Max number payload of a DCI scrambled with INT-RNTI

maxINT-DCI-PayloadSize-1                INTEGER ::= 125     -- Max number payload of a DCI scrambled with INT-RNTI minus 1

maxNrofRateMatchPatterns                INTEGER ::= 4       -- Max number of rate matching patterns that may be configured

maxNrofRateMatchPatterns-1              INTEGER ::= 3       -- Max number of rate matching patterns that may be configured minus 1

maxNrofRateMatchPatternsPerGroup        INTEGER ::= 8       -- Max number of rate matching patterns that may be configured in one group

maxNrofCSI-ReportConfigurations         INTEGER ::= 48      -- Maximum number of report configurations

maxNrofCSI-ReportConfigurations-1       INTEGER ::= 47      -- Maximum number of report configurations minus 1

maxNrofCSI-ResourceConfigurations       INTEGER ::= 112     -- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1     INTEGER ::= 111     -- Maximum number of resource configurations minus 1

maxNrofAP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrOfCSI-AperiodicTriggers            INTEGER ::= 128     -- Maximum number of triggers for aperiodic CSI reporting

maxNrofReportConfigPerAperiodicTrigger  INTEGER ::= 16      -- Maximum number of report configurations per trigger state for aperiodic reporting

maxNrofNZP-CSI-RS-Resources             INTEGER ::= 192     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofNZP-CSI-RS-Resources-1           INTEGER ::= 191     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofNZP-CSI-RS-ResourcesPerSet       INTEGER ::= 64      -- Maximum number of NZP CSI-RS resources per resource set

maxNrofNZP-CSI-RS-ResourceSets          INTEGER ::= 64      -- Maximum number of NZP CSI-RS resource sets per cell

maxNrofNZP-CSI-RS-ResourceSets-1        INTEGER ::= 63      -- Maximum number of NZP CSI-RS resource sets per cell minus 1

maxNrofNZP-CSI-RS-ResourceSetsPerConfig INTEGER ::= 16      -- Maximum number of resource sets per resource configuration

maxNrofNZP-CSI-RS-ResourcesPerConfig    INTEGER ::= 128     -- Maximum number of resources per resource configuration

maxNrofZP-CSI-RS-Resources              INTEGER ::= 32      -- Maximum number of Zero-Power (ZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1            INTEGER ::= 31      -- Maximum number of Zero-Power (ZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-ResourceSets-1         INTEGER ::= 15

maxNrofZP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrofZP-CSI-RS-ResourceSets           INTEGER ::= 16

maxNrofCSI-IM-Resources                 INTEGER ::= 32      -- Maximum number of CSI-IM resources

maxNrofCSI-IM-Resources-1               INTEGER ::= 31      -- Maximum number of CSI-IM resources minus 1

maxNrofCSI-IM-ResourcesPerSet           INTEGER ::= 8       -- Maximum number of CSI-IM resources per set

maxNrofCSI-IM-ResourceSets              INTEGER ::= 64      -- Maximum number of NZP CSI-IM resource sets per cell

maxNrofCSI-IM-ResourceSets-1            INTEGER ::= 63      -- Maximum number of NZP CSI-IM resource sets per cell minus 1

maxNrofCSI-IM-ResourceSetsPerConfig     INTEGER ::= 16      -- Maximum number of CSI IM resource sets per resource configuration

maxNrofCSI-SSB-ResourcePerSet           INTEGER ::= 64      -- Maximum number of SSB resources in a resource set

maxNrofCSI-SSB-ResourceSets             INTEGER ::= 64      -- Maximum number of CSI SSB resource sets per cell

maxNrofCSI-SSB-ResourceSets-1           INTEGER ::= 63      -- Maximum number of CSI SSB resource sets per cell minus 1

maxNrofCSI-SSB-ResourceSetsPerConfig    INTEGER ::= 1       -- Maximum number of CSI SSB resource sets per resource configuration

maxNrofFailureDetectionResources        INTEGER ::= 10      -- Maximum number of failure detection resources

maxNrofFailureDetectionResources-1      INTEGER ::= 9       -- Maximum number of failure detection resources minus 1

maxNrofFreqSL-r16                       INTEGER ::= 8       -- Maximum number of carrier frequency for NR sidelink communication

maxNrofSL-BWPs-r16                      INTEGER ::= 4       -- Maximum number of BWP for NR sidelink communication

maxFreqSL-EUTRA-r16                     INTEGER ::= 8       -- Maximum number of EUTRA anchor carrier frequency for NR sidelink communication

maxNrofSL-MeasId-r16                    INTEGER ::= 64      -- Maximum number of sidelink measurement identity (RSRP) per destination

maxNrofSL-ObjectId-r16                  INTEGER ::= 64      -- Maximum number of sidelink measurement objects (RSRP) per destination

maxNrofSL-ReportConfigId-r16            INTEGER ::= 64      -- Maximum number of sidelink measurement reporting configuration(RSRP) per destination

maxNrofSL-PoolToMeasureNR-r16           INTEGER ::= 8       -- Maximum number of resource pool for NR sidelink measurement to measure for

-- each measurement object (for CBR)

maxFreqSL-NR-r16                        INTEGER ::= 8       -- Maximum number of NR anchor carrier frequency for NR sidelink communication

maxNrofSL-QFIs-r16                      INTEGER ::= 2048    -- Maximum number of QoS flow for NR sidelink communication per UE

maxNrofSL-QFIsPerDest-r16               INTEGER ::= 64      -- Maximum number of QoS flow per destination for NR sidelink communication

maxNrofObjectId                         INTEGER ::= 64      -- Maximum number of measurement objects

maxNrofPageRec                          INTEGER ::= 32      -- Maximum number of page records

maxNrofPCI-Ranges                       INTEGER ::= 8       -- Maximum number of PCI ranges

maxPLMN                                 INTEGER ::= 12      -- Maximum number of PLMNs broadcast and reported by UE at establishment

maxNrofCSI-RS-ResourcesRRM              INTEGER ::= 96      -- Maximum number of CSI-RS resources per cell for an RRM measurement object

maxNrofCSI-RS-ResourcesRRM-1            INTEGER ::= 95      -- Maximum number of CSI-RS resources per cell for an RRM measurement object minus 1

maxNrofMeasId                           INTEGER ::= 64      -- Maximum number of configured measurements

maxNrofQuantityConfig                   INTEGER ::= 2       -- Maximum number of quantity configurations

maxNrofCSI-RS-CellsRRM                  INTEGER ::= 96      -- Maximum number of cells with CSI-RS resources for an RRM measurement object

maxNrofSL-Dest-r16                      INTEGER ::= 32      -- Maximum number of destination for NR sidelink communication

maxNrofSL-Dest-1-r16                    INTEGER ::= 31      -- Highest index of destination for NR sidelink communication

maxNrofSLRB-r16                         INTEGER ::= 512     -- Maximum number of radio bearer for NR sidelink communication per UE

maxSL-LCID-r16                          INTEGER ::= 512     -- Maximum number of RLC bearer for NR sidelink communication per UE

maxSL-SyncConfig-r16                    INTEGER ::= 16      -- Maximum number of sidelink Sync configurations

maxNrofRXPool-r16                       INTEGER ::= 16      -- Maximum number of Rx resource pool for NR sidelink communication

maxNrofTXPool-r16                       INTEGER ::= 8       -- Maximum number of Tx resource pool for NR sidelink communication

maxNrofPoolID-r16                       INTEGER ::= 16      -- Maximum index of resource pool for NR sidelink communication

maxNrofSRS-PathlossReferenceRS-r16      INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for SRS power control.

maxNrofSRS-PathlossReferenceRS-1-r16    INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for SRS power control minus 1.

maxNrofSRS-ResourceSets                 INTEGER ::= 16      -- Maximum number of SRS resource sets in a BWP.

maxNrofSRS-ResourceSets-1               INTEGER ::= 15      -- Maximum number of SRS resource sets in a BWP minus 1.

maxNrofSRS-PosResourceSets-r16          INTEGER ::= 16      -- Maximum number of SRS Positioning resource sets in a BWP.

maxNrofSRS-PosResourceSets-1-r16        INTEGER ::= 15      -- Maximum number of SRS Positioning resource sets in a BWP minus 1.

maxNrofSRS-Resources                    INTEGER ::= 64      -- Maximum number of SRS resources.

maxNrofSRS-Resources-1                  INTEGER ::= 63      -- Maximum number of SRS resources minus 1.

maxNrofSRS-PosResources-r16             INTEGER ::= 64      -- Maximum number of SRS Positioning resources.

maxNrofSRS-PosResources-1-r16           INTEGER ::= 63      -- Maximum number of SRS Positioning resources in an SRS Positioning

-- resource set minus 1.

maxNrofSRS-ResourcesPerSet              INTEGER ::= 16      -- Maximum number of SRS resources in an SRS resource set

maxNrofSRS-TriggerStates-1              INTEGER ::= 3       -- Maximum number of SRS trigger states minus 1, i.e., the largest code point.

maxNrofSRS-TriggerStates-2              INTEGER ::= 2       -- Maximum number of SRS trigger states minus 2.

maxRAT-CapabilityContainers             INTEGER ::= 8       -- Maximum number of interworking RAT containers (incl NR and MRDC)

maxSimultaneousBands                    INTEGER ::= 32      -- Maximum number of simultaneously aggregated bands

maxULTxSwitchingBandPairs               INTEGER ::= 32      -- Maximum number of band pairs supporting dynamic UL Tx switching in a band combination

maxNrofSlotFormatCombinationsPerSet     INTEGER ::= 512     -- Maximum number of Slot Format Combinations in a SF-Set.

maxNrofSlotFormatCombinationsPerSet-1   INTEGER ::= 511     -- Maximum number of Slot Format Combinations in a SF-Set minus 1.

maxNrofTrafficPattern-r16               INTEGER ::= 8       -- Maximum number of Traffic Pattern for NR sidelink communication.

maxNrofPUCCH-Resources                  INTEGER ::= 128

maxNrofPUCCH-Resources-1                INTEGER ::= 127

maxNrofPUCCH-ResourceSets               INTEGER ::= 4       -- Maximum number of PUCCH Resource Sets

maxNrofPUCCH-ResourceSets-1             INTEGER ::= 3       -- Maximum number of PUCCH Resource Sets minus 1.

maxNrofPUCCH-ResourcesPerSet            INTEGER ::= 32      -- Maximum number of PUCCH Resources per PUCCH-ResourceSet

maxNrofPUCCH-P0-PerSet                  INTEGER ::= 8       -- Maximum number of P0-pucch present in a p0-pucch set

maxNrofPUCCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUCCH power control.

maxNrofPUCCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUCCH power control minus 1.

maxNrofPUCCH-PathlossReferenceRSs-r16   INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUCCH power control extended.

maxNrofPUCCH-PathlossReferenceRSs-1-r16 INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- minus 1 extended.

maxNrofPUCCH-PathlossReferenceRSsDiff-r16 INTEGER ::= 60    -- Difference between the extended maximum and the non-extended maximum

maxNrofPUCCH-ResourceGroups-r16         INTEGER ::= 4       -- Maximum number of PUCCH resources groups.

maxNrofPUCCH-ResourcesPerGroup-r16      INTEGER ::= 128     -- Maximum number of PUCCH resources in a PUCCH group.

maxNrofMultiplePUSCHs-r16               INTEGER ::= 8       -- Maximum number of multiple PUSCHs in PUSCH TDRA list

maxNrofP0-PUSCH-AlphaSets               INTEGER ::= 30      -- Maximum number of P0-pusch-alpha-sets (see TS 38.213 [13], clause 7.1)

maxNrofP0-PUSCH-AlphaSets-1             INTEGER ::= 29      -- Maximum number of P0-pusch-alpha-sets minus 1 (see TS 38.213 [13], clause 7.1)

maxNrofPUSCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUSCH power control.

maxNrofPUSCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUSCH power control minus 1.

maxNrofPUSCH-PathlossReferenceRSs-r16   INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUSCH power control extended

maxNrofPUSCH-PathlossReferenceRSs-1-r16 INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUSCH power control

-- extended minus 1

maxNrofPUSCH-PathlossReferenceRSsDiff-r16  INTEGER ::= 60   -- Difference between maxNrofPUSCH-PathlossReferenceRSs-r16 and

-- maxNrofPUSCH-PathlossReferenceRSs

maxNrofNAICS-Entries                    INTEGER ::= 8       -- Maximum number of supported NAICS capability set

maxBands                                INTEGER ::= 1024    -- Maximum number of supported bands in UE capability.

maxBandsMRDC                            INTEGER ::= 1280

maxBandsEUTRA                           INTEGER ::= 256

maxCellReport                           INTEGER ::= 8

maxDRB                                  INTEGER ::= 29      -- Maximum number of DRBs (that can be added in DRB-ToAddModList).

maxFreq                                 INTEGER ::= 8       -- Max number of frequencies.

maxFreqLayers                           INTEGER ::= 4       -- Max number of frequency layers.

maxFreqIDC-r16                          INTEGER ::= 128     -- Max number of frequencies for IDC indication.

maxCombIDC-r16                          INTEGER ::= 128     -- Max number of reported UL CA for IDC indication.

maxFreqIDC-MRDC                         INTEGER ::= 32      -- Maximum number of candidate NR frequencies for MR-DC IDC indication

maxNrofCandidateBeams                   INTEGER ::= 16      -- Max number of PRACH-ResourceDedicatedBFR in BFR config.

maxNrofCandidateBeams-r16               INTEGER ::= 64      -- Max number of candidate beam resources in BFR config.

maxNrofCandidateBeamsExt-r16            INTEGER ::= 48      -- Max number of PRACH-ResourceDedicatedBFR in the CandidateBeamRSListExt

maxNrofPCIsPerSMTC                      INTEGER ::= 64      -- Maximum number of PCIs per SMTC.

maxNrofQFIs                             INTEGER ::= 64

maxNrofResourceAvailabilityPerCombination-r16 INTEGER ::= 256

maxNrOfSemiPersistentPUSCH-Triggers     INTEGER ::= 64      -- Maximum number of triggers for semi persistent reporting on PUSCH

maxNrofSR-Resources                     INTEGER ::= 8       -- Maximum number of SR resources per BWP in a cell.

maxNrofSlotFormatsPerCombination        INTEGER ::= 256

maxNrofSpatialRelationInfos             INTEGER ::= 8

maxNrofSpatialRelationInfos-plus-1      INTEGER ::= 9

maxNrofSpatialRelationInfos-r16         INTEGER ::= 64

maxNrofSpatialRelationInfosDiff-r16     INTEGER ::= 56      -- Difference between maxNrofSpatialRelationInfos-r16 and maxNrofSpatialRelationInfos

maxNrofIndexesToReport                  INTEGER ::= 32

maxNrofIndexesToReport2                 INTEGER ::= 64

maxNrofSSBs-r16                         INTEGER ::= 64      -- Maximum number of SSB resources in a resource set.

maxNrofSSBs-1                           INTEGER ::= 63      -- Maximum number of SSB resources in a resource set minus 1.

maxNrofS-NSSAI                          INTEGER ::= 8       -- Maximum number of S-NSSAI.

maxNrofTCI-StatesPDCCH                  INTEGER ::= 64

maxNrofTCI-States                       INTEGER ::= 128     -- Maximum number of TCI states.

maxNrofTCI-States-1                     INTEGER ::= 127     -- Maximum number of TCI states minus 1.

maxNrofUL-Allocations                   INTEGER ::= 16      -- Maximum number of PUSCH time domain resource allocations.

maxQFI                                  INTEGER ::= 63

maxRA-CSIRS-Resources                   INTEGER ::= 96

maxRA-OccasionsPerCSIRS                 INTEGER ::= 64      -- Maximum number of RA occasions for one CSI-RS

maxRA-Occasions-1                       INTEGER ::= 511     -- Maximum number of RA occasions in the system

maxRA-SSB-Resources                     INTEGER ::= 64

maxSCSs                                 INTEGER ::= 5

maxSecondaryCellGroups                  INTEGER ::= 3

maxNrofServingCellsEUTRA                INTEGER ::= 32

maxMBSFN-Allocations                    INTEGER ::= 8

maxNrofMultiBands                       INTEGER ::= 8

maxCellSFTD                             INTEGER ::= 3       -- Maximum number of cells for SFTD reporting

maxReportConfigId                       INTEGER ::= 64

maxNrofCodebooks                        INTEGER ::= 16      -- Maximum number of codebooks supported by the UE

maxNrofCSI-RS-ResourcesExt-r16          INTEGER ::= 16      -- Maximum number of codebook resources supported by the UE for eType2/Codebook combo

maxNrofCSI-RS-Resources                 INTEGER ::= 7       -- Maximum number of codebook resources supported by the UE

maxNrofCSI-RS-ResourcesAlt-r16          INTEGER ::= 512     -- Maximum number of alternative codebook resources supported by the UE

maxNrofCSI-RS-ResourcesAlt-1-r16        INTEGER ::= 511     -- Maximum number of alternative codebook resources supported by the UE minus 1

maxNrofSRI-PUSCH-Mappings               INTEGER ::= 16

maxNrofSRI-PUSCH-Mappings-1             INTEGER ::= 15

maxSIB                                  INTEGER::= 32       -- Maximum number of SIBs

maxSI-Message                           INTEGER::= 32       -- Maximum number of SI messages

maxPO-perPF                             INTEGER ::= 4       -- Maximum number of paging occasion per paging frame

maxAccessCat-1                          INTEGER ::= 63      -- Maximum number of Access Categories minus 1

maxBarringInfoSet                       INTEGER ::= 8       -- Maximum number of access control parameter sets

maxCellEUTRA                            INTEGER ::= 8       -- Maximum number of E-UTRA cells in SIB list

maxEUTRA-Carrier                        INTEGER ::= 8       -- Maximum number of E-UTRA carriers in SIB list

maxPLMNIdentities                       INTEGER ::= 8       -- Maximum number of PLMN identities in RAN area configurations

maxDownlinkFeatureSets                  INTEGER ::= 1024    -- (for NR DL) Total number of FeatureSets (size of the pool)

maxUplinkFeatureSets                    INTEGER ::= 1024    -- (for NR UL) Total number of FeatureSets (size of the pool)

maxEUTRA-DL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxEUTRA-UL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxFeatureSetsPerBand                   INTEGER ::= 128     -- (for NR) The number of feature sets associated with one band.

maxPerCC-FeatureSets                    INTEGER ::= 1024    -- (for NR) Total number of CC-specific FeatureSets (size of the pool)

maxFeatureSetCombinations               INTEGER ::= 1024    -- (for MR-DC/NR)Total number of Feature set combinations (size of the pool)

maxInterRAT-RSTD-Freq                   INTEGER ::= 3

maxHRNN-Len-r16                         INTEGER ::= 48      -- Maximum length of HRNNs

maxNPN-r16                              INTEGER ::= 12      -- Maximum number of NPNs broadcast and reported by UE at establishment

maxNrOfMinSchedulingOffsetValues-r16    INTEGER ::= 2       -- Maximum number of min. scheduling offset (K0/K2) configurations

maxK0-SchedulingOffset-r16              INTEGER ::= 16      -- Maximum number of slots configured as min. scheduling offset (K0)

maxK2-SchedulingOffset-r16              INTEGER ::= 16      -- Maximum number of slots configured as min. scheduling offset (K2)

maxDCI-2-6-Size-r16                     INTEGER ::= 140     -- Maximum size of DCI format 2-6

maxDCI-2-6-Size-1-r16                   INTEGER ::= 139     -- Maximum DCI format 2-6 size minus 1

maxNrofUL-Allocations-r16               INTEGER ::= 64      -- Maximum number of PUSCH time domain resource allocations

maxNrofP0-PUSCH-Set-r16                 INTEGER ::= 2       -- Maximum number of P0 PUSCH set(s)

maxOnDemandSIB-r16                      INTEGER ::= 8       -- Maximum number of SIB(s) that can be requested on-demand

maxOnDemandPosSIB-r16                   INTEGER ::= 32      -- Maximum number of posSIB(s) that can be requested on-demand

maxCI-DCI-PayloadSize-r16               INTEGER ::= 126     -- Maximum number of the DCI size for CI

maxCI-DCI-PayloadSize-1-r16             INTEGER ::= 125     -- Maximum number of the DCI size for CI minus 1

maxWLAN-Id-Report-r16                   INTEGER ::= 32      -- Maximum number of WLAN IDs to report

maxWLAN-Name-r16                        INTEGER ::= 4       -- Maximum number of WLAN name

maxRAReport-r16                         INTEGER ::= 8       -- Maximum number of RA procedures information to be included in the RA report

maxTxConfig-r16                         INTEGER ::= 64      -- Maximum number of sidelink transmission parameters configurations

maxTxConfig-1-r16                       INTEGER ::= 63      -- Maximum number of sidelink transmission parameters configurations minus 1

maxPSSCH-TxConfig-r16                   INTEGER ::= 16      -- Maximum number of PSSCH TX configurations

maxNrofCLI-RSSI-Resources-r16           INTEGER ::= 64      -- Maximum number of CLI-RSSI resources for UE

maxNrofCLI-RSSI-Resources-1-r16         INTEGER ::= 63      -- Maximum number of CLI-RSSI resources for UE minus 1

maxNrofCLI-SRS-Resources-r16            INTEGER ::= 32      -- Maximum number of SRS resources for CLI measurement for UE

maxCLI-Report-r16                       INTEGER ::= 8

maxNrofConfiguredGrantConfig-r16        INTEGER ::= 12      -- Maximum number of configured grant configurations per BWP

maxNrofConfiguredGrantConfig-1-r16      INTEGER ::= 11      -- Maximum number of configured grant configurations per BWP minus 1

maxNrofCG-Type2DeactivationState        INTEGER ::= 16      -- Maximum number of deactivation state for type 2 configured grants per BWP

maxNrofConfiguredGrantConfigMAC-1-r16   INTEGER ::= 31      -- Maximum number of configured grant configurations per MAC entity minus 1

maxNrofSPS-Config-r16                   INTEGER ::= 8       -- Maximum number of SPS configurations per BWP

maxNrofSPS-Config-1-r16                 INTEGER ::= 7       -- Maximum number of SPS configurations per BWP minus 1

maxNrofSPS-DeactivationState            INTEGER ::= 16      -- Maximum number of deactivation state for SPS per BWP

maxNrofDormancyGroups                   INTEGER ::= 5       --

maxNrofPUCCH-ResourceGroups-1-r16       INTEGER ::= 3       --

maxNrofServingCellsTCI-r16              INTEGER ::= 32      -- Maximum number of serving cells in simultaneousTCI-UpdateList

maxNrofTxDC-TwoCarrier-r16              INTEGER ::= 64      -- Maximum number of UL Tx DC locations reported by the UE for 2CC uplink CA

maxNrofGapId-r17                        INTEGER ::= FFS     -- Maximum number of measurement gap ID

maxNrofGapId-1-r17                      INTEGER ::= FFS     -- Maximum number of measurement gap ID minus 1

maxNrOfGapPri-r17                       INTEGER ::= FFS     -- Maximum number of gap priority level

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

<Skip>

## 11.2	Inter-node RRC messages

### 11.2.1	General

This clause specifies RRC messages that are sent either across the X2-, Xn- or the NG-interface, either to or from the gNB, i.e. a single 'logical channel' is used for all RRC messages transferred across network nodes. The information could originate from or be destined for another RAT.

-- ASN1START

-- TAG-NR-INTER-NODE-DEFINITIONS-START

NR-InterNodeDefinitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ARFCN-ValueNR,

ARFCN-ValueEUTRA,

CellIdentity,

CGI-InfoEUTRA,

CGI-InfoNR,

CSI-RS-Index,

CSI-RS-CellMobility,

DRX-Config,

EUTRA-PhysCellId,

FeatureSetDownlinkPerCC-Id,

FeatureSetUplinkPerCC-Id,

FreqBandIndicatorNR,

GapConfig,

maxBandComb,

maxBands,

maxBandsEUTRA,

maxCellSFTD,

maxFeatureSetsPerBand,

maxFreqIDC-MRDC,

maxNrofCombIDC,

maxNrofPhysicalResourceBlocks,

maxNrofSCells,

maxNrofServingCells,

maxNrofServingCells-1,

maxNrofServingCellsEUTRA,

maxNrofIndexesToReport,

maxSimultaneousBands,

MeasQuantityResults,

MeasResultCellListSFTD-EUTRA,

MeasResultCellListSFTD-NR,

MeasResultList2NR,

MeasResultSCG-Failure,

MeasResultServFreqListEUTRA-SCG,

NeedForGapsInfoNR-r16,

NeedForNCSG-InfoNR-r17,

NeedForNCSG-InfoEUTRA-r17,

OverheatingAssistance,

P-Max,

PhysCellId,

RadioBearerConfig,

RAN-NotificationAreaInfo,

RRCReconfiguration,

ServCellIndex,

SetupRelease,

SSB-Index,

SSB-MTC,

SSB-ToMeasure,

SS-RSSI-Measurement,

ShortMAC-I,

SubcarrierSpacing,

UEAssistanceInformation,

UE-CapabilityRAT-ContainerList,

maxNrofCLI-RSSI-Resources-r16,

maxNrofCLI-SRS-Resources-r16,

RSSI-ResourceId-r16,

SidelinkUEInformationNR-r16,

SRS-ResourceId

FROM NR-RRC-Definitions;

-- TAG-NR-INTER-NODE-DEFINITIONS-STOP

-- ASN1STOP

### 11.2.2	Message definitions

<Skip>

#### –	HandoverPreparationInformation

This message is used to transfer the NR RRC information used by the target gNB during handover preparation or UE context retrieval, e.g. in case of resume or re-establishment, including UE capability information. This message is also used for transferring the information between the CU and DU.

Direction: source gNB/source RAN to target gNB or CU to DU.

HandoverPreparationInformation message

-- ASN1START

-- TAG-HANDOVER-PREPARATION-INFORMATION-START

HandoverPreparationInformation ::=      SEQUENCE {

criticalExtensions                      CHOICE {

c1                                      CHOICE{

handoverPreparationInformation          HandoverPreparationInformation-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

HandoverPreparationInformation-IEs ::=  SEQUENCE {

ue-CapabilityRAT-List                   UE-CapabilityRAT-ContainerList,

sourceConfig                            AS-Config                                       OPTIONAL, -- Cond HO

rrm-Config                              RRM-Config                                      OPTIONAL,

as-Context                              AS-Context                                      OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                     OPTIONAL

}

AS-Config ::=                           SEQUENCE {

rrcReconfiguration                      OCTET STRING (CONTAINING RRCReconfiguration),

...,

[[

sourceRB-SN-Config                      OCTET STRING (CONTAINING RadioBearerConfig)     OPTIONAL,

sourceSCG-NR-Config                     OCTET STRING (CONTAINING RRCReconfiguration)    OPTIONAL,

sourceSCG-EUTRA-Config                  OCTET STRING                                    OPTIONAL

]],

[[

sourceSCG-Configured                    ENUMERATED {true}                               OPTIONAL

]]

}

AS-Context ::=                          SEQUENCE {

reestablishmentInfo                     ReestablishmentInfo                                 OPTIONAL,

configRestrictInfo                      ConfigRestrictInfoSCG                               OPTIONAL,

...,

[[  ran-NotificationAreaInfo            RAN-NotificationAreaInfo                            OPTIONAL

]],

[[  ueAssistanceInformation             OCTET STRING (CONTAINING UEAssistanceInformation)   OPTIONAL   -- Cond HO2

]],

[[

selectedBandCombinationSN               BandCombinationInfoSN                               OPTIONAL

]],

[[

configRestrictInfoDAPS-r16              ConfigRestrictInfoDAPS-r16                          OPTIONAL,

sidelinkUEInformationNR-r16             OCTET STRING                                        OPTIONAL,

sidelinkUEInformationEUTRA-r16          OCTET STRING                                        OPTIONAL,

ueAssistanceInformationEUTRA-r16        OCTET STRING                                        OPTIONAL,

ueAssistanceInformationSCG-r16          OCTET STRING (CONTAINING UEAssistanceInformation)   OPTIONAL,   -- Cond HO2

needForGapsInfoNR-r16                   NeedForGapsInfoNR-r16                               OPTIONAL

]],

[[

configRestrictInfoDAPS-v1640            ConfigRestrictInfoDAPS-v1640                        OPTIONAL

]],

[[

needForNCSG-InfoNR-r17                  NeedForNCSG-InfoNR-r17                              OPTIONAL,

needForNCSG-InfoEUTRA-r17               NeedForNCSG-InfoEUTRA-r17                           OPTIONAL

]]

}

ConfigRestrictInfoDAPS-r16 ::=          SEQUENCE {

powerCoordination-r16                   SEQUENCE {

p-DAPS-Source-r16                       P-Max,

p-DAPS-Target-r16                       P-Max,

uplinkPowerSharingDAPS-Mode-r16          ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic }

}                                                                                                       OPTIONAL

}

ConfigRestrictInfoDAPS-v1640 ::=    SEQUENCE {

sourceFeatureSetPerDownlinkCC-r16   FeatureSetDownlinkPerCC-Id,

sourceFeatureSetPerUplinkCC-r16     FeatureSetUplinkPerCC-Id

}

ReestablishmentInfo ::=             SEQUENCE {

sourcePhysCellId                        PhysCellId,

targetCellShortMAC-I                    ShortMAC-I,

additionalReestabInfoList               ReestabNCellInfoList                            OPTIONAL

}

ReestabNCellInfoList ::=             SEQUENCE ( SIZE (1..maxCellPrep) ) OF ReestabNCellInfo

ReestabNCellInfo::= SEQUENCE{

cellIdentity                            CellIdentity,

key-gNodeB-Star                         BIT STRING (SIZE (256)),

shortMAC-I                              ShortMAC-I

}

RRM-Config ::=              SEQUENCE {

ue-InactiveTime             ENUMERATED {

s1, s2, s3, s5, s7, s10, s15, s20,

s25, s30, s40, s50, min1, min1s20, min1s40,

min2, min2s30, min3, min3s30, min4, min5, min6,

min7, min8, min9, min10, min12, min14, min17, min20,

min24, min28, min33, min38, min44, min50, hr1,

hr1min30, hr2, hr2min30, hr3, hr3min30, hr4, hr5, hr6,

hr8, hr10, hr13, hr16, hr20, day1, day1hr12, day2,

day2hr12, day3, day4, day5, day7, day10, day14, day19,

day24, day30, dayMoreThan30}                            OPTIONAL,

candidateCellInfoList       MeasResultList2NR                                           OPTIONAL,

...,

[[

candidateCellInfoListSN-EUTRA      MeasResultServFreqListEUTRA-SCG                      OPTIONAL

]]

}

-- TAG-HANDOVER-PREPARATION-INFORMATION-STOP

-- ASN1STOP

HandoverPreparationInformation field descriptions

as-Context Local RAN context required by the target gNB or DU.

rrm-Config Local RAN context used mainly for RRM purposes.

sourceConfig The radio resource configuration as used in the source cell.

ue-CapabilityRAT-List The UE radio access related capabilities concerning RATs supported by the UE. A gNB that retrieves MRDC related capability containers ensures that the set of included MRDC containers is consistent w.r.t. the feature set related information.

ue-InactiveTime Duration while UE has not received or transmitted any user data. Thus the timer is still running in case e.g., UE measures the neighbour cells for the HO purpose. Value s1 corresponds to 1 second, s2 corresponds to 2 seconds and so on. Value min1 corresponds to 1 minute, value min1s20 corresponds to 1 minute and 20 seconds, value min1s40 corresponds to 1 minute and 40 seconds and so on. Value hr1 corresponds to 1 hour, hr1min30 corresponds to 1 hour and 30 minutes and so on.

AS-Config field descriptions

rrcReconfiguration Contains the RRCReconfiguration configuration as generated entirely by the MN.

sourceRB-SN-Config Contains the IE RadioBearerConfig as generated entirely by the SN. This field is only used when the UE is configured with SN terminated RB(s).

sourceSCG-Configured Value true indicates that the UE is configured with NR or EUTRA SCG in source configuration. The field is only used in NR-DC and NE-DC and is included only if the fields sourceSCG-NR-Config and sourceSCG-EUTRA-Config are absent.

sourceSCG-EUTRA-Config Contains the current dedicated SCG configuration in RRCConnectionReconfiguration message as specified in TS 36.331 [10] and generated entirely by the SN. In this version of the specification, the E-UTRA RRCConnectionReconfiguration message can only include the field scg-Configuration . This field is only used in NE-DC.

sourceSCG-NR-Config Contains the current dedicated SCG configuration in RRCReconfiguration message as generated entirely by the SN. In this version of the specification, the RRCReconfiguration message can only include fields secondaryCellGroup and measConfig. This field is only used in NR-DC.

AS-Context field descriptions

configRestrictInfoDAPS Includes fields for which source cell explicitly indicates the restriction to be observed by target cell during DAPS handover.

needForGapsInfoNR Includes measurement gap requirement information of the UE for NR target bands.

selectedBandCombinationSN Indicates the band combination selected by SN in (NG)EN-DC, NE-DC, and NR-DC.

sidelinkUEInformationEUTRA This field includes SidelinkUEInformation IE as specified in TS 36.331 [10].

sidelinkUEInformationNR This field includes SidelinkUEInformationNR IE.

ueAssistanceInformation Includes for each UE assistance feature the information last reported by the UE, if any.

ueAssistanceInformationSCG Includes for each UE assistance feature associated with the SCG, the information last reported by the UE in the NR UEAssistanceInformation message for the SCG, if any.

ConfigRestrictInfoDAPS field descriptions

sourceFeatureSetPerUplinkCC/sourceFeatureSetPerDownlinkCC Indicates an index referring to the position of the FeatureSetUplinkPerCC/FeatureSetDownlinkPerCC selected by source in the featureSetsUplinkPerCC/featureSetsDownlinkPerCC.

RRM-Config field descriptions

candidateCellInfoList A list of the best cells on each frequency for which measurement information was available

candidateCellInfoListSN-EUTRA A list of EUTRA cells including serving cells and best neighbour cells on each serving frequency, for which measurement results were available. This field is only used in NE-DC.

| Conditional Presence | Explanation |
| --- | --- |
| HO | The field is mandatory present in case of handover within NR or UE context retrieval, e.g. in case of resume or re-establishment. The field is optionally present in case of handover from E-UTRA/5GC. Otherwise the field is absent. |
| HO2 | The field is optionally present in case of handover within NR; otherwise the field is absent. |

NOTE 1:	The following table indicates per source RAT whether RAT capabilities are included or not.

| Source RAT | NR capabilities | E-UTRA capabilities | MR-DC capabilities | UTRA capabilities |
| --- | --- | --- | --- | --- |
| NR | May be included if UE Radio Capability ID as specified in 23.502 [43] is used for the UE. Included otherwise. | May be included | May be included | May be included, ignored by gNB if received |
| E-UTRAN | May be included if UE Radio Capability ID as specified in 23.502 [43] is used for the UE. Included otherwise. | May be included | May be included | May be included, ignored by gNB if received |

NOTE 2:	The following table indicates, in case of inter-RAT handover from E-UTRA, which additional IEs are included or not:

| Source system | sourceConfig | rrm-Config | as-Context |
| --- | --- | --- | --- |
| E-UTRA/EPC | Not included | May be included | Not included |
| E-UTRA/5GC | May be included, but only radioBearerConfig is included in the RRCReconfiguration. | May be included | Not included |

<Skip>

# Appendix A: Agreements for MGE

## RAN2#116

Pre-configure MG

R2-2111517	Pre-Configured MG (Intel)	Intel

DISCUSSION

At least case 5 is supported for pre-configured gap. FFS for case 4.

- Case 4: NW signals the pre-configured gap (A+B in Q1) via RRC, then UE follows BWP status (B) to activates/deactivates gap upon BWP switching

- Case 5: NW signals the pre-configured gap (A in Q1) via RRC, then UE determines whether the pre-configured gap should be activated or not upon BWP switching.  For example, if it is overlapped with SSB, then pre-configured gap is deactivated, otherwise it is activated.

RAN2 hasn't seen any usefulness of MAC-CE based activation/deactivation and prefers to not support it.

Send LS to RAN4 including the agreements above and to clarify:

- Can FR1 gap and FR2 gap be configured simultaneously for pre-configured gap?

- Can legacy gap and pre-configured gap be configured simultaneously?

Concurrent Gap

R2-2111471	Report of [AT116-e][041][MGE] Concurrent MG (MediaTek)	MediaTek Inc.

RAN2 confirms the following understanding for concurrent gap operation:

- 1. Concurrent gaps are multiple measurement gaps and each gap pattern could be associated with one or multiple frequency layers.

- 2. Each frequency layer can be associated with only one of the concurrent gaps.

- 3. Without considering pre-configured MG, concurrent gaps are always activated if it is setup by the network.

- 4. No new gap pattern is introduced for concurrent gap, the existing R15/R16 gap pattern could be configured for the concurrent gaps.

RAN2 to clarify “frequency layer” and limitations as below:

- PRS measurement can be associated with one gap pattern, no matter how many frequencies are measured for PRS.

- Each measured SSB or LTE frequency is considered as one frequency layer.

- Measured CSI-RS resources with the same center frequency is considered as one frequency layer. It is possible to have Multiple MOs including CSI-RS resources with same center frequency.

- SSB and CSI-RS measurement in one MO are considered as different frequency layers.

For current gap, reply RAN4 LS with the following clarification questions

- Q1 – Could RAN4 confirm the RAN2 understanding above (P1 to P2)?

- Q2 – Could concurrent gap be configured together with legacy gap (i.e. gap without associated frequency layer(s))? Could some of the concurrent gaps be configured without associated frequency layer? If yes, how does UE use the concurrent gaps together with gap without associated frequency layer?

- Q3 – How many number of concurrent gap could be configured?

- Q4 – Could concurrent gaps be configured with different gap types (i.e. some gaps are per-UE while some gaps are Per-FR)?

- Q5 – The impact to gap sharing configuration (MeasGapSharingConfig) due to concurrent gap is unclear to RAN2. Should we also have multiple gap sharing configuration?

- Q6 – ask about applicability to UTRA

R2-2111472	Reply LS on R17 NR MG enhancements – Concurrent MG	RAN2	LS out

[041] approved

## RAN2#116bis

General

From RRC signaling design, RAN2 aim to support joint working among Pre-MG, concurrent gaps, and NCSG

For all the 3 objectives in MG enh. WI, RAN2 prioritize the design in NR SA.

Pre-configure MG

Add 1 bit indication in gapConfig to indicate pre-configured measurement gap.

In case of simultaneous support of legacy gap and pre-configured gap, it is agreed to support option 2: combine concurrent gap to indicate pre-configured gap if both are enabled.

RAN2 confirms that MAC-CE based activation/deactivation for pre-configured MG is NOT supported for MGE WI.

Concurrent Gap

R2-2201672	[Pre116bis][012][MGE] Summary of 8.22.3 Multiple concurrent and independent MG patterns (MediaTek)	MediaTek Inc.

Introduce multiple gap configuration in IE MeasGapConfig (i.e. by configuring multiple GapConfig).

- FFS Whether to use ToAddModList and ToReleaseList structure

- FFS to add gap ID in GapConfig

FFS if In addition to the per frequency layer association in P3, define ASN.1 for per use case (e.g. PRS, SSB, CSI-RS, EUTRA) association with concurrent gaps.

RAN2 don’t supports concurrent gap association to 3G/2G from signalling perspective, but the signalling shall be extendable if this need to be introduced.

For association between concurrent MG and measured frequencies: Indicate the associated gaps (via “gap ID”) in MO; (for PRS measurement, indicating in the association in MG configuration).

NCSG

R2-2201678	Summary of AI 8.22.4 Network Controlled Small Gap (Apple)	Apple

DISCUSSION

Can work offline on LS out.

Re-use the Rel-16 NeedForGap reporting like procedure for NCSG reporting:

- - UE indicates capability on NCSG support in UE capability reporting (FFS on UE capability reporting details).

- - NW configures the NCSG reporting in RRCReconfiguration and RRCResume message.

- - UE reports the NCSG capabilities in RRCReconfigurationComplete and RRCResumeComplete messages.

Agree that NCSG can be configured as per UE, (per FR1 and per FR2 patterns is FFS).

FFS if  NCSG patterns corresponding to legacy patterns #0 and #1 are mandatorily supported if UE supports NCSG. And to further discuss UE capability between reporting an indicator of NCSG feature support and reporting supported NCSG patterns

Detailed design Same as Rel-16 NeedForGap, support NCSG reporting for both intra-frequency and inter-frequency. FFS Inter RAT

## RAN2#117

Pre-configured MG

RAN2 introduces support of NW-Controlled activation/deactivation pre-configured gap

RAN2 agree to support of CA scenario for pre-configured gap.

RAN2 agree to support in signalling CA scenario for pre-configured MG with no limitation on BWP switching operation as in legacy. (not limited to only single CC switch at a time)

Network configures per BWP per MG status for each CC to support CA scenario for Network-controlled activation/ deactivation support of pre-configured MG.

RAN2 can support UE autonomous activation/deactivation mechanism without specification impact under CA case if the UE combines rule will be in RAN4 spec.

If network-controlled activation/deactivation (explicit indicator) is provided, UE follow the explicit status indicator and does not use implicit rule (UE autonomous activation/deactivation).

The activation/deactivation status of the specific BWP is included in the configuration of BWP

On R2-2203845:

For first part RAN2 assumes triggering can be handled by network impl, UE combination may need to be reflected, discuss this in the RRC discussion.

Separate offline discussion on last part, if we decide to not challenge the second part in this offline it can be part of the RRC CR.

Concurrent MG

For additional gap configuration in concurrent gap, use ToAddModList and ToReleaseList structure for each gap type to add or release the additional gaps, and gap sharing configuration to be consistent.

For concurrent gap, RAN2 confirms that there is no need to support coarse granularity association (i.e. per use case such as CSI-RS, SSB measurement) since the agreed fine granularity (per frequency layer) could cover this case.

FFS the maximum number of measurement gap ID. This could be discussed in gap coordination section.

Baseline assumption When multiple MOs (with the same SSB frequency) are configured, the network associates the same MG for the SSB measurement in each MO. Details sorted out in CR disc

NCSG

RAN2 confirms that reporting of NCSG for E-UTRA target bands is supported. RAN2 assumes that support for EUTRA target bands can be a separate UE cap

RAN2 confirms that NCSG could be configured as per FR gap.

Reuse the legacy GapConfig with some extension for NCSG gap configuration.

RAN2 to support enabling derivation of SSB indexes of target cell on inter-frequency from serving cell timing.

Introduce one new field (e.g, deriveSSB-IndexFromCell-Inter-r17) which refers to IE ServCellIndex, into SSB-ConfigMobility inside MeasObjectNR.

Do not need to support the optimized RRC configuration to allow NW to indicate the new field (e.g, deriveSSB-IndexFromCell-Inter-r17) even if the MO is regarded as intra-frequency MO.

Do not introduce the new field (e.g, deriveSSB-IndexFromCell-Inter-r17) into SIB4/RRCRelease message.

Proposal 6: To support independent Rel-17 NCSG reporting from Rel-16 NeedForGap reporting.

Proposal 7: There is no need to allow simultaneous configurations on Rel-16 NeedForGap and Rel-17 NCSG reporting.

Proposal 8: Agree to introduce R17 NCSG information into inter-node HandoverPreparationInformation message.

UE Capability

introduce (A) UE capability to support NCSG as indicated in RAN4 feature list as a baseline. It can be removed if no longer needed after more input from RAN4 on (B) UE capability to support NCSG pattern.

introduce 1 bit UE capability to support concurrent gap.

introduce separate UE capability for pre-configured measurement gap as follow:

- Pre-configured measurement gap with network-controlled activation and deactivation mechanism

- Pre-configured measurement gap with UE autonomous activation and deactivation mechanism

Wait for more input from RAN4 on (B) UE capability to support NCSG pattern.

FFS additional UE capability for support perUE concurrent gap for index 2 only in addition to concurrent gap.

FFS on if CA and non-CA case should have separate UE capability.

FFS pre-configured gap should be FR differentiated.

# Appendix B: Remaining FFS Issues

Pre-MG

FFS the reference section in R4 to determine the gap status

FFS whether and how to capture the UE behavior on PRS measurements within measurement gaps when a Pre-configured MG is provided by the network (as indicated in RAN4 LS R4-2206789).

Concurrent MG (and gap coordination)

FFS whether this gap Id is also needed for feature other than concurrent gap

FFS on maximum number of gap priority

FFS on maximum number of gap ID

NCSG

N.A.

UE Capabiltiy

FFS whether pre-configured gap should be FR differentiated.

FFS whether to have separate bit to indicate support of CA and non-CA case for pre-configured gap

FFS whether to have separate capability bit for UE supporting only two per UE concurrent gap.