3GPP TSG-RAN2 Meeting # 117-e	R2-220xxx

# Online, 21 Feb - 3 Mar, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.300 CR for Introduction of QoE measurements in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-Core |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CR for introduction of QoE measurements in NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR captures the agreements made since RAN2#113b-e meeting. The CR introduces stage-2 description of Application Layer Measurement Collection feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QoE measurements will not be defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.331 CRxxxx TS 38.306 CRxxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

3	Abbreviations and Definitions

3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

GFBR	Guaranteed Flow Bit Rate

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

LDPC	Low Density Parity Check

MDBV	Maximum Data Burst Volume

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MT	Mobile Termination

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

NB-IoT	Narrow Band Internet of Things

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QoE	Quality of Experience

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TPC	Transmit Power Control

TRP	Transmit/Receive Point

UCI	Uplink Control Information

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

V2X	Vehicle-to-Everything

VR	Virtual Reality

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

<Next modification>

X	Application Layer Measurement Collection

X.1	Overview

This function enables collection of application layer measurements from the UE. The supported service types are:

-	QoE Measurement Collection for streaming services;

-	QoE Measurement Collection for MTSI services.

-	QoE Measurement Collection for VR services.

Both signalling based and management based QoE measurement collection are supported.

X.2	Configuration aspects

X.2.1	General

The application layer measurement configuration and measurement reporting are supported in RRC_CONNECTED state only. Application layer measurement configuration received by the gNB from OAM or CN is encapsulated in a transparent container, which is forwarded to a UE in the RRCReconfiguration message (there can be multiple configurations in the same message). Application layer measurement reports received from UE's higher layer are encapsulated in a transparent container and sent to the network in the MeasurementReportAppLayer message, as specified in TS 38.331 [12]. The UE can send multiple application layer measurement reports to the gNB in one MeasurementReportAppLayerRRC message. In order to allow the transmission of application layer measurement reports which exceed the maximum PDCP SDU size, segmentation of the MeasurementReportAppLayer message can be enabled by the gNB.Uplink RRC segmentation is optionally supported for transmission of application layer measurement report message. An RRC identifier conveyed in the RRC signalling is used to identify the application layer measurement configuration and report between the gNB and the UE. The RRC identifier is also passed from the UE’s AS layer to the UE’s higher layer together with associated transparent container of the application layer measurement configuration, and from the UE’s higher layer to the UE’s AS layer together with the associated transparent container of the application layer measurement report. The RRC identifier is mapped to the QoE Reference in the gNB. The application layer measurement report is forwarded to OAM together with the QoE Reference. gNB can release one or multiple application layer measurement configurations from the UE in one RRCReconfiguration RRC message at any time. The UE may additionally be configured by the gNB to report when a QoE measurement session starts or stops for a certain application layer measurement configuration. If there already is an ongoing QoE measurement session in the UE when the gNB configures the UE to report when the session starts or stops, the UE transmits a session start indication directly upon configuration.

Upon reception of QoE release command in an application layer measurement configurationmessage, the UE discards any unsent application layer measurement reports corresponding to the released application layer configuration. The UE discards the reports received from application layer when it has no associated application layer measurement configuration configured.

X.2.2	Pause and resume

gNB can use the RRCReconfiguration message to temporarily stop application layer measurement reports associated to one or multiple application layer measurement configurations from being sent from the UE to the gNBnetwork. When the UE receives the QoE pause indication, UE temporarily stores application layer measurement reports in AS layer. When the UE receives the QoE resume indication, UE sends the stored application layer measurement reports to the gNBnetwork.

The UE keeps on reporting the configured RAN visible application layer measurements, even though the corresponding non RAN visible application layer measurement reporting is paused.

X.2.3	Measurement handling in RRC_IDLE and RRC_INACTIVE

If the UE enters RRC_INACTIVE, the UE AS configuration for the QoE is stored in the UE Inactive AS context.

If the UE enters RRC_IDLE state, the UE releases all the application layer measurement configurations.

X.2.4	Application layer measurement configuration handling during mobility

For signalling based QoE, at handover to a target gNB which supports QoE, the target gNB decides which application layer measurement configurations to keep and which to release, e.g. based on application layer measurement configuration information received from the source gNB in Xn/NG signalling.

When the UE resumes the connection in a gNB supporting QoE, the UE keeps application layer measurement configurations indicated by the target gNB and releases the application layer measurement configurations which are not indicated by the target gNB for restoration. When the UE resumes the connection in a gNB not supporting QoE, the UE releases all application layer measurement configurations.

X.3	RAN visible application layer measurement

RAN visible application layer measurement is supported only for streaming and VR services. The gNB can use RAN visible application layer measurement configurations to instruct the UE to collect application layer measurements for RRM purposes.The RAN visible application layer measurement configuration utilizes explicit RRC IEs. Multiple simultaneous RAN visible application layer measurements configuration and reports can be supported for RAN visible application layer measurement, and each RAN visible application layer measurement configuration and report is identified by the same RRC identifier as the application layer measurement configuration and measurement report. gNB configures the required RAN visible QoE metrics in the RAN visible application measurement configuration for the UE to report. After receiving the RAN visible application layer measurement configuration, the UE RRC layer forwards the configuration to the application layer, indicating the service type and the RRC identifier. RAN visible application layer configuration can only be configured if there is a corresponding application layer measurement configuration for the same service type configured at the UE. The application layer sends the RAN visible application layer measurement report associated with the RRC identifier to the UE’s AS layer. UE can send both RAN visible application layer measurement reports and the application layer measurement reports to the gNB in the same MeasurementReportAppLayer message one RRC message. gNB can release one or multiple RAN visible application layer measurement configurations from the UE in one RRC message at any time. gNB shall release RAN visible application layer measurement configuration if the application layer measurement configuration with the same identifier is released.

The UE keeps on reporting the configured RAN visible application layer measurements, even though the corresponding non RAN visible application layer measurement reporting is paused.

Annex	RAN2 agreements on NR QoE

RAN2 agreements for information, which will be removed from final CR.

A.1 Configuration architecture general aspect

RAN2#117-e Agreements

1-bit indication added in the MeasurementReportAppLayer message is used to indicate session start/stop for each QoE configuration, sent with Meas ID (as other reports)

Indication of Session start/stop is configurable per QoE configuration.

RAN2#116b-e Agreements

Send LS to SA4 to explain that with RRC segmentation the max container size (for the report container) can be different and can change by AS reconfigurations. Ask whether the application can/would take this into account and whether this need explicit indication.

Mulitple QoE reports can be sent in one MeasurementReportAppLayer message.

There can be both multiple QoE reports with different measConfigAppLayerId and multiple QoE reports with the same measConfigAppLayerId in the MeasurementReportAppLayer message.

The maximum size of the QoE configuration container is specified as a maximum size 8000 (Bytes) of the OCTET STRING in ASN.1.

No max size of the OCTET STRING for the QoE report container is specified in ASN.1.

Send a reply LS to SA4 with the RAN2 agreements related to RRC segmentations and container size limitations.

Inform CT1 that the service type does not need to be forwarded to the application layer at release.

Inform CT1 that the QoE configurations can be configured as a list in NR and ask them to take this into account when specifying the AT-command.

Inform CT1 that all QoE configurations may need to be released without any measConfigAppLayerId being indicated from the AS-layer and ask them to take this into account when specifying the AT-command.

Send an LS to CT1 and inform them of the RAN2 agreements with impact on AT-commands.

RAN2#116-e Agreements

Forward the measConfigAppLayerId from the AS layer to the application layer together with the QoE configuration.

Forward the measConfigAppLayerId from the application layer to the AS layer together with the QoE report.

Support RRC segmentation for the Reporting

Reply to SA4 that the size limitation of the QoE report has chanegd. RAN2 has agreed to optionally support RRC segmentation for transmission of QoE reports, and we indicate the new limits

Size limit of QoE configuration = size of one PDCP SDU.

Inform CT1 and SA4 of these agreements and ask them to specify the measConfigAppLayerId (e.g. in AT command). Can also discuss whether we need to have an action related to size limitation (whether to inform application of the size that is supported).

FFS if to allow multiple QoE reports in the same RRC message, but leave it to UE.

RAN2#115-e Agreements

It is the RAN2 understanding that the QoE Reference does not need to be sent to or from the UE in RRC signaling for QoE measurements in RRC_CONNECTED. The RRC ID, MeasConfigAppLayerId, is sufficient to identify the QoE configuration between UE and gNB.

RAN2 assumes that gNB keeps the mapping between MeasConfigAppLayerId and QoE Reference. The mapping is sent to the target gNB as part of QoE configuration and information at handover.

Send an LS to SA5 (cc R3) to confirm proposals (agreements) 1 and 2.

FFS if the RRC layer forwards the MeasConfigAppLayerId together with the QoE configuration to the application layer.

Confirm that RAN2 deprioritizes QoE measurement in RRC_IDLE/RRC_INACTIVE in Rel-17.

Send an LS to SA5 for confirmation of max number of QoE configurations per UE. Number 8 could be assumed, to be finally concluded offline. R2 has not concluded the max no of QoE configs per UE, numbers in the range 8 - 64 are discussed. The outgoing LS R2-2109200 is approved.

RAN2#114-e Agreements

gNB can release a list of QoE measurement configurations in one RRCReconfiguration message.

If a QoE measurement configuration is released, RRC layer informs the upper layer to release the QoE measurement configuration. This could be revisited based on other issues’ progress.

If the UE enters IDLE state, UE should release all of the QoE measurement configurations.

QoE configuration and report are encapsulated in a transparent container in the RRC messages. It is FFS for RAN-visible QoE configuration and report (dep on R3).

At lease service type and RRC level ID (Reference ID or shorten ID) together with corresponding QMC configuration container should be included for each QoE configuration in RRCReconfiguration message when the network setups QoE measurement to the UE.

At least RRC level ID (Reference ID or shorten ID) together with corresponding QMC report container should be included in MeasReportAppLayer message for each QoE report.

RAN2 confirms logged MDT framework for QoE data retrieval and reporting is not supported in Rel-17.

RAN2 assumes that QoE configuration modification does not need to be supported from RAN2 signalling point of view (in RRC), and send LS to SA5/SA4 to confirm the assumption.

Send LS to SA4/SA5/RAN3 ask whether multiple QoE measurement configurations can be configured for a certain service type.

RAN2 assumes to re-use the maximum container size of 1000 bytes for QoE measurements configuration and send LS to SA4 to confirm the assumption.

Send LS to SA4 to check the necessity of the maximum container size of QoE measurements report beyond than 8000 bytes.

RAN2#113b-e Agreements

Configure QoE measurements for NR in RRCReconfiguration.

Add configuration of QoE measurements in OtherConfig in RRCReconfiguration.

Add the configuration of QoE measurements by means of list to enable configuration of multiple simultaneous measurements.

R2 assumes that for RRC an ID is required to identify a measurement, FFS whether this is the QoE reference ID or something else.

Define SRB4 for transmission of QoE reports in NR.

Define an RRC message MeasReportAppLayer for the transmission of QoE reports in NR.

RAN2 assumes that QoE support for NR includes (as the LTE framework): activation by Trace Function, both signalling and management-based configuration and RRC procedures supporting AppLayer config and report.

From RAN2 point of view, the UE shall follow gNB commands and, NG-RAN can in principle release by RRC the application layer measurement configuration towards the UE at any time, e.g. if required due to load or other reasons (Note that other WGs are responsible to define the normal system procedures for release and which nodes are responsible etc).

The UE Inactive AS context includes the UE AS configuration for the QoE (it is not released when UE goes to Inactive).

A.2 Start and stop

RAN2#117-e Agreements

Pause Resume is not applicable to RVQoE

The UE keeps stored QoE reports (while in Paused state) when going to RRC_INACTIVE if the UE also keeps the AS QoE configuration. If or when the configuration is released, then stored QoE reports if any are discarded.

RAN2#116b-e Agreements

AS layer is responsible for storing QoE reports when the UE receives QoE pause indication at RAN overload (overrides earlier decisions)

There is no need for interaction between AS and Application for Pause Resume (overrides earlier decisions)

The minimal memory size of QoE paused measurements report is 64KB

At RAN overload scenarios, when the memory reserved for the QoE paused measurements becomes full, the UE is allowed to discard extra QoE paused measurements report. The action of how UE AS layer discards extra QoE paused measurements report is based on UE implementation.

When the UE receives QoE resume indication after RAN overload, AS layer should send the stored QoE paused measurements report to the RAN.

RAN2#116-e Agreements

We go with selective pause resume (with the understanding that we will not work further on the information the gNB may use for election).

RAN2#115-e Agreements

No progress and no contributions were treated.

RAN2#114-e Agreements

At reception of QoE release, the UE shall discard any unsent QoE reports corresponding to the released QoE configuration.

FFS whether pause resume will affect all configurations or whether pause resume can act selectively per configuration.

On whether to store reports in the AS or the application layer at Pause, Send LS to SA4/SA5/SA3 to inform them about the options and their pros/cons (if possible) and ask them for feedback. RAN2 will continue work on this topic based on the feedback received.

RAN2#113b-e Agreements

“QoE pause” indication from the network is used to temporarily stop QoE reports from being sent from the UE to the network. Application layer behaviour upon UE receiving “pause/resume” indications is out of RAN2 scope.

The following are options considered by RAN2 for QoE report handling during RAN overload via “QoE report pause indication”:

Option 1: Application layer is responsible for storing QoE reports when the UE receives QoE pause indication.

Option 2: AS layer is responsible for storing QoE reports when the UE receives QoE pause indication.

Option 3: The QoE container received from application layer is discarded during pause.

A.3 Support for Mobility

RAN2#116b-e Agreements

Upper layers are informed of the release of the application layer measurements at RRCSetup (can be done if RRC setup is provided as a response to RRCresumerequest or RRC reestablishmentrequest).

At Resume with delta configuration the network indicates possible differences to the QoE configurations.

At mobility with fullConfig, upper layers are informed of the release of the application layer measurements if no measConfigAppLayerId is indicated by the network.

Except for restarts transmission of QoE reports after handover, The TP in the Annex of R2-2200011 is included in the running CR for QoE measurements.

RAN2#115-e Agreements

RAN2 assumes that all QoE mobility related agreements made by RAN2 are applicable at least to signalling based QoE. Whether the same applies to management-based QoE is pending further input from SA5 and RAN3.

Area scope parameter is not introduced in RRC procedures supporting QoE.

When the UE resumes the connection in a gNB supporting QoE, the target gNB should explicitly indicate which QoE measurement configurations should be kept by the UE during RRC resume procedure, e.g. in RRCResume message. The UE shall release all QoE measurement configurations not indicated by the gNB for restoration. FFS how the indication looks like, e.g. granularity per QoE configuration or common for all QoE configurations.

During the handover to target gNB which supports QoE, the target gNB decides which QoE configurations to keep and which to release during a handover, e.g. based on QoE configuration information received from the source gNB in Xn/Ng signalling (exact information is up to RAN3) including the RRC container.

The UE discards the reports received from application layer in case it has no associated QoE configuration configured.

FFS whether the gNB needs to know the QoE configurations for which there are ongoing QoE sessions, e.g. to enable QoE configuration handling upon mobility (pending SA4 reply on the ongoing QoE measurement session continuity requirement).

In case the UE resumes the connection in a gNB not supporting QoE, the UE should release all QoE measurement configurations.

A.4 RAN visible QoE

RAN2#117-e Agreements

SRB4 is used to transmit RAN visible QoE measurements.

RAN2#116b-e Agreements

On RVQoE metrics reporting, RAN2 arrived at the following possible assumptions as starting points.

- Assumption 1a: RAN2 specifies the maximum number of buffer level entries (ASN.1 value) for each buffer level metric report in one reporting message.

- Assumption 1c: It is UE implementation on which buffer level entries should be reported for each buffer level metric report when the received number of buffer level entries exceeds the maximum number.

- Assumption 2a: The parameter “t” is not reported for each buffer level entry.

- Assumption 2b: It is expected that application layer does not send parameter “t” to AS layer.

- Assumption 3: Taking the granularity 10ms for level value as baseline, i.e. integer value 1 correspnds to 10ms, value 2 corresponds to 20ms, and so on.

- Assumption 4a: Taking the maximum value of 5min as baseline for level value range.

- Assumption 4b: UE sets the value to 5min if the received level value is more than 5min.

- Assumption 5: Taking the maximum value 30 seconds as baseline for playout delay for media startup value range.

- Assumption 6: Taking the granularity 1ms as baseline for playout delay, i.e. integer value 1 correspnds to 1ms, value 2 corresponds to 2ms, and so on.

Send LS to SA4 and to RAN3 about the above assumptions, and also indicate that RAN2 doesn't consider itself as the main responsible group for definition of RV QoE metrics, so the decision whether to use these assumptions is in the hands of the receiving group(s). Can also include other agreements on RV QoE.

RVQoE configuration can share the same measConfigAppLayerId and service type RRC IEs with legacy QoE configuration.

Modification of RVQoE configuration can be supported from RRC layer point of view, it can be revisited if any problem according to further stage 3.

RAN2 confirm it is feasible that NG-RAN can release a list of RAN visible QoE configurations while not releasing the corresponding legacy QoE configuration and if the corresponding legacy QoE configuration is released, the RAN visible QoE configuration is released as well.

RVQoE measurements can be included into MeasurementReportAppLayer message.

MeasConfigAppLayerId can be used to identify both of associated legacy QoE report and RVQoE report, and it is irrespective whether RVQoE should be reported independently or together with legacy QoE.

Multiple RVQoE reports can be included in one MeasurementReportAppLayer message, and can be revisited according to legacy QoE reporting progress.

The above agreements uses somewhat incorrectly the word “legacy” to denote the non-RAN-Visible QoE (in this release). Note that the word legacy is forbidden in TSes.

RAN2#116-e Agreements

RAN2 assumes that RAN2 is responsible to define the procedure to support RVQOE configuration and reporting, and leave the definition of RAN QoE metrics and what should be included in RVQOE configuration and report to other WGs, e.g. RAN3, SA4.

RAN2 confirms the following is feasible from RAN2 point of view.

It is feasible to configure RVQOE using explicit RRC IEs

Multiple simultaneous QoE measurements can be supported for RVQOE.  Each RVQOE measurement configuration is identified by the MeasConfigAppLayerId (or change to another generic term) corresponding to the regular QoE configuration.

UE RRC layer forwards the received RVQOE configuration to the upper (application) layer, indicating the service type.

RAN configures the required RVQOE metrics in the RVQOE configuration for UE to report.

A.5 UE capabilities

RAN2#117-e Agreements

A parameter per service type indicating whether UE supports RAN visible QoE capability.

RAN2 assumes that No UE capability parameters of the alignment of QoE and MDT need to be introduced.

RRC segmentation capability can be optional with UE capability parameter (one extra bit).

R2 assumes Pause and resume capability is one of basic sub-features of QoE. (This may be revisited in Q2, if UE vendors find that this requirement is a blocker for wide deployment of QoE reporting).

RAN2 assumes that AS layer capability will be indicated to network only if the UE is capable also on higher layers

RAN2 assumes that how AS layer obtain application capability is based on UE implementation (with no AS spec impact).

RAN2 send LS to SA4 (and cc CT1), can elaborate on detailed Questions offline, if needed.

RAN2#116b-e Agreements

For QoE capable UE, Mandatory to support 16 QoE configs (signalling limitation), include this info in LS out to SA4.

Introduce QoE UE capability parameters for each service type i.e., streaming, MTSI and VR.

Introduce UE capability parameter(s) for RAN visible QoE.

Introduce a new sub-section in TS 38.306 to capture QoE related capabilities.

Agree that no differentiation for FDD/TDD or FR1/FR2 is needed for QoE related capabilities.

FFS on whether the Pause and resume capability is one of basic sub-features.

FFS on which of the following option to choose for RVQoE capability,

- Option 1: One parameter indicating whether UE supports RVQoE.

- Option 2: Separate parameters indicating whether UE supports RVQoE for each service type.

FFS on RRC segmentation capability for QoE report, and the following three directions are considered:

- Option 1: Conditional mandatory without UE capability parameter (no extra bit)

- Option 2: Optional without UE capability parameter (no extra bit)

- Option 3: Optional with UE capability parameter (one extra bit)