3GPP TSG-RAN WG2 Meeting #117-e		R2-220xxxx

Electronic, 21st Feb. – 3rd Mar. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0227 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of ePowSav in TS 38.304 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-03-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To capture agreements for ePowSav in NR into TS 38.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of ePowSav features in NR. This CR captures the idle/inactive aspects of enhanced power saving and it is based on the RAN2/RAN1 agreements made so far, which could be found in Annex at the end of this document. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ePowSav is not supported in NR in TS 38.304. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 7.x, 7.y, Z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.331 CR TBD TS/TR 38.306 CR TBD TS/TR 38.300 CR TBD |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the updated version of running CR for TS 38.304 for ePowSav WI considering conclusions from RAN1#108-e and RAN2#117-e. |  |  |  |  |  |  |  |  |

Start of change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AS	Access Stratum

CAG	Closed Access Group

CAG-ID	Closed Access Group Identifier

CMAS	Commercial Mobile Alert System

CN	Core Network

DCI	Downlink Control Information

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

IMSI	International Mobile Subscriber Identity

MCC	Mobile Country Code

MICO	Mobile Initiated Connection Only

NAS	Non-Access Stratum

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

PEI	Paging Early Indication

PEI-O	Paging Early Indication-Occasion

PLMN	Public Land Mobile Network

RAT	Radio Access Technology

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RRC	Radio Resource Control

SNPN	Stand-alone Non-Public Network

TRS	Tracking Reference Signal

UAC	Unified Access Control

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

V2X	Vehicle to Everything

Next change

# 7	Paging

## 7.1	Discontinuous Reception for paging

The UE may use Discontinuous Reception (DRX) in RRC_IDLE and RRC_INACTIVE state in order to reduce power consumption. The UE monitors one paging occasion (PO) per DRX cycle. A PO is a set of PDCCH monitoring occasions and can consist of multiple time slots (e.g. subframe or OFDM symbol) where paging DCI can be sent (TS 38.213 [4]). One Paging Frame (PF) is one Radio Frame and may contain one or multiple PO(s) or starting point of a PO.

In multi-beam operations, the UE assumes that the same paging message and the same Short Message are repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the paging message and Short Message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.

The UE initiates RRC Connection Resume procedure upon receiving RAN initiated paging. If the UE receives a CN initiated paging in RRC_INACTIVE state, the UE moves to RRC_IDLE and informs NAS.

The PF and PO for paging are determined by the following formulae:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

The PDCCH monitoring occasions for paging are determined according to pagingSearchSpace as specified in TS 38.213 [4] and firstPDCCH-MonitoringOccasionOfPO and nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured as specified in TS 38.331 [3]. When SearchSpaceId = 0 is configured for pagingSearchSpace, the PDCCH monitoring occasions for paging are same as for RMSI as defined in clause 13 in TS 38.213 [4].

When SearchSpaceId = 0 is configured for pagingSearchSpace, Ns is either 1 or 2. For Ns = 1, there is only one PO which starts from the first PDCCH monitoring occasion for paging in the PF. For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF.

When SearchSpaceId other than 0 is configured for pagingSearchSpace, the UE monitors the (i_s + 1)th PO. A PO is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH monitoring occasions for paging which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF. When firstPDCCH-MonitoringOccasionOfPO is present, the starting PDCCH monitoring occasion number of (i_s + 1)th PO is the (i_s + 1)th value of the firstPDCCH-MonitoringOccasionOfPO parameter; otherwise, it is equal to i_s * S*X. If X > 1, when the UE detects a PDCCH transmission addressed to P-RNTI within its PO, the UE is not required to monitor the subsequent PDCCH monitoring occasions for this PO.

NOTE 1:	A PO associated with a PF may start in the PF or after the PF.

NOTE 2:	The PDCCH monitoring occasions for a PO can span multiple radio frames. When SearchSpaceId other than 0 is configured for paging-SearchSpace the PDCCH monitoring occasions for a PO can span multiple periods of the paging search space.

The following parameters are used for the calculation of PF and i_s above:

T: DRX cycle of the UE (T is determined by the shortest of the UE specific DRX value(s), if configured by RRC and/or upper layers, and a default DRX value broadcast in system information. In RRC_IDLE state, if UE specific DRX is not configured by upper layers, the default value is applied).

N: number of total paging frames in T

Ns: number of paging occasions for a PF

PF_offset: offset used for PF determination

UE_ID: 5G-S-TMSI mod 1024

Parameters Ns, nAndPagingFrameOffset, nrofPDCCH-MonitoringOccasionPerSSB-InPO, and the length of default DRX Cycle are signaled in SIB1. The values of N and PF_offset are derived from the parameter nAndPagingFrameOffset as defined in TS 38.331 [3]. The parameter first-PDCCH-MonitoringOccasionOfPO is signalled in SIB1 for paging in initial DL BWP. For paging in a DL BWP other than the initial DL BWP, the parameter first-PDCCH-MonitoringOccasionOfPO is signaled in the corresponding BWP configuration.

If the UE has no 5G-S-TMSI, for instance when the UE has not yet registered onto the network, the UE shall use as default identity UE_ID = 0 in the PF and i_s formulas above.

5G-S-TMSI is a 48 bit long bit string as defined in TS 23.501 [10]. 5G-S-TMSI shall in the formulae above be interpreted as a binary number where the left most bit represents the most significant bit.

## 7.x	PEI

### 7.x.1	PEI reception

The UE may use PEI in RRC_IDLE and RRC_INACTIVE states in order to reduce power consumption. If PEI configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state supporting PEI can monitor PEI using PEI parameters in system information according to the procedure described below.

If lastUsedCellOnly is configured in system information, the UE will use PEI only in the cell in which the UE most recently entered RRC_IDLE or RRC_INACTIVE states. Otherwise, the UE will use PEI in the cell regardless of which cell the UE most recently entered RRC_IDLE or RRC_INACTIVE states.

The UE monitors one PEI occasion per DRX cycle. A PEI occasion (PEI-O) is a set of PDCCH monitoring occasions and can consist of multiple time slots (e.g. subframe or OFDM symbol) where PEI can be sent (TS 38.213 [4]).

The time location of PEI-O for UE's PO is determined by a reference point and an offset from the reference point to the start of the first PDCCH monitoring occasion of this PEI-O:

- The reference point is the start of a reference frame determined by a frame-level offset from the start of the first PF of the PF(s) associated with the PEI-O, provided by PEI-F_offset and configured via SIB for the cell;

- The offset is a symbol-level offset from the reference point to the start of the first PDCCH MO of PEI-O, provided by firstPDCCH-MonitoringOccasionOfPEI-O and configured via SIB for the cell.

If one PEI-O is associated with POs of two PFs, the two PFs are consecutive PFs calculated by the parameters PF_offset, T, Ns, and N configured via SIB. The first PF of the PFs associated with the PEI-O is provided by ((SFN for PF) - floor (iPO/Ns)*T/N), where SFN for PF is determined in clause 7.1, iPO = ((UE_ID mod N) * Ns + i_s) mod POnumPerPEI, and POnumPerPEI is configured via SIB.

The PDCCH monitoring occasions for PEI are determined according to pei-SearchSpace as specified in TS 38.213 [4], PEI-F_offset, firstPDCCH-MonitoringOccasionOfPEI-O and nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured as specified in TS 38.331 [3].

When SearchSpaceId = 0 is configured for peiSearchSpace for the case of CORESET multiplexing pattern 2 or 3, the UE monitors the PEI-O according to searchSpaceZero. When SearchSpaceId other than 0 is configured for peiSearchSpace, the UE monitors the PEI-O according to the SearchSpace of the configured SearchSpaceId.

A PEI occasion is a set of 'S*X' consecutive PDCCH monitoring occasions, where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for PEI in the PEI occasion corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH monitoring occasions for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for PEI in the PEI-O. If X > 1, when the UE detects a PEI within its PEI-O, the UE is not required to monitor the subsequent monitoring occasion(s) associated with the same PEI-O.

Editor’s NOTE:	The detailed parameters for PEI reception will be aligned with RRC specification.

If the UE detects PEI and the PEI indicates that the UE (or the subgroup the UE belongs to, as specified in clause 7.y) to monitor the associated PO, as specified in clause 10.4a in TS 38.213 [4], the UE monitors the associated PO as specified in clause 7.1. If the UE does not detect PEI on the monitored PEI occasion or the PEI does not indicate the UE (or the subgroup the UE belongs to, as specified in clause 7.y) to monitor the associated PO, as specified in clause 10.4a in TS 38.213 [4], the UE is not required to monitor the associated PO as specified in clause 7.1.

If the UE is unable to monitor the PEI occasion (i.e. all valid PDCCH monitoring occasion for PEI) corresponding to its PO, e.g. during cell re-selection, the UE monitors the associated PO according to clause 7.1.

## 7.y	Subgrouping

### 7.y.0	General

If subgroupConfig is configured, UEs monitoring the same PO can be divided into one or more subgroups. With subgrouping, the UE (except UEs expecting multicast service start notification) monitors PO if the corresponding bit for subgroup the UE belongs to is indicated as 1 by PEI corresponding to its PO, as specified in clause 10.4a in TS 38.213 [4]. UE's subgroup can be either assigned by CN as specified in clause 7.y.1 or determined based on UE_ID as specified in clause 7.y.2:

-	If subgroupsNumForUEID is absent in subgroupConfig, the subgroup ID based on CN assigned subgrouping as specified in clause 7.y.1 is used in the cell.

-	If both subgroupsNumPerPO and subgroupsNumForUEID are configured, and subgroupsNumForUEID has the same value as subgroupsNumPerPO, the subgroup ID based on UE_ID based subgrouping as specified in clause 7.y.2 is used in the cell.

-	If both subgroupsNumPerPO and subgroupsNumForUEID are configured, and 0 < subgroupsNumForUEID < subgroupsNumPerPO, the subgroup ID based on CN assigned subgrouping as specified in clause 7.y.1 and the subgroup ID based on UE_ID based subgrouping as specified in clause 7.y.2 are used in the cell.

The following parameters are used for the determination of subgroup ID:

subgroupsNumPerPO: number of subgroups for total CN assigned subgrouping (if any) and UE_ID based subgrouping (if any) in a PO, which is broadcasted in system information

subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information

If a UE cannot find its subgroup ID, the UE monitors its associated PO as specified in clause 7.1.

### 7.y.1	CN assigned subgrouping

Paging with CN assigned subgrouping is only used in the cell which supports CN assigned subgrouping, as described in clause 7.y.0. A UE supporting CN assigned subgrouping in RRC_IDLE or RRC_INACTIVE state can be assigned a subgroup ID (between 0 to 7) by AMF through NAS signalling. The UE belonging to the assigned subgroup ID monitors its associated PEI which indicates the corresponding subgroup of UEs to monitor the associated PO as specified in clause 7.x.

### 7.y.2	UE_ID based subgrouping

Paging with UE_ID based subgrouping is only used in the cell which supports UE_ID based subgrouping, as described in clause 7.y.0.

- If a UE supports UE_ID based subgrouping:

- if the UE is not configured with a CN assigned subgroup ID; or,

- if the UE is configured with a CN assigned subgroup ID in a cell supporting only UE_ID based subgrouping:

- the subgroup ID of the UE is determined by below formula:

SubgroupID = (floor(UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID),

where:

N: number of total paging frames in T

Ns: number of paging occasions for a PF

UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; or otherwise, X is 8192

subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information

Editor’s NOTE:	The detailed parameters for UE_ID based subgrouping will be aligned with RRC specification.

The UE belonging to the SubgroupID monitors its associated PEI which includes the paged subgroup(s) as specified in clause 7.x.

Next change

# Z	TRS

## Z.1	General

The UE in RRC_IDLE and RRC_INACTIVE state may use TRS whose configurations are provided in system information by network during its paging reception to save power. In a cell in which TRS are available for the UE in RRC_IDLE and RRC_INACTIVE states to use, the availability of indicated TRS is informed to the RRC_IDLE and RRC_INATIVE state UEs based on explicit L1 based availability indication defined in TS 38.213 [4]. A UE which acquired system information with TRS configuration but has not yet receives an associated L1 based availability indication considers the corresponding TRS resource set(s) as unavailable.

## Z.2	TRS reception

Editor’s NOTE:	Companies are invited to provide comments on whether this part needs to be captured here or only captured in PHY specification.

After the UE receives the configuration of TRS in SIBx,

-	If PEI is detected as specified in clause 7.x.1, and the availability indication(s) in PEI indicates that the TRS at the configured occasion(s) is available, as specified in TS 38.213 [4]; or,

-	If the availability indication(s) in paging PDCCH (according to clause 7) indicates that the TRS at the configured occasion(s) is available, as specified in TS 38.213 [4]:

-	the UE considers the corresponding TRS resource set(s) associated with the availability indication(s) is available for a time duration associated with the availability indication(s), as specified in TS 38.213 [4].

End of change

# Annex A– RAN2 agreements

### Agreements on paging enhancement

| RAN2 agreements | Impacted specification | Comments, if any |
| --- | --- | --- |
| RAN2#111e |  |  |
| For PowSav solutions for Idle/Inactive (for smart phones) that can easily also be applied to redcap, R2 assume they may be applied. Details FFS and to be discuss case by case when the maturity is high (might in the end just be a question of UE caps). Dual DRX not in the scope of current WID. | No impact |  |
| RAN2#112e |  |  |
| Confirm that UE grouping is considered a candidate of paging enhancement for UE power saving | Not yet captured to wait for further progress. |  |
| RAN2 have discussed and considered “paging indication for UE subgroups using paging DCI”, “paging early indication or wake-up signal (WUS) for UE subgroups”, “cross-slot scheduling of paging for UE subgroups”. RAN2 understands that RAN1 have started to evaluate performance and complexity. RAN2 assumes that RAN1 continues with this evaluation, in order that decisions can be made regarding the paging indication/scheduling solution. As R2 is the leading group for this WI objective it is expected that final decisions are made by R2. Will send an LS to R1 (action to be discussed offline). The solution of PRNTI based group discrimination is deprioritized from RAN2 perspective The solution of “paging for UE subgroups using different time/frequency resources” is de-prioritized from RAN2 perspective. | No impact |  |
| RAN2#113e |  |  |
| There is support to have UE ID based enhancement There is still significant interest to have other additional methods (but also some concerns). The approach to have a single mechanism that can take several aspects into account can be a way forward. There are still questions on the details, e.g. whether CN or RAN would provide a parameter. | No impact |  |
| RAN2#113bis-e |  |  |
| If we go for network controlled subgrouping, If the network chooses to not provide specific subgrouping information, there will be configuration option where subgrouping can be supported by randomization (by UE-ID). | Captured in 7.y.2 |  |
| We adopt Network controlled subgrouping (based on individual UE characteristics, not specified or limited to paging prob as EUTRA, possibly with additional randomization) | Not yet captured to wait for further progress. |  |
| RAN2#114e |  |  |
| CN is responsible for allocating UEs to UE paging subgroups based on UE characteristics | Captured in 7.y.1 |  |
| Use same UE subgroups when in RRC_IDLE and RRC_INACTIVE | Captured in 7.y |  |
| RAN2#115e |  |  |
| When AMF has assigned a UE with a Paging subgroup, some NAS signaling should be supported between AMF and UE to convey the related information to the UE. Exact information is FFS. The design and procedure are up to SA2/CT1. | Captured in 7.y.1 |  |
| When AMF has assigned a UE with a Paging subgroup, some signaling should be supported between AMF and gNB(s) to inform gNB(s) about the related subgroup information for paging a UE in RRC_IDLE/RRC_INACTIVE. Exact information is FFS. The message(s) and associated design are up to RAN3. | Captured in 7.y.1 |  |
| It is FFS when a UE in RRC_INACTIVE has been assigned by CN a Paging subgroup, whether some signaling should be introduced between gNBs to inform each other about the UE’s subgroup for RAN paging. | No impact by now |  |
| If RAN2 agrees to support UE assistance information to CN in support of Paging subgroup assignment, RAN2 will focus on the paging probability and power profile attributes. | No impact by now |  |
| UEID-based subgroup method requires, in addition to the already available information for legacy UEID-based grouping in PO, the total number of supported UEID-based subgroups by the network. | Captured in 7.y.2 |  |
| At least for UEID-based subgroup method the total number, Nsg, of supported subgroups by the network is decided by RAN and broadcasted in System Information. | Captured in 7.y.2 |  |
| At least for UEID-based subgroup method the total number, Nsg, of supported subgroups is controlled on a cell basis and can be different in different cells. | Captured in 7.y.2 |  |
| Option 2 is excluded We go with Option 1 Editor’s NOTE: option 1 and option 2 come from R2-2109094, which are: - Option 1: CN assigns subgroup ID - possible with or without remapping to RAN subgroup ID depends on the sub-options - Option 2: CN assigns a set of subgroup IDs - Similar to option 1 but with multiple subgroup IDs assigned from CN and the UE needs to choose the corresponding subgroup ID based on RAN configuration | Captured in 7.y.1 |  |
| R2 assumes that All the cells within the registration area supports the same number of CN assigned subgroups, i.e. no remapping of CN assigned group ID to RAN subgroup ID (will revisit only if serious issues are found). | Captured in 7.y.1 |  |
| For the purpose of continued discussions, R2 assumes that UE has separate UE caps for CN assigned and UEID based subgrouping, the actual decision to be taken later. | Not yet captured to wait for further progress. |  |
| RAN capability is known based on broadcast information. FFS with explicit indication or implicitly based configuration. | Captured in 7.y |  |
| RAN2#116e |  |  |
| Assume that one subgroup indication refer to either CN assigned subgroups or UE-ID based subgroup (no overlapping) Both UE ID based and CN based subgrouping can be supported simultaneously in a cell, it is allowed to just support one of them. | Captured in 7.y.0 |  |
| FFS if the total number of CN-assigned subgroups is OAM configured. Max would be 8 as this is what RAN support. | No impact |  |
| The total number of CN-assigned subgroups that is used is not fixed can be configured up to 8 (e.g. by OAM). No impact on signalling is assumed. | No impact |  |
| RAN introduces a new parameter Nsg-UEID to indicate its support of UE-ID based subgrouping. | Captured in 7.y.2 |  |
| RAN does not support any type of subgrouping if its configuration for subgrouping is either absent or nullified (e.g. subgroupsNumPerPO is either absent or set to zero). FFS for the signalling details. | Captured in 7.y.0 |  |
| We assume separate indications for UE capability of CN based subgrouping and UEID based subgrouping. UE’s capability of supporting the UE ID based subgrouping is reported to RAN by AS UE capability signalling while R2 assumes that UE’s capability of supporting the CN-assigned subgrouping is reported to CN by NAS signalling. | Captured in 7.y.1 and 7.y.2 |  |
| RAN2 assumes that if PEI is detected, and the PEI indicates that the UE has to monitor the associated PO, then the UE monitors paging DCI in the associated PO, including scheduling information for paging PDSCH (if included) as in legacy. This assumption may be updated based on RAN1 agreements. | Captured in 7.x.1 |  |
| As a baseline RAN2 has a preference to support PEI with both DRX and eDRX, but potential issues (e.g. PEI and PTW) are FFS. | Captured in 7.x.1 |  |
| For UE-ID based subgroups the UE identity is UE_ID = 5G-S-TMSI mod X, where X is 8192 (1024*8). | Captured in 7.y.2 |  |
| Introduce a UERadioPagingInfo IE in the UECapabilityInformation message in NR in Rel-17. | No impact |  |
| If the UE was not able to monitor the PEI occasion corresponding to its PO the UE shall monitor the PO. | Captured in 7.x.1 |  |
| RAN2#116bis-e |  |  |
| RAN configuration (of subgrouping) includes the two parameters Nsg-UEID (number of UEID-based subgroups) and subgroupsNumPerPO (total number of subgroups in a PO): - If only CN-assigned subgrouping is used, subgroupsNumPerPO is present (the value then equals to the number of CN-assigned subgroups), and Nsg-UEID is absent. - If only UEID-based subgrouping is used, subgroupsNumPerPO and Nsg-UEID are present, and Nsg-UEID has the same value as subgroupsNumPerPO. - If both subgrouping methods are used, both subgroupsNumPerPO and Nsg-UEID are present, and 0 < Nsg-UEID < subgroupsNumPerPO. | Captured in 7.y.0 |  |
| RAN2 aims to Support PEI and subgrouping with eDRX. FFS the impact. | Captured in 7.x.1 as an EN to wait for further progress. |  |
| RAN2 assumes that there is no particular impact to Uu signalling to support RAN sharing. It is further assumed that Core Networks must have consistent policy if subgrouping is used by multiple Core Networks. | No impact |  |
| RAN2 assumes that PEI can be used “without” subgrouping. FFS whether the bits in the PEI for subgrouping then need to have any particular meaning, or whether this would be done by just having one subgroup. | Captured in 7.y.0 FFS part is assumed to be captured by RAN1. |  |
| RAN2 assumes that PEI monitoring can not be specifically enabled/disabled for individual UEs. | No impact |  |
| PEI subgroup indices are allocated to CN-assigned subgroups first. The 1st bit in the PEI bitmap corresponds to the CN-assigned subgroup #1, 2nd bit in the bitmap corresponds to the CN-assigned subgroup #2, and so on. After CN-assigned subgroups, the subsequent bits, if any, correspond to the UEID-based subgroup #1, #2, and so on. | Assuming to be Captured in RAN1 specification TS 38.212 and 38.213, and no impact to TS 38.304 |  |
| Both subgroupNumPerPO and Nsg-UEID range from 1 to 8. | Assuming to be Captured in RRC, and no impact to TS 38.304 |  |
| If network supports PEI but not subgrouping, the whole SubgroupConfig-r17 is absent. The parameter subgroupsNumPerPO is mandatory present if subgroupConfig-r17 is configured. | Captured in 7.y.0 |  |
| UE is configured to monitor PEI, either only in the last used cell or any other cells (after cell reselection). FFS how the configuration is provided in [SI, RRCRelease, or NAS message]. | Captured in 7.x.1 |  |
| If a cell supports both UE identity based and CN assigned subgrouping, for UEID based paging subgrouping, UE belongs to k-th paging subgroup, where - k = [floor (UE Identity/(N*Ns)) mod Nsg-UEID] + Nsg-CN, - N is the number of Paging frames, - Ns is the number of POs per paging frame, - Nsg-UEID is the number of UEID-based paging subgroups, and - Nsg-CN is the number of CN assigned paging subgroups (= subgroupNumPerPO - Nsg-UEID). | Captured in 7.y.2 |  |
| Paging enhancement capability(-ies) (e.g. PEI capability, UEID based subgrouping capability or the combined capability of PEI and UEID based subgrouping) are ‘optional with capability signalling’ as gNB needs to know the paging enhancement capability(-ies) to page the UE Separate indications for UE capability of CN based subgrouping and UEID based subgrouping (confirms earlier assumption) UE’s capability of supporting the UE ID based subgrouping is reported to RAN by AS UE capability signalling while UE’s capability of supporting the CN-assigned subgrouping is reported to CN by NAS signalling. (confirms earlier assumption). | Partially captured in 7.x.1, 7.y.1, 7.y.2. Details assuming to be captured in RRC and TS 38.306. |  |
| Paging enhancement capability(-ies) can be included into the UERadioPagingInfo IE in the UECapabilityInformation message as agreed in RAN2#116 (i.e. Introduce a UERadioPagingInfo IE in the UECapabilityInformation message in NR in Rel-17) gNB interprets UE’s reported UECapabilityInformation, copies the UERadioPagingInfo IE out and includes it as a container UE-RadioPagingInfo IE in the UERadioPagingInformation inter-node message to AMF | No impact |  |
| Specify that the UEs that expect group notification ignores PEI (and just monitor paging as usual) Editor’s NOTE: This conclusion was made in MBS WI session | Captured in 7.y.0 |  |
| RAN2#117e |  |  |
| Network indicates whether UE monitors PEI in last used cell in system information. | Captured in 7.x.0 |  |
| A PEI-capable UE stores its “last used cell” information. FFS on how to capture this in the specifications. | Suppose to be captured in the field description in RRC |  |
| Do not introduce an associated timer for the “last used cell” information stored by UE. The “lastUsedCellOnly” indication is a cell-level configuration and there is no per-subgroup indication. Introduce a one-bit indication of lastUsedCellOnly in PEI-Config. | No impact |  |
| RAN2 clarifies the meaning of “last used cell only”: When a cell broadcasts “last used cell only”, a UE monitors PEI only if its last connection was released by this cell. | Captured in 7.x.0 |  |
| A PEI-capable UE must support at least UEID-based subgrouping method. | No impact |  |
| RAN2 confirms that “PEI without subgrouping” can be implemented by configuring PEI plus UEID subgrouping with one subgroup. “PEI without subgrouping” can be configured by only one method. | Captured in 7.x.0 |  |
| When PEI is applied with eDRX, the UEID for UEID-based subgrouping is determined by 5G-S-TMSI mod 32768. | Captured in 7.y.2 |  |
| No special handling or configuration is introduced for PEI monitoring with PTW (i.e., PEI is applicable to each PO within PTW) Send an informative LS to RAN3/SA2/CT1 for RAN2 agreements about PEI and paging subgrouping and ask question about mismatched understanding of “last cell” between network and UE. | No impact |  |
| If a UE cannot find its subgroup ID with the PEI configurations in a cell, it monitors legacy paging. | Captured in 7.y.0 |  |
| Agree the easy agreement proposals (removing whether from P1). On P4 send an LS to RAN3 cc SA2, asking about the issue, can also indicate tentative solution and invite for feedback. Ask R3 also on P13, to what extent this may be a problem. | No impact |  |
| A UE with CN-assigned subgroup ID should derive UEID-based subgroup ID in a cell supporting only UEID-based subgrouping. | Captured in 7.y.2 |  |
| PEI + UEID subgrouping is one capability | No impact |  |

### Agreements on TRS/CSI-RS in idle/inactive

| RAN2 agreements | Impacted specification | Comments, if any |
| --- | --- | --- |
| RAN2#113e |  |  |
| [041] On signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s): SIB signalling is the baseline; Other dedicated high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) can be additionally considered with justification. It is assumed they do not work alone. | Not yet captured to wait for further progress. |  |
| [041] RAN2 will down select from the following options on SIB signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s): Option 2: Existing SIB, other than SIB1; Option 3: New SIB type, e.g. SIB-x; | Not yet captured to wait for further progress. |  |
| RAN2#114e |  |  |
| N/A |  |  |
| RAN2#115e |  |  |
| The TRS/CSI-RS configuration is provided in a new SIB. On demand SI should be possible for the SIB with TRS/CSI-RS information. | Captured in Z.2 |  |
| RAN2 assumes that TRS/CSI-RS configurations are broadcasted. Potential addition of dedicated signalling can be discussed in a later meeting based on company contributions. | Not yet captured to wait for further progress. |  |
| The legacy SI update procedure is used for changing TRS/CSI-RS configurations. | No impact by now. |  |
| Postpone the topic about TRS/CSI-RS availability until a later meeting when RAN1 also has progressed. Postpone the discussion on segmentation of the new SIB until RAN1 has sent the list of the parameters and a potential structure. Postpone the discussion on splitting the TRS/CSI-RS information to a common and RS-specific part until RAN1 has sent the list of the parameters and a potential structure. | Not yet captured to wait for further progress. |  |
| RAN2#116e |  |  |
| The scope of the new SIB-X is configurable (either cell or area scope) based on NW implementation. RAN2 to wait for additional RAN1 feedback, before finalizing aspects on SIB-X sizing, segmentation etc. RAN2 to wait for further RAN1 input on whether TRS/CSI-RS configuration can be split as common and TRS specific part. The new SIB-X can be made on demand, and it is up to NW configuration. There are no UE side impacts due to any additional NW side restriction on on-demand SIB-X. IDLE/INACTIVE UEs do NOT have to report any feedback on its TRS/CSI-RS resource usage. | No impact |  |
| RAN2 assumes to support current RAN1 working agreement of L1 based signalling for TRS/CSI-RS availability indication. FFS whether it should be possible to enable / disable the TRS/CSI-RS L1 based availability mechanism by broadcast signalling. | Captured in Z.1 |  |
| R2 assumes that additional TRS/CSI-RS configuration by dedicated signalling is not supported. Can revisit e.g. based on R1 provided info if needed. Postpone further discussion on TRS/CSI-RS applicability for eDRX UEs. Can consider later | Not yet captured to wait for further progress. |  |
| RAN2#116bis-e |  |  |
| The number of bits N in the bitmap used for L1 availability indication is derived implicitly from the number of different values of indBitID. There is no need for an explicit parameter. | No impact by now |  |
| RAN2 confirm TRS/CSI-RS can be applied to eDRX UEs. | Not yet captured to wait for further progress. |  |
| Confirm that there will be no particular mechanism for availability indication based on SIB (beyond the presence of the RS configuration) Indicating the TRS/CSI-RS availability in Idle/Inactive when releasing the UE to Idle/Inactive in the RRCRelease message is not pursued. RAN2 follows RAN1 agreement that if TRS resource is configured in SIB, L1 based availability indication is always enabled based on that configuration. RAN2 waits for RAN1 to finalize the contents of SIB-X before finalizing aspects on SIB-X sizing, segmentation etc | No impact |  |
| P1 is FFS Editor’s NOTE: P1 comes from R2-2201918, which is: A UE which acquired SIB-X with a TRS/CSI-RS configuration but didn’t yet receive an associated L1-based availability indication considers the configured TRS/CSI-RS as “unavailable”. | Captured in z.2 as an EN to wait for further progress. |  |
| RAN2#117e |  |  |
| A UE which acquired SIB-X with a TRS/CSI-RS configuration but didn’t yet receive an associated L1-based availability indication considers the configured TRS/CSI-RS as “unavailable”. |  |  |
| RAN2 reuses the existing mechanism used for SIB12 for implementing the SIBX segmentation | No impact |  |
| Not sufficient support to introduce any special functionality for UEs in eDRX. Rely on UE determination on whether he has up to date information or not. Can think about whether clarification is needed. | Not yet captured to wait for further progress. |  |
| gNB does not need to know the UE capability for TRS/CSI-RS in idle and inactive mode. Introduce R1 29-2 as optional without capability signalling | No impact |  |

# Annex B– RAN1 agreements (only RAN2 relevant part)

### Agreements on paging enhancement

| RAN1 agreements | Impacted specification | Comments, if any |
| --- | --- | --- |
| RAN1#102e |  |  |
| N/A |  |  |
| RAN1#103e |  |  |
| N/A |  |  |
| RAN1#104e and RAN#93e |  |  |
| Agreements: For the evaluation and comparison of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, the following are assumed: - Behv-A: - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO - Behv-B: - PEI indicates whether or not UE should monitor a PO - UE is required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO Rapporteur: Further conclusion in RAN#93 below should be considered together. - Support PDCCH-based PEI as the only option • Only essential function for PEI is support • New DCI format • Higher layer configuration, including SS • Details of the procedures of PEI monitoring, and identification of MOs before PO • Only Behv-A (per RAN1#104e agreement) is supported • If TRS availability indication is agreed to be supported in both paging DCI and the DCI format for PEI, same mechanism/principle for TRS availability indication is adopted for the two DCI formats • Supporting TRS availability indication in DCI format for PEI shall not delay the completion of essential functionality of PEI | Captured in 7.x.1 |  |
| RAN1#104bis-e |  |  |
| N/A |  |  |
| RAN1#105e |  |  |
| For UE subgroups indication in physical layer, maximum of 8 subgroups per PO is supported. | Not yet captured to wait for further progress. |  |
| RAN1#106e |  |  |
| N/A |  |  |
| RAN1#106bis-e |  |  |
| For NR Rel-17, paging indications to UE subgroups are carried only in PEI. | Captured in 7.y.0 |  |
| For PEI, a new DCI format is supported to include at least paging indications to UE group(s)/subgroups of the associated PO(s) - One bit in the DCI payload indicating one UE subgroup of a PO or one UE group/PO - The maximum number of total bits for paging indication field in PEI DCI format is x - One PEI can be configured to indicate up to 4 PO(s) in a PF - FFS whether to supporting map PEI to 3 POs in a PF - FFS: 1 PEI for POs across multiple PFs - FFS: value of x | Captured in 7.x.1 |  |
| A PEI occasion (PEI-O) is a set of S consecutive PDCCH monitoring occasions when nrofPDCCH-MonitoringOccasionPerSSB-InPO is not configured - S is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 - The K-th PDCCH monitoring occasion for PEI in the PEI-O has the same QCL assumption as that of the K-th PDCCH monitoring occasion for paging in the PO. - Note: QCL reference is SSB - FFS: Determination of the PEI-O location - FFS: Support of unlicensed spectrum operation with nrofPDCCH-MonitoringOccasionPerSSB-InPO configured | Captured in 7.x.1 |  |
| Agreement CORESET # 0 or commonControlResourceSet in SIB1 can be used for PEI - Note: The number of CORESETs configured for a UE follows the requirement of UE feature 3-1 Agreement Support configuration of a dedicated search space (‘peiSearchSpace’) for PEI - FFS: Configuration details and whether and how to reuse legacy search space sets, including pagingSearchSpace and searchSpaceSetZero | Captured in 7.x.1 |  |
| RAN1#107e |  |  |
| Agreement For unlicensed operation, A PEI-O is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for PEI in the PEI-O corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. - If X > 1, when the UE detects a PEI within its PEI-O, the UE is not required to monitor the subsequent monitoring occasion(s) associated with the same PEI-O Note: The QCL reference is SSB | Captured in 7.x.1 |  |
| Agreement Support mapping one PEI to POnumPerPEI PO(s) in one or multiple PF(s)  POnumPerPEI is a factor of (total PO number in a paging cycle) and configurable via SIB for the cell with the value range of {1, 2, 4, 8} - The Maximum number of PF associated with one PEI is up to 2  Note: Maximum number of paging indication bits in DCI format 2_7 can be kept the same for any configuration of POnumPerPEI, e.g., by applying a smaller subgroupsNumPerPO and a larger POnumPerPEI.  Note: Larger value of POnumPerPEI can reduce the average PEI overhead per PO, but there can also cause potentially larger paging latency and larger UE power consumption due to longer UE wake-up time before PO monitoring, which can be significant with large value of (T/N). | Partially captured in 7.x.1 |  |
| Agreement Confirm the following working assumption: Working Assumption - The paging indication field of PEI DCI format comprises of POnumPerPEI segment(s) of K bit - K = 1, if is absent or set to 0 or 1, - K = , if is configured. - UE identifies its paging indication bit as follows: - Let denote the relative PO index, with starting value of 0, among the POs associated with the PEI - , where are as defined in clause 7 of TS 38.304 - when K = 1 and UE is not provided a subgroup index - when UE is provided a subgroup index - UE checks the corresponding paging indication from -th bit of the paging indication field where the starting bit index is 0 - If the corresponding paging indication value is set to ‘1’, it indicates the UE to monitor the PO - If the corresponding paging indication value is set to ‘0’, it indicates the UE is not required to monitor the PO | No impact. Already captured in TS 38.213. |  |
| Agreement For PEI DCI format, defined as DCI format 2_7, - Total number of bits for paging indication filed is POnumPerPEI, if is absent or set to 0 or 1, and the number is , if is configured. - For Rel-17, UE does not expect paging indication filed size is larger than the DCI payload size - Whether and how TRS availability indication field is included is up to Agenda Item 8.7.1.2 - Support configurable DCI payload size which should be no larger than payload size of paging DCI - Unused bits, when applicable, are regarded as reserved bits - Note: A smaller payload size is beneficial for PEI detection performance | No impact. |  |
| Agreement - Determination of PEI-O location for UE’s PO is based on deciding a reference point and an offset from the reference point to the start of the first PDCCH MO of the PEI-O. o The reference point is the start of a reference frame determined by a frame-level offset from the start of the first PF of the PF(s) associated with the PEI-O and configured via SIB for the cell. - FFS: The range of the frame-level offset o There is a symbol-level offset from the reference point to the start of the first PDCCH MO of PEI-O, provided by firstPDCCH-MonitoringOccasionOfPEI-O and configured via SIB for the cell. - FFS: The range of the symbol-level offset - Note: When PEI-O is placed close to or overlapped with an earlier SS burst before its associated POs, the total UE wake-up time can be reduced for better power saving gain. Network can configure the PEI-O location accounting the power saving benefit and potential impact on gNB flexibility. | Captured in 7.x.1 |  |
| Agreement The CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level for PEI PDCCH monitoring occasion are given as the following table. Actual aggregation levels and PDCCH candidates are provided by ‘peiSearchSpace’ configuration in SIB. CE Aggregation Level; Number of Candidates. 4; 4. 8; 2. 16; 1. | No impact |  |
| Agreement ‘peiSearchSpace’ can be configured to one of up to 4 common SS sets configured by commonSearchSpaceList with SearchSpaceId > 0 | Captured in 7.x.1 |  |
| RAN1#107bis-e |  |  |
| Agreement A new PEI-RNTI is supported for DCI format 2_7. Agreement For whether and how to accommodate PEI-O location determination for the case POnumPerPEI is smaller than Ns, decide one of the following alternatives - Alt-2: It is supported, and UE applies the single value in PEI-F_offset for the frame-level offset and the (flooris/POnumPerPEI+1) -th value out of Ns/POnumPerPEI configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset Note: The number of PO mapping to one PEI should be multiple of Ns when POnumPerPEI is larger than Ns | No impact |  |
| Agreement For PEI-O location determination: - Range of PEI-F_offset, in unit of frame, is 0, 1,2, …, 16 - Range of firstPDCCH-MonitoringOccasionOfPEI-O, in unit of symbol, is decided as one of the following alternatives: - Alt-1: The same value range as firstPDCCH-MonitoringOccasionOfPO | No impact |  |
| Agreement For the value PEI-RNTI, decide one of the following: - Alt-1: PEI-RNTI is of fixed value, and value design is up to RAN2. Conclusion For a UE supporting R17 paging enhancement feature, it is up to UE implementation whether the UE monitors the MO(s) for a PEI. - If UE decides to not to monitor PEI, it has to monitor UE’s PO as defined in 38.304. - Note: No specification impact | No impact |  |
| Agreement - The PDCCH monitoring occasions defined by peiSearchSpace colliding with UL symbols are omitted from the determination of the PEI monitoring occasions. The PDCCH monitoring occasions for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for PEI in the PEI-O. | Captured in 7.x.1 |  |
| Agreement The transmission power of PEI is based on the same determination method as paging PDCCH, | No impact |  |
| Agreement UE expects at most one PEI outcome from each PDCCH monitoring occasion of PEI-O, | No impact |  |
| Agreement If one PEI-O is associated with POs of 2 PFs, - The two PFs are consecutive PFs configured in SIB - FFS: two PFs are consecutive PFs within the same paging cycle - Note: As an example, SFN of the first PF of the PF(s) associated with the PEI-O can be obtained by: (SFN of UE’s PF) - iPO/Ns⋅T/N | Captured in 7.x.1 |  |
| Conclusion - For Rel-17, inclusion of SI change indication or ETWS/CMAS notification in DCI format 2_7 is not supported. Conclusion For Rel-17 DCI format 2_7, - Explicit starting position configuration for paging indication field or TRS availability indication field is not supported. - Paging indication field starts from the first bit, and TRS availability indication field starts from the next bit after the end of the paging indication field - UE does not expect the total size of paging indication filed and TRS availability indication field is larger than the DCI payload size | No impact |  |
| Working assumption SearchSpaceId = 0 can be configured forpeiSearchSpace for the case of CORESET multiplexing pattern 2 or 3 | Not yet captured to wait for further progress. |  |
| RAN1#108e |  |  |
| Conclusion For Rel-17, the following for paging early indication design are not supported: - Multipe PEI-Os for one PO - UE assistance information for preferred offset between PEI-O and PO - Different PEI configurations for different devices types | No impact |  |
| Conclusion SFN of the first PF of the PF(s) associated with the PEI-O is provided by (SFN of UE’s PF) - | Captured in 7.x.1 |  |
| Agreement - Confirm the following working assumption: Working assumption SearchSpaceId = 0 can be configured for peiSearchSpace for the case of CORESET multiplexing pattern 2 or 3. - When SearchSpaceId = 0 is configured for peiSearchSpace, subject to SS/PBCH block and CORESET multiplexing pattern 2 or 3, the PDCCH monitoring occasions for PEI-O are same as for RMSI as defined in clause 13 in TS 38.213. - UE determines first PDCCH MO for PEI-O based on PEI-F_offset and firstPDCCH-MonitoringOccasionOfPEI-O, as previously agreed for the case with SearchSpaceId > 0. - Note: UE expects the first PDCCH MO for PEI-O determined from PEI-F_offset and firstPDCCH-MonitoringOccasionOfPEI-O is aligned with the MOs for RMSI as defined in clause 13 in TS 38.213. | Captured in 7.x.1 |  |

### Agreements on TRS/CSI-RS in idle/inactive

| RAN1 agreements | Impacted specification | Comments, if any |
| --- | --- | --- |
| RAN1#104e |  |  |
| N/A |  |  |
| RAN1#104bis-e |  |  |
| N/A |  |  |
| RAN1#105e |  |  |
| Agreement: Confirm the following working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling Agreement: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint - e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources - e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources - FFS maximum number of configured RS resources per physical layer availability indication to support. - FFS whether availability/unavailability information is for all or some of configured RS resources | Captured in Z.1 |  |
| Working assumption: Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected. - FFS whether and how to enable/disable L1 based availability indication configurable by SIB | Captured in Z.2 |  |
| RAN1#106e |  |  |
| N/A |  |  |
| RAN1#106bis-e |  |  |
| Agreement At least for paging PDCCH based L1 availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, the L1 availability indication is valid for a time duration starting from a reference point, where - the time duration is a validity duration configured by higher layer, - FFS applicable values, e.g. # of DRX cycles, or multiple of default paging cycle duration (i.e. modification period) - FFS UE doesn’t expect inconsistent L1 based indication during the time duration - the reference point for start of the validity duration is one of the following alternatives: - Alt1: SFN of the first PF from the next DRX cycle - Alt2: SFN of the first PF from the current DRX cycle where UE receives the indication - Alt3: based on SFN configured by higher layer, i.e. modification period configured as multiple of default paging cycle duration - Alt4: start of the PF for the PO where UE receives the indication - Note: the DRX cycle in Alt1 and Alt2 is the default paging cycle broadcast in SIB - Note: The SFN for the first PF is for (UE mod N) = 0, and can be calculated by (SFN + PF_offset) mod T = 0 - the time duration can be optionally configured by gNB - when the time duration is not configured, one of the following alternatives can be considered: - Alt1: the availability indication is valid until when the UE receives another availability indication. - Alt2: the availability indication is valid until L1 availability indication is changed by network - Alt3: default time duration e.g. default paging cycle - FFS whether and how to handle the miss detection issue of L1 signaling | Partially captured in Z.2 |  |
| RAN1#107e |  |  |
| Agreement For the maximum number of TRS resource sets configured by higher layer, X, - X = 64 - FFS: the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 | No impact |  |
| Agreement For L1 availability indication using a bitmap, the following is supported: - Number of bits in the bitmap, N, is up to 6 bits - a bit is associated with a group of TRS resource sets. The associated TRS resource sets for each bit can be based on - explicit configuration of TRS resource set group, where - each TRS resource set is configured with a ID i, with value from {0, …, N-1}, for the association with an indication bit in TRS availability indication field. - the ith bit maps to all the TRS resource set(s) associated with ID i. - start of the bitmap is the first bit of the reserved bits in paging PDCCH - Note: It is left to RAN2 decision on whether explicit parameter is used for N or it can be implicitly determined by the TRS resource set configurations. | Already captured in TS 38.213. FFS part is captured |  |
| Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication - FFS: Whether the availability indication is transmitted [only once] during the validity duration | Already captured in TS 38.213. |  |
| Agreement For the validity duration configured by higher layer at least for paging PDCCH based L1 availability indication, support - time unit is one default paging cycle, - applicable values: {1, 2, 4, 8, 16, 32, [64], [128], [256],[512]} When the validity duration is not configured, UE assumes a default time duration to be 2 default paging cycle(s). | Partially captured in Z.2 |  |
| Agreement The following working assumption is confirmed Working Assumption - Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. - Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected. | Already captured in TS 38.213. |  |
| RAN1#107bis-e |  |  |
| Agreement Support a configuration parameter for the number of, X, TRS resources for a TRS resource set at least for FR2 - Applicable values for X: {2, 4} - For FR1, X is based on configuration parameter with applicable value {2, 4} Note: Configuration follows restriction specified in sub-clause 5.1.6.1.1 in TS38.214 for connected mode TRS Agreement One or more scrambling IDs is configured for a TRS resource set. - If a single scrambling ID is configured, it applies to all the TRS resources. - Otherwise, each TRS resource is provided with a scrambling ID. Agreement The applicable values:{64, 128, 256, 512} are supported for the validity duration configured by higher layer, - Note: If UE is provided a configuration of validity duration longer than 10.24s, and the UE does not support eDRX, it is up to UE implementation whether to assume the validity duration length is no larger than 10.24s | No impact |  |
| Agreement UE can receive L1 based signaling for TRS availability indication before the expiration/end of validity duration associated with previous L1 based signaling for TRS availability indication - For each bit indicated as ‘1’ in the availability indication field of the current L1 based signaling TRS availability indication, the UE assumes the corresponding TRS resource set(s) are available from the reference point until the end of the validity duration associated with the current L1 based signaling indication. - For each bit indicated as ‘0’ in the availability indication field of the current L1 based signaling, the UE keeps the existing current assumption on the availability or unavailability of the corresponding TRS resource set(s). Note: the validity duration for different group of TRS resources sets correspond to different bits in the availability indication field can be different and are maintained independently. Note: CATT has concern on the power saving gain based on this agreement. | Partially captured in Z.2, others already captured in TS 38.213. |  |
| Agreement Confirm the following working assumption Working Assumption If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. | No impact |  |
| Agreement The parameter of periodicityAndOffset is used to determine the location of the first slot of TRS resource set. | No impact |  |
| Agreement If SIB configures TRS resource, TRS availability indication field is present in DCI format 2_7 (if configured) with CRC scrambled by PEI-RNTI and DCI format 1_0 with CRC scrambled by P-RNTI Note: Huawei, MTK and ZTE have concern on the agreement. | Already captured in Z.2 |  |
| RAN1#108e |  |  |
| Conclusion No consensus to support - PEI DCI provides L1 availability indication information only for RS resources with QCL references to be the same as for the L1 availability indication occasion Conclusion A UE can receive different TRS availability indication content from PEI and associated paging PDCCH. - Note: It is up to UE implementation whether UE receives both PEI and the associated paging PDCCH to receive TRS availability indication Conclusion No consensus to support - For PEI based availability indication, the reference time of the start of validity duration follows the reference time for associated monitoring occasion for the paging DCI. Agreement Down-select one of the TPs to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication - Alt1: Adopt TP#1 - Alt2: Adopt TP#2 - Note: TRS availability indication transmitted in all PDCCH monitoring occasions of the same PEI-O or PO is same. - Note: Reference point for TRS availability indication in a given DCI is identical for all UEs regardless of UE specific DRX value. Agreement TRS-ResourceSetConfig IE is not accounted in determination of available resource elements for PDSCH. | No impact |  |