3GPP TSG-RAN WG2 Meeting #117-e	R2-220xxxx

# Electronic meeting, 21st February – 3rd March  2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Slicing UE Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Slice-Core |  |  |  |  |  | Date: |  |  | 2022-03-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UE capability signalling for Slicing support |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the UE capabilities agreements from RAN2#116-bis-e for Slicing. ## Agreements in RAN2#116-e #1: UE indicates its support of slice based cell reselection in the UE capability signalling with the following TS38.306 description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No UE capabilities for Slicing will be defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | UE-NR-Capability in Section 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 6.3.3	UE capability information elements

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [26].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::=            SEQUENCE {

accessStratumRelease            AccessStratumRelease,

pdcp-Parameters                 PDCP-Parameters,

rlc-Parameters                  RLC-Parameters                                                        OPTIONAL,

mac-Parameters                  MAC-Parameters                                                        OPTIONAL,

phy-Parameters                  Phy-Parameters,

rf-Parameters                   RF-Parameters,

measAndMobParameters            MeasAndMobParameters                                                  OPTIONAL,

fdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                           OPTIONAL,

tdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                           OPTIONAL,

fr1-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                           OPTIONAL,

fr2-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                           OPTIONAL,

featureSets                     FeatureSets                                                           OPTIONAL,

featureSetCombinations          SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination         OPTIONAL,

lateNonCriticalExtension        OCTET STRING (CONTAINING UE-NR-Capability-v15c0)                      OPTIONAL,

nonCriticalExtension            UE-NR-Capability-v1530                                                OPTIONAL

}

-- Regular non-critical extensions:

UE-NR-Capability-v1530 ::=               SEQUENCE {

fdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                            OPTIONAL,

tdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                            OPTIONAL,

dummy                                    ENUMERATED {supported}                                       OPTIONAL,

interRAT-Parameters                      InterRAT-Parameters                                          OPTIONAL,

inactiveState                            ENUMERATED {supported}                                       OPTIONAL,

delayBudgetReporting                     ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1540                                       OPTIONAL

}

UE-NR-Capability-v1540 ::=              SEQUENCE {

sdap-Parameters                         SDAP-Parameters                                               OPTIONAL,

overheatingInd                          ENUMERATED {supported}                                        OPTIONAL,

ims-Parameters                          IMS-Parameters                                                OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr1-fr2-Add-UE-NR-Capabilities          UE-NR-CapabilityAddFRX-Mode                                   OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1550                                        OPTIONAL

}

UE-NR-Capability-v1550 ::=               SEQUENCE {

reducedCP-Latency                        ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1560                                       OPTIONAL

}

UE-NR-Capability-v1560 ::=               SEQUENCE {

nrdc-Parameters                         NRDC-Parameters                                               OPTIONAL,

receivedFilters                         OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)       OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1570                                        OPTIONAL

}

UE-NR-Capability-v1570 ::=               SEQUENCE {

nrdc-Parameters-v1570                   NRDC-Parameters-v1570                                         OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1610                                        OPTIONAL

}

-- Late non-critical extensions:

UE-NR-Capability-v15c0 ::=               SEQUENCE {

nrdc-Parameters-v15c0                    NRDC-Parameters-v15c0                                        OPTIONAL,

partialFR2-FallbackRX-Req                ENUMERATED {true}                                            OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v15g0                                       OPTIONAL

}

UE-NR-Capability-v15g0 ::=               SEQUENCE {

rf-Parameters-v15g0                      RF-Parameters-v15g0                                          OPTIONAL,

nonCriticalExtension                     SEQUENCE {}                                                  OPTIONAL

}

-- Regular non-critical extensions:

UE-NR-Capability-v1610 ::=               SEQUENCE {

inDeviceCoexInd-r16                     ENUMERATED {supported}                                        OPTIONAL,

dl-DedicatedMessageSegmentation-r16     ENUMERATED {supported}                                        OPTIONAL,

nrdc-Parameters-v1610                   NRDC-Parameters-v1610                                         OPTIONAL,

powSav-Parameters-r16                   PowSav-Parameters-r16                                         OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1610        UE-NR-CapabilityAddFRX-Mode-v1610                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1610        UE-NR-CapabilityAddFRX-Mode-v1610                             OPTIONAL,

bh-RLF-Indication-r16                   ENUMERATED {supported}                                        OPTIONAL,

directSN-AdditionFirstRRC-IAB-r16       ENUMERATED {supported}                                        OPTIONAL,

bap-Parameters-r16                      BAP-Parameters-r16                                            OPTIONAL,

referenceTimeProvision-r16              ENUMERATED {supported}                                        OPTIONAL,

sidelinkParameters-r16                  SidelinkParameters-r16                                        OPTIONAL,

highSpeedParameters-r16                 HighSpeedParameters-r16                                       OPTIONAL,

mac-Parameters-v1610                    MAC-Parameters-v1610                                          OPTIONAL,

mcgRLF-RecoveryViaSCG-r16               ENUMERATED {supported}                                        OPTIONAL,

resumeWithStoredMCG-SCells-r16          ENUMERATED {supported}                                        OPTIONAL,

resumeWithStoredSCG-r16                 ENUMERATED {supported}                                        OPTIONAL,

resumeWithSCG-Config-r16                ENUMERATED {supported}                                        OPTIONAL,

ue-BasedPerfMeas-Parameters-r16         UE-BasedPerfMeas-Parameters-r16                               OPTIONAL,

son-Parameters-r16                      SON-Parameters-r16                                            OPTIONAL,

onDemandSIB-Connected-r16               ENUMERATED {supported}                                        OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1640                                        OPTIONAL

}

UE-NR-Capability-v1640 ::=               SEQUENCE {

redirectAtResumeByNAS-r16               ENUMERATED {supported}                                        OPTIONAL,

phy-ParametersSharedSpectrumChAccess-r16  Phy-ParametersSharedSpectrumChAccess-r16                    OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1650                                        OPTIONAL

}

UE-NR-Capability-v1650 ::=               SEQUENCE {

mpsPriorityIndication-r16                ENUMERATED {supported}                                       OPTIONAL,

highSpeedParameters-v1650                HighSpeedParameters-v1650                                    OPTIONAL,

nonCriticalExtension                      UE-NR-Capability-v17xy                            OPTIONAL

}

UE-NR-Capability-v17xy ::=               SEQUENCE {

sliceInfoforCellReselection-r17         ENUMERATED {supported}                                        OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                                   OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=         SEQUENCE {

phy-ParametersXDD-Diff                  Phy-ParametersXDD-Diff                                        OPTIONAL,

mac-ParametersXDD-Diff                  MAC-ParametersXDD-Diff                                        OPTIONAL,

measAndMobParametersXDD-Diff            MeasAndMobParametersXDD-Diff                                  OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode-v1530 ::=    SEQUENCE {

eutra-ParametersXDD-Diff                 EUTRA-ParametersXDD-Diff

}

UE-NR-CapabilityAddFRX-Mode ::= SEQUENCE {

phy-ParametersFRX-Diff              Phy-ParametersFRX-Diff                                            OPTIONAL,

measAndMobParametersFRX-Diff        MeasAndMobParametersFRX-Diff                                      OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1540 ::=    SEQUENCE {

ims-ParametersFRX-Diff                   IMS-ParametersFRX-Diff                                       OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1610 ::=    SEQUENCE {

powSav-ParametersFRX-Diff-r16            PowSav-ParametersFRX-Diff-r16                                OPTIONAL,

mac-ParametersFRX-Diff-r16               MAC-ParametersFRX-Diff-r16                                   OPTIONAL

}

BAP-Parameters-r16 ::=                   SEQUENCE {

flowControlBH-RLC-ChannelBased-r16       ENUMERATED {supported}                                       OPTIONAL,

flowControlRouting-ID-Based-r16          ENUMERATED {supported}                                       OPTIONAL

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

UE-NR-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for supportedBandCombinationList in UE-NR-Capability. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

UE-NR-Capability-v1540 field descriptions

fr1-fr2-Add-UE-NR-Capabilities This instance of UE-NR-CapabilityAddFRX-Mode does not include any other fields than csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework.

END OF CHANGE