3GPP TSG-RAN WG2 Meeting #117 Electronic	R2-2203766

# 21st February 2022 - 3rd March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2887 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of enhanced IIoT&URLLC support for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-03-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture the features introduced in the work item: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN1 impacts: The below RAN1 RRC paramters are captured. - RAN1 RRC parameter list R1-2202759 (sent in the LS R1-2202760) Field description update to reflect RAN1 agremeents on NR-U. RAN2 impacts: - Clasue 5.5: Add the UE Rx-Tx time difference measurement reporting - Clause 5.7.1.3: Add the UE action related with the reception of the gNB Rx-Tx time difference measurement, the SIB9 fallback indication, the dedicated signalling in the DL information transfer message - In clause 6.3.2, in the IE DL Information Transfer, add the fields: to activate/de-activate UE-side TA-based PDC; to indicate gNB Rx-TX time difference measurement; SIB9 fallback indication. - In clause 6.3.2, add the IE MeasResultRxTxTimeDiff-r17, MeasObjectRxTxDiff-r17, RxTxPeriodical-r17, RxTxTimeDiff-r17 to suport RTT-based PDC - In clause 6.3.2, add intraCG-Prioritization in the MAC-CellGroupConfig to - In Clause 6.3.2, add survival time support indication in PDCP-Config - In Clause 6.3.2, in the IE ReferenceTimeInfo clarify that the time can also be compensated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The features introduced in Rel-17 NR_IIOT_URLLC_enh are not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.1, 5.5.2.1, 5.5.3.1, 5.5.4.1, 5.5.5.1, 5.7.1.3, 6.2.2, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.300 CR 0416r1 TS 38.321 CR 1200r1 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision “1”: This version updates the endorsed CR R2-2202325 with the RAN2#117-e meeting agreements and the updated RAN1 parameter list in R1-2202759. |  |  |  |  |  |  |  |  |

first change

## 5.5	Measurements

### 5.5.1	Introduction

The network may configure an RRC_CONNECTED UE to perform measurements. The network may configure the UE to report them in accordance with the measurement configuration or perform conditional reconfiguration evaluation in accordance with the conditional reconfiguration. The measurement configuration is provided by means of dedicated signalling i.e. using the RRCReconfiguration or RRCResume.

The network may configure the UE to perform the following types of measurements:

-	NR measurements;

-	Inter-RAT measurements of E-UTRA frequencies.

-	Inter-RAT measurements of UTRA-FDD frequencies.

The network may configure the UE to report the following measurement information based on SS/PBCH block(s):

-	Measurement results per SS/PBCH block;

-	Measurement results per cell based on SS/PBCH block(s);

-	SS/PBCH block(s) indexes.

The network may configure the UE to report the following measurement information based on CSI-RS resources:

-	Measurement results per CSI-RS resource;

-	Measurement results per cell based on CSI-RS resource(s);

-	CSI-RS resource measurement identifiers.

The network may configure the UE to perform the following types of measurements for NR sidelink and V2X sidelink:

-	CBR measurements.

The network may configure the UE to report the following CLI measurement information based on SRS resources:

-	Measurement results per SRS resource;

-	SRS resource(s) indexes.

The network may configure the UE to report the following CLI measurement information based on CLI-RSSI resources:

-	Measurement results per CLI-RSSI resource;

-	CLI-RSSI resource(s) indexes.

The network may configure the UE to report the following Rx-Tx time difference measurement information based on CSI-RS for tracking or PRS:

-	UE Rx-Tx time difference measurement result.

The measurement configuration includes the following parameters:

1.	Measurement objects: A list of objects on which the UE shall perform the measurements.

-	For intra-frequency and inter-frequency measurements a measurement object indicates the frequency/time location and subcarrier spacing of reference signals to be measured. Associated with this measurement object, the network may configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting. Whitelisted cells are the only ones applicable in event evaluation or measurement reporting.

-	The measObjectId of the MO which corresponds to each serving cell is indicated by servingCellMO within the serving cell configuration.

-	For inter-RAT E-UTRA measurements a measurement object is a single E-UTRA carrier frequency. Associated with this E-UTRA carrier frequency, the network can configure a list of cell specific offsets and a list of 'blacklisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting.

-	For inter-RAT UTRA-FDD measurements a measurement object is a set of cells on a single UTRA-FDD carrier frequency.

-	For CBR measurement of NR sidelink communication, a measurement object is a set of transmission resource pool(s) on a single carrier frequency for NR sidelink communication.

-	For CLI measurements a measurement object indicates the frequency/time location of SRS resources and/or CLI-RSSI resources, and subcarrier spacing of SRS resources to be measured.

2.	Reporting configurations: A list of reporting configurations where there can be one or multiple reporting configurations per measurement object. Each measurement reporting configuration consists of the following:

-	Reporting criterion: The criterion that triggers the UE to send a measurement report. This can either be periodical or a single event description.

-	RS type: The RS that the UE uses for beam and cell measurement results (SS/PBCH block or CSI-RS).

-	Reporting format: The quantities per cell and per beam that the UE includes in the measurement report (e.g. RSRP) and other associated information such as the maximum number of cells and the maximum number beams per cell to report.

In case of conditional reconfiguration, each configuration consists of the following:

-	Execution criteria: The criteria the UE uses for conditional reconfiguration execution.

-	RS type: The RS that the UE uses for obtaining beam and cell measurement results (SS/PBCH block-based or CSI-RS-based), used for evaluating conditional reconfiguration execution condition.

3.	Measurement identities: For measurement reporting, a list of measurement identities where each measurement identity links one measurement object with one reporting configuration. By configuring multiple measurement identities, it is possible to link more than one measurement object to the same reporting configuration, as well as to link more than one reporting configuration to the same measurement object. The measurement identity is also included in the measurement report that triggered the reporting, serving as a reference to the network. For conditional reconfiguration triggering, one measurement identity links to exactly one conditional reconfiguration trigger configuration. And up to 2 measurement identities can be linked to one conditional reconfiguration execution condition.

4.	Quantity configurations: The quantity configuration defines the measurement filtering configuration used for all event evaluation and related reporting, and for periodical reporting of that measurement. For NR measurements, the network may configure up to 2 quantity configurations with a reference in the NR measurement object to the configuration that is to be used. In each configuration, different filter coefficients can be configured for different measurement quantities, for different RS types, and for measurements per cell and per beam.

5.	Measurement gaps: Periods that the UE may use to perform measurements.

A UE in RRC_CONNECTED maintains a measurement object list, a reporting configuration list, and a measurement identities list according to signalling and procedures in this specification. The measurement object list possibly includes NR measurement object(s), CLI measurement object(s) and inter-RAT objects. Similarly, the reporting configuration list includes NR and inter-RAT reporting configurations. Any measurement object can be linked to any reporting configuration of the same RAT type. Some reporting configurations may not be linked to a measurement object. Likewise, some measurement objects may not be linked to a reporting configuration.

The measurement procedures distinguish the following types of cells:

1.	The NR serving cell(s) – these are the SpCell and one or more SCells.

2.	Listed cells – these are cells listed within the measurement object(s).

3.	Detected cells – these are cells that are not listed within the measurement object(s) but are detected by the UE on the SSB frequency(ies) and subcarrier spacing(s) indicated by the measurement object(s).

For NR measurement object(s), the UE measures and reports on the serving cell(s), listed cells and/or detected cells. For inter-RAT measurements object(s) of E-UTRA, the UE measures and reports on listed cells and detected cells and, for RSSI and channel occupancy measurements, the UE measures and reports on the configured resources on the indicated frequency. For inter-RAT measurements object(s) of UTRA-FDD, the UE measures and reports on listed cells. For CLI measurement object(s), the UE measures and reports on configured measurement resources (i.e. SRS resources and/or CLI-RSSI resources).

Whenever the procedural specification, other than contained in sub-clause 5.5.2, refers to a field it concerns a field included in the VarMeasConfig unless explicitly stated otherwise i.e. only the measurement configuration procedure covers the direct UE action related to the received measConfig.

In NR-DC, the UE may receive two independent measConfig:

-	a measConfig, associated with MCG, that is included in the RRCReconfiguration message received via SRB1; and

-	a measConfig, associated with SCG, that is included in the RRCReconfiguration message received via SRB3, or, alternatively, included within a RRCReconfiguration message embedded in a RRCReconfiguration message received via SRB1.

In this case, the UE maintains two independent VarMeasConfig and VarMeasReportList, one associated with each measConfig, and independently performs all the procedures in clause 5.5 for each measConfig and the associated VarMeasConfig and VarMeasReportList, unless explicitly stated otherwise.

The configurations related to CBR measurements are only included in the measConfig associated with MCG.

The configurations related to Rx-Tx time difference measurement are only included in the measConfig associated with MCG.

next change

### 5.5.3	Performing measurements

#### 5.5.3.1	General

An RRC_CONNECTED UE shall derive cell measurement results by measuring one or multiple beams associated per cell as configured by the network, as described in 5.5.3.3. For all cell measurement results, except for RSSI, and CLI measurement results in RRC_CONNECTED, the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria, measurement reporting or the criteria to trigger conditional reconfiguration execution. For cell measurements, the network can configure RSRP, RSRQ, SINR, RSCP or EcN0 as trigger quantity. For CLI measurements, the network can configure SRS-RSRP or CLI-RSSI as trigger quantity. For cell and beam measurements, reporting quantities can be any combination of quantities (i.e. only RSRP; only RSRQ; only SINR; RSRP and RSRQ; RSRP and SINR; RSRQ and SINR; RSRP, RSRQ and SINR; only RSCP; only EcN0; RSCP and EcN0), irrespective of the trigger quantity, and for CLI measurements, reporting quantities can be either SRS-RSRP or CLI-RSSI. For conditional reconfiguration execution, the network can configure up to 2 quantities, both using same RS type. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the CBR measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the Rx-Tx time difference measurements.

The network may also configure the UE to report measurement information per beam (which can either be measurement results per beam with respective beam identifier(s) or only beam identifier(s)), derived as described in 5.5.3.3a. If beam measurement information is configured to be included in measurement reports, the UE applies the layer 3 beam filtering as specified in 5.5.3.2. On the other hand, the exact L1 filtering of beam measurements used to derive cell measurement results is implementation dependent.

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

2>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to csi-rs:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

1>	for each serving cell for which servingCellMO is configured, if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains SINR as trigger quantity and/or reporting quantity:

2>	if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO:

3>	if the reportConfigcontains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

4>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

2>	if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO:

3>	if the reportConfigcontains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

4>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

3>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the reportType for the associated reportConfig is set to reportCGI and timer T321 is running:

3>	if useAutonomousGaps is configured for the associated reportConfig:

4>	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using autonomous gaps as necessary;

3>	else:

4>	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using available idle periods;

3>	if the cell indicated by reportCGI field for the associated measObject is an NR cell and that indicated cell is broadcasting SIB1 (see TS 38.213 [13], clause 13):

4>	try to acquire SIB1 in the concerned cell;

3>	if the cell indicated by reportCGI field is an E-UTRA cell:

4>	try to acquire SystemInformationBlockType1 in the concerned cell;

2>	if the ul-DelayValueConfig is configured for the associated reportConfig:

3>	ignore the measObject;

3>	for each of the configured DRBs, configure the PDCP layer to perform corresponding average UL PDCP packet delay measurement per DRB;

2>	if the reportType for the associated reportConfig is periodical, eventTriggered or condTriggerConfig:

3>	if a measurement gap configuration is setup, or

3>	if the UE does not require measurement gaps to perform the concerned measurements:

4>	if s-MeasureConfig is not configured, or

4>	if s-MeasureConfig is set to ssb-RSRP and the NR SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP, or

4>	if s-MeasureConfig is set to csi-RSRP and the NR SpCell RSRP based on CSI-RS, after layer 3 filtering, is lower than csi-RSRP:

5>	if the measObject is associated to NR and the rsType is set to csi-rs:

6>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 filtered beam measurements only based on CSI-RS for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on CSI-RS for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to NR and the rsType is set to ssb:

6>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on SS/PBCH block for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to E-UTRA:

6>	perform the corresponding measurements associated to neighbouring cells on the frequencies indicated in the concerned measObject, as described in 5.5.3.2;

5>	if the measObject is associated to UTRA-FDD:

6>	perform the corresponding measurements associated to neighbouring cells on the frequencies indicated in the concerned measObject, as described in 5.5.3.2;

4>	if the measRSSI-ReportConfig is configured in the associated reportConfig:

5>	perform the RSSI and channel occupancy measurements on the frequency indicated in the associated measObject;

2>	if the reportType for the associated reportConfig is set to reportSFTD and the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than one:

3>	if the reportSFTD-Meas is set to true:

4>	if the measObject is associated to E-UTRA:

5>	perform SFTD measurements between the PCell and the E-UTRA PSCell;

5>	if the reportRSRP is set to true;

6>	perform RSRP measurements for the E-UTRA PSCell;

4>	else if the measObject is associated to NR:

5>	perform SFTD measurements between the PCell and the NR PSCell;

5>	if the reportRSRP is set to true;

6>	perform RSRP measurements for the NR PSCell based on SSB;

3>	else if the reportSFTD-NeighMeas is included:

4>	if the measObject is associated to NR:

5>	if the drx-SFTD-NeighMeas is included:

6>	perform SFTD measurements between the PCell and the NR neighbouring cell(s) detected based on parameters in the associated measObject using available idle periods;

5>	else:

6>	perform SFTD measurements between the PCell and the NR neighbouring cell(s) detected based on parameters in the associated measObject;

5>	if the reportRSRP is set to true:

6>	perform RSRP measurements based on SSB for the NR neighbouring cell(s) detected based on parameters in the associated measObject;

2>	if the reportType for the associated reportConfig is cli-Periodical or cli-EventTriggered:

3>	perform the corresponding measurements associated to CLI measurement resources indicated in the concerned measObjectCLI;

2>	perform the evaluation of reporting criteria as specified in 5.5.4, except if reportConfig is condTriggerConfig.

NOTE 1:	The evaluation of conditional reconfiguration execution criteria is specified in 5.3.5.13.

The UE capable of Rx-Tx time difference measurement when configured with measObjectRxTxDiff shall:

1>	perform the corresponding Rx-Tx time difference measurements associated with downlink reference signals indicated in the concerned measObjectRxTxDiff.

The UE capable of CBR measurement when configured to transmit NR sidelink communication shall:

1>	If the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE:

3>	if the cell chosen for NR sidelink communication provides SIB12 which includes sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency:

4>	perform CBR measurement on pools in sl-TxPoolSelectedNormal and sl-TxPoolExceptional for the concerned frequency in SIB12;

2>	if the UE is in RRC_CONNECTED:

3>	if tx-PoolMeasToAddModList is included in VarMeasConfig:

4>	perform CBR measurements on each transmission resource pool indicated in the tx-PoolMeasToAddModList;

3>	if sl-TxPoolSelectedNormal, sl-TxPoolScheduling or sl-TxPoolExceptional is included in sl-ConfigDedicatedNR for the concerned frequency within RRCReconfiguration:

4>	perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal, sl-TxPoolScheduling or sl-TxPoolExceptional if included in sl-ConfigDedicatedNR for the concerned frequency within RRCReconfiguration;

3>	else if the cell chosen for NR sidelink communication provides SIB12 which includes sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency:

4>	perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal and sl-TxPoolExceptional for the concerned frequency in SIB12;

1>	else:

2>	perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal and sl-TxPoolExceptional in SidelinkPreconfigNR for the concerned frequency.

NOTE 2:	In case the configurations for NR sidelink communication and CBR measurement are acquired via the E-UTRA, configurations for NR sidelink communication in SIB12, sl-ConfigDedicatedNR within RRCReconfiguration used in this subclause are provided by the configurations in SystemInformationBlockType28, sl-ConfigDedicatedForNR within RRCConnectionReconfiguration as specified in TS 36.331[10], respectively.

NOTE 3:	If a UE that is configured by upper layers to transmit V2X sidelink communication is configured by NR with transmission resource pool(s) and the measurement objects concerning V2X sidelink communication (i.e. by sl-ConfigDedicatedEUTRA-Info), it shall perform CBR measurement as specified in subclause 5.5.3 of TS 36.331 [10], based on the transmission resource pool(s) and the measurement object(s) concerning V2X sidelink communication configured by NR.

NOTE 4:	For V2X sidelink communication, each of the CBR measurement results is associated with a resource pool, as indicated by the poolReportId (see TS 36.331 [10]), that refers to a pool as included in sl-ConfigDedicatedEUTRA-Info or SIB13.

next change

### 5.5.4	Measurement report triggering

#### 5.5.4.1	General

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the corresponding reportConfig includes a reportType set to eventTriggered or periodical:

3>	if the corresponding measObject concerns NR:

4>	if the corresponding reportConfig includes measRSSI-ReportConfig:

5>	consider the resource indicated by the rmtc-Config on the associated frequency to be applicable;

4>	if the eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	if the eventA3 or eventA5 is configured in the corresponding reportConfig:

5>	if a serving cell is associated with a measObjectNR and neighbours are associated with another measObjectNR, consider any serving cell associated with the other measObjectNR to be a neighbouring cell as well;

4>	if corresponding reportConfig includes reportType set to periodical; or

4>	for measurement events other than eventA1 or eventA2:

5>	if useWhiteCellList is set to true:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is included in the whiteCellsToAddModList defined within the VarMeasConfig for this measId;

5>	else:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is not included in the blackCellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns E-UTRA:

4>	if eventB1 or eventB2 is configured in the corresponding reportConfig:

5>	consider a serving cell, if any, on the associated E-UTRA frequency as neighbour cell;

4>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the blackCellsToAddModListEUTRAN defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns UTRA-FDD:

4>	if eventB1-UTRA-FDD or eventB2-UTRA-FDD is configured in the corresponding reportConfig; or

4>	if corresponding reportConfig includes reportType set to periodical:

5>	consider a neighbouring cell on the associated frequency to be applicable when the concerned cell is included in the cellsToAddModList defined within the VarMeasConfig for this measId;

2>	else if the corresponding reportConfig includes a reportType set to reportCGI:

3>	consider the cell detected on the associated measObject which has a physical cell identity matching the value of the cellForWhichToReportCGI included in the corresponding reportConfig within the VarMeasConfig to be applicable;

2>	else if the corresponding reportConfig includes a reportType set to reportSFTD:

3>	if the corresponding measObject concerns NR:

4>	if the reportSFTD-Meas is set to true:

5>	consider the NR PSCell to be applicable;

4>	else if the reportSFTD-NeighMeas is included:

5>	if cellsForWhichToReportSFTD is configured in the corresponding reportConfig:

6>	consider any NR neighbouring cell detected on the associated measObjectNR which has a physical cell identity that is included in the cellsForWhichToReportSFTD to be applicable;

5>	else:

6>	consider up to 3 strongest NR neighbouring cells detected based on parameters in the associated measObjectNR to be applicable when the concerned cells are not included in the blackCellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns E-UTRA:

4>	if the reportSFTD-Meas is set to true:

5>	consider the E-UTRA PSCell to be applicable;

2>	else if the corresponding reportConfig includes a reportType set to cli-Periodical or cli-EventTriggered:

3>	consider all CLI measurement resources included in the corresponding measObject to be applicable;

2>	else if the corresponding reportConfig includes a reportType set to rxTxPeriodical:

3>	consider all Rx-Tx time difference measurement resources included in the corresponding measObject to be applicable;

2>	if the corresponding reportConfig concerns the reporting for NR sidelink communication (i.e. reportConfigNR-SL):

3>	consider the transmission resource pools indicated by the tx-PoolMeasToAddModList defined within the VarMeasConfig for this measId to be applicable;

2>	if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if useT312 is set to true in reportConfig for this event:

4>	if T310 for the corresponding SpCell is running; and

4>	if T312 is not running for corresponding SpCell:

5>	start timer T312 for the corresponding SpCell with the value of T312 configured in the corresponding measObjectNR;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if useT312 is set to true in reportConfig for this event:

4>	if T310 for the corresponding SpCell is running; and

4>	if T312 is not running for corresponding SpCell:

5>	start timer T312 for the corresponding SpCell with the value of T312 configured in the corresponding measObjectNR;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable transmission resource pools for all measurements taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include an measurement reporting entry for this measId (a first transmission resource pool triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable transmission resource pools not included in the poolsTriggeredList for all measurements taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent transmission resource pool triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more applicable transmission resource pools included in the poolsTriggeredList defined within the VarMeasReportList for this measId for all measurements taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	if the poolsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running

NOTE 1:	Void.

2>	if reportType is set to periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	if the corresponding reportConfig includes measRSSI-ReportConfig:

4>	initiate the measurement reporting procedure as specified in 5.5.5 immediately when RSSI sample values are reported by the physical layer after the first L1 measurement duration;

3>	else if the corresponding reportConfig includes the ul-DelayValueConfig:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after a first measurement result is provided from lower layers of the associated DRB identity;

3>	else if the reportAmount exceeds 1:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell;

3>	else (i.e. the reportAmount is equal to 1):

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell and for the strongest cell among the applicable cells;

2>	if, in case the corresponding reportConfig concerns the reporting for NR sidelink communication, reportType is set to periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell and CBR measurement results become available;

2>	if the reportType is set to cli-EventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable CLI measurement resources for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first CLI measurement resource triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned CLI measurement resource(s) in the cli-TriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to cli-EventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more CLI measurement resources not included in the cli-TriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent CLI measurement resource triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned CLI measurement resource(s) in the cli-TriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to cli-EventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the CLI measurement resources included in the cli-TriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned CLI measurement resource(s) in the cli-TriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cli-TriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	if reportType is set to cli-Periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for at least one CLI measurement resource;

2>	if reportType is set to rxTxPeriodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	upon expiry of the periodical reporting timer for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5.

2>	if the corresponding reportConfig includes a reportType is set to reportSFTD:

3>	if the corresponding measObject concerns NR:

4>	if the drx-SFTD-NeighMeas is included:

5>	if the quantity to be reported becomes available for each requested pair of PCell and NR cell:

6>	stop timer T322;

6>	initiate the measurement reporting procedure, as specified in 5.5.5;

4>	else

5>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for each requested pair of PCell and NR cell or the maximal measurement reporting delay as specified in TS 38.133 [14];

3>	else if the corresponding measObject concerns E-UTRA:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the pair of PCell and E-UTRA PSCell or the maximal measurement reporting delay as specified in TS 38.133 [14];

2>	if reportType is set to reportCGI:

3>	if the UE acquired the SIB1 or SystemInformationBlockType1 for the requested cell; or

3>	if the UE detects that the requested NR cell is not transmitting SIB1 (see TS 38.213 [13], clause 13):

4>	stop timer T321;

4>	include a measurement reporting entry within the VarMeasReportList for this measId;

4>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	upon the expiry of T321 for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5.

2>	upon the expiry of T322 for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5.

next change

### 5.5.5	Measurement reporting

#### 5.5.5.1	General

Figure 5.5.5.1-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to the network. The UE shall initiate this procedure only after successful AS security activation.

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	for each serving cell configured with servingCellMO:

2>	if the reportConfig associated with the measId that triggered the measurement reporting includes rsType:

3>	if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report;

2>	else:

3>	if SSB based serving cell measurements are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB;

3>	else if CSI-RS based serving cell measurements are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

1>	set the servCellId within measResultServingMOList to include each NR serving cell that is configured with servingCellMO, if any;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

2>	for each serving cell configured with servingCellMO, include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

2>	for each measObjectId referenced in the measIdList which is also referenced with servingCellMO, other than the measObjectId corresponding with the measId that triggered the measurement reporting:

3>	if the measObjectNR indicated by the servingCellMO includes the RS resource configuration corresponding to the rsType indicated in the reportConfig:

4>	set the measResultBestNeighCell within measResultServingMOList to include the physCellId and the available measurement quantities based on the reportQuantityCell and rsType indicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNR with the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR;

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	for each best non-serving cell included in the measurement report:

6>	include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if the reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5, or eventB1, or eventB2:

2>	if the UE is in NE-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListEUTRA-SCG to include an entry for each E-UTRA SCG serving frequency with the following:

4>	include carrierFreq of the E-UTRA serving frequency;

4>	set the measResultServingCell to include the available measurement quantities that the UE is configured to measure by the measurement configuration associated with the SCG;

4>	if reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

5>	set the measResultServFreqListEUTRA-SCG to include within measResultBestNeighCell the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	if reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5:

2>	if the UE is in NR-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListNR-SCG to include for each NR SCG serving cell that is configured with servingCellMO, if any, the following:

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes rsType:

5>	if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report;

4>	else:

5>	if SSB based serving cell measurements are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB;

5>	else if CSI-RS based serving cell measurements are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

4>	if results for the serving cell derived based on SSB are included:

5>	include the ssbFrequency to the value indicated by ssbFrequency as included in the MeasObjectNR of the serving cell;

4>	if results for the serving cell derived based on CSI-RS are included:

5>	include the refFreqCSI-RS to the value indicated by refFreqCSI-RS as included in the MeasObjectNR of the serving cell;

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	for each serving cell configured with servingCellMO, include beam measurement information according to the associated reportConfig as described in 5.5.5.2, where availability is considered according to the measurement configuration associated with the SCG;

4>	if reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

5>	if the measObjectNR indicated by the servingCellMO includes the RS resource configuration corresponding to the rsType indicated in the reportConfig:

6>	set the measResultBestNeighCellListNR within measResultServFreqListNR-SCG to include one entry with the physCellId and the available measurement quantities based on the reportQuantityCell and rsType indicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNR with the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR, where availability is considered according to the measurement configuration associated with the SCG;

7>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

8>	for each best non-serving cell included in the measurement report:

9>	include beam measurement information according to the associated reportConfig as described in 5.5.5.2, where availability is considered according to the measurement configuration associated with the SCG;

1>	if the measRSSI-ReportConfig is configured within the corresponding reportConfig for this measId:

2>	set the rssi-Result to the linear average of sample value(s) provided by lower layers in the reportInterval;

2>	set the channelOccupancy to the rounded percentage of sample values which are beyond the channelOccupancyThreshold within all the sample values in the reportInterval;

1>	if there is at least one applicable neighbouring cell to report:

2>	if the reportType is set to eventTriggered or periodical:

3>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

4>	if the reportType is set to eventTriggered:

5>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

4>	else:

5>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

4>	for each cell that is included in the measResultNeighCells, include the physCellId;

4>	if the reportType is set to eventTriggered or periodical:

5>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

6>	if the measObject associated with this measId concerns NR:

7>	if rsType in the associated reportConfig is set to ssb:

8>	set resultsSSB-Cell within the measResult to include the SS/PBCH block based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

8>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

7>	else if rsType in the associated reportConfig is set to csi-rs:

8>	set resultsCSI-RS-Cell within the measResult to include the CSI-RS based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

8>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

6>	if the measObject associated with this measId concerns E-UTRA:

7>	set the measResult to include the quantity(ies) indicated in the reportQuantity within the concerned reportConfigInterRAT in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

6>	if the measObject associated with this measId concerns UTRA-FDD and if ReportConfigInterRAT includes the reportQuantityUTRA-FDD:

7>	set the measResult to include the quantity(ies) indicated in the reportQuantityUTRA-FDD within the concerned reportConfigInterRAT in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

2>	else:

3>	if the cell indicated by cellForWhichToReportCGI is an NR cell:

4>	if plmn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained:

5>	include the plmn-IdentityInfoList including plmn-IdentityList, trackingAreaCode (if available), ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the plmn-IdentityInfoList;

5>	include frequencyBandList if available;

4>	if nr-CGI-Reporting-NPN is supported by the UE and npn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained:

5>	include the npn-IdentityInfoList including npn-IdentityList, trackingAreaCode, ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the npn-IdentityInfoList;

5>	include cellReservedForOtherUse if available;

4>	else if MIB indicates the SIB1 is not broadcast:

5>	include the noSIB1 including the ssb-SubcarrierOffset and pdcch-ConfigSIB1 obtained from MIB of the concerned cell;

3>	if the cell indicated by cellForWhichToReportCGI is an E-UTRA cell:

4>	if all mandatory fields of the cgi-Info-EPC for the concerned cell have been obtained:

5>	include in the cgi-Info-EPC the fields broadcasted in E-UTRA SystemInformationBlockType1 associated to EPC;

4>	if the UE is E-UTRA/5GC capable and all mandatory fields of the cgi-Info-5GC for the concerned cell have been obtained:

5>	include in the cgi-Info-5GC the fields broadcasted in E-UTRA SystemInformationBlockType1 associated to 5GC;

4>	if the mandatory present fields of the cgi-Info for the cell indicated by the cellForWhichToReportCGI in the associated measObject have been obtained:

5>	include the freqBandIndicator;

5>	if the cell broadcasts the multiBandInfoList, include the multiBandInfoList;

5>	if the cell broadcasts the freqBandIndicatorPriority, include the freqBandIndicatorPriority;

1>	if the corresponding measObject concerns NR:

2>	if the reportSFTD-Meas is set to true within the corresponding reportConfigNR for this measId:

3>	set the measResultSFTD-NR in accordance with the following:

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true;

5>	set rsrp-Result to the RSRP of the NR PSCell derived based on SSB;

2>	else if the reportSFTD-NeighMeas is included within the corresponding reportConfigNR for this measId:

3>	for each applicable cell which measurement results are available, include an entry in the measResultCellListSFTD-NR and set the contents as follows:

4>	set physCellId to the physical cell identity of the concerned NR neighbour cell.

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true:

5>	set rsrp-Result to the RSRP of the concerned cell derived based on SSB;

1>	else if the corresponding measObject concerns E-UTRA:

2>	if the reportSFTD-Meas is set to true within the corresponding reportConfigInterRAT for this measId:

3>	set the measResultSFTD-EUTRA in accordance with the following:

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true;

5>	set rsrpResult-EUTRA to the RSRP of the EUTRA PSCell;

1>	if average uplink PDCP delay values are available:

2>	set the ul-PDCP-DelayValueResultList to include the corresponding average uplink PDCP delay values;

1>	if the includeCommonLocationInfo is configured in the corresponding reportConfig for this measId and detailed location information that has not been reported is available, set the content of commonLocationInfo of the locationInfo as follows:

2>	include the locationTimestamp;

2>	include the locationCoordinate, if available;

2>	include the velocityEstimate, if available;

2>	include the locationError, if available;

2>	include the locationSource, if available;

2>	if available, include the gnss-TOD-msec,

1>	if the includeWLAN-Meas is configured in the corresponding reportConfig for this measId, set the wlan-LocationInfo of the locationInfo in the measResults as follows:

2>	if available, include the LogMeasResultWLAN, in order of decreasing RSSI for WLAN APs;

1>	if the includeBT-Meas is configured in the corresponding reportConfig for this measId, set the BT-LocationInfo of the locationInfo in the measResults as follows:

2>	if available, include the LogMeasResultBT, in order of decreasing RSSI for Bluetooth beacons;

1>	if the includeSensor-Meas is configured in the corresponding reportConfig for this measId, set the sensor-LocationInfo of the locationInfo in the measResults as follows:

2>	if available, include the sensor-MeasurementInformation;

2>	if available, include the sensor-MotionInformation;

1>	if there is at least one applicable transmission resource pool for NR sidelink communication (for measResultsSL):

2>	set the measResultsListSL to include the CBR measurement results in accordance with the following:

3>	if the reportType is set to eventTriggered:

4>	include the transmission resource pools included in the poolsTriggeredList as defined within the VarMeasReportList for this measId;

3>	else:

4>	include the applicable transmission resource pools for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

3>	if the corresponding measObject concerns NR sidelink communication, then for each transmission resource pool to be reported:

4>	set the sl-poolReportIdentity to the identity of this transmission resource pool;

4>	set the sl-CBR-ResultsNR to the CBR measurement results on PSSCH and PSCCH of this transmission resource pool provided by lower layers, if available;

NOTE 1:	Void.

1>	if there is at least one applicable CLI measurement resource to report:

2>	if the reportType is set to cli-EventTriggered or cli-Periodical:

3>	set the measResultCLI to include the most interfering SRS resources or most interfering CLI-RSSI resources up to maxReportCLI in accordance with the following:

4>	if the reportType is set to cli-EventTriggered:

5>	if trigger quantity is set to srs-RSRP i.e. i1-Threshold is set to srs-RSRP:

6>	include the SRS resource included in the cli-TriggeredList as defined within the VarMeasReportList for this measId;

5>	if trigger quantity is set to cli-RSSI i.e. i1-Threshold is set to cli-RSSI:

6>	include the CLI-RSSI resource included in the cli-TriggeredList as defined within the VarMeasReportList for this measId;

4>	else:

5>	if reportQuantityCLI is set to srs-rsrp:

6>	include the applicable SRS resources for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

5>	else:

6>	include the applicable CLI-RSSI resources for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

4>	for each SRS resource that is included in the measResultCLI:

5>	include the srs-ResourceId;

5>	set srs-RSRP-Result to include the layer 3 filtered measured results in decreasing order, i.e. the most interfering SRS resource is included first;

4>	for each CLI-RSSI resource that is included in the measResultCLI:

5>	include the rssi-ResourceId;

5>	set cli-RSSI-Result to include the layer 3 filtered measured results in decreasing order, i.e. the most interfering CLI-RSSI resource is included first;

1>	if there is at least one applicable UE Rx-Tx time difference measurement to report:

2> set measResultRxTxTimeDiff to the latest measurement result;

Editor’s note (IIoT): It is assumed (without explicit RAN2 agreements) that only the latest measurement is included in the report, e.g., no filtered measurement, no multiple measurement reports. RAN2 to discuss if others are needed.

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1>	else:

2>	if the reportType is set to periodical or cli-Periodical or rxTxPeriodical:

3>	remove the entry within the VarMeasReportList for this measId;

3>	remove this measId from the measIdList within VarMeasConfig;

1>	if the measurement reporting was configured by a sl-ConfigDedicatedNR received within the RRCConnectionReconfiguration:

2>	submit the MeasurementReport message to lower layers for transmission via SRB1, embedded in E-UTRA RRC message ULInformationTransferIRAT as specified TS 36.331 [10], clause 5.6.28;

1>	else if the UE is in (NG)EN-DC:

2>	if SRB3 is configured:

3>	submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends;

2>	else:

3>	submit the MeasurementReport message via E-UTRA embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

1>	else if the UE is in NR-DC:

2>	if the measurement configuration that triggered this measurement report is associated with the SCG:

3>	if SRB3 is configured:

4>	submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends;

3>	else:

4>	submit the MeasurementReport message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3;

2>	else:

3>	submit the MeasurementReport message via SRB1 to lower layers for transmission, upon which the procedure ends;

1>	else:

2>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends.

next change

5.7	Other

5.7.1	DL information transfer

5.7.1.3	Reception of the DLInformationTransfer by the UE

Upon receiving DLInformationTransfer message, the UE shall:

1>	if dedicatedNAS-Message is included:

2>	forward dedicatedNAS-Message to upper layers.

1>	if referenceTimeInfo is included:

2>	calculate the reference time based on the time, referenceSFN and timeInfoType if it is included;

2>	calculate the uncertainty of the reference time based on the uncertainty, if uncertainty is included;

2>	inform upper layers of the reference time and, if uncertainty is included, of the uncertainty;

2>	ignore all further referenceTimeInfo received in SIB9, if any.

1>	if sib9Fallback is included:

2>	 fallback to receive referenceTimeInfo in SIB9.

1>	if rxTxTimeDiff-gNB is included:

2>	calculate the propagation delay based on the UE Rx-Tx time difference measurement and the received Rx-Tx time difference measurement;

2>	inform upper layers of the propagation delay.

next change

6	Protocol data units, formats and parameters (ASN.1)

6.2	RRC messages

6.2.2	Message definitions

–	DLInformationTransfer

The DLInformationTransfer message is used for the downlink transfer of NAS dedicated information and timing information for the 5G internal system clock.

Signalling radio bearer: SRB2 or SRB1 (only if SRB2 not established yet. If SRB2 is suspended, the network does not send this message until SRB2 is resumed.)

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

DLInformationTransfer message

-- ASN1START

-- TAG-DLINFORMATIONTRANSFER-START

DLInformationTransfer ::=           SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

dlInformationTransfer           DLInformationTransfer-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

DLInformationTransfer-IEs ::=       SEQUENCE {

dedicatedNAS-Message                DedicatedNAS-Message                OPTIONAL,   -- Need N

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                DLInformationTransfer-v1610-IEs     OPTIONAL

}

DLInformationTransfer-v1610-IEs ::= SEQUENCE {

referenceTimeInfo-r16               ReferenceTimeInfo-r16               OPTIONAL,   -- Need R

nonCriticalExtension                DLInformationTransfer                         OPTIONAL

}

DLInformationTransfer-v17xx-IEs ::= SEQUENCE {

rxTxTimeDiff-gNB-r17                RxTxTimeDiff-r17                    OPTIONAL,   -- Need R

ta-PDC-r17                          ENUMERATED {activate,deactivate}    OPTIONAL,   -- Need R

sib9Fallback-r17                    ENUMERATED {true}                   OPTIONAL,   -- Need R

nonCriticalExtension                SEQUENCE {}                         OPTIONAL

}

-- TAG-DLINFORMATIONTRANSFER-STOP

-- ASN1STOP

DLInformationTransfer field descriptions

rxTxTimeDiff-gNB Indicates the Rx-Tx time difference measurement at the gNB (see clause 5.2.3, TS 38.215 [9]). Upon receiving this field, the UE calculates the propagation delay based on the RTT-method. The network does not configure this field, if the UE is configured with ta-PDC with value activate.

sib9Fallback Indicates that the UE fallbacks to receive referenceTimeInfo in SIB9.

ta-PDC Indicates whether the UE-side TA-based propagation delay compensation (PDC) is activated or de-activated. The network does not configure this field with activate, if the field rxTxTimeDiff-gNB is configured.

next change

6.3.2	Radio resource control information elements

–	BetaOffsetsCrossPri

The IE BetaOffsetsCrossPri is used to configure beta-offset values for cross-priority HARQ-ACK multiplexing on PUSCH.

BetaOffsetsCrossPri information element

-- ASN1START

-- TAG-BETAOFFSETSCROSSPRI-START

BetaOffsetsCrossPri-r17 ::= SEQUENCE (SIZE(3)) OF INTEGER(0..31)

-- TAG-BETAOFFSETSCROSSPRI-STOP

-- ASN1STOP

next change

–	BWP-UplinkDedicated

The IE BWP-UplinkDedicated is used to configure the dedicated (UE specific) parameters of an uplink BWP.

BWP-UplinkDedicated information element

-- ASN1START

-- TAG-BWP-UPLINKDEDICATED-START

BWP-UplinkDedicated ::=             SEQUENCE {

pucch-Config                        SetupRelease { PUCCH-Config }                                           OPTIONAL,   -- Need M

pusch-Config                        SetupRelease { PUSCH-Config }                                           OPTIONAL,   -- Need M

configuredGrantConfig               SetupRelease { ConfiguredGrantConfig }                                  OPTIONAL,   -- Need M

srs-Config                          SetupRelease { SRS-Config }                                             OPTIONAL,   -- Need M

beamFailureRecoveryConfig           SetupRelease { BeamFailureRecoveryConfig }                              OPTIONAL,   -- Cond SpCellOnly

...,

[[

sl-PUCCH-Config-r16                 SetupRelease { PUCCH-Config }                                           OPTIONAL,   -- Need M

cp-ExtensionC2-r16                  INTEGER (1..28)                                                         OPTIONAL,   -- Need R

cp-ExtensionC3-r16                  INTEGER (1..28)                                                         OPTIONAL,   -- Need R

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

pucch-ConfigurationList-r16         SetupRelease { PUCCH-ConfigurationList-r16 }                            OPTIONAL,   -- Need M

lbt-FailureRecoveryConfig-r16       SetupRelease { LBT-FailureRecoveryConfig-r16 }                          OPTIONAL,   -- Need M

configuredGrantConfigToAddModList-r16                 ConfiguredGrantConfigToAddModList-r16                 OPTIONAL,   -- Need N

configuredGrantConfigToReleaseList-r16                ConfiguredGrantConfigToReleaseList-r16                OPTIONAL,   -- Need N

configuredGrantConfigType2DeactivationStateList-r16   ConfiguredGrantConfigType2DeactivationStateList-r16   OPTIONAL    -- Need R

]]

}

ConfiguredGrantConfigToAddModList-r16    ::= SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig

ConfiguredGrantConfigToReleaseList-r16   ::= SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfigIndex-r16

ConfiguredGrantConfigType2DeactivationState-r16 ::= SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfigIndex-r16

ConfiguredGrantConfigType2DeactivationStateList-r16  ::=

SEQUENCE (SIZE (1..maxNrofCG-Type2DeactivationState)) OF ConfiguredGrantConfigType2DeactivationState-r16

-- TAG-BWP-UPLINKDEDICATED-STOP

-- ASN1STOP

BWP-UplinkDedicated field descriptions

beamFailureRecoveryConfig Configuration of beam failure recovery. If supplementaryUplink is present, the field is present only in one of the uplink carriers, either UL or SUL.

configuredGrantConfig A Configured-Grant of type1 or type2. It may be configured for UL or SUL but in case of type1 not for both at a time. Except for reconfiguration with sync, the NW does not reconfigure configuredGrantConfig when there is an active configured uplink grant Type 2 (see TS 38.321 [3]). However, the NW may release the configuredGrantConfig at any time. Network can only configure configured grant in one BWP using either this field or configuredGrantConfigToAddModList.

configuredGrantConfigToAddModList Indicates a list of one or more configured grant configurations to be added or modified for one BWP. Except for reconfiguration with sync, the NW does not reconfigure a Type 2 configured grant configuration when it is active (see TS 38.321 [3]). The network configures multiple CG configurations with either all configurations or no configurations configured with cg-RetransmissionTimer-r16.

configuredGrantConfigToReleaseList Indicates a list of one or more UL Configured Grant configurations to be released. The NW may release a configured grant configuration at any time.

configuredGrantConfigType2DeactivationStateList Indicates a list of the deactivation states in which each state can be mapped to a single or multiple Configured Grant type 2 configurations to be deactivated when the corresponding deactivation DCI is received, see clause 7.3.1 in TS 38.212 [17] and clause 10.2 in TS 38.213 [13].

cp-ExtensionC2, cp-ExtensionC3 Configures the cyclic prefix (CP) extension (see TS 38.211 [16], clause 5.3.1). For 15 kHz SCS, {1..28} are valid for both cp-ExtensionC2 and cp-ExtensionC3. For 30 kHz SCS, {1..28} are valid for cp-ExtensionC2 and {2..28} are valid for cp-ExtensionC3. For 60 kHz SCS, {2..28} are valid for cp-ExtensionC2 and {3..28} are valid for cp-ExtensionC3.

lbt-FailureRecoveryConfig Configures parameters used for detection of consistent uplink LBT failures for operation with shared spectrum channel access, as specified in TS 38.321 [3].

pucch-Config PUCCH configuration for one BWP of the normal UL or SUL of a serving cell. If the UE is configured with SUL, the network configures PUCCH only on the BWPs of one of the uplinks (normal UL or SUL). The network configures PUCCH-Config at least on non-initial BWP(s) for SpCell and PUCCH SCell. If supported by the UE, the network may configure at most one additional SCell of a cell group with PUCCH-Config (i.e. PUCCH SCell); if PUCCH cell switching is supported by the UE, the network may configure at most one additional SCell with PUCCH-Config within each PUCCH group. In (NG)EN-DC and NE-DC, the NW configures at most one serving cell per frequency range with PUCCH. In (NG)EN-DC and NE-DC, if two PUCCH groups are configured, the serving cells of the NR PUCCH group in FR2 use the same numerology. For NR-DC, the maximum number of PUCCH groups in each cell group is one, and only the same numerology is supported for the cell group with carriers only in FR2. The NW may configure PUCCH for a BWP when setting up the BWP. The network may also add/remove the pucch-Config in an RRCReconfiguration with reconfigurationWithSync (for SpCell or PUCCH SCell) or with SCell release and add (for PUCCH SCell) to move the PUCCH between the UL and SUL carrier of one serving cell. In other cases, only modifications of a previously configured pucch-Config are allowed. If one (S)UL BWP of a serving cell is configured with PUCCH, all other (S)UL BWPs must be configured with PUCCH, too.

pucch-ConfigurationList PUCCH configurations for two simultaneously constructed HARQ-ACK codebooks (see TS 38.213 [13], clause 9.1). Different PUCCH Resource IDs are configured in different PUCCH-Config within the pucch-ConfigurationList if configured.

pusch-Config PUSCH configuration for one BWP of the normal UL or SUL of a serving cell. If the UE is configured with SUL and if it has a PUSCH-Config for both UL and SUL, an UL/SUL indicator field in DCI indicates which of the two to use. See TS 38.212 [17], clause 7.3.1.

sl-PUCCH-Config Indicates the UE specific PUCCH configurations used for the HARQ-ACK feedback reporting for NR sidelink communication.

srs-Config Uplink sounding reference signal configuration.

useInterlacePUCCH-PUSCH If the field is present, the UE uses uplink frequency domain resource allocation Type 2 for PUSCH (see 38.213 clause 8.3 and 38.214 clause 6.1.2.2) and uses interlaced PUCCH Format 0, 1, 2, and 3 for PUCCH (see TS 38.213 [13], clause 9.2.1).

| Conditional Presence | Explanation |
| --- | --- |
| SpCellOnly | The field is optionally present, Need M, in the BWP-UplinkDedicated of an SpCell. It is absent otherwise. |

NOTE 1:	In case of RRCReconfiguration with reconfigurationWithSync, the UE performs a MAC reset, which involves releasing the PUCCH-CSI/SRS/SR configuration in accordance with clause 5.3.12 and TS 38.321 [6], clauses 5.12 and 5.2. Hence, for these parts of the dedicated radio resource configuration, delta signalling is not supported in the message when reconfigurationWithSync is included.

next change

–	ConfiguredGrantConfig

The IE ConfiguredGrantConfig is used to configure uplink transmission without dynamic grant according to two possible schemes. The actual uplink grant may either be configured via RRC (type1) or provided via the PDCCH (addressed to CS-RNTI) (type2). Multiple Configured Grant configurations may be configured in one BWP of a serving cell.

ConfiguredGrantConfig information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

ConfiguredGrantConfig ::=           SEQUENCE {

frequencyHopping                    ENUMERATED {intraSlot, interSlot}                                       OPTIONAL,   -- Need S

cg-DMRS-Configuration               DMRS-UplinkConfig,

mcs-Table                           ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

mcs-TableTransformPrecoder          ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

uci-OnPUSCH                         SetupRelease { CG-UCI-OnPUSCH }                                         OPTIONAL,   -- Need M

resourceAllocation                  ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch },

rbg-Size                            ENUMERATED {config2}                                                    OPTIONAL,   -- Need S

powerControlLoopToUse               ENUMERATED {n0, n1},

p0-PUSCH-Alpha                      P0-PUSCH-AlphaSetId,

transformPrecoder                   ENUMERATED {enabled, disabled}                                          OPTIONAL,   -- Need S

nrofHARQ-Processes                  INTEGER(1..16),

repK                                ENUMERATED {n1, n2, n4, n8},

repK-RV                             ENUMERATED {s1-0231, s2-0303, s3-0000}                                  OPTIONAL,   -- Need R

periodicity                         ENUMERATED {

sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14,

sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14,

sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14,

sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12,

sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12,

sym1280x12, sym2560x12

},

configuredGrantTimer                INTEGER (1..64)                                                         OPTIONAL,   -- Need R

rrc-ConfiguredUplinkGrant           SEQUENCE {

timeDomainOffset                    INTEGER (0..5119),

timeDomainAllocation                INTEGER (0..15),

frequencyDomainAllocation           BIT STRING (SIZE(18)),

antennaPort                         INTEGER (0..31),

dmrs-SeqInitialization              INTEGER (0..1)                                                          OPTIONAL,   -- Need R

precodingAndNumberOfLayers          INTEGER (0..63),

srs-ResourceIndicator               INTEGER (0..15)                                                         OPTIONAL,   -- Need R

mcsAndTBS                           INTEGER (0..31),

frequencyHoppingOffset              INTEGER (1.. maxNrofPhysicalResourceBlocks-1)                           OPTIONAL,   -- Need R

pathlossReferenceIndex              INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1),

...,

[[

pusch-RepTypeIndicator-r16          ENUMERATED {pusch-RepTypeA,pusch-RepTypeB}                              OPTIONAL,   -- Need M

frequencyHoppingPUSCH-RepTypeB-r16  ENUMERATED {interRepetition, interSlot}                                 OPTIONAL,   -- Cond RepTypeB

timeReferenceSFN-r16                ENUMERATED {sfn512}                                                     OPTIONAL    -- Need S

]]

}                                                                                                           OPTIONAL,   -- Need R

...,

[[

cg-RetransmissionTimer-r16              INTEGER (1..64)                                                     OPTIONAL,   -- Need R

cg-minDFI-Delay-r16                     ENUMERATED

{sym7, sym1x14, sym2x14, sym3x14, sym4x14, sym5x14, sym6x14, sym7x14, sym8x14,

sym9x14, sym10x14, sym11x14, sym12x14, sym13x14, sym14x14,sym15x14, sym16x14

}                                                   OPTIONAL,   -- Need R

cg-nrofPUSCH-InSlot-r16                 INTEGER (1..7)                                              OPTIONAL,   -- Need R

cg-nrofSlots-r16                        INTEGER (1..40)                                             OPTIONAL,   -- Need R

cg-StartingOffsets-r16                  CG-StartingOffsets-r16                                      OPTIONAL,   -- Need R

cg-UCI-Multiplexing-r16                 ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

cg-COT-SharingOffset-r16                INTEGER (1..39)                                             OPTIONAL,   -- Need R

betaOffsetCG-UCI-r16                    INTEGER (0..31)                                            OPTIONAL,   -- Need R

cg-COT-SharingList-r16                  SEQUENCE (SIZE (1..1709)) OF CG-COT-Sharing-r16             OPTIONAL,   -- Need R

harq-ProcID-Offset-r16                  INTEGER (0..15)                                             OPTIONAL,   -- Need M

harq-ProcID-Offset2-r16                 INTEGER (0..15)                                             OPTIONAL,   -- Need M

configuredGrantConfigIndex-r16          ConfiguredGrantConfigIndex-r16                              OPTIONAL,   -- Cond CG-List

configuredGrantConfigIndexMAC-r16       ConfiguredGrantConfigIndexMAC-r16                           OPTIONAL,   -- Cond CG-IndexMAC

periodicityExt-r16                      INTEGER (1..5120)                                           OPTIONAL,   -- Need R

startingFromRV0-r16                     ENUMERATED {on, off}                                        OPTIONAL,   -- Need R

phy-PriorityIndex-r16                   ENUMERATED {p0, p1}                                         OPTIONAL,   -- Need R

autonomousTx-r16                        ENUMERATED {enabled}                                        OPTIONAL    -- Cond LCH-BasedPrioritization

]]

[[

cg-betaOffsetsCrossPri0-r17                SetupRelease { BetaOffsetsCrossPriSelCG-r17 }                     OPTIONAL,  -- Need M

cg-betaOffsetsCrossPri1-r17                SetupRelease { BetaOffsetsCrossPriSelCG-r17 }                     OPTIONAL  -- Need M

]]

}

CG-UCI-OnPUSCH ::= CHOICE {

dynamic                                 SEQUENCE (SIZE (1..4)) OF BetaOffsets,

semiStatic                              BetaOffsets

}

CG-COT-Sharing-r16 ::= CHOICE {

noCOT-Sharing-r16                   NULL,

cot-Sharing-r16                     SEQUENCE {

duration-r16                       INTEGER (1..39),

offset-r16                         INTEGER (1..39),

channelAccessPriority-r16          INTEGER (1..4)

}

}

CG-StartingOffsets-r16 ::= SEQUENCE {

cg-StartingFullBW-InsideCOT-r16         SEQUENCE (SIZE (1..7)) OF INTEGER (0..6)             OPTIONAL,   -- Need R

cg-StartingFullBW-OutsideCOT-r16        SEQUENCE (SIZE (1..7)) OF INTEGER (0..6)             OPTIONAL,   -- Need R

cg-StartingPartialBW-InsideCOT-r16      INTEGER (0..6)                                       OPTIONAL,   -- Need R

cg-StartingPartialBW-OutsideCOT-r16     INTEGER (0..6)                                       OPTIONAL    -- Need R

}

BetaOffsetsCrossPriSelCG-r17 ::= CHOICE {

dynamic-r17         SEQUENCE (SIZE (1..4)) OF BetaOffsetsCrossPri-r17,

semiStatic-r17      BetaOffsetsCrossPri-r17

}

-- TAG-CONFIGUREDGRANTCONFIG-STOP

-- ASN1STOP

ConfiguredGrantConfig field descriptions

antennaPort Indicates the antenna port(s) to be used for this configuration, and the maximum bitwidth is 5. See TS 38.214 [19], clause 6.1.2, and TS 38.212 [17], clause 7.3.1.

autonomousTx If this field is present, the Configured Grant configuration is configured with autonomous transmission, see TS 38.321 [3].

betaOffsetCG-UCI Beta offset for CG-UCI in CG-PUSCH, see TS 38.213 [13], clause 9.3

cg-betaOffsetsCrossPri0, cg-betaOffsetsCrossPri1 Selection between and configuration of dynamic and semi-static beta-offset for multiplexing HARQ-ACK in CG-PUSCH with different priorities. The field cg-betaOffsetsCrossPri0 indicates multiplexing LP HARQ-ACK in HP CG-PUSCH. This field is configured only if phy-PriorityIndex-r16 is configured with value p1. The field cg-betaOffsetsCrossPri1 indicates multiplexing HP HARQ-ACK in LP CG-PUSCH. This field is configured only if phy-PriorityIndex-r16 is configured with value p0.

cg-COT-SharingList Indicates a table for COT sharing combinations (see 37.213 [48], clause 4.1.3). One row of the table can be set to noCOT-Sharing to indicate that there is no channel occupancy sharing. If the cg-RetransmissionTimer-r16 is configured and the UE operates as an initiating device in semi-static channel access mode (see TS 37.213 [48], clause 4.3), then cg-COT-SharingList-r16 is configured and the UE ignores the field channelAccessPriority-r16. Editor’s note (IIoT): The last part “the UE ignores the field channelAccessPriority-r16” may be already implemented by RAN1 spec 37.213. RAN2 to discuss further if it needs to be removed to avoid mis-interpretation.

cg-COT-SharingOffset Indicates the offset from the end of the slot where the COT sharing indication in UCI is enabled where the offset in symbols is equal to 14*n, where n is the signaled value for cg-COT-SharingOffset. Applicable when ul-toDL-COT-SharingED-Threshold-r16 is not configured (see 37.213 [48], clause 4.1.3).

cg-DMRS-Configuration DMRS configuration (see TS 38.214 [19], clause 6.1.2.3).

cg-minDFI-Delay Indicates the minimum duration (in unit of symbols) from the ending symbol of the PUSCH to the starting symbol of the PDCCH containing the downlink feedback indication (DFI) carrying HARQ-ACK for this PUSCH. The HARQ-ACK received before this minimum duration is not considered as valid for this PUSCH (see TS 38.213 [13], clause 10.5). The following minimum duration values are supported, depending on the configured subcarrier spacing [symbols]: 15 kHz: 7, m*14, where m = {1, 2, 3, 4} 30 kHz: 7, m*14, where m = {1, 2, 3, 4, 5, 6, 7, 8} 60 kHz: 7, m*14, where m = {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16}

cg-nrofPUSCH-InSlot Indicates the number of consecutive PUSCH configured to CG within a slot where the SLIV indicating the first PUSCH and additional PUSCH appended with the same length (see TS 38.214 [19], clause 6.1.2.3).

cg-nrofSlots Indicates the number of allocated slots in a configured grant periodicity following the time instance of configured grant offset (see TS 38.214 [19], clause 6.1.2.3).

cg-RetransmissionTimer Indicates the initial value of the configured retransmission timer (see TS 38.321 [3]) in multiples of periodicity. The value of cg-RetransmissionTimer is always less than or equal to the value of configuredGrantTimer. This field is always configured together with harq-ProcID-Offset. This field is not configured for operation in licensed spectrum or simultaneously with harq-ProcID-Offset2.

cg-StartingOffsets This field is not applicable for a UE which is allowed to operate as an initiating device in semi-static channel access mode, i.e., not applicable for a UE configured with UE FFP parameters (e.g. period, offset) regardless whether the UE would initiate its own COT or would share gNB’s COT.

cg-UCI-Multiplexing If present, this field indicates that in the case of PUCCH overlapping with CG-PUSCH(s) within a PUCCH group, the CG-UCI and HARQ-ACK are jointly encoded (see TS 38.213 [13], clause 9).

configuredGrantConfigIndex Indicates the index of the Configured Grant configurations within the BWP.

configuredGrantConfigIndexMAC Indicates the index of the Configured Grant configurations within the MAC entity.

configuredGrantTimer Indicates the initial value of the configured grant timer (see TS 38.321 [3]) in multiples of periodicity. When cg-RetransmissonTimer is configured, if HARQ processes are shared among different configured grants on the same BWP, configuredGrantTimer * periodicity is set to the same value for the configurations that share HARQ processes on this BWP.

dmrs-SeqInitialization The network configures this field if transformPrecoder is disabled. Otherwise the field is absent.

frequencyDomainAllocation Indicates the frequency domain resource allocation, see TS 38.214 [19], clause 6.1.2, and TS 38.212 [17], clause 7.3.1).

frequencyHopping The value intraSlot enables 'Intra-slot frequency hopping' and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured. The field frequencyHopping applies to configured grant for 'pusch-RepTypeA' (see TS 38.214 [19], clause 6.3.1).

frequencyHoppingOffset Frequency hopping offset used when frequency hopping is enabled (see TS 38.214 [19], clause 6.1.2 and clause 6.3).

frequencyHoppingPUSCH-RepTypeB Indicates the frequency hopping scheme for Type 1 CG when pusch-RepTypeIndicator is set to 'pusch-RepTypeB' (see TS 38.214 [19], clause 6.1). The value interRepetition enables 'Inter-repetition frequency hopping', and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, the frequency hopping is not enabled for Type 1 CG.

harq-ProcID-Offset For operation with shared spectrum channel access configured with cg-RetransmissionTimer-r16, this configures the range of HARQ process IDs which can be used for this configured grant where the UE can select a HARQ process ID within [harq-procID-offset, .., (harq-procID-offset + nrofHARQ-Processes – 1)].

harq-ProcID-Offset2 Indicates the offset used in deriving the HARQ process IDs, see TS 38.321 [3], clause 5.4.1. This field is not configured together with cg-RetransmissionTimer-r16.

mcs-Table Indicates the MCS table the UE shall use for PUSCH without transform precoding. If the field is absent the UE applies the value qam64.

mcs-TableTransformPrecoder Indicates the MCS table the UE shall use for PUSCH with transform precoding. If the field is absent the UE applies the value qam64.

mcsAndTBS The modulation order, target code rate and TB size (see TS 38.214 [19], clause 6.1.2). The NW does not configure the values 28~31 in this version of the specification.

nrofHARQ-Processes The number of HARQ processes configured. It applies for both Type 1 and Type 2. See TS 38.321 [3], clause 5.4.1.

p0-PUSCH-Alpha Index of the P0-PUSCH-AlphaSet to be used for this configuration.

periodicity Periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5.8.2). The following periodicities are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 320, 640} 30 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 640, 1280} 60 kHz with normal CP 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 60 kHz with ECP: 2, 6, n*12, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 120 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120}

periodicityExt This field is used to calculate the periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5,8.2). If this field is present, the field periodicity is ignored. The following periodicites are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 640. 30 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 1280. 60 kHz with normal CP: periodicityExt*14, where periodicityExt has a value between 1 and 2560. 60 kHz with ECP: periodicityExt*12, where periodicityExt has a value between 1 and 2560. 120 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 5120.

phy-PriorityIndex Indicates the PHY priority of CG PUSCH at least for PHY-layer collision handling. Value p0 indicates low priority and value p1 indicates high priority.

powerControlLoopToUse Closed control loop to apply (see TS 38.213 [13], clause 7.1.1).

pusch-RepTypeIndicator Indicates whether UE follows the behavior for PUSCH repetition type A or the behavior for PUSCH repetition type B for each Type 1 configured grant configuration. The value pusch-RepTypeA enables the 'PUSCH repetition type A' and the value pusch-RepTypeB enables the 'PUSCH repetition type B' (see TS 38.214 [19], clause 6.1.2.3). The value pusch-RepTypeB is not configured simultaneously with cg-nrofPUSCH-InSlot-r16 and cg-nrofSlots-r16. The network does not configure this field if cg-RetransmissionTimer-r16 is configured for CG operation with shared spectrum channel access.

rbg-Size Selection between configuration 1 and configuration 2 for RBG size for PUSCH. The UE does not apply this field if resourceAllocation is set to resourceAllocationType1. Otherwise, the UE applies the value config1 when the field is absent. Note: rbg-Size is used when the transformPrecoder parameter is disabled.

repK-RV The redundancy version (RV) sequence to use. See TS 38.214 [19], clause 6.1.2. The network configures this field if repetitions are used, i.e., if repK is set to n2, n4 or n8. This field is not configured when cg-RetransmissionTimer is configured. Otherwise, the field is absent.

repK Number of repetitions K, see TS 38.214 [19].

resourceAllocation Configuration of resource allocation type 0 and resource allocation type 1. For Type 1 UL data transmission without grant, resourceAllocation should be resourceAllocationType0 or resourceAllocationType1.

rrc-ConfiguredUplinkGrant Configuration for "configured grant" transmission with fully RRC-configured UL grant (Type1). If this field is absent the UE uses UL grant configured by DCI addressed to CS-RNTI (Type2).

srs-ResourceIndicator Indicates the SRS resource to be used.

startingFromRV0 This field is used to determine the initial transmission occasion of a transport block for a given RV sequence, see TS 38.214 [19], clause 6.1.2.3.1. The network does not configure this field if cg-RetransmissionTimer-r16 is configured for CG operation.

timeDomainAllocation Indicates a combination of start symbol and length and PUSCH mapping type, see TS 38.214 [19], clause 6.1.2 and TS 38.212 [17], clause 7.3.1.

timeDomainOffset Offset related to the reference SFN indicated by timeReferenceSFN, see TS 38.321 [3], clause 5.8.2.

timeReferenceSFN Indicates SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration, see TS 38.321 [3], clause 5.8.2. If the field timeReferenceSFN is not present, the reference SFN is 0.

transformPrecoder Enables or disables transform precoding for type1 and type2. If the field is absent, the UE enables or disables transform precoding in accordance with the field msg3-transformPrecoder in RACH-ConfigCommon, see TS 38.214 [19], clause 6.1.3.

uci-OnPUSCH Selection between and configuration of dynamic and semi-static beta-offset. For Type 1 UL data transmission without grant, uci-OnPUSCH should be set to semiStatic.

CG-COT-Sharing field descriptions

channelAccessPriority Indicates the Channel Access Priority Class that the gNB can assume when sharing the UE initiated COT (see 37.213 [48], clause 4.1.3).

duration Indicates the number of DL transmission slots within UE initiated COT (see 37.213 [48], clause 4.1.3).

offset Indicates the number of DL transmission slots from the end of the slot where CG-UCI is detected after which COT sharing can be used (see 37.213 [48], clause 4.1.3).

CG-StartingOffsets field descriptions

cg-StartingFullBW-InsideCOT A set of configured grant PUSCH transmission starting offsets which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation includes all interlaces in the allocated RB set(s) and the CG PUSCH resource is inside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

cg-StartingFullBW-OutsideCOT A set of configured grant PUSCH transmission starting offset indices (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation includes all interlaces in the allocated RB set(s) and the CG PUSCH resource is outside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

cg-StartingPartialBW-InsideCOT A set of configured grant PUSCH transmission starting offset index (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation does not include all interlaces in the allocated RB set(s) and the CG PUSCH resource is inside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

cg-StartingPartialBW-OutsideCOT A set of configured grant PUSCH transmission starting offset index (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation does not include all interlaces in the allocated RB set(s) and the CG PUSCH resource is outside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

| Conditional Presence | Explanation |
| --- | --- |
| LCH-BasedPrioritization | This field is optionally present, Need R, if lch-BasedPrioritization is configured in the MAC entity. It is absent otherwise. |
| RepTypeB | The field is optionally present if pusch-RepTypeIndicator is set to pusch-RepTypeB, Need S, and absent otherwise. |
| CG-List | The field is mandatory present when included in configuredGrantConfigToAddModList-r16, otherwise the field is absent. |
| CG-IndexMAC | The field is mandatory present if at least one configured grant is configured by configuredGrantConfigToAddModList-r16 in any BWP of this MAC entity, otherwise it is optionally present, need R. |

next change

–	CSI-ReportConfig

The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). See TS 38.214 [19], clause 5.2.1.

CSI-ReportConfig information element

-- ASN1START

-- TAG-CSI-REPORTCONFIG-START

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

carrier                                 ServCellIndex                   OPTIONAL,   -- Need S

resourcesForChannelMeasurement          CSI-ResourceConfigId,

csi-IM-ResourcesForInterference         CSI-ResourceConfigId            OPTIONAL,   -- Need R

nzp-CSI-RS-ResourcesForInterference     CSI-ResourceConfigId            OPTIONAL,   -- Need R

reportConfigType                        CHOICE {

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

},

reportQuantity                          CHOICE {

none                                    NULL,

cri-RI-PMI-CQI                          NULL,

cri-RI-i1                               NULL,

cri-RI-i1-CQI                           SEQUENCE {

pdsch-BundleSizeForCSI                  ENUMERATED {n2, n4}                                         OPTIONAL    -- Need S

},

cri-RI-CQI                              NULL,

cri-RSRP                                NULL,

ssb-Index-RSRP                          NULL,

cri-RI-LI-PMI-CQI                       NULL

},

reportFreqConfiguration                 SEQUENCE {

cqi-FormatIndicator                     ENUMERATED { widebandCQI, subbandCQI }                          OPTIONAL,   -- Need R

pmi-FormatIndicator                     ENUMERATED { widebandPMI, subbandPMI }                          OPTIONAL,   -- Need R

csi-ReportingBand                       CHOICE {

subbands3                               BIT STRING(SIZE(3)),

subbands4                               BIT STRING(SIZE(4)),

subbands5                               BIT STRING(SIZE(5)),

subbands6                               BIT STRING(SIZE(6)),

subbands7                               BIT STRING(SIZE(7)),

subbands8                               BIT STRING(SIZE(8)),

subbands9                               BIT STRING(SIZE(9)),

subbands10                              BIT STRING(SIZE(10)),

subbands11                              BIT STRING(SIZE(11)),

subbands12                              BIT STRING(SIZE(12)),

subbands13                              BIT STRING(SIZE(13)),

subbands14                              BIT STRING(SIZE(14)),

subbands15                              BIT STRING(SIZE(15)),

subbands16                              BIT STRING(SIZE(16)),

subbands17                              BIT STRING(SIZE(17)),

subbands18                              BIT STRING(SIZE(18)),

...,

subbands19-v1530                        BIT STRING(SIZE(19))

}   OPTIONAL    -- Need S

}                                                                                                           OPTIONAL,   -- Need R

timeRestrictionForChannelMeasurements           ENUMERATED {configured, notConfigured},

timeRestrictionForInterferenceMeasurements      ENUMERATED {configured, notConfigured},

codebookConfig                                  CodebookConfig                                              OPTIONAL,   -- Need R

dummy                                           ENUMERATED {n1, n2}                                         OPTIONAL,   -- Need R

groupBasedBeamReporting                     CHOICE {

enabled                                     NULL,

disabled                                    SEQUENCE {

nrofReportedRS                          ENUMERATED {n1, n2, n3, n4}                                 OPTIONAL    -- Need S

}

},

cqi-Table                   ENUMERATED {table1, table2, table3, spare1}                                     OPTIONAL,   -- Need R

subbandSize                 ENUMERATED {value1, value2},

non-PMI-PortIndication      SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks OPTIONAL,   -- Need R

...,

[[

semiPersistentOnPUSCH-v1530         SEQUENCE {

reportSlotConfig-v1530              ENUMERATED {sl4, sl8, sl16}

}                                                                                                           OPTIONAL    -- Need R

]],

[[

semiPersistentOnPUSCH-v1610         SEQUENCE {

reportSlotOffsetListDCI-0-2-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL,    -- Need R

reportSlotOffsetListDCI-0-1-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL     -- Need R

}                                                                                                           OPTIONAL,    -- Need R

aperiodic-v1610                     SEQUENCE {

reportSlotOffsetListDCI-0-2-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL,    -- Need R

reportSlotOffsetListDCI-0-1-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL     -- Need R

}                                                                                                           OPTIONAL,    -- Need R

reportQuantity-r16                  CHOICE {

cri-SINR-r16                         NULL,

ssb-Index-SINR-r16                   NULL

}                                                                                                           OPTIONAL,   -- Need R

codebookConfig-r16                          CodebookConfig-r16                                              OPTIONAL,    -- Need R

]]

[[

cqi-BitsPerSubband-r17              ENUMERATED {bits4}                                           OPTIONAL   -- Need R

]]

}

CSI-ReportPeriodicityAndOffset ::=  CHOICE {

slots4                              INTEGER(0..3),

slots5                              INTEGER(0..4),

slots8                              INTEGER(0..7),

slots10                             INTEGER(0..9),

slots16                             INTEGER(0..15),

slots20                             INTEGER(0..19),

slots40                             INTEGER(0..39),

slots80                             INTEGER(0..79),

slots160                            INTEGER(0..159),

slots320                            INTEGER(0..319)

}

PUCCH-CSI-Resource ::=              SEQUENCE {

uplinkBandwidthPartId               BWP-Id,

pucch-Resource                      PUCCH-ResourceId

}

PortIndexFor8Ranks ::=              CHOICE {

portIndex8                          SEQUENCE{

rank1-8                             PortIndex8                                                      OPTIONAL,   -- Need R

rank2-8                             SEQUENCE(SIZE(2)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank3-8                             SEQUENCE(SIZE(3)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank4-8                             SEQUENCE(SIZE(4)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank5-8                             SEQUENCE(SIZE(5)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank6-8                             SEQUENCE(SIZE(6)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank7-8                             SEQUENCE(SIZE(7)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank8-8                             SEQUENCE(SIZE(8)) OF PortIndex8                                 OPTIONAL    -- Need R

},

portIndex4                          SEQUENCE{

rank1-4                             PortIndex4                                                      OPTIONAL,   -- Need R

rank2-4                             SEQUENCE(SIZE(2)) OF PortIndex4                                 OPTIONAL,   -- Need R

rank3-4                             SEQUENCE(SIZE(3)) OF PortIndex4                                 OPTIONAL,   -- Need R

rank4-4                             SEQUENCE(SIZE(4)) OF PortIndex4                                 OPTIONAL    -- Need R

},

portIndex2                          SEQUENCE{

rank1-2                             PortIndex2                                                      OPTIONAL,   -- Need R

rank2-2                             SEQUENCE(SIZE(2)) OF PortIndex2                                 OPTIONAL    -- Need R

},

portIndex1                          NULL

}

PortIndex8::=                       INTEGER (0..7)

PortIndex4::=                       INTEGER (0..3)

PortIndex2::=                       INTEGER (0..1)

-- TAG-CSI-REPORTCONFIG-STOP

-- ASN1STOP

CSI-ReportConfig field descriptions

carrier Indicates in which serving cell the CSI-ResourceConfig indicated below are to be found. If the field is absent, the resources are on the same serving cell as this report configuration.

codebookConfig Codebook configuration for Type-1 or Type-2 including codebook subset restriction. Network does not configure codebookConfig and codebookConfig-r16 simultaneously to a UE

cqi-BitsPerSubband This field can only be present if cqi-FormatIndicator is set to subbandCQI. If the field is configured with bits4, the UE uses 4-bit sub-band CQI. If the field is not present and cqi-FormatIndicator is set to subbandCQI, the UE uses 2-bit sub-band differential CQI.

cqi-FormatIndicator Indicates whether the UE shall report a single (wideband) or multiple (subband) CQI (see TS 38.214 [19], clause 5.2.1.4).

cqi-Table Which CQI table to use for CQI calculation (see TS 38.214 [19], clause 5.2.2.1).

csi-IM-ResourcesForInterference CSI IM resources for interference measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only CSI-IM resources. The bwp-Id in that CSI-ResourceConfig is the same value as the bwp-Id in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement.

csi-ReportingBand Indicates a contiguous or non-contiguous subset of subbands in the bandwidth part which CSI shall be reported for. Each bit in the bit-string represents one subband. The right-most bit in the bit string represents the lowest subband in the BWP. The choice determines the number of subbands (subbands3 for 3 subbands, subbands4 for 4 subbands, and so on) (see TS 38.214 [19], clause 5.2.1.4). This field is absent if there are less than 24 PRBs (no sub band) and present otherwise (see TS 38.214 [19], clause 5.2.1.4).

dummy This field is not used in the specification. If received it shall be ignored by the UE.

groupBasedBeamReporting Turning on/off group beam based reporting (see TS 38.214 [19], clause 5.2.1.4).

non-PMI-PortIndication Port indication for RI/CQI calculation. For each CSI-RS resource in the linked ResourceConfig for channel measurement, a port indication for each rank R, indicating which R ports to use. Applicable only for non-PMI feedback (see TS 38.214 [19], clause 5.2.1.4.2). The first entry in non-PMI-PortIndication corresponds to the NZP-CSI-RS-Resource indicated by the first entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the CSI-ResourceConfig whose CSI-ResourceConfigId is indicated in a CSI-MeasId together with the above CSI-ReportConfigId; the second entry in non-PMI-PortIndication corresponds to the NZP-CSI-RS-Resource indicated by the second entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig, and so on until the NZP-CSI-RS-Resource indicated by the last entry in nzp-CSI-RS-Resources in the in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig. Then the next entry corresponds to the NZP-CSI-RS-Resource indicated by the first entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the second entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig and so on.

nrofReportedRS The number (N) of measured RS resources to be reported per report setting in a non-group-based report. N <= N_max, where N_max is either 2 or 4 depending on UE capability. (see TS 38.214 [19], clause 5.2.1.4) When the field is absent the UE applies the value 1.

nzp-CSI-RS-ResourcesForInterference NZP CSI RS resources for interference measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only NZP-CSI-RS resources. The bwp-Id in that CSI-ResourceConfig is the same value as the bwp-Id in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement.

p0alpha Index of the p0-alpha set determining the power control for this CSI report transmission (see TS 38.214 [19], clause 6.2.1.2).

pdsch-BundleSizeForCSI PRB bundling size to assume for CQI calculation when reportQuantity is CRI/RI/i1/CQI. If the field is absent, the UE assumes that no PRB bundling is applied (see TS 38.214 [19], clause 5.2.1.4.2).

pmi-FormatIndicator Indicates whether the UE shall report a single (wideband) or multiple (subband) PMI. (see TS 38.214 [19], clause 5.2.1.4).

pucch-CSI-ResourceList Indicates which PUCCH resource to use for reporting on PUCCH.

reportConfigType Time domain behavior of reporting configuration.

reportFreqConfiguration Reporting configuration in the frequency domain. (see TS 38.214 [19], clause 5.2.1.4).

reportQuantity The CSI related quantities to report. see TS 38.214 [19], clause 5.2.1. If the field reportQuantity-r16 is present, UE shall ignore reportQuantity (without suffix).

reportSlotConfig Periodicity and slot offset (see TS 38.214 [19], clause 5.2.1.4). If the field reportSlotConfig-v1530 is present, the UE shall ignore the value provided in reportSlotConfig (without suffix).

reportSlotOffsetList, reportSlotOffsetListDCI-0-1, reportSlotOffsetListDCI-0-2 Timing offset Y for semi persistent reporting using PUSCH. This field lists the allowed offset values. This list must have the same number of entries as the pusch-TimeDomainAllocationList in PUSCH-Config. A particular value is indicated in DCI. The network indicates in the DCI field of the UL grant, which of the configured report slot offsets the UE shall apply. The DCI value 0 corresponds to the first report slot offset in this list, the DCI value 1 corresponds to the second report slot offset in this list, and so on. The first report is transmitted in slot n+Y, second report in n+Y+P, where P is the configured periodicity. Timing offset Y for aperiodic reporting using PUSCH. This field lists the allowed offset values. This list must have the same number of entries as the pusch-TimeDomainAllocationList in PUSCH-Config. A particular value is indicated in DCI. The network indicates in the DCI field of the UL grant, which of the configured report slot offsets the UE shall apply. The DCI value 0 corresponds to the first report slot offset in this list, the DCI value 1 corresponds to the second report slot offset in this list, and so on (see TS 38.214 [19], clause 6.1.2.1). The field reportSlotOffsetListDCI-0-1 applies to DCI format 0_1 and the field reportSlotOffsetListDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.2.1).

resourcesForChannelMeasurement Resources for channel measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only NZP-CSI-RS resources and/or SSB resources. This CSI-ReportConfig is associated with the DL BWP indicated by bwp-Id in that CSI-ResourceConfig.

subbandSize Indicates one out of two possible BWP-dependent values for the subband size as indicated in TS 38.214 [19], table 5.2.1.4-2 . If csi-ReportingBand is absent, the UE shall ignore this field.

timeRestrictionForChannelMeasurements Time domain measurement restriction for the channel (signal) measurements (see TS 38.214 [19], clause 5.2.1.1).

timeRestrictionForInterferenceMeasurements Time domain measurement restriction for interference measurements (see TS 38.214 [19], clause 5.2.1.1).

PortIndexFor8Ranks field descriptions

portIndex8 Port-Index configuration for up to rank 8. If present, the network configures port indexes for at least one of the ranks.

portIndex4 Port-Index configuration for up to rank 4. If present, the network configures port indexes for at least one of the ranks.

portIndex2 Port-Index configuration for up to rank 2. If present, the network configures port indexes for at least one of the ranks.

portIndex1 Port-Index configuration for rank 1.

PUCCH-CSI-Resource field descriptions

pucch-Resource PUCCH resource for the associated uplink BWP. Only PUCCH-Resource of format 2, 3 and 4 is supported. The actual PUCCH-Resource is configured in PUCCH-Config and referred to by its ID. When two PUCCH-Config are configured within PUCCH-ConfigurationList, PUCCH-ResourceId in a PUCCH-CSI-Resource refers to a PUCCH-Resource in the PUCCH-Config used for HARQ-ACK with low priority.

next change

–	MAC-CellGroupConfig

The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX.

MAC-CellGroupConfig information element

-- ASN1START

-- TAG-MAC-CELLGROUPCONFIG-START

MAC-CellGroupConfig ::=             SEQUENCE {

drx-Config                          SetupRelease { DRX-Config }                                     OPTIONAL,   -- Need M

schedulingRequestConfig             SchedulingRequestConfig                                         OPTIONAL,   -- Need M

bsr-Config                          BSR-Config                                                      OPTIONAL,   -- Need M

tag-Config                          TAG-Config                                                      OPTIONAL,   -- Need M

phr-Config                          SetupRelease { PHR-Config }                                     OPTIONAL,   -- Need M

skipUplinkTxDynamic                 BOOLEAN,

...,

[[

csi-Mask                            BOOLEAN                                                         OPTIONAL,   -- Need M

dataInactivityTimer                 SetupRelease { DataInactivityTimer }                            OPTIONAL    -- Cond MCG-Only

]],

[[

usePreBSR-r16                       ENUMERATED {true}                                               OPTIONAL,   -- Need R

schedulingRequestID-LBT-SCell-r16   SchedulingRequestId                                             OPTIONAL,   -- Need R

lch-BasedPrioritization-r16         ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

schedulingRequestID-BFR-SCell-r16   SchedulingRequestId                                             OPTIONAL,   -- Need R

drx-ConfigSecondaryGroup-r16        SetupRelease { DRX-ConfigSecondaryGroup }                       OPTIONAL    -- Need M

]],

[[

enhancedSkipUplinkTxDynamic-r16     ENUMERATED {true}                                               OPTIONAL,   -- Need R

enhancedSkipUplinkTxConfigured-r16  ENUMERATED {true}                                               OPTIONAL,    -- Need R

]]

[[

intraCG-Prioritization-r17              ENUMERATED {enabled}                                        OPTIONAL    -- Cond LCH-PrioWithReTxTimer

]]

}

DataInactivityTimer ::=         ENUMERATED {s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60, s80, s100, s120, s150, s180}

-- TAG-MAC-CELLGROUPCONFIG-STOP

-- ASN1STOP

MAC-CellGroupConfig field descriptions

usePreBSR If set to true, the MAC entity of the IAB-MT may use the Pre-emptive BSR, see TS 38.321 [3].

csi-Mask If set to true, the UE limits CSI reports to the on-duration period of the DRX cycle, see TS 38.321 [3].

dataInactivityTimer Releases the RRC connection upon data inactivity as specified in clause 5.3.8.5 and in TS 38.321 [3]. Value s1 corresponds to 1 second, value s2 corresponds to 2 seconds, and so on.

drx-Config Used to configure DRX as specified in TS 38.321 [3].

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

lch-BasedPrioritization If this field is present, the corresponding MAC entity of the UE is configured with prioritization between overlapping grants and between scheduling request and overlapping grants based on LCH priority, see TS 38.321 [3].

schedulingRequestID-BFR-SCell Indicates the scheduling request configuration applicable for BFR on SCell, as specified in TS 38.321 [3].

schedulingRequestID-LBT-SCell Indicates the scheduling request configuration applicable for consistent uplink LBT recovery on SCell, as specified in TS 38.321 [3].

skipUplinkTxDynamic, enhancedSkipUplinkTxDynamic, enhancedSkipUplinkTxConfigured If set to true, the UE skips UL transmissions as described in TS 38.321 [3]. If the UE is configured with enhancedSkipUplinkTxDynamic or enhancedSkipUplinkTxConfigured with value true, REPETITION_NUMBER (as specified in TS 38.321 [3], clause 5.4.2.1) of the corresponding PUSCH transmission of the uplink grant shall be equal to 1.

tag-Config The field is used to configure parameters for a time-alignment group. The field is not present if any DAPS bearer is configured.

| Conditional Presence | Explanation |
| --- | --- |
| MCG-Only | This field is optionally present, Need M, for the MAC-CellGroupConfig of the MCG. It is absent otherwise. |
| LCH-PrioWithReTxTimer | This field is optionally present, Need R, if lch-BasedPrioritization-r16 is configured in this MAC entity and cg-RetransmissionTimer-r16 is configured for any configured grant configuration associated with this MAC entity. It is absent otherwise. |

next change

–	MeasObjectRxTxDiff

The IE MeasObjectRxTxDiff is used to configure the measurement object for UE Rx-Tx time difference measurement.

MeasObjectRxTxDiff information element

-- ASN1START

-- TAG-MEASOBJECTRXTXDIFF-START

MeasObjectRxTxDiff-r17 ::=      SEQUENCE {

dl-Ref-r17      CHOICE {

prs-Ref-r17             null,

csi-RS-Ref-r17          null,

...

}                               OPTIONAL,   -- Need R

...

}

-- TAG-MEASOBJECTRXTXDIFF-STOP

-- ASN1STOP

MeasObjectRxTxDiff field descriptions

dl-Ref-r17 configures the DL references signals to measure Rx-Tx time difference. prs-Ref-r17 indicates PRS is chosen, and csi-RS-Ref-17 indicates that CSI-RS for tracking is chosen. Only one PRS resource is configured by the network. Only one NZP-CSI-RS-ResourceSet can be configured with pdc-Info-r17 set to true and it is used for UE Rx-Tx time difference measurement. Only reference signals from the PCell of the MCG can be configured by the network.

next change

–	MeasObjectToAddModList

The IE MeasObjectToAddModList concerns a list of measurement objects to add or modify.

MeasObjectToAddModList information element

-- ASN1START

-- TAG-MEASOBJECTTOADDMODLIST-START

MeasObjectToAddModList ::=                  SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectToAddMod

MeasObjectToAddMod ::=                      SEQUENCE {

measObjectId                                MeasObjectId,

measObject                                  CHOICE {

measObjectNR                                MeasObjectNR,

...,

measObjectEUTRA                             MeasObjectEUTRA,

measObjectUTRA-FDD-r16                      MeasObjectUTRA-FDD-r16,

measObjectNR-SL-r16                         MeasObjectNR-SL-r16,

measObjectCLI-r16                           MeasObjectCLI-r16,

measObjectRxTxDiff-r17                      MeasObjectRxTxDiff-r17

}

}

-- TAG-MEASOBJECTTOADDMODLIST-STOP

-- ASN1STOP

next change

–	MeasResults

The IE MeasResults covers measured results for intra-frequency, inter-frequency, inter-RAT mobility and measured results for NR sidelink communication.

MeasResults information element

-- ASN1START

-- TAG-MEASRESULTS-START

MeasResults ::=                         SEQUENCE {

measId                                  MeasId,

measResultServingMOList                 MeasResultServMOList,

measResultNeighCells                    CHOICE {

measResultListNR                        MeasResultListNR,

...,

measResultListEUTRA                     MeasResultListEUTRA,

measResultListUTRA-FDD-r16              MeasResultListUTRA-FDD-r16

}                                                                                                                   OPTIONAL,

...,

[[

measResultServFreqListEUTRA-SCG         MeasResultServFreqListEUTRA-SCG                                             OPTIONAL,

measResultServFreqListNR-SCG            MeasResultServFreqListNR-SCG                                                OPTIONAL,

measResultSFTD-EUTRA                    MeasResultSFTD-EUTRA                                                        OPTIONAL,

measResultSFTD-NR                       MeasResultCellSFTD-NR                                                       OPTIONAL

]],

[[

measResultCellListSFTD-NR               MeasResultCellListSFTD-NR                                                   OPTIONAL

]],

[[

measResultForRSSI-r16                   MeasResultForRSSI-r16                                                       OPTIONAL,

locationInfo-r16                        LocationInfo-r16                                                            OPTIONAL,

ul-PDCP-DelayValueResultList-r16        UL-PDCP-DelayValueResultList-r16                                            OPTIONAL,

measResultsSL-r16                       MeasResultsSL-r16                                                           OPTIONAL,

measResultCLI-r16                       MeasResultCLI-r16                                                           OPTIONAL,

]]

[[

measResultRxTxTimeDiff-r17             MeasResultRxTxTimeDiff-r17                                                   OPTIONAL

]]

}

MeasResultServMOList ::=                SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO

MeasResultServMO ::=                    SEQUENCE {

servCellId                              ServCellIndex,

measResultServingCell                   MeasResultNR,

measResultBestNeighCell                 MeasResultNR                                                                OPTIONAL,

...

}

MeasResultListNR ::=                    SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR

MeasResultNR ::=                        SEQUENCE {

physCellId                              PhysCellId                                                                  OPTIONAL,

measResult                              SEQUENCE {

cellResults                             SEQUENCE{

resultsSSB-Cell                         MeasQuantityResults                                                 OPTIONAL,

resultsCSI-RS-Cell                      MeasQuantityResults                                                 OPTIONAL

},

rsIndexResults                          SEQUENCE{

resultsSSB-Indexes                      ResultsPerSSB-IndexList                                             OPTIONAL,

resultsCSI-RS-Indexes                   ResultsPerCSI-RS-IndexList                                          OPTIONAL

}                                                                                                               OPTIONAL

},

...,

[[

cgi-Info                                CGI-InfoNR                                                                    OPTIONAL

]]

}

MeasResultListEUTRA ::=                 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA

MeasResultEUTRA ::=                     SEQUENCE {

eutra-PhysCellId                        PhysCellId,

measResult                              MeasQuantityResultsEUTRA,

cgi-Info                                CGI-InfoEUTRA                                                               OPTIONAL,

...

}

MultiBandInfoListEUTRA ::=              SEQUENCE (SIZE (1..maxMultiBands)) OF FreqBandIndicatorEUTRA

MeasQuantityResults ::=                 SEQUENCE {

rsrp                                    RSRP-Range                                                                  OPTIONAL,

rsrq                                    RSRQ-Range                                                                  OPTIONAL,

sinr                                    SINR-Range                                                                  OPTIONAL

}

MeasQuantityResultsEUTRA ::=            SEQUENCE {

rsrp                                    RSRP-RangeEUTRA                                                             OPTIONAL,

rsrq                                    RSRQ-RangeEUTRA                                                             OPTIONAL,

sinr                                    SINR-RangeEUTRA                                                             OPTIONAL

}

ResultsPerSSB-IndexList::=              SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index

ResultsPerSSB-Index ::=                 SEQUENCE {

ssb-Index                               SSB-Index,

ssb-Results                             MeasQuantityResults                                                         OPTIONAL

}

ResultsPerCSI-RS-IndexList::=           SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerCSI-RS-Index

ResultsPerCSI-RS-Index ::=              SEQUENCE {

csi-RS-Index                            CSI-RS-Index,

csi-RS-Results                          MeasQuantityResults                                                         OPTIONAL

}

MeasResultServFreqListEUTRA-SCG ::= SEQUENCE (SIZE (1..maxNrofServingCellsEUTRA)) OF MeasResult2EUTRA

MeasResultServFreqListNR-SCG ::= SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResult2NR

MeasResultListUTRA-FDD-r16 ::=          SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultUTRA-FDD-r16

MeasResultUTRA-FDD-r16 ::=              SEQUENCE {

physCellId-r16                          PhysCellIdUTRA-FDD-r16,

measResult-r16                          SEQUENCE {

utra-FDD-RSCP-r16                       INTEGER (-5..91)          OPTIONAL,

utra-FDD-EcN0-r16                       INTEGER (0..49)           OPTIONAL

}

}

MeasResultForRSSI-r16 ::=        SEQUENCE {

rssi-Result-r16                  RSSI-Range-r16,

channelOccupancy-r16             INTEGER (0..100)

}

MeasResultCLI-r16 ::=            SEQUENCE {

measResultListSRS-RSRP-r16       MeasResultListSRS-RSRP-r16                                                         OPTIONAL,

measResultListCLI-RSSI-r16       MeasResultListCLI-RSSI-r16                                                         OPTIONAL

}

MeasResultListSRS-RSRP-r16 ::=   SEQUENCE (SIZE (1.. maxCLI-Report-r16)) OF MeasResultSRS-RSRP-r16

MeasResultSRS-RSRP-r16 ::=       SEQUENCE {

srs-ResourceId-r16               SRS-ResourceId,

srs-RSRP-Result-r16              SRS-RSRP-Range-r16

}

MeasResultListCLI-RSSI-r16 ::=   SEQUENCE (SIZE (1.. maxCLI-Report-r16)) OF MeasResultCLI-RSSI-r16

MeasResultCLI-RSSI-r16 ::=       SEQUENCE {

rssi-ResourceId-r16              RSSI-ResourceId-r16,

cli-RSSI-Result-r16              CLI-RSSI-Range-r16

}

UL-PDCP-DelayValueResultList-r16 ::= SEQUENCE (SIZE (1..maxDRB)) OF UL-PDCP-DelayValueResult-r16

UL-PDCP-DelayValueResult-r16 ::= SEQUENCE {

drb-Id-r16                       DRB-Identity,

averageDelay-r16                 INTEGER (0..10000),

...

}

-- TAG-MEASRESULTS-STOP

-- ASN1STOP

MeasResultEUTRA field descriptions

eutra-PhysCellId Identifies the physical cell identity of the E-UTRA cell for which the reporting is being performed. The UE reports a value in the range 0..503, other values are reserved.

MeasResultNR field descriptions

averageDelay Indicates average delay for the packets during the reporting period, as specified in TS 38.314 [53]. Value 0 corresponds to 0 millisecond, value 1 corresponds to 0.1 millisecond, value 2 corresponds to 0.2 millisecond, and so on.

cellResults Cell level measurement results.

drb-Id Indicates DRB value for which uplink PDCP delay ratio or value is provided, according to TS 38.314 [53].

locationInfo Positioning related information and measurements.

physCellId The physical cell identity of the NR cell for which the reporting is being performed.

resultsSSB-Cell Cell level measurement results based on SS/PBCH related measurements.

resultsSSB-Indexes Beam level measurement results based on SS/PBCH related measurements.

resultsCSI-RS-Cell Cell level measurement results based on CSI-RS related measurements.

resultsCSI-RS-Indexes Beam level measurement results based on CSI-RS related measurements.

rsIndexResults Beam level measurement results.

MeasResultUTRA-FDD field descriptions

physCellId The physical cell identity of the UTRA-FDD cell for which the reporting is being performed.

utra-FDD-EcN0 According to CPICH_Ec/No in TS 25.133 [46] for FDD.

utra-FDD-RSCP According to CPICH_RSCP in TS 25.133 [46] for FDD.

MeasResults field descriptions

measId Identifies the measurement identity for which the reporting is being performed.

measQuantityResults The value sinr is not included when it is used for LogMeasReport-r16.

measResultCellListSFTD-NR SFTD measurement results between the PCell and the NR neighbour cell(s) in NR standalone.

measResultCLI CLI measurement results.

measResultEUTRA Measured results of an E-UTRA cell.

measResultForRSSI Includes measured RSSI result in dBm (see TS 38.215 [9]) and channelOccupancy which is the percentage of samples when the RSSI was above the configured channelOccupancyThreshold for the associated reportConfig.

measResultListEUTRA List of measured results for the maximum number of reported best cells for an E-UTRA measurement identity.

measResultListNR List of measured results for the maximum number of reported best cells for an NR measurement identity.

measResultListUTRA-FDD List of measured results for the maximum number of reported best cells for a UTRA-FDD measurement identity.

measResultNR Measured results of an NR cell.

measResultServFreqListEUTRA-SCG Measured results of the E-UTRA SCG serving frequencies: the measurement result of PSCell and each SCell, if any, and of the best neighbouring cell on each E-UTRA SCG serving frequency.

measResultServFreqListNR-SCG Measured results of the NR SCG serving frequencies: the measurement result of PSCell and each SCell, if any, and of the best neighbouring cell on each NR SCG serving frequency.

measResultServingMOList Measured results of measured cells with reference signals indicated in the serving cell measurement objects including measurement results of SpCell, configured SCell(s) and best neighbouring cell within measured cells with reference signals indicated in on each serving cell measurement object. If the sending of the MeasurementReport message is triggered by a measurement configured by the field sl-ConfigDedicatedForNR received within an E-UTRA RRCConnectionReconfiguration message (i.e. CBR measurements), this field is not applicable and its contents is ignored by the network.

measResultSFTD-EUTRA SFTD measurement results between the PCell and the E-UTRA PScell in NE-DC.

measResultSFTD-NR SFTD measurement results between the PCell and the NR PScell in NR-DC.

measResultsSL CBR measurements results for NR sidelink communication.

measResultUTRA-FDD Measured result of a UTRA-FDD cell.

next change

–	MeasResultRxTxTimeDiff

The IE MeasResultRxTxTimeDiff is used to provide Rx-Tx time difference measurement result.

MeasResultRxTxTimeDiff information element

-- ASN1START

-- TAG-MEASRESULTRXTXTIMEDIFF-START

MeasResultRxTxTimeDiff-r17 ::=   SEQUENCE {

rxTxTimeDiff-ue-r17               RxTxTimeDiff-r17      OPTIONAL,    -- Need R

...

}

-- TAG-MEASRESULTRXTXTIMEDIFF-STOP

-- ASN1STOP

MeasResultRxTxTimeDiff field descriptions

rxTxTimeDiff-ue indicates the Rx-Tx Time difference measurement at the UE (see clause 5.1.30, TS 38.215 [9]).

next change

–	NR-DL-PRS-PDC-Info

The IE NR-DL-PRS-PDC-Info defines downlink PRS configuration for PDC.

NR-DL-PRS-PDC-Info information element

-- ASN1START

-- TAG-NR-DL-PRS-PDC-INFO-START

NR-DL-PRS-PDC-Info-r17 :: =     SEQUENCE {

nr-DL-PRS-PDC-ResourceSet-r17          NR-DL-PRS-PDC-ResourceSet-r17                    OPTIONAL, -- Need R

...

}

NR-DL-PRS-PDC-ResourceSet-r17 :: = 	SEQUENCE {

periodicityAndOffset-r17     NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r17,

numSymbols-r17               ENUMERATED {n2, n4, n6, n12, spare1, spare2, spare3, spare4},

dl-PRS-ResourceBandwidth-r17        INTEGER (1..63),

dl-PRS-StartPRB-r17                 INTEGER (0..2176),

resourceList-r17             SEQUENCE (SIZE (1..maxNrofPRS-ResourcesPerSet-r17)) OF NR-DL-PRS-Resource-r17,

repetitionFactor-r17         ENUMERATED {n2, n4, n6, n8, n16, n32, spare1, spare2}      OPTIONAL,   -- Need S

timeGap-r17                  ENUMERATED {s1, s2, s4, s8, s16, s32, spare1, spare2}      OPTIONAL,   -- Need S

...

}

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r17 ::= CHOICE {

scs15-r17       CHOICE {

n4-r17                  INTEGER (0..3),

n5-r17                  INTEGER (0..4),

n8-r17                  INTEGER (0..7),

n10-r17                 INTEGER (0..9),

n16-r17                 INTEGER (0..15),

n20-r17                 INTEGER (0..19),

n32-r17                 INTEGER (0..31),

n40-r17                 INTEGER (0..39),

n64-r17                 INTEGER (0..63),

n80-r17                 INTEGER (0..79),

n160-r17                INTEGER (0..159),

n320-r17                INTEGER (0..319),

n640-r17                INTEGER (0..639),

n1280-r17               INTEGER (0..1279),

n2560-r17               INTEGER (0..2559),

n5120-r17               INTEGER (0..5119),

n10240-r17              INTEGER (0..10239),

...

},

scs30-r17       CHOICE {

n8-r17                  INTEGER (0..7),

n10-r17                 INTEGER (0..9),

n16-r17                 INTEGER (0..15),

n20-r17                 INTEGER (0..19),

n32-r17                 INTEGER (0..31),

n40-r17                 INTEGER (0..39),

n64-r17                 INTEGER (0..63),

n80-r17                 INTEGER (0..79),

n128-r17                INTEGER (0..127),

n160-r17                INTEGER (0..159),

n320-r17                INTEGER (0..319),

n640-r17                INTEGER (0..639),

n1280-r17               INTEGER (0..1279),

n2560-r17               INTEGER (0..2559),

n5120-r17               INTEGER (0..5119),

n10240-r17              INTEGER (0..10239),

n20480-r17              INTEGER (0..20479),

...

},

scs60-r17       CHOICE {

n16-r17                 INTEGER (0..15),

n20-r17                 INTEGER (0..19),

n32-r17                 INTEGER (0..31),

n40-r17                 INTEGER (0..39),

n64-r17                 INTEGER (0..63),

n80-r17                 INTEGER (0..79),

n128-r17                INTEGER (0..127),

n160-r17                INTEGER (0..159),

n256-r17                INTEGER (0..255),

n320-r17                INTEGER (0..319),

n640-r17                INTEGER (0..639),

n1280-r17               INTEGER (0..1279),

n2560-r17               INTEGER (0..2559),

n5120-r17               INTEGER (0..5119),

n10240-r17              INTEGER (0..10239),

n20480-r17              INTEGER (0..20479),

n40960-r17              INTEGER (0..40959),

...

},

scs120-r17      CHOICE {

n32-r17                 INTEGER (0..31),

n40-r17                 INTEGER (0..39),

n64-r17                 INTEGER (0..63),

n80-r17                 INTEGER (0..79),

n128-r17                INTEGER (0..127),

n160-r17                INTEGER (0..159),

n256-r17                INTEGER (0..255),

n320-r17                INTEGER (0..319),

n512-r17                INTEGER (0..511),

n640-r17                INTEGER (0..639),

n1280-r17               INTEGER (0..1279),

n2560-r17               INTEGER (0..2559),

n5120-r17               INTEGER (0..5119),

n10240-r17              INTEGER (0..10239),

n20480-r17              INTEGER (0..20479),

n40960-r17              INTEGER (0..40959),

n81920-r17              INTEGER (0..81919),

...

},

...

}

NR-DL-PRS-Resource-r17 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r17            NR-DL-PRS-ResourceID-r17,

dl-PRS-SequenceID-r17               INTEGER (0..4095),

dl-PRS-CombSizeN-AndReOffset-r17    CHOICE {

n2-r17                          INTEGER (0..1),

n4-r17                          INTEGER (0..3),

n6-r17                          INTEGER (0..5),

n12-r17                         INTEGER (0..11),

...

},

dl-PRS-ResourceSlotOffset-r17       INTEGER (0..maxNrofPRS-ResourceOffsetValue-1-r17),

dl-PRS-ResourceSymbolOffset-r17     INTEGER (0..12),

dl-PRS-QCL-Info-r17                 DL-PRS-QCL-Info-r17                         OPTIONAL, -- Need N

...

}

DL-PRS-QCL-Info-r17 ::= CHOICE {

ssb-r17                     SEQUENCE {

pci-r17                         PhysCellId,

ssb-Index-r17                   INTEGER (0..63),

rs-Type-r17                     ENUMERATED {typeC, typeD, typeC-plus-typeD}

},

dl-PRS-r17                  SEQUENCE {

qcl-DL-PRS-ResourceID-r17       NR-DL-PRS-ResourceID-r17,

...

}

...

}

NR-DL-PRS-ResourceID-r17 ::= INTEGER (0..maxNrofPRS-ResourcesPerSet-1-r17)

-- TAG-NR-DL-PRS-PDC-INFO-STOP

-- ASN1STOP

NR-DL-PRS-PDC-ResourceSet field descriptions

dl-PRS-ResourceBandwidth This field specifies the number of PRBs allocated for all the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs in this resource set. All DL-PRS Resources of the DL-PRS-PDC Resource Set have the same bandwidth. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

dl-PRS-StartPRB This field specifies the start PRB index defined as offset with respect to subcarrier 0 in common resource block 0 for the DL-PRS Resource. All DL-PRS Resources of the DL-PRS-PDC Resource Set have the same value of dl-PRS-StartPRB.

numSymbols This field specifies the number of symbols per DL-PRS Resource within a slot.

periodicityAndOffset This field specifies the periodicity of DL-PRS allocation in slots and the slot offset with respect to SFN #0 slot #0 in the PCell where the DL-PRS-PDC Resource Set is configured (i.e., slot where the first DL-PRS Resource of DL-PRS-PDC Resource Set occurs).

repetitionFactor This field specifies how many times each DL-PRS Resource is repeated for a single instance of the DL-PRS Resource Set. It is applied to all resources of the DL-PRS Resource Set. Enumerated values n2, n4, n6, n8, n16, n32 correspond to 2, 4, 6, 8, 16, 32 resource repetitions, respectively. If this field is absent, the value for dl-PRS-ResourceRepetitionFactor is 1 (i.e., no resource repetition).

timeGap This field specifies the offset in units of slots between two repeated instances of a DL-PRS Resource corresponding to the same DL-PRS Resource ID within a single instance of the DL-PRS Resource Set. The time duration spanned by one DL-PRS Resource Set containing repeated DL-PRS Resources should not exceed the periodicity configured by periodicityAndOffset. The field is mandatory present, if repetitionFactor is present. Otherwise, it is not present.

next change

–	NZP-CSI-RS-ResourceSet

The IE NZP-CSI-RS-ResourceSet is a set of Non-Zero-Power (NZP) CSI-RS resources (their IDs) and set-specific parameters.

NZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESET-START

NZP-CSI-RS-ResourceSet ::=          SEQUENCE {

nzp-CSI-ResourceSetId               NZP-CSI-RS-ResourceSetId,

nzp-CSI-RS-Resources                SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId,

repetition                          ENUMERATED { on, off }                                                  OPTIONAL,   -- Need S

aperiodicTriggeringOffset           INTEGER(0..6)                                                           OPTIONAL,   -- Need S

trs-Info                            ENUMERATED {true}                                                       OPTIONAL,   -- Need R

...,

[[

aperiodicTriggeringOffset-r16       INTEGER(0..31)                                                          OPTIONAL,   -- Need S

]]

[[

pdc-Info-r17                        ENUMERATED {true}                                                       OPTIONAL   -- Need R

]]

}

-- TAG-NZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

NZP-CSI-RS-ResourceSet field descriptions

aperiodicTriggeringOffset, aperiodicTriggeringOffset-r16 Offset X between the slot containing the DCI that triggers a set of aperiodic NZP CSI-RS resources and the slot in which the CSI-RS resource set is transmitted. For aperiodicTriggeringOffset, the value 0 corresponds to 0 slots, value 1 corresponds to 1 slot, value 2 corresponds to 2 slots, value 3 corresponds to 3 slots, value 4 corresponds to 4 slots, value 5 corresponds to 16 slots, value 6 corresponds to 24 slots. For aperiodicTriggeringOffset-r16, the value indicates the number of slots. The network configures only one of the fields. When neither field is included, the UE applies the value 0.

nzp-CSI-RS-Resources NZP-CSI-RS-Resources associated with this NZP-CSI-RS resource set (see TS 38.214 [19], clause 5.2). For CSI, there are at most 8 NZP CSI RS resources per resource set.

pdc-Info Indicates that this NZP-CSI-RS-ResourceSet, if configured also with trs-Info, is used for propagation delay compensation. The field can be present only if trs-info is present. The field can be present in only one NZP-CSI-RS-ResourceSet. If network configures this field for an NZP-CSI-RS-ResourceSet, the UE measures the UE Rx-Tx time difference based on resources configured in this resource set.

Repetition Indicates whether repetition is on/off. If the field is set to off or if the field is absent, the UE may not assume that the NZP-CSI-RS resources within the resource set are transmitted with the same downlink spatial domain transmission filter (see TS 38.214 [19], clauses 5.2.2.3.1 and 5.1.6.1.2). It can only be configured for CSI-RS resource sets which are associated with CSI-ReportConfig with report of L1 RSRP, L1 SINR or "no report".

trs-Info Indicates that the antenna port for all NZP-CSI-RS resources in the CSI-RS resource set is same. If the field is absent or released the UE applies the value false (see TS 38.214 [19], clause 5.2.2.3.1).

next change

–	PDCP-Config

The IE PDCP-Config is used to set the configurable PDCP parameters for signalling and data radio bearers.

PDCP-Config information element

-- ASN1START

-- TAG-PDCP-CONFIG-START

PDCP-Config ::=         SEQUENCE {

drb                     SEQUENCE {

discardTimer            ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200,

ms250, ms300, ms500, ms750, ms1500, infinity}       OPTIONAL, -- Cond Setup

pdcp-SN-SizeUL          ENUMERATED {len12bits, len18bits}                               OPTIONAL, -- Cond Setup2

pdcp-SN-SizeDL          ENUMERATED {len12bits, len18bits}                               OPTIONAL, -- Cond Setup2

headerCompression       CHOICE {

notUsed                 NULL,

rohc                    SEQUENCE {

maxCID                  INTEGER (1..16383)                                      DEFAULT 15,

profiles                SEQUENCE {

profile0x0001           BOOLEAN,

profile0x0002           BOOLEAN,

profile0x0003           BOOLEAN,

profile0x0004           BOOLEAN,

profile0x0006           BOOLEAN,

profile0x0101           BOOLEAN,

profile0x0102           BOOLEAN,

profile0x0103           BOOLEAN,

profile0x0104           BOOLEAN

},

drb-ContinueROHC            ENUMERATED { true }                                 OPTIONAL    -- Need N

},

uplinkOnlyROHC          SEQUENCE {

maxCID                  INTEGER (1..16383)                                      DEFAULT 15,

profiles                SEQUENCE {

profile0x0006           BOOLEAN

},

drb-ContinueROHC            ENUMERATED { true }                                 OPTIONAL    -- Need N

},

...

},

integrityProtection     ENUMERATED { enabled }                                          OPTIONAL,   -- Cond ConnectedTo5GC1

statusReportRequired    ENUMERATED { true }                                             OPTIONAL,   -- Cond Rlc-AM-UM

outOfOrderDelivery      ENUMERATED { true }                                             OPTIONAL    -- Need R

}                                                                                           OPTIONAL,   -- Cond DRB

moreThanOneRLC          SEQUENCE {

primaryPath             SEQUENCE {

cellGroup               CellGroupId                                                 OPTIONAL,   -- Need R

logicalChannel          LogicalChannelIdentity                                      OPTIONAL    -- Need R

},

ul-DataSplitThreshold   UL-DataSplitThreshold                                           OPTIONAL,   -- Cond SplitBearer

pdcp-Duplication            BOOLEAN                                                     OPTIONAL    -- Need R

}                                                                                           OPTIONAL,   -- Cond MoreThanOneRLC

t-Reordering                ENUMERATED {

ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40,

ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220,

ms240, ms260, ms280, ms300, ms500, ms750, ms1000, ms1250,

ms1500, ms1750, ms2000, ms2250, ms2500, ms2750,

ms3000, spare28, spare27, spare26, spare25, spare24,

spare23, spare22, spare21, spare20,

spare19, spare18, spare17, spare16, spare15, spare14,

spare13, spare12, spare11, spare10, spare09,

spare08, spare07, spare06, spare05, spare04, spare03,

spare02, spare01 }                                          OPTIONAL, -- Need S

...,

[[

cipheringDisabled       ENUMERATED {true}                                                   OPTIONAL    -- Cond ConnectedTo5GC

]],

[[

discardTimerExt-r16     SetupRelease { DiscardTimerExt-r16 }                                OPTIONAL,    -- Cond DRB2

moreThanTwoRLC-DRB-r16  SEQUENCE {

splitSecondaryPath-r16  LogicalChannelIdentity                                          OPTIONAL,   -- Cond SplitBearer2

duplicationState-r16    SEQUENCE (SIZE (3)) OF BOOLEAN                                  OPTIONAL    -- Need S

}                                                                                           OPTIONAL,   -- Cond MoreThanTwoRLC-DRB

ethernetHeaderCompression-r16  SetupRelease { EthernetHeaderCompression-r16 }               OPTIONAL,    -- Need M

]]

[[

survivalTimeStateSupport-r17     ENUMERATED {true}                                          OPTIONAL    -- Cond Drb-Duplication

]]

}

EthernetHeaderCompression-r16 ::=  SEQUENCE {

ehc-Common-r16                     SEQUENCE {

ehc-CID-Length-r16                 ENUMERATED { bits7, bits15 },

...

},

ehc-Downlink-r16               SEQUENCE {

drb-ContinueEHC-DL-r16         ENUMERATED { true }                                      OPTIONAL,   -- Need N

...

}                                                                                           OPTIONAL,   -- Need M

ehc-Uplink-r16                 SEQUENCE {

maxCID-EHC-UL-r16              INTEGER (1..32767),

drb-ContinueEHC-UL-r16         ENUMERATED { true }                                      OPTIONAL,   -- Need N

...

}                                                                                           OPTIONAL    -- Need M

}

UL-DataSplitThreshold ::= ENUMERATED {

b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800,

b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400,

b6553600, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

DiscardTimerExt-r16 ::= ENUMERATED {ms0dot5, ms1, ms2, ms4, ms6, ms8, spare2, spare1}

-- TAG-PDCP-CONFIG-STOP

-- ASN1STOP

PDCP-Config field descriptions

cipheringDisabled If included, ciphering is disabled for this DRB regardless of which ciphering algorithm is configured for the SRB/DRBs. The field may only be included if the UE is connected to 5GC. Otherwise the field is absent. The network configures all DRBs with the same PDU-session ID with same value for this field. The value for this field cannot be changed after the DRB is set up.

discardTimer Value in ms of discardTimer specified in TS 38.323 [5]. Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms and so on. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

discardTimerExt Value in ms of discardTimer specified in TS 38.323 [5]. Value ms0dot5 corresponds to 0.5 ms, value ms1 corresponds to 1ms and so on. If this field is present, the field discardTimer is ignored and discardTimerExt is used instead.

drb-ContinueROHC Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. This field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated. The network does not include the field if the bearer is configured as DAPS bearer.

duplicationState This field indicates the uplink PDCP duplication state for the associated RLC entities at the time of receiving this IE. If set to true, the PDCP duplication state is activated for the associated RLC entity. The index for the indication is determined by ascending order of logical channel ID of all RLC entities other than the primary RLC entity indicated by primaryPath in the order of MCG and SCG, as in clause 6.1.3.32 of TS 38.321 [3]. If the number of associated RLC entities other than the primary RLC entity is two, UE ignores the value in the largest index of this field. If the field is absent, the PDCP duplication states are deactivated for all associated RLC entities.

ethernetHeaderCompression This fields configures Ethernet Header Compression. This field can only be configured for a bi-directional DRB. The network reconfigures ethernetHeaderCompression only upon reconfiguration involving PDCP re-establishment and with neither drb-ContinueEHC-DL nor drb-ContinueEHC-UL configured.

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. If uplinkOnlyROHC is configured, the UE shall apply the configured ROHC profile(s) in uplink (there is no header compression in downlink). ROHC can be configured for any bearer type. ROHC and EHC can be both configured simultaneously for a DRB. The network reconfigures headerCompression only upon reconfiguration involving PDCP re-establishment, and without any drb-ContinueROHC. Network configures headerCompression to notUsed when outOfOrderDelivery is configured.

integrityProtection Indicates whether or not integrity protection is configured for this radio bearer. The network configures all DRBs with the same PDU-session ID with same value for this field. The value for this field cannot be changed after the DRB is set up.

maxCID Indicates the value of the MAX_CID parameter as specified in TS 38.323 [5]. The total value of MAX_CIDs across all bearers for the UE should be less than or equal to the value of maxNumberROHC-ContextSessions parameter as indicated by the UE.

moreThanOneRLC This field configures UL data transmission when more than one RLC entity is associated with the PDCP entity. This field is not present if the bearer is configured as DAPS bearer.

moreThanTwoRLC-DRB This field configures UL data transmission when more than two RLC entities are associated with the PDCP entity for DRBs.

outOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured. This field should be either always present or always absent, after the radio bearer is established.

pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates that duplication is configured. PDCP duplication is not configured for CA packet duplication of LTE RLC bearer. The value of this field, when the field is present, indicates the state of the duplication at the time of receiving this IE. If set to true, duplication is activated. The value of this field is always true, when configured for a SRB. For PDCP entity with more than two associated RLC entities for UL transmission, this field is always present. If the field moreThanTwoRLC-DRB is present, the value of this field is ignored and the state of the duplication is indicated by duplicationState. For PDCP entity with more than two associated RLC entities, only NR RLC bearer is supported.

survivalTimeStateSupport Indicates whether the DRB associated with this PDCP entity has survival time state support. If this field is configured to be true, all associated RLC entities are activated for PDCP duplication upon reception of a retransmission grant, as specified in TS 38.321 [3].

pdcp-SN-SizeDL PDCP sequence number size for downlink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value len12bits is applicable. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

pdcp-SN-SizeUL PDCP sequence number size for uplink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value len12bits is applicable. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

primaryPath Indicates the cell group ID and LCID of the primary RLC entity as specified in TS 38.323 [5], clause 5.2.1 for UL data transmission when more than one RLC entity is associated with the PDCP entity. In this version of the specification, only cell group ID corresponding to MCG is supported for SRBs. The NW indicates cellGroup for split bearers using logical channels in different cell groups. The NW always indicates logicalChannel if CA based PDCP duplication is configured in the cell group indicated by cellGroup of this field.

splitSecondaryPath Indicates the LCID of the split secondary RLC entity as specified in TS 38.323 [5] for fallback to split bearer operation when UL data transmission with more than two RLC entities is associated with the PDCP entity. This RLC entity belongs to a cell group that is different from the cell group indicated by cellGroup in the field primaryPath.

statusReportRequired For AM DRBs and DAPS UM DRBs, indicates whether the DRB is configured to send a PDCP status report in the uplink, as specified in TS 38.323 [5]. For DAPS AM DRBs, it also indicates whether the DRB is configured to send a second PDCP status report in the uplink, as specified in TS 38.323 [5].

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms0 corresponds to 0 ms, value ms20 corresponds to 20 ms, value ms40 corresponds to 40 ms, and so on. When the field is absent the UE applies the value infinity. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

ul-DataSplitThreshold Parameter specified in TS 38.323 [5]. Value b0 corresponds to 0 bytes, value b100 corresponds to 100 bytes, value b200 corresponds to 200 bytes, and so on. The network sets this field to infinity for UEs not supporting splitDRB-withUL-Both-MCG-SCG. If the field is absent when the split bearer is configured for the radio bearer first time, then the default value infinity is applied.

EthernetHeaderCompression field descriptions

drb-ContinueEHC-DL Indicates whether the PDCP entity continues or resets the downlink EHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

drb-ContinueEHC-UL Indicates whether the PDCP entity continues or resets the uplink EHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

ehc-CID-Length Indicates the length of the CID field for EHC packet. The value bits7 indicates the length is 7 bits, and the value bits15 indicates the length is 15 bits. Once the field ethernetHeaderCompression-r16 is configured for a DRB, the value of the field ehc-CID-Length for this DRB is not reconfigured to a different value.

ehc-Common Indicates the configurations that apply for both downlink and uplink.

ehc-Downlink Indicates the configurations that apply for only downlink. If the field is configured, then Ethernet header compression is configured for downlink. Otherwise, it is not configured for downlink.

ehc-Uplink Indicates the configurations that apply for only uplink. If the field is configured, then Ethernet header compression is configured for uplnik. Otherwise, it is not configured for uplink.

maxCID-EHC-UL Indicates the value of the MAX_CID_EHC_UL parameter as specified in TS 38.323 [5]. The total value of MAX_CID_EHC_UL across all bearers for the UE should be less than or equal to the value of maxNumberEHC-Contexts parameter as indicated by the UE.

| Conditional presence | Explanation |
| --- | --- |
| DRB | This field is mandatory present when the corresponding DRB is being set up, absent for SRBs. Otherwise this field is optionally present, need M. |
| DRB2 | This field is optionally present in case of DRB, need M. Otherwise, it is absent for SRBs. |
| MoreThanOneRLC | This field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer with more than one associated logical channel and upon RRC reconfiguration with the association of additional logical channels to the PDCP entity. The field is also mandatory present in case the field moreThanTwoRLC-DRB is included in PDCP-Config. Upon RRC reconfiguration when a PDCP entity is associated with multiple logical channels, this field is optionally present need M. Otherwise, this field is absent. Need R. |
| MoreThanTwoRLC-DRB | For SRBs, this field is absent. For DRBs, this field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer with more than two associated logical channels and upon RRC reconfiguration with the association of one or more additional logical channel(s) to the PDCP entity so that the PDCP entity has more than two associated logical channels. Upon RRC reconfiguration when a PDCP entity is associated with more than two logical channels, this field is optionally present, Need M. Otherwise, the field is absent, Need R. |
| Rlc-AM-UM | For RLC UM (if the UE supports DAPS handover) or RLC AM, the field is optionally present, need R. Otherwise, the field is absent. |
| Setup | The field is mandatory present in case of radio bearer setup. Otherwise the field is optionally present, need M. |
| SplitBearer | The field is absent for SRBs. Otherwise, the field is optional present, need M, in case of radio bearer with more than one associated RLC mapped to different cell groups. |
| SplitBearer2 | The field is mandatory present, in case of a split bearer. Otherwise the field is absent. |
| ConnectedTo5GC | The field is optionally present, need R, if the UE is connected to 5GC. Otherwise the field is absent. |
| ConnectedTo5GC1 | The field is optionally present, need R, if the UE is connected to NR/5GC. Otherwise the field is absent. |
| Setup2 | This field is mandatory present in case for radio bearer setup for RLC-AM and RLC-UM. Otherwise, this field is absent, Need M. |
| Drb-Duplication | For SRBs, this field is absent. For DRBs with only one associated logical channel, this field is absent. Otherwise, this field is optional, need R. |

–	PDSCH-Config

The PDSCH-Config IE is used to configure the UE specific PDSCH parameters.

PDSCH-Config information element

-- ASN1START

-- TAG-PDSCH-CONFIG-START

PDSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPDSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

dmrs-DownlinkForPDSCH-MappingTypeA      SetupRelease { DMRS-DownlinkConfig }                                OPTIONAL,   -- Need M

dmrs-DownlinkForPDSCH-MappingTypeB      SetupRelease { DMRS-DownlinkConfig }                                OPTIONAL,   -- Need M

tci-StatesToAddModList                  SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-State                  OPTIONAL,   -- Need N

tci-StatesToReleaseList                 SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-StateId                OPTIONAL,   -- Need N

vrb-ToPRB-Interleaver                   ENUMERATED {n2, n4}                                                 OPTIONAL,   -- Need S

resourceAllocation                      ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pdsch-TimeDomainAllocationList          SetupRelease { PDSCH-TimeDomainResourceAllocationList }             OPTIONAL,   -- Need M

pdsch-AggregationFactor                 ENUMERATED { n2, n4, n8 }                                           OPTIONAL,   -- Need S

rateMatchPatternToAddModList            SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL,   -- Need N

rateMatchPatternToReleaseList           SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL,   -- Need N

rateMatchPatternGroup1                  RateMatchPatternGroup                                               OPTIONAL,   -- Need R

rateMatchPatternGroup2                  RateMatchPatternGroup                                               OPTIONAL,   -- Need R

rbg-Size                                ENUMERATED {config1, config2},

mcs-Table                               ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

maxNrofCodeWordsScheduledByDCI          ENUMERATED {n1, n2}                                                 OPTIONAL,   -- Need R

prb-BundlingType                        CHOICE {

staticBundling                          SEQUENCE {

bundleSize                              ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

},

dynamicBundling                     SEQUENCE {

bundleSizeSet1                      ENUMERATED { n4, wideband, n2-wideband, n4-wideband }           OPTIONAL,   -- Need S

bundleSizeSet2                      ENUMERATED { n4, wideband }                                     OPTIONAL    -- Need S

}

},

zp-CSI-RS-ResourceToAddModList                  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-Resource

OPTIONAL,   -- Need N

zp-CSI-RS-ResourceToReleaseList                 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-ResourceId

OPTIONAL,   -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList    SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

sp-ZP-CSI-RS-ResourceSetsToAddModList   SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

sp-ZP-CSI-RS-ResourceSetsToReleaseList  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

p-ZP-CSI-RS-ResourceSet                 SetupRelease { ZP-CSI-RS-ResourceSet }

OPTIONAL,   -- Need M

...,

[[

maxMIMO-Layers-r16                      SetupRelease { MaxMIMO-LayersDL-r16 }                               OPTIONAL,   -- Need M

minimumSchedulingOffsetK0-r16           SetupRelease { MinSchedulingOffsetK0-Values-r16 }                   OPTIONAL,   -- Need M

-- Start of the parameters for DCI format 1_2 introduced in V16.1.0

antennaPortsFieldPresenceDCI-1-2-r16            ENUMERATED {enabled}                                        OPTIONAL,   -- Need S

aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2-r16  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

aperiodicZP-CSI-RS-ResourceSetsToReleaseListDCI-1-2-r16 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2-r16  SetupRelease { DMRS-DownlinkConfig }                        OPTIONAL,   -- Need M

dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2-r16  SetupRelease { DMRS-DownlinkConfig }                        OPTIONAL,   -- Need M

dmrs-SequenceInitializationDCI-1-2-r16          ENUMERATED {enabled}                                        OPTIONAL,   -- Need S

harq-ProcessNumberSizeDCI-1-2-r16               INTEGER (0..4)                                              OPTIONAL,   -- Need R

mcs-TableDCI-1-2-r16                            ENUMERATED {qam256, qam64LowSE}                             OPTIONAL,   -- Need S

numberOfBitsForRV-DCI-1-2-r16                   INTEGER (0..2)                                              OPTIONAL,   -- Need R

pdsch-TimeDomainAllocationListDCI-1-2-r16       SetupRelease { PDSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,   -- Need M

prb-BundlingTypeDCI-1-2-r16             CHOICE {

staticBundling-r16                      SEQUENCE {

bundleSize-r16                          ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

},

dynamicBundling-r16                     SEQUENCE {

bundleSizeSet1-r16                      ENUMERATED { n4, wideband, n2-wideband, n4-wideband }       OPTIONAL,   -- Need S

bundleSizeSet2-r16                      ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

}

}                                                                                                           OPTIONAL,   -- Need R

priorityIndicatorDCI-1-2-r16                ENUMERATED {enabled}                                            OPTIONAL,   -- Need S

rateMatchPatternGroup1DCI-1-2-r16           RateMatchPatternGroup                                           OPTIONAL,   -- Need R

rateMatchPatternGroup2DCI-1-2-r16           RateMatchPatternGroup                                           OPTIONAL,   -- Need R

resourceAllocationType1GranularityDCI-1-2-r16  ENUMERATED {n2,n4,n8,n16}                                    OPTIONAL,   -- Need S

vrb-ToPRB-InterleaverDCI-1-2-r16            ENUMERATED {n2, n4}                                             OPTIONAL,   -- Need S

referenceOfSLIVDCI-1-2-r16                  ENUMERATED {enabled}                                            OPTIONAL,   -- Need S

resourceAllocationDCI-1-2-r16               ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch}

OPTIONAL,   -- Need M

-- End of the parameters for DCI format 1_2 introduced in V16.1.0

priorityIndicatorDCI-1-1-r16             ENUMERATED {enabled}                                               OPTIONAL,   -- Need S

dataScramblingIdentityPDSCH2-r16         INTEGER (0..1023)                                                  OPTIONAL,   -- Need R

pdsch-TimeDomainAllocationList-r16       SetupRelease { PDSCH-TimeDomainResourceAllocationList-r16 }        OPTIONAL,   -- Need M

repetitionSchemeConfig-r16               SetupRelease { RepetitionSchemeConfig-r16}                         OPTIONAL    -- Need M

]],

[[

repetitionSchemeConfig-v1630             SetupRelease { RepetitionSchemeConfig-v1630}                       OPTIONAL,    -- Need M

]]

[[

pdsch-HARQ-ACK-OneShotFeedbackDCI-1-2-r17    ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-EnhType3DCI-1-2-r17           ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-EnhType3DCI-Field-1-2-r17     ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-RetxDCI-1-2-r17               ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pucch-sSCellDynDCI-1-2-r17                   ENUMERATED {enabled}                                           OPTIONAL   -- Need R

]]

}

RateMatchPatternGroup ::=               SEQUENCE (SIZE (1..maxNrofRateMatchPatternsPerGroup)) OF CHOICE {

cellLevel                               RateMatchPatternId,

bwpLevel                                RateMatchPatternId

}

MinSchedulingOffsetK0-Values-r16 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK0-SchedulingOffset-r16)

MaxMIMO-LayersDL-r16 ::=                INTEGER (1..8)

-- TAG-PDSCH-CONFIG-STOP

-- ASN1STOP

PDSCH-Config field descriptions

antennaPortsFieldPresenceDCI-1-2 Configure the presence of "Antenna ports" field in DCI format 1_2. When the field is configured, then the "Antenna ports" field is present in DCI format 1_2. Otherwise, the field size is set to 0 for DCI format 1_2 (See TS 38.212 [17], clause 7.3.1.1.3). If neither dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 nor dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 is configured, this field is absent.

Aperiodic-ZP-CSI-RS-ResourceSetsToAddModList, aperiodic-ZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 AddMod/Release lists for configuring aperiodically triggered zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network configures the UE with at most 3 aperiodic ZP-CSI-RS-ResourceSets and it uses only the ZP-CSI-RS-ResourceSetId 1 to 3. The network triggers a set by indicating its ZP-CSI-RS-ResourceSetId in the DCI payload. The DCI codepoint '01' triggers the resource set with ZP-CSI-RS-ResourceSetId 1, the DCI codepoint '10' triggers the resource set with ZP-CSI-RS-ResourceSetId 2, and the DCI codepoint '11' triggers the resource set with ZP-CSI-RS-ResourceSetId 3 (see TS 38.214 [19], clause 5.1.4.2). The field aperiodic-ZP-CSI-RS-ResourceSetsToAddModList applies to DCI format 1_1 and the field aperiodic-ZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.4.2 and TS 38.212 [17] clause 7.3.1).

dataScramblingIdentityPDSCH, dataScramblingIdentityPDSCH2 Identifier(s) used to initialize data scrambling (c_init) for PDSCH as specified in TS 38.211 [16], clause 7.3.1.1. The dataScramblingIdentityPDSCH2 is configured if coresetPoolIndex is configured with 1 for at least one CORESET in the same BWP.

Dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 DMRS configuration for PDSCH transmissions using PDSCH mapping type A (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-DownlinkForPDSCH-MappingTypeA applies to DCI format 1_1 and the field dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 DMRS configuration for PDSCH transmissions using PDSCH mapping type B (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-DownlinkForPDSCH-MappingTypeB applies to DCI format 1_1 and the field dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

dmrs-SequenceInitializationDCI-1_2 Configure whether the field "DMRS Sequence Initialization" is present or not in DCI format 1_2 If the field is absent, then the UE applies the value of 0 bit for the field "DMRS Sequence Initialization" in DCI format 1_2. If the field is present, then the UE applies the value of 1 bit as in DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

harq-ProcessNumberSizeDCI-1-2 Configure the number of bits for the field "HARQ process number" in DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

maxMIMO-Layers Indicates the maximum number of MIMO layers to be used for PDSCH in this DL BWP. If not configured, the UE uses the maxMIMO-Layers configuration in IE PDSCH-ServingCellConfig of the serving cell to which this BWP belongs, when the UE operates in this BWP. The value of maxMIMO-Layers for a DL BWP shall be smaller than or equal to the value of maxMIMO-Layers configured in IE PDSCH-ServingCellConfig of the serving cell to which this BWP belongs.

maxNrofCodeWordsScheduledByDCI Maximum number of code words that a single DCI may schedule. This changes the number of MCS/RV/NDI bits in the DCI message from 1 to 2.

mcs-Table, mcs-TableDCI-1-2 Indicates which MCS table the UE shall use for PDSCH. (see TS 38.214 [19], clause 5.1.3.1). If the field is absent the UE applies the value 64QAM. The field mcs-Table applies to DCI format 1_0 and DCI format 1_1, and the field mcs-TableDCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.3.1).

minimumSchedulingOffsetK0 List of minimum K0 values. Minimum K0 parameter denotes minimum applicable value(s) for the TDRA table for PDSCH and for A-CSI RS triggering Offset(s) (see TS 38.214 [19], clause 5.3.1).

numberOfBitsForRV-DCI-1-2 Configures the number of bits for "Redundancy version" in the DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 5.1.2.1).

pdsch-AggregationFactor Number of repetitions for data (see TS 38.214 [19], clause 5.1.2.1). When the field is absent the UE applies the value 1.

pdsch-HARQ-ACK-EnhType3DCI-1-2 When configured, enhanced Type 3 HARQ-ACK codebook triggering by DCI format 1_2 is enabled.

pdsch-HARQ-ACK-EnhType3DCI-Field-1-2 Enables the enhanced Type 3 codebook through a new DCI field to indicate the enhanced Type 3 HARQ-ACK codebook in DCI format 1_2 if the more than one enhanced Type 3 HARQ-ACK codebook is configured for the primary PUCCH cell group.

pdsch-HARQ-ACK-OneShotFeedbackDCI-1-2 When configured, DCI format 1_2 can request the UE to report A/N for all HARQ processes and all component carriers configured in the PUCCH group (see TS 38.212 [17], clause 7.3.1).

pdsch-HARQ-ACK-RetxDCI-1-2 When configured, DCI format 1_2 can request the UE to perform a HARQ-ACK re-transmission on a PUCCH resource (see TS 38.213 [13], clause 9.1.5).

pucch-sSCellDynDCI-1-2 When configured, PUCCH cell switching based on dynamic indication in DCI format 1_2 is enabled (see TS 38.213 [13], clause 9.A).

pdsch-TimeDomainAllocationList, pdsch-TimeDomainAllocationListDCI-1-2 List of time-domain configurations for timing of DL assignment to DL data. The field pdsch-TimeDomainAllocationList (with or without suffix) applies to DCI format 1_0 and DCI format 1_1 (see table 5.1.2.1.1-1 in TS 38.214 [19]), and if the field pdsch-TimeDomainAllocationListDCI-1-2 is not configured, to DCI format 1_2. If the field pdsch-TimeDomainAllocationListDCI-1-2 is configured, it applies to DCI format 1_2 (see table 5.1.2.1.1-1A in TS 38.214 [19]). The network does not configure the pdsch-TimeDomainAllocationList-r16 simultaneously with the pdsch-TimeDomainAllocationList (without suffix) in the same PDSCH-Config.

prb-BundlingType, prb-BundlingTypeDCI-1-2 Indicates the PRB bundle type and bundle size(s) (see TS 38.214 [19], clause 5.1.2.3). If dynamic is chosen, the actual bundleSizeSet1 or bundleSizeSet2 to use is indicated via DCI. Constraints on bundleSize(Set) setting depending on vrb-ToPRB-Interleaver and rbg-Size settings are described in TS 38.214 [19], clause 5.1.2.3. If a bundleSize(Set) value is absent, the UE applies the value n2. The field prb-BundlingType applies to DCI format 1_1, and the field prb-BundlingTypeDCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 5.1.2.3).

priorityIndicatorDCI-1-1, priorityIndicatorDCI-1-2 Configure the presence of "priority indicator" in DCI format 1_1/1_2. When the field is absent in the IE, then 0 bit for "priority indicator" in DCI format 1_1/1_2. The field priorityIndicatorDCI-1-1 applies to DCI format 1_1 and the field priorityIndicatorDCI-1-2 applies to DCI format 1_2, respectively (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9).

p-ZP-CSI-RS-ResourceSet A set of periodically occurring ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network uses the ZP-CSI-RS-ResourceSetId=0 for this set.

rateMatchPatternGroup1, rateMatchPatternGroup1DCI-1-2 The IDs of a first group of RateMatchPatterns defined in PDSCH-Config->rateMatchPatternToAddModList (BWP level) or in ServingCellConfig ->rateMatchPatternToAddModList (cell level). These patterns can be activated dynamically by DCI (see TS 38.214 [19], clause 5.1.4.1). The field rateMatchPatternGroup1 applies to DCI format 1_1, and the field rateMatchPatternGroup1DCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.4.1).

rateMatchPatternGroup2, rateMatchPatternGroup2DCI-1-2 The IDs of a second group of RateMatchPatterns defined in PDSCH-Config->rateMatchPatternToAddModList (BWP level) or in ServingCellConfig ->rateMatchPatternToAddModList (cell level). These patterns can be activated dynamically by DCI (see TS 38.214 [19], clause 5.1.4.1). The field rateMatchPatternGroup2 applies to DCI format 1_1, and the field rateMatchPatternGroup2DCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.4.1).

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the rate match patterns (see TS 38.214 [19], clause 5.1.4.1).

rbg-Size Selection between config 1 and config 2 for RBG size for PDSCH. The UE ignores this field if resourceAllocation is set to resourceAllocationType1 (see TS 38.214 [19], clause 5.1.2.2.1).

referenceOfSLIVDCI-1-2 Enable using the starting symbol of the PDCCH monitoring occasion in which the DL assignment is detected as the reference of the SLIV for DCI format 1_2. When the RRC parameter enables the utilization of the new reference, the new reference is applied for TDRA entries with K0=0. For other entries (if any) in the same TDRA table, the reference is slot boundary as in Rel-15. PDSCH mapping type A is not supported with the new reference. The new reference of SLIV is not configured for a serving cell configured to be scheduled by cross-carrier scheduling on a scheduling cell with different numerology (see TS 38.212 [17] clause 7.3.1 and TS 38.214 [19] clause 5.1.2.1).

repetitionSchemeConfig Configure the UE with repetition schemes. The network does not configure repetitionSchemeConfig-r16 and repetitionSchemeConfig-v1630 simultaneously to setup in the same PDSCH-Config.

resourceAllocation, resourceAllocationDCI-1-2 Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI (see TS 38.214 [19], clause 5.1.2.2). The field resourceAllocation applies to DCI format 1_1, and the field resourceAllocationDCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.2.2).

resourceAllocationType1GranularityDCI-1-2 Configure the scheduling granularity applicable for both the starting point and length indication for resource allocation type 1 in DCI format 1_2. If this field is absent, the granularity is 1 PRB (see TS 38.214 [19], clause 5.1.2.2.2).

sp-ZP-CSI-RS-ResourceSetsToAddModList AddMod/Release lists for configuring semi-persistent zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList) (see TS 38.214 [19], clause 5.1.4.2).

tci-StatesToAddModList A list of Transmission Configuration Indicator (TCI) states indicating a transmission configuration which includes QCL-relationships between the DL RSs in one RS set and the PDSCH DMRS ports (see TS 38.214 [19], clause 5.1.5).

vrb-ToPRB-Interleaver, vrb-ToPRB-InterleaverDCI-1-2 Interleaving unit configurable between 2 and 4 PRBs (see TS 38.211 [16], clause 7.3.1.6). When the field is absent, the UE performs non-interleaved VRB-to-PRB mapping.

zp-CSI-RS-ResourceToAddModList A list of Zero-Power (ZP) CSI-RS resources used for PDSCH rate-matching. Each resource in this list may be referred to from only one type of resource set, i.e., aperiodic, semi-persistent or periodic (see TS 38.214 [19]).

next change

–	PhysicalCellGroupConfig

The IE PhysicalCellGroupConfig is used to configure cell-group specific L1 parameters.

PhysicalCellGroupConfig information element

-- ASN1START

-- TAG-PHYSICALCELLGROUPCONFIG-START

PhysicalCellGroupConfig ::=         SEQUENCE {

harq-ACK-SpatialBundlingPUCCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

harq-ACK-SpatialBundlingPUSCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

p-NR-FR1                            P-Max                                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-Codebook             ENUMERATED {semiStatic, dynamic},

tpc-SRS-RNTI                        RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUCCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUSCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

sp-CSI-RNTI                         RNTI-Value                                                      OPTIONAL,   -- Need R

cs-RNTI                             SetupRelease { RNTI-Value }                                     OPTIONAL,   -- Need M

...,

[[

mcs-C-RNTI                          RNTI-Value                                                      OPTIONAL,   -- Need R

p-UE-FR1                            P-Max                                                           OPTIONAL    -- Cond MCG-Only

]],

[[

xScale                              ENUMERATED {dB0, dB6, spare2, spare1}                           OPTIONAL    -- Cond SCG-Only

]],

[[

pdcch-BlindDetection                SetupRelease { PDCCH-BlindDetection }                           OPTIONAL    -- Need M

]],

[[

dcp-Config-r16                      SetupRelease { DCP-Config-r16 }                                 OPTIONAL,   -- Need M

harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup-r16    ENUMERATED {enabled, disabled}             OPTIONAL,   -- Cond twoPUCCHgroup

harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup-r16    ENUMERATED {enabled, disabled}             OPTIONAL,   -- Cond twoPUCCHgroup

pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16          ENUMERATED {semiStatic, dynamic}           OPTIONAL,   -- Cond twoPUCCHgroup

p-NR-FR2-r16                                              P-Max                                     OPTIONAL,   -- Need R

p-UE-FR2-r16                                              P-Max                                     OPTIONAL,   -- Cond MCG-Only

nrdc-PCmode-FR1-r16                ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}       OPTIONAL,   -- Cond MCG-Only

nrdc-PCmode-FR2-r16                ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}       OPTIONAL,   -- Cond MCG-Only

pdsch-HARQ-ACK-Codebook-r16            ENUMERATED {enhancedDynamic}                                 OPTIONAL,   -- Need R

nfi-TotalDAI-Included-r16              ENUMERATED {true}                                            OPTIONAL,   -- Need R

ul-TotalDAI-Included-r16               ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedback-r16     ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackNDI-r16  ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackCBG-r16  ENUMERATED {true}                                            OPTIONAL,   -- Need R

downlinkAssignmentIndexDCI-0-2-r16     ENUMERATED { enabled }                                       OPTIONAL,   -- Need S

downlinkAssignmentIndexDCI-1-2-r16     ENUMERATED {n1, n2, n4}                                      OPTIONAL,   -- Need S

pdsch-HARQ-ACK-CodebookList-r16        SetupRelease {PDSCH-HARQ-ACK-CodebookList-r16}               OPTIONAL,   -- Need M

ackNackFeedbackMode-r16                ENUMERATED {joint, separate}                                 OPTIONAL,   -- Need R

pdcch-BlindDetectionCA-CombIndicator-r16 SetupRelease { PDCCH-BlindDetectionCA-CombIndicator-r16 }  OPTIONAL,   -- Need M

pdcch-BlindDetection2-r16                SetupRelease { PDCCH-BlindDetection2-r16 }                 OPTIONAL,   -- Need M

pdcch-BlindDetection3-r16                SetupRelease { PDCCH-BlindDetection3-r16 }                 OPTIONAL,   -- Need M

bdFactorR-r16                          ENUMERATED {n1}                                              OPTIONAL,    -- Need R

]]

[[

-- start of enhanced Type3 feedback

pdsch-HARQ-ACK-EnhType3ToAddModList-r17   SEQUENCE (SIZE(1..maxNrofEnhType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3-r17

OPTIONAL,   -- Need N

pdsch-HARQ-ACK-EnhType3ToReleaseList-r17  SEQUENCE (SIZE(1..maxNrofEnhType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3Index-r17

OPTIONAL,    -- Need N

pdsch-HARQ-ACK-EnhType3SecondaryToAddModList-r17   SEQUENCE (SIZE(1..maxEnhNrofType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3-r17

OPTIONAL,    -- Cond twoPUCCHgroup

pdsch-HARQ-ACK-EnhType3SecondaryToReleaseList-r17  SEQUENCE (SIZE(1..maxEnhNrofType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3Index-r17

OPTIONAL,    -- Cond twoPUCCHgroup

pdsch-HARQ-ACK-EnhType3SecondaryPUCCHgroup          ENUMERATED {enabled}                          OPTIONAL,   -- Cond twoPUCCHgroup

pdsch-HARQ-ACK-EnhType3DCI-Field-r17                ENUMERATED {enabled}                          OPTIONAL,   -- Need R

-- end of enhanced Type3 feedback

-- start of triggering of HARQ-ACK re-transmission on a PUCCH resource

pdsch-HARQ-ACK-Retx-r17                   ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

pdsch-HARQ-ACK-RetxSecondaryPUCCHgroup-r17  ENUMERATED {enabled}                                   OPTIONAL,   -- Cond twoPUCCHgroup

-- end of triggering of HARQ-ACK re-transmission on a PUCCH resource

-- start of PUCCH Cell switching

pucch-sSCell-r17                         SCellIndex                                                    OPTIONAL,   -- Need R

pucch-sSCellSecondaryPUCCHgroup-r17      SCellIndex                                                    OPTIONAL,   -- Cond twoPUCCHgroup

pucch-sSCellDyn-r17                      ENUMERATED {enabled}                                       OPTIONAL,   -- Need R

pucch-sSCellDynSecondaryPUCCHgroup-r17   ENUMERATED {enabled}                                       OPTIONAL,   -- Cond twoPUCCHgroup

pucch-sSCellPattern-r17                      SEQUENCE (SIZE(1..maxNrofSlots)) of INTEGER (0..1)        OPTIONAL,   -- Need R

pucch-sSCellPatternSecondaryPUCCHgroup-r17   SEQUENCE (SIZE(1..maxNrofSlots)) of INTEGER (0..1)        OPTIONAL,   -- Cond twoPUCCHgroup

-- end of PUCCH Cell switching

uci-MuxWithDiffPrio-r17           ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

uci-MuxWithDiffPrioSecondaryPUCCHgroup-r17     ENUMERATED {enabled}                        OPTIONAL,   -- Cond twoPUCCHgroup

simultaneousPUCCH-PUSCH-r17       ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup-r17       ENUMERATED {enabled}                  OPTIONAL,   -- Cond twoPUCCHgroup

prioLowDG-HighCG-r17            ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

prioHighDG-LowCG-r17            ENUMERATED {enabled}                                      OPTIONAL   -- Need R

]]

}

PDSCH-HARQ-ACK-EnhType3-r17 ::=         SEQUENCE {

pdsch-HARQ-ACK-EnhType3Index-r17    PDSCH-HARQ-ACK-EnhType3Index-r17,

applicable-r17   CHOICE {

perCC                            SEQUENCE (SIZE (1..maxNrofServingCells)) OF INTEGER (0..1),

perHARQ                          SEQUENCE (SIZE (1..maxNrofServingCells)) OF BIT STRING (SIZE (16))

}

pdsch-HARQ-ACK-EnhType3NDI-r17         ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-EnhType3CBG-r17         ENUMERATED {true}                                            OPTIONAL   -- Need S

...

}

PDSCH-HARQ-ACK-EnhType3Index-r17 ::=    INTEGER (0... maxNrofEnhType3HARQ-ACK-r17-1)

PDCCH-BlindDetection ::=                INTEGER (1..15)

DCP-Config-r16 ::=                  SEQUENCE {

ps-RNTI-r16                         RNTI-Value,

ps-Offset-r16                       INTEGER (1..120),

sizeDCI-2-6-r16                     INTEGER (1..maxDCI-2-6-Size-r16),

ps-PositionDCI-2-6-r16              INTEGER (0..maxDCI-2-6-Size-1-r16),

ps-WakeUp-r16                       ENUMERATED {true}                                               OPTIONAL,   -- Need S

ps-TransmitPeriodicL1-RSRP-r16      ENUMERATED {true}                                               OPTIONAL,   -- Need S

ps-TransmitOtherPeriodicCSI-r16     ENUMERATED {true}                                               OPTIONAL    -- Need S

}

PDSCH-HARQ-ACK-CodebookList-r16 ::=     SEQUENCE (SIZE (1..2)) OF ENUMERATED {semiStatic, dynamic}

PDCCH-BlindDetectionCA-CombIndicator-r16 ::= SEQUENCE {

pdcch-BlindDetectionCA1-r16                  INTEGER (1..15),

pdcch-BlindDetectionCA2-r16                  INTEGER (1..15)

}

PDCCH-BlindDetection2-r16 ::=                INTEGER (1..15)

PDCCH-BlindDetection3-r16 ::=                INTEGER (1..15)

-- TAG-PHYSICALCELLGROUPCONFIG-STOP

-- ASN1STOP

PhysicalCellGroupConfig field descriptions

ackNackFeedbackMode Indicates which among the joint and separate ACK/NACK feedback modes to use within a slot as specified in TS 38.213 [13] (clause 9).

bdFactorR Parameter for determining and distributing the maximum numbers of BD/CCE for mPDCCH based mPDSCH transmission as specified in TS 38.213 [13] Clause 10.1.

cs-RNTI RNTI value for downlink SPS (see SPS-Config) and uplink configured grant (see ConfiguredGrantConfig).

downlinkAssignmentIndexDCI-0-2 Indicates if "Downlink assignment index" is present or absent in DCI format 0_2. If the field "downlinkAssignmentIndexDCI-0-2" is absent, then 0 bit for "Downlink assignment index" in DCI format 0_2. If the field "downlinkAssignmentIndexDCI-0-2" is present, then the bitwidth of "Downlink assignment index" in DCI format 0_2 is defined in the same was as that in DCI format 0_1 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.1).

downlinkAssignmentIndexDCI-1-2 Configures the number of bits for "Downlink assignment index" in DCI format 1_2. If the field is absent, then 0 bit for "Downlink assignment index" in DCI format 1_2. Note that 1 bit and 2 bits are applied if only one serving cell is configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic. 4 bits is applied if more than one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook is set to dynamic (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.1).

harq-ACK-SpatialBundlingPUCCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUCCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling of PUCCH HARQ ACKs for the primary PUCCH group is disabled (see TS 38.213 [13], clause 9.1.2.1). If the field harq-ACK SpatialBundlingPUCCH-secondaryPUCCHgroup is present, harq-ACK-SpatialBundlingPUCCH is only applied to primary PUCCH group. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup Indicates whether spatial bundling of PUCCH HARQ ACKs for the secondary PUCCH group is enabled or disabled. The field is only applicable when more than 4 layers are possible to schedule (see TS 38.213 [13], clause 9.1.2.1). When the field is absent, the use of spatial bundling of PUCCH HARQ ACKs for the secondary PUCCH group is indicated by harq-ACK-SpatialBundlingPUCCH. See TS 38.213 [13], clause 9.1.2.1. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUSCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUSCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling of PUSCH HARQ ACKs for the primary PUCCH group is disabled (see TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2). If the field harq-ACK SpatialBundlingPUSCH-secondaryPUCCHgroup is present, harq-ACK-SpatialBundlingPUSCH is only applied to primary PUCCH group. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup Indicates whether spatial bundling of PUSCH HARQ ACKs for the secondary PUCCH group is enabled or disabled. The field is only applicable when more than 4 layers are possible to schedule (see TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2). When the field is absent, the use of spatial bundling of PUSCH HARQ ACKs for the secondary PUCCH group is indicated by harq-ACK-SpatialBundlingPUSCH. See TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

mcs-C-RNTI RNTI to indicate use of qam64LowSE for grant-based transmissions. When the mcs-C-RNTI is configured, RNTI scrambling of DCI CRC is used to choose the corresponding MCS table.

nfi-TotalDAI-Included Indicates whether the NFI and total DAI fields of the non-scheduled PDSCH group is included in the non-fallback DL grant DCI (see TS 38.212 [17], clause 7.3.1). The network configures this only when enhanced dynamic codebook is configured (pdsch-HARQ-ACK-Codebook is set to enhancedDynamic).

nrdc-PCmode-FR1 Indicates the uplink power sharing mode that the UE uses in NR-DC in frequency range 1 (FR1) (see TS 38.213 [13], clause 7.6).

nrdc-PCmode-FR2 Indicates the uplink power sharing mode that the UE uses in NR-DC in frequency range 2 (FR2) (see TS 38.213 [13], clause 7.6).

pdcch-BlindDetection, pdcch-BlindDetection2, pdcch-BlindDetection3 Indicates the reference number of cells for PDCCH blind detection for the CG. Network configures the field for each CG when the UE is in NR DC and sets the value in accordance with the constraints specified in TS 38.213 [13]. The network configures pdcch-BlindDetection only if the UE is in NR-DC. The network configures pdcch-BlindDetection2 only if the UE is in NR-DC with at least one downlink cell using Rel-16 PDCCH monitoring capability. The network configures pdcch-BlindDetection3 only if the UE is in NR-DC with at least one downlink cell using Rel-15 PDCCH monitoring capability.

pdcch-BlindDetectionCA-CombIndicator Configure one combination of pdcch-BlindDetectionCA1 (for R15) and pdcch-BlindDetectionCA2 (for R16) for UE to use for scaling PDCCH monitoring capability if the number of serving cells configured to a UE is larger than the reported capability, and if UE reports more than one combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2 as UE capability. The combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2) configured by pdcch-BlindDetectionCACombIndicator is from the more than one combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2 reported by UE (see TS 38.213 [13], clause 10).

p-NR-FR1 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 1 (FR1). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR1 (configured total for all serving cells operating on FR1).

p-NR-FR2 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 2 (FR2). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR2 (configured total for all serving cells operating on FR2). This field is only used in NR-DC.

prioLowDG-HighCG Enable PHY prioritization for the case where low-priority dynamic grant-PUSCH collides with high-priority configured grant-PUSCH on a BWP of a serving cell (see TS 38.213 [13], clause 9), when the UE has generated transport blocks for both DG-PUSCH and CG-PUSCH as described in TS 38.321 [3].

prioHighDG-LowCG Enable PHY prioritization for the case where high-priority dynamic grant PUSCH collides with low-priority configured grant PUSCH on a BWP of a serving cell (see TS 38.213 [13], clause 9), when the UE has generated transport blocks for both DG-PUSCH and CG-PUSCH as described in TS 38.321 [3].

ps-RNTI RNTI value for scrambling CRC of DCI format 2-6 used for power saving (see TS 38.213 [13], clause 10.1).

ps-Offset The start of the search-time of DCI format 2-6 with CRC scrambled by PS-RNTI relative to the start of the drx-onDurationTimer of Long DRX (see TS 38.213 [13], clause 10.3). Value in multiples of 0.125ms (milliseconds). 1 corresponds to 0.125 ms, 2 corresponds to 0.25 ms, 3 corresponds to 0.375 ms and so on.

ps-WakeUp Indicates the UE to wake-up if DCI format 2-6 is not detected outside active time (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not wake-up if DCI format 2-6 is not detected outside active time.

ps-PositionDCI-2-6 Starting position of UE wakeup and SCell dormancy indication in DCI format 2-6 (see TS 38.213 [13], clause 10.3).

ps-TransmitPeriodicL1-RSRP Indicates the UE to transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start.

ps-TransmitOtherPeriodicCSI Indicates the UE to transmit periodic CSI report(s) other than L1-RSRP reports when the drx-onDurationTimer does not start (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic CSI report(s) other than L1-RSRP reports when the drx-onDurationTimer does not start.

p-UE-FR1 The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR1 (configured for the cell group).

p-UE-FR2 The maximum total transmit power to be used by the UE across all serving cells in frequency range 2 (FR2) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR2 (configured for the cell group).

pdsch-HARQ-ACK-Codebook The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to both CA and none CA operation (see TS 38.213 [13], clauses 9.1.2 and 9.1.3). If pdsch-HARQ-ACK-Codebook-r16 is signalled, UE shall ignore the pdsch-HARQ-ACK-Codebook (without suffix). For the HARQ-ACK for sidelink, if pdsch-HARQ-ACK-Codebook-r16 is signalled, the UE uses pdsch-HARQ-ACK-Codebook (without suffix) and ignores pdsch-HARQ-ACK-Codebook-r16. If the field pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup is present, pdsch-HARQ-ACK-Codebook is applied to primary PUCCH group. Otherwise, this field is applied to the cell group (i.e. for all the cells within the cell group). For the HARQ-ACK for sidelink, if the field pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup is present, pdsch-HARQ-ACK-Codebook is applied to primary and secondary PUCCH group and the UE ignores pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup.

pdsch-HARQ-ACK-CodebookList A list of configurations for one or two HARQ-ACK codebooks. Each configuration in the list is defined in the same way as pdsch-HARQ-ACK-Codebook (see TS 38.212 [17], clause 7.3.1.2.2 and TS 38.213 [13], clauses 7.2.1, 9.1.2, 9.1.3 and 9.2.1). If this field is present, the field pdsch-HARQ-ACK-Codebook is ignored. If this field is present, the value of this field is applied for primary PUCCH group and for secondary PUCCH group (if configured). For the HARQ-ACK for sidelink, the UE uses pdsch-HARQ-ACK-Codebook and ignores pdsch-HARQ-ACK-CodebookList if this field is present.

pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to CA operation (see TS 38.213 [13], clauses 9.1.2 and 9.1.3). It is configured for secondary PUCCH group.

pdsch-HARQ-ACK-EnhType3DCI-Field Indicates the enhanced Type 3 codebook through a new DCI field to indicate the enhanced Type 3 HARQ-ACK codebook in the secondary PUCCH group if the more than one enhanced Type HARQ-ACK codebook is configured for the primary PUCCH group.

pdsch-HARQ-ACK-EnhType3ToAddModList, pdsch-HARQ-ACK-EnhType3SecondaryToAddModList Configure the list of enhanced Type 3 HARQ-ACK codebooks for the primary PUCCH group and the secondary PUCCH group, respectively. When configured, DCI_format 1_1 can request the UE to report A/N for one of the configured enhanced Type 3 HARQ-ACK codebooks in the corresponding PUCCH group (see TS 38.213 [13], clause 9.1.4).

pdsch-HARQ-ACK-EnhType3SecondaryPUCCHgroup Enables the enhanced Type 3 codebook through a DCI field to indicate the enhanced Type 3 HARQ-ACK codebook in the secondary PUCCH group if the more than one enhanced Type 3 HARQ-ACK codebook is configured for the secondary PUCCH group.

pdsch-HARQ-ACK-OneShotFeedback When configured, the DCI_format 1_1 can request the UE to report A/N for all HARQ processes and all CCs configured in the PUCCH group (see TS 38.212 [17], clause 7.3.1).

pdsch-HARQ-ACK-OneShotFeedbackCBG When configured, the DCI_format 1_1 can request the UE to include CBG level A/N for each CC with CBG level transmission configured. When not configured, the UE will report TB level A/N even if CBG level transmission is configured for a CC. The network configures this only when pdsch-HARQ-ACK-OneShotFeedback is configured.

pdsch-HARQ-ACK-OneShotFeedbackNDI When configured, the DCI_format 1_1 can request the UE to include NDI for each A/N reported. The network configures this only when pdsch-HARQ-ACK-OneShotFeedback is configured.

pdsch-HARQ-ACK-Retx, pdsch-HARQ-ACK-RetxSecondaryPUCCHgroup When configured, the DCI format 1_1 can request the UE to perform a HARQ-ACK re-transmission on a PUCCH resource in the primary PUCCH group and the secondary PUCCH group, respectively (see TS 38.213 [13], clause 9.1.5).

pucch-sSCell, pucch-sSCellSecondaryPUCCHgroup indictates the alternative PUCCH cells for PUCCH cell switching in the primary and the secondary PUCCH group, respectively. For the primary PUCCH group, it is configured for cells on top of SpCell. For the secondary PUCCH group, it is configured for cell on top of the PUCCH SCell.

pucch-sSCellDyn, pucch-sSCellDynsecondaryPUCCHgroup When configured, PUCCH cell switching based on dynamic indication in DCI format 1_1 is enabled (see TS 38.213 [13], clause 9.A, clause 9.1.5), respectively for the primary PUCCH group and the secondary PUCCH group.

pucch-sSCellPattern, pucch-sSCellPatternSecondaryPUCCHgroup When configured, the UE applies the semi-static PUCCH cell switching (see TS 38.213 [13], clause 9.A) using the time domain pattern of applicable PUCCH cells indicated by this field, respectively for the primary PUCCH group and the secondary PUCCH group.

simultaneousPUCCH-PUSCH, simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup Enables simultaneous PUCCH and PUSCH transmissions with different priorities for the primary PUCCH group and the secondary PUCCH group, respectively.

sizeDCI-2-6 Size of DCI format 2-6 (see TS 38.213 [13], clause 11.5).

sp-CSI-RNTI RNTI for Semi-Persistent CSI reporting on PUSCH (see CSI-ReportConfig) (see TS 38.214 [19], clause 5.2.1.5.2). Network always configures the UE with a value for this field when at least one CSI-ReportConfig with reportConfigType set to semiPersistentOnPUSCH is configured.

tpc-PUCCH-RNTI RNTI used for PUCCH TPC commands on DCI (see TS 38.213 [13], clause 10.1).

tpc-PUSCH-RNTI RNTI used for PUSCH TPC commands on DCI (see TS 38.213 [13], clause 10.1).

tpc-SRS-RNTI RNTI used for SRS TPC commands on DCI (see TS 38.213 [13], clause 10.1).

ul-TotalDAI-Included Indicates whether the total DAI fields of the additonal PDSCH group is included in the non-fallback UL grant DCI (see TS 38.212 [17], clause 7.3.1). The network configures this only when enhanced dynamic codebook is configured (pdsch-HARQ-ACK-Codebook is set to enhancedDynamic).

uci-MuxWithDiffPrio, uci-MuxWithDiffPrio-secondaryPUCCHgroup When configured, enables multiplexing a high-priority (HP) HARQ-ACK UCI and a low-priority (LP) HARQ-ACK UCI into a PUCCH or PUSCH for the primary PUCCH group and the secondary PUCCH group, respectively.

xScale The UE is allowed to drop NR only if the power scaling applied to NR results in a difference between scaled and unscaled NR UL of more than xScale dB (see TS 38.213 [13]). If the value is not configured for dynamic power sharing, the UE assumes default value of 6 dB.

| Conditional Presence | Explanation |
| --- | --- |
| MCG-Only | This field is optionally present, Need R, in the PhysicalCellGroupConfig of the MCG. It is absent otherwise. |
| SCG-Only | This field is optionally present, Need S, in the PhysicalCellGroupConfig of the SCG in (NG)EN-DC as defined in TS 38.213 [13]. It is absent otherwise. |
| twoPUCCHgroup | This field is optionally present, Need R, if secondary PUCCH group is configured. It is absent otherwise. |

PDSCH-HARQ-ACK-EnhType3 field descriptions

pdsch-HARQ-ACK-EnhType3CBG When configured, the DCI_format 1_1 or DCI format 1_2 can request the UE to include CBG level A/N for each CC with CBG level transmission configured of the enhanced Type 3 HARQ-ACK codebook. When not configured, the UE will report TB level A/N even if CBG level transmission is configured for a CC.

pdsch-HARQ-ACK-EnhType3NDI When configured, the DCI_format 1_1 or DCI format 1_2 can request the UE to include NDI for each A/N reported of the enhanced Type 3 HARQ-ACK codebook.

perCC Configures enhanced Type 3 HARQ-ACK codebook using per CC configuration.

perHARQ Configures enhanced Type 3 HARQ-ACK codebook using per HARQ process and CC configuration.

next change

–	PUCCH-Config

The IE PUCCH-Config is used to configure UE specific PUCCH parameters (per BWP).

PUCCH-Config information element

-- ASN1START

-- TAG-PUCCH-CONFIG-START

PUCCH-Config ::=                        SEQUENCE {

resourceSetToAddModList                 SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSet   OPTIONAL, -- Need N

resourceSetToReleaseList                SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSetId OPTIONAL, -- Need N

resourceToAddModList                    SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource         OPTIONAL, -- Need N

resourceToReleaseList                   SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-ResourceId       OPTIONAL, -- Need N

format1                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format2                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format3                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format4                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

schedulingRequestResourceToAddModList   SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig

OPTIONAL, -- Need N

schedulingRequestResourceToReleaseList  SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceId

OPTIONAL, -- Need N

multi-CSI-PUCCH-ResourceList            SEQUENCE (SIZE (1..2)) OF PUCCH-ResourceId                            OPTIONAL, -- Need M

dl-DataToUL-ACK                         SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)                             OPTIONAL, -- Need M

spatialRelationInfoToAddModList         SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfo

OPTIONAL, -- Need N

spatialRelationInfoToReleaseList        SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfoId

OPTIONAL, -- Need N

pucch-PowerControl                      PUCCH-PowerControl                                                    OPTIONAL, -- Need M

...,

[[

resourceToAddModListExt-v1610           SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-ResourceExt-v1610  OPTIONAL, -- Need N

dl-DataToUL-ACK-r16                     SetupRelease { DL-DataToUL-ACK-r16 }                                  OPTIONAL, -- Need M

ul-AccessConfigListDCI-1-1-r16          SetupRelease { UL-AccessConfigListDCI-1-1-r16 }                       OPTIONAL, -- Need M

subslotLengthForPUCCH-r16               CHOICE {

normalCP-r16                        ENUMERATED {n2,n7},

extendedCP-r16                      ENUMERATED {n2,n6}

}                                                                                                             OPTIONAL, -- Need R

dl-DataToUL-ACK-DCI-1-2-r16             SetupRelease { DL-DataToUL-ACK-DCI-1-2-r16}                           OPTIONAL, -- Need M

numberOfBitsForPUCCH-ResourceIndicatorDCI-1-2-r16  INTEGER (0..3)                                             OPTIONAL, -- Need R

dmrs-UplinkTransformPrecodingPUCCH-r16  ENUMERATED {enabled}                                                  OPTIONAL,  -- Cond PI2-BPSK

spatialRelationInfoToAddModListSizeExt-v1610    SEQUENCE (SIZE (1..maxNrofSpatialRelationInfosDiff-r16)) OF PUCCH-SpatialRelationInfo

OPTIONAL, -- Need N

spatialRelationInfoToReleaseListSizeExt-v1610   SEQUENCE (SIZE (1..maxNrofSpatialRelationInfosDiff-r16)) OF PUCCH-SpatialRelationInfoId

OPTIONAL, -- Need N

spatialRelationInfoToAddModListExt-v1610  SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos-r16)) OF PUCCH-SpatialRelationInfoExt-r16

OPTIONAL, -- Need N

spatialRelationInfoToReleaseListExt-v1610    SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos-r16)) OF

PUCCH-SpatialRelationInfoId-r16       OPTIONAL, -- Need N

resourceGroupToAddModList-r16           SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceGroups-r16)) OF PUCCH-ResourceGroup-r16

OPTIONAL, -- Need N

resourceGroupToReleaseList-r16          SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceGroups-r16)) OF PUCCH-ResourceGroupId-r16

OPTIONAL, -- Need N

sps-PUCCH-AN-List-r16                   SetupRelease { SPS-PUCCH-AN-List-r16 }                                OPTIONAL,  -- Need M

schedulingRequestResourceToAddModListExt-v1610   SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfigExt-v1610

OPTIONAL, -- Need N

]]

[[

format0-r17                             SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format2Ext-r17                          SetupRelease { PUCCH-FormatConfigExt-r17 }                            OPTIONAL, -- Need M

format3Ext-r17                          SetupRelease { PUCCH-FormatConfigExt-r17 }                            OPTIONAL, -- Need M

format4Ext-r17                          SetupRelease { PUCCH-FormatConfigExt-r17 }                            OPTIONAL, -- Need M

ul-AccessConfigListDCI-1-2-r17          SetupRelease { UL-AccessConfigListDCI-1-2-r17 }                       OPTIONAL -- Need M

]]

}

PUCCH-FormatConfig ::=                  SEQUENCE {

interslotFrequencyHopping               ENUMERATED {enabled}                                                  OPTIONAL, -- Need R

additionalDMRS                          ENUMERATED {true}                                                     OPTIONAL, -- Need R

maxCodeRate                             PUCCH-MaxCodeRate                                                     OPTIONAL, -- Need R

nrofSlots                               ENUMERATED {n2,n4,n8}                                                 OPTIONAL, -- Need S

pi2BPSK                                 ENUMERATED {enabled}                                                  OPTIONAL, -- Need R

simultaneousHARQ-ACK-CSI                ENUMERATED {true}                                                     OPTIONAL  -- Need R

}

PUCCH-FormatConfigExt-V17xy ::=               SEQUENCE {

maxCodeRateLP-v17xy                       PUCCH-MaxCodeRate                                                   OPTIONAL,  -- Need R

...

}

PUCCH-MaxCodeRate ::=                   ENUMERATED {zeroDot08, zeroDot15, zeroDot25, zeroDot35, zeroDot45, zeroDot60, zeroDot80}

-- A set with one or more PUCCH resources

PUCCH-ResourceSet ::=                   SEQUENCE {

pucch-ResourceSetId                     PUCCH-ResourceSetId,

resourceList                            SEQUENCE (SIZE (1..maxNrofPUCCH-ResourcesPerSet)) OF PUCCH-ResourceId,

maxPayloadSize                          INTEGER (4..256)                                                      OPTIONAL  -- Need R

}

PUCCH-ResourceSetId ::=                 INTEGER (0..maxNrofPUCCH-ResourceSets-1)

PUCCH-Resource ::=                      SEQUENCE {

pucch-ResourceId                        PUCCH-ResourceId,

startingPRB                             PRB-Id,

intraSlotFrequencyHopping               ENUMERATED { enabled }                                                OPTIONAL, -- Need R

secondHopPRB                            PRB-Id                                                                OPTIONAL, -- Need R

format                                  CHOICE {

format0                                 PUCCH-format0,

format1                                 PUCCH-format1,

format2                                 PUCCH-format2,

format3                                 PUCCH-format3,

format4                                 PUCCH-format4

}

}

PUCCH-ResourceExt-v1610 ::=               SEQUENCE {

interlaceAllocation-r16                   SEQUENCE {

rb-SetIndex                             INTEGER (0..4),

interlace0                              CHOICE {

scs15                                   INTEGER (0..9),

scs30                                   INTEGER (0..4)

}

}                                                                                                             OPTIONAL,  --Need R

format-v1610                              CHOICE {

interlace1-v1610                            INTEGER (0..9),

occ-v1610                                   SEQUENCE {

occ-Length-v1610                                ENUMERATED {n2,n4}                                       OPTIONAL, -- Need M

occ-Index-v1610                                 ENUMERATED {n0,n1,n2,n3}                                 OPTIONAL  -- Need M

}

}                                                                                                            OPTIONAL,  -- Need R

...

}

PUCCH-ResourceId ::=                    INTEGER (0..maxNrofPUCCH-Resources-1)

PUCCH-format0 ::=                               SEQUENCE {

initialCyclicShift                              INTEGER(0..11),

nrofSymbols                                     INTEGER (1..2),

startingSymbolIndex                             INTEGER(0..13)

}

PUCCH-format1 ::=                               SEQUENCE {

initialCyclicShift                              INTEGER(0..11),

nrofSymbols                                     INTEGER (4..14),

startingSymbolIndex                             INTEGER(0..10),

timeDomainOCC                                   INTEGER(0..6)

}

PUCCH-format2 ::=                               SEQUENCE {

nrofPRBs                                        INTEGER (1..16),

nrofSymbols                                     INTEGER (1..2),

startingSymbolIndex                             INTEGER(0..13)

}

PUCCH-format3 ::=                               SEQUENCE {

nrofPRBs                                        INTEGER (1..16),

nrofSymbols                                     INTEGER (4..14),

startingSymbolIndex                             INTEGER(0..10)

}

PUCCH-format4 ::=                               SEQUENCE {

nrofSymbols                                     INTEGER (4..14),

occ-Length                                      ENUMERATED {n2,n4},

occ-Index                                       ENUMERATED {n0,n1,n2,n3},

startingSymbolIndex                             INTEGER(0..10)

}

PUCCH-ResourceGroup-r16 ::=                SEQUENCE {

pucch-ResourceGroupId-r16                  PUCCH-ResourceGroupId-r16,

resourcePerGroupList-r16                   SEQUENCE (SIZE (1..maxNrofPUCCH-ResourcesPerGroup-r16)) OF PUCCH-ResourceId

}

PUCCH-ResourceGroupId-r16 ::=              INTEGER (0..maxNrofPUCCH-ResourceGroups-1-r16)

DL-DataToUL-ACK-r16 ::=                    SEQUENCE (SIZE (1..8)) OF INTEGER (-1..15)

DL-DataToUL-ACK-DCI-1-2-r16 ::=            SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)

UL-AccessConfigListDCI-1-1-r16 ::=         SEQUENCE (SIZE (1..16)) OF INTEGER (0..15)

UL-AccessConfigListDCI-1-2-r17 ::=         SEQUENCE (SIZE (1..16)) OF INTEGER (0..15)

-- TAG-PUCCH-CONFIG-STOP

-- ASN1STOP

PUCCH-Config field descriptions

dl-DataToUL-ACK, dl-DataToUL-ACK-DCI-1-2 List of timing for given PDSCH to the DL ACK (see TS 38.213 [13], clause 9.1.2). The field dl-DataToUL-ACK applies to DCI format 1_1 and the field dl-DataToUL-ACK-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3). If dl-DataToUL-ACK-r16 is signalled, UE shall ignore the dl-DataToUL-ACK (without suffix). The value -1 corresponds to "non-numerical value" for the case where the A/N feedback timing is not explicitly included at the time of scheduling PDSCH.

dmrs-UplinkTransformPrecodingPUCCH This field is used for PUCCH formats 3 and 4 according to TS 38.211, Clause 6.4.1.3.3.1.

format0 Parameters that are common for all PUCCH resources of format 0.

format1 Parameters that are common for all PUCCH resources of format 1.

format2 Parameters that are common for all PUCCH resources of format 2.

format3 Parameters that are common for all PUCCH resources of format 3.

format4. Parameters that are common for all PUCCH resources of format 4

numberOfBitsForPUCCH- ResourceIndicatorDCI-1-2 Configuration of the number of bits for "PUCCH resource indicator" in DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3).

resourceGroupToAddModList, resourceGroupToReleaseList Lists for adding and releasing groups of PUCCH resources that can be updated simultaneously for spatial relations with a MAC CE

resourceSetToAddModList, resourceSetToReleaseList Lists for adding and releasing PUCCH resource sets (see TS 38.213 [13], clause 9.2).

resourceToAddModList, resourceToAddModListExt, resourceToReleaseList Lists for adding and releasing PUCCH resources applicable for the UL BWP and serving cell in which the PUCCH-Config is defined. The resources defined herein are referred to from other parts of the configuration to determine which resource the UE shall use for which report. If the network includes of resourceToAddModListExt, it includes the same number of entries, and listed in the same order, as in resourceToAddModList.

spatialRelationInfoToAddModList, spatialRelationInfoToAddModListSizeExt , spatialRelationInfoToAddModListExt Configuration of the spatial relation between a reference RS and PUCCH. Reference RS can be SSB/CSI-RS/SRS. If the list has more than one element, MAC-CE selects a single element (see TS 38.321 [3], clause 5.18.8 and TS 38.213 [13], clause 9.2.2). The UE shall consider entries in spatialRelationInfoToAddModList and in spatialRelationInfoToAddModListSizeExt as a single list, i.e. an entry created using spatialRelationInfoToAddModList can be modified using spatialRelationInfoToAddModListSizeExt (or deleted using spatialRelationInfoToReleaseListSizeExt) and vice-versa. If the network includes spatialRelationInfoToAddModListExt, it includes the same number of entries, and listed in the same order, as in the concatenation of spatialRelationInfoToAddModList and of spatialRelationInfoToAddModListSizeExt.

spatialRelationInfoToReleaseList, spatialRelationInfoToReleaseListSizeExt, spatialRelationInfoToReleaseListExt Lists of spatial relation configurations between a reference RS and PUCCH to be released by the UE.

sps-PUCCH-AN-List Indicates a list of PUCCH resources for DL SPS HARQ ACK. The field maxPayloadSize is absent for the first and the last SPS-PUCCH-AN in the list. If configured, this overrides n1PUCCH-AN in SPS-config.

subslotLengthForPUCCH Indicate the sub-slot length for sub-slot based PUCCH feedback in number of symbols (see TS 38.213 [13], clause 9). Value n2 corresponds to 2 symbols, value n6 corresponding to 6 symbols, value n7 corresponds to 7 symbols. For normal CP, the value is either n2 or n7. For extended CP, the value is either n2 or n6.

ul-AccessConfigListDCI-1-1, ul-AccessConfigListDCI-1-2 List of the combinations of cyclic prefix extension and UL channel access type (See TS 38.212 [17], Clause 7.3.1) applicable, respectively, to DCI format 1_1 and DCI format 1_2.

PUCCH-format3 field descriptions

nrofPRBs The supported values are 1,2,3,4,5,6,8,9,10,12,15 and 16. The UE shall ignore this field when formatExt is configured.

PUCCH-FormatConfig, PUCCH-FormatConfigExt field descriptions

additionalDMRS If the field is present, the UE enables 2 DMRS symbols per hop of a PUCCH Format 3 or 4 if both hops are more than X symbols when FH is enabled (X=4). And it enables 4 DMRS symbols for a PUCCH Format 3 or 4 with more than 2X+1 symbols when FH is disabled (X=4). The field is not applicable for format 0, 1 and 2. See TS 38.213 [13], clause 9.2.2.

interslotFrequencyHopping If the field is present, the UE enables inter-slot frequency hopping when PUCCH Format 0, 1, 3 or 4 is repeated over multiple slots. For long PUCCH over multiple slots, the intra and inter slot frequency hopping cannot be enabled at the same time for a UE. The field is not applicable for format 2. See TS 38.213 [13], clause 9.2.6.

maxCodeRate Max coding rate to determine how to feedback UCI on PUCCH for format 2, 3 or 4. The field is not applicable for format 0 and 1. See TS 38.213 [13], clause 9.2.5.

maxCodeRateLP Max coding rate to determine how to feedback UCI on PUCCH for format 2, 3 or 4. The field is not applicable for format 0 and 1. This field configures additional max code rate in the second entry of PUCCH-ConfigurationList-r16 for multiplexing low-priority (LP) HARQ-ACK and high-priority (HP) UCI in a PUCCH as described Clause 9.2.5.3 of TS 38.213. The field is absent for the first entry of PUCCH-ConfigurationList-r16.

nrofSlots Number of slots with the same PUCCH. When the field is absent the UE applies the value n1. The field is not applicable for format 2. See TS 38.213 [13], clause 9.2.6.

pi2BPSK If the field is present, the UE uses pi/2 BPSK for UCI symbols instead of QPSK for PUCCH. The field is not applicable for format 0, 1 and 2. See TS 38.213 [13], clause 9.2.5.

rb-SetIndex Indicates the RB set where PUCCH resource is allocated.

simultaneousHARQ-ACK-CSI If the field is present, the UE uses simultaneous transmission of CSI and HARQ-ACK feedback with or without SR with PUCCH Format 2, 3 or 4. See TS 38.213 [13], clause 9.2.5. When the field is absent the UE applies the value off. The field is not applicable for format 0 and 1.

PUCCH-Resource, PUCCH-ResourceExt field descriptions

format, formatExt Selection of the PUCCH format (format 0 – 4) and format-specific parameters, see TS 38.213 [13], clause 9.2. format0 and format1 are only allowed for a resource in a first PUCCH resource set. format2, format3 and format4 are only allowed for a resource in non-first PUCCH resource set. The network can only configure formatExt when format is set to format2 or format3.

interlace0 This is the only interlace of interlaced PUCCH Format 0 and 1 and the first interlace for interlaced PUCCH Format 2 and 3.

interlace1 A second interlace, in addition to interlace 0, as specified in TS 38.213 [13], clause 9.2.1. For 15KHz SCS, values {0..9} are applicable; for 30Khz SCS, values {0..4} are applicable. For 15kHz SCS, the values of interlace1 shall satisfy interlace1=mod(interlace0+X,10) where X=1, -1, or 5.

intraSlotFrequencyHopping Enabling intra-slot frequency hopping, applicable for all types of PUCCH formats. For long PUCCH over multiple slots, the intra and inter slot frequency hopping cannot be enabled at the same time for a UE. See TS 38.213 [13], clause 9.2.1.

occ-Index Indicates the orthogonal cover code index (see TS 38.213 [13], clause 9.2.1). This field is Applicable when useInterlacePUCCH-Dedicated-r16 is configured.

occ-Length Indicates the orthogonal cover code length (see TS 38.213 [13], clause 9.2.1). Applicable when useInterlacePUCCH-Dedicated-r16 is configured.

pucch-ResourceId Identifier of the PUCCH resource.

secondHopPRB Index of first PRB after frequency hopping of PUCCH. This value is applicable for intra-slot frequency hopping (see TS 38.213 [13], clause 9.2.1) or inter-slot frequency hopping (see TS 38.213 [13], clause 9.2.6).

PUCCH-ResourceSet field descriptions

maxPayloadSize Maximum number of UCI information bits that the UE may transmit using this PUCCH resource set (see TS 38.213 [13], clause 9.2.1). In a PUCCH occurrence, the UE chooses the first of its PUCCH-ResourceSet which supports the number of bits that the UE wants to transmit. The field is absent in the first set (Set0) and in the last configured set since the UE derives the maximum number of UCI information bits as specified in TS 38.213 [13], clause 9.2.1. This field can take integer values that are multiples of 4.

resourceList PUCCH resources of format0 and format1 are only allowed in the first PUCCH resource set, i.e., in a PUCCH-ResourceSet with pucch-ResourceSetId = 0. This set may contain between 1 and 32 resources. PUCCH resources of format2, format3 and format4 are only allowed in a PUCCH-ResourceSet with pucch-ResourceSetId > 0. If present, these sets contain between 1 and 8 resources each. The UE chooses a PUCCH-Resource from this list as specified in TS 38.213 [13], clause 9.2.3. Note that this list contains only a list of resource IDs. The actual resources are configured in PUCCH-Config.

| Conditional Presence | Explanation |
| --- | --- |
| PI2-BPSK | The field is optionally present, Need R, if format3 and/or format4 are configured and pi2BPSK is configured in each of them. It is absent, Need R otherwise. |

next change

–	PUCCH-TPC-CommandConfig

The IE PUCCH-TPC-CommandConfig is used to configure the UE for extracting TPC commands for PUCCH from a group-TPC messages on DCI.

PUCCH-TPC-CommandConfig information element

-- ASN1START

-- TAG-PUCCH-TPC-COMMANDCONFIG-START

PUCCH-TPC-CommandConfig ::=             SEQUENCE {

tpc-IndexPCell                          INTEGER (1..15)                         OPTIONAL,   -- Cond PDCCH-OfSpcell

tpc-IndexPUCCH-SCell                    INTEGER (1..15)                         OPTIONAL,   -- Cond PDCCH-ofSpCellOrPUCCH-SCell

...

[[

tpc-IndexPUCCH-sSCell-r17                              INTEGER (1..15)                         OPTIONAL,   -- Need R

tpc-IndexPUCCH-sScellSecondaryPUCCHgroup-r17          INTEGER (1..15)                         OPTIONAL   -- Cond twoPUCCHgroup

]]

}

-- TAG-PUCCH-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

PUCCH-TPC-CommandConfig field descriptions

tpc-IndexPCell An index determining the position of the first bit of TPC command (applicable to the SpCell) inside the DCI format 2-2 payload.

tpc-IndexPUCCH-SCell An index determining the position of the first bit of TPC command (applicable to the PUCCH SCell) inside the DCI format 2-2 payload.

tpc-IndexPUCCH-sSCell, tpc-IndexPUCCH-sSCellSecondaryPUCCHgroup An index determining the position of the first bit of TPC command (applicable to the alternative PUCCH cell for PUCCH cell switching) inside the DCI format 2-2 payload, for the primary PUCCH group and the secondary PUCCH group respectively.

| Conditional Presence | Explanation |
| --- | --- |
| PDCCH-OfSpcell | The field is mandatory present if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the SpCell. Otherwise, the field is absent, Need R. |
| PDCCH-ofSpCellOrPUCCH-SCell | The field is mandatory present if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the PUCCH-SCell. The field is optionally present, need R, if the UE is configured with a PUCCH SCell in this cell group and if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the SpCell. Otherwise, the field is absent, Need R. |
| twoPUCCHgroup | This field is optionally present, Need R, if secondary PUCCH group is configured. It is absent otherwise. |

–	PUSCH-Config

The IE PUSCH-Config is used to configure the UE specific PUSCH parameters applicable to a particular BWP.

PUSCH-Config information element

-- ASN1START

-- TAG-PUSCH-CONFIG-START

PUSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPUSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

txConfig                                ENUMERATED {codebook, nonCodebook}                                  OPTIONAL,   -- Need S

dmrs-UplinkForPUSCH-MappingTypeA        SetupRelease { DMRS-UplinkConfig }                                  OPTIONAL,   -- Need M

dmrs-UplinkForPUSCH-MappingTypeB        SetupRelease { DMRS-UplinkConfig }                                  OPTIONAL,   -- Need M

pusch-PowerControl                      PUSCH-PowerControl                                                  OPTIONAL,   -- Need M

frequencyHopping                        ENUMERATED {intraSlot, interSlot}                                   OPTIONAL,   -- Need S

frequencyHoppingOffsetLists             SEQUENCE (SIZE (1..4)) OF INTEGER (1.. maxNrofPhysicalResourceBlocks-1)

OPTIONAL,   -- Need M

resourceAllocation                      ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pusch-TimeDomainAllocationList          SetupRelease { PUSCH-TimeDomainResourceAllocationList }             OPTIONAL,   -- Need M

pusch-AggregationFactor                 ENUMERATED { n2, n4, n8 }                                           OPTIONAL,   -- Need S

mcs-Table                               ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

mcs-TableTransformPrecoder              ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

transformPrecoder                       ENUMERATED {enabled, disabled}                                      OPTIONAL,   -- Need S

codebookSubset                          ENUMERATED {fullyAndPartialAndNonCoherent, partialAndNonCoherent,nonCoherent}

OPTIONAL, -- Cond codebookBased

maxRank                                 INTEGER (1..4)                                                OPTIONAL, -- Cond codebookBased

rbg-Size                                ENUMERATED { config2}                                         OPTIONAL, -- Need S

uci-OnPUSCH                             SetupRelease { UCI-OnPUSCH}                                   OPTIONAL, -- Need M

tp-pi2BPSK                              ENUMERATED {enabled}                                          OPTIONAL, -- Need S

...,

[[

minimumSchedulingOffsetK2-r16           SetupRelease { MinSchedulingOffsetK2-Values-r16 }             OPTIONAL,  -- Need M

ul-AccessConfigListDCI-0-1-r16          SetupRelease { UL-AccessConfigListDCI-0-1-r16 }               OPTIONAL,  -- Need M

-- Start of the parameters for DCI format 0_2 introduced in V16.1.0

harq-ProcessNumberSizeDCI-0-2-r16                       INTEGER (0..4)                                OPTIONAL,   -- Need R

dmrs-SequenceInitializationDCI-0-2-r16                  ENUMERATED {enabled}                          OPTIONAL,   -- Need S

numberOfBitsForRV-DCI-0-2-r16                           INTEGER (0..2)                                OPTIONAL,   -- Need R

antennaPortsFieldPresenceDCI-0-2-r16                    ENUMERATED {enabled}                          OPTIONAL,   -- Need S

dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2-r16            SetupRelease { DMRS-UplinkConfig }            OPTIONAL,   -- Need M

dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2-r16            SetupRelease { DMRS-UplinkConfig }            OPTIONAL,   -- Need M

frequencyHoppingDCI-0-2-r16                             CHOICE {

pusch-RepTypeA                                          ENUMERATED {intraSlot, interSlot},

pusch-RepTypeB                                          ENUMERATED {interRepetition, interSlot}

}                                                                                                     OPTIONAL,   -- Need S

frequencyHoppingOffsetListsDCI-0-2-r16  SetupRelease { FrequencyHoppingOffsetListsDCI-0-2-r16}        OPTIONAL,  -- Need M

codebookSubsetDCI-0-2-r16               ENUMERATED {fullyAndPartialAndNonCoherent, partialAndNonCoherent,nonCoherent}

OPTIONAL,   -- Cond codebookBased

invalidSymbolPatternIndicatorDCI-0-2-r16                ENUMERATED {enabled}                          OPTIONAL,   -- Need S

maxRankDCI-0-2-r16                                      INTEGER (1..4)                                OPTIONAL,   -- Cond codebookBased

mcs-TableDCI-0-2-r16                                    ENUMERATED {qam256, qam64LowSE}               OPTIONAL,   -- Need S

mcs-TableTransformPrecoderDCI-0-2-r16                   ENUMERATED {qam256, qam64LowSE}               OPTIONAL,   -- Need S

priorityIndicatorDCI-0-2-r16                            ENUMERATED {enabled}                          OPTIONAL,   -- Need S

pusch-RepTypeIndicatorDCI-0-2-r16                       ENUMERATED { pusch-RepTypeA, pusch-RepTypeB}  OPTIONAL,  -- Need R

resourceAllocationDCI-0-2-r16                           ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch}

OPTIONAL,   -- Need M

resourceAllocationType1GranularityDCI-0-2-r16           ENUMERATED { n2,n4,n8,n16 }                   OPTIONAL,   -- Need S

uci-OnPUSCH-ListDCI-0-2-r16                             SetupRelease { UCI-OnPUSCH-ListDCI-0-2-r16}   OPTIONAL,   -- Need M

pusch-TimeDomainAllocationListDCI-0-2-r16               SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,   -- Need M

-- End of the parameters for DCI format 0_2 introduced in V16.1.0

-- Start of the parameters for DCI format 0_1 introduced in V16.1.0

pusch-TimeDomainAllocationListDCI-0-1-r16               SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,   -- Need M

invalidSymbolPatternIndicatorDCI-0-1-r16          ENUMERATED {enabled}                                OPTIONAL,   -- Need S

priorityIndicatorDCI-0-1-r16                      ENUMERATED {enabled}                                OPTIONAL,   -- Need S

pusch-RepTypeIndicatorDCI-0-1-r16                 ENUMERATED { pusch-RepTypeA, pusch-RepTypeB}        OPTIONAL,   -- Need R

frequencyHoppingDCI-0-1-r16                 ENUMERATED {interRepetition, interSlot}                   OPTIONAL,   -- Cond RepTypeB

uci-OnPUSCH-ListDCI-0-1-r16                 SetupRelease { UCI-OnPUSCH-ListDCI-0-1-r16  }             OPTIONAL,  -- Need M

-- End of the parameters for DCI format 0_1 introduced in V16.1.0

invalidSymbolPattern-r16                    InvalidSymbolPattern-r16                                  OPTIONAL,   -- Need S

pusch-PowerControl-v1610                SetupRelease {PUSCH-PowerControl-v1610}                       OPTIONAL,   -- Need M

ul-FullPowerTransmission-r16            ENUMERATED {fullpower, fullpowerMode1, fullpowerMode2}         OPTIONAL,   -- Need R

pusch-TimeDomainAllocationListForMultiPUSCH-r16  SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,  --  Need M

numberOfInvalidSymbolsForDL-UL-Switching-r16        INTEGER (1..4)                                    OPTIONAL,    -- Cond RepTypeB2

]]

[[

ul-AccessConfigListDCI-0-2-r17          SetupRelease { UL-AccessConfigListDCI-0-2-r17 }               OPTIONAL,  -- Need M

betaOffsetsCrossPri0-r17                SetupRelease { BetaOffsetsCrossPriSel-r17 }                     OPTIONAL,  -- Need M

betaOffsetsCrossPri1-r17                SetupRelease { BetaOffsetsCrossPriSel-r17 }                     OPTIONAL,  -- Need M

betaOffsetsCrossPri0DCI-0-2-r17         SetupRelease { BetaOffsetsCrossPriSelDCI-0-2-r17 }                     OPTIONAL,  -- Need M

betaOffsetsCrossPri1DCI-0-2-r17         SetupRelease { BetaOffsetsCrossPriSelDCI-0-2-r17 }                     OPTIONAL  -- Need M

]]

}

UCI-OnPUSCH ::=                         SEQUENCE {

betaOffsets                             CHOICE {

dynamic                             SEQUENCE (SIZE (4)) OF BetaOffsets,

semiStatic                          BetaOffsets

}                                                                                                 OPTIONAL, -- Need M

scaling                                 ENUMERATED { f0p5, f0p65, f0p8, f1 }

}

MinSchedulingOffsetK2-Values-r16 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK2-SchedulingOffset-r16)

UCI-OnPUSCH-DCI-0-2-r16 ::=             SEQUENCE {

betaOffsetsDCI-0-2-r16                  CHOICE {

dynamicDCI-0-2-r16                      CHOICE {

oneBit-r16                              SEQUENCE (SIZE (2)) OF BetaOffsets,

twoBits-r16                             SEQUENCE (SIZE (4)) OF BetaOffsets

},

semiStaticDCI-0-2-r16          BetaOffsets

}                                                                                                 OPTIONAL,   -- Need M

scalingDCI-0-2-r16                 ENUMERATED { f0p5, f0p65, f0p8, f1 }

}

FrequencyHoppingOffsetListsDCI-0-2-r16 ::=  SEQUENCE (SIZE (1..4)) OF INTEGER (1.. maxNrofPhysicalResourceBlocks-1)

UCI-OnPUSCH-ListDCI-0-2-r16 ::=  SEQUENCE (SIZE (1..2)) OF UCI-OnPUSCH-DCI-0-2-r16

UCI-OnPUSCH-ListDCI-0-1-r16 ::=  SEQUENCE (SIZE (1..2)) OF UCI-OnPUSCH

UL-AccessConfigListDCI-0-1-r16 ::= SEQUENCE (SIZE (1..64)) OF INTEGER (0..63)

UL-AccessConfigListDCI-0-2-r17 ::= SEQUENCE (SIZE (1..64)) OF INTEGER (0..63)

BetaOffsetsCrossPriSel-r17 ::= CHOICE {

dynamic-r17         SEQUENCE (SIZE (4)) OF BetaOffsetsCrossPri-r17,

semiStatic-r17          BetaOffsetsCrossPri-r17

}

BetaOffsetsCrossPriSelDCI-0-2-r17 ::= CHOICE {

dynamicDCI-0-2-r17         CHOICE {

oneBit-r17              SEQUENCE (SIZE (2)) OF BetaOffsetsCrossPri-r17,

twoBits-r17             SEQUENCE (SIZE (4)) OF BetaOffsetsCrossPri-r17

}

semiStaticDCI-0-2-r17          BetaOffsetsCrossPri-r17

}

-- TAG-PUSCH-CONFIG-STOP

-- ASN1STOP

PUSCH-Config field descriptions

antennaPortsFieldPresenceDCI-0-2 Configure the presence of "Antenna ports" field in DCI format 0_2. When the field is configured, then the "Antenna ports" field is present in DCI format 0_2. Otherwise, the field size is set to 0 for DCI format 0_2 (See TS 38.212 [17], clause 7.3.1.1.3). If neither dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 nor dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 is configured, this field is absent.

betaOffsetsCrossPri0, betaOffsetsCrossPri1, betaOffsetsCrossPri0DCI-0-2, betaOffsetsCrossPri1DCI-0-2 Selection between and configuration of dynamic and semi-static beta-offset for multiplexing HARQ-ACK on dynamically scheduled PUSCH with different priorities, see TS 38.213 [13], clause 9.3. The field betaOffsetsCrossPrio0 indicates multiplexing low priority (LP) HARQ-ACK on dynamically scheduled high priority (HP) PUSCH. The field betaOffsetsCrossPrio1 indicates multiplexing HP HARQ-ACK on dynamically scheduled LP PUSCH. The field betaOffsetsCrossPrio0DCI-0-2 indicates multiplexing LP HARQ-ACK on dynamically scheduled HP PUSCH by DCI format 0_2. The field betaOffsetsCrossPrio1DCI-0-2 indicates multiplexing HP HARQ-ACK on dynamically scheduled LP PUSCH by DCI format 0_2.

codebookSubset, codebookSubsetDCI-0-2 Subset of PMIs addressed by TPMI, where PMIs are those supported by UEs with maximum coherence capabilities (see TS 38.214 [19], clause 6.1.1.1). The field codebookSubset applies to DCI format 0_1 and the field codebookSubsetDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.1.1).

dataScramblingIdentityPUSCH Identifier used to initialise data scrambling (c_init) for PUSCH. If the field is absent, the UE applies the physical cell ID. (see TS 38.211 [16], clause 6.3.1.1).

dmrs-SequenceInitializationDCI-0-2 Configure whether the field "DMRS Sequence Initialization" is present or not in DCI format 0_2. If the field is absent, then 0 bit for the field "DMRS Sequence Initialization" in DCI format 0_2. If the field is present, then the number of bits is determined in the same way as DCI format 0_1 (see TS 38.212 [17], clause 7.3.1).

dmrs-UplinkForPUSCH-MappingTypeA, dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 DMRS configuration for PUSCH transmissions using PUSCH mapping type A (chosen dynamically via PUSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-UplinkForPUSCH-MappingTypeA applies to DCI format 0_1 and the field dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

dmrs-UplinkForPUSCH-MappingTypeB, dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 DMRS configuration for PUSCH transmissions using PUSCH mapping type B (chosen dynamically via PUSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-UplinkForPUSCH-MappingTypeB applies to DCI format 0_1 and the field dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

frequencyHopping The value intraSlot enables 'Intra-slot frequency hopping' and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured for 'pusch-RepTypeA' (see TS 38.214 [19], clause 6.3). The field frequencyHopping applies to DCI format 0_0 and 0_1 for 'pusch-RepTypeA'.

frequencyHoppingDCI-0-1 Indicates the frequency hopping scheme for DCI format 0_1 when pusch-RepTypeIndicatorDCI-0-1 is set to 'pusch-RepTypeB', The value interRepetition enables 'Inter-repetition frequency hopping', and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured for DCI format 0_1 (see TS 38.214 [19], clause 6.1).

frequencyHoppingDCI-0-2 Indicate the frequency hopping scheme for DCI format 0_2. The value intraSlot enables 'intra-slot frequency hopping', and the value interRepetition enables 'Inter-repetition frequency hopping', and the value interSlot enables 'Inter-slot frequency hopping'. When pusch-RepTypeIndicatorDCI-0-2 is set to 'pusch-RepTypeA', the frequency hopping scheme can be chosen between 'intra-slot frequency hopping and 'inter-slot frequency hopping' if enabled. When pusch-RepTypeIndicatorDCI-0-2 is set to 'pusch-RepTypeB', the frequency hopping scheme can be chosen between 'inter-repetition frequency hopping' and 'inter-slot frequency hopping' if enabled. If the field is absent, frequency hopping is not configured for DCI format 0_2 for 'pusch-RepTypeB' (see TS 38.214 [19], clause 6.3).

frequencyHoppingOffsetLists, frequencyHoppingOffsetListsDCI-0-2 Set of frequency hopping offsets used when frequency hopping is enabled for granted transmission (not msg3) and type 2 configured grant activation (see TS 38.214 [19], clause 6.3). The field frequencyHoppingOffsetLists applies to DCI format 0_0 and DCI format 0_1 and the field frequencyHoppingOffsetListsDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.3).

harq-ProcessNumberSizeDCI-0-2 Configure the number of bits for the field "HARQ process number" in DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

invalidSymbolPattern Indicates one pattern for invalid symbols for PUSCH transmission repetition type B applicable to both DCI format 0_1 and 0_2. If InvalidSymbolPattern is not configured, semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols (see TS 38.214 [19] clause 6.1).

invalidSymbolPatternIndicatorDCI-0-1, invalidSymbolPatternIndicatorDCI-0-2 Indicates the presence of an additional bit in the DCI format 0_1/0_2. If invalidSymbolPattern is absent, then both invalidSymbolPatternIndicatorDCI-0-1 and invalidSymbolPatternIndicatorDCI-0-2 are absent. The field invalidSymbolPatternIndicatorDCI-0-1 applies to the DCI format 0_1 and the field invalidSymbolPatternIndicatorDCI-0-1 applies to DCI format 0_2 (see TS 38.214 [19] clause 6.1).

maxRank, maxRankDCI-0-2 Subset of PMIs addressed by TRIs from 1 to ULmaxRank (see TS 38.214 [19], clause 6.1.1.1). The field maxRank applies to DCI format 0_1 and the field maxRankDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.1.1).

mcs-Table, mcs-TableFormat0-2 Indicates which MCS table the UE shall use for PUSCH without transform precoder (see TS 38.214 [19], clause 6.1.4.1). If the field is absent the UE applies the value 64QAM. The field mcs-Table applies to DCI format 0_0 and DCI format 0_1 and the field mcs-TableDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.4.1).

mcs-TableTransformPrecoder, mcs-TableTransformPrecoderDCI-0-2 Indicates which MCS table the UE shall use for PUSCH with transform precoding (see TS 38.214 [19], clause 6.1.4.1) If the field is absent the UE applies the value 64QAM. The field mcs-TableTransformPrecoder applies to DCI format 0_0 and DCI format 0_1 and the field mcs-TableTransformPrecoderDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.4.1).

minimumSchedulingOffsetK2 List of minimum K2 values. Minimum K2 parameter denotes minimum applicable value(s) for the Time domain resource assignment table for PUSCH (see TS 38.214 [19], clause 6.1.2.1).

numberOfBitsRV-DCI-0-2 Configures the number of bits for "Redundancy version" in the DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 6.1.2.1).

numberOfInvalidSymbolsForDL-UL-Switching Indicates the number of symbols after the last semi-static DL symbol that are invalid symbols for PUSCH repetition Type B. If it is absent, no symbol is explicitly defined for DL-to-UL switching (see TS 38.214 [19], clause 6.1).

priorityIndicatorDCI-0-1, priorityIndicatorDCI-0-2 Configures the presence of "priority indicator" in DCI format 0_1/0_2. When the field is absent in the IE, then the UE shall apply 0 bit for "Priority indicator" in DCI format 0_1/0_2. The field priorityIndicatorDCI-0-1 applies to DCI format 0_1 and the field priorityIndicatorDCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17] clause 7.3.1 and TS 38.213 [13] clause 9).

pusch-AggregationFactor Number of repetitions for data (see TS 38.214 [19], clause 6.1.2.1). If the field is absent the UE applies the value 1.

pusch-RepTypeIndicatorDCI-0-1, pusch-RepTypeIndicatorDCI-0-2 Indicates whether UE follows the behavior for "PUSCH repetition type A" or the behavior for "PUSCH repetition type B" for the PUSCH scheduled by DCI format 0_1/0_2 and for Type 2 CG associated with the activating DCI format 0_1/0_2.The value pusch-RepTypeA enables the 'PUSCH repetition type A' and the value pusch-RepTypeB enables the 'PUSCH repetition type B'. The field pusch-RepTypeIndicatorDCI-0-1 applies to DCI format 0_1 and the field pusch-RepTypeIndicatorDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.2.1).

pusch-TimeDomainAllocationList List of time domain allocations for timing of UL assignment to UL data (see TS 38.214 [19], table 6.1.2.1.1-1). The field pusch-TimeDomainAllocationList applies to DCI formats 0_0 or DCI format 0_1 when the field pusch-TimeDomainAllocationListDCI-0-1 is not configured (see TS 38.214 [19], table 6.1.2.1.1-1 and table 6.1.2.1.1-1A). The network does not configure the pusch-TimeDomainAllocationList (without suffix) simultaneously with the pusch-TimeDomainAllocationListDCI-0-2-r16 or pusch-TimeDomainAllocationListDCI-0-1-r16 or pusch-TimeDomainAllocationListForMultiPUSCH-r16.

pusch-TimeDomainAllocationListDCI-0-1 Configuration of the time domain resource allocation (TDRA) table for DCI format 0_1 (see TS 38.214 [19], clause 6.1, table 6.1.2.1.1-1A).

pusch-TimeDomainAllocationListDCI-0-2 Configuration of the time domain resource allocation (TDRA) table for DCI format 0_2 (see TS 38.214 [19], clause 6.1.2, table 6.1.2.1.1-1B).

pusch-TimeDomainAllocationListForMultiPUSCH Configuration of the time domain resource allocation (TDRA) table for multiple PUSCH (see TS 38.214 [19], clause 6.1.2). The network configures at most 16 rows in this TDRA table in PUSCH-TimeDomainResourceAllocationList-r16 configured by this field. This field is not configured simultaneously with pusch-AggregationFactor.

rbg-Size Selection between configuration 1 and configuration 2 for RBG size for PUSCH. The UE does not apply this field if resourceAllocation is set to resourceAllocationType1. Otherwise, the UE applies the value config1 when the field is absent (see TS 38.214 [19], clause 6.1.2.2.1).

resourceAllocation, resourceAllocationDCI-0-2 Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI (see TS 38.214 [19], clause 6.1.2). The field resourceAllocation applies to DCI format 0_1 and the field resourceAllocationDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.2).

resourceAllocationType1GranularityDCI-0-2 Configures the scheduling granularity applicable for both the starting point and length indication for resource allocation type 1 in DCI format 0_2. If this field is absent, the granularity is 1 PRB (see TS 38.214 [19], clause 6.1.2.2.2).

tp-pi2BPSK Enables pi/2-BPSK modulation with transform precoding if the field is present and disables it otherwise.

transformPrecoder The UE specific selection of transformer precoder for PUSCH (see TS 38.214 [19], clause 6.1.3). When the field is absent the UE applies the value of the field msg3-transformPrecoder.

txConfig Whether UE uses codebook based or non-codebook based transmission (see TS 38.214 [19], clause 6.1.1). If the field is absent, the UE transmits PUSCH on one antenna port, see TS 38.214 [19], clause 6.1.1.

uci-OnPUSCH-ListDCI-0-1, uci-OnPUSCH-ListDCI-0-2 Configuration for up to 2 HARQ-ACK codebooks specific to DCI format 0_1/0_2. The field uci-OnPUSCH-ListDCI-0-1 applies to DCI format 0_1 and the field uci-OnPUSCH-ListDCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9.3).

ul-AccessConfigListDCI-0-1, ul-AccessConfigListDCI-0-2 List of the combinations of cyclic prefix extension, channel access priority class (CAPC), and UL channel access type (see TS 38.212 [17], Table 7.3.1.1.2-35) applicable for DCI format 0_1 and DCI format 0_2, respectively.

ul-FullPowerTransmission Configures the UE with UL full power transmission mode as specified in TS 38.213.

UCI-OnPUSCH field descriptions

betaOffsets Selection between and configuration of dynamic and semi-static beta-offset for DCI formats other than DCI format 0_2. If the field is not configured, the UE applies the value 'semiStatic' (see TS 38.213 [13], clause 9.3).

scaling Indicates a scaling factor to limit the number of resource elements assigned to UCI on PUSCH for DCI formats other than DCI format 0_2. Value f0p5 corresponds to 0.5, value f0p65 corresponds to 0.65, and so on. The value configured herein is applicable for PUSCH with configured grant (see TS 38.212 [17], clause 6.3).

UCI-OnPUSCH-DCI-0-2 field descriptions

betaOffsetsDCI-0-2 Configuration of beta-offset for DCI format 0_2. If semiStaticDCI-0-2 is chosen, the UE shall apply the value of 0 bit for the field of beta offset indicator in DCI format 0_2. If dynamicDCI-0-2 is chosen, the UE shall apply the value of 1 bit or 2 bits for the field of beta offset indicator in DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9.3).

dynamicDCI-0-2 Indicates the UE applies the value 'dynamic' for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.3).

semiStaticDCI-0-2 Indicates the UE applies the value 'semiStatic' for DCI format 0_2. (see TS 38.212 [17], clause 7.3.1 and see TS 38.213 [13], clause 9.3).

scalingDCI-0-2 Indicates a scaling factor to limit the number of resource elements assigned to UCI on PUSCH for DCI format 0_2. Value f0p5 corresponds to 0.5, value f0p65 corresponds to 0.65, and so on (see TS 38.212 [17], clause 6.3).

| Conditional Presence | Explanation |
| --- | --- |
| codebookBased | The field is mandatory present if txConfig is set to codebook and absent otherwise. |
| RepTypeB | The field is optionally present, Need S, if pusch-RepTypeIndicatorDCI-0-1 is set to pusch-RepTypeB. It is absent otherwise. |
| RepTypeB2 | The field is optionally present, Need S, if pusch-RepTypeIndicatorDCI-0-1 or pusch-RepTypeIndicatorDCI-0-2 is set to pusch-RepTypeB. It is absent otherwise. |

next change

–	ReferenceTimeInfo

The IE ReferenceTimeInfo contains timing information for 5G internal system clock used for, e.g., time stamping, see TS 23.501 [32], clause 5.27.1.2.

ReferenceTimeInfo information element

-- ASN1START

-- TAG-REFERENCETIMEINFO-START

ReferenceTimeInfo-r16 ::= SEQUENCE {

time-r16                            ReferenceTime-r16,

uncertainty-r16                     INTEGER (0..32767)          OPTIONAL,   -- Need S

timeInfoType-r16                    ENUMERATED {localClock}     OPTIONAL,   -- Need S

referenceSFN-r16                    INTEGER (0..1023)           OPTIONAL    -- Cond RefTime

}

ReferenceTime-r16 ::=           SEQUENCE {

refDays-r16                         INTEGER (0..72999),

refSeconds-r16                      INTEGER (0..86399),

refMilliSeconds-r16                 INTEGER (0..999),

refTenNanoSeconds-r16               INTEGER (0..99999)

}

-- TAG-REFERENCETIMEINFO-STOP

-- ASN1STOP

ReferenceTimeInfo field descriptions

referenceSFN This field indicates the reference SFN corresponding to the reference time information. If referenceTimeInfo field is received in DLInformationTransfer message, this field indicates the SFN of PCell.

Time This field indicates time reference with 10ns granularity. If included in DLInformationTransfer and if UE-side TA PDC is de-activated, the indicated time may not be referenced at the network, i.e., gNB may pre-compensate. If included in DLInformationTransfer and if UE is requested to transmit UE Rx-Tx time difference measurement, the indicated time may not be referenced at the network, i.e., gNB may pre-compensate. Otherwise, the indicated time is referenced at the network, i.e., without compensating for RF propagation delay. The indicated time in 10ns unit from the origin is refDays*86400*1000*100000 + refSeconds*1000*100000 + refMilliSeconds*100000 + refTenNanoSeconds. The refDays field specifies the sequential number of days (with day count starting at 0) from the origin of the time field. If the referenceTimeInfo field is received in DLInformationTransfer message, the time field indicates the time at the ending boundary of the system frame indicated by referenceSFN. The UE considers this frame (indicated by referenceSFN) to be the frame which is nearest to the frame where the message is received (which can be either in the past or in the future). If the referenceTimeInfo field is received in SIB9, the time field indicates the time at the SFN boundary at or immediately after the ending boundary of the SI-window in which SIB9 is transmitted. If referenceTimeInfo field is received in SIB9, this field is excluded when determining changes in system information, i.e. changes of time should neither result in system information change notifications nor in a modification of valueTag in SIB1.

timeInfoType If timeInfoType is not included, the time indicates the GPS time and the origin of the time field is 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time). If timeInfoType is set to localClock, the origin of the time is unspecified.

Uncertainty This field indicates the uncertainty of the reference time information provided by the time field. The uncertainty is 25ns multiplied by this field. If this field is absent, the uncertainty is unspecified.

| Conditional Presence | Explanation |
| --- | --- |
| RefTime | The field is mandatory present if referenceTimeInfo is included in DLInformationTransfer message; otherwise the field is absent. |

next change

–	ReportConfigNR

The IE ReportConfigNR specifies criteria for triggering of an NR measurement reporting event or of a CHO or CPC event. For events labelled AN with N equal to 1, 2 and so on, measurement reporting events and CHO or CPC events are based on cell measurement results, which can either be derived based on SS/PBCH block or CSI-RS.

Event A1:	Serving becomes better than absolute threshold;

Event A2:	Serving becomes worse than absolute threshold;

Event A3:	Neighbour becomes amount of offset better than PCell/PSCell;

Event A4:	Neighbour becomes better than absolute threshold;

Event A5:	PCell/PSCell becomes worse than absolute threshold1 AND Neighbour/SCell becomes better than another absolute threshold2;

Event A6:	Neighbour becomes amount of offset better than SCell;

CondEvent A3: Conditional reconfiguration candidate becomes amount of offset better than PCell/PSCell;

CondEvent A5: PCell/PSCell becomes worse than absolute threshold1 AND Conditional reconfiguration candidate becomes better than another absolute threshold2;

For event I1, measurement reporting event is based on CLI measurement results, which can either be derived based on SRS-RSRP or CLI-RSSI.

Event I1:	Interference becomes higher than absolute threshold.

ReportConfigNR information element

-- ASN1START

-- TAG-REPORTCONFIGNR-START

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16,

rxTxPeriodical-r17                          RxTxPeriodical-r17

}

}

ReportCGI ::=                     SEQUENCE {

cellForWhichToReportCGI          PhysCellId,

...,

[[

useAutonomousGaps-r16            ENUMERATED {setup}                OPTIONAL   -- Need R

]]

}

ReportSFTD-NR ::=                 SEQUENCE {

reportSFTD-Meas                  BOOLEAN,

reportRSRP                       BOOLEAN,

...,

[[

reportSFTD-NeighMeas             ENUMERATED {true}                                OPTIONAL,   -- Need R

drx-SFTD-NeighMeas               ENUMERATED {true}                                OPTIONAL,   -- Need R

cellsForWhichToReportSFTD        SEQUENCE (SIZE (1..maxCellSFTD)) OF PhysCellId   OPTIONAL    -- Need R

]]

}

CondTriggerConfig-r16 ::=        SEQUENCE {

condEventId                      CHOICE {

condEventA3                      SEQUENCE {

a3-Offset                        MeasTriggerQuantityOffset,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

condEventA5                      SEQUENCE {

a5-Threshold1                    MeasTriggerQuantity,

a5-Threshold2                    MeasTriggerQuantity,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

...

},

rsType-r16                       NR-RS-Type,

...

}

EventTriggerConfig::=                       SEQUENCE {

eventId                                     CHOICE {

eventA1                                     SEQUENCE {

a1-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

eventA2                                     SEQUENCE {

a2-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

eventA3                                     SEQUENCE {

a3-Offset                                   MeasTriggerQuantityOffset,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA4                                     SEQUENCE {

a4-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA5                                     SEQUENCE {

a5-Threshold1                               MeasTriggerQuantity,

a5-Threshold2                               MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA6                                     SEQUENCE {

a6-Offset                                   MeasTriggerQuantityOffset,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

...

},

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                     MeasReportQuantity                                            OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

reportAddNeighMeas                          ENUMERATED {setup}                                             OPTIONAL,   -- Need R

...,

[[

measRSSI-ReportConfig-r16                   MeasRSSI-ReportConfig-r16                                      OPTIONAL,   -- Need R

useT312-r16                                 BOOLEAN                                                        OPTIONAL,   -- Need M

includeCommonLocationInfo-r16               ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                          SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                        SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16                      SetupRelease {Sensor-NameList-r16}                             OPTIONAL    -- Need M

]]

}

PeriodicalReportConfig ::=                  SEQUENCE {

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                    MeasReportQuantity                                             OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

useWhiteCellList                            BOOLEAN,

...,

[[

measRSSI-ReportConfig-r16                   MeasRSSI-ReportConfig-r16                                      OPTIONAL,   -- Need R

includeCommonLocationInfo-r16               ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                          SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                        SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16                      SetupRelease {Sensor-NameList-r16}                             OPTIONAL,   -- Need M

ul-DelayValueConfig-r16                     SetupRelease { UL-DelayValueConfig-r16 }                       OPTIONAL,   -- Need M

reportAddNeighMeas-r16                      ENUMERATED {setup}                                             OPTIONAL    -- Need R

]]

}

NR-RS-Type ::=                              ENUMERATED {ssb, csi-rs}

MeasTriggerQuantity ::=                     CHOICE {

rsrp                                        RSRP-Range,

rsrq                                        RSRQ-Range,

sinr                                        SINR-Range

}

MeasTriggerQuantityOffset ::=               CHOICE {

rsrp                                        INTEGER (-30..30),

rsrq                                        INTEGER (-30..30),

sinr                                        INTEGER (-30..30)

}

MeasReportQuantity ::=                      SEQUENCE {

rsrp                                        BOOLEAN,

rsrq                                        BOOLEAN,

sinr                                        BOOLEAN

}

MeasRSSI-ReportConfig-r16 ::=               SEQUENCE {

channelOccupancyThreshold-r16               RSSI-Range-r16         OPTIONAL   -- Need R

}

CLI-EventTriggerConfig-r16 ::=              SEQUENCE {

eventId-r16                                 CHOICE {

eventI1-r16                                 SEQUENCE {

i1-Threshold-r16                            MeasTriggerQuantityCLI-r16,

reportOnLeave-r16                           BOOLEAN,

hysteresis-r16                              Hysteresis,

timeToTrigger-r16                           TimeToTrigger

},

...

},

reportInterval-r16                          ReportInterval,

reportAmount-r16                            ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

maxReportCLI-r16                            INTEGER (1..maxCLI-Report-r16),

...

}

CLI-PeriodicalReportConfig-r16 ::=          SEQUENCE {

reportInterval-r16                          ReportInterval,

reportAmount-r16                            ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCLI-r16                       MeasReportQuantityCLI-r16,

maxReportCLI-r16                            INTEGER (1..maxCLI-Report-r16),

...

}

RxTxPeriodical-r17  ::=          SEQUENCE {

rxTxReportInterval-r17                      RxTxReportInterval-r17,

reportAmount-r17                            ENUMERATED {r1, infinity, spare6, spare5, spare4, spare3, spare2, spare1},

...

}

RxTxReportInterval-r17 ::= ENUMERATED {ms80,ms120,ms160,ms240,ms320,ms480,ms640,ms1024,ms1280,ms2048,ms2560,ms5120,spare4,spare3,spare2,spare1}

MeasTriggerQuantityCLI-r16 ::=              CHOICE {

srs-RSRP-r16                                SRS-RSRP-Range-r16,

cli-RSSI-r16                                CLI-RSSI-Range-r16

}

MeasReportQuantityCLI-r16 ::=               ENUMERATED {srs-rsrp, cli-rssi}

-- TAG-REPORTCONFIGNR-STOP

-- ASN1STOP

CondTriggerConfig field descriptions

a3-Offset Offset value(s) to be used in NR conditional reconfiguration triggering condition for cond event a3. The actual value is field value * 0.5 dB.

a5-Threshold1/ a5-Threshold2 Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR conditional reconfiguration triggering condition for cond event a5. In the same condeventA5, the network configures the same quantity for the MeasTriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2.

condEventId Choice of NR conditional reconfiguration event triggered criteria.

timeToTrigger Time during which specific criteria for the event needs to be met in order to execute the conditional reconfiguration evaluation.

ReportConfigNR field descriptions

reportType Type of the configured measurement report. In MR-DC, network does not configure report of type reportCGI using SRB3. The condTriggerConfig is used for CHO or CPC configuration.

ReportCGI field descriptions

useAutonomousGaps Indicates whether or not the UE is allowed to use autonomous gaps in acquiring system information from the NR neighbour cell. When the field is included, the UE applies the corresponding value for T321.

EventTriggerConfig field descriptions

a3-Offset/a6-Offset Offset value(s) to be used in NR measurement report triggering condition for event a3/a6. The actual value is field value * 0.5 dB.

aN-ThresholdM Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR measurement report triggering condition for event number aN. If multiple thresholds are defined for event number aN, the thresholds are differentiated by M. The network configures aN-Threshold1 only for events A1, A2, A4, A5 and a5-Threshold2 only for event A5. In the same eventA5, the network configures the same quantity for the MeasTriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2.

channelOccupancyThreshold RSSI threshold which is used for channel occupancy evaluation.

eventId Choice of NR event triggered reporting criteria.

maxNrofRS-IndexesToReport Max number of RS indexes to include in the measurement report for A1-A6 events.

maxReportCells Max number of non-serving cells to include in the measurement report.

reportAddNeighMeas Indicates that the UE shall include the best neighbour cells per serving frequency.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types.

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList, as specified in 5.5.4.1.

reportQuantityCell The cell measurement quantities to be included in the measurement report.

reportQuantityRS-Indexes Indicates which measurement information per RS index the UE shall include in the measurement report.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

useT312 If value TRUE is configured, the UE shall use the timer T312 with the value t312 as specified in the corresponding measObjectNR. If value FALSE is configured, the timer T312 is considered as disabled. Network configures value TRUE only if reportType is set to eventTriggered.

useWhiteCellList Indicates whether only the cells included in the white-list of the associated measObject are applicable as specified in 5.5.4.1.

CLI-EventTriggerConfig field descriptions

i1-Threshold Threshold value associated to the selected trigger quantity (e.g. SRS-RSRP, CLI-RSSI) to be used in CLI measurement report triggering condition for event i1.

eventId Choice of CLI event triggered reporting criteria.

maxReportCLI Max number of CLI measurement resource to include in the measurement report.

reportAmount Number of measurement reports.

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a CLI measurement resource in srsTriggeredList or rssiTriggeredList, as specified in 5.5.4.1.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

CLI-PeriodicalReportConfig field descriptions

maxReportCLI Max number of CLI measurement resource to include in the measurement report.

reportAmount Number of measurement reports.

reportQuantityCLI The CLI measurement quantities to be included in the measurement report.

PeriodicalReportConfig field descriptions

maxNrofRS-IndexesToReport Max number of RS indexes to include in the measurement report.

maxReportCells Max number of non-serving cells to include in the measurement report.

reportAddNeighMeas Indicates that the UE shall include the best neighbour cells per serving frequency.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportQuantityCell The cell measurement quantities to be included in the measurement report.

reportQuantityRS-Indexes Indicates which measurement information per RS index the UE shall include in the measurement report.

ul-DelayValueConfig If the field is present, the UE shall perform the actual UL PDCP Packet Average Delay measurement per DRB as specified in TS 38.314 [53] and the UE shall ignore the fields reportQuantityCell and maxReportCells. The applicable values for the corresponding reportInterval are (one of the) {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,min6, min12, min30}. The reportInterval indicates the periodicity for performing and reporting of UL PDCP Packet Average Delay per DRB measurement as specified in TS 38.314 [53].

useWhiteCellList Indicates whether only the cells included in the white-list of the associated measObject are applicable as specified in 5.5.4.1.

ReportSFTD-NR field descriptions

cellForWhichToReportSFTD Indicates the target NR neighbour cells for SFTD measurement between PCell and NR neighbour cells.

drx-SFTD-NeighMeas Indicates that the UE shall use available idle periods (i.e. DRX off periods) for the SFTD measurement in NR standalone. The network only includes drx-SFTD-NeighMeas field when reprtSFTD-NeighMeas is set to true.

reportSFTD-Meas Indicates whether UE is required to perform SFTD measurement between PCell and NR PSCell in NR-DC.

reportSFTD-NeighMeas Indicates whether UE is required to perform SFTD measurement between PCell and NR neighbour cells in NR standalone. The network does not include this field if reportSFTD-Meas is set to true.

reportRSRP Indicates whether UE is required to include RSRP result of NR PSCell or NR neighbour cells in SFTD measurement result, derived based on SSB. If it is set to true, the network should ensure that ssb-ConfigMobility is included in the measurement object for NR PSCell or NR neighbour cells.

other field descriptions

MeasTriggerQuantity SINR is applicable only for CONNECTED mode events.

next change

–	RxTxTimeDiff

The IE RxTxTimeDiff contains the Rx-Tx time difference measurement at either the UE or the gNB.

RxTxTimeDiff information element

-- ASN1START

-- TAG-RXTXTIMEDIFF-START

RxTxTimeDiff-r17  ::= SEQUENCE {

result-k5             INTEGER (0..61565),

...

}

-- TAG-RXTXTIMEDIFF-STOP

-- ASN1STOP

RxTxTimeDiff field descriptions

result-k5 This field indicates the Rx-Tx time difference measurement, see TS 38.133 [14].

next change

–	SemiStaticChannelAccessConfigUE

The IE SemiStaticChannelAccessConfigUE is used to configure channel access parameters for UE initiated semi-static channel access.

SemiStaticChannelAccessConfigUE information element

-- ASN1START

-- TAG-SEMISTATICCHANNELACCESSCONFIGUE-START

SemiStaticChannelAccessConfigUE-r17 ::=    SEQUENCE {

periodUE-r17                                 ENUMERATED {ms1, ms2, ms2dot5, ms4, ms5, ms10, spare1, spare2},

offsetUE-r17                                 INTEGER (0..1119)

}

-- TAG-SEMISTATICCHANNELACCESSCONFIGUE-STOP

-- ASN1STOP

SemiStaticChannelAccessConfigUE field descriptions

periodUE Indicates the period of a channel occupancy that the UE can initiate as described in TS 37.213 [48], clause 4.3. Value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, value ms2dot5 corresponds to 2.5 ms, and so on.

offsetUE Indicates the number of symbols from the beginning of the even indexed radio frame to the start of the first period within that radio frame that the UE can initiate a channel occupancy (see TS 37.213 [48], clause 4.3), based on the smallest SCS among the configured SCSs in the serving cell. The offset duration indicated by this field is less than the period duration indicated by periodUE. The maximum value is 279, 559 and 1119 for 15, 30 and 60 kHz subcarrier spacing, respectively.

next change

–	ServingCellConfig

The IE ServingCellConfig is used to configure (add or modify) the UE with a serving cell, which may be the SpCell or an SCell of an MCG or SCG. The parameters herein are mostly UE specific but partly also cell specific (e.g. in additionally configured bandwidth parts). Reconfiguration between a PUCCH and PUCCHless SCell is only supported using an SCell release and add.

ServingCellConfig information element

-- ASN1START

-- TAG-SERVINGCELLCONFIG-START

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                                OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated                                                    OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                               OPTIONAL,   -- Need N

downlinkBWP-ToAddModList            SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink                         OPTIONAL,   -- Need N

firstActiveDownlinkBWP-Id           BWP-Id                                                                   OPTIONAL,   -- Cond SyncAndCellAdd

bwp-InactivityTimer                 ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30,

ms40,ms50, ms60, ms80,ms100, ms200,ms300, ms500,

ms750, ms1280, ms1920, ms2560, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1 }    OPTIONAL,   --Need R

defaultDownlinkBWP-Id               BWP-Id                                                                  OPTIONAL,   -- Need S

uplinkConfig                        UplinkConfig                                                            OPTIONAL,   -- Need M

supplementaryUplink                 UplinkConfig                                                            OPTIONAL,   -- Need M

pdcch-ServingCellConfig             SetupRelease { PDCCH-ServingCellConfig }                                OPTIONAL,   -- Need M

pdsch-ServingCellConfig             SetupRelease { PDSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

csi-MeasConfig                      SetupRelease { CSI-MeasConfig }                                         OPTIONAL,   -- Need M

sCellDeactivationTimer              ENUMERATED {ms20, ms40, ms80, ms160, ms200, ms240,

ms320, ms400, ms480, ms520, ms640, ms720,

ms840, ms1280, spare2,spare1}       OPTIONAL,   -- Cond ServingCellWithoutPUCCH

crossCarrierSchedulingConfig        CrossCarrierSchedulingConfig                                            OPTIONAL,   -- Need M

tag-Id                              TAG-Id,

dummy1                              ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

pathlossReferenceLinking            ENUMERATED {spCell, sCell}                                              OPTIONAL,   -- Cond SCellOnly

servingCellMO                       MeasObjectId                                                            OPTIONAL,   -- Cond MeasObject

...,

[[

lte-CRS-ToMatchAround               SetupRelease { RateMatchPatternLTE-CRS }                                OPTIONAL,   -- Need M

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern       OPTIONAL,   -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId     OPTIONAL,   -- Need N

downlinkChannelBW-PerSCS-List       SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier                     OPTIONAL    -- Need S

]],

[[

supplementaryUplinkRelease-r16      ENUMERATED {true}                                                       OPTIONAL,   -- Need N

tdd-UL-DL-ConfigurationDedicated-IAB-MT-r16    TDD-UL-DL-ConfigDedicated-IAB-MT-r16                         OPTIONAL,   -- Cond TDD_IAB

dormantBWP-Config-r16               SetupRelease { DormantBWP-Config-r16 }                                  OPTIONAL,   -- Need M

ca-SlotOffset-r16                   CHOICE {

refSCS15kHz                         INTEGER (-2..2),

refSCS30KHz                         INTEGER (-5..5),

refSCS60KHz                         INTEGER (-10..10),

refSCS120KHz                        INTEGER (-20..20)

}                                                                                                           OPTIONAL,   -- Cond AsyncCA

dummy2                              SetupRelease { DummyJ }                                                 OPTIONAL,   -- Need M

intraCellGuardBandsDL-List-r16      SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16           OPTIONAL,   -- Need S

intraCellGuardBandsUL-List-r16      SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16           OPTIONAL,   -- Need S

csi-RS-ValidationWithDCI-r16       ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

lte-CRS-PatternList1-r16            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

lte-CRS-PatternList2-r16            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

crs-RateMatch-PerCORESETPoolIndex-r16  ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

enableTwoDefaultTCI-States-r16      ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultTCI-StatePerCoresetPoolIndex-r16 ENUMERATED {enabled}                                          OPTIONAL,   -- Need R

enableBeamSwitchTiming-r16          ENUMERATED {true}                                                       OPTIONAL,   -- Need R

cbg-TxDiffTBsProcessingType1-r16    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

cbg-TxDiffTBsProcessingType2-r16    ENUMERATED {enabled}                                                    OPTIONAL    -- Need R

]],

[[

directionalCollisionHandling-r16    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

channelAccessConfig-r16             SetupRelease { ChannelAccessConfig-r16 }                                OPTIONAL,    -- Need M

]]

[[

nr-dl-PRS-PDC-Info-r17                 SetupRelease {NR-DL-PRS-PDC-Info-r17}                                      OPTIONAL,    -- Need M

semiStaticChannelAccessConfigUE-r17     SetupRelease {SemiStaticChannelAccessConfigUE-r17}                  OPTIONAL    -- Need M

]]

}

UplinkConfig ::=                    SEQUENCE {

initialUplinkBWP                    BWP-UplinkDedicated                                                     OPTIONAL,   -- Need M

uplinkBWP-ToReleaseList             SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                              OPTIONAL,   -- Need N

uplinkBWP-ToAddModList              SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink                          OPTIONAL,   -- Need N

firstActiveUplinkBWP-Id             BWP-Id                                                                  OPTIONAL,   -- Cond SyncAndCellAdd

pusch-ServingCellConfig             SetupRelease { PUSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

carrierSwitching                    SetupRelease { SRS-CarrierSwitching }                                   OPTIONAL,   -- Need M

...,

[[

powerBoostPi2BPSK                   BOOLEAN                                                                 OPTIONAL,   -- Need M

uplinkChannelBW-PerSCS-List         SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier                     OPTIONAL    -- Need S

]],

[[

enablePL-RS-UpdateForPUSCH-SRS-r16  ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForPUSCH0-0-r16 ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForPUCCH-r16    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForSRS-r16      ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

uplinkTxSwitching-r16               SetupRelease { UplinkTxSwitching-r16 }                                  OPTIONAL,   -- Need M

mpr-PowerBoost-FR2-r16              ENUMERATED {true}                                                       OPTIONAL    -- Need R

]]

}

DummyJ ::=                          SEQUENCE {

maxEnergyDetectionThreshold-r16         INTEGER(-85..-52),

energyDetectionThresholdOffset-r16      INTEGER (-20..-13),

ul-toDL-COT-SharingED-Threshold-r16     INTEGER (-85..-52)                                                  OPTIONAL,   -- Need R

absenceOfAnyOtherTechnology-r16         ENUMERATED {true}                                                   OPTIONAL    -- Need R

}

ChannelAccessConfig-r16 ::=         SEQUENCE {

energyDetectionConfig-r16           CHOICE {

maxEnergyDetectionThreshold-r16         INTEGER (-85..-52),

energyDetectionThresholdOffset-r16      INTEGER (-13..20)

}                                                                                                           OPTIONAL,   -- Need R

ul-toDL-COT-SharingED-Threshold-r16         INTEGER (-85..-52)                                              OPTIONAL,   -- Need R

absenceOfAnyOtherTechnology-r16             ENUMERATED {true}                                               OPTIONAL    -- Need R

}

IntraCellGuardBandsPerSCS-r16 ::=      SEQUENCE {

guardBandSCS-r16                       SubcarrierSpacing,

intraCellGuardBands-r16                SEQUENCE (SIZE (1..4)) OF GuardBand-r16

}

GuardBand-r16 ::=                      SEQUENCE {

startCRB-r16                          INTEGER (0..274),

nrofCRBs-r16                          INTEGER (0..15)

}

DormancyGroupID-r16 ::=         INTEGER (0..4)

DormantBWP-Config-r16::=               SEQUENCE {

dormantBWP-Id-r16                      BWP-Id                                                           OPTIONAL,   -- Need M

withinActiveTimeConfig-r16             SetupRelease { WithinActiveTimeConfig-r16 }                      OPTIONAL,   -- Need M

outsideActiveTimeConfig-r16            SetupRelease { OutsideActiveTimeConfig-r16 }                     OPTIONAL    -- Need M

}

WithinActiveTimeConfig-r16 ::=         SEQUENCE {

firstWithinActiveTimeBWP-Id-r16         BWP-Id                                                           OPTIONAL,   -- Need M

dormancyGroupWithinActiveTime-r16       DormancyGroupID-r16                                              OPTIONAL    -- Need R

}

OutsideActiveTimeConfig-r16 ::=        SEQUENCE {

firstOutsideActiveTimeBWP-Id-r16        BWP-Id                                                           OPTIONAL,   -- Need M

dormancyGroupOutsideActiveTime-r16      DormancyGroupID-r16                                              OPTIONAL    -- Need R

}

UplinkTxSwitching-r16 ::=              SEQUENCE {

uplinkTxSwitchingPeriodLocation-r16    BOOLEAN,

uplinkTxSwitchingCarrier-r16           ENUMERATED {carrier1, carrier2}

}

-- TAG-SERVINGCELLCONFIG-STOP

-- ASN1STOP

ChannelAccessConfig field descriptions

absenceOfAnyOtherTechnology Presence of this field indicates absence on a long term basis (e.g. by level of regulation) of any other technology sharing the carrier; absence of this field indicates the potential presence of any other technology sharing the carrier, as specified in TS 37.213 [48] clauses 4.2.1 and 4.2.3.

energyDetectionConfig Indicates whether to use the maxEnergyDetectionThreshold or the energyDetectionThresholdOffset (see TS 37.213 [48], clause 4.2.3).

energyDetectionThresholdOffset Indicates the offset to the default maximum energy detection threshold value. Unit in dB. Value -13 corresponds to -13dB, value -12 corresponds to -12dB, and so on (i.e. in steps of 1dB) as specified in TS 37.213 [48], clause 4.2.3.

maxEnergyDetectionThreshold Indicates the absolute maximum energy detection threshold value. Unit in dBm. Value -85 corresponds to -85 dBm, value -84 corresponds to -84 dBm, and so on (i.e. in steps of 1dBm) as specified in TS 37.213 [48], clause 4.2.3.

ul-toDL-COT-SharingED-Threshold Maximum energy detection threshold that the UE should use to share channel occupancy with gNB for DL transmission as specified in TS 37.213 [48], clause 4.1.3 for downlink channel access and clause 4.2.3 for uplink channel access. This field is not applicable in semi-static channel access mode.

ServingCellConfig field descriptions

bwp-InactivityTimer The duration in ms after which the UE falls back to the default Bandwidth Part (see TS 38.321 [3], clause 5.15). When the network releases the timer configuration, the UE stops the timer without switching to the default BWP.

ca-SlotOffset Slot offset between the primary cell (PCell/PSCell) and the SCell in unaligned frame boundary with slot alignment and partial SFN alignment inter-band CA. Based on this field, the UE determines the time offset of the SCell as specified in clause 4.5 of TS 38.211 [16]. The granularity of this field is determined by the reference SCS for the slot offset (i.e. the maximum of PCell/PSCell lowest SCS among all the configured SCSs in DL/UL SCS-SpecificCarrierList in ServingCellConfigCommon or ServingCellConfigCommonSIB and this serving cell's lowest SCS among all the configured SCSs in DL/UL SCS-SpecificCarrierList in ServingCellConfigCommon or ServingCellConfigCommonSIB). The Network configures at most single non-zero offset duration in ms (independent on SCS) among CCs in the unaligned CA configuration. If the field is absent, the UE applies the value of 0. The slot offset value can only be changed with SCell release and add.

cbg-TxDiffTBsProcessingType1, cbg-TxDiffTBsProcessingType2 Indicates whether processing types 1 and 2 based CBG based operation is enabled according to Rel-16 UE capabilities.

channelAccessConfig List of parameters used for access procedures of operation with shared spectrum channel access (see TS 37.213 [48).

crossCarrierSchedulingConfig Indicates whether this serving cell is cross-carrier scheduled by another serving cell or whether it cross-carrier schedules another serving cell.

crs-RateMatch-PerCORESETPoolIndex Indicates how UE performs rate matching when both lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 are configured as specified in TS 38.214 [19], clause 5.1.4.2.

csi-RS-ValidationWithDCI Indicates how the UE performs periodic and semi-persistent CSI-RS reception in a slot. The presence of this field indicates that the UE uses DCI detection to validate whether to receive CSI-RS (see TS 38.213 [13], clause 11.1).

defaultDownlinkBWP-Id The initial bandwidth part is referred to by BWP-Id = 0. ID of the downlink bandwidth part to be used upon expiry of the BWP inactivity timer. This field is UE specific. When the field is absent the UE uses the initial BWP as default BWP. (see TS 38.213 [13], clause 12 and TS 38.321 [3], clause 5.15).

directionalCollisionHandling Indicates that this serving cell is using directional collision handling between a reference and other cell(s) for half-duplex operation in TDD CA with same SCS as specified in TS 38.213 [13], clause 11.1. The half-duplex operation only applies within the same frequency range and cell group. The network only configures this field for TDD serving cells that are using the same SCS.

dormantBWP-Config The dormant BWP configuration for an SCell. This field can be configured only for a (non-PUCCH) SCell.

downlinkBWP-ToAddModList List of additional downlink bandwidth parts to be added or modified. (see TS 38.213 [13], clause 12).

downlinkBWP-ToReleaseList List of additional downlink bandwidth parts to be released. (see TS 38.213 [13], clause 12).

downlinkChannelBW-PerSCS-List A set of UE specific channel bandwidth and location configurations for different subcarrier spacings (numerologies). Defined in relation to Point A. The UE uses the configuration provided in this field only for the purpose of channel bandwidth and location determination. If absent, UE uses the configuration indicated in scs-SpecificCarrierList in DownlinkConfigCommon / DownlinkConfigCommonSIB. Network only configures channel bandwidth that corresponds to the channel bandwidth values defined in TS 38.101-1 [15] and TS 38.101-2 [39].

dummy1, dummy 2 This field is not used in the specification. If received it shall be ignored by the UE.

enableBeamSwitchTiming Indicates the aperiodic CSI-RS triggering with beam switching triggering behaviour as defined in clause 5.2.1.5.1 of TS 38.214 [19].

enableDefaultTCI-StatePerCoresetPoolIndex Presence of this field indicates the UE shall follow the release 16 behavior of default TCI state per CORESETPoolindex when the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet is enabled.

enableTwoDefaultTCI-States Presence of this field indicates the UE shall follow the release 16 behavior of two default TCI states for PDSCH when at least one TCI codepoint is mapped to two TCI states is enabled

firstActiveDownlinkBWP-Id If configured for an SpCell, this field contains the ID of the DL BWP to be activated upon performing the RRC (re-)configuration. If the field is absent, the RRC (re-)configuration does not impose a BWP switch. If configured for an SCell, this field contains the ID of the downlink bandwidth part to be used upon activation of an SCell. The initial bandwidth part is referred to by BWP-Id = 0. Upon reconfiguration with reconfigurationWithSync, the network sets the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id to the same value.

initialDownlinkBWP The dedicated (UE-specific) configuration for the initial downlink bandwidth-part (i.e. DL BWP#0). If any of the optional IEs are configured within this IE, the UE considers the BWP#0 to be an RRC configured BWP (from UE capability viewpoint). Otherwise, the UE does not consider the BWP#0 as an RRC configured BWP (from UE capability viewpoint). Network always configures the UE with a value for this field if no other BWPs are configured. NOTE1

intraCellGuardBandsDL-List, intraCellGuardBandsUL-List List of intra-cell guard bands in a serving cell for operation with shared spectrum channel access. If not configured, the guard bands are defined according to 38.101-1 [15], see TS 38.214 [19], clause 7. For operation in licensed spectrum, this field is absent, and no UE action is required.

lte-CRS-PatternList1 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. The network does not configure this field and lte-CRS-ToMatchAround simultaneously.

lte-CRS-PatternList2 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH scheduled with a DCI detected on a CORESET with CORESETPoolIndex configured with 1. This list is configured only if CORESETPoolIndex configured with 1. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList1, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList1, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and there is at least one ControlResourceSet in one DL BWP of this serving cell with coresetPoolIndex set to 1.

lte-CRS-ToMatchAround Parameters to determine an LTE CRS pattern that the UE shall rate match around.

nr-dl-PRS-PDC-Info Configures the DL PRS for propagation delay compensation. When configured, the UE measures the UE Rx-Tx time difference based on the reference signals configured in this field.

pathlossReferenceLinking Indicates whether UE shall apply as pathloss reference either the downlink of SpCell (PCell for MCG or PSCell for SCG) or of SCell that corresponds with this uplink (see TS 38.213 [13], clause 7).

pdsch-ServingCellConfig PDSCH related parameters that are not BWP-specific.

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the rate match patterns. Rate match patterns defined here on cell level apply only to PDSCH of the same numerology. See TS 38.214 [19], clause 5.1.4.1.

sCellDeactivationTimer SCell deactivation timer in TS 38.321 [3]. If the field is absent, the UE applies the value infinity.

semiStaticChannelAccessConfigUE When this field is configured and when channelAccessMode-r16 (see IE ServingCellConfigCommon and IE ServingCellConfigCommonSIB) is configured to semiStatic, the UE operates in semi-static channel access mode and can initiate a channel occupancy periodically (see TS 37.213 [48], Clause 4.3). The period can be configured independently from period configured in SemiStaticChannelAccessConfig-r16 if the UE indicates the corresponding capability. Otherwise, the periodicity configured by periodUE-r17 is an integer multiple of or an integter factor of the periodicity indicated by period in SemiStaticChannelAccessConfig-r16.

servingCellMO measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell. For this MeasObjectNR, the following relationship applies between this MeasObjectNR and frequencyInfoDL in ServingCellConfigCommon of the serving cell: if ssbFrequency is configured, its value is the same as the absoluteFrequencySSB and if csi-rs-ResourceConfigMobility is configured, the value of its subcarrierSpacing is present in one entry of the scs-SpecificCarrierList, csi-RS-CellListMobility includes an entry corresponding to the serving cell (with cellId equal to physCellId in ServingCellConfigCommon) and the frequency range indicated by the csi-rs-MeasurementBW of the entry in csi-RS-CellListMobility is included in the frequency range indicated by in the entry of the scs-SpecificCarrierList.

supplementaryUplink Network may configure this field only when supplementaryUplinkConfig is configured in ServingCellConfigCommon or supplementaryUplink is configured in ServingCellConfigCommonSIB.

supplementaryUplinkRelease If this field is included, the UE shall release the uplink configuration configured by supplementaryUplink. The network only includes either supplementaryUplinkRelease or supplementaryUplink at a time.

tag-Id Timing Advance Group ID, as specified in TS 38.321 [3], which this cell belongs to.

tdd-UL-DL-ConfigurationDedicated-IAB-MT Resource configuration per IAB-MT D/U/F overrides all symbols (with a limitation that effectively only flexible symbols can be overwritten in Rel-16) per slot over the number of slots as provided by TDD-UL-DL ConfigurationCommon.

uplinkConfig Network may configure this field only when uplinkConfigCommon is configured in ServingCellConfigCommon or ServingCellConfigCommonSIB. Addition or release of this field can only be done upon SCell addition or release (respectively).

UplinkConfig field descriptions

carrierSwitching Includes parameters for configuration of carrier based SRS switching (see TS 38.214 [19], clause 6.2.1.3.

enableDefaultBeamPL-ForPUSCH0-0, enableDefaultBeamPL-ForPUCCH, enableDefaultBeamPL-ForSRS When the parameter is present, UE derives the spatial relation and the corresponding pathloss reference Rs as specified in 38.213, clauses 7.1.1, 7.2.1, 7.3.1 and 9.2.2. The network only configures these parameters for FR2.

enablePL-RS-UpdateForPUSCH-SRS When this parameter is present, the Rel-16 feature of MAC CE based pathloss RS updates for PUSCH/SRS is enabled. Network only configures this parameter when the UE is configured with sri-PUSCH-PowerControl. If this field is not configured, network configures at most 4 pathloss RS resources for PUSCH/PUCCH/SRS transmissions per BWP, not including pathloss RS resources for SRS transmissions for positioning. (See TS 38.213 [13], clause 7).

firstActiveUplinkBWP-Id If configured for an SpCell, this field contains the ID of the UL BWP to be activated upon performing the RRC (re-)configuration. If the field is absent, the RRC (re-)configuration does not impose a BWP switch. If configured for an SCell, this field contains the ID of the uplink bandwidth part to be used upon activation of an SCell. The initial bandwidth part is referred to by BandiwdthPartId = 0.

initialUplinkBWP The dedicated (UE-specific) configuration for the initial uplink bandwidth-part (i.e. UL BWP#0). If any of the optional IEs are configured within this IE as part of the IE uplinkConfig, the UE considers the BWP#0 to be an RRC configured BWP (from UE capability viewpoint). Otherwise, the UE does not consider the BWP#0 as an RRC configured BWP (from UE capability viewpoint). Network always configures the UE with a value for this field if no other BWPs are configured. NOTE1

mpr-PowerBoost-FR2 Indicates whether UE is allowed to boost uplink transmission power by suspending in-band emission (IBE) requirements as specified in TS 38.101-2 [39]. Network only configures this field for FR2 serving cells.

powerBoostPi2BPSK If this field is set to true, the UE determines the maximum output power for PUCCH/PUSCH transmissions that use pi/2 BPSK modulation according to TS 38.101-1 [15], clause 6.2.4.

pusch-ServingCellConfig PUSCH related parameters that are not BWP-specific.

uplinkBWP-ToAddModList The additional bandwidth parts for uplink to be added or modified. In case of TDD uplink- and downlink BWP with the same bandwidthPartId are considered as a BWP pair and must have the same center frequency.

uplinkBWP-ToReleaseList The additional bandwidth parts for uplink to be released.

uplinkChannelBW-PerSCS-List A set of UE specific channel bandwidth and location configurations for different subcarrier spacings (numerologies). Defined in relation to Point A. The UE uses the configuration provided in this field only for the purpose of channel bandwidth and location determination. If absent, UE uses the configuration indicated in scs-SpecificCarrierList in UplinkConfigCommon / UplinkConfigCommonSIB. Network only configures channel bandwidth that corresponds to the channel bandwidth values defined in TS 38.101-1 [15] and TS 38.101-2 [39].

uplinkTxSwitchingPeriodLocation Indicates whether the location of UL Tx switching period is configured in this uplink carrier in case of inter-band UL CA, SUL, or (NG)EN-DC, as specified in TS 38.101-1 [15] and TS 38.101-3 [34]. In case of inter-band UL CA or SUL, network configures this field to TRUE for one of the uplink carriers involved in dynamic UL TX switching and configures this field in the other carrier to FALSE. In case of (NG)EN-DC, network always configures this field to TRUE for NR carrier (i.e. with (NG)EN-DC, the UL switching period always occurs on the NR carrier).

uplinkTxSwitchingCarrier Indicates that the configured carrier is carrier1 or carrier2 for dynamic uplink Tx switching, as defined in TS 38.101-1 [15] and TS 38.101-3 [34]. In case of inter-band UL CA or SUL, network configures one of the two uplink carriers involved in dynamic UL TX switching as carrier1 and the other as carrier2. In case of (NG)EN-DC, network always configures the NR carrier as carrier 2.

DormantBWP-Config field descriptions

dormancyGroupWithinActiveTime This field contains the ID of an SCell group for Dormancy within active time, to which this SCell belongs. The use of the Dormancy within active time SCell groups is specified in TS 38.213 [13].

dormancyGroupOutsideActiveTime This field contains the ID of an SCell group for Dormancy outside active time, to which this SCell belongs. The use of the Dormancy outside active time SCell groups is specified in TS 38.213 [13].

dormantBWP-Id This field contains the ID of the downlink bandwidth part to be used as dormant BWP. If this field is configured, its value is different from defaultDownlinkBWP-Id, and at least one of the withinActiveTimeConfig and outsideActiveTimeConfig should be configured.

firstOutsideActiveTimeBWP-Id This field contains the ID of the downlink bandwidth part to be activated when receiving a DCI indication for SCell dormancy outside active time.

firstWithinActiveTimeBWP-Id This field contains the ID of the downlink bandwidth part to be activated when receiving a DCI indication for SCell dormancy within active time.

outsideActiveTimeConfig This field contains the configuration to be used for SCell dormancy outside active time, as specified in TS 38.213 [13]. The field can only be configured when the cell group the SCell belongs to is configured with dcp-Config.

withinActiveTimeConfig This field contains the configuration to be used for SCell dormancy within active time, as specified in TS 38.213 [13].

GuardBand field descriptions

startCRB Indicates the starting RB of the guard band.

nrofCRB Indicates the length of the guard band in RBs. When set to 0, zero-size guard band is used.

NOTE 1:	If the dedicated part of initial UL/DL BWP configuration is absent, the initial BWP can be used but with some limitations. For example, changing to another BWP requires RRCReconfiguration since DCI format 1_0 doesn't support DCI-based switching.

| Conditional Presence | Explanation |
| --- | --- |
| AsyncCA | This field is mandatory present for SCells whose slot offset between the SpCell is not 0. Otherwise it is absent, Need S. |
| MeasObject | This field is mandatory present for the SpCell if the UE has a measConfig, and it is optionally present, Need M, for SCells. |
| SCellOnly | This field is optionally present, Need R, for SCells. It is absent otherwise. |
| ServingCellWithoutPUCCH | This field is optionally present, Need S, for SCells except PUCCH SCells. It is absent otherwise. |
| SyncAndCellAdd | This field is mandatory present for a SpCell upon reconfiguration with reconfigurationWithSync and upon RRCSetup/RRCResume. The field is optionally present for an SpCell, Need N, upon reconfiguration without reconfigurationWithSync. The field is mandatory present for an SCell upon addition, and absent for SCell in other cases, Need M. |
| TDD | This field is optionally present, Need R, for TDD cells. It is absent otherwise. |
| TDD_IAB | For IAB-MT, this field is optionally present, Need R, for TDD cells. It is absent otherwise. |

next change

–	SlotFormatCombinationsPerCell

The IE SlotFormatCombinationsPerCell is used to configure the SlotFormatCombinations applicable for one serving cell (see TS 38.213 [13], clause 11.1.1).

SlotFormatCombinationsPerCell information element

-- ASN1START

-- TAG-SLOTFORMATCOMBINATIONSPERCELL-START

SlotFormatCombinationsPerCell ::=   SEQUENCE {

servingCellId                       ServCellIndex,

subcarrierSpacing                   SubcarrierSpacing,

subcarrierSpacing2                  SubcarrierSpacing                                                         OPTIONAL, -- Need R

slotFormatCombinations              SEQUENCE (SIZE (1..maxNrofSlotFormatCombinationsPerSet)) OF SlotFormatCombination

OPTIONAL, -- Need M

positionInDCI                       INTEGER(0..maxSFI-DCI-PayloadSize-1)                                      OPTIONAL, -- Need M

...,

[[

enableConfiguredUL-r16              ENUMERATED {enabled}                                                      OPTIONAL  -- Need R

]]

}

SlotFormatCombination ::=           SEQUENCE {

slotFormatCombinationId             SlotFormatCombinationId,

slotFormats                         SEQUENCE (SIZE (1..maxNrofSlotFormatsPerCombination)) OF INTEGER (0..255)

}

SlotFormatCombinationId ::=         INTEGER (0..maxNrofSlotFormatCombinationsPerSet-1)

-- TAG-SLOTFORMATCOMBINATIONSPERCELL-STOP

-- ASN1STOP

SlotFormatCombination field descriptions

slotFormatCombinationId This ID is used in the DCI payload to dynamically select this SlotFormatCombination (see TS 38.213 [13], clause 11.1.1).

slotFormats Slot formats that occur in consecutive slots in time domain order as listed here (see TS 38.213 [13], clause 11.1.1 and TS 38.213 [13], clause 14 for IAB-MT).

SlotFormatCombinationsPerCell field descriptions

enableConfiguredUL If configured, the UE is allowed to transmit uplink signals/channels (SRS, PUCCH, CG-PUSCH) in the set of symbols of the slot when the UE does not detect a DCI format 2_0 providing a slot format for the set of symbols (see TS 38.213 [13], 11.1.1). This field is applicable only if cg-RetransmissionTimer-r16 is configured.

positionInDCI The (starting) position (bit) of the slotFormatCombinationId (SFI-Index) for this serving cell (servingCellId) within the DCI payload (see TS 38.213 [13], clause 11.1.1).

servingCellId The ID of the serving cell for which the slotFormatCombinations are applicable.

slotFormatCombinations A list with SlotFormatCombinations. Each SlotFormatCombination comprises of one or more SlotFormats (see TS 38.211 [16], clause 4.3.2). The total number of slotFormats in the slotFormatCombinations list does not exceed 512.

subcarrierSpacing2 Reference subcarrier spacing for a Slot Format Combination on an FDD or SUL cell (see TS 38.213 [13], clause 11.1.1). For FDD, subcarrierSpacing (SFI-scs) is the reference SCS for DL BWP and subcarrierSpacing2 (SFI-scs2) is the reference SCS for UL BWP. For SUL, subcarrierSpacing (SFI-scs) is the reference SCS for non-SUL carrier and subcarrierSpacing2 (SFI-scs2) is the reference SCS for SUL carrier. The network configures a value that is smaller than or equal to any SCS of configured BWPs of the serving cell that the command applies to. And the network configures a value that is smaller than or equal to the SCS of the serving cell which the UE monitors for SFI indications.

subcarrierSpacing Reference subcarrier spacing for this Slot Format Combination. The network configures a value that is smaller than or equal to any SCS of configured BWPs of the serving cell that the command applies to. And the network configures a value that is smaller than or equal to the SCS of the serving cell which the UE monitors for SFI indications (see TS 38.213 [13], clause 11.1.1).

next change

–	SPS-Config

The IE SPS-Config is used to configure downlink semi-persistent transmission. Multiple Downlink SPS configurations may be configured in one BWP of a serving cell.

SPS-Config information element

-- ASN1START

-- TAG-SPS-CONFIG-START

SPS-Config ::=                  SEQUENCE {

periodicity                     ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640,

spare6, spare5, spare4, spare3, spare2, spare1},

nrofHARQ-Processes              INTEGER (1..8),

n1PUCCH-AN                      PUCCH-ResourceId                                                                OPTIONAL,   -- Need M

mcs-Table                       ENUMERATED {qam64LowSE}                                                         OPTIONAL,   -- Need S

...,

[[

sps-ConfigIndex-r16             SPS-ConfigIndex-r16                                                             OPTIONAL,   -- Cond SPS-List

harq-ProcID-Offset-r16          INTEGER (0..15)                                                                 OPTIONAL,   -- Need R

periodicityExt-r16              INTEGER (1..5120)                                                               OPTIONAL,   -- Need R

harq-CodebookID-r16             INTEGER (1..2)                                                                  OPTIONAL,   -- Need R

pdsch-AggregationFactor-r16     ENUMERATED {n1, n2, n4, n8 }                                                    OPTIONAL,    -- Need S

]]

[[

sps-HARQ-Deferral-r17           INTEGER (1..32)              OPTIONAL,    -- Need R

n1PUCCH-AN-PUCCHsSCell-r17          PUCCH-ResourceId             OPTIONAL     -- Need R

]]

}

-- TAG-SPS-CONFIG-STOP

-- ASN1STOP

SPS-Config field descriptions

harq-CodebookID Indicates the HARQ-ACK codebook index for the corresponding HARQ-ACK codebook for SPS PDSCH and ACK for SPS PDSCH release.

harq-ProcID-Offset Indicates the offset used in deriving the HARQ process IDs, see TS 38.321 [3], clause 5.3.1.

mcs-Table Indicates the MCS table the UE shall use for DL SPS (see TS 38.214 [19],clause 5.1.3.1. If present, the UE shall use the MCS table of low-SE 64QAM table indicated in Table 5.1.3.1-3 of TS 38.214 [19]. If this field is absent and field mcs-table in PDSCH-Config is set to 'qam256' and the activating DCI is of format 1_1, the UE applies the 256QAM table indicated in Table 5.1.3.1-2 of TS 38.214 [19]. Otherwise, the UE applies the non-low-SE 64QAM table indicated in Table 5.1.3.1-1 of TS 38.214 [19].

n1PUCCH-AN HARQ resource for PUCCH for DL SPS. The network configures the resource either as format0 or format1. The actual PUCCH-Resource is configured in PUCCH-Config and referred to by its ID. See TS 38.213 [13], clause 9.2.3.

n1PUCCH-AN-PUCCHsSCell HARQ resource for PUCCH on PUCCH switching SCell (sSCell) for DL SPS. The network configures the resource either as format 0 or format 1. The actual PUCCH-Resource is configured in PUCCH-Config of the PUCCH sSCell and referred to by its ID. See TS 38.213 [13], clause 9.2.3.

nrofHARQ-Processes Number of configured HARQ processes for SPS DL (see TS 38.321 [3], clause 5.8.1).

pdsch-AggregationFactor Number of repetitions for SPS PDSCH (see TS 38.214 [19], clause 5.1.2.1). When the field is absent, the UE applies PDSCH aggregation factor of PDSCH-Config.

periodicity Periodicity for DL SPS (see TS 38.214 [19] and TS 38.321 [3], clause 5.8.1).

periodicityExt This field is used to calculate the periodicity for DL SPS (see TS 38.214 [19] and see TS 38.321 [3], clause 5,8.1). If this field is present, the field periodicity is ignored. The following periodicities are supported depending on the configured subcarrier spacing [ms]: 15 kHz: periodicityExt, where periodicityExt has a value between 1 and 640. 30 kHz: 0.5 x periodicityExt, where periodicityExt has a value between 1 and 1280. 60 kHz with normal CP. 0.25 x periodicityExt, where periodicityExt has a value between 1 and 2560. 60 kHz with ECP: 0.25 x periodicityExt, where periodicityExt has a value between 1 and 2560. 120 kHz: 0.125 x periodicityExt, where periodicityExt has a value between 1 and 5120.

sps-ConfigIndex Indicates the index of one of multiple SPS configurations.

sps-HARQ-Deferral Indicates the maximum number of slots or subslots the transmission of DL SPS HARQ-ACK in a slot or subslot can be deferred (see TS 38.213 [13], clause 9.2.5.4).

| Conditional Presence | Explanation |
| --- | --- |
| SPS-List | The field is mandatory present when included in sps-ConfigToAddModList-r16, otherwise the field is absent. |

next change

–	SRS-Config

The IE SRS-Config is used to configure sounding reference signal transmissions. The configuration defines a list of SRS-Resources, a list of SRS-PosResources, a list of SRS-PosResourceSets and a list of SRS-ResourceSets. Each resource set defines a set of SRS-Resources or SRS-PosResources. The network triggers the transmission of the set of SRS-Resources or SRS-PosResources using a configured aperiodicSRS-ResourceTrigger (L1 DCI).

SRS-Config information element

-- ASN1START

-- TAG-SRS-CONFIG-START

SRS-Config ::=                          SEQUENCE {

srs-ResourceSetToReleaseList            SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId                OPTIONAL,   -- Need N

srs-ResourceSetToAddModList             SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet                  OPTIONAL,   -- Need N

srs-ResourceToReleaseList               SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId                      OPTIONAL,   -- Need N

srs-ResourceToAddModList                SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource                        OPTIONAL,   -- Need N

tpc-Accumulation                        ENUMERATED {disabled}                                                           OPTIONAL,   -- Need S

...,

[[

srs-RequestDCI-1-2-r16                  INTEGER (1..2)                                                          OPTIONAL, -- Need S

srs-RequestDCI-0-2-r16                  INTEGER (1..2)                                                          OPTIONAL, -- Need S

srs-ResourceSetToAddModListDCI-0-2-r16  SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet          OPTIONAL, -- Need N

srs-ResourceSetToReleaseListDCI-0-2-r16 SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId        OPTIONAL, -- Need N

srs-PosResourceSetToReleaseList-r16     SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSetId-r16

OPTIONAL, -- Need N

srs-PosResourceSetToAddModList-r16      SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16        OPTIONAL,-- Need N

srs-PosResourceToReleaseList-r16        SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16            OPTIONAL,-- Need N

srs-PosResourceToAddModList-r16         SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16              OPTIONAL -- Need N

]]

}

SRS-ResourceSet ::=                     SEQUENCE {

srs-ResourceSetId                       SRS-ResourceSetId,

srs-ResourceIdList                      SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId    OPTIONAL, -- Cond Setup

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

aperiodicSRS-ResourceTrigger            INTEGER (1..maxNrofSRS-TriggerStates-1),

csi-RS                                  NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

slotOffset                              INTEGER (1..32)                                        OPTIONAL, -- Need S

...,

[[

aperiodicSRS-ResourceTriggerList            SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL  -- Need M

]]

},

semi-persistent                         SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

},

periodic                                SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

}

},

usage                                   ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching},

alpha                                   Alpha                                                          OPTIONAL, -- Need S

p0                                      INTEGER (-202..24)                                             OPTIONAL, -- Cond Setup

pathlossReferenceRS                     PathlossReferenceRS-Config                                     OPTIONAL, -- Need M

srs-PowerControlAdjustmentStates        ENUMERATED { sameAsFci2, separateClosedLoop}                   OPTIONAL, -- Need S

...,

[[

pathlossReferenceRSList-r16             SetupRelease { PathlossReferenceRSList-r16}                    OPTIONAL,  -- Need M

]]

[[

usagePDC-r17             ENUMERATED {true}                    OPTIONAL  -- Need R

]]

}

PathlossReferenceRS-Config ::=              CHOICE {

ssb-Index                                   SSB-Index,

csi-RS-Index                                NZP-CSI-RS-ResourceId

}

PathlossReferenceRSList-r16 ::=             SEQUENCE (SIZE (1..maxNrofSRS-PathlossReferenceRS-r16)) OF PathlossReferenceRS-r16

PathlossReferenceRS-r16 ::=                 SEQUENCE {

srs-PathlossReferenceRS-Id-r16              SRS-PathlossReferenceRS-Id-r16,

pathlossReferenceRS-r16                     PathlossReferenceRS-Config

}

SRS-PathlossReferenceRS-Id-r16 ::=          INTEGER (0..maxNrofSRS-PathlossReferenceRS-1-r16)

SRS-PosResourceSet-r16 ::=                  SEQUENCE {

srs-PosResourceSetId-r16                    SRS-PosResourceSetId-r16,

srs-PosResourceIdList-r16                   SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-PosResourceId-r16

OPTIONAL, -- Cond Setup

resourceType-r16                            CHOICE {

aperiodic-r16                               SEQUENCE {

aperiodicSRS-ResourceTriggerList-r16        SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-1))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL, -- Need M

...

},

semi-persistent-r16                         SEQUENCE {

...

},

periodic-r16                                SEQUENCE {

...

}

},

alpha-r16                                   Alpha                                                      OPTIONAL, -- Need S

p0-r16                                      INTEGER (-202..24)                                         OPTIONAL, -- Cond Setup

pathlossReferenceRS-Pos-r16                 CHOICE {

ssb-IndexServing-r16                        SSB-Index,

ssb-Ncell-r16                               SSB-InfoNcell-r16,

dl-PRS-r16                                  DL-PRS-Info-r16

}                                                                                                      OPTIONAL, -- Need M

...

}

SRS-ResourceSetId ::=                   INTEGER (0..maxNrofSRS-ResourceSets-1)

SRS-PosResourceSetId-r16 ::=            INTEGER (0..maxNrofSRS-PosResourceSets-1-r16)

SRS-Resource ::=                        SEQUENCE {

srs-ResourceId                          SRS-ResourceId,

nrofSRS-Ports                           ENUMERATED {port1, ports2, ports4},

ptrs-PortIndex                          ENUMERATED {n0, n1 }                                           OPTIONAL,   -- Need R

transmissionComb                        CHOICE {

n2                                      SEQUENCE {

combOffset-n2                           INTEGER (0..1),

cyclicShift-n2                          INTEGER (0..7)

},

n4                                      SEQUENCE {

combOffset-n4                           INTEGER (0..3),

cyclicShift-n4                          INTEGER (0..11)

}

},

resourceMapping                         SEQUENCE {

startPosition                           INTEGER (0..5),

nrofSymbols                             ENUMERATED {n1, n2, n4},

repetitionFactor                        ENUMERATED {n1, n2, n4}

},

freqDomainPosition                      INTEGER (0..67),

freqDomainShift                         INTEGER (0..268),

freqHopping                             SEQUENCE {

c-SRS                                   INTEGER (0..63),

b-SRS                                   INTEGER (0..3),

b-hop                                   INTEGER (0..3)

},

groupOrSequenceHopping                  ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

...

},

semi-persistent                         SEQUENCE {

periodicityAndOffset-sp                     SRS-PeriodicityAndOffset,

...

},

periodic                                SEQUENCE {

periodicityAndOffset-p                      SRS-PeriodicityAndOffset,

...

}

},

sequenceId                              INTEGER (0..1023),

spatialRelationInfo                     SRS-SpatialRelationInfo                                        OPTIONAL,   -- Need R

...,

[[

resourceMapping-r16                     SEQUENCE {

startPosition-r16                       INTEGER (0..13),

nrofSymbols-r16                         ENUMERATED {n1, n2, n4},

repetitionFactor-r16                    ENUMERATED {n1, n2, n4}

}                                                                                                      OPTIONAL,    -- Need R

]]

[[

spatialRelationInfo-PDC-r17             SetupRelease { SpatialRelationInfo-PDC-r17 }                   OPTIONAL     -- Need M

]]

}

SRS-PosResource-r16::=                  SEQUENCE {

srs-PosResourceId-r16                   SRS-PosResourceId-r16,

transmissionComb-r16                    CHOICE {

n2-r16                                  SEQUENCE {

combOffset-n2-r16                       INTEGER (0..1),

cyclicShift-n2-r16                      INTEGER (0..7)

},

n4-r16                                  SEQUENCE {

combOffset-n4-r16                        INTEGER (0..3),

cyclicShift-n4-r16                      INTEGER (0..11)

},

n8-r16                                  SEQUENCE {

combOffset-n8-r16                       INTEGER (0..7),

cyclicShift-n8-r16                      INTEGER (0..5)

},

...

},

resourceMapping-r16                       SEQUENCE {

startPosition-r16                           INTEGER (0..13),

nrofSymbols-r16                             ENUMERATED {n1, n2, n4, n8, n12}

},

freqDomainShift-r16                       INTEGER (0..268),

freqHopping-r16                           SEQUENCE {

c-SRS-r16                                 INTEGER (0..63),

...

},

groupOrSequenceHopping-r16                ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType-r16                          CHOICE {

aperiodic-r16                             SEQUENCE {

slotOffset-r16                            INTEGER (1..32)                                      OPTIONAL,   -- Need S

...

},

semi-persistent-r16                       SEQUENCE {

periodicityAndOffset-sp-r16               SRS-PeriodicityAndOffset-r16,

...

},

periodic-r16                              SEQUENCE {

periodicityAndOffset-p-r16                SRS-PeriodicityAndOffset-r16,

...

}

},

sequenceId-r16                            INTEGER (0..65535),

spatialRelationInfoPos-r16                SRS-SpatialRelationInfoPos-r16                               OPTIONAL,   -- Need R

...

}

SRS-SpatialRelationInfo ::=     SEQUENCE {

servingCellId                       ServCellIndex                                                      OPTIONAL,   -- Need S

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId,

srs                                 SEQUENCE {

resourceId                          SRS-ResourceId,

uplinkBWP                           BWP-Id

}

}

}

SRS-SpatialRelationInfoPos-r16 ::=      CHOICE {

servingRS-r16                           SEQUENCE {

servingCellId                           ServCellIndex                                              OPTIONAL,   -- Need S

referenceSignal-r16                     CHOICE {

ssb-IndexServing-r16                    SSB-Index,

csi-RS-IndexServing-r16                 NZP-CSI-RS-ResourceId,

srs-SpatialRelation-r16                 SEQUENCE {

resourceSelection-r16                   CHOICE {

srs-ResourceId-r16                      SRS-ResourceId,

srs-PosResourceId-r16                   SRS-PosResourceId-r16

},

uplinkBWP-r16                           BWP-Id

}

}

},

ssb-Ncell-r16                           SSB-InfoNcell-r16,

dl-PRS-r16                              DL-PRS-Info-r16

}

SSB-Configuration-r16  ::=          SEQUENCE {

ssb-Freq-r16                     ARFCN-ValueNR,

halfFrameIndex-r16                  ENUMERATED {zero, one},

ssbSubcarrierSpacing-r16            SubcarrierSpacing,

ssb-Periodicity-r16                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2,spare1 }   OPTIONAL, -- Need S

sfn0-Offset-r16                     SEQUENCE {

sfn-Offset-r16                      INTEGER (0..1023),

integerSubframeOffset-r16           INTEGER (0..9)                                                 OPTIONAL  -- Need R

}                                                                                                      OPTIONAL, -- Need R

sfn-SSB-Offset-r16                  INTEGER (0..15),

ss-PBCH-BlockPower-r16              INTEGER (-60..50)                                                  OPTIONAL  -- Cond Pathloss

}

SSB-InfoNcell-r16  ::=              SEQUENCE {

physicalCellId-r16                  PhysCellId,

ssb-IndexNcell-r16                  SSB-Index                                                          OPTIONAL, -- Need S

ssb-Configuration-r16               SSB-Configuration-r16                                              OPTIONAL  -- Need S

}

DL-PRS-Info-r16  ::=                SEQUENCE {

dl-PRS-ID-r16                      INTEGER (0..255),

dl-PRS-ResourceSetId-r16           INTEGER (0..7),

dl-PRS-ResourceId-r16              INTEGER (0..63)                                                     OPTIONAL  -- Need S

}

SRS-ResourceId ::=                      INTEGER (0..maxNrofSRS-Resources-1)

SRS-PosResourceId-r16 ::=               INTEGER (0..maxNrofSRS-PosResources-1-r16)

SRS-PeriodicityAndOffset ::=            CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559)

}

SRS-PeriodicityAndOffset-r16 ::=        CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559),

sl5120                                  INTEGER(0..5119),

sl10240                                 INTEGER(0..10239),

sl40960                                 INTEGER(0..40959),

sl81920                                 INTEGER(0..81919),

...

}

SpatialRelationInfo-PDC-r17 ::=         SEQUENCE {

referenceSignal                     CHOICE {

ssb-Index            SSB-Index,

csi-RS-Index         NZP-CSI-RS-ResourceId,

dl-PRS-PDC           NR-DL-PRS-ResourceID-r17

srs                  SEQUENCE {

resourceId             SRS-ResourceId,

uplinkBWP                BWP-Id

}

...

},

...

}

-- TAG-SRS-CONFIG-STOP

-- ASN1STOP

SRS-Config field descriptions

tpc-Accumulation If the field is absent, UE applies TPC commands via accumulation. If disabled, UE applies the TPC command without accumulation (this applies to SRS when a separate closed loop is configured for SRS) (see TS 38.213 [13], clause 7.3).

SRS-Resource, SRS-PosResource field descriptions

cyclicShift-n2 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

cyclicShift-n4 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

freqHopping Includes parameters capturing SRS frequency hopping (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, the network always configures this field such that b-hop > b-SRS.

groupOrSequenceHopping Parameter(s) for configuring group or sequence hopping (see TS 38.211 [16], clause 6.4.1.4.2). For CLI SRS-RSRP measurement, the network always configures this parameter to 'neither'.

nrofSRS-Ports Number of ports. For CLI SRS-RSRP measurement, the network always configures this parameter to 'port1'.

periodicityAndOffset-p Periodicity and slot offset for this SRS resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, sl1280 and sl2560 cannot be configured.

periodicityAndOffset-sp Periodicity and slot offset for this SRS resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots (see TS 38.214 [19], clause 6.2.1).

ptrs-PortIndex The PTRS port index for this SRS resource for non-codebook based UL MIMO. This is only applicable when the corresponding PTRS-UplinkConfig is set to CP-OFDM. The ptrs-PortIndex configured here must be smaller than the maxNrofPorts configured in the PTRS-UplinkConfig (see TS 38.214 [19], clause 6.2.3.1). This parameter is not applicable to CLI SRS-RSRP measurement.

resourceMapping OFDM symbol location of the SRS resource within a slot including nrofSymbols (number of OFDM symbols), startPosition (value 0 refers to the last symbol, value 1 refers to the second last symbol, and so on) and repetitionFactor (see TS 38.214 [19], clause 6.2.1 and TS 38.211 [16], clause 6.4.1.4). The configured SRS resource does not exceed the slot boundary. If resourceMapping-r16 is signalled, UE shall ignore the resourceMapping (without suffix). For CLI SRS-RSRP measurement, the network always configures nrofSymbols and repetitionFactor to 'n1'.

resourceType Periodicity and offset for semi-persistent and periodic SRS resource (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, only 'periodic' is applicable for resourceType.

sequenceId Sequence ID used to initialize pseudo random group and sequence hopping (see TS 38.214 [19], clause 6.2.1).

servingCellId The serving Cell ID of the source SSB, CSI-RS, or SRS for the spatial relation of the target SRS resource. If this field is absent the SSB, the CSI-RS, or the SRS is from the same serving cell where the SRS is configured.

spatialRelationInfo Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS (see TS 38.214 [19], clause 6.2.1). This parameter is not applicable to CLI SRS-RSRP measurement.

spatialRelationInfo-PDC Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS/DL-PRS-PDC (see TS 38.214 [19], clause 6.2.1). The field is present in case of resourceType=periodic and usagePDC-r17=true in the SRS-ResourceSet, otherwise the field is absent.

spatialRelationInfoPos Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS/DL-PRS (see TS 38.214 [19], clause 6.2.1). If the IE srs-ResourceId-Ext is present, the IE srs-ResourceId in spatialRelationInfoPos represents the index from 0 to 63. Otherwise the IE srs-ResourceId in spatialRelationInfoPos represents the index from 0 to 31.

srs-RequestDCI-0-2 Indicate the number of bits for "SRS request"in DCI format 0_2. When the field is absent, then the value of 0 bit for "SRS request" in DCI format 0_2 is applied. If the parameter srs-RequestDCI-0-2 is configured to value 1, 1 bit is used to indicate one of the first two rows of Table 7.3.1.1.2-24 in TS 38.212 [17] for triggered aperiodic SRS resource set. If the value 2 is configured, 2 bits are used to indicate one of the rows of Table 7.3.1.1.2-24 in TS 38.212 [17]. When UE is configured with supplementaryUplink, an extra bit (the first bit of the SRS request field) is used for the non-SUL/SUL indication.

srs-RequestDCI-1-2 Indicate the number of bits for "SRS request" in DCI format 1_2. When the field is absent, then the value of 0 bit for "SRS request" in DCI format 1_2 is applied. When the UE is configured with supplementaryUplink, an extra bit (the first bit of the SRS request field) is used for the non-SUL/SUL indication (see TS 38.214 [19], clause 6.1.1.2).

srs-ResourceSetToAddModListDCI-0-2 List of SRS resource set to be added or modified for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

srs-ResourceSetToReleaseListDCI-0-2 List of SRS resource set to be released for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

transmissionComb Comb value (2 or 4 or 8) and comb offset (0..combValue-1) (see TS 38.214 [19], clause 6.2.1).

SRS-ResourceSet, SRS-PosResourceSet field descriptions

alpha alpha value for SRS power control (see TS 38.213 [13], clause 7.3). When the field is absent the UE applies the value 1.

aperiodicSRS-ResourceTriggerList An additional list of DCI "code points" upon which the UE shall transmit SRS according to this SRS resource set configuration (see TS 38.214 [19], clause 6). When the field is not included during a reconfiguration of SRS-ResourceSet of resourceType set to aperiodic, UE maintains this value based on the Need M; that is, this list is not considered as an extension of aperiodicSRS-ResourceTrigger for purpose of applying the general rule for extended list in clause 6.1.3.

aperiodicSRS-ResourceTrigger The DCI "code point" upon which the UE shall transmit SRS according to this SRS resource set configuration (see TS 38.214 [19], clause 6).

associatedCSI-RS ID of CSI-RS resource associated with this SRS resource set in non-codebook based operation (see TS 38.214 [19], clause 6.1.1.2).

csi-RS ID of CSI-RS resource associated with this SRS resource set. (see TS 38.214 [19], clause 6.1.1.2).

csi-RS-IndexServingcell Indicates CSI-RS index belonging to a serving cell

p0 P0 value for SRS power control. The value is in dBm. Only even values (step size 2) are allowed (see TS 38.213 [13], clause 7.3).

pathlossReferenceRS A reference signal (e.g. a CSI-RS config or a SS block) to be used for SRS path loss estimation (see TS 38.213 [13], clause 7.3).

pathlossReferenceRS-Pos A reference signal (e.g. a SS block or a DL-PRS config) to be used for SRS path loss estimation (see TS 38.213 [13], clause 7.3).

pathlossReferenceRSList Multiple candidate pathloss reference RS(s) for SRS power control, where one candidate RS can be mapped to SRS Resource Set via MAC CE (clause 6.1.3.27 in TS 38.321 [3]). The network can only configure this field if pathlossReferenceRS is not configured in the same SRS-ResourceSet.

resourceSelection Indicates whether the configured SRS spatial relation resource is a SRS-Resource or SRS-PosResource.

resourceType Time domain behavior of SRS resource configuration, see TS 38.214 [19], clause 6.2.1. The network configures SRS resources in the same resource set with the same time domain behavior on periodic, aperiodic and semi-persistent SRS.

slotOffset An offset in number of slots between the triggering DCI and the actual transmission of this SRS-ResourceSet. If the field is absent the UE applies no offset (value 0).

srs-PowerControlAdjustmentStates Indicates whether hsrs,c(i) = fc(i,1) or hsrs,c(i) = fc(i,2) (if twoPUSCH-PC-AdjustmentStates are configured) or separate close loop is configured for SRS. This parameter is applicable only for Uls on which UE also transmits PUSCH. If absent or release, the UE applies the value sameAs-Fci1 (see TS 38.213 [13], clause 7.3).

srs-ResourceIdList, srs-PosResourceIdList The IDs of the SRS-Resources/SRS-PosResource used in this SRS-ResourceSet/SRS-PosResourceSet. If this SRS-ResourceSet/SRS-PosResourceSet is configured with usage set to codebook, the srs-ResourceIdList/srs-PosResourceIdList contains at most 2 entries. If this SRS-ResourceSet/SRS-PosResourceSet is configured with usage set to nonCodebook, the srs-ResourceIdList/srs-PosResourceIdList contains at most 4 entries.

srs-ResourceSetId, srs-PosResourceSetId The ID of this resource set. It is unique in the context of the BWP in which the parent SRS-Config is defined.

ssb-IndexSevingcell Indicates SSB index belonging to a serving cell

ssb-NCell This field indicates a SSB configuration from neighboring cell

usage Indicates if the SRS resource set is used for beam management, codebook based or non-codebook based transmission or antenna switching. See TS 38.214 [19], clause 6.2.1. Reconfiguration between codebook based and non-codebook based transmission is not supported.

usagePDC If configured, it indicates that this SRS resource set is used for propagation delay compensation. The field can be present in only one SRS-ResourceSet. If this field is present, the UE ignore the field usage.

SSB-InfoNCell field descriptions

physicalCellId This field specifies the physical cell ID of the neighbour cell for which SSB configuration is provided.

ssb-IndexNcell This field specifies the index of the SSB for a neighbour cell. See TS 38.213 [13]. If this field is absent, the UE determines the ssb-IndexNcell of the physicalCellId based on its SSB measurement from the cell.

ssb-Configuration This field specifies the full configuration of the SSB. If this field is absent, the UE obtains the configuration for the SSB from nr-SSB-Config received as part of DL-PRS assistance data in LPP, see TS 37.355 [49], by looking up the corresponding SSB configuration using the field physicalCellId.

DL-PRS-Info field descriptions

dl-PRS-ID This field specifies the UE specific TRP ID (see TS 37.355 [49]) for which PRS configuration is provided.

dl-PRS-ResourceSetId This field specifies the PRS-ResourceSet ID of a PRS resourceSet.

dl-PRS-ResourceId This field specifies the PRS-Resource ID of a PRS resource. If this field is absent, the UE determines the dl-PRS-ResourceID based on its PRS measurement from the TRP (see TS 37.355 [49]) and DL-PRS Resource Set.

SSB-Configuration field descriptions

halfFrameIndex Indicates whether SSB is in the first half or the second half of the frame. Value zero indicates the first half and value 1 indicates the second half.

integerSubframeOffset Indicates the subframe boundary offset of the cell in which SSB is transmited.

sfn0-Offset Indiactes the time offset of the SFN0 slot 0 for the cell with respect to SFN0 slot 0 of serving cell.

sfn-Offset Specifies the SFN offset between the cell in which SSB is transmited and serving cell. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of serving cell to the beginning of the closest subsequent radio frame #0 of the cell in which SSB is transmitted.

sfn-SSB-Offset Indicates the SFN offset of the transmitted SSB relative to the start of the SSB period. Value 0 indicates that the SSB is transmitted in the first system frame, value 1 indicates that SSB is transmitted in the second system frame and so on. The network configures this field according to the field ssb-Periodicity such that the indicated system frame does not exceed the configured SSB periodicity.

ssb-Freq Indicates the frequency of the SSB.

ssb-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

ssb-Periodicity Indicates the periodicity of the SSB. If the field is absent, the UE applies the value ms5. (see TS 38.213 [13], clause 4.1)

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the values 15 kHz or 30 kHz (FR1), and 120 kHz or 240 kHz (FR2) are applicable.

| Conditional Presence | Explanation |
| --- | --- |
| Setup | This field is mandatory present upon configuration of SRS-ResourceSet or SRS-Resource and optionally present, Need M, otherwise. |
| NonCodebook | This field is optionally present, Need M, in case of non-codebook based transmission, otherwise the field is absent. |
| Pathloss | The field is mandatory present if the IE SSB-InfoNcell is included in pathlossReferenceRS-Pos; otherwise it is optionally present, Need R |

next change

6.4	RRC multiplicity and type constraint values

–	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxAI-DCI-PayloadSize-r16               INTEGER ::= 128      --Maximum size of the DCI payload scrambled with ai-RNTI

maxAI-DCI-PayloadSize-1-r16             INTEGER ::= 127      --Maximum size of the DCI payload scrambled with ai-RNTI minus 1

maxBandComb                             INTEGER ::= 65536   -- Maximum number of DL band combinations

maxBandsUTRA-FDD-r16                    INTEGER ::= 64      -- Maximum number of bands listed in UTRA-FDD UE caps

maxBH-RLC-ChannelID-r16                 INTEGER ::= 65536   -- Maximum value of BH RLC Channel ID

maxBT-IdReport-r16                      INTEGER ::= 32      -- Maximum number of Bluetooth IDs to report

maxBT-Name-r16                          INTEGER ::= 4       -- Maximum number of Bluetooth name

maxCAG-Cell-r16                         INTEGER ::= 16      -- Maximum number of NR CAG cell ranges in SIB3, SIB4

maxTwoPUCCH-Grp-ConfigList-r16          INTEGER ::= 32      -- Maximum number of supported configuration(s) of {primary PUCCH group

-- config, secondary PUCCH group config}

maxCBR-Config-r16                       INTEGER ::= 8       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control

maxCBR-Config-1-r16                     INTEGER ::= 7       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control minus 1

maxCBR-Level-r16                        INTEGER ::= 16      -- Maximum number of CBR levels

maxCBR-Level-1-r16                      INTEGER ::= 15      -- Maximum number of CBR levels minus 1

maxCellBlack                            INTEGER ::= 16      -- Maximum number of NR blacklisted cell ranges in SIB3, SIB4

maxCellGroupings-r16                    INTEGER ::= 32      -- Maximum number of cell groupings for NR-DC

maxCellHistory-r16                      INTEGER ::= 16      -- Maximum number of visited cells reported

maxCellInter                            INTEGER ::= 16      -- Maximum number of inter-Freq cells listed in SIB4

maxCellIntra                            INTEGER ::= 16      -- Maximum number of intra-Freq cells listed in SIB3

maxCellMeasEUTRA                        INTEGER ::= 32      -- Maximum number of cells in E-UTRAN

maxCellMeasIdle-r16                     INTEGER ::= 8       -- Maximum number of cells per carrier for idle/inactive measurements

maxCellMeasUTRA-FDD-r16                 INTEGER ::= 32      -- Maximum number of cells in FDD UTRAN

maxCellWhite                            INTEGER ::= 16      -- Maximum number of NR whitelisted cell ranges in SIB3, SIB4

maxEARFCN                               INTEGER ::= 262143  -- Maximum value of E-UTRA carrier frequency

maxEUTRA-CellBlack                      INTEGER ::= 16      -- Maximum number of E-UTRA blacklisted physical cell identity ranges

-- in SIB5

maxEUTRA-NS-Pmax                        INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxLogMeasReport-r16                    INTEGER ::= 520     -- Maximum number of entries for logged measurements

maxMultiBands                           INTEGER ::= 8       -- Maximum number of additional frequency bands that a cell belongs to

maxNARFCN                               INTEGER ::= 3279165 -- Maximum value of NR carrier frequency

maxNR-NS-Pmax                           INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxFreqIdle-r16                         INTEGER ::= 8       -- Maximum number of carrier frequencies for idle/inactive measurements

maxNrofServingCells                     INTEGER ::= 32      -- Max number of serving cells (SpCells + SCells)

maxNrofServingCells-1                   INTEGER ::= 31      -- Max number of serving cells (SpCells + SCells) minus 1

maxNrofAggregatedCellsPerCellGroup      INTEGER ::= 16

maxNrofAggregatedCellsPerCellGroupMinus4-r16   INTEGER ::= 12

maxNrofDUCells-r16                      INTEGER ::= 512     -- Max number of cells configured on the collocated IAB-DU

maxNrofAvailabilityCombinationsPerSet-r16   INTEGER ::= 512 -- Max number of AvailabilityCombinationId used in the DCI format 2_5

maxNrofAvailabilityCombinationsPerSet-1-r16 INTEGER ::= 511 -- Max number of AvailabilityCombinationId used in the DCI format 2_5 minus 1

maxNrofSCells                           INTEGER ::= 31      -- Max number of secondary serving cells per cell group

maxNrofCellMeas                         INTEGER ::= 32      -- Maximum number of entries in each of the cell lists in a measurement object

maxNrofCG-SL-r16                        INTEGER ::= 8       -- Max number of sidelink configured grant

maxNrofCG-SL-1-r16                      INTEGER ::= 7       -- Max number of sidelink configured grant minus 1

maxNrofSS-BlocksToAverage               INTEGER ::= 16      -- Max number for the (max) number of SS blocks to average to determine cell measurement

maxNrofCondCells-r16                    INTEGER ::= 8       -- Max number of conditional candidate SpCells

maxNrofCSI-RS-ResourcesToAverage        INTEGER ::= 16      -- Max number for the (max) number of CSI-RS to average to determine cell measurement

maxNrofDL-Allocations                   INTEGER ::= 16      -- Maximum number of PDSCH time domain resource allocations

maxNrofSR-ConfigPerCellGroup            INTEGER ::= 8       -- Maximum number of SR configurations per cell group

maxLCG-ID                               INTEGER ::= 7       -- Maximum value of LCG ID

maxLC-ID                                INTEGER ::= 32      -- Maximum value of Logical Channel ID

maxLC-ID-Iab-r16                        INTEGER ::= 65855   -- Maximum value of BH Logical Channel ID extension

maxLTE-CRS-Patterns-r16                 INTEGER ::= 3       -- Maximum number of additional LTE CRS rate matching patterns

maxNrofTAGs                             INTEGER ::= 4       -- Maximum number of Timing Advance Groups

maxNrofTAGs-1                           INTEGER ::= 3       -- Maximum number of Timing Advance Groups minus 1

maxNrofBWPs                             INTEGER ::= 4       -- Maximum number of BWPs per serving cell

maxNrofCombIDC                          INTEGER ::= 128     -- Maximum number of reported MR-DC combinations for IDC

maxNrofSymbols-1                        INTEGER ::= 13      -- Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13)

maxNrofSlots                            INTEGER ::= 320     -- Maximum number of slots in a 10 ms period

maxNrofSlots-1                          INTEGER ::= 319     -- Maximum number of slots in a 10 ms period minus 1

maxNrofPhysicalResourceBlocks           INTEGER ::= 275     -- Maximum number of PRBs

maxNrofPhysicalResourceBlocks-1         INTEGER ::= 274     -- Maximum number of PRBs minus 1

maxNrofPhysicalResourceBlocksPlus1      INTEGER ::= 276     -- Maximum number of PRBs plus 1

maxNrofControlResourceSets              INTEGER ::= 12      -- Max number of CoReSets configurable on a serving cell

maxNrofControlResourceSets-1            INTEGER ::= 11      -- Max number of CoReSets configurable on a serving cell minus 1

maxNrofControlResourceSets-1-r16        INTEGER ::= 15      -- Max number of CoReSets configurable on a serving cell extended in minus 1

maxNrofCoresetPools-r16                 INTEGER ::= 2       -- Maximum number of CORESET pools

maxCoReSetDuration                      INTEGER ::= 3       -- Max number of OFDM symbols in a control resource set

maxNrofSearchSpaces-1                   INTEGER ::= 39      -- Max number of Search Spaces minus 1

maxSFI-DCI-PayloadSize                  INTEGER ::= 128     -- Max number payload of a DCI scrambled with SFI-RNTI

maxSFI-DCI-PayloadSize-1                INTEGER ::= 127     -- Max number payload of a DCI scrambled with SFI-RNTI minus 1

maxIAB-IP-Address-r16                   INTEGER ::= 32      -- Max number of assigned IP addresses

maxINT-DCI-PayloadSize                  INTEGER ::= 126     -- Max number payload of a DCI scrambled with INT-RNTI

maxINT-DCI-PayloadSize-1                INTEGER ::= 125     -- Max number payload of a DCI scrambled with INT-RNTI minus 1

maxNrofRateMatchPatterns                INTEGER ::= 4       -- Max number of rate matching patterns that may be configured

maxNrofRateMatchPatterns-1              INTEGER ::= 3       -- Max number of rate matching patterns that may be configured minus 1

maxNrofRateMatchPatternsPerGroup        INTEGER ::= 8       -- Max number of rate matching patterns that may be configured in one group

maxNrofCSI-ReportConfigurations         INTEGER ::= 48      -- Maximum number of report configurations

maxNrofCSI-ReportConfigurations-1       INTEGER ::= 47      -- Maximum number of report configurations minus 1

maxNrofCSI-ResourceConfigurations       INTEGER ::= 112     -- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1     INTEGER ::= 111     -- Maximum number of resource configurations minus 1

maxNrofAP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrOfCSI-AperiodicTriggers            INTEGER ::= 128     -- Maximum number of triggers for aperiodic CSI reporting

maxNrofReportConfigPerAperiodicTrigger  INTEGER ::= 16      -- Maximum number of report configurations per trigger state for aperiodic reporting

maxNrofNZP-CSI-RS-Resources             INTEGER ::= 192     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofNZP-CSI-RS-Resources-1           INTEGER ::= 191     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofNZP-CSI-RS-ResourcesPerSet       INTEGER ::= 64      -- Maximum number of NZP CSI-RS resources per resource set

maxNrofNZP-CSI-RS-ResourceSets          INTEGER ::= 64      -- Maximum number of NZP CSI-RS resource sets per cell

maxNrofNZP-CSI-RS-ResourceSets-1        INTEGER ::= 63      -- Maximum number of NZP CSI-RS resource sets per cell minus 1

maxNrofNZP-CSI-RS-ResourceSetsPerConfig INTEGER ::= 16      -- Maximum number of resource sets per resource configuration

maxNrofNZP-CSI-RS-ResourcesPerConfig    INTEGER ::= 128     -- Maximum number of resources per resource configuration

maxNrofZP-CSI-RS-Resources              INTEGER ::= 32      -- Maximum number of Zero-Power (ZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1            INTEGER ::= 31      -- Maximum number of Zero-Power (ZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-ResourceSets-1         INTEGER ::= 15

maxNrofZP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrofZP-CSI-RS-ResourceSets           INTEGER ::= 16

maxNrofCSI-IM-Resources                 INTEGER ::= 32      -- Maximum number of CSI-IM resources

maxNrofCSI-IM-Resources-1               INTEGER ::= 31      -- Maximum number of CSI-IM resources minus 1

maxNrofCSI-IM-ResourcesPerSet           INTEGER ::= 8       -- Maximum number of CSI-IM resources per set

maxNrofCSI-IM-ResourceSets              INTEGER ::= 64      -- Maximum number of NZP CSI-IM resource sets per cell

maxNrofCSI-IM-ResourceSets-1            INTEGER ::= 63      -- Maximum number of NZP CSI-IM resource sets per cell minus 1

maxNrofCSI-IM-ResourceSetsPerConfig     INTEGER ::= 16      -- Maximum number of CSI IM resource sets per resource configuration

maxNrofCSI-SSB-ResourcePerSet           INTEGER ::= 64      -- Maximum number of SSB resources in a resource set

maxNrofCSI-SSB-ResourceSets             INTEGER ::= 64      -- Maximum number of CSI SSB resource sets per cell

maxNrofCSI-SSB-ResourceSets-1           INTEGER ::= 63      -- Maximum number of CSI SSB resource sets per cell minus 1

maxNrofCSI-SSB-ResourceSetsPerConfig    INTEGER ::= 1       -- Maximum number of CSI SSB resource sets per resource configuration

maxNrofFailureDetectionResources        INTEGER ::= 10      -- Maximum number of failure detection resources

maxNrofFailureDetectionResources-1      INTEGER ::= 9       -- Maximum number of failure detection resources minus 1

maxNrofFreqSL-r16                       INTEGER ::= 8       -- Maximum number of carrier frequency for NR sidelink communication

maxNrofSL-BWPs-r16                      INTEGER ::= 4       -- Maximum number of BWP for NR sidelink communication

maxFreqSL-EUTRA-r16                     INTEGER ::= 8       -- Maximum number of EUTRA anchor carrier frequency for NR sidelink communication

maxNrofSL-MeasId-r16                    INTEGER ::= 64      -- Maximum number of sidelink measurement identity (RSRP) per destination

maxNrofSL-ObjectId-r16                  INTEGER ::= 64      -- Maximum number of sidelink measurement objects (RSRP) per destination

maxNrofSL-ReportConfigId-r16            INTEGER ::= 64      -- Maximum number of sidelink measurement reporting configuration(RSRP) per destination

maxNrofSL-PoolToMeasureNR-r16           INTEGER ::= 8       -- Maximum number of resource pool for NR sidelink measurement to measure for

-- each measurement object (for CBR)

maxFreqSL-NR-r16                        INTEGER ::= 8       -- Maximum number of NR anchor carrier frequency for NR sidelink communication

maxNrofSL-QFIs-r16                      INTEGER ::= 2048    -- Maximum number of QoS flow for NR sidelink communication per UE

maxNrofSL-QFIsPerDest-r16               INTEGER ::= 64      -- Maximum number of QoS flow per destination for NR sidelink communication

maxNrofObjectId                         INTEGER ::= 64      -- Maximum number of measurement objects

maxNrofPageRec                          INTEGER ::= 32      -- Maximum number of page records

maxNrofPCI-Ranges                       INTEGER ::= 8       -- Maximum number of PCI ranges

maxPLMN                                 INTEGER ::= 12      -- Maximum number of PLMNs broadcast and reported by UE at establishment

maxNrofCSI-RS-ResourcesRRM              INTEGER ::= 96      -- Maximum number of CSI-RS resources per cell for an RRM measurement object

maxNrofCSI-RS-ResourcesRRM-1            INTEGER ::= 95      -- Maximum number of CSI-RS resources per cell for an RRM measurement object minus 1

maxNrofMeasId                           INTEGER ::= 64      -- Maximum number of configured measurements

maxNrofQuantityConfig                   INTEGER ::= 2       -- Maximum number of quantity configurations

maxNrofCSI-RS-CellsRRM                  INTEGER ::= 96      -- Maximum number of cells with CSI-RS resources for an RRM measurement object

maxNrofSL-Dest-r16                      INTEGER ::= 32      -- Maximum number of destination for NR sidelink communication

maxNrofSL-Dest-1-r16                    INTEGER ::= 31      -- Highest index of destination for NR sidelink communication

maxNrofSLRB-r16                         INTEGER ::= 512     -- Maximum number of radio bearer for NR sidelink communication per UE

maxSL-LCID-r16                          INTEGER ::= 512     -- Maximum number of RLC bearer for NR sidelink communication per UE

maxSL-SyncConfig-r16                    INTEGER ::= 16      -- Maximum number of sidelink Sync configurations

maxNrofRXPool-r16                       INTEGER ::= 16      -- Maximum number of Rx resource pool for NR sidelink communication

maxNrofTXPool-r16                       INTEGER ::= 8       -- Maximum number of Tx resource pool for NR sidelink communication

maxNrofPoolID-r16                       INTEGER ::= 16      -- Maximum index of resource pool for NR sidelink communication

maxNrofSRS-PathlossReferenceRS-r16      INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for SRS power control.

maxNrofSRS-PathlossReferenceRS-1-r16    INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for SRS power control minus 1.

maxNrofSRS-ResourceSets                 INTEGER ::= 16      -- Maximum number of SRS resource sets in a BWP.

maxNrofSRS-ResourceSets-1               INTEGER ::= 15      -- Maximum number of SRS resource sets in a BWP minus 1.

maxNrofSRS-PosResourceSets-r16          INTEGER ::= 16      -- Maximum number of SRS Positioning resource sets in a BWP.

maxNrofSRS-PosResourceSets-1-r16        INTEGER ::= 15      -- Maximum number of SRS Positioning resource sets in a BWP minus 1.

maxNrofSRS-Resources                    INTEGER ::= 64      -- Maximum number of SRS resources.

maxNrofSRS-Resources-1                  INTEGER ::= 63      -- Maximum number of SRS resources minus 1.

maxNrofSRS-PosResources-r16             INTEGER ::= 64      -- Maximum number of SRS Positioning resources.

maxNrofSRS-PosResources-1-r16           INTEGER ::= 63      -- Maximum number of SRS Positioning resources in an SRS Positioning

-- resource set minus 1.

maxNrofSRS-ResourcesPerSet              INTEGER ::= 16      -- Maximum number of SRS resources in an SRS resource set

maxNrofSRS-TriggerStates-1              INTEGER ::= 3       -- Maximum number of SRS trigger states minus 1, i.e., the largest code point.

maxNrofSRS-TriggerStates-2              INTEGER ::= 2       -- Maximum number of SRS trigger states minus 2.

maxRAT-CapabilityContainers             INTEGER ::= 8       -- Maximum number of interworking RAT containers (incl NR and MRDC)

maxSimultaneousBands                    INTEGER ::= 32      -- Maximum number of simultaneously aggregated bands

maxULTxSwitchingBandPairs               INTEGER ::= 32      -- Maximum number of band pairs supporting dynamic UL Tx switching in a band combination

maxNrofSlotFormatCombinationsPerSet     INTEGER ::= 512     -- Maximum number of Slot Format Combinations in a SF-Set.

maxNrofSlotFormatCombinationsPerSet-1   INTEGER ::= 511     -- Maximum number of Slot Format Combinations in a SF-Set minus 1.

maxNrofTrafficPattern-r16               INTEGER ::= 8       -- Maximum number of Traffic Pattern for NR sidelink communication.

maxNrofPUCCH-Resources                  INTEGER ::= 128

maxNrofPUCCH-Resources-1                INTEGER ::= 127

maxNrofPUCCH-ResourceSets               INTEGER ::= 4       -- Maximum number of PUCCH Resource Sets

maxNrofPUCCH-ResourceSets-1             INTEGER ::= 3       -- Maximum number of PUCCH Resource Sets minus 1.

maxNrofPUCCH-ResourcesPerSet            INTEGER ::= 32      -- Maximum number of PUCCH Resources per PUCCH-ResourceSet

maxNrofPUCCH-P0-PerSet                  INTEGER ::= 8       -- Maximum number of P0-pucch present in a p0-pucch set

maxNrofPUCCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUCCH power control.

maxNrofPUCCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUCCH power control minus 1.

maxNrofPUCCH-PathlossReferenceRSs-r16   INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUCCH power control extended.

maxNrofPUCCH-PathlossReferenceRSs-1-r16 INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- minus 1 extended.

maxNrofPUCCH-PathlossReferenceRSsDiff-r16 INTEGER ::= 60    -- Difference between the extended maximum and the non-extended maximum

maxNrofPUCCH-ResourceGroups-r16         INTEGER ::= 4       -- Maximum number of PUCCH resources groups.

maxNrofPUCCH-ResourcesPerGroup-r16      INTEGER ::= 128     -- Maximum number of PUCCH resources in a PUCCH group.

maxNrofMultiplePUSCHs-r16               INTEGER ::= 8       -- Maximum number of multiple PUSCHs in PUSCH TDRA list

maxNrofP0-PUSCH-AlphaSets               INTEGER ::= 30      -- Maximum number of P0-pusch-alpha-sets (see TS 38.213 [13], clause 7.1)

maxNrofP0-PUSCH-AlphaSets-1             INTEGER ::= 29      -- Maximum number of P0-pusch-alpha-sets minus 1 (see TS 38.213 [13], clause 7.1)

maxNrofPUSCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUSCH power control.

maxNrofPUSCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUSCH power control minus 1.

maxNrofPUSCH-PathlossReferenceRSs-r16   INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUSCH power control extended

maxNrofPUSCH-PathlossReferenceRSs-1-r16 INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUSCH power control

-- extended minus 1

maxNrofPUSCH-PathlossReferenceRSsDiff-r16  INTEGER ::= 60   -- Difference between maxNrofPUSCH-PathlossReferenceRSs-r16 and

-- maxNrofPUSCH-PathlossReferenceRSs

maxNrofNAICS-Entries                    INTEGER ::= 8       -- Maximum number of supported NAICS capability set

maxBands                                INTEGER ::= 1024    -- Maximum number of supported bands in UE capability.

maxBandsMRDC                            INTEGER ::= 1280

maxBandsEUTRA                           INTEGER ::= 256

maxCellReport                           INTEGER ::= 8

maxDRB                                  INTEGER ::= 29      -- Maximum number of DRBs (that can be added in DRB-ToAddModList).

maxFreq                                 INTEGER ::= 8       -- Max number of frequencies.

maxFreqLayers                           INTEGER ::= 4       -- Max number of frequency layers.

maxFreqIDC-r16                          INTEGER ::= 128     -- Max number of frequencies for IDC indication.

maxCombIDC-r16                          INTEGER ::= 128     -- Max number of reported UL CA for IDC indication.

maxFreqIDC-MRDC                         INTEGER ::= 32      -- Maximum number of candidate NR frequencies for MR-DC IDC indication

maxNrofCandidateBeams                   INTEGER ::= 16      -- Max number of PRACH-ResourceDedicatedBFR in BFR config.

maxNrofCandidateBeams-r16               INTEGER ::= 64      -- Max number of candidate beam resources in BFR config.

maxNrofCandidateBeamsExt-r16            INTEGER ::= 48      -- Max number of PRACH-ResourceDedicatedBFR in the CandidateBeamRSListExt

maxNrofPCIsPerSMTC                      INTEGER ::= 64      -- Maximum number of PCIs per SMTC.

maxNrofQFIs                             INTEGER ::= 64

maxNrofResourceAvailabilityPerCombination-r16 INTEGER ::= 256

maxNrOfSemiPersistentPUSCH-Triggers     INTEGER ::= 64      -- Maximum number of triggers for semi persistent reporting on PUSCH

maxNrofSR-Resources                     INTEGER ::= 8       -- Maximum number of SR resources per BWP in a cell.

maxNrofSlotFormatsPerCombination        INTEGER ::= 256

maxNrofSpatialRelationInfos             INTEGER ::= 8

maxNrofSpatialRelationInfos-plus-1      INTEGER ::= 9

maxNrofSpatialRelationInfos-r16         INTEGER ::= 64

maxNrofSpatialRelationInfosDiff-r16     INTEGER ::= 56      -- Difference between maxNrofSpatialRelationInfos-r16 and maxNrofSpatialRelationInfos

maxNrofIndexesToReport                  INTEGER ::= 32

maxNrofIndexesToReport2                 INTEGER ::= 64

maxNrofSSBs-r16                         INTEGER ::= 64      -- Maximum number of SSB resources in a resource set.

maxNrofSSBs-1                           INTEGER ::= 63      -- Maximum number of SSB resources in a resource set minus 1.

maxNrofS-NSSAI                          INTEGER ::= 8       -- Maximum number of S-NSSAI.

maxNrofTCI-StatesPDCCH                  INTEGER ::= 64

maxNrofTCI-States                       INTEGER ::= 128     -- Maximum number of TCI states.

maxNrofTCI-States-1                     INTEGER ::= 127     -- Maximum number of TCI states minus 1.

maxNrofUL-Allocations                   INTEGER ::= 16      -- Maximum number of PUSCH time domain resource allocations.

maxQFI                                  INTEGER ::= 63

maxRA-CSIRS-Resources                   INTEGER ::= 96

maxRA-OccasionsPerCSIRS                 INTEGER ::= 64      -- Maximum number of RA occasions for one CSI-RS

maxRA-Occasions-1                       INTEGER ::= 511     -- Maximum number of RA occasions in the system

maxRA-SSB-Resources                     INTEGER ::= 64

maxSCSs                                 INTEGER ::= 5

maxSecondaryCellGroups                  INTEGER ::= 3

maxNrofServingCellsEUTRA                INTEGER ::= 32

maxMBSFN-Allocations                    INTEGER ::= 8

maxNrofMultiBands                       INTEGER ::= 8

maxCellSFTD                             INTEGER ::= 3       -- Maximum number of cells for SFTD reporting

maxReportConfigId                       INTEGER ::= 64

maxNrofCodebooks                        INTEGER ::= 16      -- Maximum number of codebooks supported by the UE

maxNrofCSI-RS-ResourcesExt-r16          INTEGER ::= 16      -- Maximum number of codebook resources supported by the UE for eType2/Codebook combo

maxNrofCSI-RS-Resources                 INTEGER ::= 7       -- Maximum number of codebook resources supported by the UE

maxNrofCSI-RS-ResourcesAlt-r16          INTEGER ::= 512     -- Maximum number of alternative codebook resources supported by the UE

maxNrofCSI-RS-ResourcesAlt-1-r16        INTEGER ::= 511     -- Maximum number of alternative codebook resources supported by the UE minus 1

maxNrofSRI-PUSCH-Mappings               INTEGER ::= 16

maxNrofSRI-PUSCH-Mappings-1             INTEGER ::= 15

maxSIB                                  INTEGER::= 32       -- Maximum number of SIBs

maxSI-Message                           INTEGER::= 32       -- Maximum number of SI messages

maxPO-perPF                             INTEGER ::= 4       -- Maximum number of paging occasion per paging frame

maxAccessCat-1                          INTEGER ::= 63      -- Maximum number of Access Categories minus 1

maxBarringInfoSet                       INTEGER ::= 8       -- Maximum number of access control parameter sets

maxCellEUTRA                            INTEGER ::= 8       -- Maximum number of E-UTRA cells in SIB list

maxEUTRA-Carrier                        INTEGER ::= 8       -- Maximum number of E-UTRA carriers in SIB list

maxPLMNIdentities                       INTEGER ::= 8       -- Maximum number of PLMN identities in RAN area configurations

maxDownlinkFeatureSets                  INTEGER ::= 1024    -- (for NR DL) Total number of FeatureSets (size of the pool)

maxUplinkFeatureSets                    INTEGER ::= 1024    -- (for NR UL) Total number of FeatureSets (size of the pool)

maxEUTRA-DL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxEUTRA-UL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxFeatureSetsPerBand                   INTEGER ::= 128     -- (for NR) The number of feature sets associated with one band.

maxPerCC-FeatureSets                    INTEGER ::= 1024    -- (for NR) Total number of CC-specific FeatureSets (size of the pool)

maxFeatureSetCombinations               INTEGER ::= 1024    -- (for MR-DC/NR)Total number of Feature set combinations (size of the pool)

maxInterRAT-RSTD-Freq                   INTEGER ::= 3

maxHRNN-Len-r16                         INTEGER ::= 48      -- Maximum length of HRNNs

maxNPN-r16                              INTEGER ::= 12      -- Maximum number of NPNs broadcast and reported by UE at establishment

maxNrOfMinSchedulingOffsetValues-r16    INTEGER ::= 2       -- Maximum number of min. scheduling offset (K0/K2) configurations

maxK0-SchedulingOffset-r16              INTEGER ::= 16      -- Maximum number of slots configured as min. scheduling offset (K0)

maxK2-SchedulingOffset-r16              INTEGER ::= 16      -- Maximum number of slots configured as min. scheduling offset (K2)

maxDCI-2-6-Size-r16                     INTEGER ::= 140     -- Maximum size of DCI format 2-6

maxDCI-2-6-Size-1-r16                   INTEGER ::= 139     -- Maximum DCI format 2-6 size minus 1

maxNrofUL-Allocations-r16               INTEGER ::= 64      -- Maximum number of PUSCH time domain resource allocations

maxNrofP0-PUSCH-Set-r16                 INTEGER ::= 2       -- Maximum number of P0 PUSCH set(s)

maxOnDemandSIB-r16                      INTEGER ::= 8       -- Maximum number of SIB(s) that can be requested on-demand

maxOnDemandPosSIB-r16                   INTEGER ::= 32      -- Maximum number of posSIB(s) that can be requested on-demand

maxCI-DCI-PayloadSize-r16               INTEGER ::= 126     -- Maximum number of the DCI size for CI

maxCI-DCI-PayloadSize-1-r16             INTEGER ::= 125     -- Maximum number of the DCI size for CI minus 1

maxWLAN-Id-Report-r16                   INTEGER ::= 32      -- Maximum number of WLAN IDs to report

maxWLAN-Name-r16                        INTEGER ::= 4       -- Maximum number of WLAN name

maxRAReport-r16                         INTEGER ::= 8       -- Maximum number of RA procedures information to be included in the RA report

maxTxConfig-r16                         INTEGER ::= 64      -- Maximum number of sidelink transmission parameters configurations

maxTxConfig-1-r16                       INTEGER ::= 63      -- Maximum number of sidelink transmission parameters configurations minus 1

maxPSSCH-TxConfig-r16                   INTEGER ::= 16      -- Maximum number of PSSCH TX configurations

maxNrofCLI-RSSI-Resources-r16           INTEGER ::= 64      -- Maximum number of CLI-RSSI resources for UE

maxNrofCLI-RSSI-Resources-1-r16         INTEGER ::= 63      -- Maximum number of CLI-RSSI resources for UE minus 1

maxNrofCLI-SRS-Resources-r16            INTEGER ::= 32      -- Maximum number of SRS resources for CLI measurement for UE

maxCLI-Report-r16                       INTEGER ::= 8

maxNrofConfiguredGrantConfig-r16        INTEGER ::= 12      -- Maximum number of configured grant configurations per BWP

maxNrofConfiguredGrantConfig-1-r16      INTEGER ::= 11      -- Maximum number of configured grant configurations per BWP minus 1

maxNrofCG-Type2DeactivationState        INTEGER ::= 16      -- Maximum number of deactivation state for type 2 configured grants per BWP

maxNrofConfiguredGrantConfigMAC-1-r16   INTEGER ::= 31      -- Maximum number of configured grant configurations per MAC entity minus 1

maxNrofSPS-Config-r16                   INTEGER ::= 8       -- Maximum number of SPS configurations per BWP

maxNrofSPS-Config-1-r16                 INTEGER ::= 7       -- Maximum number of SPS configurations per BWP minus 1

maxNrofSPS-DeactivationState            INTEGER ::= 16      -- Maximum number of deactivation state for SPS per BWP

maxNrofDormancyGroups                   INTEGER ::= 5       --

maxNrofPUCCH-ResourceGroups-1-r16       INTEGER ::= 3       --

maxNrofServingCellsTCI-r16              INTEGER ::= 32      -- Maximum number of serving cells in simultaneousTCI-UpdateList

maxNrofTxDC-TwoCarrier-r16              INTEGER ::= 64      -- Maximum number of UL Tx DC locations reported by the UE for 2CC uplink CA

maxNrofEnhType3HARQ-ACK-r17             INTEGER ::= 8       -- Maximum number of enhanced type 3 HARQ-ACK codebook

maxNrofEnhType3HARQ-ACK-r17-1           INTEGER ::= 7       -- Maximum number of enhanced type 3 HARQ-ACK codebook minus 1

maxNrofPRS-ResourcesPerSet-r17          INTEGER ::= 64      -- Maximum number of PRS resources for one set

maxNrofPRS-ResourcesPerSet-1-r17        INTEGER ::= 63      -- Maximum number of PRS resources for one set minus 1

maxNrofPRS-ResourceOffsetValue-1-r17    INTEGER ::= 511

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

–	End of NR-RRC-Definitions

-- ASN1START

END

-- ASN1STOP

Next change