3GPP TSG-RAN WG2 Meeting #117-e	R2-220xxxx

E-meeting, February 21 - March 3, 2022

Title:	DRAFT LS Reply to CT1 on Small Data Transmission

Response to:	R2-2203859/ C1-221891) NAS's trigger for resume for SDT

Release:	Release-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	Apple [To be RAN2]

To:	CT1

Cc:

Contact Person:

- Name:	Fangli XU

- E-mail Address:	fangli_xu@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R2-2204234 (38.300 stage-2 CR: Introduction of SDT)

1. Overall Description:

RAN2 thanks CT1 for their LS in C1-221891 on the topic of NAS's trigger for resume for SDT.

Based on CT1’s answers, RAN2 has made further agreements related to AS/NAS interaction in RAN2#117 meeting as follows:

- If UE detects an SDT failure of ongoing SDT session for the transfer of NAS message, RRC informs NAS about the failure for NAS message transfer. - For non-SDT data arrival indication, ResumeCause value is included in UAI as an optional IE (and hence is provided to the network if upper layers provide it to the AS). If NAS provides it the UE shall include it in resume cause.

For the NAS-AS interaction to support the SDT operation, RAN2 has observed from previous CT1 reply LS that CT1 did not reach consensus regarding NAS-AS interactions for SDT. RAN2 assumption on the NAS-AS interaction is as follow:

RAN2 assumes the NAS-AS interaction for SDT is same as the interaction for the legacy NAS triggered RRC resume procedure, and the data transmission during the SDT procedure is up to UE implementation.

- Small Data Transmission (SDT) is a procedure allowing data and/or signalling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state).

- During an SDT session, the UE is kept in RRC_INACTIVE state, whilst exchanging UL and DL packets over radio bearers configured for SDT; the AS responds to the trigger from NAS layer only after the SDT session is completed or when SDT failure is detected. The AS does not indicate to upper layers whether the suspended RRC connection has been resumed before that.

- Upon unsuccessful completion of the SDT procedure, the UE transitions to RRC_IDLE.

- The data and signaling transmission during the SDT procedure is up to UE implementation.

RAN2 would like CT1 to check whether RAN2 assumption is feasible or whether there is any specification impact required (e.g. to the NAS/AS interaction) for SDT.

The agreed RAN2 stage-2 CR for SDT in R2-2204234 is attached for reference.

2. Actions:

To CT1 group:

Action: RAN2 respectfully asks CT1 to answer the following questions:

Q: Whether are the RAN2 assumptions as indicated above on the NAS-AS interaction are feasible from CT1 perspective or whether are there any specification impact required (e.g. to the NAS/AS interaction) for SDT?

3. Date of Next RAN2 Meetings:

TSG-RAN2 Meeting #118-e   16-27 May 2022	Electronic Meeting

TSG-RAN2 Meeting#119-e    22-26 Aug 2022	TBD