3GPP TSG-RAN WG2 Meeting #119-e		R2-220xxxx

Online, August 17-29, 2022

Agenda Item:	8.4.2.1

Source: 	MediaTek Inc.

Title:  	Report of [Post119-e][036][feMob] Time Chart

Document for:	Discussion and decision

# Introduction

In RAN2#119-e meeting, we discussed the latency and interruption of L1/L2-based inter-cell mobility objective of Rel-18 NR further mobility enhancement WI. Then we have the following post-meeting email discussion.

[Post119-e][036][feMob] Agreements, time chart, LS out (MediaTek) Scope: Capture WI agreements, Capture a mobility timing chart for L1L2 mobility, as a reference - include all pieces of procedures that may be optimized impacted FFS etc (acc to current agreements). LS out to RAN1 and RAN3 on the RAN2 progress, and ask to take into account. Intended outcome: Endorsed Report or Stage-2 CR with appendix etc, Approved LS out Deadline: Short (Can start before the meeting has ended).

In this document, we discuss the timing chart for L1/L2-based inter-cell mobility. The concluded timing chart will be captured as a reference in a running stage-2 CR or a report.

Related assumptions in Chair’s note are copied below.

Assumption: HO interruption time for L1/L2-based inter-cell mobility is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. FFS if TRS tracking after HO and CSI RS measurement should also be included, i.e. the time to use a high-performance beam (can be clarified further). Assumption: To reduce HO interruption time, investigate e.g. solutions to reduce the time for UE reconfiguration (already in the WID), downlink and uplink synchronization after handover decision (other parts of dynamic switch not precluded). R2 assumes that L2 is continued whenever possible (e.g. intra-DU), without Reset, with the target to avoid data loss, and the additional delay of data recovery. Measurement delay can/may be considered in this work

Contact information:

| Company | Name <email> |
| --- | --- |
| MediaTek | Li-Chuan TSENG <li-chuan.tseng@mediatek.com> |
| Huawei, HiSilicon | David Lecompte (david.lecompte@huawei.com) |
| Xiaomi | Yumin Wu (wuyumin@xiaomi.com) |

# Discussion

Based on procedure and latency analyses in companies’ contributions and online discussions, Rapporteur prepares the following time chart model, in an attempt to include components of mobility latency mentioned by companies. Notice that we do not intend to define any kind of delay requirements in RAN2; the purpose of this discussion is to have a reference model about the components that contribute to mobility latency, based on which we can study enhancements for mobility latency reduction.

Figure 1. Components of mobility latency for L1/L2-based inter-cell mobility (before enhancement)

The meaning of components is shown below.

| Component | Meaning | Value |
| --- | --- | --- |
| TRRC | Processing time for RRCReconfiguration carrying candidate configurations | Up to 10ms |
| Tprocessing,1 / Tprocessing,2 | Time for UE processing, before and after cell switch command, respectively. This may include L2/3 reconfiguration, RF retuning, baseband retuning, security update if needed, etc. | Up to 20ms |
| Tmeas | The time UE measures target cell (from target appears to cell switch command) |  |
| Tcmd | Time for processing L1/L2-command (HARQ and parsing) | Up to 5ms |
| Tsearch | Time required to search the target cell | 0ms (if cell is known) Up to 15ms (if cell is unknown) |
| TΔ | Time for fine tracking and acquiring full timing information | SMTC periodicity (typ. 20ms) |
| Tmargin | Time for SSB post-processing | Up to 2ms |
| TIU | interruption uncertainty in acquiring the first available PRACH occasion in the new cell | Typ. 15ms |
| TRAR | Time for RAR delay | Typ. 4ms |

Note: Tprocessing is divided into two parts if some processing can be done before cell switch command.

Definition of HO interruption

According to Chair’s note, HO interruption time for L1/L2-based inter-cell mobility is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. This is similar to the definitions used in previous works (e.g., TR 36.881 and Rel-16 DAPS). However, there is also an FFS: if TRS tracking after HO and CSI RS measurement should also be included, i.e., the time to use a high-performance beam. We first invite companies to comment on this FFS.

Q1: Should the time to use a high-performance beam be included in HO interruption time model?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | FFS | The intention of this email seems not to conclude the FFS part. Maybe we can just capture it as FFS and further update if needed in future meetings. |
| Xiaomi | Yes | Many companies are proposing the solution of avoiding RACH to the target cell. If the RACH is not used, the UE would anyway require the TCI switching time in order to switch to the target cell. We see no strong motivation of considering the TCI switching delay after the RACH, since the RACH procedure will provide a proper DL beam to the UE. |
| MediaTek | See comments | Our understanding is that with L1/L2 mobility, L1 measurement and tracking of RS corresponding to the target beam should be done before the cell switch command. Then if UE is to use a CSI-RS (high performance) beam, the CSI-RS measurement is covered in Tmeas, and TRS tracking /post-processing can be covered in TΔ and Tmargin. The exactly naming and value should be discussed in RAN4. From RAN2 perspective, our point is that we do not need to have separate components for SSB and CSI-RS measurement/tracking. |

The term “UE processing” considers the steps to configure the UE for target cell, such as L2/3 reconfiguration, RF retuning, baseband retuning, security update, etc. The exact steps may depend on the scenario (intra- vs. inter-frequency, intra- vs. inter-DU), as analyzed in [5]. We now discuss the details of UE processing time.

Q2: What steps are included in the time for UE processing? Please consider different scenarios.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | ASN.1 decoding and validity checking (in existing T38.331, TRRC only includes processing of the configuration to be applied immediately, e.g. in CHO, the UE is not required to decode and do validity checking within TRRC), L2/3 reconfiguration, baseband retuning, RF retuning |
| Xiaomi | We think that this “UE processing” is the same as Tprocessing defined in RAN4 TS 38.133. |
| MediaTek | L2/3 reconfigurations, baseband retuning, RF retuning. ASN.1 decoding and validity checking can also be considered if we mandate UE to do that right after receiving candidate configurations. |

In legacy handover delay requirements, the time for UE processing (Tprocessing) is considered after receiving handover command (see e.g., Clause 6.1.1 in TS 38.133). For L1/L2-based inter-cell mobility, we see some different views. For example, it is mentioned in [16] that UE may process and apply the configuration(s) for candidate target cells for L1/L2 based inter-cell mobility right away when this as received. In other words, UE processing in L1/L2-based inter-cell mobility may be done (partially) before or after cell switch command. To address this, in Figure 1, we divide the “Tprocessing” into “Tprocessing,1” and “Tprocessing,2”, capturing UE processing before and after cell switch command, respectively.

We now discuss how to model UE processing time in L1/L2-based mobility latency model. We see three options:

- Option 1:	UE processing happens after cell switch command, and is considered as a part of the interruption

- Option 2:	UE processing happens both before and after cell switch command, and only the latter part is considered as a part of the interruption

- Option 3:	UE processing happens before cell switch command, and is NOT considered as a part of the interruption

If Option 2 is preferred, we should also discuss which parts are done after cell switch command (i.e., included in handover interruption)

Q3: How should UE processing be modelled in L1/L2-based inter-cell mobility latency analysis? If Option 2 is preferred, please indicate which steps are done after cell switch command.

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option2 | We assume RRC ASN.1 decoding and validity check of the pre-configuration shall be before L1/L2 HO CMD, in order to reduce the interruption time. We expect this to be the dominant component of UE processing delay. Applying parameters is after the L1/L2 HO CMD. |
| Xiaomi | Option 1 | We think that “Tprocessing” as defined in RAN4 is for the RF retuning, which is difficult to be performed before the reception of the L1/L2 cell switching command, because the UE may only use single RF chain while switching from once cell to another. If companies consider to reduce the “Tprocessing”, we may need to ask RAN4 to evaluate the feasibility. |
| MediaTek | Option 2 | UE can actually do beyond RRC ASN.1 decoding and validity check before cell switch command if UE’s hardware/software supports that. For example, UE can configure RLC/MAC/PHY for candidate CG if it has two protocol stacks. |

Measurement delay

Chair’s note mentions that measurement delay may also be considered in this work. Rapporteur’s understanding (based on e.g., [10]) is that measurement delay means the time it takes for UE to perform measurement and reporting to trigger cell switch after a better cell (target) appears. Since it is before the cell switch command, it may not be a part of HO interruption, but it does contribute to the overall latency for UE to access a better cell.

Q4: How should measurement delay be considered in the illustration for components of mobility latency?

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | We could distinguish the following components - Time between "target cell appears" and "UE measures the target cell" - Time between "UE measures the target cell" and "UE reports the measurement" - Time between "UE reports the measurement "and "UE receives the L1/L2 handover command" We could add these steps on the figure. |
| Xiaomi | We agree with the Rapporteur that the measurement delay does contribute to the overall latency required for accessing the target cell, and we are open for solutions reducing such latency. However, while providing the analysis for interruption, we think that we could focus on the user plane interruption time, which does not include the measurement delay,. |
| MediaTek | Different kinds/parts of measurement delay may be shown in the illustration. But we need not to count measurement delay in any kind of formula for mobility latency, since the value varies a lot. The main purpose is to show that a too long measurement period may delay UE’s access towards a better cell. |

Finally, we’d like to know if the example analysis of components for mobility latency is agreeable, or any modification is needed.

Q5: Any suggestions for the analysis of components for mobility latency

| Company | Comments |
| --- | --- |

# Conclusion

It is proposed to discuss and decide on the following proposals:

# Reference

- R2-2206982	Target Performance Enhancements for L1L2-based Inter-cell Mobility	MediaTek Inc.	discussion

- R2-2206992	On the Target Performance Enhancements for L1L2 based Mobility	CATT	discussion	Rel-18	NR_mob_enh2-Core

- R2-2207315	NR mobility issues and goals for improvement	Futurewei	discussion	Rel-18	NR_mob_enh2-Core

- R2-2207340	L1/L2 Mobility – General Concepts and Configuration	Qualcomm Incorporated	discussion	Rel-18

- R2-2207380	Discussion on latency model of L1 L2 mobility	Intel Corporation	discussion	Rel-18	NR_mob_enh2-Core

- R2-2207407	Consideration on L1/L2 signalling based mobility	Fujitsu	discussion	Rel-18	NR_mob_enh2-Core

- R2-2207466	Latency reduction aspects of L2/L1 mobility	Apple	discussion	Rel-18	NR_mob_enh2-Core

- R2-2207496	Target scenario and latency reduction in L1/L2 based mobility	NEC	discussion	Rel-18	NR_mob_enh2-Core

- R2-2207537	Discussion on Dynamic switch mechanism among candidate serving cells	KDDI Corporation	discussion

- R2-2207637	L1/L2 mobility target performance enhancements	Huawei, HiSilicon	discussion	Rel-18	NR_mob_enh2-Core

- R2-2207655	Analysis of HO latency and possible enhancements for L1/L2 mobility	OPPO	discussion	Rel-18	NR_mob_enh2-Core

- R2-2207752	Discussion on basic model for L1 L2 mobility	vivo	discussion	Rel-18	NR_mob_enh2-Core

- R2-2207806	Latency Evaluation of L1 or L2 based mobility	Xiaomi	discussion	Rel-18	NR_mob_enh2-Core

- R2-2207857	Initial discussion of L1/L2 mobility	Sharp	discussion	Rel-18	NR_mob_enh2-Core

- R2-2208185	Target enhancements and latency model for L1/2 triggered handover	Interdigital, Inc.	discussion	Rel-18	NR_mob_enh2-Core

- R2-2208200	Latency analysis for L1/L2 based inter-cell mobility	Ericsson	discussion	Rel-18	NR_mob_enh2-Core

- R2-2208212	Prerequisites and benefits of Lower Layer Mobility	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_mob_enh2-Core

- R2-2208213	Basic details of Lower Layer L1/L2 Mobility	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_mob_enh2-Core

- R2-2208367	Discussion on L1 L2 mobility performance enhancement	ASUSTeK	discussion	Rel-16	NR_mob_enh2-Core

- R2-2208455	Initial considerations on L1L2 mobility	CMCC	discussion	Rel-18	NR_mob_enh2-Core

- R2-2208522	L1/L2 mobility scenarios and latency	LG Electronics	discussion	Rel-18

- R2-2208528	Scenario and Target Performance Enhancements for L1/L2 mobility	Samsung	discussion	NR_mob_enh2-Core

- R2-2208559	Initial Consideration on L1-L2 Signaling Based Mobility	ZTE Corporation,Sanechips	discussion	Rel-18	NR_mob_enh2-Core