3GPP TSG-RAN WG2 Meeting #119-e	R2-22xxxxx

Online, 17-26 August 2022

Source: 	Huawei, HiSilicon

Title:	Summary of [POST119-e][302][Sdata] UP open issues and CR to 38.321 (Huawei)

- Issues for the CR

| Company | Change1-17 | Issue in the Change | Propose TP |
| --- | --- | --- | --- |
| LG | Change5 | We think “if CG-SDT procedure is ongoing” and “if cg-SDT-TimeAlignmentTimer is running” are same conditions. The proposed text may cause misunderstanding that there is a case when the CG-SDT procedure is ongoing but the cg-SDT-TimeAlignmentTimer is not running [Nokia] Agree. | Remove one of “CG-SDT procedure is ongoing” or “cg-SDT-TimeAlignmentTimer is running”. We slightly prefer to remove the second one. “If CG-SDT procedure is ongoing” is sufficient. [Rapp] Corrected [Langbo] We don’t think “if CG-SDT procedure is ongoing” and “if cg-SDT-TimeAlignmentTimer is running” are same conditions. If one of the two conditions has to been deleted, we suggest to retain “cg-SDT-TimeAlignmentTimer is running” in order to avoid the potential risks. One example is: (1) cg-SDT-TimeAlignmentTimer expires (or not running) during the ongoing CG-SDT procedure; and then (2) UE performs 2-step RA as specified in clause 5.2 of TS38.321 "The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission when the cg-SDT-TimeAlignmentTimer is not running during the ongoing CG-SDT procedure......."; and then (3) a PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission is received by the UE; and then (4) the UE considers this Random Access procedure successfully completed according to the updated version v01; and then (5) hop to step (2) again due to the above mentioned specification in Clause 5.2 because cg-SDT-TimeAlignmentTimer is NOT started from step (1) to step (4). In addition, for the case “when the CG-SDT procedure is ongoing but the cg-SDT-TimeAlignmentTimer is not running”, the UE may expect an Absolute Timing Advance Command MAC CE to complete the Random Access. |
| LG | Change6 | “triggered as in clause 5.27” is not needed. [Nokia] Agree. | Remove “triggered as in clause 5.27”. “If CG-SDT procedure is ongoing” is sufficient. [Rapp] Corrected [Langbo] Agree with the updated version v01. |
| LG | Change4 | Same text as in Chang5 and Change6 is preferred. “if there is an on-going CG-SDT procedure as in clause 5.27”. [Nokia] Agree. | “If CG-SDT procedure is ongoing” is sufficient. [Rapp] Corrected [Langbo] Agree with the updated version v01. |
| LG | Change8 | To simplify the change, the cg-SDT-TAT can be added to the existing text. Then, new text can be removed. The retransmission is allowed only for initial CG-SDT transmission is clear from the procedure text. [Nokia] We prefer rapporteur’s new text to clarify this is only possible for re-tx of initial CG-SDT transmission. | “If cg-RetransmissionTimer or cg-SDT-RetransmissionTimer is configured, retransmissions with the same HARQ process may be performed on any configured grant configuration if the configured grant configurations have the same TBS.” [Rapp] Keep as it is [Langbo] agree with Rapp. |
| LG | Change13 | We think 8356 like change is better because the logicalChannelSR-DelayTimeer itself is a timer used for SDT. [Nokia] We would be OK with this approach as well, however, that will require changes also in the SR triggering part. So rapporteur’s proposal could be simpler. | Suggest 8356 like change. 1> else if BSR is triggered for a logical channel for which logicalChannelSR-DelayTimerApplied with value true is configured by upper layers and SDT procedure is on-going according to clause 5.27: 2> start or restart the sdt-LogicalChannelSR-DelayTimer. 1> else: 2> if running, stop the logicalChannelSR-DelayTimer. 2> if running, stop the sdt-LogicalChannelSR-DelayTimer. [Rapp] Keep as it is [Langbo] agree with Rapp. |
| LG | Change10 | “if configured” is redundant. | Remove “if configured”. [Rapp] This has been dicsused and agreed during online dicsussion. Check the agreement [Langbo] agree with Rapp. |
| LG | Change14 | We don’t see the need to add “if CG-SDT-TAT is running”. What’s the problem with considering the CG-SDT-TAT expired even if it is not running? [Nokia] Agree. | Undo the change. [Rapp] it is possible that CG_SDT is not configured at all on the selected UL carrier. Why should we consider cg-SDT-TAT expried when there is no configured CG-SDT at all? [Langbo] agree with Rapp. |
| Xiaomi | Change2 | “the HARQ process” is not clear, as the sentence does not mention whether “the initial CG-SDT transmission for CCCH message” is included in “the HARQ process”. | - cg-SDT-RetransmissionTimer: the duration after a configured grant (re)trasnmission of the initial CG-SDT transmission for CCCH message when the UE shall not autonomously retransmit the initial CG-SDT transmission for CCCH message. [Rapp] The current text aligns with the definition of cg-RT. So prefer to keep the current text cg-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process when the UE shall not autonomously retransmit that HARQ process [Langbo] Agree with the updated version v01. |
| Nokia | Change2 | Additional “of a HARQ process” as in the original timer text should be added to make the sentence clear. | cg-SDT-RetransmissionTimer: the duration after a configured grant (re)trasnmission of a HARQ process of the initial CG-SDT transmission for CCCH message when the UE shall not autonomously retransmit the HARQ process. [Rapp] corrected [Langbo] Agree with the updated version v01. |
| vivo | Change2 | In CG-SDT, the first initial trasnmission is not only for CCCH message, we prefer the the wording “the initial CG-SDT transmission with CCCH message”, aligning with the others in the MAC spec. | - cg-SDT-RetransmissionTimer: the duration after a configured grant (re)trasnmission of the initial CG-SDT transmission withfor CCCH message when the UE shall not autonomously retransmit the HARQ process. [Rapp] corrected [Langbo] Agree with the updated version v01. |

- Summary