3GPP TSG-RAN WG2 Meeting #120															R2-220xxxx

Toulouse, France, 14 – 18 November, 2022

Agenda item:	x.x.x

Source: 	Xiaomi

Title: 	(Draft) - Summary of [Post119-e][651][IDC] Comparison of TDM solutions (Xiaomi)

Document for:		Discussion

# 1.	Introduction

This paper is to trigger the following email discussion for IDC TDM solutions:

[Post119-e][651][IDC] Comparison of TDM solutions (Xiaomi)

Scope: Analyse the details of following TDM candidate solutions, and compare solutions , e.g. applied scenarios (e.g. BT voice, BT eSCO and WLAN beacon), complexity, etc;

- DRX solution;

- MUSIM gap like solution;

- UL and/or DL transmission occasion(s);

- Autonomous denial solution;

Intended outcome: Report to RAN2#120

Deadline:  Nov 3rd (Rapporteur may introduce intermediate deadlines, but no deadline during an inactive period, and no deadline in the period from Submisssion deadline to EOM of R2-119bis).

The guidance provided by the Chair is quoted below:

Extra Long email discussions after R2-119-e, for R2-120, Deadline: Nov 3rd Outcome tdocs for long email discussions shall be submitted to RAN2 120-e (Nov meeting). Please request tdoc numbers as for any other input tdoc to next meeting, i.e. by 3GU. NOTE that these discussions shall consider the duration of R2 119bis-e to be an inactive period (in addition to the general 3GPP inactive periods).

To facilitate the discussion as indicated above, the rapporteur would like to split the discussion into the following two phases. The intention is to understand how the candidate solution would work before comparing the solution with other candidates.

Phase-1 discussion: Scope: Analyse the details of the following TDM candidate solutions: - DRX solution; - MUSIM gap like solution; - UL and/or DL transmission occasion(s); - Autonomous denial solution; Intended outcome: The solution details (e.g. specification impacts) from the Phase-1 discussion are to be provided based on the companies’ feedbacks. The solution details provided via Phase-1 discussion will be used as the baseline for further polishment if the solution is adopted in the Rel-18 IDC. Deadline for the Phase-1 discussion: Sept 23rd 00:30 AM UTC.

Phase-2 discussion: The phase-2 discussion will be kicked off once the solutions details (e.g. specification impacts) provided from the Phase-1 discussion are relatively stable. Scope: Based on the solution details provided by the Phase-1 discussion, compare solutions, e.g. applied scenarios (e.g. BT voice, BT eSCO and WLAN beacon), complexity, etc. Selection of TDM solutions to be specified in Rel-18. Deadline for the Phase-2 discussion: Nov 3rd 00:30 AM UTC.

## 1.1	Contacts

Contact person for each participating company:

| Company | Name | Email Address |
| --- | --- | --- |
| Xiaomi | Yumin Wu | wuyumin@xiaomi.com |
| Ericsson | Henrik Enbuske | henrik.enbuske@ericsson.com |
| Intel | Yujian Zhang | yujian.zhang@intel.com |

# 2.	Phase-1 discussion

Phase-1 discussion: Scope: Analyse the details of the following TDM candidate solutions: - DRX solution; - MUSIM gap like solution; - UL and/or DL transmission occasion(s); - Autonomous denial solution; Intended outcome: The solution details (e.g. specification impacts) from the Phase-1 discussion are to be provided based on the companies’ feedbacks. The solution details provided via Phase-1 discussion will be used as the baseline for further polishment if the solution is adopted in the Rel-18 IDC. Deadline for the Phase-1 discussion: Sept 23rd 00:30 AM UTC.

## 2.1 DRX solution

The LTE DRX solution is quoted as follows:

Procedure: Step 1: The eNB indicates whether the IDC reporting for TDM assistance information is allowed. Step 2: When detecting the IDC issue, the UE reports the DRX assistance information for the affected frequencies. The TDM assistance information uses the MCG as timing reference. The DRX assistance information includes: drx-CycleLength, drx-Offset and drx-ActiveTime in the granularity of subframe.

ASN.1 signaling: Step 1: Network configuration idc-Indication-r11 ENUMERATED {setup} OPTIONAL, -- Need OR Step 2: UE reporting signaling drx-AssistanceInfo-r11 SEQUENCE { drx-CycleLength-r11 ENUMERATED {sf40, sf64, sf80, sf128, sf160, sf256, spare2, spare1}, drx-Offset-r11 INTEGER (0..255) OPTIONAL, drx-ActiveTime-r11 ENUMERATED {sf20, sf30, sf40, sf60, sf80, sf100, spare2, spare1} },

The candidate DRX solution for NR is as follows:

Option 1: (By using the LTE DRX solution as the baseline)

Procedure: Step 1: The gNB indicates whether the IDC reporting for TDM assistance information is allowed. Step 2: When detecting the IDC issue, the UE reports the DRX assistance information for the affected frequencies. The TDM assistance information uses the MCG as timing reference. The DRX assistance information includes: drx-LongCycleStartOffset including drx-LongCycle and drx-StartOffset in the granularity of ms, drx-SlotOffset in the granularity of 1/32 ms (subMilliSeconds), and drx-ActiveTime in the granularity of ms or 1/32 ms (subMilliSeconds).

ASN.1 signaling example: Step 1: Network configuration idc-TDM-Indication-r18 ENUMERATED {setup} OPTIONAL, -- Need R Step 2: UE reporting signaling DRX-AssistanceInfo-r18 ::= SEQUENCE { drx-LongCycleStartOffset CHOICE { ms10 INTEGER(0..9), ms20 INTEGER(0..19), ms32 INTEGER(0..31), ms40 INTEGER(0..39), ms60 INTEGER(0..59), ms64 INTEGER(0..63), ms70 INTEGER(0..69), ms80 INTEGER(0..79), ms128 INTEGER(0..127), ms160 INTEGER(0..159), ms256 INTEGER(0..255), ms320 INTEGER(0..319), ms512 INTEGER(0..511), ms640 INTEGER(0..639), ms1024 INTEGER(0..1023), ms1280 INTEGER(0..1279), ms2048 INTEGER(0..2047), ms2560 INTEGER(0..2559), ms5120 INTEGER(0..5119), ms10240 INTEGER(0..10239) }, drx-SlotOffset INTEGER (0..31), drx-ActiveTime-r18 CHOICE { subMilliSeconds INTEGER (1..31), milliSeconds ENUMERATED {ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200, ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } } }

#### Task 1: Please provide your suggested modification for above Option 1 or other options for the DRX solution for NR.

(Rapporteur’s comment: Please provide the changes for the above option or other options in the “Comments” column. The ASN.1 signaling example provided above is to be polished further while drafting the CR. Companies can provide FFS for specific technical points. However the increasing number of FFS(s) could also cause the dropping of the solution due to the very limted TU for the Rel-18 IDC work.)

| Company | Comments |
| --- | --- |
| ZTE | Generally agree, but we don’t think the slot offset with 1/32ms granularity is necessary. |
| Ericsson | It seems that the rapporteur has simply copied from the IE DRX-Config, and removed field which the UE shall not have an opinion about. This looks like a decent starting-point. |
| Intel | Generally OK with Option 1. For the UE report signalling, our understanding is that we can use the values in LTE signalling (e.g. DRX cycle length in the range of 40 ms ~ 256 ms) as baseline. The reason is that the LTE signalling values are selected based on coexistence scenarios (clause 4.2 of TR 36.816), and were agreed in email discussion “[79#33] [LTE/IDC] IDC Open issues” (R2-124404). We also agree with ZTE that slot offset is not necessary. |

## 2.2 MUSIM gap like solution

The Rel-17 MUSIM gap solution is quoted as follows:

Procedure: Step 1: The gNB indicates whether the MUSIM reporting for gap assistance information is allowed. And a prohibit timer is provided for gap assistance information . Step 2: When detecting the need for gap for MUSIM, the UE reports its preferred gap configuration when the prohibit timer is not running. If the UE prefers periodic gap, the preferred gap configuration includes musim-GapPreferenceList with an entry for each periodic gap. Each peridodic gap configuration includes musim-GapLength and musim-GapRepetitionAndOffset including the gap repetition period and the gap offset in the granularity of ms. If the UE prefers aperiodic gap, the preferred gap configuration includes musim-GapPreferenceList with an entry for each aperiodic gap. Each aperidodic gap configuration includes musim-GapLength in the granularity of ms and musim-Starting-SFN-AndSubframe including starting-SFN in the granularity of SFN and startingSubframe in the granularity of subframe.

ASN.1 signaling: Step 1: Network configuration MUSIM-GapAssistanceConfig-r17 ::= SEQUENCE { musim-GapProhibitTimer-r17 ENUMERATED {s0, s0dot1, s0dot2, s0dot3, s0dot4, s0dot5, s1, s2, s3, s4, s5, s6, s7, s8, s9, s10} } Step 2: UE reporting signaling MUSIM-GapPreferenceList-r17 ::= SEQUENCE (SIZE (1..4)) OF MUSIM-GapInfo-r17 MUSIM-GapInfo-r17 ::= SEQUENCE { musim-Starting-SFN-AndSubframe-r17 MUSIM-Starting-SFN-AndSubframe-r17 OPTIONAL, -- Cond aperiodic musim-GapLength-r17 ENUMERATED {ms3, ms4, ms6, ms10, ms20} OPTIONAL, -- Need S musim-GapRepetitionAndOffset-r17 CHOICE { ms20-r17 INTEGER (0..19), ms40-r17 INTEGER (0..39), ms80-r17 INTEGER (0..79), ms160-r17 INTEGER (0..159), ms320-r17 INTEGER (0..319), ms640-r17 INTEGER (0..639), ms1280-r17 INTEGER (0..1279), ms2560-r17 INTEGER (0..2559), ms5120-r17 INTEGER (0..5119), ... } OPTIONAL -- Cond periodic } MUSIM-Starting-SFN-AndSubframe-r17 ::= SEQUENCE { starting-SFN-r17 INTEGER (0..1023), startingSubframe-r17 INTEGER (0..9) }

The candidate MUSIM gap like solution for NR is as follows:

Option 1: (By using the Rel-17 MUSIM gap solution as the baseline)

Procedure: Step 1: The gNB indicates whether the IDC reporting for gap assistance information is allowed. And a prohibit timer is provided for gap assistance information. Step 2: When detecting the IDC issue, the UE reports its preferred gap configuration for the affected frequencies when the prohibit timer is not running. The TDM assistance information uses the MCG as timing reference. If the UE prefers periodic gap, the preferred gap configuration includes idc-GapPreferenceList with an entry for each periodic gap. Each peridodic gap configuration includes idc-GapLength and idc-GapRepetitionAndOffset including the gap repetition period and the gap offset in the granularity of ms. If the UE prefers aperiodic gap, the preferred gap configuration includes idc-GapPreferenceList with an entry for each aperiodic gap. Each aperidodic gap configuration includes idc-GapLength in the granularity of ms and idc-Starting-SFN-AndSubframe including starting-SFN in the granularity of SFN and startingSubframe in the granularity of subframe.

ASN.1 signaling example: Step 1: IDC-GapAssistanceConfig-r17 ::= SEQUENCE { musim-GapProhibitTimer-r17 ENUMERATED {s0, s0dot1, s0dot2, s0dot3, s0dot4, s0dot5, s1, s2, s3, s4, s5, s6, s7, s8, s9, s10} } Step 2: UE reporting signaling IDC-GapPreferenceList-r17 ::= SEQUENCE (SIZE (1..4)) OF IDC-GapInfo-r17 IDC-GapInfo-r17 ::= SEQUENCE { idc-Starting-SFN-AndSubframe-r17 IDC-Starting-SFN-AndSubframe-r17 OPTIONAL, -- Cond aperiodic idc-GapLength-r17 ENUMERATED {ms3, ms4, ms6, ms10, ms20} OPTIONAL, -- Need S idc-GapRepetitionAndOffset-r17 CHOICE { ms20-r17 INTEGER (0..19), ms40-r17 INTEGER (0..39), ms80-r17 INTEGER (0..79), ms160-r17 INTEGER (0..159), ms320-r17 INTEGER (0..319), ms640-r17 INTEGER (0..639), ms1280-r17 INTEGER (0..1279), ms2560-r17 INTEGER (0..2559), ms5120-r17 INTEGER (0..5119), ... } OPTIONAL -- Cond periodic } IDC-Starting-SFN-AndSubframe-r17 ::= SEQUENCE { starting-SFN-r17 INTEGER (0..1023), startingSubframe-r17 INTEGER (0..9) }

#### Task 2: Please provide your suggested modification for above Option 1 or other options for the MUSIM gap like solution for IDC.

(Rapporteur’s comment: Please provide the changes for the above option or other options in the “Comments” column. The ASN.1 signaling example provided above is to be polished further while drafting the CR. Companies can provide FFS for specific technical points. However the increasing number of FFS(s) could also cause the dropping of the solution due to the very limted TU for the Rel-18 IDC work.)

| Company | Comments |
| --- | --- |
| ZTE | Generally OK with the ASN.1 structure in the Step 2, as Rapporteur said, the detail values can be further polished. For the step 1, it’s about whether a prohibit timer is needed for the IDC reporting, we think it can be discussed as a separate topic. |
| Ericsson | Unclear if any change is needed since this solution needs more stage-2 type of discussion. The MUSIM-gap solution requires additional discussion to evaluate required gap-lenghts and periodicities as they have now been defined for another use case, including RAN4 work. It is also unclear to us if/how the gap prohibit timer should be applied. More study/discussion would be needed if the gap-solution should be adopted. But more importantly, we are not sure whether this solution adds anything benefit beyond DRX. Is it not just yet another time-pattern? |
| Intel | Similar to comment to Task 1 (DRX), the values of gap period (repetition) and length need to be aligned with the LTE DRX values as the value range is related to the coexistence scenarios. We don’t think a prohibit timer is needed. In LTE IDC, there was discussion on whether a prohibit timer is needed or not and the final decision was that prohibit timer is not needed. In addition, there is no prohibit timer for NR Rel-16 IDC with FDM solution. |

## 2.3 UL and/or DL transmission occasion(s)

The LTE HARQ reservation solution is quoted as follows:

Procedure: Step 1: The eNB indicates whether the IDC reporting for TDM assistance information is allowed. Step 2: When detecting the IDC issue, the UE reports the desired subframe reservation patterns for the affected frequencies. The TDM assistance information uses the MCG as timing reference. The desired subframe reservation patterns is included in idc-SubframePatternList. A bitmap with value 0 indicates that E-UTRAN is requested to abstain from using the subframe. For FDD, the radio frame in which the pattern starts (i.e. the radio frame in which the first/leftmost bit of the subframePatternFDD corresponds to subframe #0) occurs when SFN mod 2 = 0. For TDD, the first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10.

ASN.1 signaling: Step 1: Network configuration idc-Indication-r11 ENUMERATED {setup} OPTIONAL, -- Need OR Step 2: UE reporting signaling IDC-SubframePatternList-r11 ::= SEQUENCE (SIZE (1..maxSubframePatternIDC-r11)) OF IDC-SubframePattern-r11 IDC-SubframePattern-r11 ::= CHOICE { subframePatternFDD-r11 BIT STRING (SIZE (4)), subframePatternTDD-r11 CHOICE { subframeConfig0-r11 BIT STRING (SIZE (70)), subframeConfig1-5-r11 BIT STRING (SIZE (10)), subframeConfig6-r11 BIT STRING (SIZE (60)) }, ... }

The candidate UL and/or DL transmission occasion solution for NR is as follows:

Option 1: (By using the LTE HARQ reservation solution as the baseline)

Procedure: Step 1: The gNB indicates whether the IDC reporting for TDM assistance information is allowed. Step 2: When detecting the IDC issue, the UE reports the desired subframe reservation patterns for the affected frequencies. The TDM assistance information uses the MCG as timing reference. The desired subframe reservation patterns is included in idc-SubframePatternList. A bitmap with value 0 indicates that NR is requested to abstain from using the subframe. For FDD, the radio frame in which the pattern starts (i.e. the radio frame in which the first/leftmost bit of the subframePatternFDD corresponds to subframe #0) occurs when SFN mod 2 = 0. For TDD, the UE indicates its preferredTDD-UL-DL-Pattern with referenceSubcarrierSpacing, and its desired subframe reservation pattern preferredSubframePatternTDD for the preferredTDD-UL-DL-Pattern. The first/leftmost bit of preferredSubframePatternTDD corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10.

ASN.1 signaling example: Step 1: Network configuration idc-TDM-Indication-r18 ENUMERATED {setup} OPTIONAL, -- Need R Step 2: UE reporting signaling IDC-SubframePatternList-r18 ::= SEQUENCE (SIZE (1..maxSubframePatternIDC-r18)) OF IDC-SubframePattern-r18 IDC-SubframePattern-r18 ::= CHOICE { subframePatternFDD-r18 BIT STRING (SIZE (4)), subframePatternTDD-r18 SubframePatternTDD-r18, ... } SubframePatternTDD-r18 ::= SEQUENCE { referenceSubcarrierSpacing-r18 SubcarrierSpacing, preferredTDD-UL-DL-Pattern-r18 TDD-UL-DL-Pattern, preferredSubframePatternTDD-r18 BIT STRING (SIZE (1..70)) } TDD-UL-DL-Pattern ::= SEQUENCE { dl-UL-TransmissionPeriodicity ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10}, nrofDownlinkSlots INTEGER (0..maxNrofSlots), nrofDownlinkSymbols INTEGER (0..maxNrofSymbols-1), nrofUplinkSlots INTEGER (0..maxNrofSlots), nrofUplinkSymbols INTEGER (0..maxNrofSymbols-1), ..., [[ dl-UL-TransmissionPeriodicity-v1530 ENUMERATED {ms3, ms4} OPTIONAL -- Need R ]] }

#### Task 3: Please provide your suggested modification for above Option 1 or other options for the UL and/or DL transmission occasion solution for NR.

(Rapporteur’s comment: Please provide the changes for the above option or other options in the “Comments” column. The ASN.1 signaling example provided above is to be polished further while drafting the CR. Companies can provide FFS for specific technical points. However the increasing number of FFS(s) could also cause the dropping of the solution due to the very limted TU for the Rel-18 IDC work.)

| Company | Comments |
| --- | --- |
| ZTE | We know that the down selection will be done in Phase 2, but we still don’t think there is a need to further discuss such HARQ based solution in NR, for that the NR take the flexible timing which is quite different from the LTE. Furthermore, as in the 36816, the HARQ based solution is mainly for the LTE+BT cases. However DRX-based scheme can also work for these cases, so we prefer to focus on DRX(or MUSIM) based scheme. Back to the option 1: For the FDD, In the LTE, the k1=4, so there is a 4 bits bitstring, in NR, K1 is configured by network, so we don’t think it’s reasonable to set size as 4. subframePatternFDD-r18 BIT STRING (SIZE (4)), For the TDD, in LTE the 70bits is adopted for the LTE TDD pattern 0,while for the pattern 6, 60bits is adopted, preferredSubframePatternTDD-r18 BIT STRING (SIZE (1..70)) LTE: subframeConfig0-r11 BIT STRING (SIZE (70)), subframeConfig1-5-r11 BIT STRING (SIZE (10)), subframeConfig6-r11 BIT STRING (SIZE (60)) so it seems hard to set bits size for the different DL/UL subframe configurations in the NR for both the TDD and FDD. |
| Ericsson | In our understanding we cannot simply reuse this solution from LTE due to in NR we have symbols instead of subframes, many and dynamic TDD patterns, a different HARQ process use, etc. We would be concerned in the amount of work this would bring compared to the DRX-solution. Also, for the gNB to actually respect this pattern it would impact the scheduler. With the DRX-solution on the other hand, it is already built-in to the DRX feature that the gNB does not schedule the UE outside active time, aside from ensuring that the inactivity timer is not extended in to the time when the UE is experiencing IDC issues. |
| Intel | First of all, we’d like to note that DRX assistance information as discussed in Task 1 has coarse granularity, and is not suitable for coexistence use case of BT voice, which requires finer granularity (that’s why HARQ process reservation solution was used in LTE in addition to DRX assistance information). Subframe pattern in LTE (as well as the Option 1 in this section) might not be directly applicable in NR since it is based on synchronous HARQ, while NR HARQ is asynchronous. In LTE, subframe pattern length is related to FDD/TDD and TDD UL-DL configuration. For example, although the periodicity of TDD UL-DL configuration is 10 ms, the subframe pattern length is 70 ms due to UL synchronous HARQ. In NR, HARQ is asynchronous. Therefore there is no need to report TDM pattern based on synchronous HARQ operation. In NR TDD, the pattern periodicity for IDC can be related to the TDD UL-DL transmission periodicity (dl-UL-TransmissionPeriodicity). A simple TDM pattern with fine granularity (compared with DRX assistance information) can be considered. For example, the pattern periodicity for NR IDC can be a single value (e.g. 20 ms) irrespective of the actual TDD UL-DL configuration since TDD transmission periodicity is a divisor of 20 ms. The pattern periodicity for FDD can the same as that of TDD, e.g. 20 ms. For each bit indicated in the pattern, the pattern unit can be subframe (ms), i.e. each bit in the pattern indicates whether NG-RAN is requested to abstain from using the subframe (ms). An example ASN.1 signaling for UE assistance information is as follows: IDC-SubframePatternList-r18 ::= SEQUENCE (SIZE (1..maxSubframePatternIDC-r18)) OF IDC-SubframePattern-r18 IDC-SubframePattern-r18 ::= BIT STRING (SIZE (1..20) |

## 2.4 Autonomous denial solution

The LTE autonomous denial solution is quoted as follows:

Procedure: Step 1: The eNB provides the autonomousDenialParameters including autonomousDenialSubframes and autonomousDenialValidity in the granularity of subframe. Step 2: The UE is allowed to deny any transmission in a particular UL subframe if during the number of subframes indicated by autonomousDenialValidity, preceeding and including this particular subframe, it autonomously denied fewer UL subframes than indicated by autonomousDenialSubframes

ASN.1 signaling: Step 1: Network configuration autonomousDenialParameters-r11 SEQUENCE { autonomousDenialSubframes-r11 ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1}, autonomousDenialValidity-r11 ENUMERATED { sf200, sf500, sf1000, sf2000, spare4, spare3, spare2, spare1} } OPTIONAL, -- Need OR

The candidate autonomous denial solution for NR is as follows:

Option 1: (By using the LTE autonomous denial solution as the baseline)

Procedure: Step 1: The gNB provides the autonomousDenialParameters including autonomousDenialSubframes and autonomousDenialValidity in the granularity of subframe. Step 2: The UE is allowed to deny any transmission in a particular UL subframe if during the number of subframes indicated by autonomousDenialValidity, preceeding and including this particular subframe, it autonomously denied fewer UL subframes than indicated by autonomousDenialSubframes

ASN.1 signaling example: Step 1: Network configuration autonomousDenialParameters-r18 SEQUENCE { autonomousDenialSubframes-r18 ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1}, autonomousDenialValidity-r18 ENUMERATED { sf200, sf500, sf1000, sf2000, spare4, spare3, spare2, spare1} } OPTIONAL, -- Need R

#### Task 4: Please provide your suggested modification for above Option 1 or other options for the autonomous denial solution for NR.

(Rapporteur’s comment: Please provide the changes for the above option or other options in the “Comments” column. The ASN.1 signaling example provided above is to be polished further while drafting the CR. Companies can provide FFS for specific technical points. However the increasing number of FFS(s) could also cause the dropping of the solution due to the very limted TU for the Rel-18 IDC work.)

| Company | Comments |
| --- | --- |
| ZTE | Generally Ok with the Option 1 if autonomous denial for the NR was supported |
| Ericsson | The RAN2 signalling part of this solution looks OK. One can of course discuss the granularity of this solution, e.g. should the validity-period/number of subframes be as proposed needs further checking. However, we expect that the autonomous denial solution alone cannot solve all possible problems. This would require significant RAN4 work and it is unclear if this solution would bring additional gains (e.g. compared to DRX) which would justify this enhancement. |
| Intel | Generally OK with Option 1 if autonomous denial will be supported for RAN2. We understand that some details will be discussed in later stage, e.g. the unit in configuration (current UL subframe is used, however there is no concept of UL subframe in NR). |

# 2.	Phase-2 discussion

Phase-2 discussion: The phase-2 discussion will be kicked off once the solutions details (e.g. specification impacts) provided from the Phase-1 discussion are relatively stable. Scope: Based on the solution details provided by the Phase-1 discussion, compare solutions, e.g. applied scenarios (e.g. BT voice, BT eSCO and WLAN beacon), complexity, etc. Selection of TDM solutions to be specified in Rel-18. Deadline for the Phase-2 discussion: Nov 3rd 00:30 AM UTC.

TBD…

# 3.	Conclusion

TBD…

# 4.	Reference

- R2-2207379	TDM Assistance Information for IDC	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_IDC_Enh-Core

- R2-2207718	TDM solution for IDC problem	Lenovo	discussion	Rel-18

- R2-2207805	Candidate TDM solutions for IDC	Xiaomi	discussion	Rel-18	NR_IDC_Enh-Core

- R2-2207845	Discussion on TDM solution for in-device co-existence interference avoidance	Samsung	discussion	Rel-18	NR_IDC_Enh-Core

- R2-2207937	Discussion on TDM solution in IDC	Apple	discussion	Rel-18	NR_IDC_Enh-Core

- R2-2207969	TDM solution for IDC	Intel Corporation	discussion	Rel-18	NR_IDC_Enh-Core

- R2-2208113	TDM Solution for NR IDC	Ericsson	discussion	Rel-18	NR_IDC_Enh-Core

- R2-2208118	TDM Solutions in IDC	Qualcomm Incorporated	discussion	Rel-18

- R2-2208231	Discussion on TDM solution for NR IDC	Huawei, HiSilicon	discussion	Rel-18	NR_IDC_Enh-Core

- R2-2208397	Discussion on TDM solution for IDC	vivo	discussion	Rel-18	NR_IDC_Enh-Core

- R2-2208525	IDC TDM solution	LG Electronics	discussion	Rel-18

- R2-2207161	Clarification on the IDC scope	ZTE Corporation, Sanechips	discussion	Rel-18	NR_IDC_Enh-Core

- R2-2208952, Xiaomi, "Summary of [AT119-e][652][IDC] TDM solution (Xiaomi)"