3GPP TSG-RAN WG2 Meeting #121bis-e	R2-230xxxx

Online, April 17 – 26, 2023

Title:	[draft] LS to SA3 on security for L2 UE-to-UE relay

Response to:

Release:	Rel-18

Work Item:	NR_SL_relay_enh, FS_5G_ProSe_Ph2

Source:	Lenovo (to be RAN2)

To:	SA3

Cc:

Contact Person:

#### Name:           Lianhai Wu

E-mail Address:   wulh5@lenovo.com

1. Overall Description:

End-to-end PC5 RRC connection between source remote UE and target remote UE is supported for L2 UE-to-UE relay operation in Rel-18. In legacy NR sidelink communication, LSB 5 bits of LCID as specified in TS 38.321 is used as input for ciphering and deciphering function at PDCP layer of Sidelink UE. However, in L2 UE-to-UE relay, LCIDs may be different in the first hop between source remote UE and L2 UE-to-UE relay UE and the second hop between L2 UE-to-UE relay UE and target remote UE.

Therefore, RAN2 has a working assumption in RAN2#121bis that end-to-end bearer ID (i.e., configuration index in the list of SLRB configurations) is used as input for the L2 U2U relay ciphering and deciphering at PDCP. RAN2 would like to ask SA3 to confirm if the above working assumption is feasible.

2. Actions:

To SA3:

ACTION: 	RAN2 respectfully asks SA3 to confirm the feasibility of using the configuration index of end-to-end bearer as input to security and provide the feedback.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #122		22-26 May 2023			Incheon, KR

TSG-RAN2 Meeting #123		21-25 August 2023			Toulouse, FR