3GPP TSG RAN WG2#121-bis-e	R2-230xxxx

Online, 17th – 26th April, 2023

# Title:	DRAFT LS to RAN1 on multicast reception in RRC_INACTIVE

# Response to:	-

# Release:	Release 18

Work Item:	     NR_MBS_enh-Core

Source:	Apple (to be RAN2)

To:	RAN1

Cc:	-

Contact Person:

Name:	Fangli XU

E-mail Address:	fangli_xu@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

For Rel-18 MBS enhancement, RAN2 has discussed multicast reception in RRC_INACTIVE and made several agreements (chair notes can be found in R2-2304207).

RAN2 would like to inform RAN1 of the following  which may be relevant to RAN1:

RAN2#121bis-e agreements: - CFR for multicast reception in RRC_INACTIVE - From the location&bandwidth and SCS configuration perspective, follow R17 MBS broadcast CFR principle (i.e. case A,C,E) to provide multicast CFR configuration in RRC_INACTIVE. - Multicast CFR in RRC_INACTIVE and broadcast CFR can be configured differently. FFS whether we need to restrict that one CFR is completely contained within the other in this case (we should understand what the issue is otherwise). - Case B and case D are not supported for multicast CFR in RRC_INACTIVE; - Whether multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different is up to NW implementation. FFS whether this causes some issues which need to be addressed. - The same CFR is used for multicast MCCH and MTCH. It can be revisited if there is any issue found, e.g. for RedCap UEs. - HARQ Operation - HARQ feedback related information in the DCI is not needed or can be ignored for multicast transmission to RRC_INACTIVE UE. - The HARQ operation for multicast reception in RRC_INACTIVE is same as the operation without HARQ feedback in RRC_CONNECTED state. - The multicast transmission in RRC_INACTIVE is performed via beam sweeping based on SSB index like broadcast MBS (i.e. beam information is not need in DCI). - For MTCH, RAN2 assumes to reuse the same DCI format of R17 multicast (i.e. DCI format 4-1/4-2) for dynamic scheduling of multicast in RRC INACTIVE. RAN2 assumes for MCCH scheduling, DCI format 4-0 is used. We will ask RAN1 to confirm whether it is feasible and whether both 4-1 and 4-2 are needed.

Additionally, RAN2 has made some assumptions on aspects relevant to RAN1 and would like to check RAN1 views on the following for multicast reception in RRC_INACTIVE:

- Question 1: RAN1 to confirm whether the following RAN2 assumption is feasible. If feasible, whether both DCI format 4-1 and 4-2 are needed?

  - For MTCH, RAN2 assumes to reuse the same DCI format of R17 multicast (i.e. DCI format 4-1/4-2) for dynamic scheduling of multicast in RRC INACTIVE. RAN2 assumes for MCCH scheduling, DCI format 4-0 is used.

- Question 2: RAN1 to confirm whether the following RAN2 understanding is correct.

  - RAN2 understanding that PDSCH aggregation is supported for multicast MTCH in RRC_INACTIVE (as that is supported in Rel-17 multicast MTCH in RRC_CONNECTED).

- Question 3: RAN1 to check the feasibility of reusing Rel-17 CSS for multicast MTCH and MCCH in RRC_INACTIVE:

  - 3.1) reusing the same CSS for multicast MTCH in RRC_INACTIVE (same as multicast MTCH in RRC_CONNECTED);

  - 3.2) separate CSS for MCCH and MTCH.

2. Actions:

To RAN1

ACTION:	RAN2 respectfully asks RAN1 to take the agreements into consideration for the future work and provide responses to the above questions.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2#122                      2023-05-22 to 2023-05-26		Incheon, KR

TSG-RAN WG2#123                      2023-08-21 to 2023-08-25		Toulouse, FR