3GPP TSG-RAN WG2 #123-bis	R2-23xxxxx

Xiamen, P.R. China, October 9th – 13th 2023

Agenda Item:	7.16.2.2

Source:	Ericsson

Title:	[Post123][059][AIML] Data Collection (Ericsson)

Document for:	Discussion, Decision

# Introduction

This document is to address the following email discussion:

[Post123][059][AIML] Data Collection (Ericsson)

Scope: Attempt to converge to agreements on outcome of discussion of R2-2308898, to have consolidated agreements.

Intended outcome: Report with agreeable proposals (agreeable as far as possible).

Deadline: Long

Let us observe that R2-2308898 [1] mainly focuses on CSI feedback enhancement/Beam management uses cases, and on one-sided models. Hence, since the scope of this email discussion is to consolidate proposals in that document, in this email discussion we only address issues related to CSI feedback enhancement/Beam management and one-sided models.

Related to R2-2308898, the following was captured in RAN2#123’s Chair Notes:

Chair: The proposals below are almost agreeable. It is a narrowing proposal (more specific than the physical entity mapping agreed) and is a reasonable baseline for further work: Proposal 1 For training of NW-side models, RAN2 prioritizes discussion on the suitability of data collection frameworks for gNB-centric data collection. Proposal 2 For training of NW-side models, the gNB-centric data collection implies that the gNB configures the UE to transfer data and initiates/terminates a data transferring session. Proposal 3 For training of NW-side models, RAN2 evaluates the suitability of data collection frameworks for OAM-centric data collection Proposal 4 For training of NW-side models, the OAM-centric data collection implies that the OAM initiates and terminates the data collection from the UE. Proposal 5 If feasibility of OAM-centric data collection for NW-side models is assessed by RAN1, RAN2 considers enhancements to logged MDT, such as logging measurements in RRC Connected mode. Proposal 6 For gNB-centric data collection for NW-side model, RAN2 to study a L3 data collection framework that allows the UE to measure and store a set of measurements (details up to RAN1) to be reported to the gNB upon request. Proposal 7 For NW-side performance monitoring, RAN2 waits for RAN1 input on the need to enhance the L1 reporting configuration or the L3 RRC measurement configuration and reporting. FFS Proposal 8 For UE-side model training, RAN2 considers (subject to RAN1 progress), the UE Assistance Information framework as a tool for the UE to request aid from the network in training at the UE. Proposal 9 For UE-side performance monitoring at NW side, RAN2 to focus on impacts in layer-2, or layer-3 (possibly including some layer-1 related measurements) for reporting of the outcome of performance monitoring (e.g. performance monitoring results, (non)applicability of AIML functionality). Layer-1 details are left to RAN1. FFS Proposal 10 The need of any enhancements to non-RAN data collection frameworks for UE-side models should be studied in SA WGs. Proposal 11 For CSI/beam management use cases, RAN2 to agree to Table 1 in Annex A which maps LCM functions to the various existing data collection frameworks considering; the sidedness of the model, and the entity terminating/initiating the data collection.

Regarding data collection, the following agreements/observations have been captured so far in Chair Notes:

From RAN2#121-bis: Observation: RAN2 may need to consider enhancements for AIML to existing functionality for data collection, e.g. for timing control (e.g. for MDT/RRM). From RAN2#122: RAN 2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios, e.g. (not exhaustive): - - For model inference of UE-sided model, input data for model inference is available inside the UE. - - For UE-side (real time) monitoring of UE-sided model, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW. P4a: For the latency requirement of data collection, RAN2 assumes: - - for all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection - - for model inference, when required data comes from other entities, there is a latency requirement for data collection - - for model monitoring, when required monitoring data (e.g., performance metric) comes from the other entities, there is a latency requirement for data collection. P6a: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement on the non-connected state can be revisited when needed. P5a: For the data generation entity and termination entity deployed at different entities, RAN2 assumes: - For CSI enhancement and beam management use cases: - - For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server. - - For NW-sided model inference, input data can be generated by UE and terminated at gNB. - - For UE-side model inference, input data/assistance information can be generated by gNB and terminated at UE. - - For model monitoring at NW side, performance metrics can be generated by UE and terminated at gNB. - For positioning enhancement use case: - - For model training, training data can be generated by UE/gNB and terminated at LMF/OTT server. - - For NW-sided model inference, input data can be generated by UE/gNB and terminated at LMF and/or gNB. - - For UE-side model inference, input data/assistance information can be generated by LMF/gNB and terminated at the UE. - - For model monitoring at NW side, performance metrics can be generated by UE/gNB and terminated at LMF.

Further the above assumptions were clarified/amended by RAN1 in the LS reply available in R1-2308730:

From RAN1 LS reply in R1-2308730: RAN1 discussed Assumption 4 of Part A for CSI compression, CSI prediction, Beam management and positioning use case separately and made following clarification for each use case based on the Assumption 4 of Part A of RAN2 LS. - For CSI compression enhancement and beam management use case: - For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server - For NW-sided model inference NW-part of two-sided model inference, input data can be generated by UE and terminated at gNB. - For UE-side model inference UE-part of two-sided model inference, input data is internally available at UEinput data/assistance information can be generated by gNB and terminated at UE. - For model performance monitoring at the NW side, calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by UE and terminated at gNB. - For CSI prediction enhancement and beam management use case: - For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server. - For NW-sided model inference, input data can be generated by UE and terminated at gNB. - For UE-side model inference, input data/assistance information is internally available at UE can be generated by gNB and terminated at UE. - For performancemodel monitoring at the NW side, calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by UE and terminated at gNB. - For CSI enhancement and beam management use case: - For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server. - For NW-sided model inference, input data can be generated by UE and terminated at gNB. - For UE-side model inference, input data/assistance information is internally available at UE. can be generated by gNB and terminated at UE. - For performancemodel monitoring at the NW side, calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by UE and terminated at gNB. - For positioning enhancement use case: - For model training, training data can be generated by UE/PRU/gNB/LMF and terminated at LMF/OTT server. - For LMFNW-sided model inference (Case 2b, Case 3b), input data can be generated by UE/gNB and terminated at LMF gNB. - For gNB-sided model inference (Case 3a), input data is internally available at gNB. - For UE-side model inference (Case 1, Case 2a), input data/assistance information is internally available at UE can be generated by LMF/gNB and terminated at the UE. - For modelperformance monitoring at the NWLMF side, calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by UE/gNB and terminated at LMF. - For modelperformance monitoring at the NWgNB side, calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by at least gNB. Note: In RAN1’s answer to Assumption 4, RAN1 did not reply on the different NW entities for training (gNB/CN/LMF/OAM) as it is out of RAN1’s expertise that RAN1 cannot confirm. Note: For the above replies for Assumption 1~4 in Part A, RAN1’s understanding is that “input data” in the RAN2 LS does not include assistance information that a model may additionally use as model input. In RAN1’s answer, RAN1 did not reply on assistance information, and informs RAN2 of related conclusions/agreements/observations in the Appendix.

Related to data collection, it is also important to refer to the tables on the mapping of functions to entities that were discussed in R2-2308286 and agreed in the last RAN2#123:

R2-2308286 Report of [Post122][060][AIML] Mapping of functions to physical entities (CMCC) CMCC report Rel-18 FS_NR_AIML_air - Quite long discussion - CMCC report that FFS items has support from 3 companies. - Chair Comment: These options represent several possibilities. RAN2 would typically have selected a specific architecture option, and for a WI, specific option(s) need to be selected. Hope it is possible to further narrow down during the SI. P1-P6 are agreed, it is expected that FFS items for which support is not increased will be removed.

# Discussion

In the following sections, the discussion is organized taking into account NW-side models and UE-side models and considering data collection for training, and data collection for performances monitoring.

## 2.1	NW-side models

Regarding NW-side models for the CSI feedback enhancement /Beam management use case, the following table from R2-2308286 [2] was agreed:

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training (offline training) | gNB, OAM, [FFS: CN, OTT server] |
| b) | Model transfer/delivery | OAM->gNB, [FFS: CN->gNB, OTT server->gNB] |
| c) | Inference | gNB |
| d) | Model/functionality monitoring | gNB |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: For b), no model transfer/delivery is expected if the entity for model training and model inference is the same one.

Note 3: Whether/how OAM is to be involved may need to consult RAN3, SA5.

Note 4: Whether/how CN is to be involved may need to consult RAN3, SA2.

### 2.1.1	Training of NW-side models

For NW-side model training data collection, both the gNB and OAM may be the entities involved. In order to have a more focused discussion, it was discussed during RAN2#123 whether any prioritization is needed in the work.

- Q1: For training of NW-side models, do you believe that there is the need to prioritize gNB- or OAM-centric data collection?

  - Yes, gNB-centric data collection should be prioritized

  - Yes, OAM-centric data collection should be prioritized

  - No, both gNB- and OAM-centric data collection should be equally prioritized

| Company | Preferred Option (a,b,c) | Comments |
| --- | --- | --- |
| OPPO | c) | In this early stage, we don’t find the strong motivation to do the down-scoping even if only RAN centric use cases are considered in R18. In R18 NG-RAN AI led by RAN3, both OAM and NG-RAN can do the model training, in which the three use cases, i.e. power saving, load balance, mobility, are also RAN centric. More addition, in MDT data collection framework, OAM can also collect many metrics from lower layer over the air, e.g. RACH report, so OAM can be a option for data collection. |
| ZTE | C | We agree the prioritization shall be discussed. As rapporteur point out, the OAM/gNB can be a logical entity where the NW-side model is trained. In this sense, the gNB and OAM is a terminated point for data collection, which means, both gNB and OAM -centric data collection shall be prioritized. |
| Xiaomi | A or c | According to my RAN1 colleagues, RAN1 don’t have preference on gNB-centric or OAM-centric data collection. It can be up to RAN2. From RAN2 point of view, I understand it’s reasonable to collect data and train AI model at gNB, considering the use case is mainly within RAN and NW-side AI model is used by gNB. In OAM-centric data collection, additional signaling would be introduced to deliver AI model or data from OAM to gNB. So, we would prefer gNB-centric data collection. OAM centric data collection can be studied as well in this phase. |
| vivo | c | For the NW-side model for CSI/Beam use cases, prefer to reuse the functionality mapping in AI for NG-RAN. That is, the model training can be located at gNB or OAM, and both gNB- and OAM-centric data collection should be prioritized. |
| NEC | C | We support to discuss both gNB- and OAM-centric data collection in general. However, for different use case, gNB- or OAM-centric data collection may be more suitable than the other. |
| Apple | c | Let's not waste time on topic prioritization discussion, which is impossible to conclude at this stage. |
| Intel | c | As it was agreed in RAN2 #123 meeting that both gNB and OAM can be the model training entity, hence we think at this stage, both gNB- and OAM-centric data collection should be equally considered. |
| LGE | c) |  |

#### 2.1.1.1 gNB-centric data collection

For gNB-centric data collection, it seems natural to assume that the gNB configures the UE to initiate the data collection and the gNB terminates the data collection.

- Q2: For training of NW-side models, do you agree that a gNB-centric data collection implies that the gNB configures the UE to initiate the data collection procedure and the gNB terminates the data collection procedure?
If no, please provide your explanation.

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes with some clarification | Usually, ‘session’ is not a concept defined/used by RAN2 even if one can see the wording ‘PDU session/Trace session’ in RAN2 spec, it’s still defined in CN domain. To remove the confusion that RAN2 may define the data collection session concept alone, it may be better to use ‘procedure/task’ instead of ‘session’ when we discuss gNB-centric data collection. |
| ZTE | Yes | The legacy framework/procedure is preferred. |
| Xiaomi | Yes | Following current logic, the UE measurement is under gNB control. |
| vivo | Yes |  |
| NEC | Yes | Maybe we can say the same gNB configures the UE to initiate the data collection and terminates the data collection |
| Apple | Yes with modified wording of Rapporteur |  |
| Intel | see comment | we agree that gNB is responsible to configure how the UE perform data collection. However, we are wondering what “data collection session” is referring to in the question. Does it refer to TCE session or a new session for data collection or others? Therefore, we propose with following updates: For training of NW-side models, a gNB-centric data collection implies that the gNB configures and stops the UE to initiate the data collection session and the gNB terminates the data collection session? |
| LGE | Yes |  |

When it comes to RAN2 impact evaluation, L3 signalling reporting framework can be considered a natural candidate for gNB-centric data collection. While L1 measurement reporting can also potentially be used for this purpose, that does not have any RAN2 specific impact, so L1 measurement reporting should not be studied by RAN2.

- Q3: Related to gNB-centric data collection for NW-side model training, do you agree that RAN2 should study the potential impact on L3 signalling reports? 
If no, please provide your explanation and which alternative framework(s) (if any) can be used.

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Comments | We’re fine that RAN2 shall not consider L1 signaling based data collection framework, but it’s also misleading to use the terminology ‘L3 measurements reporting framework’ as people in RAN2 may specifically refer to L3 RRM measurements reporting framework which may be not the intention here, so we suggest to use L3 signaling based data collection framework instead of L3 measurements reporting framework. More addition, L3 measurements reporting framework is mainly used for RRM purpose, which cannot be used to report multiple samples per time. MDT-like framework is better than RRM framework because MDT-like framework allows to report multiple collected/stored entries per time which can be considered as a baseline. |
| ZTE | Yes with comments | For gNB-centric data collection, we generally agree that RAN2 should focus on the L3 signaling based report framework. Among the current candidates, only the followings are L3 signaling based reporting framework; 1: L3 measurement (RRM) 2: Early measurement 3: UAI To our understanding, the early measurement is mainly for the measurement for idle and inactive state which has been deprioritized before. - P6a: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement on the non-connected state can be revisited when needed. Regarding the UAI, it is not born as measurement/report framework, it can be deprioritized if there is no any strong motivation can be found. Maybe we can improve our ambition level: Regarding the gNB-centric data collection, RAN2 should study the potential impact on the L3 measurement at a higher priority. |
| Xiaomi | Yes | L3 signaling reporting is a natural choice. |
| vivo | OK to make the WA from RAN2 perspective | For CSI/Beam use cases, one straightforward way is utilizing the L1 measurement for data collection. However, if the data size exceeds the maximum threshold of the L1 report, L3 report can be considered by RAN2. If RAN2 assume that L3 signaling report is used for gNB-centric data collection for model training, RAN2 can further discuss the principle and potential specification impact related to L3 signaling. |
| NEC | Yes | We agree to focus on L3 based report, but we do not think we need to limit to L3 signaling based report |
| Apple | Yes | As usual, RAN2 starts working on L1 measurement only when RAN1 sent LS to RAN2 (mostly on RRC or ASN.1 implementation). On signaling of data collection, we think RAN2 is not in position to decide whether L1 measurement / signaling can be used (for one LCM purpose). Instead, RAN2 can only make conclusion on whether L3 measurement / signaling can be used (for one LCM purpose), and corresponding signaling change. |
| Intel | Yes |  |
| LGE | Yes | We are fine with the revised wording, but we believe L3 measurement reporting (RRM MR) framework can be a baseline for this question. |

If the impact on L3 signalling reports is agreed to be studied for gNB-centric data collection of NW-side models, the Rapporteur suggests discussing the main principles that the L3 signalling report framework should support in order to fulfil the requirements of NW-side model training. Once such principles are assessed, RAN2 can discuss whether any enhancement to the current L3 measurement reporting framework is needed.

Taking into account proposals from different papers submitted to RAN2#123, the following principles may be envisaged for the L3 signalling reporting for NW-side model training:

- The L3 signalling reporting framework for NW-side model training should not interfere with the ordinary L3 measurement reporting signalling used for RRM purposes and mobility.

- The L3 signalling reporting framework for NW-side model training should allow the UE to store sets of measurements and then report them to the gNB in multiple RRC segments (which might be needed if the UE has collected lots of data).

- The L3 signalling reporting framework for NW-side model training should allow the UE to report in a single RRC report multiple measurements taken at different points in time.

- The L3 signalling reporting framework for NW-side model training implies that the UE may be configured to report measurements periodically.

- The L3 signalling reporting framework for NW-side model training implies that the UE may be configured to report measurements upon fulfilling certain events.

Companies are invited to evaluate the above principles and provide their views.

- Q4: Related to gNB-centric data collection for NW-side model training, which of the above principles should be considered for the L3 signalling reporting framework?

| Company | Option (a,b,c,d,e) | Comments |
| --- | --- | --- |
| OPPO | b) and c) with clarification | Overall comments: We can use ‘L3 signaling based data collection framework’ instead of ‘L3 measurements reporting’. For a), RAN2 has not yet agreed to use L3 RRM reporting as the data collection baseline, so it’s too far to have the principle at this early stage. More addition, even if L3 RRM reporting may be considered as the baseline, it’s always followed by default that one new feature will not interfere with the legacy feature, which will be addressed during normative work as usual, so not so urgent for now to have bullet a); For b), collecting measurements will narrow down the scope for future extension, we believe this data collection framework is not only aimed for CSI and BM use cases, which may be extended to collect metrics for future use cases, in this sense, we suggest to use more generic wording; Another point is that we think it’s too early to assume RRC segments are needed. Let’s take MDT as an example, although MDT reports may be reported via multiple RRC signalling, no explicit RRC segments is used, instead only one indicator is embedded into one report to indicate whether MDT report is still available at UE side and NW can choose to re-acquire or not to re-acquire the stored data at UE side in which one single message is triggered without RRC segment marking. Based on above, we suggest the following: b) The L3 signaling based data collection report for NW-side model training should allow the UE to store multiple collected metric samples and then report them to the gNB in multiple RRC procedures (which might be needed if the UE has collected lots of data); For c), the similar suggestions as bullet b): c) The L3 signaling based data collection report for NW-side model training should allow the UE to report in a single RRC report including multiple collected metric samples taken at different points in time. For bullet d) and e), it’s more related to normative work details, not so urgent to address. |
| ZTE | a,b (FFS), c,d,e(FFS) | For a: As we comment above, L3 measurement can be studied at a higher priority, the first rule we need to follow is that the RRM and mobility shall not be impact by the data collection for model training. For b: Whether the RRC segment is supported depends on the data size requirement for model training at each report instance. It is not sure before the reception of LS for PART B from RAN1. For c, in the legacy L3 measurement/report, the UE filter the historical L1 measurement result into L3 measurement result and then report to NW, theoretically speaking, it already support UE to collect the L1 measurement result at different timing points. In our understanding, consider the data collection for model training is not delay sensitive, there is no need for UE to frequently report the measurement result per measurement occasion which is power-consuming, in this sense, c can be supported For d: this is legacy behavior and for AI based temporal beam prediction, the continuous data is very much important for model training. So the periodic data collection is needed. For e: Even though this is also legacy, so far the motivation of the event triggered data collection for model training is not clear. We suggest to mark this as FFS. |
| Xiaomi | B,c,d,e | For a), it’s a bit unclear on ‘interfere’. This depends on the design of data collection. We understand it may be possible that UE can’t perform data collection and ordinary L3 RRM due to capability restriction. It’s too early to say no interference. B and c are beneficial to reduce additional signaling. D and e are beneficial to meet the delay requirement |
| vivo | b,c,d,e with comments | For a), agree with the above that the “interfere” should be clarified, does it imply that the model training data cannot be reported in RRM report? For b), agree with the first part. Whether segmentation is needed can be revisited. b) The L3 signalling reporting framework for NW-side model training should allow the UE to store sets of measurements and then report them to the gNB in multiple RRC segments (which might be needed if the UE has collected lots of data). For c,d,e), in addition to reporting, “recording” can be added. c) The L3 signalling reporting framework for NW-side model training should allow the UE to record and report in a single RRC report multiple measurements taken at different points in time. d) The L3 signalling reporting framework for NW-side model training implies that the UE may be configured to record and report measurements periodically. e) The L3 signalling reporting framework for NW-side model training implies that the UE may be configured to record and report measurements upon fulfilling certain events. |
| NEC | a/d/e | For b/c, we propose to also consider the data based report considering the large amount of data report for AIML training. Even though we can consider L3 signalling based data collection, that should be based on a special SRB, to avoiding overwhelming the normal signalling transmission. |
| Apple | b, c (with wording modification) , d, e | For a), we think it is a general requirement for all new signaling (i.e. we should ensure the new signaling can work with legacy signaling), which doesn't need to capture explicitly. And it is not a specific requirement for data collection of AI/ML, For b), we agree. And we are fine to keep segmentation, which we think it is obvious. For c), we agree, but want to clarify that c) also allow the UE to store the measurements taken in different time points, right? Otherwise (if not allow UE to store), how the UE can report measurements taken in different time? The issue is that: if we review b) and c) together, it may be misunderstood that c) don't allow UE to store measurements because b) mentions "store" but c) doesn't. So, we suggest below change: c) The L3 signalling reporting framework for NW-side model training should allow the UE to store and report in a single RRC report multiple measurements taken at different points in time. For d) and e), although RAN2 generally don't make decision of signaling at this stage, latest TR 38.843 has captured both options in bottom of section 5.1: - Performance metric including at least intermediate KPI (e.g., NMSE or SGCS) - UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report. - Note: down selection is not precluded. - Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW. So, we are fine to capture d) and e) as principles. |
| Intel | b, c, d, e | For a), it is not clear how to understand “should not interfere with the ordinary L3 measurement reporting”, does it mean the NW should not change configuration for existing L3 measurement reporting for AI/ML data collection purpose or others? |
| LGE | b,c,d,e | For a), we think RRM is the simplest extensible procedure since RRM can easily accommodate future expansions or modifications related to beam measurements. For b) and c), both can be considered as solutions to store(collect) and report data set. For d) and e), both are general solutions to trigger storing/reporting data. We can take both general methods in this stage. |

Besides the above listed principles, Rapporteur would like to ask companies if there is any other principle that RAN2 should study for the L3 signalling reporting framework.

- Q5: Related to gNB-centric data collection for NW-side model training, is there any other principle that RAN2 should take into account for the L3 signalling reporting framework? Please describe.

| Company | Comments |
| --- | --- |
| OPPO | - The L3 signaling based data collection report for NW-side model training should have the flexibility to allow the UE to collect metrics in all RRC states or part of RRC states, which is under NW control. - Because the data collected in different RRC state may be used by different target model training, one can not assume UE will always collect data in one RRC state for all use cases. - The L3 signaling based data collection report for NW-side model training should have the flexibility to allow the UE to collect metrics for specific feature/procedure/function, which is under NW control. - Because different use cases may collect similar metrics but shaing a unified data collection framework, for instance, UE may collect L3 measurement during RRC establishment(including RRC setup/resume/re-establish cases) to optimize the performance during RRC establishment procedure while the L3 measurements collected during RRC connected state may be used to optimize the performance for UE mobility, UE should know from NW which procedure NW aims for when setting data collection metrics. |
| ZTE | None according to the current situation. |
| Apple | We agree with 1st point of OPPO (i.e. allow the UE to store metrics in all RRC states). Note that RAN1 has explicitly informed RAN2 in reply LS (R1-2308730): Regarding Assumption 3 of Part A, Assumption 3: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement of the non-connected state can be revisited when needed.. RAN1 confirms RAN2’s Assumption 3 for CSI compression, CSI prediction, beam prediction and Positioning use cases. For positioning, it is noted that existing specification supports DL PRS measurement and UE positioning in both RRC_CONNECTED and RRC_INACTIVE state. |
| Intel | For gNB-centric data collection and model training at gNB, since the collected data is used by gNB for model training. The collected data from UE should be visible and able to be used by gNB for model training directly. |

#### 2.1.1.2 OAM-centric data collection

For OAM-centric data collection, it seems natural to assume that the OAM configures the UE to initiate the data collection session, and it terminates such data collection session.

- Q6: For training of NW-side models, do you agree that an OAM-centric data collection implies that the OAM configures the UE to initiate the data collection procedure and the OAM terminates the data collection procedure?
If no, please provide your explanation.

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes with clarification | In legacy, TCE in SA5/OAM domain is the node hosting the data collection procedure, so we suggest to make the wording more generic: An OAM-centric data collection implies that one node in OAM domain configures the UE to initiate the data collection task/procedure and the node in OAM domain terminates the data collection task/procedure. |
| ZTE | Yes | Among the current candidates, only the followings are the OAM-Centric data collection: 1: Logged MDT 2: Immediate MDT. |
| Xiaomi | Yes |  |
| vivo | Yes, with comments | The management-based MDT framework can be reused as a baseline. To be specific, the OAM sends the data collection configuration to NG-RAN node. NG-RAN sends the configuration to the qualified UEs (e.g., UE capability, user consent) and the UEs feedback the data report when available. For the interaction between UE and NG-RAN, the configuration and reporting mechanisms for gNB-centric data collection can be reused. |
| NEC | Yes |  |
| Apple | Yes |  |
| Intel | see comment | In our understanding, there’s no direct interface between OAM and UE. According to current MDT mechanism, the configuration towards UE is transmitted by gNB, e.g. LoggedMeasurementConfiguration. For OAM-centric data collection, we expect this principle (no direct interface between OAM and UE) will not be changed. Furthermore, similar to our comment to gNB-centric data collection, following changes are suggested: For training of NW-side models, a OAM-centric data collection implies that the OAM configures and stops the UE to initiate the data collection via gNB session and the OAM terminates the data collection session? |
| LGE | Yes |  |

Related to which framework to adopt for the OAM-centric data collection, the MDT seems to be a natural candidate, since the MDT was designed to allow the OAM to configure the UE to perform a data collection session and to terminate such data collection session (in the TCE). In particular, as showed in the endorsed tables in R2-2302286, MDT consists of immediate MDT and logged MDT which have the following characteristics:

| Logged MDT | TCE/OAM (It can be utilized by gNB) | RRC_IDLE/RRRC_INACTIVE | <9kbyte | L3 cell/beam measurements, location info, sensor info, timing info | - Procedure latency***: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency****: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message, Privacy via user consent |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Immediate MDT | TCE/OAM (It can be utilized by gNB) | RRC_CONNECTED | <9kbyte | L3 cell/beam measurements, location info, sensor info | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Event triggered report, Periodic reporting | AS security via RRC message, Privacy via user consent |

In a nutshell:

- Logged MDT allows the UE to collect and store multiple measurements, and then report such logged measurements to the gNB upon request, potentially in multiple RRC segments. However, the measurements included in the logged MDT are only performed by the UE in IDLE/INACTIVE state.

- Immediate MDT is based on the existing RRC measurement procedures for configuration and reporting. It works in RRC_CONNECTED state, however, only a single measurement result can be conveyed in one measurement report, i.e., unlike logged MDT it does not support the possibility for the UE to collect and store multiple measurements taken at different points in time, and then report those measurements to the network in multiple RRC segments (which might be necessary when the UE has collected lots of data).

Given the above, the Rapporteur would like to ask the following question:

- Q7: Related to OAM-centric data collection for NW-side model training, do you agree that RAN2 should study the potential impact on MDT?

  - Yes, the study of immediate MDT should be prioritized.

  - Yes, the study of logged MDT should be prioritized.

  - Yes, both the immediate MDT and logged MDT should be equally prioritized.

  - No, alternative collection framework should be studied. Please specify what.

| Company | Option (a,b,c,d) | Comments |
| --- | --- | --- |
| OPPO | d) | Although CSI and BM use cases are related to connected mode, only one single sample can be reported via immediate MDT framework while Logged MDT allows to collect/store multiple samples which is also beneficial for model training purpose, so a combined solution, i.e. new framework, may be more suitable to leverage the pros for both frameworks. It should be noted that immediate MDT and logged MDT can work as the reference when designing the new data collection framework for model training. |
| ZTE | A | As the we mentioned in above, RAN2 have assumed the data collection shall focus on RRC connected mode. Then a) is an option that complies the RAN2 understanding. However, we also need to dig the necessities about the logged-MDT, that is, whether the data collection at UE idle/inactive state is necessary. For now, we think a should be prioritized. |
| Xiaomi | C | Both can be studied as baseline. |
| vivo | a) with comments | AI for NG-RAN may introduce enhancement on MDT in Rel-19 WI. Coordination is needed to avoid duplicated discussions. |
| NEC | d) | We think the OAM based data collection is a different framework from the traditional data collection from MDT in terms of the triggering for data collection and the characteristics of the data collection. And meanwhile, we may adopt some mechanism from immediate MDT and logged MDT. With this said, we support to study a new framework for AIML data collection on top of MDT. This work can also be coordinated with SA5 and RAN3. |
| Apple | c) or d) (new MDT framework across all RRC states) | RAN1 has explicitly informed RAN2 to consider data collection for both CONNECTED state and INACTIVE state in reply LS (R1-2308730): Regarding Assumption 3 of Part A, Assumption 3: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement of the non-connected state can be revisited when needed.. RAN1 confirms RAN2’s Assumption 3 for CSI compression, CSI prediction, beam prediction and Positioning use cases. For positioning, it is noted that existing specification supports DL PRS measurement and UE positioning in both RRC_CONNECTED and RRC_INACTIVE state. So, we think it is important to collect data in all RRC states. Regarding to specific solution, we are fine with both below ways: - c) consider both immediate MDT (for data in CONNECTED state) and logged MDT (for data in INACTIVE/IDLD state) - d) introduce a new MDT type which allows the UE to store and report data collected in all RRC states (CONNECTED state and INACTIVE/IDLD state) |
| Intel | a | As replied in RAN1 reply LS R1-2308730, data collection should focus on only RRC_CONNECTED state (except for positioning that INACTIVE state is already supported by existing PRS measurement and UE positioning mechanism). Furthermore, as rapporteur analyzed in Q8, there are some commonalities between immediate MDT enhancement and gNB-centric data collection enhancement. It can also save effort/time for RAN2 to study. Therefore, we think only immediate MDT enhancement should be prioritized. RAN1 reply LS: Regarding Assumption 3 of Part A, Assumption 3: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement of the non-connected state can be revisited when needed.. RAN1 confirms RAN2’s Assumption 3 for CSI compression, CSI prediction, beam prediction and Positioning use cases. For positioning, it is noted that existing specification supports DL PRS measurement and UE positioning in both RRC_CONNECTED and RRC_INACTIVE state.. |
| LGE | d) | For b) logged MDT, considering motivation/efforts for specifying beam measurement mechanisms in the existing logged MDT as follows, we think logged MDT is not suitable for model training: - Specific beam measurement settings and reports: Since the purpose of logged MDT is to perform measurement in RRC idle/inactive state, there are no beam specific configurations/reports. For example, measurement/report related to SSB/CSI-RS is not considered. - When/How to report the logged measurement results (for training) in connected state: In the current procedure, the UE can notify the network that the logged measurement result is available when entering connected state. The network can bring the logged measurement result through the UE information request/response procedure. In the connected state, additional consideration is needed to inform when it is available and when to report. For a) immediate MDT, Immediate MDT utilizes the RRM procedure to report data. From the RAN2 point of view, we think immediate MDT can be analyzed with RRM analysis in 2.1.1.1 gNB-centric data collection. |

Irrespective of whether immediate or logged MDT is taken into account for the OAM-centric data collection of NW-side models, the Rapporteur suggests discussing what are the main principles that the MDT framework should support in order to fulfil the requirements of OAM-centric data collection. Once such principles are assessed, RAN2 can discuss whether any enhancement to Immediate or Logged MDT is needed.

For example, for Immediate MDT, we note that the current framework is based on the existing RRC measurement procedures for configuration and reporting. Hence, the following principles may need to be studied:

- The Immediate MDT framework for NW-side model training should allow the UE to store sets of measurements and then report them in multiple RRC segments.

- The Immediate MDT framework for NW-side model training should allow the UE to report in a single RRC report multiple measurements taken at different points in time.

- The Immediate MDT framework for NW-side model training should allow the network to configure the UE to report measurements periodically.

- The Immediate MDT framework for NW-side model training should allow the network to configure the UE to report measurements upon fulfilling certain events.

- Q8: Related to OAM-centric data collection for NW-side model training, which of the above principles should be considered if Immediate MDT is used?

| Company | Option (a,b,c,d) | Comments |
| --- | --- | --- |
| OPPO | a) and b) with clarification | The similar suggestion as Q4. No need to differentiate Immediate MDT and logged MDT when setting principles/requirements For a) and b) suggest the following wording: - The OAM-centric data collection reporting for NW-side model training should allow the UE to store multiple collected metric samples and then report them in multiple RRC procedures. - The OAM-centric data collection reporting for NW-side model training should allow the UE to report in a single RRC report including multiple collected metric samples taken at different points in time. For c) and d), postpone to normative work stage. |
| ZTE | a (FFS), b,c,d (FFS) | The similar suggestion in Q4. a: Whether the RRC segments are supported depends on the requirement of data size for each report instance. d: it is not still clear what is the motivation of event triggered data collection for model training. |
| Xiaomi | All | I understand these principles are beneficial to avoid signaling overhead and meet latency requirement. However, with such principles, MDT and L3 measurement report becomes similar. Therefore, I understand these principles are agreeable regardless MDT or L3 measurement report is used. |
| vivo | All with comments | Similar comments in Q4. And for the interaction between UE and NG-RAN, the configuration and reporting mechanisms for gNB-centric data collection can be reused as baseline. a) The Immediate MDT framework for NW-side model training should allow the UE to store sets of measurements and then report them in multiple RRC segments. b) The Immediate MDT framework for NW-side model training should allow the UE to record and report in a single RRC report multiple measurements taken at different points in time. c) The Immediate MDT framework for NW-side model training should allow the network to configure the UE to record and report measurements periodically. d) The Immediate MDT framework for NW-side model training should allow the network to configure the UE to record and report measurements upon fulfilling certain events. |
| NEC | All with comments | As said, we support study a different framework for data collection. However, if immediate MDT is studied as the start point, we may need to consider the RRC Idle/Inactive mode based data collection and report |
| Apple | All with wording modification in b) | Similar comments in Q4. We also support NEC raised point (i.e. need to consider the RRC Idle/Inactive mode based data collection and report). For b), we suggest below change: b) The Immediate MDT framework for NW-side model training should allow the UE to store and report in a single RRC report multiple measurements taken at different points in time. |
| Intel | a, b, c, d | Regarding to d), since event trigger reporting is now supported by immediate MDT, we are wondering whether this is for new event or existing event to be reused? If yes for new event, it would be good to clarify in the principle. |
| LGE | a,b,c,d | Similar answer with Q4. For a) and b), both can be considered for solutions to store(collect) and report data set. For c) and d), both are general solutions to trigger storing/reporting data. We can take both general methods in this stage. |

Besides the above listed principles, Rapporteur would like to ask companies if there is any other principle that RAN2 should study related to the immediate MDT.

- Q9: Related to OAM-centric data collection for NW-side model training, is there any other principle that RAN2 should take into account if the Immediate MDT is used? Please describe.

| Company | Comments |
| --- | --- |
| OPPO | The similar suggestions as Q5. - The OAM-centric data collection report for NW-side model training should have the flexibility to allow the UE to collect metrics in all RRC states or part of RRC states, which is under OAM control. - The OAM-centric data collection report for NW-side model training should have the flexibility to allow the UE to collect metrics for specific feature/procedure/function, which is under OAM control. |
| ZTE | None according to the current situation. |
| vivo | For the interaction between UE and NG-RAN, the configuration and reporting mechanisms for gNB-centric data collection can be reused. That is, OAM- or gNB-centric data collection can be transparent to UE. |
| Apple | We agree with OPPO's 1st point: The UE is allowed to store data across different RRC states without pause. |

On the other hand, for Logged MDT, it is already possible for the UE to store sets of measurements and then report them to the network in multiple RRC segments. However, it only works for UEs in IDLE/INACTIVE state (which is not in line with the RAN2 understanding that the CONNECTED mode use cases should be prioritized). Additionally, the logged MDT reports can be transmitted to the network only upon gNB request. Hence, the following principles may need to be studied:

- The Logged MDT framework for NW-side model training should also work for UEs in RRC_CONNECTED state.

- The Logged MDT framework for NW-side model training should allow the network to collect the UE measurement reports periodically.

- The Logged MDT framework for NW-side model training should allow the network to collect the UE measurement reports upon fulfilling certain events.

- Q10: Related to OAM-centric data collection for NW-side model training, which of the above principles should be considered if Logged MDT is used?

| Company | Option (a,b,c) | Comments |
| --- | --- | --- |
| OPPO | None | For OAM-centric data collection, no need to differentiate Immediate MDT and logged MDT when setting principles/requirements, the following suggestion can directly reflect the data collection requirement for OAM-centric data collection: - The OAM-centric data collection reporting for NW-side model training should allow the UE to store multiple collected metric samples and then report them in multiple RRC procedures. - The OAM-centric data collection reporting for NW-side model training should allow the UE to report in a single RRC report including multiple collected metric samples taken at different points in time. - The OAM-centric data collection report for NW-side model training should have the flexibility to allow the UE to collect metrics in all RRC states or part of RRC states, which is under OAM control; - The OAM-centric data collection report for NW-side model training should have the flexibility to allow the UE to collect metrics for specific feature/procedure/function, which is under OAM control. |
| ZTE | None | The logged MDT is applied for UE to collect the data during the idle/inactive state and report to NW by gNB request in RRC connected mode. It cannot be foreseen to enhance the logged MDT to work on RRC connected mode. |
| Xiaomi | All | I understand these principles are beneficial to avoid signaling overhead and meet latency requirement. However, with such principles, MDT and L3 measurement report becomes similar. Therefore, I understand these principles are agreeable regardless MDT or L3 measurement report is used. |
| vivo | None | Prefer to keep logged MDT only involving measurement logging by UE in IDLE and INACTIVE state. The analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). |
| NEC | All with comments | As said, we support study a different framework for data collection. However, if logged MDT is studied as the start point, we may need to consider the data collection for the UE during state transition between RRC Idle/Inactive and connected in terms of data collection. |
| Apple | All | Similar view as NEC. We need to take RAN1 Reply LS into account. |
| Intel | others | We don’t think enhancement for logged MDT is needed, as discussed in Q7. |
| LGE | None | See Q7. Considering motivation/efforts for specifying beam measurement mechanisms in the existing logged MDT as follows, we think logged MDT is not suitable for model training. |

Besides the above listed principles, Rapporteur would like to ask companies if there is any other principle that RAN2 should study related to the logged MDT.

- Q11: Related to OAM-centric data collection for NW-side model training, is there any other principle that RAN2 should take into account if the Logged MDT is used? Please describe.

| Company | Comments |
| --- | --- |
| OPPO | See answer in Q10 |
| Apple | The UE is allowed to store data across different RRC states without pause. |

### 2.1.2 Performance Monitoring of NW-side models

On the topic of performance monitoring of NW-side models, RAN2 assumed in RAN2#122 that:

“- for model monitoring, when required monitoring data (e.g., performance metric) comes from the other entities, there is a latency requirement for data collection.”

Additionally, Rapporteur believes that also the signalling overhead could be a concern, especially if the monitoring should be done frequently. 
If L3 measurements are used for this purpose, the above requirements should be taken into account. Rapporteur would like to ask companies if there is any impact that can be foreseen in RAN2 protocols related to performance monitoring of NW-side model, taking into account the above expected requirements.

- Q12: Related to performance monitoring of NW-side models, do you foresee any impact in RAN2 protocols that RAN2 should study? Please describe.

| Company | Comments |
| --- | --- |
| OPPO | No yet. |
| Xiaomi | To assist performance monitoring of NW-side models, UE may need to collect and report monitoring data/assistance information to gNB. There may be spec impact to RAN2 on the signaling and procedure. |
| vivo | On this issue we can wait for RAN1 feedback on the requirement of data collection for model monitoring. E.g., Using L3 or L1 signaling for model monitoring relies on the content and size of the monitoring data. Potential RAN2 impacts may include monitoring configuration and reporting. Agreement In CSI compression using two-sided model use case, further study the necessity, complexity, overhead, latency and potential specification impact on ground truth CSI report for NW side data collection for model performance monitoring, including: - Scalar quantization for ground-truth CSI - FFS: any processing applied to the ground-truth CSI before scalar quantization - Codebook-based quantization for ground-truth CSI - FFS: Parameter set enhancement of existing eType II codebook, based on evaluation results in 9.2.2.1 - RRC signaling and/or L1 signaling procedure to enable fast identification of AI/ML model performance - Aperiodic/semi-persistent or periodic ground-truth CSI report.. |
| Apple | Not for now. We tend to think RAN2 study on data collection for inference and performance monitor should not start before reception of RAN1 specific requirement (i.e. RAN2 can only study data collection for offline training at this stage). |
| Intel | prefer to wait for RAN1 requirement and input regarding to performance monitoring. |
| LGE | Cannot see yet. It depends on the RAN1 decision. According to RAN1 LS, there would reports for calculated performance metrics or data needed for performance metric. |

## 2.2 UE-side models

Related to UE-side models for the CSI feedback enhancement/Beam management use case, the following table from R2-2308286 [2] was agreed:

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training(offline training) | UE-side OTT server, UE, [FFS: gNB, OAM, CN] |
| b) | Model transfer/delivery | UE-side OTT server->UE, [FFS: gNB->UE, or OAM->UE, or CN->UE] |
| c) | Inference | UE |
| d) | Model/functionality monitoring | UE (UE monitors the performance, and may report to gNB), gNB (gNB monitors the performance) |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB if monitoring resides at UE or gNB, UE if monitoring resides at UE |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: For b), no model transfer/delivery is expected if the entity for model training and model inference is the same one.

Note 3: Whether/how OAM is to be involved may need to consult RAN3, SA5.

Note 4: Whether/how CN is to be involved may need to consult RAN3, SA2.

### 2.2.1	Training of UE-side models

As shown in the table above, RAN2 has so far assumed that the UE itself or the UE-side OTT server will oversee the model training for UE-side models. 
It was discussed at length in RAN2 whether data collection protocols available outside RAN could be used for the purpose of UE-side model training when the UE-side OTT server is involved. However, no conclusion was made. At the same time, it is not clear what could be the impact in RAN2 protocols if the UE-side OTT server is responsible for the UE-side model training.

- Q13: If the UE-side OTT server is responsible for the UE-side model training, do you agree that the way the OTT server collects data should not be studied in RAN2?
If no, please explain which data collection framework should RAN2 study when the UE-side OTT server is responsible for the UE-side model training.

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO |  | For this scenario, SA work may be involved but RAN2 is not involved based on current observation. |
| ZTE | Yes | The OTT server collects data should not be studied in RAN. |
| Xiaomi | Yes |  |
| vivo | Yes, with comments | Potential discussion on data collection framework for the UE-side model training at UE-side OTT server should involve SA WG(s), such as SA2. RAN2 should send Ls to SA2 to trigger the discussion in SA2. |
| NEC | Yes |  |
| Apple | Yes | Not in RAN scope, and no need to send LS to SA2 because SA2 is having such discussion. |
| Intel | Yes |  |
| LGE | Yes, but | To avoid affecting user data transmission (data performance), we can discuss whether the AIML data specific management is necessary. |

Even if the UE-side model training is performed by the UE itself or the UE-side OTT server, RAN2 may need to discuss whether any impact is expected in RAN2 protocols, e.g. in order to allow the UE to properly perform the training. To this end, Rapporteur notes that in the TR 38.843, RAN1 mentions “UE reporting to NW supported/preferred configurations of DL RS transmission”, “data collection initiated/triggered by configuration from NW”, “request from UE for data collection”, etc.:

In light of what is already captured above by RAN1 in the TR, the Rapporteur would like to ask companies if there is any other impact foreseen in RAN2 protocols related to UE-side model training, e.g. if the RRC UE Assistance Information can be reused and enhanced for the above purposes mentioned in the TR.

- Q14: Related to UE-side model training, do you foresee any potential impact in existing RAN2 protocols, e.g. impact in the legacy RRC UE Assistance Information, that RAN2 should study? Please describe.

| Company | Comments |
| --- | --- |
| OPPO | At least data collection initiated/triggered by configuration from NW may have RAN2 impact, but the details may have RAN1 dependency for CSI and BM use cases. |
| ZTE | We are open for UE to provide the UE assistance information to acquire their wanted measurement configuration for the UE sided model training, but it shall be discussed in normative phase. |
| Xiaomi | We would not prefer UE to do the model training due to limited capability on computation and storage. |
| vivo | UE may send the request for assistance information to the network, e.g., to categorize the collected data. And network may provide the assistance information to UE via dedicated signaling or broadcast SIB. |
| NEC | We think UE-side model training, the UE may need to report the information of the UE-side model via legacy message e.g. legacy RRC UE Assistance Information. |
| Apple | We agree that UAI may be used to send at least assistance information. However, as usual, such RAN2 work should be started only after reception if RAN1 LS. For now, we don't think RAN2 can do anything. |
| Intel | Not for now. It is not clear at this stage what is the association between model and its data collection. It is possible that the network can defer the requirement for data collection configuration based on the UE reported model meta info, applicable condition, etc. For this case, there’s no need to let UE report additional duplicated preferred information to NW. Hence, we prefer to wait until it is clear what the meta info will be. |
| LGE | Agree with OPPO |

### 2.2.2 Performance Monitoring of UE-side models

Related to performance monitoring for UE-side models, the following is captured in the TR 38.843:

From TR 38.843: For performance monitoring at the NW side, calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by UE and terminated at gNB.

The methods for conveying the performance metrics or the data needed for performance metric calculations requires further discussion in RAN1 and RAN2. The Rapporteur believes that RAN2 could study the impact on RRC protocols for the reporting of calculated performance metrics or for the reporting of the data necessary for performance metric calculations.

- Q15: Related to UE-side performance monitoring, do you foresee any potential impact in existing RAN2 protocols to convey data from the UE to the gNB with the calculated performance metrics or with the data needed for performance metric calculation? Please describe.

| Company | Comments |
| --- | --- |
| OPPO | First, we’d like to clarify that UE sided model monitored by NW is one candidate option raised by RAN1, so it’s quite clear that this scenario may have RAN2 impact. We also believe that L3 signaling is better and more future proof than L1 signaling for model monitoring metrics reporting. As for the reporting details, more RAN1 guidance is needed before RAN2 can go through the detailed discussion. Another point is that RAN2 should clarify that NW based model monitoring can be applicable to both activated model and inactive model as RAN1 already made the following agreements: Agreement For the purpose of activation/selection/switching of UE-side models/UE-part of two-sided models /functionalities (if applicable), study necessity, feasibility and potential specification impact for methods to assess/monitor the applicability and expected performance of an inactive model/functionality, including the following examples: - Assessment/Monitoring based on the additional conditions associated with the model/functionality - Assessment/Monitoring based on input/output data distribution - Assessment/Monitoring using the inactive model/functionality for monitoring purpose and measuring the inference accuracy - Assessment/Monitoring based on past knowledge of the performance of the same model/functionality (e.g., based on other UEs) FFS: Requirements for the assessment/monitoring to be reliable (e.g., sufficient data coverage during evaluation) FFS: Additional aspects specific to the case where the inactive model has never been activated before, if any. So we suggest RAN2 to consider the following proposal for UE side model monitoring: Proposal: UE sided model monitored by NW can be applicable to both activated model and inactive model. |
| ZTE | In general, both L3 signaling and MAC CE is applicable for this intention anyway from RAN2 perspective. However, so far as now, due to the lack of detail information about the model monitoring, we cannot identify any further impacts on the RAN2 protocol for UE to report the performance metrics and/or data for NW. |
| Xiaomi | As captured in TR, UE may need to report calculated performance metrics or data needed for performance metric calculation to gNB. Existing signaling may not support the required metrics or data. And the report procedure may also need to be enhanced. |
| vivo | See comments in Q12. |
| Apple | Not for now. We tend to think RAN2 study on data collection for inference and performance monitor should not start before reception of RAN1 specific requirement (i.e. RAN2 can only study data collection for offline training at this stage). |
| Intel | Without RAN1 further input on data content, typical data size and latency requirement, which are relevant to part B in RAN2 LS, though RAN2 might have some impact, we prefer to wait for RAN1 input and discuss based on reply to part B. |
| LGE | In case of monitoring, network may configure a threshold criterion to facilitate UE to perform model monitoring. However, if UE sends monitoring-related reports whenever conducting model monitoring operations, there would be a lot of power consumption in UE and signalling overhead. Note that several models can be activated at once for various functionality. As many models are activated, many monitoring-related reports can be triggered. - RAN2 can consider when storing/reporting monitoring-related results Regarding to contents of the reports, - RAN2 can consider how to combine the monitoring-related results of Activated and Inactivated model-based results (align with RAN1 agreement) If the priority of the RRC message including monitoring-related information is always lower, it can lead to delays in LCM operation, such as model switching, (de)activation, fallback, etc. - RAN2 can consider adopting SRB depending on model-related information If the model is changed due to switching/update/etc, stored monitoring-related results can be removed in UE. - RAN2 can consider how to manage outdated monitoring information |

# Conclusion

TBD

# 4. References

- R2-2308898, Data collection for AI/ML, Ericsson

- R2-2308286, Report of [Post122][060][AIML] Mapping of functions to physical entities (CMCC), CMCC