3GPP TSG-RAN WG2 Meeting #123bis	R2-2309316

# Xiamen, China, 9~13 October 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | Num | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of XR enhancements to TS 38.321 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2023-09-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New mechanisms have been agreed to support XR services in NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The agreements and the clauses in which they are implemented are tabulated in the following: Agreement; Clause. From RAN1#113: From RAN1 perspective, for determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH: - The HARQ process ID for the first configured PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable - HARQ Process ID = [X*floor( (CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes - HARQ Process ID = [X*floor((CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2 - X= the number of configured PUSCHs in the CG period - The HARQ process ID of the remaining configured and valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable. - Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB. From RAN2#123: - Send LS to RAN1 informing them of the error in the formula and tell RAN2 will capture the HARQ process formula in RAN2 specifications. - Ask RAN1 to capture the definition and clarify the text accordingly (i.e. no “yes” or “no” parts) so that RAN2 can refer to it in our specifications - Add “RAN2 would request RAN1 to capture the definition an invalid CG PUSCH as below” to the start of the the 2nd part, i.e. “RAN2 would respectfully request RAN1 to capture the definition an invalid CG PUSCH as below, and would also like to know where the definition will be captured in the RAN1 specifications (so that RAN2 specifications can refer to that definition).”; 5.4.1. From RAN1#113: - When a CG PUSCH occasion is indicated as “unused”, the UE is not allowed to transmit CG PUSCH on that CG PUSCH occasion. - For any other CG PUSCH occasion that is NOT indicated as “unused”, the UE is allowed to transmit or not to transmit CG PUSCH on that CG PUSCH occasion as per legacy specification. - A CG PUSCH occasion indicated as “unused” earlier, is not allowed to be indicated as “NOT unused later”. - A CG PUSCH occasion indicated as “NOT unused” earlier, can be indicated as “unused” later.; 5.4.3.1.3, 5.8.2. From RAN2#121bis-e: - To address SFN wrap around, it is proposed to adopt option with a counter in DRX formula that increments at every SFN wrap around and an DRX reference SFN signalled by network. From RAN2#122: - Not use broadcast signalling for counter and reference SFN in XR. - Define DRX cycle based on rational numbers. From RAN2#123: - The maximum value of the counter (NSFN) is 2^16 = 65536 - Network sets DRX reference SFN (drx-ReferenceSFN) to either 0 or 512, in the same way as in Rel-16 IIoT Use the following option (option A): both the counter NSFN and the DRX reference SFN drx-ReferenceSFN are added to the DRX formula. NSFN is initialized to 0.; 5.7. From RAN2#122: - For retransmission less CG enhancement in XR, adapt the NTN solution by disabling the HARQ RTT timer per CG configuration. Specifically, the following modifications shall be introduced: - A new RRC parameter for disabling drx-HARQ-RTT-TimerUL for a CG configuration; - Changes in the procedural text of DRX operations for CG in the MAC specification; - A new UE capability for supporting disabling drx-HARQ-RTT-TimerUL for a CG configuration.; 5.7. From RAN1#113: - Multi-PUSCHs CG is supported for Type-1 configured grant. Multi-PUSCHs CG is supported for Type-2 configured grant. [RAN1#112] • To determine corresponding slots for CG PUSCHs in a period of a multi-PUSCH CG configuration: o For the first PUSCH in the period, follow the legacy procedures. o For remaining PUSCHs in the period - ForType-1 and Type-2, reuse the corresponding procedures for NR-U by applying the RRC parameters N and M, instead of cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively.; 5.8.2. From RAN2#119bis-e: - Delay information consists of at least “remaining time”. - If we have delay information, it needs to distinguish how much data is buffered for which delay value. From RAN2#122: - UE calculates the remaining time based on the PDCP discard timer value. - When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. From RAN2#123 - Support threshold based DSR reporting, e.g. DSR reporting is triggered when remaining delay of a PDU/PDU set is below a NW configured threshold. The threshold is configured per LCG. FFS whether configuring multiple thresholds for a LCG is supported. Definition of remaining time is FFS. - Network can configure the UE whether to trigger delay status reporting. FFS if we have some thresholds per LCG.; 5.x. From RAN2#119bis-e: - Introduce new BS table(s) to reduce the quantisation errors (e.g. for high bit rates). From RAN2#120: - RAN2 thinks we need one or more additional BSR table(s) for XR. From RAN2#121bis-e: - As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. - Support of new BSR table(s) is based on NW configuration and UE capability. - Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. - At least linear distribution is used for generating code points in new BSR table(s). - New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements. - Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. From RAN2#122: - Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases).; 6.1.3.1a, 6.2.1. From RAN2#121: - RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. From RAN2#123 - Working assumption: Define a new, separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting. Detailed Definition of associated data volume is FFS. - When UE triggers reporting delay information for a LCG, and UE also reports the buffer status associated with the remaining time.; 6.1.3.x, 6.2.1. In addition, a definition of multi-PUSCH CG is added to 3.1 and an abbreviation for delay status report is added to 3.2. |
| Consequences if not approved: |  | MAC-layer procedures for the agreed mechanisms to support XR services will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 5.4.1, 5.7, 5.8.2, 5.4.X, 6.1.3.1a, 6.1.3.x, 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.300 … CR … TS 38.306 … CR … TS 38.322 … CR … TS 38.323 … CR … TS 38.331 … CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Dormant BWP: The dormant BWP is one of downlink BWPs configured by the network via dedicated RRC signaling. In the dormant BWP, the UE stop monitoring PDCCH on/for the SCell, but continues performing CSI measurements, Automatic Gain Control (AGC) and beam management, if configured.

DRX group: A group of Serving Cells that is configured by RRC and that have the same DRX Active Time.

HARQ information: HARQ information for DL-SCH, for UL-SCH, or for SL-SCH transmissions consists of New Data Indicator (NDI), Transport Block Size (TBS), Redundancy Version (RV), and HARQ process ID.

IAB-donor: gNB that provides network access to UEs via a network of backhaul and access links.

IAB-node: RAN node that supports NR access links to UEs and NR backhaul links to parent nodes and child nodes.

Listen Before Talk: A procedure according to which transmissions are not performed if the channel is identified as being occupied, see TS 37.213 [18].

Msg3: Message transmitted on UL-SCH containing a C-RNTI MAC CE or CCCH SDU, submitted from upper layer and associated with the UE Contention Resolution Identity, as part of a Random Access procedure.

Multi-PUSCH configured grant: A configured grant configuration that includes multiple consecutive configured uplink grants within a single periodicity.

Non-terrestrial network: An NG-RAN consisting of gNBs, which provide non-terrestrial NR access to UEs by means of an NTN payload embarked on an airborne or space-borne NTN vehicle and an NTN Gateway.

NR backhaul link: NR link used for backhauling between an IAB-node and an IAB-donor, and between IAB-nodes in case of a multi-hop backhauling.

NR sidelink communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [19] and ProSe communication (including ProSe non-Relay and UE-to-Network Relay communication) as defined in TS 23.304 [26], between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink discovery: AS functionality enabling ProSe non-Relay discovery and ProSe UE-to-Network Relay discovery for Proximity based Services as defined in TS 23.304 [26], between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink transmission: Any NR Sidelink-based transmission, including both transmission for NR sidelink discovery and transmission for NR sidelink communication.

PDCCH occasion: A time duration (i.e. one or a consecutive number of symbols) during which the MAC entity is configured to monitor the PDCCH.

PRS Processing Window: A time window during which UE may perform PRS measurement inside the active DL BWP with the same numerology as the active DL BWP without measurement gap.

RedCap UE: A UE with reduced capabilities as specified in clause 4.2.21.1 in TS 38.306 [25].

Serving Cell: A PCell, a PSCell, or an SCell in TS 38.331 [5].

Sidelink transmission information: Sidelink transmission information included in an SCI for an SL-SCH transmission as specified in clause 8.3 and 8.4 of TS 38.212 [9] consists of Sidelink HARQ information including NDI, RV, Sidelink process ID, HARQ feedback enabled/disabled indicator, Sidelink identification information including cast type indicator, Source Layer-1 ID and Destination Layer-1 ID, and Sidelink other information including CSI request, a priority, a communication range requirement and Zone ID.

Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG depending on if the MAC entity is associated to the MCG or the SCG, respectively. Otherwise the term Special Cell refers to the PCell. A Special Cell supports PUCCH transmission and contention-based Random Access, and is always activated.

Timing Advance Group: A group of Serving Cells that is configured by RRC and that, for the cells with a UL configured, using the same timing reference cell and the same Timing Advance value. A Timing Advance Group containing the SpCell of a MAC entity is referred to as Primary Timing Advance Group (PTAG), whereas the term Secondary Timing Advance Group (STAG) refers to other TAGs.

UE-gNB RTT: For non-terrestrial networks, the sum of the UE's Timing Advance value (see TS 38.211 [8] clause 4.3.1) and kmac.

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [20], between nearby UEs, using E-UTRA technology but not traversing any network node.

NOTE 1:	A timer is running once it is started, until it is stopped or until it expires; otherwise it is not running. A timer can be started if it is not running or restarted if it is running. A Timer is always started or restarted from its initial value. The duration of a timer is not updated until it is stopped or expires (e.g. due to BWP switching). When the MAC entity applies zero value for a timer, the timer shall be started and immediately expire unless explicitly stated otherwise.

NOTE 2:	In this version of the specification, the SRS in the procedural description includes Positioning SRS except for the Positioning SRS for transmission in RRC_INACTIVE as in clause 5.26. Positioning SRS except for the Positioning SRS for transmission in RRC_INACTIVE is treated the same as SRS by the UE unless explicitly stated otherwise.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AP	Aperiodic

BFR	Beam Failure Recovery

BSR	Buffer Status Report

BWP	Bandwidth Part

CE	Control Element

CG	Cell Group

CG-SDT	Configured Grant-based SDT

CI-RNTI	Cancellation Indication RNTI

CSI	Channel State Information

CSI-IM	CSI Interference Measurement

CSI-RS	CSI Reference Signal

CS-RNTI	Configured Scheduling RNTI

DAPS	Dual Active Protocol Stack

DCP	DCI with CRC scrambled by PS-RNTI

DL-PRS	DownLink-Positioning Reference Signal

DSR	Delay Status Report

G-CS-RNTI	Group Configured Scheduling RNTI

G-RNTI	Group RNTI

IAB	Integrated Access and Backhaul

INT-RNTI	Interruption RNTI

LBT	Listen Before Talk

LCG	Logical Channel Group

LCP	Logical Channel Prioritization

MBS	Multicast/Broadcast Services

MCCH	MBS Control Channel

MCCH-RNTI	MBS Control Channel RNTI

MCG	Master Cell Group

MPE	Maximum Permissible Exposure

MTCH	MBS Traffic Channel

NCD-SSB	Non Cell Defining SSB

NSAG	Network Slice AS Group

NUL	Normal Uplink

NZP CSI-RS	Non-Zero Power CSI-RS

PDB	Packet Delay Budget

PEI-RNTI	Paging Early Indication RNTI

PHR	Power Headroom Report

PS-RNTI	Power Saving RNTI

PTAG	Primary Timing Advance Group

PTM	Point to Multipoint

PTP	Point to Point

QCL	Quasi-colocation

PPW	PRS Processing Window

PRS	Positioning Reference Signal

RA-SDT	Random Access-based SDT

RS	Reference Signal

SCG	Secondary Cell Group

SDT	Small Data Transmission

SFI-RNTI	Slot Format Indication RNTI

SI	System Information

SL-RNTI	Sidelink RNTI

SLCS-RNTI	Sidelink Configured Scheduling RNTI

SpCell	Special Cell

SP	Semi-Persistent

SP-CSI-RNTI	Semi-Persistent CSI RNTI

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SS	Synchronization Signals

SSB	Synchronization Signal Block

STAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TPC-SRS-RNTI	Transmit Power Control-Sounding Reference Signal-RNTI

TRIV	Time Resource Indicator Value

TRP	Transmit/Receive Point

TRS	CSI-RS for tracking

U2N	UE-to-Network

UCI	Uplink Control Information

V2X	Vehicle-to-Everything

ZP CSI-RS	Zero Power CSI-RS

Next Modified Subclause

### 5.4.1	UL Grant reception

Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, configured semi-persistently by RRC or determined to be associated with the PUSCH resource of MSGA as specified in clause 5.1.2a. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers. An uplink grant addressed to CS-RNTI with NDI = 0 is considered as a configured uplink grant. An uplink grant addressed to CS-RNTI with NDI = 1 is considered as a dynamic uplink grant.

If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer or a running cg-SDT-TimeAlignmentTimer and for each grant received for this PDCCH occasion:

1>	if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or

1>	if an uplink grant has been received in a Random Access Response:

2>	if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant:

3>	consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

2>	if the uplink grant is for MAC entity's C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured;

3>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running.

2>	stop the cg-SDT-RetransmissionTimer for the corresponding HARQ process, if running.

2>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

1>	else if an uplink grant for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured;

3>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running;

3>	stop the cg-SDT-RetransmissionTimer for the corresponding HARQ process, if running;

3>	deliver the uplink grant and the associated HARQ information to the HARQ entity;

3>	if a logical channel associated with a DRB configured with survivalTimeStateSupport is multiplexed in the MAC PDU stored in the HARQ buffer for the corresponding HARQ process:

4>	trigger activation of PDCP duplication for all configured RLC entities of the DRB.

2>	else if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate configured grant Type 2 deactivation:

4>	trigger configured uplink grant confirmation.

3>	else if PDCCH contents indicate configured grant Type 2 activation:

4>	trigger configured uplink grant confirmation;

4>	store the uplink grant for this Serving Cell and the associated HARQ information as configured uplink grant;

4>	initialise or re-initialise the configured uplink grant for this Serving Cell to start in the associated PUSCH duration and to recur according to rules in clause 5.8.2;

4>	stop the configuredGrantTimer for the corresponding HARQ process, if running;

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running.

For each Serving Cell and each configured uplink grant, if configured and activated and has not been indicated by the MAC entity to the lower layers as to be unused for PUSCH transmission, the MAC entity shall:

1>	if the MAC entity is configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or the PUSCH duration of a MSGA payload for this Serving Cell; or

- if the MAC entity is not configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response or the PUSCH duration of a MSGA payload for this Serving Cell:

2>	set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;

2>	if, for the corresponding HARQ process, the configuredGrantTimer is not running and cg-RetransmissionTimer is not configured and cg-SDT-RetransmissionTimer is not configured (i.e. new transmission):

3>	if there is an on-going CG-SDT procedure and PDCCH addressed to the MAC entity's C-RNTI has been received; or

3>	if there is no on-going CG-SDT procedure:

4>	consider the NDI bit for the corresponding HARQ process to have been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the cg-RetransmissionTimer for the corresponding HARQ process is configured and not running, then for the corresponding HARQ process:

3>	if the configuredGrantTimer is not running, and the HARQ process is not pending (i.e. new transmission):

4>	consider the NDI bit to have been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

3>	else if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant (i.e. retransmission on configured grant):

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the cg-SDT-RetransmissionTimer is configured and not running for the corresponding HARQ process;

3>	if the configured uplink grant is for the initial transmission for the CG-SDT with CCCH message (i.e., initial new transmission); or

3>	if the configuredGrantTimer is not running or not configured, and PDCCH addressed to the MAC entity's C-RNTI has been received after the initial transmission of the CG-SDT with CCCH message (i.e., subsequent new transmission):

4>	consider the NDI bit to have been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

3>	else if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant for initial transmission of CG-SDT with CCCH message or for its retransmission; and

3>	if PDCCH addressed to the MAC entity's C-RNTI has not been received (i.e., retransmission for initial CG-SDT transmission):

4>	consider the NDI bit to have not been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

For configured uplink grants that are not part of a multi-PUSCH configured grant and neither configured with harq-ProcID-Offset2 nor with cg-RetransmissionTimer, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

For configured uplink grants that are not part of a multi-PUSCH configured grant and configured with harq-ProcID-Offset2, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

For a multi-PUSCH configured grant (as specified in clause 5.8.2) configured with neither harq-ProcID-Offset2 nor cg-RetransmissionTimer, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [numberOfPUSCH-PerPeriod × floor (CURRENT_symbol/periodicity) + ID_OFFSET] modulo nrofHARQ-Processes

For a multi-PUSCH configured grant configured with harq-ProcID-Offset2, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [numberOfPUSCH-PerPeriod × floor (CURRENT_symbol/periodicity) + ID_OFFSET] modulo nrofHARQ-Processes + harq-ProcID-Offset2

where CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8]. For a multi-PUSCH configured grant, ID_OFFSET equals 0 for the first configured uplink grant within a periodicity of the configuration and K for the Kth (1 ≤ K < numberOfPUSCH_PerPeriod) valid configured uplink grant after the first configured uplink grant within the same periodicity. A configured uplink grant in a multi-PUSCH configured grant is not considered valid if it satisfies the conditions specified in clause x.x.x in TS 38.214 [7].

Editor’s note:  The reference for the validality of a CG occasion is to be provided by RAN1.

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection among initial transmission and retransmission with equal priority, the UE shall prioritize retransmissions before initial transmissions. The priority of a HARQ Process for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than the priority of a HARQ Process for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU. If the MAC entity is not configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize retransmissions before initial transmissions. The UE shall toggle the NDI in the CG-UCI for new transmissions and not toggle the NDI in the CG-UCI in retransmissions.

NOTE 1:	CURRENT_symbol refers to the symbol index of the first transmission occasion of a bundle of configured uplink grant.

NOTE 2:	A HARQ process is configured for a configured uplink grant where neither harq-ProcID-Offset nor harq-ProcID-Offset2 is configured, if the configured uplink grant is activated and the associated HARQ process ID is less than nrofHARQ-Processes. A HARQ process is configured for a configured uplink grant where harq-ProcID-Offset2 is configured, if the configured uplink grant is activated and the associated HARQ process ID is greater than or equal to harq-ProcID-Offset2 and less than sum of harq-ProcID-Offset2 and nrofHARQ-Processes for the configured grant configuration.

NOTE 3:	If the MAC entity receives a grant in a Random Access Response (i.e. MAC RAR or fallbackRAR), or addressed to Temporary C-RNTI or determines a grant as specified in clause 5.1.2a for MSGA payload and if the MAC entity also receives an overlapping grant for its C-RNTI or CS-RNTI, requiring concurrent transmissions on the SpCell, the MAC entity may choose to continue with either the grant for its RA-RNTI/Temporary C-RNTI/MSGB-RNTI/the MSGA payload transmission or the grant for its C-RNTI or CS-RNTI.

NOTE 4:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the HARQ Process ID used for configured uplink grants.

NOTE 5:	If cg-RetransmissionTimer is not configured, a HARQ process is not shared between different configured grant configurations in the same BWP.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

For the MAC entity configured with lch-basedPrioritization, if the corresponding PUSCH transmission of a configured uplink grant is cancelled by CI-RNTI as specified in clause 11.2A of TS 38.213 [6] or cancelled by a high PHY-priority PUCCH transmission as specified in clause 9 of TS 38.213 [6], this configured uplink grant is considered as a de-prioritized uplink grant. If this de-prioritized uplink grant is configured with autonomousTx, the configuredGrantTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running. If this de-prioritized uplink grant is configured with autonomousTx, the cg-RetransmissionTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running.

When the MAC entity is configured with lch-basedPrioritization, for each uplink grant delivered to the HARQ entity and whose associated PUSCH can be transmitted by lower layers, the MAC entity shall:

1>	if this uplink grant is received in a Random Access Response (i.e. in a MAC RAR or fallback RAR), or addressed to Temporary C-RNTI, or is determined as specified in clause 5.1.2a for the transmission of the MSGA payload:

2>	consider this uplink grant as a prioritized uplink grant.

1>	else if this uplink grant is addressed to CS-RNTI with NDI = 1 or C-RNTI:

2>	if there is no overlapping PUSCH duration of a configured uplink grant which was not already de-prioritized, in the same BWP, whose priority is higher than the priority of the uplink grant; and

2>	if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups, and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

3>	consider this uplink grant as a prioritized uplink grant;

3>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s);

3>	consider the other overlapping SR transmission(s), if any, as a de-prioritized SR transmission(s), except for the SR transmission(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups;

3>	if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started:

4>	stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s);

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

1>	else if this uplink grant is a configured uplink grant:

2>	if there is no overlapping PUSCH duration of another configured uplink grant which was not already de-prioritized, in the same BWP, whose priority is higher than the priority of the uplink grant; and

2>	if there is no overlapping PUSCH duration of an uplink grant addressed to CS-RNTI with NDI = 1 or C-RNTI which was not already de-prioritized, in the same BWP, whose priority is higher than or equal to the priority of the uplink grant; and

2>	if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups, and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

3>	consider this uplink grant as a prioritized uplink grant;

3>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s);

3>	if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started:

4>	stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s);

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

3>	consider the other overlapping SR transmission(s), if any, as a de-prioritized SR transmission(s), except for the SR transmission(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups.

NOTE 6:	If the MAC entity is configured with lch-basedPrioritization and if there is overlapping PUSCH duration of at least two configured uplink grants whose priorities are equal, the prioritized uplink grant is determined by UE implementation.

NOTE 7:	If the MAC entity is not configured with lch-basedPrioritization and if there is overlapping PUSCH duration of at least two configured uplink grants, it is up to UE implementation to choose one of the configured uplink grants.

NOTE 8:	If the MAC entity is configured with lch-basedPrioritization, the MAC entity does not take UCI multiplexing according to the procedure specified in TS 38.213 [6] into account when determining whether the PUSCH duration of an uplink grant overlaps with the PUCCH resource for an SR transmission.

Next Modified Subclause

## 5.7	Discontinuous Reception (DRX)

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, AI-RNTI, SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

NOTE 1:	Void

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL, DL or SL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle start;

- 	drx-NonIntegerLongCycleStartOffset (optional): the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle start, when the length of the Long DRX cycle and/or the short DRX cycle is a rational number;

-	drx-ShortCycle (optional): the Short DRX cycle;

- 	drx-NonIntegerShortCycle (optional): the Short DRX cycle whose length is a rational number;

Editor’s note: It is FFS whether non-integer DRX cycles can be configured for both short and long DRX cycles or only one of them.

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity;

-	drx-RetransmissionTimerSL (per SL HARQ process): the maximum duration until a grant for SL retransmission is received;

-	drx-HARQ-RTT-TimerSL (per SL HARQ process): the minimum duration before an SL retransmission grant is expected by the MAC entity;

-	drx-LastTransmissionUL (optional): the configuration to start drx-HARQ-RTT-TimerUL after the last transmission within a bundle;

-	ps-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

-	ps-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	ps-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	downlinkHARQ-FeedbackDisabled (optional): the configuration to disable HARQ feedback per DL HARQ process;

-	uplinkHARQ-Mode (optional): the configuration to set HARQmodeA or HARQmodeB per UL HARQ process;

-	disableCG-RetransmissionMonitoring (optional): the configuration to disable starting drx-HARQ-RTT-TimerUL for UL transmission over a configured uplink grant;

-	drx-TimeReferenceSFN (optional): the reference SFN used in determining the start time of DRX on durations when short and/or long DRX cycle is a rational number.

The following UE variable is used for the DRX operation if drx-NonIntegerLongCycleStartOffset and/or drx-NonIntegerShortCycle is configured:

-	DRX_SFN_COUNTER: the counter that increments when SFN changes to 0. This counter can be implemented with a maximum value of 65536.

Serving Cells of a MAC entity may be configured by RRC in two DRX groups with separate DRX parameters. When RRC does not configure a secondary DRX group, there is only one DRX group and all Serving Cells belong to that one DRX group. When two DRX groups are configured, each Serving Cell is uniquely assigned to either of the two groups. The DRX parameters that are separately configured for each DRX group are: drx-onDurationTimer, drx-InactivityTimer. The DRX parameters that are common to the DRX groups are: drx-SlotOffset, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-LongCycleStartOffset, drx-ShortCycle (optional), drx-ShortCycleTimer (optional), drx-HARQ-RTT-TimerDL, and drx-HARQ-RTT-TimerUL.

Editor’s note: FFS whether the new DRX parameters such as drx-NonIntegerLongCycleStartOffset and drx-NonIntegerShortCycle are shared by both DRX groups or can be configured separately for different DRX groups.

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or

-	drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a).

The following MAC timers are used for DRX operation in a non-terrestrial network:

-	HARQ-RTT-TimerDL-NTN (per DL HARQ process configured with HARQ feedback enabled): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	HARQ-RTT-TimerUL-NTN (per UL HARQ process configured with HARQModeA): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When DRX is not configured and multicast DRX is configured for a G-RNTI or G-CS-RNTI, the MAC entity shall:

1>	monitor the PDCCH as specified in TS 38.213 [6];

1>	if a MAC PDU is received in a configured downlink assignment for unicast; or

1>	if the PDCCH indicates a DL unicast transmission:

2>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process.

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment for unicast:

2>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

3>	if the corresponding HARQ process is configured with HARQ feedback enabled:

4>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

4>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

2>	else:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 1a:	Void.

NOTE 1b:	Void.

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process;

2>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers:

2>	if this Serving Cell is configured with uplinkHARQ-Mode:

3>	if the corresponding HARQ process is configured as HARQModeA:

4>	set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value;

4>	if drx-LastTransmissionUL is configured:

5>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

2>	else:

3>	if disableCG-RetransmissionMonitoring is not configured for the configured uplink grant:

4>	if drx-LastTransmissionUL is configured:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process at the first transmission (within a bundle) of the corresponding PUSCH transmission.

1>	if a MAC PDU is transmitted in a configured sidelink grant:

2>	if the PUCCH resource is configured:

3>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH transmission carrying the SL HARQ feedback; or

3>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH resource for the SL HARQ feedback when the PUCCH is not transmitted;

3>	stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

2>	else:

3>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process at the first symbol after the end of the corresponding PSSCH transmission;

3>	stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.

1>	if a HARQ-RTT-TimerDL-NTN expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of HARQ-RTT-TimerDL-NTN.

1>	if a drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

1>	if a HARQ-RTT-TimerUL-NTN expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of HARQ-RTT-TimerUL-NTN.

1>	if a drx-HARQ-RTT-TimerSL expires:

2>	if a HARQ NACK feedback for the corresponding HARQ process is transmitted on PUCCH; or

2>	if a HARQ NACK feedback for the corresponding HARQ process is generated but not transmitted on PUCCH; or

2>	if the PUCCH resource is not configured for the SL grant:

3>	start the drx-RetransmissionTimerSL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerSL.

NOTE 1c:	The UE handles the drx-RetransmissionTimerSL operation when sl-PUCCH-Config is configured by RRC but PUCCH resource is not scheduled same as when sl-PUCCH-Config is not configured.

1>	if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer for each DRX group;

2>	stop drx-InactivityTimer for each DRX group.

1>	if drx-InactivityTimer for a DRX group expires:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer for this DRX group in the first symbol after the expiry of drx-InactivityTimer;

3>	use the Short DRX cycle for this DRX group.

2>	else:

3>	use the Long DRX cycle for this DRX group.

1>	if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer for each DRX group in the first symbol after the end of DRX Command MAC CE reception;

3>	use the Short DRX cycle for each DRX group.

2>	else:

3>	use the Long DRX cycle for each DRX group.

1>	if drx-ShortCycleTimer for a DRX group expires:

2>	use the Long DRX cycle for this DRX group.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer for each DRX group;

2>	use the Long DRX cycle for each DRX group.

1> if the drx-NonIntegerLongCycle is used for a DRX group:

2> set DRX_SFN_COUNTER to 0 upon successful reception of RRC (re-)configuration for DRX;

- increment DRX_SFN_COUNTER by 1 in the first symbol of a slot in which SFN changes to 0;

1>	if the Short DRX cycle is used for a DRX group and the drx-NonIntegerShortCycle is not configured for the DRX group, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

- if the Short DRX cycle is used for a DRX group and the drx-NonIntegerShortCycle is configured for the DRX group, and [(DRX_SFN_COUNTER  × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerShortCycle) = [(drx-TimeReferenceSFN × 10) + drx-StartOffset] modulo (drx-NonIntegerShortCycle):

2>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

Editor’s note:  Whether drx-NonIntegerShortCycle can be configured and the final formula for determining the start time of the drx-onDurationTimer when the drx-NonIntegerShortCycle is configured is pending further agreements.

1>	if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycle is not configured for the DRX group, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset; or

- if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycle is configured for the DRX group, and [(DRX_SFN_COUNTER  × 10240)  + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle) = [(drx-TimeReferenceSFN × 10) + drx-StartOffset] modulo (drx-NonIntegerLongCycle):

Editor’s note:  The final formula for determining the start time of the drx-onDurationTimer when the drx-NonIntegerLongCycle is used is pending further agreements.

2>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3:

3>	if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

3>	if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or

3>	if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers:

4>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

2>	else:

3>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration.

1>	if a DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission; or

2>	if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or

2>	if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]:

3>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

4>	if the corresponding HARQ process is configured with HARQ feedback enabled:

5>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

5>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

3>	else:

4>	start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 3:	When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating an inapplicable k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback.

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported;

3>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

3>	if the PDSCH-to-HARQ_feedback timing indicate an inapplicable k1 value as specified in TS 38.213 [6]:

4>	start the drx-RetransmissionTimerDL in the first symbol after the (end of the last) PDSCH transmission (within a bundle) for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	if this Serving Cell is configured with uplinkHARQ-Mode:

4>	if the corresponding HARQ process is configured as HARQModeA:

5>	set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value;

5>	if drx-LastTransmissionUL is configured:

6>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

5>	else:

6>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

3>	else:

4>	if drx-LastTransmissionUL is configured:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates an SL transmission:

3>	if the PUCCH resource is configured:

4>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH transmission carrying the SL HARQ feedback; or

4>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH resource for the SL HARQ feedback when the PUCCH is not transmitted;

4>	stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

3>	else:

4>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process at the first symbol after end of PDCCH occasion;

4>	stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL, UL or SL) on a Serving Cell in this DRX group:

3>	start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH reception.

NOTE 3a:	A PDCCH indicating activation of SPS, configured grant type 2, or configured sidelink grant of configured grant Type 2 is considered to indicate a new transmission.

NOTE 3b:	If the PDCCH reception includes two PDCCH candidates from corresponding search spaces, as described in clause 10.1 in 38.213, start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH candidate that ends later in time.

2>	if a HARQ process receives downlink feedback information and acknowledgement is indicated:

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and

1>	if the current symbol n occurs within drx-onDurationTimer duration; and

1>	if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause:

2>	if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

2>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or if all multicast DRXes would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions are configured with multicast DRX:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7];

3>	not report semi-persistent CSI configured on PUSCH;

3>	if ps-TransmitPeriodicL1-RSRP is not configured with value true:

4>	not report periodic CSI that is L1-RSRP on PUCCH.

3>	if ps-TransmitOtherPeriodicCSI is not configured with value true:

4>	not report periodic CSI that is not L1-RSRP on PUCCH.

1>	else:

2>	in current symbol n, if a DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

2>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if all multicast DRXes corresponding to the DRX group would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group;

3>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group.

2>	if CSI masking (csi-Mask) is setup by upper layers:

3>	in current symbol n, if drx-onDurationTimer of a DRX group would not be running considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

3>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if drx-onDurationTimerPTM(s) of all multicast DRXes corresponding to the DRX group would not be running considering DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX:

4>	not report CSI on PUCCH in this DRX group.

NOTE 4:	If a UE multiplexes a CSI configured on PUCCH with other overlapping UCI(s) according to the procedure specified in TS 38.213 [6] clause 9.2.5 and this CSI multiplexed with other UCI(s) would be reported on a PUCCH resource either outside DRX Active Time of the DRX group in which this PUCCH is configured or outside the on-duration period of the DRX group in which this PUCCH is configured if CSI masking is setup by upper layers, it is up to UE implementation whether to report this CSI multiplexed with other UCI(s).

Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in the DRX group when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

Next Modified Subclause

### 5.8.2	Uplink

There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	configured grant Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating configured uplink grant activation or deactivation.

Type 1 and Type 2 are configured by RRC for a Serving Cell per BWP. Multiple configurations can be active simultaneously in the same BWP. For Type 2, activation and deactivation are independent among the Serving Cells. For the same BWP, the MAC entity can be configured with both Type 1 and Type 2.

A multi-PUSCH configured grant has multiple consecutive configured uplink grants within a periodicity. Both Type 1 and Type 2 can be configured for a multi-PUSCH configured grant by RRC.

Editor’s note:  This change is based on RAN1’s agreement. It needs to be confirmed by RAN2.

Only configured grant Type 1 can be configured for CG-SDT. CG-SDT can only be configured on initial BWP.

RRC configures the following parameters when the configured grant Type 1 is configured:

-	cs-RNTI: CS-RNTI for retransmission;

-	cg-SDT-CS-RNTI: CS-RNTI for CG-SDT retransmission;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT;

-	periodicity: periodicity of the configured grant Type 1;

-	timeDomainOffset: Offset of a resource with respect to SFN = timeReferenceSFN in time domain;

-	timeDomainAllocation: Allocation of configured uplink grant in time domain which contains startSymbolAndLength (i.e. SLIV in TS 38.214 [7]) or startSymbol (i.e. S in TS 38.214 [7]);

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer;

-	timeReferenceSFN: SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration.

RRC configures the following parameters when the configured grant Type 2 is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	periodicity: periodicity of the configured grant Type 2;

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer.

RRC configures the following parameter when retransmissions on configured uplink grant is configured:

-	cg-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process when the UE shall not autonomously retransmit that HARQ process;

-	cg-SDT-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process of the initial CG-SDT transmission with CCCH message when the UE shall not autonomously retransmit the HARQ process.

RRC configures the following additional parameter for a multi-PUSCH configured grant:

- numberOfPUSCH-PerPeriod: the number of configured uplink grants in a periodicity of a multi-PUSCH configured grant.

Upon configuration of a configured grant Type 1 for a BWP of a Serving Cell by upper layers, the MAC entity shall:

1>	store the uplink grant provided by upper layers as a configured uplink grant for the indicated BWP of the Serving Cell;

1>	initialise or re-initialise the configured uplink grant to start in the symbol according to timeDomainOffset, timeReferenceSFN, and S (derived from SLIV or provided by startSymbol as specified in TS 38.214 [7]), and to reoccur with periodicity.

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the configured uplink grant, or the first configured uplink grant in a multi-PUSCH configured grant, in the Nth (N ≥ 0) periodicity occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	(timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity)
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

For a multi-PUSCH configured grant Type 1, the Kth (1 < K ≤ numberOfPUSCH-PerPeriod) configured uplink grant within a periodicity occurs (K-1) × numberOfSymbolsPerSlot symbols after the symbol in which the first configured uplink grant in that periodicity occurs.

Editor’s Note: This change is based on RAN1’s agreement. It needs to be confirmed by RAN2.

For an uplink grant configured for configured grant Type 1 for CG-SDT on the selected uplink carrier as in clause 5.27, when CG-SDT is triggered and not terminated, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if, after initial transmission for CG-SDT with CCCH message has been performed according to clause 5.4.1, PDCCH addressed to the MAC entity's C-RNTI has not been received:

2>	if the SSB corresponding to the configured UL grant has the same SSB index as the SSB selected for initial transmission for CG-SDT with CCCH message (i.e., retransmission of initial transmission of CG-SDT):

3>	select this SSB;

3>	indicate the SSB index corresponding to the configured uplink grant to the lower layer;

3>	consider this configured uplink grant as valid.

1>	else if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

2>	if at least one SSB corresponding to the configured uplink grant with SS-RSRP above the cg-SDT-RSRP-ThresholdSSB is available:

3>	if this is the initial transmission of CG-SDT with CCCH message after the CG-SDT procedure is initiated as in clause 5.27 (i.e., initial transmission for CG-SDT):

4>	select an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant.

3>	else if PDCCH addressed to C-RNTI has been received after the initial transmission of CG-SDT with CCCH message (i.e., subsequent new transmission for CG-SDT):

4>	if SS-RSRP of the SSB selected for the previous transmission for CG-SDT is above cg-SDT-RSRP-ThresholdSSB and this SSB is associated with this configured uplink grant:

5>	select this SSB.

4>	else if SS-RSRP of the SSB selected for the previous transmission for CG-SDT is not above cg-SDT-RSRP-ThresholdSSB:

5>	select an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant.

3>	if SSB is selected above:

4>	indicate the SSB index to the lower layer;

4>	consider this configured uplink grant as valid.

1>	else:

2>	consider this configured uplink grant as not valid.

2>	if PDCCH addressed to C-RNTI after the initial transmission of the CG-SDT with CCCH message has been received:

3>	if there is data available for transmission for at least one RB configured for SDT:

4>	initiate Random Access procedure in clause 5.1.

NOTE 1:	When the UE determines if there is an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider sequentially that the configured uplink grant, or the first configured uplink grant in a multi-PUSCH configured grant, in the Nth (N ≥ 0) periodicity occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity]
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, of the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised.

For a multi-PUSCH configured grant Type 2, the Kth (1 < K ≤ numberOfPUSCH-PerPeriod) configured uplink grant within the same periodicity occurs (K-1) × numberOfSymbolsPerSlot symbols after the symbol in which the first configured uplink grant in that periodicity occurs.

Editor’s Note: This change is based on RAN1’s agreement. It needs to be confirmed by RAN2.

If cg-nrofPUSCH-InSlot or cg-nrofSlots is configured for a configured grant Type 1 or Type 2, the MAC entity shall consider the uplink grants occur in those additional PUSCH allocations as specified in clause 6.1.2.3 of TS 38.214 [7].

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the occurrences of configured uplink grants.

If the MAC entity determines that a configured uplink grant is not going to be used for PUSCH transmission, it sends an indication to lower layers about this decision.

Editor’s note:  	FFS how the MAC entity determines whether a configured uplink grant is going to be used for PUSCH transmission or not.

When the configured uplink grant is released by upper layers, all the corresponding configurations shall be released and all corresponding uplink grants shall be cleared.

The MAC entity shall:

1>	if at least one configured uplink grant confirmation has been triggered and not cancelled; and

1>	if the MAC entity has UL resources allocated for new transmission:

2>	if, in this MAC entity, at least one configured uplink grant is configured by configuredGrantConfigToAddModList:

3>	instruct the Multiplexing and Assembly procedure to generate a Multiple Entry Configured Grant Confirmation MAC CE as defined in clause 6.1.3.31.

2>	else:

3>	instruct the Multiplexing and Assembly procedure to generate a Configured Grant Confirmation MAC CE as defined in clause 6.1.3.7.

2>	cancel all triggered configured uplink grant confirmation(s).

For a configured grant Type 2, the MAC entity shall clear the configured uplink grant(s) immediately after first transmission of Configured Grant Confirmation MAC CE or Multiple Entry Configured Grant Confirmation MAC CE which confirms the configured uplink grant deactivation.

Retransmissions use:

-	repetition of configured uplink grants; or

-	received uplink grants addressed to CS-RNTI; or

-	configured uplink grants with cg-RetransmissionTimer or cg-SDT-RetransmissionTimer configured.

Next Modified Subclause (new)

## 5.4.X	Delay status reporting

The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of UL data. This delay status includes remaining time of UL data, which is based on the value of its associated PDCP discardTimer (as specified in clause 7.3 in TS 38.323 [4]) at the time of the first symbol of the PUSCH transmission in which the DSR is sent, as well as the amount of data associated with the reported remaining time.

RRC controls the DSR operation by configuring the following parameter:

-	remainingTimeThreshold: the threshold on remaining time of UL data configured for triggering DSR for an LCG.

The MAC entity triggers a DSR when the remaining time of a PDU in an LCG, if configured for DSR, becomes shorter than its associated remainingTimeThreshold.

Editor’s note:  This section will be updated after more agreements are made, e.g. a more precise definition on “the amount of data associated with the reported remaining time” needs to be specified. FFS Definition of remaining time. FFS if one or more values are reported for an LCG. FFS if data with delay below the threshold is reported.

Next Modified Subclause (new)

#### 6.1.3.1a (TBD)

Editor’s note:   It is FFS whether a new, enhanced BSR MAC is needed or not to support the use of new BSR table.

#### For a logical channel group, network can configure whether its buffer status is reported based on either Table 6.1.3.1-2 or Table 6.1.3.1a-x.

#### Editor’s Note:  It is FFS how the MAC entity determines which BSR table a logical channel group is eligble to use.

Table 6.1.3.1a-x: Buffer size levels (in bytes) in the new BSR table

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | 64 |  | 128 |  | 192 |  |
| 1 |  | 65 |  | 129 |  | 193 |  |
| 2 |  | 66 |  | 130 |  | 194 |  |
| 3 |  | 67 |  | 131 |  | 195 |  |
| 4 |  | 68 |  | 132 |  | 196 |  |
| 5 |  | 69 |  | 133 |  | 197 |  |
| 6 |  | 70 |  | 134 |  | 198 |  |
| 7 |  | 71 |  | 135 |  | 199 |  |
| 8 |  | 72 |  | 136 |  | 200 |  |
| 9 |  | 73 |  | 137 |  | 201 |  |
| 10 |  | 74 |  | 138 |  | 202 |  |
| 11 |  | 75 |  | 139 |  | 203 |  |
| 12 |  | 76 |  | 140 |  | 204 |  |
| 13 |  | 77 |  | 141 |  | 205 |  |
| 14 |  | 78 |  | 142 |  | 206 |  |
| 15 |  | 79 |  | 143 |  | 207 |  |
| 16 |  | 80 |  | 144 |  | 208 |  |
| 17 |  | 81 |  | 145 |  | 209 |  |
| 18 |  | 82 |  | 146 |  | 210 |  |
| 19 |  | 83 |  | 147 |  | 211 |  |
| 20 |  | 84 |  | 148 |  | 212 |  |
| 21 |  | 85 |  | 149 |  | 213 |  |
| 22 |  | 86 |  | 150 |  | 214 |  |
| 23 |  | 87 |  | 151 |  | 215 |  |
| 24 |  | 88 |  | 152 |  | 216 |  |
| 25 |  | 89 |  | 153 |  | 217 |  |
| 26 |  | 90 |  | 154 |  | 218 |  |
| 27 |  | 91 |  | 155 |  | 219 |  |
| 28 |  | 92 |  | 156 |  | 220 |  |
| 29 |  | 93 |  | 157 |  | 221 |  |
| 30 |  | 94 |  | 158 |  | 222 |  |
| 31 |  | 95 |  | 159 |  | 223 |  |
| 32 |  | 96 |  | 160 |  | 224 |  |
| 33 |  | 97 |  | 161 |  | 225 |  |
| 34 |  | 98 |  | 162 |  | 226 |  |
| 35 |  | 99 |  | 163 |  | 227 |  |
| 36 |  | 100 |  | 164 |  | 228 |  |
| 37 |  | 101 |  | 165 |  | 229 |  |
| 38 |  | 102 |  | 166 |  | 230 |  |
| 39 |  | 103 |  | 167 |  | 231 |  |
| 40 |  | 104 |  | 168 |  | 232 |  |
| 41 |  | 105 |  | 169 |  | 233 |  |
| 42 |  | 106 |  | 170 |  | 234 |  |
| 43 |  | 107 |  | 171 |  | 235 |  |
| 44 |  | 108 |  | 172 |  | 236 |  |
| 45 |  | 109 |  | 173 |  | 237 |  |
| 46 |  | 110 |  | 174 |  | 238 |  |
| 47 |  | 111 |  | 175 |  | 239 |  |
| 48 |  | 112 |  | 176 |  | 240 |  |
| 49 |  | 113 |  | 177 |  | 241 |  |
| 50 |  | 114 |  | 178 |  | 242 |  |
| 51 |  | 115 |  | 179 |  | 243 |  |
| 52 |  | 116 |  | 180 |  | 244 |  |
| 53 |  | 117 |  | 181 |  | 245 |  |
| 54 |  | 118 |  | 182 |  | 246 |  |
| 55 |  | 119 |  | 183 |  | 247 |  |
| 56 |  | 120 |  | 184 |  | 248 |  |
| 57 |  | 121 |  | 185 |  | 249 |  |
| 58 |  | 122 |  | 186 |  | 250 |  |
| 59 |  | 123 |  | 187 |  | 251 |  |
| 60 |  | 124 |  | 188 |  | 252 |  |
| 61 |  | 125 |  | 189 |  | 253 |  |
| 62 |  | 126 |  | 190 |  | 254 |  |
| 63 |  | 127 |  | 191 |  | 255 |  |

Editor’s note:   The entries in the table will be populated after RAN2 agree on the range and distribution of buffer sizes.

Next Modified Subclause (new)

#### 6.1.3.x	Delay Status Report MAC CE

The Delay Status Report (DSR) MAC CE is identified by MAC subheader with an (e)LCID as specified in Table 6.2.1-2.

Editor’s note:   It is FFS whether the DSR MAC CE uses a legacy LCID or one-octet eLCID.

The fields in the DSR MAC CE are defined as follows:

Editor’s note:   It is FFS the fields included in the DSR MAC CE and their respective definitions.

Next Modified Subclause

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1, 6.2.1-1c and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The size of the LCID field is 6 bits. If the LCID field is set to 34, one additional octet is present in the MAC subheader containing the eLCID field and follow the octet containing LCID field. If the LCID field is set to 33, two additional octets are present in the MAC subheader containing the eLCID field and these two additional octets follow the octet containing LCID field;

NOTE 1:	For MBS broadcast, a logical channel is identified based on G-RNTI and LCID if the same LCID is allocated for logical channels corresponding to different G-RNTIs.

-	eLCID: The extended Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE as described in tables 6.2.1-1a, 6.2.1-1b, 6.2.1-2a and 6.2.1-2b for the DL-SCH and UL-SCH respectively. The size of the eLCID field is either 8 bits or 16 bits.

NOTE 2:	The extended Logical Channel ID space using two-octet eLCID and the relevant MAC subheader format is used, only when configured, on the NR backhaul links between IAB nodes or between IAB node and IAB Donor, or for multicast MTCHs.

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to 0.

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel of DCCH, DTCH and multicast MTCH |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35–46 | Reserved |
| 47 | Recommended bit rate |
| 48 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 49 | PUCCH spatial relation Activation/Deactivation |
| 50 | SP SRS Activation/Deactivation |
| 51 | SP CSI reporting on PUCCH Activation/Deactivation |
| 52 | TCI State Indication for UE-specific PDCCH |
| 53 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 54 | Aperiodic CSI Trigger State Subselection |
| 55 | SP CSI-RS/CSI-IM Resource Set Activation/Deactivation |
| 56 | Duplication Activation/Deactivation |
| 57 | SCell Activation/Deactivation (four octets) |
| 58 | SCell Activation/Deactivation (one octet) |
| 59 | Long DRX Command |
| 60 | DRX Command |
| 61 | Timing Advance Command |
| 62 | UE Contention Resolution Identity |
| 63 | Padding |

Table 6.2.1-1a Values of two-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to (216 – 1) | 320 to (216 + 319) | Identity of the logical channel |

Table 6.2.1-1b Values of one-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 226 | 64 to 290 | Reserved |
| 227 | 291 | Serving Cell Set based SRS TCI State Indication MAC CE |
| 228 | 292 | SP/AP SRS TCI State Indication MAC CE |
| 229 | 293 | BFD-RS Indication MAC CE |
| 230 | 294 | Differential Koffset |
| 231 | 295 | Enhanced SCell Activation/Deactivation MAC CE with one octet Ci field |
| 232 | 296 | Enhanced SCell Activation/Deactivation MAC CE with four octet Ci field |
| 233 | 297 | Unified TCI States Activation/Deactivation MAC CE |
| 234 | 298 | PUCCH Power Control Set Update for multiple TRP PUCCH repetition MAC CE |
| 235 | 299 | PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE |
| 236 | 300 | Enhanced TCI States Indication for UE-specific PDCCH |
| 237 | 301 | Positioning Measurement Gap Activation/Deactivation Command |
| 238 | 302 | PPW Activation/Deactivation Command |
| 239 | 303 | DL Tx Power Adjustment |
| 240 | 304 | Timing Case Indication |
| 241 | 305 | Child IAB-DU Restricted Beam Indication |
| 242 | 306 | Case-7 Timing advance offset |
| 243 | 307 | Provided Guard Symbols for Case-6 timing |
| 244 | 308 | Provided Guard Symbols for Case-7 timing |
| 245 | 309 | Serving Cell Set based SRS Spatial Relation Indication |
| 246 | 310 | PUSCH Pathloss Reference RS Update |
| 247 | 311 | SRS Pathloss Reference RS Update |
| 248 | 312 | Enhanced SP/AP SRS Spatial Relation Indication |
| 249 | 313 | Enhanced PUCCH Spatial Relation Activation/Deactivation |
| 250 | 314 | Enhanced TCI States Activation/Deactivation for UE-specific PDSCH |
| 251 | 315 | Duplication RLC Activation/Deactivation |
| 252 | 316 | Absolute Timing Advance Command |
| 253 | 317 | SP Positioning SRS Activation/Deactivation |
| 254 | 318 | Provided Guard Symbols |
| 255 | 319 | Timing Delta |

Table 6.2.1-1c Values of LCID for MBS broadcast on DL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | MCCH |
| 1–32 | Identity of the logical channel of broadcast MTCH |
| 33–63 | Reserved |

Table 6.2.1-2 Values of LCID for UL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]), except for a RedCap UE |
| 1–32 | Identity of the logical channel of DCCH and DTCH |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) for a RedCap UE |
| 36 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) for a RedCap UE |
| 37–42 | Reserved |
| 43 | Truncated Enhanced BFR (one octet Ci) |
| 44 | Timing Advance Report |
| 45 | Truncated Sidelink BSR |
| 46 | Sidelink BSR |
| 47 | Reserved |
| 48 | LBT failure (four octets) |
| 49 | LBT failure (one octet) |
| 50 | BFR (one octet Ci) |
| 51 | Truncated BFR (one octet Ci) |
| 52 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]), except for a RedCap UE |
| 53 | Recommended bit rate query |
| 54 | Multiple Entry PHR (four octets Ci) |
| 55 | Configured Grant Confirmation |
| 56 | Multiple Entry PHR (one octet Ci) |
| 57 | Single Entry PHR |
| 58 | C-RNTI |
| 59 | Short Truncated BSR |
| 60 | Long Truncated BSR |
| 61 | Short BSR |
| 62 | Long BSR |
| 63 | Padding |

Table 6.2.1-2a Values of two-octet eLCID for UL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to (216 – 1) | 320 to (216 + 319) | Identity of the logical channel |

Table 6.2.1-2b Values of one-octet eLCID for UL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 228 | 64 to 292 | Reserved |
| 229 | 293 | Enhanced Multiple Entry PHR for multiple TRP (four octets Ci) |
| 230 | 294 | Enhanced Multiple Entry PHR for multiple TRP (one octets Ci) |
| 231 | 295 | Enhanced Single Entry PHR for multiple TRP |
| 232 | 296 | Enhanced Multiple Entry PHR (four octets Ci) |
| 233 | 297 | Enhanced Multiple Entry PHR (one octets Ci) |
| 234 | 298 | Enhanced Single Entry PHR |
| 235 | 299 | Enhanced BFR (one octet Ci) |
| 236 | 300 | Enhanced BFR (four octet Ci) |
| 237 | 301 | Truncated Enhanced BFR (four octet Ci) |
| 238 | 302 | Positioning Measurement Gap Activation/Deactivation Request |
| 239 | 303 | IAB-MT Recommended Beam Indication |
| 240 | 304 | Desired IAB-MT PSD range |
| 241 | 305 | Desired DL Tx Power Adjustment |
| 242 | 306 | Case-6 Timing Request |
| 243 | 307 | Desired Guard Symbols for Case 6 timing |
| 244 | 308 | Desired Guard Symbols for Case 7 timing |
| 245 | 309 | Extended Short Truncated BSR |
| 246 | 310 | Extended Long Truncated BSR |
| 247 | 311 | Extended Short BSR |
| 248 | 312 | Extended Long BSR |
| 249 | 313 | Extended Pre-emptive BSR |
| 250 | 314 | BFR (four octets Ci) |
| 251 | 315 | Truncated BFR (four octets Ci) |
| 252 | 316 | Multiple Entry Configured Grant Confirmation |
| 253 | 317 | Sidelink Configured Grant Confirmation |
| 254 | 318 | Desired Guard Symbols |
| 255 | 319 | Pre-emptive BSR |

Editor’s note: It is FFS which LCID or eLCID should be assigned to the DSR MAC CE.

End of Changes