3GPP TSG-RAN WG2 Meeting #123		R2-230xxx

Toulouse, France, 21st – 25th Aug. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.322 | CR | DraftCR | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running RLC CR for XR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2023-08-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18, new functionalities were agreed for NR to provide enhanced support of XR services, which require RLC protocol modifications. This is a draft of the running RLC CR for XR. To be updated based on the progress on XR in the following meetings. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of Delay Status Report. This CR captures the RLC aspects of XR and it is based on RAN2 agreements made so far, which could be found in Annex at the end of this document. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR enhancements related to XR services cannot be supported in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.300 CR TBD TS/TR 38.306 CR TBD TS/TR 38.321 CR TBD TS/TR 38.323 CR TBD |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR 38.331 CR TBD TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be lifted to the latest version of the specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the initial version of running CR for TS 38.322 for XR WI. |  |  |  |  |  |  |  |  |

Start of change

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Data field element: An RLC SDU or an RLC SDU segment that is mapped to the Data field.

NR sidelink communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [6] and ProSe communication (including ProSe non-Relay and UE-to-Network Relay communication) as defined in TS 23.304 [8], between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink discovery: AS functionality enabling ProSe non-Relay Discovery and ProSe UE-to-Network Relay discovery for Proximity based Services as defined in TS 23.304 [8] between two or more nearby UEs, using NR technology but not traversing any network node.

RLC data volume: The amount of data available for transmission in an RLC entity.

RLC SDU segment: A segment of an RLC SDU.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AM	Acknowledged Mode

AMD	AM Data

ARQ	Automatic Repeat request

DSR	Delay Status Report

gNB	NR Node B

MBS	Multicast/Broadcast Services

MCCH	MBS Control Channel

MTCH	MBS Traffic Channel

PDU	Protocol Data Unit

RLC	Radio Link Control

SBCCH	Sidelink Broadcast Control Channel

SCCH	Sidelink Control Channel

SDU	Service Data Unit

SN	Sequence Number

STCH	Sidelink Traffic Channel

TB	Transport Block

TM	Transparent Mode

TMD	TM Data

UE	User Equipment

UM	Unacknowledged Mode

UMD	UM Data

Editor’s NOTE:	The abbreviation of DSR will be aligned with other specifications (e.g. 38.321).

Next change

## 5.5	Data volume calculation

For the purpose of MAC buffer status reporting, the UE shall consider the following as RLC data volume:

-	RLC SDUs and RLC SDU segments that have not yet been included in an RLC data PDU;

-	RLC data PDUs that are pending for initial transmission;

-	RLC data PDUs that are pending for retransmission (RLC AM).

[For the purpose of MAC delay status reporting (DSR), the UE shall consider the following as delay-critical RLC data volume:]

-	[RLC SDUs for which the remaining discardTimer values are less than a threshold], [according to the indication from upper layer (e.g. PDCP)].

Editor's Notes: it is a placeholder for new mechanism for DSR. FFS how to calculate the buffer status associated with the remaining time. Depending on further progress, the exact procedure and location of this text may need to be changed.

Editor's Notes: FFS how to determine the remaining discardTimer value is less than a [threshold] in RLC, e.g. based on an indication from PDCP simiar as legacy.

Editor's Notes: FFS whether the data with discardTimer expired indicated by PDCP is also included in the above case that discardTimer value is less than a threshold.

In addition, if a STATUS PDU has been triggered and t-StatusProhibit is not running or has expired, the UE shall estimate the size of the STATUS PDU that will be transmitted in the next transmission opportunity, and consider this as part of RLC data volume.

End of change

# Annex A– RAN2 agreements related to RLC

| RAN2 agreements | Impacted specification | Comments, if any |
| --- | --- | --- |
| RAN2#121 |  |  |
| 5. Introduce UL PDU Set Importance. How UE derives this will be handled in UE implementation. Can indicate that in RAN2 considers PDU set concept applicable to both UL and DL in LS to SA2. | No impact |  |
| RAN2#121bis-e |  |  |
| N/A |  |  |
| RAN2#122 |  |  |
| 1: UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this. When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this. | Partially captured in 5.5 |  |
| RAN2#123 |  |  |
| Network can configure the UE whether to trigger delay status reporting. FFS if we have some thresholds per LCG. | No impact |  |
| When UE triggers reporting delay information for a LCG, and UE also reports the buffer status associated with the remaining time. | Captured in 5.5 |  |
| Working assumption: Define a new separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting. Detailed Definition of associated data volume is FFS. Support threshold based DSR reporting, e.g. DSR reporting is triggered when remaining delay of a PDU/PDU set is below a NW configured threshold. The threshold is configured per LCG. FFS whether configuring multiple thresholds for a LCG is supported. Definition of remaining time is FFS. | Captured in 5.5 |  |
| RAN2#123bis |  |  |