3GPP TSG RAN WG2 Meeting #123bis                                                       R2-230xxxx

# Xiamen, China, October 09-13, 2023

Agenda item:	7.18.2

Source:	Intel Corporation

Title:	Outcome of email discussion [POST123][303][MT-SDT] on UE Capabilities for Rel-18 MT SDT

Document for:	Discussion and decision

# Introduction

This document aims to review the running CRs on UE capabilities for Rel-18 MT-SDT and to discuss one open topic identified during RAN2#123 meeting.

[POST123][303][MT-SDT] CR to 38.306 (Intel)

Scope : review running CR

Outcome: CR to be submitted to next meeting

The following are RAN2#123 agreements relevant to this discussion:

- A new optional radio capability signaling (e.g., MT-SDT-r18) is defined to indicate UE’s support of Rel-18 MT-RA-SDT-r18.  MT-RA-SDT-r18 indicates whether the UE supports initiation of MT-SDT procedure and transmission/reception in RRC_INACTIVE state via Random Access procedure (i.e., RACH).

- Separate UE capability is used to indicate support MT-CG-SDT.  If the UE support MT-CG-SDT, it has to support MT-RA-SDT.

- SRB2 capability will be captured by adding “or MT-SDT” Rel-17 srb-sdt-r-17 capability.  (i.e. no separate capability)

- FFS how/whether we define UE capability to indicate UE’s support to select RACH resources instead of configured grant type 1 resource when triggering resume for SDT or MT-SDT and next CG-SDT resource is too far as specified in TS 38.331.

Please provide your inputs before/by Tuesday September 19th EOD PST to have few days afterwards to further review the report and updated CRs (as official email discussion deadline is Friday September 22nd, 2023).

# Companies’ point of contact (PoC)

| Company’s name | PoC’s name | PoC’s email address |
| --- | --- | --- |
| Ericsson | Oskar Myrberg | oskar.myrberg@ericsson.com |
| Huawei, HiSilicon | Dawid Koziol | dawid.koziol@huawei.com |

# Discussion

## Drafted running CRs to TS 38.306 and 38.331 on UE Capabilities for MT-SDT

- Please indicate if you have any input/comments on the drafted running CRs to TS 38.306 and 38.331 which captures the agreed details on UE Capabilities for MT-SDT during RAN2#123 meeting.

| Company’s name | TS # | Section | Comments, if any |
| --- | --- | --- | --- |
| Ericsson |  |  | No comments on .331 and .306 CRs. |
| Huawei | 38.306 | mt-CG-SDT-r18 definition | The capability agreed by RAN2 was supposed to be an MT-SDT equivalent of cg-SDT-r17 capability, it should not be related to switching from CG-SDT to RA-SDT. We should simply have a capability description basing on cg-SDT-r17, but with MT-SDT in mind, e.g.: Indicates whether the UE supports initiating MT-SDT procedure over configured grant type 1, in response to reception of MT-SDT indication in paging message, as specified in TS 38.331 [9].”. A UE supporting this feature shall also support mt-SDT-r18. |
| Huawei | 38.306 | mt-SDT-r18 | The description of this capability sounds as if the only thing the UE can do in reply to Paging with MT-SDT is to send RACH, but the UE can also perform CG-SDT. We suggest rephrasing as: “Indicates whether the UE supports initiating MT-SDT procedure over RACH, in response to reception of MT-SDT indication in paging message, as specified in TS 38.331 [9].” |
| Huawei | 38.306 | cg-SDT-r17 ra-SDT-r17 | The descriptions of these capabilities need to be updated to clarify they are for MO-SDT. |
| Huawei | 38.306 | srb-SDT-r17 | RA-SDT and CG-SDT are applicable to both MO-SDT and MT-SDT. And for MT-SDT, SRB2 can be applicable for either RA-SDt or CG-SDT. Suggest to reword as: “srb-SDT-r17 Indicates whether the UE supports the usage of signalling radio bearer SRB2 over RA-SDT or CG-SDT for MO-SDT and/or MT-SDT, as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate support of ra-SDT-r17 or cg-SDT-r17 or mt-SDT-r18 or mt-CG-SDT-r18.” |

## Open topics identified on UE Capabilities for MT-SDT

On the usage of CG-SDT resources to initiate SDT that it is equally applicable for MT-SDT and MO-SDT, the following was agreed in RAN2#122 meeting:

- For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first.   This is checked at the point of initial resource selection (e.g. CG SDT selection).   FFS what is too far and how this is configured.   Assumption is that we will continue this discussion in SDT session.  CONFIRM with main session [CB]

During RAN2#123 meeting, the corresponding new UE capability was discussed and the following FFS was captured:

- FFS how/whether we define UE capability to indicate UE’s support to select RACH resources instead of configured grant type 1 resource when triggering resume for SDT or MT-SDT and next CG-SDT resource is too far as specified in TS 38.331.

In addition, during the online discussion, it was mentioned that same/common UE capability might be preferable with this and the related CG-SDT topic discussed as part of TEI18. This is on the extension of CG-SDT periodicities, the following was agreed in RAN2#122 meeting (as part of TEI18) and RAN2 sent an LS [1] to RAN1 asking to provide any necessary feedback or concerns on impacts, if any:

- Agreeable, under condition that RAN1 impact is very small (e.g. update of a table): Extend the maximum periodicity for CG-SDT to cover longer periodicities.

- Send LS to R1 ask about impact.

RAN1 corresponding respond LS [2] will be discussed in next RAN2#123bis meeting which indicates that the foreseen impact is low (details copied below for reference).

“RAN1 confirms that extension of CG-SDT periodicities would have low impact on RAN1 specifications”.

The following discussion points aim to get inputs on those open topics raised during RAN2#123 online to easy related capability discussion in next RAN2#123bis meeting with the assumption that RAN2 will go ahead and extend the maximum periodicity for CG-SDT.

### Support to select RACH instead of CG resources for (MT-)SDT and the extension of CG-SDT periodicity

- Do you prefer defining a single or two separate new UE capabilities for UE to indicate its Rel-18 support to select RACH resources instead of configured grant type 1 resource when triggering resume for SDT or MT-SDT and next CG-SDT resource is too far and to use extended values of the CG-SDT periodicities? If applicable, please indicate whether any pre-requirement or relation to other UE capability that needs to also be considered.

- Single new UE capability is defined. For example, cg-SDT-Enh-r18 could indicate the support of both enhancements related to CG-SDT operation (i.e., support to select RACH resources instead of configured grant type 1 resource when triggering resume for SDT or MT-SDT and next CG-SDT resource is too far and support to extend the range of CG-SDT periodicities).

- Two separate new UE capabilities are defined. E.g., ra-InsteadCG-SDT-r18 could indicate the support to select RACH resources instead of configured grant type 1 resource when triggering resume for SDT or MT-SDT and next CG-SDT resource is too far and cg-SDT-ExtendedPeriodicity-r18 could indicate support to extend the range of CG-SDT periodicities.

- No new/additional capability is defined but instead its support is defined as part of Rel-18 UE capability MT-CG-SDT-r18. Note that based on current RAN2 agreements, UE capability draft CR to 38.306 defines MT-CG-SDT-r18 capability to indicates whether the UE supports the selection of configured grant type 1 resource (instead of RACH) to perform MT-SDT procedure as specified in TS 38.331 (moreover a UE supporting this MT-CG-SDT-r18 feature shall also support mt-SDT-r18). This option c) would mean that the UE capability definition will also mandate that UE supporting this MT-CG-SDT-r18 feature shall also support the two features here explained in this discussion point 2).

In addition, if option c) is preferable, please clarify how to allow the usage of this feature explained in this discussion point 2) by UEs only performing MO-SDT.

- Other approach is preferred.

| Company’s name | Option | Comments, if any |
| --- | --- | --- |
| Ericsson | A | We think a single capability for the SDT enhancements would be enough, given that the longer periodicities are designed to work together with the RA fallback. However, if the majority view would be B, we think this is also an acceptable option. |
| Huawei | b) | This is a standalone feature which can be used for both MO-SDT and MT-SDT with both existing CG-SDT periodicities and extended periodicities. There is no reason to bind it to any of other capabilities. |

# Report <<To be added>>

<<Report from this email discussion [to be added by rapporteur] >>

- xxxx.

# Conclusion

The proposals captured are the following:

Proposal 1.	xxxx.

# Reference

- R2-2306904, LS on longer CG-SDT periodicities, TEI18, From: RAN2, To: RAN1, May 2023.

- R1-2308487, Reply LS on longer CG-SDT periodicities, TEI18, From: RAN1, To: RAN2, August 2023.