3GPP TSG-RAN WG2 Meeting #123	R2-230

Toulouse , France, 21st Aug– 25th Aug, 2023

Title:		Summary of 	[Post123][414][POS] Rel-18 positioning MAC CRs (Huawei)

Source:	Huawei, HiSilicon

Agenda item:	8.2.1

Document for:	Discussion and Decision

- Discussion

1.1	MAC CR for sidelink positioning

| Company+index (e,g, HW000) | Excerpted spec with issues | Comments |
| --- | --- | --- |

1.2	MAC CR for LPHAP

| Company+index (e,g, HW000) | Excerpted spec with issues | Comments |
| --- | --- | --- |

2	Summary

NADA

Annex

Discussion during R2#123 for MAC CR for LPHAP

| Company+index (e,g, HW000) | Excerpted spec with issues | Comments |
| --- | --- | --- |
| ERIC000 | The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission when the cg-SDT-TimeAlignmentTimer is not running during the ongoing CG-SDT procedure as triggered in clause 5.27 and the inactivePosSRS-TimeAlignmentTimer or srs-ValidityAreaTimeAlignmentTimer is not running | These two timers are not running at the same time, so here should be "and" "are" instead of "or" "is" [Rapp] Corrected |
| ERIC001 | 2> if there is ongoing Positioning SRS Transmission in RRC_INACTIVE as in clause 5.26: 3> if SRS positioning validity area is configured: 4> start or restart the srs-ValidityAreaTimeAlignmentTimer associated with the indicated TAG. 3> else: 4> start or restart the inactivePosSRS-TimeAlignmentTimer associated with the indicated TAG. 2> if CG-SDT procedure is ongoing: | 3> if SRS positioning validity area is configured And UE is in validityArea as determined by upper layers: We may need above?? [Rapp] agree the previous text is a bit ambiguous. The followig has been chagned 2> if there is ongoing Positioning SRS Transmission in RRC_INACTIVE as in clause 5.26: 3> if srs-ValidityAreaTimeAlignmentTimer is configured: 4> start or restart the srs-ValidityAreaTimeAlignmentTimer associated with the indicated TAG. 3> else: 4> start or restart the inactivePosSRS-TimeAlignmentTimer associated with the indicated TAG. |
| ZTE001 | 1> when the indication is received from upper layer for stopping the inactivePosSRS-TimeAlignmentTimer or srs-ValidityArea-TimerAlignmentTimer: 2> stop the inactivePosSRS-TimeAlignmentTimer or srs-ValidityArea-TimerAlignmentTimer. 1> when the indication is received from upper layer for starting the inactivePosSRS-TimeAlignmentTimer: 2> start or restart the inactivePosSRS-TimeAlignmentTimer. 1> when instruction from the upper layer has been received for starting the cg-SDT-TimeAlignmentTimer: 2> start the cg-SDT-TimeAlignmentTimer. | Should add - when the indication is received from upper layer for starting the inactivePosSRS-TimeAlignmentTimer or srs-ValidityArea-TimerAlignmentTimer 2> start or restart the inactivePosSRS-TimeAlignmentTimer or srs-ValidityArea-TimerAlignmentTimer [Rapp] Not clear which agreement supports this one. Currently we have only agreed to start the TAT when TA command is received. |
| ZTE | 2> if there is ongoing Positioning SRS Transmission in RRC_INACTIVE as in clause 5.26: 3> if SRS positioning validity area is configured: 4> start or restart the srs-ValidityAreaTimeAlignmentTimer associated with the indicated TAG. 3> else: 4> start or restart the inactivePosSRS-TimeAlignmentTimer associated with the indicated TAG. | We should first agree with Rel-17 timer and Rel-18 timer cannot work together first [Rapp] Did anyone propose to use them at the same time? |
| Lenovo000 | 2> if there is ongoing Positioning SRS Transmission in RRC_INACTIVE as in clause 5.26: 3> if SRS positioning validity area is configured(excluding pre-configured SRS): 4> start or restart the srs-ValidityAreaTimeAlignmentTimer associated with the indicated TAG. 3> else: 4> start or restart the inactivePosSRS-TimeAlignmentTimer associated with the indicated TAG. | As the agreement made this meeting: The following criterion needs to be defined for the start/re-start of the area-specific TA timer: Reception of RRCRelease message containing the SRS configuration (excluding pre-configured SRS). The SRS pre-configuration cannot be regarded as the start/restart conditions as the agreements, which should be captured in spec. [Rapp] This is the agreement during this meeting and will be captured during the post meeting email discussion. |

Discussion during R2#123 for MAC CR for sidelink positioning

| Company+index (e,g, HW000) | Excerpted spec with issues | Comments |
| --- | --- | --- |
| vivo01 | Change 0 If the MAC entity has been configured with Sidelink resource allocation mode 1 as indicated in TS 38.331 or the MAC entity has been configured with resource allocation Scheme 2 and the PDCCH is received for the resource allocation on shared resource pool for SL-PRS transmission [5], | resource allocation Scheme 2 -> resource allocation mode 1 {Rapp} corrected to resource allocation scheme 1 |
| vivo02 | Editor's NOTE: FFS retransmission of PRS on shared and dedicated resource pool. | retransmission -> repetition? Conclusion Do not support ACK/NACK feedback for SL-PRS or lower-layer feedback-based retransmissions in Release 18. [Rapp] Should be retransmission. |
| vivo03 | Editor's NOTE: FFS SL-PRS transmission on shared resource pool when the MAC PDU has been positively acked. | The intention of this FFS is not clear. [Rapp] The intention is that if on shared RP, SL-PRS and data area transmitted together and data is acked with PSFCH, what should be done to SL-PRS |
| vivo04 | Editor's NOTE: FFS relation with HARQ for SL-PRS transmission on shared resource pool | See vivo02 |
| vivo05 | For each SL-PRS occasion, the MAC entity shall: 1> for each SCI valid for this SL-PRS occasion: 2> instruct the physical layer to perform SL-PRS reception on the SL-PRS occasion. 1> if this SL-PRS transmission is associated to unicast: 2> if the destination ID in the corresponding SCI is equal to the UE's source ID and source ID in the corresponding SCI is equal to the UE's destination ID: 3> instruct the physical layer to perform SL-PRS reception on the SL-PRS transmission occasion. 1> else if this SL-PRS transmission is associated to broadcast or groupcast: 2> if the destination ID in the corresponding SCI is equal to the UE's source ID 3> instruct the physical layer to perform SL-PRS reception on the SL-PRS transmission occasion. | Seems duplicated instruction to perform SL-PRS reception. [Rapp] One is for unicast, another is for broadcast and groupcast. |