| Company | Clause | Comment | Rapp Response |
| --- | --- | --- | --- |
| Lenovo | 6.3.1 | For the field description for frequency information, “In this release” has ambiguity after introducing new IE sl-FreqInfoListSizeExt and suggest to remove the wording sl-FreqInfoList, sl-FreqInfoListSizeExt This field indicates the NR sidelink communication/discovery configuration on some carrier frequency (ies). In this release, Only one entry can be configured in the sl-FreqInfoList. |  |
| Lenovo | 5.2.2.4.13 | New added IE sl-FreqInfoListSizeExt and sl-RLC-BearerConfigListSizeExt are defined in SIB12 but not in sl-ConfigCommonNR. Needs to be aligned with 6.3.1 2> if sl-FreqInfoList/sl-FreqInfoListSizeExt is included in sl-ConfigCommonNR: … 2> if sl-RadioBearerConfigList or sl-RLC-BearerConfigList/sl-RLC-BearerConfigListSizeExt is included in sl-ConfigCommonNR: |  |
| Lenovo | 6.3.5 and 5.8.9.1a.5.1 | Additional RLC configuration for SRB/SCCH is directly specified, so we understand there not need RLC configuration index for SRB RLC configuration. I guess the purpose to include this index for SRB is for unified release condition/operation of additional RLC bearer for both DRB and SRB? 6.3.5 SL-RLC-BearerConfig-r18 ::= CHOICE { srb SEQUENCE { sl-SRB-IdentityWithDuplication INTEGER (1..3), sL-RLC-BearerConfigIndex-r16 SL-RLC-BearerConfigIndex-r18, ... }, drb SEQUENCE { slrb-PC5-ConfigIndex-r18 SLRB-PC5-ConfigIndex-r16, sL-RLC-BearerConfigIndex-r16 SL-RLC-BearerConfigIndex-r18, sl-RLC-ConfigPC5-r18 SL-RLC-ConfigPC5-r16 OPTIONAL, -- Need M sl-MAC-LogicalChannelConfigPC5-r18 SL-LogicalChannelConfigPC5-r16 OPTIONAL, -- Need M ... } } 5.8.9.1a.5.1 1> for unicast, if SL-RLC-BearerConfigIndex (if any) of the sidelink DRB or SRB is included in sl-RLC-BearerToReleaseList/sl-RLC-BearerToAddModListSizeExt in RRCReconfigurationSidelink; |  |
| Lenovo | 5.8.9.1.1 | Since following two sentences are basically same, seems they can be combined for concise text. No strong view though - the addition of sidelink carrier associated with the peer UE, as specified in clause 5.8.9.1b.2; - the modification of sidelink carrier associated with the peer UE, as specified in clause 5.8.9.1b.2; => - the addition or modification of sidelink carrier associated with the peer UE, as specified in clause 5.8.9.1b.2; |  |
| Lenovo | 5.8.9.1a.6 | A typo 3> configure the MAC entity with a logical channel associated with the sidelink RLC entity, as specified in clause 9.1.1.4. |  |
| Lenovo | 5.8.9.1b.2 | Typo and rewording 1> for unicast, after receiving the RRCReconfigurationCompleteSidelink message, 2> for each sl-Carrier-Id value included in the sl-CarrierToReleaseList sl-CarrierToAddModList that is not part of the current UE configuration (sidelink carrier addition): … 1> for unicast, if the sidelink carrier addition added sidelink carrier was modified due to the reception of the RRCReconfigurationSidelink message; or 1> for unicast, after receiving the RRCReconfigurationCompleteSidelink message, 2> for each sl-Carrier-Id value included in the sl-CarrierToReleaseList sl-CarrierToAddModList that is part of the current UE configuration (sidelink carrier modification): 3> modify the sidelink carrier configuration in accordance with sl-AbsoluteFrequencyPointA and sl-AbsoluteFrequencySSB; |  |
| Lenovo | 6.3.5 | The new added timer is for C-LBT cancellation, so suggest following update for the description of IE and also the name of timer: sl-LBT-FailureRecoveryConfig Configures parameters used for detection and cancellation of consistent sidelink LBT failures for operation with shared spectrum channel access, as specified in TS 38.321 [3]. – SL-LBT-FailureRecoveryConfig The IE SL-LBT-FailureRecoveryConfig-r18 is used to configure the parameters used for detection and cancellation of sidelink consistent LBT failures for operation with shared spectrum channel access, as specified in TS 38.321 [3]. SL-LBT-FailureRecoveryConfig information element -- ASN1START -- TAG-SL-LBT-FAILURERECOVERYCONFIG-START SL-LBT-FailureRecoveryConfig-r18 ::= SEQUENCE { sl-lbt-FailureInstanceMaxCount-r18 ENUMERATED {n4, n8, n16, n32, n64, n128} OPTIONAL, -- Need M sl-lbt-FailureDetectionTimer-r18 ENUMERATED {ms10, ms20, ms40, ms80, ms160, ms320} OPTIONAL, -- Need M sl-LBT-RecoveryCancellationTimer-r18 ENUMERATED {ms10, ms20, ms40, ms80, ms160, ms320} OPTIONAL, -- Need M... } -- TAG-SL-LBT-FAILURERECOVERYCONFIG-STOP -- ASN1STOP |  |