3GPP TSG-RAN WG2 Meeting #124	R2-231xxxx

# Chicago, USA, Nov. 13th – 17th, 2023

Agenda Item:	   x.x.x

Source:	   Huawei, HiSilicon

Title:	   Summary of [POST123bis][851][CE_enh]  CP running CR and open issues (Huawei)

Document for:	   Decision

- 1	Introduction

This document aims at discussing the following RAN2#123bis Post discussion.

[POST123bis][851][CE_enh]  CP running CR and open issues (Huawei)

Scope and intended outcome:

1.     Update the running CR with agreements from the meeting

2.     Rapporteur to propose resolutions for straightforward open issues which can already be included in the running CR

3.     For Stage 3 running CRs, get input on stage-3 issues that require further input from companies to make a decision:

- Focus on stage-3 issues which are better handled via offline, e.g. signaling details, parameter values/ranges, NOT functionality discussion. For these issues, if any, the CR rapporteur should submit a separate report with proposals to the next meeting by the submission deadline, while input via company Tdocs should be avoided

4.     Identify the remaining open issues that need to be solved for WI completion in the next meeting:

- Company Tdocs for the next meeting should focus on these issues

Deadline:  Long (until next meeting)

| Company | Name | Email |
| --- | --- | --- |
| Ericsson | Oskar Myrberg | oskar.myrberg@ericsson.com |
| Samsung | Anil Agiwal | anilag@samsung.com |
| Qualcomm | Sherif ElAzzouni | selazzou@qti.qualcomm.com |
| ZTE | LiuJing | liu.jing30@zte.com.cn |
| Huawei, Hisilicon | You Chunhua | youchunhua@huawei.com |

- 2	Remaining open issues

2.1 Parameter configuration

- groupBconfigured, rsrp-ThresholdSSB

RAN2 discussed how to configure parameters for different repetition number:

From RAN2 CE perspective, deltaPreamble IE in FeatureCombinationPreambles are common for repetition number 2, 4 and 8 - FFS for groupBconfigured, rsrp-ThresholdSSB

The moderator tend to think there can be benefit/flexible to allow separate configurations of groupBconfigued, rsrp-ThresholdSSB for different repetition number. An EN has been also added.

Editor’s Note1: FFS on separate groupBconfigure, rsrp-ThresholdSSB for different repetition number.

Companies are encouraged to provide views if you are fine with this proposal.

Question 1:	Do companies agree that groupBconfigured can be separately configured for different repetition number?

| Company | Options (Y or N) | Comments |
| --- | --- | --- |
| Ericsson | N | In order to keep the specification impact smaller we prefer that groupBconfigured is same for all repetitions. We think the added benefit from this configured separately for each repetition factor is low. |
| Samsung | N | Its not needed. Its going to create more issues. For example, if ra-Msg3SizeGroupA is different, preamble group reselection and Msg3 MAC PDU generation (rebuilding) needs to be performed again when UE fallbacks from lower to higher repetition. |
| Qualcomm | No | Not needed. Group B if configured can use the same repetition factor of group A. No issues foreseen. |
| ZTE | Yes with comments | groupBconfigured-r17 SEQUENCE { ra-SizeGroupA-r17 ENUMERATED {b56, b144, b208, b256, b282, b480, b640, b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1}, messagePowerOffsetGroupB-r17 ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18}, numberOfRA-PreamblesGroupA-r17 INTEGER (1..64) } OPTIONAL, -- Need R groupBConfigured has 3 sub IEs, in our view, only numberOfRA-PreamblesGroupA-r17 can be different for different repetition numbers. Since we use separate featureCombinationPreambles IEs to configure RACH resources for different repetition numbers, considering the different amount of UEs at cell edge, the network may reserve different number of preamble indexes for different repetition numbers. For example, 8 preamble indexes for Num 2, 4 preamble indexes for Num4, but only 2 preamble indexes for Num 8. In this case, it does not make sense to apply the same numberOfRA-PreamblesGroupA for different repetition numbers. In our understanding, different numberOfRA-PreamblesGroupA does not bring additional impact to MAC spec. |
| Huawei, Hisilicon | Y | Agree with the moderator. However comon configuration is acceptable to us if the majority prefer it. |

Question 2:	Do companies agree that rsrp-ThresholdSSB can be separately configured for different repetition number?

| Company | Options (Y or N) | Comments |
| --- | --- | --- |
| Ericsson | N | As the current running CR for MAC looks, we fail to see the use of separate rsrp-ThresholdSSB for when 5.1.2 in MAC specification is run again. If the SSB selection does not find any SSB above the threshold the UE will any way select “any SSB” so we think this will be enough. |
| Samsung | N | Benefit is unclear. |
| Qualcomm | No | No clear motivation for RSRP SSB threshold to depend on repetition number. RSRP threshold will be set conservatively to allow enough signal for no repetition case, and lower than that repetition thresholds can cover. |
| ZTE | N | Same view as Ericsson. |
| Huawei, Hisilicon | Y | Agree with the moderator. However comon configuration is acceptable to us if the majority prefer it. |

- MSG3 repetition parameters in relation with MSG1 repetition

In the previous meeting, the moderator observes some proposals on MSG1 and MSG3 co-existence, and think there is no discussions on this point from CP prespective. Some companies argue that it would be beneificial to distinguish Msg3 parameters w/wo Msg1 repetition. Therefore, the moderator would like to see company’s views on this. Companies are encouraged to provide views on the following question. An EN has been also added.

Editor’s Note3: FFS on separate numberOfMsg3-RepetitionsList, mcs-Msg3-Repetitions when MSG1 repetition is applicable.

Question 3:	Do companies think if separate Msg3 repetition parameter (e.g. numberOfMsg3-RepetitionsList and mcs-Msg3-Repetitions) should be configured if Msg 1 repetition is applicable to distinguish that from the case when Msg1 repetition is not applicable.

| Company | Options (Y or N) | Comments |
| --- | --- | --- |
| Ericsson | N | We think we don’t have to separate the cases to reduce complexity. |
| Samsung | N |  |
| Qualcomm | No | Side note: If a partition is configured with Msg1/Msg3 based repetition, we think there is some value in Msg 1 repetition automatically triggering Msg3 repetition. |
| ZTE | N | This looks like an optimization, not sure if we should check with RAN1 first. |
| Huawei, Hisilicon | Y | Agree with the moderator. The MSG3 repeition number value in MSG1 + MSG3 repetition for R18 should be different from the R17 since differnet coverage level are assumed in R18. |

2.2 Value ranges

- The values of the threshold of fallback from lower number to higher number

RAN2 agreed to introduce a RRC configured threshold to control the fallback, which is common for different repetition number. However, the value needs to be determined. An EN has been also added.

Introduce a RRC configured threshold (e.g. TransMax-Msg1RepNum), the field is used for deciding whether to trigger fallback from with lower number to higher number when the number of Msg1 transmission exceeds this threshold. This parameter is common for different repetition numbers configured in one RACH partition.

Editor’s Note4: FFS on values of msg1-RepetitionTransMax.

Regarding the values, the moderator suggest to adtop the same values of 2-step switch to 4-step, i.e. n1, n2, n4, n6, n100, n200. In case this filed is absent, the switch from lower number to higher number is not allowed.

msgA-TransMax-r16                                    ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}     OPTIONAL, -- Need R

The suggested TP is as follows.

BWP-UplinkCommon information element

-- ASN1START

-- TAG-BWP-UPLINKCOMMON-START

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...,

[[

rach-ConfigCommonIAB-r16            SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

msgA-ConfigCommon-r16               SetupRelease { MsgA-ConfigCommon-r16 }                                  OPTIONAL    -- Cond SpCellOnly2

]],

[[

enableRA-PrioritizationForSlicing-r17 BOOLEAN                                                    OPTIONAL, -- Cond RA-PrioSliceAI

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }               OPTIONAL, -- Cond SpCellOnly2

rsrp-ThresholdMsg3-r17              RSRP-Range                                                   OPTIONAL, -- Need R

numberOfMsg3-RepetitionsList-r17    SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17                  OPTIONAL,  -- Cond Msg3Rep

mcs-Msg3-Repetitions-r17            SEQUENCE (SIZE (8)) OF INTEGER (0..31)                               OPTIONAL   -- Cond Msg3Rep

msg1-RepetitionTransMax-r18			       ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}           OPTIONAL   -- Cond Msg1Rep1

]]

}

AdditionalRACH-ConfigList-r17 ::=       SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17

AdditionalRACH-Config-r17 ::=       SEQUENCE {

rach-ConfigCommon-r17               RACH-ConfigCommon                                                   OPTIONAL,  -- Need R

msgA-ConfigCommon-r17               MsgA-ConfigCommon-r16                                               OPTIONAL,  -- Need R

...

}

NumberOfMsg3-Repetitions-r17::=         ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16}

-- TAG-BWP-UPLINKCOMMON-STOP

-- ASN1STOP

msg1-RepetitionTransMax Max number of transmissions of MSG1 repetitions number (2, 4 and 8) performed before switching to higher repetition number (see TS 38.321 [3], clauses 5.1.1). This field is only applicable when more than 2 repetition numbers are configured in shared RO. If the field is absent, switching from lower repetition number to higher repetition number is not allowed.

Question 4:	Do companies agree the values of msg1-RepetitionTransMax, which is used to decide whether to trigger fallback from with lower number to higher number when the number of Msg1 transmission exceeds this threshold can be { n1, n2, n4, n6, n100, n200} ?

| Company | Options (Y or N) | Comments |
| --- | --- | --- |
| Ericsson | Y | Proposed values seems ok. |
| Samsung | Y |  |
| Qualcomm | Yes |  |
| ZTE | Yes |  |
| Huawei, Hisilicon | Yes |  |

- The number of RACH configurations

Currently there is a limitation of maximum 16 of RACH configurations in the RRC spec as follows. However, considering Msg1 repetition can be combinated with any feature, which would at most 3 times the number of RACH configurations (e.g. separate RO). Therefore, the moderator would like to see if there is a need to extend this limitation. Two ENs has been also added.

Editor’s Note5: FFS on limitation of the max number of entries of additionalRACH-ConfigList

additionalRACH-ConfigList List of feature or feature combination-specific RACH configurations, i.e. the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to one or more feature(s) or feature combination(s). The network does not configure this list to have more than 16 entries. If both rach-ConfigCommon and msgA-ConfigCommon are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.

Editor’s Note6: FFS on limitation of the max number of entries of featureCombinationPreamblesList

RACH-ConfigCommon field descriptions

featureCombinationPreamblesList Specifies a series of preamble partitions each associated to a combination of features and 4-step RA. The network does not configure this list to have more than 16 entries.

Question 5:	Do companies think if the limitation of 16 entries for RACH configurations should be extended for MSG1 repetition. If yes, what would be your suggested number of entries of RACH configurations.

| Company | Options (Y or N) | Comments (If you indicate Y, pls also indicate your suggested number of entries, e.g. 3*16=48?) |
| --- | --- | --- |
| Ericsson | Comments | In principle we agree that we need to expand this. Suggested value is 32 since it seems unlikely that networks will configure partitions equal for all repetition factors and featurecombinations. |
| Samsung | Comments | It is unlikely that network will configure all features/feature combinations simultaneously. We are ok to keep the maximum number to 16. |
| Qualcomm | No strong view |  |
| ZTE | Yes | We propose 16* (3+1) = 64. (1 for without repetition, 3 for repetition Num2,4,8) |
| Huawei, Hisilicon | Y | Agreed with Ericsson, 32 is fine. |

2.3 CHO support

RAN2 discussed the support of CHO for CFRA with MSG1 repetition, and no consensus is made.

CHO support

[Online]Proposal 4 [6/9]: RAN2 to discuss and confirm that for CHO, if multiple repetition number configuration and UE selection is not considered, i.e. no further optimization is needed in R18.

Discussion

Samsung: network doesn’t know the exact repetition number at the time of configuration so, it is better not to support this. But one repetition number is supported.

The moderator think we can try again to see if we can converge on this point given that CHO with MSG1 repetition can come for free based on previous agreement on CFRA. The further optimization can be discussed in TEI or R19. Note that only a large majority view support this direction, the proposal can be maded. Otherwise, it should go to online discussion for the next meeting. An EN has been also added.

Editor’s Note7: FFS on support of CFRA with MSG1-repetition for CHO

Question 6:	Do companies is okay with the proposal that the previous agreement on CFRA is applied to CHO, i.e. no additional optimization for CFRA is needed for CHO.

| Company | Options (Y or N) | Comments |
| --- | --- | --- |
| Ericsson | Y with comments | For CHO we whink it is enough with one configured repetition factor. |
| Samsung | N | In case of CFRA configuration for CHO case CFRA configuration is provided in advance and this configuration is used much later when certain conditions (CHO execution condition) are met. So just indication of one repetition number for CHO case is not sufficient. Multiple repetitions number, different CFRA configuration for each of these repetition number and selection based on RSRP seems needed for CHO case. This requires additional work. So as agreed for BFR, we propose to not support CFRA with Msg1 repetitions for CHO |
| Qualcomm | Yes | As the moderator says, it comes for free, so we see no strong reasons to deliberately restrict it. It should be treated as a normal HO of which we allow the NW to indicate a single msg1 repetition number. Also we are not convinced that the NW not knowing the exact UE measurements are an issue, the UE executes HO when a (possibly RSRP) condition is true, most of the time the RSRP measured by the UE would be very close to the configured condition. If NW has a great deal of variability it may choose not to allow this feature. |
| ZTE | N | We are fine to not support CFRA with Msg1 repetition for CHO. we understand usually CHO is triggered when the target cell is good enough. Msg1 repetition seems not so essential. If it is going to be supported, we have strong willingness to only support one repetition number. There is no need to consider pre-configuring multiple repetition numbers, and it will requires more discussion on how to support the fallback in MAC spec. |
| Huawei, Hisilicon | Yes | Agree with Ericsson. If there is only one repetition number, CHO execution condition parameters (e.g. threshold for CHO events A5) can be linked to RSRP threshold for selecting single multiple repetition number. However if there are multiple repetition number, we wonder how the execution condition parameter is linked to thresholds for multiple repetition numbers. |

2.4 Other issues

In case if any company see any other critical issue worthy to be discussed in “remaining open issues”, please provide it by below.

| Company | Issue | Comments |
| --- | --- | --- |

- 3	Running CR implementation issues

3.1 RA framework

- Agreement

Adopt Alt 2.3 for Msg1 repetition framework

Separate RO for different number is supported;

      - For sharedRO and separateRO case, different repetition numbers are configured via separate featureCombinationPreamble IEs only for CE.

      - RACH resources of RACH partitions that are configured with the same “featureCombination” are considered to be within the same set of RACH resources;

      - Fallback from lower number to higher number is performed within the selected set of RACH resources.

      - Alt1: Fallback is only supported for sharedRO case

Regarding how to implement the agreed RA framework, the RRC rapporteur propose the following TP with changes:

1) Adding MSG1 repetition number to the corresponding FeatureCombinationPreambles, and whether separate RO or shared RO for different repetition number is configured is up to NW to configure the different FeatureCombinationPreambles (corresponding to different repetition number) under one or different rach-ConfigCommon, which is illustrated in the following figure from [Post123][801] email discussion.

2) Meanwhile, the restriction that one FeatureCombinationPreambles is associated with a given feature combination per RA type should be also updated.

3) Regarding Alt1: Fallback is only supported for sharedRO case, the RRC rapporteur thinks the parameters of the threshould for fallback should reflect this agreement, and thus it can be captured into the corresponding field description.

#### –	FeatureCombinationPreambles

The IE FeatureCombinationPreambles associates a set of preambles with a feature combination. For parameters which can be provided in this IE, the UE applies this field value when performing Random Access using a preamble in this featureCombinationPreambles, otherwise the UE applies the corresponding value as determined by applicable Need Code, e.g. Need S. On a specific BWP, there can be at most one set of preambles associated with a given feature combination per RA Type (i.e. 4-step RACH or 2-step RACH) per MSG1 repetition number.

FeatureCombinationPreambles information element

-- ASN1START

-- TAG-FEATURECOMBINATIONPREAMBLES-START

FeatureCombinationPreambles-r17 ::=   SEQUENCE {

featureCombination-r17                FeatureCombination-r17,

startPreambleForThisPartition-r17     INTEGER (0..63),

numberOfPreamblesPerSSB-ForThisPartition-r17 INTEGER (1..64),

ssb-SharedRO-MaskIndex-r17            INTEGER (1..15)                                           OPTIONAL, -- Need S

groupBconfigured-r17                  SEQUENCE {

ra-SizeGroupA-r17                     ENUMERATED {b56, b144, b208, b256, b282, b480, b640,

b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB-r17          ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA-r17        INTEGER (1..64)

}                                                                                               OPTIONAL, -- Need R

separateMsgA-PUSCH-Config-r17         MsgA-PUSCH-Config-r16                                     OPTIONAL, -- Cond MsgAConfigCommon

msgA-RSRP-Threshold-r17               RSRP-Range                                                OPTIONAL, -- Need R

rsrp-ThresholdSSB-r17                 RSRP-Range                                                OPTIONAL, -- Need R

deltaPreamble-r17                     INTEGER (-1..6)                                           OPTIONAL, -- Need R

...,

[[

msg1-RepetitionNum-r18                     ENUMERATED {2, 4, 8}                                             OPTIONAL, -- Cond Msg1Rep2

]]

}

-- TAG-FEATURECOMBINATIONPREAMBLES-STOP

-- ASN1STOP

msg1-RepetitionTransMax Max number of transmissions of MSG1 repetitions number (2, 4 and 8) performed before switching to higher repetition number (see TS 38.321 [3], clauses 5.1.1). This field is only applicable when more than 2 repetition numbers are configured in shared RO. If the field is absent, switching from lower repetition number to higher repetition number is not allowed.

Question 7:	Do companies agree with above 3 changes to implement the RA framework of MSG1 repetition, and if not, please indicate your detailed suggestions in the following table.

| Company | Comments |
| --- | --- |
| Ericsson | Seems like this solution forces many parameters to be defined “per repetition factor” like rsrp-ThresholdSSB etc. Alternative is to define msg1-RepetitionNum in such a way so that it can support one or several repetition factors (like msg1-RepetitionNum-2 {supported} and signal preamble range within the IE.. At least for the case where all repetition factors are on shared RO or all are on separate RO then there will be no duplicate values. Otherwise we need a discussion about how the duplicate parameters should be treated. |

3.2 SI request framework

Separate SI-RequestResources is configured for different repetition number (2,4,8), under a common SI-RequestConfig which is different from legacy SI-RequestConfig

Regarding how to implement the SI request framework as agreed above which is illustrated in the following feature. The RRC rapporteur propose a TP with the following changes..

1) Extending si-RequestResources to a list to include the configurations of individual repetition number si-RequestResourcesList-MSG1-Repetition-r18

2) Adding msg1-RepetitionNum to the configuration of RA configuations for each repetition number

#### –	SI-RequestConfig

The IE SI-RequestConfig contains configuration for Msg1 based SI request.

SI-RequestConfig information element

-- ASN1START

-- TAG-SI-REQUESTCONFIG-START

SI-RequestConfig ::=                SEQUENCE {

rach-OccasionsSI                    SEQUENCE {

rach-ConfigSI                       RACH-ConfigGeneric,

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                       OPTIONAL,   -- Need R

si-RequestPeriod                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

si-RequestResources                 SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

}

SI-RequestResources ::=             SEQUENCE {

ra-PreambleStartIndex               INTEGER (0..63),

ra-AssociationPeriodIndex           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

SI-RequestConfig-v18xy ::=            SEQUENCE {

si-RequestResources-r18               SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResourcesForMSG1-Repetition-r18

}

SI-RequestResourcesForMSG1-Repetition-r18 ::=             SEQUENCE {

si-RequestResourcesList-MSG1-Repetition-r18               SEQUENCE (SIZE (1.. maxNrofMSG1-Repetitions-r18)) OF SI-RequestResourcesForMSG1-RepetitionNum-r18

}

SI-RequestResourcesForMSG1-RepetitionNum-r18 ::=             SEQUENCE {

si-RequestResources-r18                 SI-RequestResources,

msg1-RepetitionNum-r18                  ENUMERATED {2, 4, 8}

}

-- TAG-SI-REQUESTCONFIG-STOP

-- ASN1STOP

Question 8:	Do companies agree with above changes to implement the SI framework of MSG1 repetition, and if not, please indicate your detailed suggestions in the following table.

| Company | Comments |
| --- | --- |
| Ericsson | Yes |
| Samsung | No Issue 1: si-RequestResources (without suffix) will be unecessary signalled as it will be ignored by UE. Please see red and yellow highlighted text below. SI-RequestConfig-r18 ::= SEQUENCE { si-RequestConfigForMSG1-Repetition-r18 SI-RequestConfig, si-RequestConfig-v18xy SI-RequestConfig-v18xy } SI-RequestConfig ::= SEQUENCE { rach-OccasionsSI SEQUENCE { rach-ConfigSI RACH-ConfigGeneric, ssb-perRACH-Occasion ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen} } OPTIONAL, -- Need R si-RequestPeriod ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen} OPTIONAL, -- Need R si-RequestResources SEQUENCE (SIZE (1.maxSI-Message)) OF SI-RequestResources } SI-RequestConfig-v18xy ::= SEQUENCE { si-RequestResources-r18 SEQUENCE (SIZE (1.maxSI-Message)) OF SI-RequestResourcesForMSG1-Repetition-r18 } SI-RequestResourcesForMSG1-Repetition-r18 ::= SEQUENCE { si-RequestResourcesList-MSG1-Repetition-r18 SEQUENCE (SIZE (1. maxNrofMSG1-Repetitions-r18)) OF SI-RequestResourcesForMSG1-RepetitionNum-r18 } SI-RequestResourcesForMSG1-RepetitionNum-r18 ::= SEQUENCE { si-RequestResources-r18 SI-RequestResources, msg1-RepetitionNum-r18 ENUMERATED {2, 4, 8} } Issue 2: SI-RequestConfig-r18 calls the legacy IE SI-RequestConfig with same name. Not sure if this is ok. Issue 3: si-RequestResources-r18 have two definitions in the CR. Is this ok? o si-RequestResources-r18 SEQUENCE (SIZE (1.maxSI-Message)) OF SI-RequestResourcesForMSG1-Repetition-r18 o si-RequestResources-r18 SI-RequestResources |
|  | Suggestion to define as follows: SI-SchedulingInfo-v18xy ::= SEQUENCE { si-RequestConfig-MSG1-Repetition-r18 SI-RequestConfig-r18 OPTIONAL, -- Cond MSG-1 si-RequestConfigRedCap-MSG1-Repetition-r18 SI-RequestConfig-r18 OPTIONAL, -- Cond SUL-MSG-1 si-RequestConfigSUL-MSG1-Repetition-r18 SI-RequestConfig-r18 OPTIONAL -- Cond REDCAP-MSG-1 } SI-RequestConfig-r18 ::= SEQUENCE { si-RequestConfigForMSG1-Repetition-r18 SI-RequestConfigRepetitions, } SI-RequestConfigRepetitions::= SEQUENCE { rach-OccasionsSI-r18 SEQUENCE { rach-ConfigSI-r18 RACH-ConfigGeneric, ssb-perRACH-Occasion-r18 ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen} } OPTIONAL, -- Need R si-RequestPeriod-r18 ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen} OPTIONAL, -- Need R si-RequestResourcesRepetitions-r18 SEQUENCE (SIZE (1.maxSI-Message)) OF SI-RequestResourcesForMSG1-Repetition-r18 } SI-RequestResourcesForMSG1-Repetition-r18 ::= SEQUENCE { si-RequestResourcesList-MSG1-Repetition-r18 SEQUENCE (SIZE (1. maxNrofMSG1-Repetitions-r18)) OF SI-RequestResourcesForMSG1-RepetitionNum-r18 } SI-RequestResourcesForMSG1-RepetitionNum-r18 ::= SEQUENCE { si-RequestResources-r18 SI-RequestResources, msg1-RepetitionNum-r18 ENUMERATED {2, 4, 8} } |

3.3 Other issues

In case if any company see any other critical issue worthy to be discussed in “remaining open issues”, please provide it by below.

| Company | Issue | Comments |
| --- | --- | --- |
| Qualcomm | separate RSRP thresholds for different UE power classes. | In our view, the framework of msg1 repetition needs to separate the repetition criteria between UE power classes, as FWA UEs can go to a much higher max EIRP than normal/handheld UEs, it should be possible for the NW to restrict repetition of such UEs since they have a much higher power limit they can ramp up to without congesting the RACH resources especially on shared ROs, in fact we assume the NW may want to discourage such UEs from using Coverage enhancements ROs so as to not worsen coverage of other UEs. |

- 4 Conclusions

- 5 References

- R2-2309776	Remaining control plane issues of further NR Coverage Enhancements	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_cov_enh2-Core

- R2-2310284	Discussion on the remaining CP issues for CE	NEC Corporation.	discussion	Rel-18	NR_cov_enh2-Core

- R2-2310198	Remaining issues of CP aspects for CE	Huawei, HiSilicon	discussion	NR_cov_enh2-Core