3GPP TSG-RAN WG2 #124	R2-23xxxxx

Chicago, USA, 13th – 17th November 2023

Agenda Item:	7.2.1

Source:	Ericsson

- Title:			[Post123bis][410][POS] Rel-18 positioning RRC CR (Ericsson)

Document for:	Discussion, Decision

- Introduction

This is to kick off the email discussion.

[Post123bis][410][POS] Rel-18 positioning RRC CR (Ericsson)

Scope: Review the running CR and develop an open issue list.

Intended outcome: Draft CR and open issue list for next meeting

Deadline: Medium (2 weeks)

2	Discussion

2.1	Sidelink CR

For Sidelink resource pool configuration ASN.1 implementation, there are two Options:

1) Reuse Legacy IE and update the field description: adding additional part into existing IEs

2) Create a new IE for SL positioning resource pool configuration

A document with both versions have been provided here.

Please provide your comments on which Option is preferred.

| Company Name | Option 1 or Option 2 | Comments on preferred Option and additionally on the CR, if any: |
| --- | --- | --- |
| ZTE | Option 2 |  |
| CATT | Option 2 |  |

2.2	Bandwidth Aggregation CR

Please provide your comments on the CR.

| Company Name | Comments |
| --- | --- |
| ZTE | For RRC INACTIVE, R1’s parameter list says to have an IE containing < Indicates the frequency information (e.g. point A, offset to carrier) of one or two additional carrier(s) with respective SRS configurations where the newly introduced carrier(s) and the carrier of the initial BWP should be intra-band contiguous carriers.>; And an IE containing < Provides positioning SRS configuration with SRS aggregation for UE in RRC_INACTIVE state>. So we think these two should be saparately configured in RRCRelease, rather than quote SRS-PosResourceSetLinkedForAggBW. |
| ZTE | SRSPosIntraBandCCForAggBW-r18 ::= SEQUENCE { servCellIndexList-r18 SEQUENCE (SIZE(1.. maxNrOfLinkedSRS-PosResourceSet-r18)) OF ServCellIndex, cc-CombinationList-r18 SEQUENCE (SIZE(1.. maxNrOfLinkedSRS-PosResourceSet-r18)) OF UplinkDedicated } Does this IE necessary? SRS-PosResourceSetLinkedForAggBW already contains serving cell index |
| ZTE | SRS-PosResourceSetLinkedForAggBW-r18 ::= SEQUENCE { srs-PosResourceSetLinked-r18 SRS-PosResourceSetId-r16, carrier-r18 ARFCN-ValueNR, OPTIONAL, --Need M servingCellIndex-r18 ServingCellIndex OPTIONAL --Need M } This should also add UL BWP ID since SRS resource set ID is unique among a BWP. |

2.3	RedCaP CR

Please provide your comments on the CR.

| Company Name | Comments |
| --- | --- |

2.4	CPP CR

For CPP CR, Pls provide your view:

1) No RRC Impact

2) Agree that there is RRC impact and Please provide your comments on the CR.

3) Wait for RAN1 Guidance

| Company Name | Option 1/2/3 | Comments |
| --- | --- | --- |
| CATT | Option 3 |  |

2.5	LPHAP

Please provide your comments on the LPHAP CR.

| Company Name | Comments |
| --- | --- |
| ZTE | #### 5.3.13.2 Initiation The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, for NR sidelink communication/discovery/V2X sidelink communication as specified in clause 5.3.13.1a, upon SRS request or activation in RRC_INACTIVE) requests the resume of a suspended RRC connection or requests the resume for initiating SDT as specified in clause 5.3.13.1b. |
| ZTE | 1> else if cell reselection occurs when srs-PosRRC-InactiveValidityArea is configured and if the cell is included in the srs-PosConfigValidityArea: 2> if autonomousTA-AdjustmentEnabled is configured; 3> autonomously adjusts the time advance value. 3> autonomously adjusts the stored RSRP for TA validation. |
| ZTE | MAC spec has the procedure to store the RSRP. We think RRC spec does not need to say it again. 38.321: The MAC entity shall: 1> if the UE receives configuration for SRS transmission in RRC_INACTIVE: 2> store the RSRP of the downlink pathloss reference with the current RSRP value of the downlink pathloss reference as in TS 38.331 [5].Q. 331 running CR: 1> acquire SIB2, if stored version is invalid; 1> if nrofSS-BlocksToAverage or absThreshSS-BlocksConsolidation is not present or if absThreshSS-BlocksConsolidation is present and the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation: 2> derive the downlink pathloss reference RSRP for TA validation as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [24]; 2> store the derived RSRP;. |
| ZTE | inactivePosSRS-ValidityAreaTAT-r18 ENUMERATED {ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity} this IE should be optional with need M |
| ZTE | R1’s parameter list says: pathlossReferenceRS-Pos may or may not be present. When pathloss RS is absent in the configuration, the UE determines the pathloss RS using a RS resource obtained from the SS/PBCH block of the camping cell that the UE uses to obtain MIB as the pathloss RS. Does this need to be captured in the field description in RRC? |
| CATT | This modification has not been agreed yet. Under what condition the lower layer will indicate SRS release request to RRC layer is not clear Upon receiving a positioning SRS configuration for RRC_INACTIVE release request from lower layers, the UE shall: 1> release the configured srs-PosRRC-Inactive, if configured; 1> release the configured srs-PosRRC-InactiveValidityArea, if configured.. |
| CATT | We wonder whether the usage of resume cause of the the RRC connection resumption should be limited to UE reselect out of the validity area. According to the modifications in the running CR, the following conditions may occur: - When UE reselects out of the validity area, the UE resume the RRC connection to request SRS, even if it does not detect a positioning event. - The UE keep staying in the validity area, and the configured SRS becomes invalid, it cannot request updated SRS with this resume cause. We think the trigger(s) of UE sending the new resume cause to request SRS configuration does not need to be reflected in the stage 3 procedure. Because it is not when UE reselects out of the validity area, it should request SRS with the resume cause. From our perspective, the trigger(s) of UE sending the new resume cause to request SRS configuration can be captured in stage 2 spec. And the stage 3 spec can be modified as follow. 1> else if the resumption of the RRC connection is triggered due to request srs-PosConfigValidityArea: 2> set the resumeCause to srs-RequestOrActivation; |
| CATT | The following agreement for TA in the parameter list R1-2310694 from RAN1 need to be reflected in RRC spec. The possible impact on RRC spec is that RRC needs indicate that to lower layer. And the field descriptiom of autonomousTA-AdjustmentEnabled also need to be updated. For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area, support the following to determine a valid TA: • The DL reference timing follows the DL timing of current camping cell. • By default, UE maintains the TA from the last serving cell. o UE can adjust its UL timing according to the change in DL reference timing. • If configured by the network, subject to UE capability, UE autonomously adjusts the TA, when cell-reselection happens.. |
| CATT | The following modifications are not needed. They are already captured in the MAC spec. 1> if nrofSS-BlocksToAverage or absThreshSS-BlocksConsolidation is not present or if absThreshSS-BlocksConsolidation is present and the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation: 2> derive the downlink pathloss reference RSRP for TA validation as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [24]; 2> store the derived RSRP; 1> else: 2> derive the downlink pathloss reference RSRP for TA validation as the linear average of the power values of up to nrofSS-BlocksToAverage of the highest beam measurement quantity values above absThreshSS-BlocksConsolidation, where each beam measurement quantity is described in TS 38.215 [24]; 2> store the derived RSRP.. |
| CATT | Update the remaining bit number. ResumeCause ::= ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, rna-Update, mps-PriorityAccess, mcs-PriorityAccess, srs-RequestOrActivation, spare1, spare2, spare3, spare4 } |

Open Questions for LPHAP

- 1)  whether SRS configuration will be released after UE moves out of validity area or only the timer is stopped.;

- 1>	else if cell reselection occurs when srs-PosRRC-InactiveValidityArea is configured and if the cell is not included in the srs-PosConfigValidityArea:

- 2>	indicate to the lower layer to stop inactivePosSRS-ValidityAreaTAT;

- 2>	release the srs-PosRRC-InactiveValidityArea.

Please provide your view on above:

1) Yes the release cause is fine

2) No the release cause is not needed

3) Other: should be discussed via contributions

| Company Name | Yes/No/Other |  |
| --- | --- | --- |
| ZTE | 2 or 3 | Prefer to not release the SRS configuration only due to moving out of validity area. In Rel-17 UE releases when cell-reselection, and to address the issue that UE moves out of the cell and quickly switches back, RRC has add a note to say gNB will always provide full SRS configuration in RRC_INACTIVE. To avoid patching in Rel-18, we should allow UE keep the SRS configuration when UE moves out of area |
| CATT | 2) and 3) | If UE release SRS when the inactivePosSRS-ValidityAreaTAT is stopped/expired, when UE restart this timer, all the gNBs within the validity area need to know that, there may need signalling enhancement on Xn and NRPPa message. More seriously, when the UE adjust TA autonomously, whether the timer need to be restarted and how RAN know that need to be discussed. If UE release SRS when it reselect out of the validity area, when UE reselect back to the validity area, it cannot use the configured SRS, and the UE needs to request SRS again. This is against with the goal of low power of LPHAP. And if delta configuration is used, there maybe some problems, because the NW does not know the UE released the SRS configuration. In summary, both of these two release causes are not suitable for SRS configuration with validity area. From our prespective, this issue needs to be discussed in next meeting with high priority. |

2)  Currently, there is an FFS on how to distinguish between preconfigured SRS validity area configuration and normal SRS configuration with validity area and further when to start/stop the timer.

2>	if srs-PosRRC-InactiveValidityArea is configured:

3>	apply the configuration and instruct MAC to start the inactivePosSRS-ValidityAreaTAT;

Editor’s Note: For preconfigured SRS, there is no need to start the inactivePosSRS-ValidityAreaTAT immediately. But for Periodic SRS the above clause would be needed. Agreement says: “Periodic SRS is supported to be configured with validity area. This agreement does not affect preconfigured SRS.” How to differentiate normal and preconfigured SRS. FFS How to start/stop the timer.

Rapporteur suggests that this should be resolved with contribution to next meeting.

2.6	Any other comments

Please provide any other comments below.

| Company Name | Comments |
| --- | --- |
| CATT | Based on our second comment for LPHAP CR, we would like to discuss whether the new resume cause for requesting SRS configuration can also be used for the case when UE stay in the validity area and the SRS becomes invalid. |

Conclusion

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

References