3GPP TSG-RAN2 #124	R2-23xxxxx

# Chicago, USA, 13~17 November 2024

Agenda item:	7.25.1.6 (NR_ATG-Core)

Source:	LG Electronics Inc.

Title: 	Introduction of ATG to 38.304_discussion

Document for:	Discussion and Decision

1. Introduction

This is to progress the following post email discussion:

[POST124][030][ATG] 38.304  CR (LG)

Intended outcome: Agree to 38.304

Deadline:  2 weeks

2. Discussion

Issue1: Codepoint of cellBarredATG in 5.3.1

RAN2#124 Agreements:

Similar to NTN, a 1 bit barring mechanism is introduced for ATG UEs.  Non-ATG UEs are barred with legacy barring bit in MIB.

RAN2 needs to introduce cellBarredATG as similar to NTN. Note that for NTN, cellBarredNTN has two codepoints {barred, notBarred}, and if cellBarredNTN is absent, UE considers the cell as barred for NTN access.

Even if cellBarredNTN has two codepoints, the necessity of “barred” codepoint for cellBarredATG is unclear (actually neither for cellBarredNTN as well), because absence of cellBarredATG already means that the cell is barred for ATG access. In other words, if a cell (ATG cell or even normal cell) wants to bar ATG access, it can choose not broadcasting cellBarredATG, rather than broadcasting cellBarredATG set to “barred”.

So, we have to options to implement the 1 bit barring field specific to ATG UEs:

- Alt1: cellBarredATG has a two codepoints {barred, notBarred}

- Alt2: cellBarredATG has a single codepoint {notBarred}

Question1: Which option do think is more reasonable?

| Company | Alt1/Alt2 | Comment |
| --- | --- | --- |

The draft CR to 38.304 uploaded to the same folder is currently written with the Alt1 as below. But, if majority companies prefer alt2, the draft CR will be modified accordingly.

5.3.1 Cell status and cell reservations Cell status and cell reservations are indicated in the MIB or SIB1 message as specified in TS 38.331 [3] by means of following fields: - cellBarred (IE type: "barred" or "not barred") Indicated in MIB message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is ignored by UEs supporting NTN while cellBarredNTN is included in SIB1. This field is ignored by UEs supporting ATG while cellBarredATG is included in SIB1. - cellBarredNTN (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is ignored if the UE does not support NTN connectivity. - cellBaredATG (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. cellBarredRedCap1Rx (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to RedCap UEs. - cellBarredRedCap2Rx (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to RedCap UEs. - cellReservedForOperatorUse (IE type: "reserved" or "not reserved") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is specified per PLMN or per SNPN. - cellReservedForOtherUse (IE type: "true") Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. - cellReservedForFutureUse (IE type: "true") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. NOTE 0: IAB-MT ignores the cellBarred, cellReservedForOperatorUse, cellReservedForFutureUse, and intraFreqReselection (i.e. treats intraFreqReselection as if it was set to allowed) as defined in TS 38.331 [3]. IAB-MT also ignores cellReservedForOtherUse for cell barring determination (i.e. NPN capable IAB-MT considers cellReservedForOtherUse for determination of an NPN-only cell) as defined in TS 38.331 [3]. - halfDuplexRedCapAllowed (IE type: "true") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to RedCap UEs. - iab-Support (IE type: "true") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is specified per PLMN or per SNPN. When cell status is indicated as "not barred" and "not reserved" for operator use and not "true" for other use and not "true" for future use, - UEs shall treat this cell as candidate during the cell selection and cell reselection procedures. When cell broadcasts any CAG-IDs or NIDs and the cell status is indicated as "not barred" and "not reserved" for operator use and "true" for other use, and not "true" for future use: - All NPN-capable UEs shall treat this cell as candidate during the cell selection and cell reselection procedures, other UEs shall treat this cell as if cell status is "barred". When cell status is indicated as "true" for other use, and either cell does not broadcast any CAG-IDs or NIDs or does not broadcast any CAG-IDs and the UE is not operating in SNPN Access Mode, - The UE shall treat this cell as if cell status is "barred". When cell status is indicated as "true" for future use, - The UE shall treat this cell as if cell status is "barred". When cellBarredNTN is not broadcast in this cell, - For NTN access, the UE shall treat this cell as if cell status is "barred". When halfDuplexRedCapAllowed is not broadcast in this cell, - The RedCap UE only capable of operating in half-duplex for FDD shall treat this cell as if cell status is "barred". When cellBarredATG is not broadcast in this cell, - For ATG access, the UE shall treat this cell as if cell status is "barred".

Issue2: PPowerClass expression in 5.2.3.2

RAN2#124 Agreements:

3	Introduce maxOutputPower-ATG-r18 with 5 bit INTEGER to indicate UE’s rate maximum output power. For ATG capable UE, it is mandatory with per band UE capability signaling.

4	With UE’s rate maximum output power, clarify the existing power class UE capabilities signaling (e.g., ue-PowerClass) does not apply to ATG. Clarify in stage 2 that CA/DC is not supported for ATG in this release

In clause 5.2.3.2 of 38.304, Srxlev derivation refers to PPowerClass. For maximum output power derivation, ATG UE does not apply ue-PowerClass capability bit but apply the new capability bit maxOutputPower-ATG-r18 field. According to ATG 38.331 UE capability CR, maximum output power value in dBm is calculated as maxOutputPower-ATG+22, given that the maxOutputPower-ATG value range of {1...18 } corresponds to {23… 40}dBm with 1dB granularity. Whether we need clarification on PPowerClass , we have two options:

- Alt1: Add clarification in 5.2.3.2 of 38.304 on PPowerClass such that for ATG UE, max RF output power. is derived based on maxOutputPower-ATG, not power class.

- Alt2: Do not add clarification in 5.2.3.2 of 38.304 on PPowerClass., but just rely on existing reference to TS 38.101-1[15].

Question2: Which option do think is more reasonable?

| Company | Alt1/Alt2 | Comment |
| --- | --- | --- |

The draft CR to 38.304 uploaded to the same folder is currently written with the Alt1 as below. But, if majority companies prefer Alt2, the draft CR will be modified accordingly.

| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in TS 38.101-1 [15]. For ATG UE, maximum RF output power of the UE is determined based on maxOutputPower-ATG as defined in TS 38.331 [3]. |
| --- | --- |

Issue3:  P-Max for ATG UE

RAN2#124 Agreements:

5	As per RAN4 LS, add clarification in the description of field P-Max that in ATG cell, actual value of P-Max = 9 + field value [dBm].

For maximum TX power level in a cell, 38.304 already refers to p-Max and NR-NS-PmaxList, and it is expected that the extra 9dB to derive P-Max value for ATG UE will be specified in 38.331. Then, there is no strong need to clarify it for Pcompensation or PEMAX1, PEMAX2 in 5.2.3.2.

| Pcompensation | For FR1, if the UE supports the additionalPmax in the NR-NS-PmaxList, if present, in SIB1, SIB2 and SIB4: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB) For FR2, Pcompensation is set to 0. For IAB-MT, Pcompensation is set to 0. |
| --- | --- |
| PEMAX1, PEMAX2 | Maximum TX power level of a UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in TS 38.101 [15]. If UE supports SUL frequency for this cell, PEMAX1 and PEMAX2 are obtained from the p-Max for SUL in SIB1 and NR-NS-PmaxList for SUL respectively in SIB1, SIB2 and SIB4 as specified in TS 38.331 [3], else PEMAX1 and PEMAX2 are obtained from the p-Max and NR-NS-PmaxList respectively in SIB1, SIB2 and SIB4 for normal UL as specified in TS 38.331 [3]. |

- Alt1: No clarification in 5.2.3.2  is needed for P-Max for ATG UE

- Alt2: Clarify in description of PEMAX1, PEMAX2 in 5.2.3.2 that P-Max for ATG UE is calculated by adding 9 dB to the p-max field.

Question3: Which option do think is more reasonable?

| Company | Alt1/Alt2 | Comment |
| --- | --- | --- |

The draft CR to 38.304 uploaded to the same folder is currently written with the Alt1. But, if majority companies prefer Alt2, the draft CR will be modified accordingly.

Issue4: Other necessary changes?

Question4: Companies are kindly requested to propose other changes to 38.304 not addressed above but necessary for ATG.

| Company | Propose changes |
| --- | --- |

3. Conclusion