3GPP TSG-RAN2 #124	R2-231xxxx

Chicago, USA, Nov 13-17, 2023

Agenda Item:	7.2.1

Source: 	Xiaomi

Title:  	TP for LPP capability signaling for bandwidth aggregation

Document for:	Discussion and Decision

# Introduction

RAN1 provided the feature list in RAN1-2312572 for bandwidth aggregation as below:

| 41. NR_pos_enh2 | 41-4-1 | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous within a MG for RRC_CONNECTED | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot | 13-1 | No | n/a | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous within a MG for RRC_CONNECTEDis not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: a) FR1 bands: {10, 20, 40, 50, 80, 100, 160, 200} b) FR2 bands: {100, 200, 400, 800} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in FG41-4-1 follows buffering capability type reported in FG13-1 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 13-1, or this value T should be equal or larger than the value T reported by FG 13-1 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each two linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 13-1 Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL)/measurement gap repetition period (MGRP) of no more than 30% | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-4-1a | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous within a MG for RRC_CONNECTED | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot | 41-4-1 | No | n/a | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous within a MG for RRC_CONNECTED is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: a) FR1 bands: {15, 20, 30, 40, 50, 60, 80, 100, 120, 140, 150, 160, 180, 200, 240, 300}} b) FR2 bands: {150, 200, 300, 400, 600, 800, 1000, 1200} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in FG41-4-1a follows buffering capability type reported in FG13-1 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 13-1 or this value T should be equal or larger than the value T reported by FG 13-1 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each three linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 13-1 Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL)/measurement gap repetition period (MGRP) of no more than 30% | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-4-1b | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot | 27-6 | No | n/a | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: a) FR1 bands: {10, 20, 40, 50, 80, 100, 160, 200} b) FR2 bands: {100, 200, 400, 800} Component 2 candidate values:a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in FG41-4-1b follows buffering capability type reported in FG13-1 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 27-6 or this value T should be equal or larger than the value T reported by FG 27-6 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each two linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 27-6 | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-4-1c | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Max number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot under it | 41-4-1b | No | n/a | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: a) FR1 bands: {15, 20, 30, 40, 50, 60, 80, 100, 120, 140, 150, 160, 180, 200, 240, 300} b) FR2 bands: {150, 200, 300, 400, 600, 800, 1000, 1200} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in FG41-4-1c follows buffering capability type reported in FG13-1 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 27-6 or this value T should be equal or larger than the value T reported by FG 27-6 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each three linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 27-6 | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-4-2 | PRS bandwidth aggregation with two PFL combinations | Support of PRS bandwidth aggregation with two PFL combinations | FFS | No | n/a | PRS bandwidth aggregation with two PFL combinations is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: More than one combination are measured in TDMed manner | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-3 | PRS bandwidth aggregation in RRC_CONNECTED — DL-TDOA | Support of PRS bandwidth aggregation in RRC_CONNECTED for DL-TDOA | 13-3, 41-4-1 | No | n/a | PRS bandwidth aggregation in RRC_CONNECTED for DL-TDOA is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-3a | PRS bandwidth aggregation in RRC_CONNECTED —Multi-RTT | Support of PRS bandwidth aggregation in RRC_CONNECTED FOR Multi-RTT | 13-4, 41-4-1 | No | n/a | PRS bandwidth aggregation in RRC_CONNECTED FOR Multi-RTT is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-4 | PRS bandwidth aggregation in RRC_ INACTIVE — DL-TDOA | Support of PRS bandwidth aggregation in RRC_ INACTIVE for DL-TDOA | 27-18a, 41-4-1b | No | n/a | PRS bandwidth aggregation in RRC_ INACTIVE for DL-TDOA is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-4a | PRS bandwidth aggregation in RRC_ INACTIVE —Multi-RTT | Support of PRS bandwidth aggregation in RRC_ INACTIVE for Multi-RTT | 27-18a, 41-4-1b | No | n/a | PRS bandwidth aggregation in RRC_ INACTIVE for Multi-RTT is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-5 | PRS bandwidth aggregation in RRC_IDLE — DL-TDOA | Support of PRS bandwidth aggregation in RRC_IDLE for DL-TDOA | 41-3-3, 41-4-1b | No | n/a | PRS bandwidth aggregation in RRC_IDLE for DL-TDOA is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-6 | Positioning SRS bandwidth aggregation in RRC_CONNECTED | 1. The number of supported aggregated carriers in intra band contiguous carriers 2. Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE 5. Max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation 6. Maximum number of aggregated SRS resources for bandwidth aggregation 7. Maximum number of aggregated SRS resources for bandwidth aggregation per slot 8. Support the same SRS power reduction across aggregated carriers | [13-8, 6-2] | Yes | n/a | Positioning SRS bandwidth aggregation in RRC_CONNECTED is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: {2,3,2and3} Component 2 candidate values: For 2 in Component 1: FR1 bands: {80, 100, 160, 200M} FR2 bands: {50, 100, 200, 400, 600, 800} For 3 in Component 1: FR1 bands: {80, 100, 160, 200, 300} FR2 bands: {50, 100, 200, 400, 600, 800, 1000, 1200} Component 5 candidate values: {1, 2, 4, 8, 12, 16} Component 6 candidate values: [Periodic: {1,2,4,8,16,32,64} Aperiodic: {0,1,2,4,8,16,32,64} Semi-persistent: {0,1,2,4,8,16,32,64}] Component 7 candidate values: [Periodic: {1,2,3,4,5,6,8,10,12,14} Aperiodic: {0,1,2,3,4,5,6,8,10,12,14} Semi-persistent: {0,1,2,3,4,5,6,8,10,12,14}] Note: The UE supports the simultaneous transmission in a coherent manner of 2 or 3 SRS resources in 2 or 3 intra-band contiguous CCs. Note: each two or three linked SRS resources are counted as 1 resource Note: A UE that support FG [13-8a] must signal a non-zero value for components 6 and 7 for aperiodic Need for location server to know if the feature is supported. UE only reports the number on bands for the current configured CA band combination. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-6a | Joint triggering by single Rel. 17 DCI | Support a Rel-17 single DCI scheduling positioning SRS resource sets across the linked carriers for SRS bandwidth aggregation in RRC_CONNECTED state | 41-4-6 | Yes | n/a | Joint triggering by single Rel. 17 DCI is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-7 | Positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED | 1. The number of supported aggregated carriers in intra band contiguous carriers 2. Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE 5. Max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation 6. Maximum number of aggregated SRS resources for bandwidth aggregation 7. Maximum number of aggregated SRS resources for bandwidth aggregation per slot 8. Support the same SRS power reduction across aggregated carriers 9. Guard period | FFS | Yes | n/a | Positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: {2,3,2and3} Component 2 candidate values: For 2 in Component 1: FR1 bands: {80, 100, 160, 200M} FR2 bands: {50, 100, 200, 400, 600, 800} For 3 in Component 1: FR1 bands: {80, 100, 160, 200, 300} FR2 bands: {50, 100, 200, 400, 600, 800, 1000, 1200} Component 5 candidate values: {1, 2, 4, 8, 12, 16} Component 6 candidate values: [Periodic: {1,2,4,8,16,32,64} Aperiodic: {0,1,2,4,8,16,32,64} Semi-persistent: {0,1,2,4,8,16,32,64}] Component 7 candidate values: [Periodic: {1,2,3,4,5,6,8,10,12,14} Aperiodic: {0,1,2,3,4,5,6,8,10,12,14} Semi-persistent: {0,1,2,3,4,5,6,8,10,12,14}] Note: The UE supports the simultaneous transmission in a coherent manner of 2 or 3 SRS resources in 2 or 3 intra-band contiguous CCs. Note: each two or three linked SRS resources are counted as 1 resource Need for location server to know if the feature is supported. UE only reports the number on bands for the current configured CA band combination. Note: Guard period is needed before and after the aggregated SRS transmissions when SRS resource is configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-8 | Positioning SRS bandwidth aggregation in RRC_INACTIVE | 1. The number of supported aggregated carriers in intra band contiguous carriers 2. Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE 5. Max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation 6. Maximum number of aggregated SRS resources for bandwidth aggregation 7. Maximum number of aggregated SRS resources for bandwidth aggregation per slot 8. Support the same SRS power reduction across aggregated carriers | [27-15b] | Yes | n/a | Positioning SRS bandwidth aggregation in RRC_INACTIVE is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {2,3,2and3} Component 2 candidate values: For 2 in Component 1: FR1 bands: {80, 100, 160, 200M} FR2 bands: {50, 100, 200, 400, 600, 800} For 3 in Component 1: FR1 bands: {80, 100, 160, 200, 300} FR2 bands: {50, 100, 200, 400, 600, 800, 1000, 1200} Component 5 candidate values: {1, 2, 4, 8, 12, 16} Component 6 candidate values: [Periodic: {1,2,4,8,16,32,64} Semi-persistent: {0,1,2,4,8,16,32,64}] Component 7 candidate values: [Periodic: {1,2,3,4,5,6,8,10,12,14} Semi-persistent: {0,1,2,3,4,5,6,8,10,12,14}] Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-19 | Use of DCI format 0_3 or 1_3 for multi-cell PDSCH/PUSCH scheduling to trigger SRS resources for bandwidth aggregation | For triggering of aperiodic SRS for positioning in RRC_CONNECTED, DCI formats 0_3/1_3 can be used to trigger aperiodic SRS for positioning across the scheduled multiple cells for bandwidth aggregation. | FFS | Yes | N.A. | DCI format 0_3 or 1_3 for multi-cell PDSCH/PUSCH scheduling cannot be used to trigger SRS resources for bandwidth aggregation in multiple CCs | Per FS | N.A. | N.A. | N.A. |  | Optional with capability signaling |

The UE features with FFS and/or marked with yellow are not implemented in the TP, therefore, the following UE features are implemented in the TP.

UE feature list:

- FG 41-4-1

- FG 41-4-1a

- FG 41-4-1b

- FG 41-4-1c

- FG 41-4-3

- FG 41-4-3a

- FG 41-4-4

- FG 41-4-4a

- FG 41-4-5

The following RAN4 features are captured as well.

| 37. NR_pos_enh2 | 37-2 | Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation for RRC_CONNECTED | 1. Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation | Component 1 RAN1 feature 41-4-1 | No | NA | UE does not support reduced number of samples in positioning measurements with PRS bandwidth aggregation | Per Band | No | No | NA | - Component 1 candidate value: true/false - Need for the LMF to know if the feature is supported: True | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37. NR_pos_enh2 | 37-2A | Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation for RRC_IDLE and RRC_INACTIVE | 1. Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation | Component 1 RAN1 feature 41-4-1 | No | NA | UE does not support reduced number of samples in positioning measurements with PRS bandwidth aggregation | Per Band | No | No | NA | - Component 1 candidate value: true/false - Need for the LMF to know if the feature is supported: True | Optional with capability signalling |

# TP

## 6.4.3	Common NR Positioning Information Elements

#### NR-DL-PRS-ProcessingCapability

The IE NR-DL-PRS-ProcessingCapability defines the common DL-PRS Processing capability. In the case of capabilities for multiple NR positioning methods are provided, the IE NR-DL-PRS-ProcessingCapability applies across the NR positioning methods and the target device shall indicate the same values for the capabilities in IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities.

The PRS-ProcessingCapabilityPerBand is defined for a single positioning frequency layer on a certain band (i.e., a target device supporting multiple positioning frequency layers is expected to process one frequency layer at a time).

-- ASN1START

NR-DL-PRS-ProcessingCapability-r16 ::= SEQUENCE {

prs-ProcessingCapabilityBandList-r16	SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

PRS-ProcessingCapabilityPerBand-r16,

maxSupportedFreqLayers-r16				INTEGER (1..4),

simulLTE-NR-PRS-r16						ENUMERATED { supported }		OPTIONAL,

...,

[[

dummy									ENUMERATED { m1, m2, ... }		OPTIONAL

]]

}

PRS-ProcessingCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16				FreqBandIndicatorNR-r16,

supportedBandwidthPRS-r16			CHOICE {

fr1										ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100},

fr2										ENUMERATED {mhz50, mhz100, mhz200, mhz400},

...

},

dl-PRS-BufferType-r16				ENUMERATED {type1, type2, ...},

durationOfPRS-Processing-r16		SEQUENCE {

durationOfPRS-ProcessingSymbols-r16	ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

durationOfPRS-ProcessingSymbolsInEveryTms-r16

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160,n320, n640, n1280},

...

},

maxNumOfDL-PRS-ResProcessedPerSlot-r16	SEQUENCE {

scs15-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs30-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs60-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs120-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

...,

[[

scs15-v1690								ENUMERATED {n6, n12}					OPTIONAL,

scs30-v1690								ENUMERATED {n6, n12}					OPTIONAL,

scs60-v1690								ENUMERATED {n6, n12}					OPTIONAL,

scs120-v1690							ENUMERATED {n6, n12}					OPTIONAL

]]

},

...,

[[

supportedDL-PRS-ProcessingSamples-RRC-CONNECTED-r17	ENUMERATED { supported }		OPTIONAL,

prs-ProcessingWindowType1A-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingWindowType1B-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingWindowType2-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingCapabilityOutsideMGinPPW-r17

SEQUENCE (SIZE(1..3)) OF

PRS-ProcessingCapabilityOutsideMGinPPWperType-r17

OPTIONAL,

dl-PRS-BufferType-RRC-Inactive-r17		ENUMERATED { type1, type2, ... }			OPTIONAL,

durationOfPRS-Processing-RRC-Inactive-r17	SEQUENCE {

durationOfPRS-ProcessingSymbols-r17			ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

durationOfPRS-ProcessingSymbolsInEveryTms-r17

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160,n320, n640, n1280},

...

}																					OPTIONAL,

maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17	SEQUENCE {

scs15-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs30-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs60-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs120-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

...

}																					OPTIONAL,

supportedLowerRxBeamSweepingFactor-FR2-r17	ENUMERATED { n1, n2, n4, n6 }			OPTIONAL

]],

[[

supportedDL-PRS-ProcessingSamples-RRC-Inactive-r17	ENUMERATED { supported }		OPTIONAL

]],

[[

prs-MeasurementWithoutMG-r17				ENUMERATED {cp, symbolDot25, symbolDot5,

slotDot5}					OPTIONAL

]],

[[

prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected-r18

PRS-BWA-TwoContiguousIntrabandInMG-r18

OPTIONAL,

prs-BWA-ThreeContiguousIntrabandInMG-RRC-Connected-r18

PRS-BWA-ThreeContiguousIntrabandInMG-r18

OPTIONAL,

prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleandInactive-r18

PRS-BWA-TwoContiguousIntrabandInMG-r18

OPTIONAL,

prs-BWA-ThreeContiguousIntrabandInMG-RRC-IdleandInactive-r18

PRS-BWA-ThreeContiguousIntrabandInMG-r18

OPTIONAL,

reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-Connected-r18		BOOLEAN          OPTIONAL,

reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleandInactive-r18    BOOLEAN    OPTIONAL

]]

}

PRS-ProcessingCapabilityOutsideMGinPPWperType-r17 ::= SEQUENCE {

prsProcessingType-r17						ENUMERATED { type1A, type1B, type2 },

ppw-dl-PRS-BufferType-r17					ENUMERATED { type1, type2, ... },

ppw-durationOfPRS-Processing1-r17			SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN-r17

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms4,

ms6, ms8, ms12, ms16, ms20, ms25, ms30, ms32, ms35,

ms40, ms45, ms50 },

ppw-durationOfPRS-ProcessingSymbolsT-r17

ENUMERATED { ms1, ms2, ms4, ms8, ms16, ms20, ms30, ms40, ms80,

ms160, ms320, ms640, ms1280 }

}																					OPTIONAL,

ppw-durationOfPRS-Processing2-r17			SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN2-r17

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms3, ms4, ms5,

ms6, ms8, ms12 },

ppw-durationOfPRS-ProcessingSymbolsT2-r17

ENUMERATED { ms4, ms5, ms6, ms8 }

}																					OPTIONAL,

ppw-maxNumOfDL-PRS-ResProcessedPerSlot-r17	SEQUENCE {

scs15-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs30-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs60-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs120-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

...

},

...,

[[

ppw-maxNumOfDL-Bandwidth-r17		CHOICE {

fr1										ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100},

fr2										ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}																					OPTIONAL

]]

}

PRS-BWA-TwoContiguoousIntrabandInMG-r18 ::= SEQUENCE {

maxNumOfTwoAggregatedDL-PRS-Bandwidth-FR1-r18   ENUMERATED {mhz10, mhz20, mhz40, mhz50,

mhz80, mhz100, mhz160, mhz200}

OPTIONAL,

maxNumOfTwoAggregatedDL-PRS-Bandwidth-FR2-r18   ENUMERATED {mhz100, mhz200, mhz400, mhz800}																							OPTIONAL,

maxNumOfDL-PRS-BandwidthPerPFL-FR1-r18

ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100}

OPTIONAL,

maxNumOfDL-PRS-BandwidthPerPFL-FR2-r18		ENUMERATED {mhz50, mhz100, mhz200, mhz400}

OPTIONAL,

dl-PRS-BufferTypeOfBWA-r18          			ENUMERATED {type1, type2},

prs-durationOfTwoPRS-BWA-Processing-r18			SEQUENCE {

prs-durationOfTwoPRS-BWA-ProcessingSymbolsN-r18

ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2, ms4, ms6, ms8, ms12,

ms16, ms20, ms25, ms30, ms32, ms35, ms40, ms45, ms50},

prs-durationOfTwoPRS-BWA-ProcessingSymbolsT-r18

ENUMERATED {ms8, ms16, ms20, ms30, ms40, ms80, ms160,

ms320, ms640, ms1280}

}                                                                                  OPTIONAL,

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r18	SEQUENCE {

scs15-r18									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs30-r18									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs60-r18									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL

},

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r18	SEQUENCE {

scs60-r18									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs120-r18									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL

}

}

PRS-BWA-ThreeContiguoousIntrabandInMG-r18 ::= SEQUENCE {

maxNumOfThreeAggregatedDL-PRS-Bandwidth-FR1-r18

ENUMERATED {mhz15, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz120, mhz140, mhz150, mhz180, mhz200, mhz240, mhz300}

OPTIONAL,

maxNumOfThreeAggregatedDL-PRS-Bandwidth-FR2-r18

ENUMERATED {mhz150, mhz200, mhz300, mhz400, mhz600, mhz800, mhz1000, mhz1200}

OPTIONAL,

maxNumOfDL-PRS-BandwidthPerPFL-FR1-r18

ENUMERATED {mhz5, mhz10, mhz20, mhz40, mhz50, mhz80, mhz100}

OPTIONAL,

maxNumOfDL-PRS-BandwidthPerPFL-FR2-r18

ENUMERATED {mhz50, mhz100, mhz200, mhz400}

OPTIONAL,

dl-PRS-BufferTypeOfBWA-r18          ENUMERATED {type1, type2},

prs-durationOfThreePRS-BWA-Processing-r18			SEQUENCE {

prs-durationOfThreePRS-BWA-ProcessingSymbolsN-r18

ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2, ms4, ms6, ms8, ms12, ms16, ms20, ms25, ms30, ms32, ms35, ms40, ms45, ms50},

prs-durationOfThreePRS-BWA-ProcessingSymbolsT-r18

ENUMERATED {ms8, ms16, ms20, ms30, ms40, ms80, ms160,

ms320, ms640, ms1280}

}                                                                             OPTIONAL,

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r18	SEQUENCE {

scs15-r18									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs30-r18									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs60-r18									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL

}

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r18	SEQUENCE {

scs60-r18									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs120-r18									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL

}

}

-- ASN1STOP

NR-DL-PRS-ProcessingCapability field descriptions

maxSupportedFreqLayers Indicates the maximum number of positioning frequency layers supported by UE.

simulLTE-NR-PRS Indicates whether the UE supports parallel processing of LTE PRS and NR PRS.

dummy This field is not used in the specification. If received it shall be ignored by the receiver.

supportedBandwidthPRS Indicates the maximum number of DL-PRS bandwidth in MHz, which is supported and reported by UE.

dl-PRS-BufferType Indicates DL-PRS buffering capability. Value type1 indicates sub-slot/symbol level buffering and value type2 indicates slot level buffering.

durationOfPRS-Processing Indicates the duration N of DL-PRS symbols in units of ms a UE can process every T ms assuming maximum DL-PRS bandwidth provided in supportedBandwidthPRS and comprises the following subfields: - durationOfPRS-ProcessingSymbols: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - durationOfPRS-ProcessingSymbolsInEveryTms: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. See NOTE 9.

maxNumOfDL-PRS-ResProcessedPerSlot Indicates the maximum number of DL-PRS resources that UE can process in a slot. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands.

supportedDL-PRS-ProcessingSamples-RRC-CONNECTED Indicates the UE capability for support of measurements based on measuring M=1 or M=2 (instances) of a DL-PRS Resource Set. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 1: This feature is supported for both UE-assisted and UE based positioning.

prs-ProcessingWindowType1A Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 1A refers to the determination of prioritization between DL-PRS and other DL signals/channels in all OFDM symbols within the PRS Processing Window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR. Enumerated value indicates supported priority handing options of DL-PRS: - option1: Support of "st1" and "st3" defined in clause 5.1.6.5 of TS 38.214 [45]. - option2: Support of "st1", "st2", and "st3" defined in clause 5.1.6.5 of TS 38.214 [45]. - option3: Support of "st1" only defined in clause 5.1.6.5 of TS 38.214 [45]. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 2: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP. NOTE 2a: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS.

prs-ProcessingWindowType1B Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 1B refers to the determination of prioritization between DL-PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected. Enumerated value indicates supported priority handing options of DL-PRS (see prs-ProcessingWindowType1A). The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 3: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP. NOTE 3a: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS.

prs-ProcessingWindowType2 Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 2 refers to the determination of prioritization between DL-PRS and other DL signals/channels only in DL-PRS symbols within the PRS processing window. Enumerated value indicates supported priority handing options of DL-PRS (see prs-ProcessingWindowType1A). The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 4: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP. NOTE 4a: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS.

prs-ProcessingCapabilityOutsideMGinPPW Indicates the DL-PRS Processing Capability outside MG of each of the supported PPW Type in the case the UE supports multiple PPW Types in a band and comprises the following subfields: - prsProcessingType: Indicates the DL-PRS Processing Window Type for which the prs-ProcessingCapabilityOutsideMGinPPW are provided. - ppw-dl-PRS-BufferType: Indicates DL-PRS buffering capability. Value 'type1' indicates sub-slot/symbol level buffering and value 'type2' indicates slot level buffering. - ppw-durationOfPRS-Processing1: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum DL-PRS bandwidth provided in ppw-maxNumOfDL-Bandwidth and comprises the following subfields: - ppw-durationOfPRS-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - ppw-durationOfPRS-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. - ppw-durationOfPRS-Processing2: Indicates the duration of DL-PRS symbols N2 in units of ms a UE can process inT2 ms assuming maximum DL-PRS bandwidth provided in ppw-maxNumOfDL-Bandwidth and comprises the following subfields: - ppw-durationOfPRS-ProcessingSymbolsN2: This field specifies the values for N2. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12 ms. - ppw-durationOfPRS-ProcessingSymbolsT2: This field specifies the values for T2. Enumerated values indicate 4, 5, 6, 8 ms. - ppw-maxNumOfDL-PRS-ResProcessedPerSlot: Indicates the maximum number of DL-PRS resources that UE can process in a slot. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands. - ppw-maxNumOfDL-Bandwidth: Indicates the maximum number of DL PRS bandwidth in MHz, which is supported and reported by UE for PRS measurement outside MG within the PPW. The UE can include this field only if the UE supports one of prs-ProcessingWindowType1A, prs-ProcessingWindowType1B and prs-ProcessingWindowType2. Otherwise, the UE does not include this field. NOTE 5: A UE that supports one of prs-ProcessingWindowType1A, prs-ProcessingWindowType1B or prs-ProcessingWindowType2 shall always include the prs-ProcessingCapabilityOutsideMGinPPW. NOTE 6: The (N, T) UE capability in ppw-durationOfPRS-Processing1 is interpreted as in NOTE 9, and the UE is expected to receive the DL-PRS within the PRS processing window but the processing of the received DL-PRS may be outside a DL-PRS processing window. NOTE 7: The (N2, T2) UE capability in ppw-durationOfPRS-Processing2 is interpreted such that the UE is capable of measuring up to N2 ms DL-PRS within a PPW and is capable of completing the DL-PRS processing within the PPW, e.g., if the time duration from the last symbol of the measured DL-PRS resource(s) inside the PPW to the end of PPW is not smaller than T2 ms. NOTE 8: A UE which supports prs-ProcessingCapabilityOutsideMGinPPW shall support either ppw-durationOfPRS-Processing1 or ppw-durationOfPRS-Processing2, but not both for each supported type in a band.

dl-PRS-BufferType-RRC-Inactive Indicates DL-PRS buffering capability in RRC_INACTIVE state. Value 'type1' indicates sub-slot/symbol level buffering and value 'type2' indicates slot level buffering.

durationOfPRS-Processing-RRC-Inactive Indicates the duration N of DL-PRS symbols in units of ms a UE can process every T ms in RRC_INACTIVE state assuming maximum DL-PRS bandwidth provided in supportedBandwidthPRS and comprises the following subfields: - durationOfPRS-ProcessingSymbols: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - durationOfPRS-ProcessingSymbolsInEveryTms: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. See NOTE 9.

maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive Indicates the maximum number of DL-PRS resources a UE can process in a slot in RRC_INACTIVE state. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands.

supportedLowerRxBeamSweepingFactor-FR2 Indicates support of the lower Rx beam sweeping factor than 8 for FR2. Enumerated value indicates the number of Rx beam sweeping factors supported.

supportedDL-PRS-ProcessingSamples-RRC-Inactive Indicates the UE capability for support of reduced number of samples for PRS measurement in RRC_INACTIVE state. The UE can include this field only if the UE supports prs-ProcessingRRC-Inactive defined in TS 38.331 [35]. Otherwise, the UE does not include this field.

prs-MeasurementWithoutMG Indicates the UE capability for support of Rx timing difference between the serving cell and non-serving cell for PRS measurement within a PPW. Value 'cp' indicates one CP length, value 'symbolDot25' indicates 0.25 symbol length, value 'symbolDot5' indicates 0.5 symbol length and value 'slotDot5' indicates 0.5 slot length. The UE can include this field only if the UE supports one of prs-ProcessingWindowType1A, prs-ProcessingWindowType1B and prs-ProcessingWindowType2. Otherwise, the UE does not include this field.

prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected Indicates the UE capability for support of DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous within a MG for RRC_CONNECTED sate and and comprises the following subfields: - maxNumOfTwoAggregatedDL-PRS-Bandwidth-FR1: Indicates the maximum aggregated DL PRS bandwidth in MHz for FR1, which is supported and reported by UE. - maxNumOfTwoAggregatedDL-PRS-Bandwidth-FR2: Indicates the maximum aggregated DL PRS bandwidth in MHz for FR2, which is supported and reported by UE. - maxNumOfDL-PRS-BandwidthPerPFL-FR1: Indicates the maximum DL PRS bandwidth in MHz for FR1, per PFL. - maxNumOfDL-PRS-BandwidthPerPFL-FR2: Indicates the maximum DL PRS bandwidth in MHz for FR2, per PFL. - dl-PRS-BufferTypeOfBWA: Indicates the DL PRS buffering capability. - prs-durationOfTwoPRS-BWA-Processing: Indicates the duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. - prs-durationOfTwoPRS-BWA-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - prs-durationOfTwoPRS-BWA-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 2560 ms. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1: Indicates the Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot for FR1. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2: Indicates the Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot for FR2. The UE can include this field only if the UE supports ProcessingCapabilityPerBand. Otherwise, the UE does not include this field. NOTE10:: dl-PRS-BufferTypeOfBWA follows buffering capability type reported in ProcessingCapabilityPerBand. NOTE11: The value N should be equal or smaller than the value N reported by ProcessingCapabilityPerBand, or this value T should be equal or larger than the value T reported by ProcessingCapabilityPerBand. NOTE12: Each two linked PRS resources are counted as 1 resource NOTE13: maxNumOfAggregatedDL-PRS-ResourcePerSlot should be equal or smaller than the value reported by ProcessingCapabilityPerBand. NOTE14: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL)/measurement gap repetition period (MGRP) of no more than 30%.

prs-BWA-ThreeContiguousIntrabandInMG-RRC-Connected Indicates the UE capability for support of DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous within a MG for RRC_CONNECTED sate and and comprises the following subfields: - maxNumOfThreeAggregatedDL-PRS-Bandwidth-FR1: Indicates the maximum aggregated DL PRS bandwidth in MHz of for FR1, which is supported and reported by UE. - maxNumOfThreeAggregatedDL-PRS-Bandwidth-FR2: Indicates the maximum aggregated DL PRS bandwidth in MHz for FR2, which is supported and reported by UE. - maxNumOfDL-PRS-BandwidthPerPFL-FR1: Indicates the maximum DL PRS bandwidth in MHz for FR1, per PFL - maxNumOfDL-PRS-BandwidthPerPFL-FR2: Indicates the maximum DL PRS bandwidth in MHz for FR2, per PFL - dl-PRS-BufferTypeOfBWA: Indicates the DL PRS buffering capability. - prs-durationOfThreePRS-BWA-Processing: Indicates the duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. - prs-durationOfThreePRS-BWA-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - prs-durationOfThreePRS-BWA-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 3840 ms. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1: Indicates the Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot for FR1. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2: Indicates the Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot for FR2. The UE can include this field only if the UE supports prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field. : Component 3 in FG41-4-1 follows buffering capability type reported in FG13-1 NOTE15:: dl-PRS-BufferTypeOfBWA follows buffering capability type reported in ProcessingCapabilityPerBand. NOTE16: The value N should be equal or smaller than the value N reported by ProcessingCapabilityPerBand, or this value T should be equal or larger than the value T reported by ProcessingCapabilityPerBand. NOTE17: Each three linked PRS resources are counted as 1 resource NOTE18: maxNumOfAggregatedDL-PRS-ResourcePerSlot should be equal or smaller than the value reported by ProcessingCapabilityPerBand. NOTE19: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL)/measurement gap repetition period (MGRP) of no more than 30%.

prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleandInactive Indicates the UE capability for support of DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous within a MG for RRC_INACTIVE and RRC_IDLE sate. The UE can include this field only if the UE supports DL PRS processing capabilities in RRC inactive state. Otherwise, the UE does not include this field.

prs-BWA-ThreeContiguousIntrabandInMG-RRC-IdleandInactive Indicates the UE capability for support of DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous within a MG for RRC_INACTIVE and RRC_IDLE sate. The UE can include this field only if the UE supports prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleandInactive. Otherwise, the UE does not include this field.

reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-Connected Indicates UE whether supports reduced number of samples in positioning measurements with PRS bandwidth aggregation for RRC_CONNECTED. TRUE means supported and FALSE means not supported. The UE can indicate TRUE only if the UE supports prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE indicates FALSE.

reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleandInactive Indicates UE whether supports reduced number of samples in positioning measurements with PRS bandwidth aggregation for RRC_IDLE and RRC_INACTIVE. TRUE means supported and FALSE means not supported. The UE can indicate TRUE only if the UE supports prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE indicates FALSE.

NOTE 9: When the target device provides the durationOfPRS-Processing capability (N, T) for any time window defined in TS 38.214 [45] clause 5.1.6.5, the target device should be capable of processing all DL-PRS resources within , if - where K is defined in the TS 38.214 [45] clause 5.1.6.5, and - the number of DL-PRS Resources in each slot does not exceed the maxNumOfDL-PRS-ResProcessedPerSlot, and - the configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) is as specified in TS 38.133 [46].

## 6.5.10.6a	NR DL-TDOA Capability Information Elements

#### NR-DL-TDOA-MeasurementCapability

The IE NR-DL-TDOA-MeasurementCapability defines the DL-TDOA measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for DL-TDOA. Otherwise, the UE does not include this IE.

-- ASN1START

NR-DL-TDOA-MeasurementCapability-r16 ::= SEQUENCE {

dl-RSTD-MeasurementPerPairOfTRP-FR1-r16			INTEGER (1..4),

dl-RSTD-MeasurementPerPairOfTRP-FR2-r16			INTEGER (1..4),

supportOfDL-PRS-RSRP-MeasFR1-r16				ENUMERATED { supported}	OPTIONAL,

supportOfDL-PRS-RSRP-MeasFR2-r16				ENUMERATED { supported}	OPTIONAL,

...,

[[

nr-UE-TEG-Capability-r17						NR-UE-TEG-Capability-r17			OPTIONAL,

dl-tdoa-MeasCapabilityBandList-r17				SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

DL-TDOA-MeasCapabilityPerBand-r17	OPTIONAL

]]

}

DL-TDOA-MeasCapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17				FreqBandIndicatorNR-r16,

supportOfDL-PRS-FirstPathRSRP-r17	ENUMERATED { supported }			OPTIONAL,

dl-PRS-MeasRRC-Inactive-r17			ENUMERATED { supported }			OPTIONAL,

...,

[[

supportOfDL-PRS-BWA-RRC-Connected-r18   ENUMERATED { supported }			OPTIONAL,

supportOfDL-PRS-BWA-RRC-Inactive-r18    ENUMERATED { supported }			OPTIONAL,

supportOfDL-PRS-BWA-RRC-Idle-r18        ENUMERATED { supported }			OPTIONAL

]]

}

-- ASN1STOP

NR-DL-TDOA-MeasurementCapability field descriptions

dl-RSTD-MeasurementPerPairOfTRP-FR1 Indicates number of DL RSTD measurements per pair of TRPs on FR1.

dl-RSTD-MeasurementPerPairOfTRP-FR2 Indicates number of DL RSTD measurements per pair of TRPs on FR2.

supportOfDL-PRS-RSRP-MeasFR1 Indicates whether the UE supports DL-PRS RSRP measurement for DL-TDOA on FR1.

supportOfDL-PRS-RSRP-MeasFR2 Indicates whether the UE supports DL-PRS RSRP measurement for DL-TDOA on FR2.

nr-UE-TEG-Capability Indicates the UE TEG capability.

supportOfDL-PRS-FirstPathRSRP Indicates whether the target device supports DL-PRS RSRPP of first path measurement for DL-TDOA. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. The UE supporting additionalPathsReport and supportOfDL-PRS-FirstPathRSRP shall support RSRPP reporting for K=1 or 2 additional paths.

dl-PRS-MeasRRC-Inactive This field, if present, indicates that the target device supports DL-PRS measurement in RRC_INACTIVE state. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and dl-PRS-BufferType-RRC-Inactive. Otherwise, the UE does not include this field. NOTE 1: This capability is applicable to both, UE-assisted and UE-based DL-TDOA. NOTE 2: The capabilities NR-DL-PRS-ResourcesCapability, dl-RSTD-MeasurementPerPairOfTRP-FR1, dl-RSTD-MeasurementPerPairOfTRP-FR2, supportOfDL-PRS-RSRP-MeasFR1, supportOfDL-PRS-RSRP-MeasFR2, simul-NR-DL-AoD-DL-TDOA are the same in RRC_INACTIVE state.

supportOfDL-PRS-BWA-RRC-Connected Indicates whether the target device supports PRS bandwidth aggregation in RRC_CONNECTED for DL-TDOA. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field.

supportOfDL-PRS-BWA-RRC-Inactive Indicates whether the target device supports PRS bandwidth aggregation in RRC_INACTIVE for DL-TDOA. The UE can include this field only if the UE supports dl-PRS-MeasRRC-Inactive and prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleandInactive. Otherwise, the UE does not include this field.

supportOfDL-PRS-BWA-RRC-Idle Indicates the target device whether supports PRS bandwidth aggregation in RRC_IDLE for DL-TDOA. The UE can include this field only if the UE supports of PRS measurement in RRC_IDLE and prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleandInactive. Otherwise, the UE does not include this field.

## 6.5.12.6a	NR Multi-RTT Capability Information Elements

#### NR-Multi-RTT-MeasurementCapability

The IE NR-Multi-RTT-MeasurementCapability defines the Multi-RTT measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for Multi-RTT. Otherwise, the UE does not include this IE;

-- ASN1START

NR-Multi-RTT-MeasurementCapability-r16 ::= SEQUENCE {

maxNrOfRx-TX-MeasFR1-r16					INTEGER (1..4)						OPTIONAL,

maxNrOfRx-TX-MeasFR2-r16					INTEGER (1..4)						OPTIONAL,

supportOfRSRP-MeasFR1-r16					ENUMERATED { supported }			OPTIONAL,

supportOfRSRP-MeasFR2-r16					ENUMERATED { supported }			OPTIONAL,

srs-AssocPRS-MultiLayersFR1-r16				ENUMERATED { supported }			OPTIONAL,

srs-AssocPRS-MultiLayersFR2-r16				ENUMERATED { supported }			OPTIONAL,

...,

[[

nr-UE-TEG-Capability-r17					NR-UE-TEG-Capability-r17			OPTIONAL,

multi-RTT-MeasCapabilityBandList-r17		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

Multi-RTT-MeasCapabilityPerBand-r17

OPTIONAL

]]

}

Multi-RTT-MeasCapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17					FreqBandIndicatorNR-r16,

supportOfDL-PRS-FirstPathRSRP-r17		ENUMERATED { supported }			OPTIONAL,

dl-PRS-MeasRRC-Inactive-r17				ENUMERATED { supported }			OPTIONAL,

...,

[[

supportOfDL-PRS-BWA-RRC-Connected-r18        ENUMERATED { supported }			OPTIONAL,

supportOfDL-PRS-BWA-RRC-Inactive-r18         ENUMERATED { supported }			OPTIONAL

]]

}

-- ASN1STOP

NR-Multi-RTT-MeasurementCapability field descriptions

maxNrOfRx-TX-MeasFR1 Indicates the maximum number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL-PRS resource/resource set on FR1.

maxNrOfRx-TX-MeasFR2 Indicates the maximum number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL-PRS resource/resource set on FR2.

srs-AssocPRS-MultiLayersFR1 Indicates whether the UE supports measurements derived on one or more DL-PRS resource/resource sets which may be in different positioning frequency layers for SRS transmitted in a single CC. PRS and SRS may be on different bands. This is for FR1 only.

srs-AssocPRS-MultiLayersFR2 Indicates whether the UE supports measurements derived on one or more DL-PRS resource/resource sets which may be in different positioning frequency layers for SRS transmitted in a single CC. PRS and SRS may be on different bands. This is for FR2 only.

supportOfRSRP-MeasFR1 Indicates whether the UE supports RSRP measurement for Multi-RTT on FR1.

supportOfRSRP-MeasFR2 Indicates whether the UE supports RSRP measurement for Multi-RTT on FR2.

nr-UE-TEG-Capability Indicates the UE TEG capability.

supportOfDL-PRS-FirstPathRSRP Indicates whether the target device supports DL-PRS RSRPP of first path measurement for Multi-RTT. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. The UE supporting additionalPathsReport and supportOfDL-PRS-FirstPathRSRP shall support RSRPP reporting for K=1 or 2 additional paths.

dl-PRS-MeasRRC-Inactive This field, if present, indicates that the target device supports DL-PRS measurement in RRC_INACTIVE state. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and dl-PRS-BufferType-RRC-Inactive. Otherwise, the UE does not include this field. NOTE: The capabilities NR-DL-PRS-ResourcesCapability, maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1, supportOfRSRP-MeasFR2, srs-AssocPRS-MultiLayersFR1, srs-AssocPRS-MultiLayersFR2, simul-NR-DL-AoD-Multi-RTT are the same in RRC_INACTIVE state.

supportOfDL-PRS-BWA-RRC-Connected Indicates whether the target device supports PRS bandwidth aggregation in RRC_CONNECTED for Multi-RTT. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field.

supportOfDL-PRS-BWA-RRC-Inactive Indicates whether the target device supports PRS bandwidth aggregation in RRC_INACTIVE for Multi-RTT. The UE can include this field only if the UE supports dl-PRS-MeasRRC-Inactive and prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleandInactive. Otherwise, the UE does not include this field.