3GPP TSG-RAN2 #124	R2-2312727

Chicage, Nov 13-17, 2023

Agenda Item:	7.2.1

Source: 	Xiaomi

Title:  	TP for SLPP capability signalling

Document for:	Discussion and Decision

# Introduction

RAN1 has sent RAN1 feature list in R1-2312572. For those RAN1 feature list marked as yellow, they are not implemented.

RAN2 also agreed some RAN2 related capabilities:

Agreements #123bis:

Introduce the UE capability on supporting positioning mode(i.e. UE based, UE assisted) per positioning method in SLPP.

Introduce the UE capability on supporting periodical reporting per positioning method in SLPP.

Introduce the UE capability on supporting lower value of response time (e.g. 10ms) per positioning method in SLPP.

Agreements #124:

SL-PRS-related capabilities are grouped according to the table in R2-2312762.

Positioning method specific capabilities are included in the positioning method specific capability IE.

RAN2 to agree that periodical reporting capability is indicated per positioning mode per positioning method.

10ms granularity response time is indicated per positioning mode per positioning method.

In this contribution, we provide the TP for 38.355 for UE capabilities based on RAN1 and RAN2 feature list.

# TP for SLPP capability--TS38.355

- –	Common-SL-PRS-Methods-ProvideCapabilities

-- ASN1START

-- TAG-CommonSLPRSMethodsProvideCapabilities-START

Common-SL-PRS-Methods-ProvideCapabilities ::= SEQUENCE {

sl-PRS-CapabilityBandList					SEQUENCE (SIZE (1..maxBands)) OF

SL-PRS-CapabilityPerBand,

...

}

SL-PRS-CapabilityPerBand ::= SEQUENCE {

--R1 41-1-19 ARP location provision for sidelink as assistance data

sl-PositioningARP-LocationProvision				ENUMATED {supported}				OPTIONAL,

--R1 41-1-19a Report of Rx ARP-ID with SL positioning measurements

sl-PositioningMeasReportWithRxARP-ID				ENUMATED {supported}				OPTIONAL,

--R1 41-1-19b Report of Tx ARP-ID  for location calculation measurements as assistance data

sl-PositioningTxARP-ID-Report				ENUMATED {supported}				OPTIONAL,

...

}

-- TAG-CommonSLPRSMethodsProvideCapabilities-STOP

-- ASN1STOP

Common-SL-PRS-Methods-ProvideCapabilities field descriptions

sl-PositioningARP-LocationProvision Indicates whether UE supports of ARP location provision for sidelink as assistance data.

sl-PositioningMeasReportWithARP-ID Indicates whether UE supports providing Rx ARP-ID with SL positioning measurements.

sl-PositioningTxARP-ID-Report Indicates whether UE supports reporting of Tx ARP-ID to LMF or another UE for location calculation measurements as assistance data.

- –	SL-TDOA-ProvideCapabilities

The IE SL-TDOA-ProvideCapabilities is used to indicate the support of SL-TDOA and to provide SL-TDOA positioning capabilities.

-- ASN1START

-- TAG-SLTDOAPROVIDECAPABILITIES-START

SL-TDOA-ProvideCapabilities ::= SEQUENCE {

positioningModes							PositioningModes,

ten-ms-unit-ResponseTime					PositioningModes						OPTIONAL,

periodicalReporting						PositioningModes						OPTIONAL,

...

}

-- TAG-SLTDOAPROVIDECAPABILITIES-STOP

-- ASN1STOP

SL-TDOA-ProvideCapabilities field descriptions

periodicalReporting This field, if present, specifies the positioning modes for which the UE supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the UE does not support periodicalReporting in CommonIEsRequestLocationInformation.

positioningModes This field specifies the SL-TDOA mode(s) supported by the UE.

ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the UE supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the UE does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

- –	SL-TOA-ProvideCapabilities

The IE SL-TOA-ProvideCapabilities is used to indicate the support of SL-TOA and to provide SL-TOA positioning capabilities.

-- ASN1START

-- TAG-SLTOAPROVIDECAPABILITIES-START

SL-TOA-ProvideCapabilities ::= SEQUENCE {

positioningModes							PositioningModes,

ten-ms-unit-ResponseTime					PositioningModes						OPTIONAL,

periodicalReporting						PositioningModes						OPTIONAL,

...

}

-- TAG-SLTOAPROVIDECAPABILITIES-STOP

-- ASN1STOP

SL-TOA-ProvideCapabilities field descriptions

periodicalReporting This field, if present, specifies the positioning modes for which the UE supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the UE does not support periodicalReporting in CommonIEsRequestLocationInformation.

positioningModes This field specifies the SL-TDOA mode(s) supported by the UE.

ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the UE supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the UE does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

- –	SL-AOA-ProvideCapabilities

The IE SL-AOA-ProvideCapabilities is used to indicate the support of SL-AOA and to provide SL-AOA positioning capabilities.

-- ASN1START

-- TAG-SLAOAPROVIDECAPABILITIES-START

SL-AOA-ProvideCapabilities ::= SEQUENCE {

sl-AOA-CapabilityBandList					SEQUENCE (SIZE (1..maxBands)) OF

SL-AOA-CapabilityPerBand,

positioningModes							PositioningModes,

ten-ms-unit-ResponseTime					PositioningModes						OPTIONAL,

periodicalReporting						PositioningModes						OPTIONAL,

...

}

SL-AOA-CapabilityPerBand ::= SEQUENCE {

--R1 41-1-22 Provision of assistance information for SL AoA measurement to measuring UE

sl-AOA-AssistantInfoProvision				ENUMATED {supported}				OPTIONAL,

...

}

-- TAG-SLPRSAOAPROVIDECAPABILITIES-STOP

-- ASN1STOP

SL-AOA-ProvideCapabilities field descriptions

periodicalReporting This field, if present, specifies the positioning modes for which the UE supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the UE does not support periodicalReporting in CommonIEsRequestLocationInformation.

positioningModes This field specifies the SL-TDOA mode(s) supported by the UE.

sl-AOA-AssistantInfoProvision Indicates whether UE supports Provision of assistance information (Expected SL-AoA value and uncertainty range) for SL AoA measurement to measuring UE.

ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the UE supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the UE does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

- –	SL-RTT-ProvideCapabilities

The IE SL-RTT-ProvideCapabilities is used to indicate the support of SL-RTT and to provide SL-RTT positioning capabilities.

-- ASN1START

-- TAG-SLRTTPROVIDECAPABILITIES-START

SL-RTT-ProvideCapabilities ::= SEQUENCE {

positioningModes							PositioningModes,

ten-ms-unit-ResponseTime					PositioningModes						OPTIONAL,

periodicalReporting						PositioningModes						OPTIONAL,

...

}

-- TAG-SLRTTPROVIDECAPABILITIES-STOP

-- ASN1STOP

SL-RTT-ProvideCapabilities field descriptions

periodicalReporting This field, if present, specifies the positioning modes for which the UE supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the UE does not support periodicalReporting in CommonIEsRequestLocationInformation.

positioningModes This field specifies the SL-TDOA mode(s) supported by the UE.

ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the UE supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the UE does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

- Change to 6.3.1	Common information elements

- –	PositioningModes

The IE PositioningModes is used to indicate several positioning modes using a bit map.

-- ASN1START

PositioningModes ::= SEQUENCE {

posModes		BIT STRING {	ue-based		(0),

ue-assisted	(1) } (SIZE (1..8)),

...

}

-- ASN1STOP

PositioningModes field descriptions

posModes This field specifies the positioning mode(s). This is represented by a bit string, with a one-value at the bit position means the particular positioning mode is addressed; a zero-value means not addressed.