3GPP TSG RAN meeting #102	 RP-23xxxx

Edinburg, GB, December 11-15, 2023

## Status Report to TSG

Agenda item: 10.2.1 IoT (Internet of Things) NTN (non-terrestrial network) enhancements

| WI / SI Name | IoT (Internet of Things) NTN (non-terrestrial network) enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: - |
| Acronym | IoT_NTN_enh |  |  |  |  |
| Unique ID | 941004 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-231407 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: - |  |
| Overall Completion level | Study Item: | Core part: Overall: 100% For information RAN1: 100% RAN2: 100% RAN3: 100% RAN4: 100% | Performance Part: 10% | Testing part: - |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Abhishek Roy |
|  | Company | MediaTek |
|  | Email | abhishek.roy@mediatek.com |
| Secondary WG |  | RAN2 |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

###### RAN1#114-bis, October 9th – October 13th, 2023, Xiamen, China

Agreements on “8.13.3 Disabling of HARQ feedback for IoT NTN”

Agreement

Confirm the following working assumptions from RAN1#113:

For single TB scheduled by DCI,

- Working assumption 2 For Option 1 + Option 3 DCI based overridden mechanism, for a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI, the NBIoT UE does not wait for an RTT+3ms (i.e., till subframe n+Kmac+3 in TS36.213 section 16.6) before monitoring NPDCCH for the same HARQ process (or monitoring any NPDCCH for the case of single HARQ process configuration).

Agreement

The TP1b in section 13 of R1-2310356 is endorsed for TS36.213 clause 7.3.

Agreement

The TP2b in R1-2310356 is endorsed for TS36.213 clause 16.4.2.

Agreement

There is ambiguity for definition of NTB in clause 7.1.7.1 and 10.2 as follows:

- For clause 10.2: NTB is the number of TBs with HARQ feedback enabled

- For clause 7.1.7.1: NTB is the number of scheduled TBs

It is recommended to the spec editor of TS36.213 to resolve that ambiguity accounting for HARQ feedback enabling/disabling.

Agreements on “8.13.4	Improved GNSS operations for IoT NTN”

Agreement

When timeAlignmentTimer is infinity, the duration X is equal to Y. Network can configure Y via a 3-bit field at least with component values [sf500, sf750, sf1280, sf1920, sf2560, sf5120, sf10240].

FFS: whether there is a new value.

Agreement

The feature of “UL transmission after original validity duration expires with duration X” can be enabled/disabled by network via RRC signalling.

Agreement

For the aperiodic GNSS measurement gap triggered by eNB with MAC CE, the start time of the gap should be at p+ X2, where p is the end of HARQ feedback transmission subframe/slot when HARQ feedback for the MAC CE is enabled and X2 is a predefined value, down select

- Alt- A: X2 = 1ms

- Alt- B: X2 = 2ms

- Alt- C: X2 = 3ms

- Alt- E: X2 = 1ms for NB-IoT, X2 = 4ms for eMTC

Agreement

New texts for GNSS measurement gaps for NB-IoT and eMTC in TS36.213 should be added to capture the determination of the start time of GNSS measurement gap triggered by MAC CE.

Agreement

For the aperiodic GNSS measurement gap triggered by eNB with MAC CE, the start time of the gap should be at p+ X2, where p is the end of HARQ feedback transmission subframe/slot when HARQ feedback for the MAC CE is enabled and X2 is predefined value, where X2 = 2ms.

Agreement

Endorsed the TP below for TS36.213, and leave it to the spec editor whether to create new clauses for this TP and if so to decide the title of the new clauses.

- Reason for change: In the TS 36.213 v18.0.0, the determination of the start time of GNSS measurement gap triggered by MAC CE for IoT NTN is not captured.

- Summary of change: Introduce the determination of the start time of GNSS measurement gap triggered by MAC CE for IoT NTN in TS36.213.

- Consequence if not approved: The RAN1 agreements for the start time of GNSS measurement gap triggered by MAC CE are not captured in specification.

================== Start of TP #1 for TS 36.213 =================== < Unchanged parts are omitted > 16.10 [GNSS measurement gap related procedures for a NB-IoT UE] For a NB-IoT UE in a NTN FDD serving cell, when the UE receives a GNSS Measurement Command MAC CE in a NPDSCH ending in DL subframe n - if the UE shall not provide HARQ-ACK information for the HARQ process associated with the transport block in the NPDSCH carrying the GNSS Measurement Command MAC CE, - the UE shall assume the start of the measurement gap in subframe n+12; - otherwise, - the UE shall assume the start of the measurement gap in subframe k+2, where k is the first DL subframe after the end of the transmission of the NPUSCH carrying ACK/NACK response for the HARQ process associated with the transport block in the NPDSCH. < Unchanged parts are omitted > 18 [GNSS measurement gap related procedures for a BL/CE UE] For a BL/CE UE in a NTN FDD serving cell, when the UE receives a GNSS Measurement Command MAC CE in a PDSCH ending in DL subframe n - if the UE shall not provide HARQ-ACK information for the HARQ process associated with the transport block in the PDSCH carrying the GNSS Measurement Command MAC CE, - the UE shall assume the start of the measurement gap in subframe n+6; - otherwise, - the UE shall assume the start of the measurement gap in subframe k+2, where k is the first DL subframe after the end of HARQ-ACK transmission for the HARQ process associated with the transport block in the PDSCH. ================End of TP #1 for TS 36.213 ===================

###### RAN1#115, November 13th – November 17th, 2023, Chicago, USA

Agreements on “8.13.3 Disabling of HARQ feedback for IoT NTN”

Agreement

When multiple TBs are scheduled by a single DCI:

- For Option 1 + Option 3 DCI based overridden mechanism, when DCI indicates HARQ feedback enabled, then the NB-IoT UE always wait for an RTT+3ms (i.e., till subframe n+Kmac+3 in TS36.213 section 16.6) before monitoring NPDCCH.

Agreement

It is up to editor to select TP 2-2b or TP 2-3a in section 7 of R1-2312389 to be endorsed for TS36.213 clause 7.3.

Agreement

The TP 4-1c in section 8 of R1-2312460 is endorsed for TS36.213 clause 7.3.1.

Agreements on “8.13.4	Improved GNSS operations for IoT NTN”

Agreement

Modify X1 value of RAN1#114 agreement on start time of GNSS measurement gap as below and endorse the corresponding TP below:

For the aperiodic GNSS measurement gap triggered by eNB with MAC CE, the start time of the gap should be at n+ X1, where n is the end of MAC CE receiving subframe/slot when HARQ feedback for the MAC CE is disabled

- X1=12ms for NB-IoT

- X1=5ms for eMTC

Reason for change:

In the endorsed CR R1-2310771 Clause 16.10, the starting point of a measurement gap is counted from the starting point of last subframe carrying triggering MAC CE, which is one subframe early than the agreement in RAN1#114. To keep the same time budget for processing the triggering MAC CE, the starting point of measurement gap should be in subframe n+13 for NB-IoT.

Summary of change:

Change the start of the measurement gap in subframe n+13 if UE shall not provide HARQ-ACK information for the NPDSCH carrying the triggering MAC CE.

Consequence if not approved:

The RAN1 agreement for GNSS measurement is not captured correctly in specification.

========================= Start of TP #1 for TS 36.213 ========================= < Unchanged parts are omitted > 16.10 GNSS measurement gap related procedures For a NB-IoT UE in a NTN FDD serving cell, when the UE receives a GNSS Measurement Command MAC CE in a NPDSCH ending in DL subframe n, - if the UE shall not provide HARQ-ACK information for the HARQ process associated with the transport block in the NPDSCH carrying GNSS Measurement Command MAC CE, - the UE shall assume the start of the measurement gap in subframe n+13 - otherwise, - the UE shall assume the start of the measurement gap in subframe k+2, where k is the first DL subframe after the end of the transmission of the NPUSCH carrying ACK/NACK response for the HARQ process associated with the transport block in the NPDSCH. ============================= End of TP #1 for TS 36.213 =============================

Agreement

Delete the TBD length in the value range column of higher layer parameters downlinkHARQ-FeedbackDisabled-Bitmap and downlinkHARQ-FeedbackDisabled-Bitmap-NB for R18 IoT NTN as follows:

| Parameter name in the text | Description | Value range |
| --- | --- | --- |
| downlinkHARQ-FeedbackDisabled-Bitmap | Used to disable the DL HARQ feedback, sent in the uplink, per HARQ process ID | Bitmap |
| downlinkHARQ-FeedbackDisabled-Bitmap-NB | Used to disable the DL HARQ feedback, sent in the uplink, per HARQ process ID | Bitmap |

Agreement

Add new higher layer parameters ULTransmissionExtension and ULTransmissionExtension-NB for R18 IoT NTN as follows:

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parent IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | Required for initial access or IDLE/INACTIVE | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IoT_NTN_enh-Core | UL transmission after original GNSS validity duration expires for eMTC | 36.213 |  |  |  | ULTransmissionExtension | New | ULTransmissionExtension | “UL transmission after original GNSS validity duration expires with duration X” is enabled/disabled by network. | BOOLEAN |  | per UE | No | 36.331 |  |
| IoT_NTN_enh-Core | UL transmission after original GNSS validity duration expires for NB-IoT | 36.213 |  |  |  | ULTransmissionExtension-NB | New | ULTransmissionExtension-NB | “UL transmission after original GNSS validity duration expires with duration X” is enabled/disabled by network. | BOOLEAN |  | per UE | No | 36.331 |  |

Agreement

From RAN1 perspective, the start time of duration X is at the point where original GNSS validity duration expires

- when timeAlignmentTimer is infinity, the end of X is at the point where new timer ULTransmissionExtentionTimer expires and ULTransmissionExtentionTimer is reset with length equal to Y every time when a MAC CE (to be defined by RAN2) is received

  - Note 1: It is up to RAN2 to decide whether the MAC CE is the legacy TAC or a new TAC or a new MAC CE.

- Note 2: It is up to RAN2 to implement the above behaviour based on new timer, existing timer, or by extending GNSS validity.

- Send an LS to RAN2

R1-2312696	LS on improved GNSS operations in Rel-18 IoT NTN	Moderator (MediaTek Inc.)

Agreement

TP#2 in section 4.2.5 of R1-2312299 is endorsed for TS36.213 Clause 16.10 and Clause 18.

Agreement

Add new higher layer parameter ul-TransmissionExtensionValue for eMTC and NB-IoT for R18 IoT NTN as follows:

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parent IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | Required for initial access or IDLE/INACTIVE | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IoT_NTN_enh-Core | UL transmission after original GNSS validity duration expires for eMTC | 36.213 |  |  |  | ul-TransmissionExtensionValue | New | ul-TransmissionExtensionValue | Indicates the duration after original GNSS validity duration expires within which UL transmission is allowed. Value in number of sub-frames, value sf500 corresponds to 500 sub-frames, sf750 corresponds to 750 sub-frames and so on. | {sf500, sf750, sf1280, sf1920, sf2560, sf5120, sf10240, spare1} |  | per UE | No | 36.331 |  |
| IoT_NTN_enh-Core | UL transmission after original GNSS validity duration expires for NB-IoT | 36.213 |  |  |  | ul-TransmissionExtensionValue-NB | New | ul-TransmissionExtensionValue | Indicates the duration after original GNSS validity duration expires within which UL transmission is allowed. Value in number of sub-frames, value sf500 corresponds to 500 sub-frames, sf750 corresponds to 750 sub-frames and so on. | {sf500, sf750, sf1280, sf1920, sf2560, sf5120, sf10240, spare1} |  | per UE | No | 36.331 |  |

#### Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

###### RAN2#123-bis, October 9th – October 13th, 2023, Xiamen, China

Agreements on “7.6.2.1 HARQ Enhancements”

Agreements:

- For NB-IoT UEs configured with two HARQ processes and at least one of them is configured with HARQ feedback disabled, RAN2 does not change the operation on drx-InactivityTimer for single-TB scheduling case.

- For a HARQ process configured as HARQ feedback disabled by RRC and further reversed to HARQ feedback enabled by DCI, UE behaviour on DRX follows the case when HARQ feedback is disabled.

- For multiple UL TBs scheduling, it is up to network implementation to configure multiple TBs using HARQ processes in the same or different HARQ modes. Start time for UL HARQ RTT timer for mode A will not change

- For multiple TB scheduling with the same HARQ feedback enabled/disabled configuration (by RRC/DCI), HARQ RTT Timer for HARQ process with HARQ feedback enabled is calculated as legacy (can further check in offline 308)

- For multiple TB scheduling with mixed HARQ feedback enabled/disabled configuration (by RRC), if HARQ-ACK bundling is configured, HARQ RTT Timer for HARQ process with HARQ feedback enabled is calculated as legacy.

- HARQ mode configuration is not applicable for PUR in IoT NTN

- For NB-IoT UE configured with two HARQ processes and at least one of them is configured with HARQ mode B, RAN2 does no change to the operation on drx-InactivityTimer for single TB scheduling case.

- For eMTC over NTN with HARQ process configured with HARQ mode B, there is no need to change drx-InactivityTimer operation.

- For NB-IoT, for a HARQ process configured as HARQ feedback enabled by RRC and further reversed to HARQ feedback disabled by DCI, UE behaviour on DRX follows the case when HARQ feedback is disabled (e.g., not start the corresponding DL HARQ RTT Timer for this HARQ process, and for NB-IoT NTN with single HARQ processes, start drx-InactivityTimer in the subframe containing the last repetition of the PDSCH plus 12 subframes plus deltaPDCCH).

- For multiple TB scheduling with the same HARQ feedback enabled configuration at least by RRC, HARQ RTT Timer for HARQ process with HARQ feedback enabled is calculated as legacy.

- In Rel-18 IoT NTN, if a NB-IoT UE receives a PDCCH indicating the transmission for multiple DL TBs, UE stops drx-InactivityTimer as legacy, regardless of the enabling/disabling HARQ feedback configuration for each of the multiple scheduled TB.

- For UL multiple TB scheduling, UE only starts the UL HARQ RTT Timer for the HARQ process(es) with HARQ mode A.

- In Rel-18 IoT NTN, if a NB-IoT UE receives a PDCCH indicating the transmission for UL multiple TBs, UE stops drx-InactivityTimer as legacy, regardless of the HARQ mode configuration for each of the multiple scheduled TB.

- HARQ feedback enabled/disabled and HARQ mode configuration related to SPS are already perfectly captured by the NOTE in stage-2 running CR (no further spec changes are needed)

Agreements on “7.6.2.2 GNSS Operation Enhancements”

- A new RRC parameter is introduced in dedicated RRC signalling to enable/disable duration X.

- A new RRC parameter is introduced in dedicated RRC signalling to configure duration Y when timeAlignmentTimer is infinity.

- GNSS Duration Report MAC CE will not trigger SR; instead CBRA will be used.

- A reserved LCID will be used for GNSS measurement command MAC CE (in DL).

- GNSS measurement validity duration report MAC CE priority is in-between TAR MAC CE and BSR MAC CE.

- The following update in NOTE in Stage 2 running CR is agreed:

NOTE: The AS operations (e.g. RLM related timers, dataInactivityTimer, CHO execution, neighbour cell measurement, RACH, SR, and BSR) are suspended when UE is performing GNSS measurement during GNSS measurement gap and resumed when the GNSS measurement is finished

- The following update in NOTE in Stage 2 running CR is agreed ((FFS whether to suspend T317, T318 during measurement gap):

NOTE: The AS operations (e.g. RLM related timers, dataInactivityTimer, CHO execution, neighbour cell measurement, RACH, SR, and BSR) are suspended when UE is performing GNSS measurement during GNSS measurement gap

- For both network-triggered and UE-autonomous Measurement Gap Length Configuration: Use MAC CE (with 1 bit indication to differentiate the two cases) (FFS if a RRC configuration is needed for NW trigger case)

Agreements on “7.6.3 Mobility Enhancements”

- Upon absence of satellite IDs for intra-frequency, the UE assumes Rel-17 behavior for intra-frequency measurements, i.e., measurement according to UE implementation.

- Upon absence of satellite IDs for inter-frequency, the UE assumes Rel-17 behavior for inter-frequency measurements, i.e., measurement according to UE implementation.

- RAN2 understands that if SIBxx is present, then satellite IDs in either SIB3, SIB5 or both SIB3 and SIB5 should be present (up to NW implementation, no spec impact)

- Satellite assistance information is provided per frequency, and not associated with PCIs. The satellite IDs for intra-frequency measurements are in SIB3 as in the current running CR (no need to place them into SIB4).

- Introduce satellite ID for serving satellite (in SIB31) as well. RAN2 does not consider implicitly reusing serving satellite assistance information.

- t-ServiceStartNeigh is signalled per satellite

- Separate reference locations are introduced for earth-quasi fixed cells and earth-moving cells.

- Working assumption:

- Adopt the same decision as for NR NTN to discriminate whether a frequency is for TN or NTN

Agreements on “7.6.4 Enhancements to discontinuous coverage”

- Provide carrier frequency for the existing satellite list in SIB32 to facilitate cell selection and reduce service interruption after an NTN coverage gap (FFS if the information can be considered as valid after the validity of SI)

###### RAN2#124, November 13th – November 17th, 2023, Chicago, USA

Agreements on “7.6.2.1 HARQ Enhancements”

Agreements:

- For multiple TB scheduling with mixed HARQ feedback enabled/disabled configuration, if HARQ-ACK bundling is not configured, HARQ RTT Timer for HARQ process with HARQ feedback enabled is calculated based on the number of scheduled TBs with HARQ feedback enabled.

- For DL multiple TB scheduling for a NB-IoT UE, if both HARQ processes are with disabled HARQ feedback, UE starts drx-InactivityTimer in the subframe containing the last repetition of the PDSCH corresponding to the last scheduled TB plus 12 subframes plus deltaPDCCH.

- For UL multiple TB scheduling for a NB-IoT UE, if both HARQ processes are configured with HARQ mode B, UE starts drx-InactivityTimer in the subframe containing the last repetition of the PUSCH corresponding to the last scheduled TB plus 1 subframe plus deltaPDCCH.

- RAN2 confirms that for both single TB scheduling and multiple TB scheduling. for a HARQ process configured as HARQ feedback enabled by RRC and further reversed to HARQ feedback disabled by DCI, UE behaviour on DRX follows the case when HARQ feedback is disabled.

Agreements on “7.6.2.2 GNSS Operation Enhancements”

Agreements:

- Upon start of GNSS measurement, UE keeps T317 running if it is currently running.

- If T317 expires during GNSS measurement gap, the SIB31 reacquisition and also T318 are not started until after GNSS measurement completion.

- Upon start of GNSS measurement, T318 needs to be stopped if it is currently running. The T318 is restarted after GNSS measurement completion.

- The duration X is not used to extend the original GNSS validity duration but at least to perform uplink transmission

- If UL transmission extension is enabled, e.g., ul-TransmissionExtensionEnabled is set to TRUE:

- If no indication of network triggered GNSS measurement is received from lower layers and gnss-AutonomousEnabled is not configured, upon duration X expires, UE moves to idle mode.

- If no indication of network triggered GNSS measurement is received from lower layers and gnss-AutonomousEnabled is configured, upon duration X expires, UE keeps in RRC_CONNECTED and triggers autonomous GNSS measurement.

- For both NB-IoT and eMTC over NTN, either we use the remaining LCID value for GNSS validity duration report MAC CE or we repurpose one of the existing codepoint.

- The start time of duration X is at the point where original GNSS validity duration expires (The wording can be rephrased based on how to capture it).

- If an indication of network triggered GNSS measurement is received from lower layers within duration X, UE triggers GNSS measurement (no specification impact).

- For both NB-IoT and eMTC over NTN, use the remaining LCID value for GNSS validity duration report MAC CE.

- UE may use the outdated GNSS position within the duration X at least for mobility. The network can limit the number of times X is extended and/or the value of X (can check if we need some different behaviour to handle the CHO in Earth Moving Cell case)

- For the case when timeAlignmentTimer is infinity, a (legacy/new) MAC CE is introduced/used to reset ULTransmissionExtentionTimer with length equal to Y).

Agreements on “7.6.3 Mobility Enhancements”

Agreements:

- For NB-IoT NTN, it is up to UE implementation which frequencies to be measured/prioritized in RRC_CONNECTED

- Same as NR NTN, the network does not configure the location-based CHO and time-based CHO simultaneously for the same candidate cell.

- For both RRC_CONNECTED and RRC_IDLE, time/location based neighbour cell measurement triggering can be configured together with the existing RSRP based triggering. If configured jointly, the UE starts measure neighbour cell when either of the triggering condition is met.

- UE may acquire SIBxx during T318.

- Clarify in the spec that RLF is not triggered if T318 expires and SIB31 has been obtained.

- UE stops T318 when both SIB31 and SIBxx have been acquired. If the UE hasn’t finished acquiring SIBxx during T318, UE aborts the acquisition of SIBxx and resumes UL operations.

- UE discriminates whether a frequency is for TN or NTN in an implicit way,  i.e., if the SIBxx is present and the satellite ID is absent for a frequency in SIB5, the UE assumes TN frequency measurement. If the SIBxx is present and the satellite ID is present for a frequency in SIB5, but the same satellite ID is absent in the SIBxx, the UE is not required to measure this frequency.

- eMTC UEs in NTN should use MeasObject(s) for performing measurements based on time/location criteria.

- We introduce the possibility to broadcast neighbor NTN cell information (SIBXX) in a TN cell (We remove the restriction in 36.331 that SIBXX can only be sent in a NTN cell. No changes expected to other Stage 3 specs)

- For CHO in EMC a new event is introduced.

- New event comprises a reference location and distance threshold for source and target cell.

- Ephemeris and epochTime information for candidate CHO cell is also provided in RRC Reconfiguration (configuring the CHO) within the configuration prepared by the source cell (outside of the new event).

Agreements on “7.6.4 Enhancements to discontinuous coverage”

- Capture a NOTE in RRC Spec “UE may directly go to RRC_IDLE after RLF is triggered if there is not enough time for the UE to finish the procedure of RRC re-establishment due to the discontinuous coverage” (actual wording can be further discussed in the CR review). We leave it to UE implementation on how to evaluate whether there is “enough time” or not

- UE should apply early stop of T310 and go to RRC_IDLE state upon expiry of t-Service for the discontinuous coverage scenario.

- UE should apply early stop of T310 and go to RRC_IDLE, if it becomes out of the current serving cell coverage for the discontinuous coverage scenario.

- UE behavior on how to use the frequency information in SIB32 for cell selection in discontinuous coverage scenario should be captured in TS 36.304. (discuss exact wording as part of the 38.304 CR review)

- For discontinuous coverage, explicit RRC Release using a new RRC Release cause is not supported.

#### 2.2.2	Remaining Open issues

- All WI RAN2 objectives are completed, with some pending clarifications, which will be discussed as part of maintenance activity:

  - Update 36.300, 36.321 and 36.331 CRs considering the following aspects:

    - DRX inactivity timer starting during HARQ enabling/disabling in multi-TB scheduling (DL and UL).

    - Select new or existing MAC CE for time alignment timer (X and/or Y) handling in GNSS improvement.

## 2.3	RAN3

#### 2.3.1	Agreements

###### RAN3#121bis, October 9th – October 13th, 2023, Xiamen, China

###### Agreements on “18.2 Support discontinuous coverage”

- Miscellaneous correction on the BL CRs.

#### 2.3.2	Remaining Open issues

- All WI RAN3 objectives are completed, no critical outstanding issues. Any further clarifications will be discussed as part of maintenance activity (if needed).

## 2.4	RAN4

#### 2.4.1	Agreements:

###### RAN4#108-bis, October 9th – October 13th, 2023, Xiamen, China

Core Part - RRM

Sub-Topic 1: K_satellite and other general issue

Agreement:

- For Idle mode, each frequency carrier

  - Ksatellite =2, if there are at least two NGSO satellites (including UE serving satellite) to be monitored by UE in this frequency carrier

  - Ksatellite =1, if there is only one NGSO satellite to be monitored in this frequency carrier

- For Connected mode, each frequency carrier

  - Ksatellite =2, if there are at least two NGSO satellites (including UE serving satellite) to be monitored by UE in this frequency carrier

  - Ksatellite =1, if there is only one NGSO satellite to be monitored in this frequency carrier

Issue 1-1-2: K_satellite for NGSO and t-ServiceStart(Neigh)

Agreement:

- For Ksatellite, all neighbor cell of neighboring NGSO satellite are not considered to be monitored before t-serviceStart(Neigh) if t-serviceStart(Neigh) is configured.

Sub-Topic 2: IDLE mode measurements

Issue 2-2: Location-based triggering cell reselection measurements

Agreement:

- For IDLE mode neighbour cell measurements for eMTC and NB-IoT, the location-based measurement initiation condition for NR-NTN in R18 (in clause 4.2C.2.3 and 4.2C.2.4 in TS 38.133) can be reused with necessary modifications of parameters names

- Note:

  - Requirements apply for quasi-fixed cell and earth-moving cell

    - For defining the requirements for earth-moving cell, further discuss based on the agreement from R18 NR NTN.

  - margin for distanceThresh is discussed in other issue

Sub-Topic 3: CONN mode measurements

Issue 3-0 (new): For NGSO, neighbor NGSO satellite measurement in connected mode

Discuss the following options until next meeting:

- Option 1: For NB-IoT, best effort measurement for inter-satellite neighbor NGSO satellite for intra-frequency measurement, similar to inter-frequency.

- Option 2: other options are not precluded.

Issue 3-1: K_satellite, update NB-IoT inter-frequency measurement in connected mode

Agreement:

- NTN NB-IoT inter-frequency measurement in connected mode, update Tdetect_inter NB1-NC  and Tmeasure _inter NB1-NC as

  - Tdetect_inter_NB1-NC=iN*K_satelliteMin(5000, Ta,i) ms

  - Tmeasure_inter_NB1-NC=iM*K_satelliteMin(5000, Tb,i) ms

Issue 3-2-1: Time-based triggering Neighbour cell measurements in connected mode for NB-IoT - requirement

Agreement:

- For NB-IoT, time-based (t-Service) measurement initiation requirements apply to earth fixed cell.

  - RAN4 understand that the exact time for UE to start the measurement is up to UE implementation according to RAN2.

  - FFS whether to capture the exact time for UE to start the measurement in RAN4 requirements.

- For earth moving cell, UE shall initiate the measurement before losing its coverage and this needs to reflected in RAN4 requirements.

  - RAN4 understand that the exact time for UE to start the measurement is up to UE implementation according to RAN2.

  - FFS whether to capture the exact time for UE to start the measurement in RAN4 requirements.

Issue 3-3-1: Time-based triggering Neighbour cell measurements in connected mode for eMTC – applicability

Agreement:

- For eMTC, time-based (t-Service) measurement initiation requirements apply to earth fixed cell.

  - RAN4 understand that the exact time for UE to start the measurement is up to UE implementation according to RAN2.

  - FFS whether to capture the exact time for UE to start the measurement in RAN4 requirements.

- For earth moving cell, UE shall initiate the measurement before losing its coverage and this needs to reflected in RAN4 requirements.

  - RAN4 understand that the exact time for UE to start the measurement is up to UE implementation according to RAN2.

  - FFS whether to capture the exact time for UE to start the measurement in RAN4 requirements.

- Further discuss for time-based neighbor cell measurement, requirements apply for the eMTC UE provided the measurement gaps are configured.

Issue 3-3-2: Time-based triggering Neighbour cell measurements in connected mode for eMTC – Trigger

Agreement:

- Trigger is defined as Max (Tidentify_intra_UE cat M1, Tidentify_inter_UE cat M1)

Issue 3-4-2: Location-based triggering Neighbour cell measurements in connected mode – requirement

FFS:

- For CONNECTED mode location-based triggering of neighbour cell measurements for eMTC and NB-IoT, add the location-conditions to the existing list of conditions to trigger the neighbor cell measurements. But the delay requirements remain same

  - Note: for eMTC, it needs to consider measurement gap. How to capture it should be discussed in the CR phase.

Agreement:

- For NB-IoT, CONNECTED mode, UE shall perform measurement when location condition is met. And the existing delay requirements apply.

FFS eMTC, whether measurement gap should be provided.

Issue 3-5 (new): Time-based triggering Neighbour cell measurements in connected mode for eMTC – impact on measurement gap

Discuss until the next meeting:

- If t-ServiceStartNeigh is provided, measurement gap pattern(s) configured for neighbour cell measurements are suspended until time t-ServiceStartNeigh.

Sub-Topic 4: eMTC, CHO

Issue 4: For eMTC, CHO requirements

Agreement:

- Introduce CHO requirements for NTN eMTC with time and location-based trigger conditions. DCHO = TRRC + Tmeasure + TEvent_DU +Tinterrupt + TCHO_execution

Where:

  - TRRC is the RRC procedure delay

  - TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until the time/location condition fulfilled.

  - Tmeasure is the measurements time. Tmeasure=0 if only condEventD1 or condEventT1 is configured.

  - TCHO_execution is the conditional execution preparation time

  - Tinterrupt is the interruption time.

  - FFS Update Tinterrupt to include Tsearch, based on handover interruption requirement as in

    - 5.5A.2.1.2	Interruption time for CEMode A

    - 5.5A.3.1.2	Interruption time for CEMode A

Sub-Topic 5: GNSS re-acquisition gap in connected mode

Issue 5-1&5-2: GNSS gap, spec impact

Discuss the following options until next meeting.

- Option 1: add generic description that the measurement delay could be longer if GNSS fix happens during measurement period.

- Option 2: The measurement delay requirements are extended by the duration of the GNSS-MG.

  - When the UE triggers an early termination of the GNSS-MG, the measurement delay requirements are extended by the duration of the early-terminated GNSS-MG.

- Option 3: The measurement delay requirements are suspended until the termination of the GNSS-MG.

Issue 5-3: GNSS gap for eMTC, overlapping between MG and GNSS gap

Agreement:

- If gaps configured for reacquiring GNSS and gaps configured for mobility measurements at least partially overlaps in time with other, then UE shall suspend the gaps configured for mobility measurements and instead prioritize the use of GNSS gaps

  - Note: it considers as no overlapping between GNSS-MG and gaps configured for mobility measurements after the UE has performed early termination of GNSS-MG.

Sub-Topic 6: CR split

| UE Cat | Requirement | Clause in TS36.133 | Volunteer |
| --- | --- | --- | --- |
| NB-IoT | IDLE mode measurements | 4.6A | MTK |
|  | CONNECTED mode measurement on neighbour cells | (New) 8.14A.X1 | Nokia |
|  | GNSS re-acqusition impact related, e.g. RLM, ... | … | Huawei |
| Cat M1 | IDLE mode measurements | 4.7A | Huawei |
|  | CONNECTED mode measurement on neighbour cells | 8.13A | Nokia |
|  | conditional HO for Cat-M1 | 5.5A.2.3/4 (CEModeA) 5.5A.3.3/4 (CEModeB) | CMCC |
|  | GNSS re-acqusition impact related, e.g. RLM, ... | ... | Ericsson |

###### RAN4#109, November 13th – November 17th, 2023, Chicago, USA

Core Part - RRM

Sub-Topic 1-1: IDLE mode measurements

Issue 1-1-1: Skipping serving cell measurement before t-service

- No agreement

Sub-Topic 1-2: CONN mode neighbour cell measurements

Issue 1-2-1: For NB-IoT NGSO, intra-frequency inter-satellite neighbour cell measurement

Agreement:

- Define requirements for NGSO intra-frequency inter-satellite neighbour cell measurement as “inter-frequency” case

- Add condition that requirements apply provided that cell is available as indicated by t-ServiceStartNeigh if indicated.

Issue 1-2-2: For NB/eMTC, time-based triggering, the exact time for UE to start the measurement

- no consensus to capture the exact time for UE to start measurement in RAN4 spec

Issue 1-2-3: For eMTC, time-based triggering and MG perspectives

Agreement:

- For eMTC neighbour cell measurement, MG shall be configured as legacy TN case.

- RAN4 agree that the MG is suspended till the time t-serviceStartNeigh

- Detailed wording to be updated in CR

Issue 1-2-4: For eMTC, location-based triggering and MG perspectives

- no consensus to introduce new conditions on when the neighbour cell measurements can be performed without gaps.

Issue 1-2-5: For eMTC, suspend MG upon t-ServiceStarNeigh

- To resolve in CR

Issue 1-2-6: For NB/eMTC NGSO, Ksatellite in Re-establishment delay requirement

- no consensus to revise Ksatellite in Re-establishment delay requirement

Sub-Topic 1-3: eMTC, CHO

Issue 1-3-1: For eMTC, CHO requirements

Agreement:

- Update Tinterrupt to include Tsearch, based on handover interruption requirement with KSAT =1, and Nfreq=1

Sub-Topic 1-4: GNSS re-acquisition gap in connected mode

Issue 1-4-1: GNSS-MG spec impact

Agreement:

- Add generic description that the measurements are suspended when UE is performing GNSS measurement during GNSS measurement gap. Wording to be discussed directly in the CR.

- For NB-IoT, UE shall restart the cell measurement when the interval between two samples are larger than 5000 ms.

- For eMTC, the modification for the case that GNSS-MG is larger than the eDRX cycle can be further study in the maintenance phase.

Issue 1-4-2: For eMTC, GNSS-MG overlapping with MG

Agreement:

- When GNSS gap overlaps with MG, MG applies if GNSS-MG is terminated earlier than MG and after RA procedure.

Issue 1-4-3: RRC re-establishment when carrierFreqList  is provided

- FFS: If the carrierFreqList in SIB32 indicates that current and target cells belong to the same carrier, then Ksatellite,I can be set to 1. Otherwise, Ksatellite,I shall correspond to the number of NGSO satellites the UE shall monitor.

Sub-Topic 1-5: Others

Issue 1-5-1: Requirement terminologies

- To resolve in CR

Issue 1-5-2: Location-based triggering cell measurements – margin for distanceThresh

- Postpone the margin discussion in performance part.

Issue 1-5-3: Clarification on more than two NGSO satellites on a frequency layer

- No agreement

Performance part - RRM

Issue 2-2-1: For NB/eMTC, test cases suspended due to lack of neighbour cell assistant information

FFS

- Define test cases which are suspended due to lack of neighbour cell assistant information.

  - Introduce inter-frequency test cases

  - Introduce NGSO configuration for the existing intra-frequency test cases

Issue 2-2-2: For NB-IoT, neighbour cell measurement in CONNNECTED mode

FFS

- For NB-IoT, introduce test cases for neighbour cell measurement in CONNNECTED mode.

Issue 2-2-3: For NB/eMTC, test cases for time/location based triggering of cell reselection in IDLE mode

FFS

- RAN4 to define test cases for time/location based triggering of cell reselection in IDLE mode based on the following table:

| Requirements | NB/eMTC |
| --- | --- |
| IDLE: Time-based measurement triggering | NB/eMTC |
| IDLE: Location-based measurement triggering | NB/eMTC |

- RAN4 to further discuss whether and how to reduce the number of test cases

Issue 2-2-4: For NB/eMTC, test cases for time/location based triggering of neighbour cell measurements in CONNECTED mode

FFS RAN4 to discuss feasibility of defining tests for time- and location based triggering of neighbour cell measurements.

- Option 1: not to define the corresponding test cases.

- Option 2: RAN4 to define test cases for time- and location based triggering of neighbour cell measurements, based on the following table:

| Requirements | NB/eMTC |
| --- | --- |
| CONN: Location-based measurement triggering [NB/eMTC] | NB/eMTC |
| CONN: Time-based measurement triggering | eMTC |

Issue 2-2-5: For eMTC, test cases for CHO

FFS

- RAN4 to define test cases for eMTC CHO based on the following table:

| Requirements | NB/eMTC |
| --- | --- |
| CHO configured with condEventD1 | eMTC |
| CHO configured with condEventT1 | eMTC |

- RAN4 to further discuss whether and how to reduce the number of test cases

Performance part - Demod

Issue 1: Work plan

- Agreement:

UE Demodulation performance part (36.102) - November, 2023 (RAN4#109) - Approve the work plan. - Discuss UE demodulation requirements for IoT-NTN enhancement. - Discuss simulation assumptions. - February 2024 (RAN4#110) - Continue discussion on UE demodulation requirements for IoT-NTN enhancement. - Agree on simulation assumptions. - Discuss possible work split for the CR work, if needed. - April 2024 (RAN4#110bis) - Collect the simulation results. - Provide CR/Draft CR based on work split and discuss CRs/Draft CRs. - May 2024 (RAN4#111) - Update simulation assumptions if necessary. - Finalize CRs and close the performance part.

Issue 2-1: Whether to define PDSCH requirements with HARQ disabled?

- Agreement:

  - Continue the discussion in the next meeting.

Issue 3-1: Whether to define PUSCH requirements with HARQ disabled

- Agreement:

  - Do not define PUSCH requirements with HARQ disabled.

#### 2.4.2	Remaining Open issues:

Core Part

All WI RAN4 objectives are completed, no critical outstanding issues. Any further proposed clarifications will be discussed as part of maintenance activity.

Performance Part

Work on Performance requirements for RRM and Demod if agreed.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN1

RAN1#114-bis, October 9th – October 13th, 2023, Xiamen, China

Submitted TDocs to AI 8.13.3

- R1-2308911	Maintenance of disabling of HARQ feedback for IoT NTN	Huawei, HiSilicon

- R1-2309000	Remaining issues on disabling of HARQ feedback for IoT NTN	Spreadtrum Communications

- R1-2309172	Remaining issue on disabling of HARQ feedback	ZTE

- R1-2309280	Disabling of HARQ feedback for IoT NTN	NEC

- R1-2309600	Discussion on remaining issue on disabling of HARQ feedback for IoT NTN	OPPO

- R1-2309651	Maintenance on disabling of HARQ feedback for IoT NTN	Nokia, Nokia Shanghai Bell

- R1-2309794	Disabling of HARQ feedback for IoT NTN	Lenovo

- R1-2309852	On Higher Layer Signaling for HARQ Feedback Disabling for IoT NTN	Apple

- R1-2309888	Maintenance on disabling HARQ feedback for IoT NTN	Ericsson

- R1-2309979	Remaining issues on disabling of HARQ for IoT NTN	MediaTek Inc.

- R1-2310161	Disabling HARQ Feedback for IoT-NTN	Qualcomm Incorporated

- R1-2310356	FLS#1 on disabling of HARQ feedback for IoT NTN		Moderator (Lenovo)

- R1-2310615	FLS#2 on disabling of HARQ feedback for IoT NTN		Moderator (Lenovo)

Submitted TDocs to AI 8.13.4

- R1-2308912	Maintenance of improved GNSS operations for IoT NTN	Huawei, HiSilicon

- R1-2309001	Remaining issues on improved GNSS operations for IoT NTN	Spreadtrum Communications

- R1-2309152	Remaining issue on improved GNSS operation	ZTE

- R1-2309395	Remaining issues on improved GNSS operations for IoT NTN	Samsung

- R1-2309436	Discussion on the remaining issues for the improved GNSS operation for IoT NTN	xiaomi

- R1-2309506	Discussion on remaining issues of improved GNSS operations for IoT NTN	CATT

- R1-2309601	Discussion on remaining issue for improved GNSS operation for IoT NTN	OPPO

- R1-2309652	Maintenance on improved GNSS operations for IoT NTN	Nokia, Nokia Shanghai Bell

- R1-2309688	Remaining issues on improved GNSS operations for IoT NTN	CMCC

- R1-2309853	On improved GNSS operations for IoT NTN	Apple

- R1-2309980	Remaining issues on improved GNSS operations for IoT NTN	MediaTek Inc.

- R1-2310162	Improved GNSS Operations for IoT-NTN	Qualcomm Incorporated

- R1-2310188	Improved GNSS operations for IoT NTN	Nordic Semiconductor ASA

- R1-2310235	On maintenance of improved GNSS operations for IoT NTN	Ericsson Limited

- R1-2310297	Feature lead summary#1 of AI 8.13.4 on improved GNSS operations	Moderator (MediaTek)

- R1-2310298	Feature lead summary#2 of AI 8.13.4 on improved GNSS operations	Moderator (MediaTek)

- R1-2310299	Feature lead summary#3 of AI 8.13.4 on improved GNSS operations	Moderator (MediaTek)

RAN1#115, November 13th – November 17th, 2023, Chicago, USA

Submitted TDocs to AI 8.13.3

- R1-2310878	Maintenance of disabling of HARQ feedback for IoT NTN	Huawei, HiSilicon

- R1-2310965	Maintenance on disabling HARQ feedback for IoT NTN	Ericsson

- R1-2311180	Remaining issues on disabling of HARQ feedback for IoT NTN	Spreadtrum Communications

- R1-2311202	Remaining issue on disabling of HARQ feedback	ZTE

- R1-2311247	Discussion on remaining issue on disabling of HARQ feedback for IoT NTN	OPPO

- R1-2311654	Maintenance on disabling of HARQ feedback for IoT NTN	Nokia, Nokia Shanghai Bell

- R1-2311702	On Higher Layer Signaling for HARQ Feedback Disabling for IoT NTN	Apple

- R1-2311728	Disabling of HARQ feedback for IoT NTN	Lenovo

- R1-2311998	Remaining issues of disabling of HARQ feedback for IoT NTN	MediaTek Inc.

- R1-2312388	FLS#1 on disabling of HARQ feedback for IoT NTN	Moderator (Lenovo)

- R1-2312389	FLS#2 on disabling of HARQ feedback for IoT NTN	Moderator (Lenovo)

- R1-2312460	FLS#3 on disabling of HARQ feedback for IoT NTN	Moderator (Lenovo)

Submitted TDocs to AI 8.13.4

- R1-2310879	Maintenance of improved GNSS operations for IoT NTN	Huawei, HiSilicon

- R1-2311181	Remaining issues on improved GNSS operations for IoT NTN	Spreadtrum Communications

- R1-2311203	Remaining issue on improved GNSS operation	ZTE

- R1-2311248	Discussion on remaining issue for improved GNSS operation for IoT NTN	OPPO

- R1-2311512	Remaining issues on Improved GNSS Operations for IoT NTN	NEC

- R1-2311586	Discussion on the remaining issues for the improved GNSS operation for IoT NTN	Beijing Xiaomi Mobile Software

- R1-2311655	Maintenance on improved GNSS operations for IoT NTN	Nokia, Nokia Shanghai Bell

- R1-2311703	Remaining issues on improved GNSS operations for IoT NTN	Apple

- R1-2311863	Remaining issues for improved GNSS operations for IoT NTN	Samsung

- R1-2311943	On maintenance of improved GNSS operations for IoT NTN	Ericsson Inc.

- R1-2311999	Remaining issues on improved GNSS operations for IoT NTN	MediaTek Inc.

- R1-2312054	Improved GNSS Operations for IoT-NTN	Qualcomm Incorporated

- R1-2312128	Improved GNSS operations for IoT NTN	Nordic Semiconductor ASA

- R1-2312298	Feature lead summary#1 of AI 8.13.4 on improved GNSS operations	Moderator (MediaTek)

- R1-2312299	Feature lead summary#2 of AI 8.13.4 on improved GNSS operations	Moderator (MediaTek)

- R1-2312300	Feature lead summary#3 of AI 8.13.4 on improved GNSS operations	Moderator (MediaTek)

- R1-2312656	Draft LS on improved GNSS operations in Rel-18 IoT NTN	Moderator (MediaTek Inc.)

- R1-2312696	LS on improved GNSS operations in Rel-18 IoT NTN	Moderator (MediaTek Inc.)

## 4.2	RAN2

RAN2#123-bis, October 9th – October 13th, 2023, Xiamen, China

Submitted TDocs to AI 7.6.1

- R2-2309416	LS on Rel-18 RAN1 UE features list for LTE after RAN1#114 (R1-2308520; contact: NTT DOCOMO, AT&T)	RAN1

- R2-2309532	Discussion on R18 IoT NTN UE capabilities	OPPO

- R2-2310180	Running CR for TS 36.306 for Rel-18 IoT NTN	Qualcomm Incorporated

- R2-2310410	36331 running CR for IOT NTN	Huawei, HiSilicon

- R2-2311070	Stage-3 running CR for TS 36.321 for Rel-18 IoT-NTN	MediaTek Inc.

- R2-2311194	36304 Running CR for IoT-NTN	Nokia Solutions & Networks (I)

- R2-2311244	Introduction of IoT NTN enhancements	Ericsson

- R2-2311245	Rapporteur input on 36.300	Ericsson

Submitted TDocs to AI 7.6.2.1

- R2-2309527	Discussion on HARQ enhancement for IoT NTN	OPPO

- R2-2309657	Remaining Issues on HARQ Enhancement for IoT NTN	vivo

- R2-2309701	Remaining issues of HARQ enhancement	Huawei, Turkcell, HiSilicon

- R2-2309752	Discussion on HARQ enhancements in IoT NTN	CATT

- R2-2309758	Discussion on HARQ enhancement	Xiaomi

- R2-2309781	On Disabling HARQ Feedback in IoT-NTN	MediaTek Inc.

- R2-2309956	Views on some remaining issues for HARQ in IoT NTN	Lenovo

- R2-2310181	DCI-based HARQ feedback overriding solution	Qualcomm Incorporated

- R2-2310269	Discussion on the HARQ enhancement for IoT-NTN	CMCC

- R2-2310651	Further discussion on HARQ enhancements for IoT NTN	Nokia, Nokia Shanghai Bell

- R2-2310805	Disabling HARQ feedback for IoT-NTN	Interdigital, Inc.

- R2-2310820	Remaining issues of HARQ enhancements	ZTE Corporation, Sanechips

- R2-2311243	R18 IoT NTN HARQ enhancements	Ericsson

Submitted TDocs to AI 7.6.2.2

- R2-2309531  Discussion on GNSS operation for IoT NTN	OPPO

- R2-2309658	 Discussion on GNSS Operation for IoT NTN	vivo

- R2-2309702	 Discussion on the impact of GNSS measurement	Huawei, Turkcell, HiSilicon

- R2-2309751	 Discussion on GNSS operation enhancements	CATT

- R2-2309757	 Discussion on GNSS operation enhancement	Xiaomi

- R2-2309782	 Enhancements on GNSS operation	MediaTek Inc.

- R2-2309957	 Views on some remaining issues for GNSS operations in IoT NTN	Lenovo

- R2-2309997	 Remaining issues on the GNSS operation	Google Inc.

- R2-2310184	 GNSS fix in RRC_CONNECTED	Qualcomm Incorporated

- R2-2310270	 Discussion on GNSS enhancement for IoT-NTN	CMCC

- R2-2310323	 Concluding critical issues in improved GNSS operation	Apple

- R2-2310650	 GNSS Validity duration Reporting	Nordic Semiconductor ASA

- R2-2310652	 Further discussion on GNSS operation enhancement in Rel-18 IoT NTN	Nokia, Nokia Shanghai Bell

- R2-2310806	 GNSS acquisition and reporting for IoT NTN	Interdigital, Inc.

- R2-2310821	 Remaining issues of GNSS enhancements	ZTE Corporation, Sanechips

- R2-2311036	 On improved GNSS operation for IoT NTN	Samsung Electronics Polska

- R2-2311086	 Discussion of GNSS operation enhancements	SHARP Corporation

- R2-2311242	 R18 IoT NTN GNSS operation enhancements	Ericsson

Submitted TDocs to AI 7.6.3.1

- R2-2309528	 Discussion on mobility enhancement for IoT NTN	OPPO

- R2-2309762	 Discussion on UE behavior when serving cell t-service expires	Xiaomi

- R2-2309783	 Enhancements on neighbor cell measurement	MediaTek Inc.

- R2-2310183	 Measurement and Mobility enhancements	Qualcomm Incorporated

- R2-2310271	 Discussion on mobility enhancements for IoT-NTN	CMCC

- R2-2310324	 Neighbour cell measurements before RLF for eMTC-NTN	Apple, MediaTek Inc.

- R2-2310411	 Remaining issues on mobility enhancements	Huawei, HiSilicon, Turkcell

- R2-2310628	 On enhancements for neighbour cell measurements	Samsung Electronics Polska

- R2-2310807	 Fast RLF and re-establishment in the discontinuous coverage scenario	Interdigital, Inc.

- R2-2310822	 Remaining issues of mobility enhancements	ZTE Corporation, Sanechips

- R2-2311012	 On Remaining issues for IoT-NTN Mobility Enhancements	Nokia, Nokia Shanghai Bell

- R2-2311069	 Discussion on gaps for neighbour cell measurements in IoT NTN	Ericsson

- R2-2311240	 Discussion on triggering RA for RRC connection re-establishment in IoT NTN	Ericsson

Submitted TDocs to AI 7.6.3.2

- R2-2309659	 Discussion on CHO Enhancement for IoT NTN	vivo

- R2-2310192	 NB-IoT NTN Coarse UE location reporting	Inmarsat, Viasat, Sateliot, Novamint, ESA, Thales

- R2-2310325	 Mobility enhancement in IoT NTN	Apple

- R2-2310629	 On other mobility enhancements for IoT NTN	Samsung Electronics Polska

Submitted TDocs to AI 7.6.4

- R2-2309660  Discussion on Discontinuous Coverage	vivo

- R2-2309703  Remaining issues of discontinuous coverage	Huawei, Turkcell, HiSilicon

- R2-2309753  Discussion on discontinuous coverage enhancement	CATT

- R2-2309882  Discussion on TN coverage for discontinuous coverage	ASUSTeK

- R2-2309959  Views on some remaining issues for discontinuous coverage in IoT NTN	Lenovo

- R2-2310021  Discussion on enhancement to discontinuous coverage for IoT NTN	Transsion Holdings

- R2-2310071  Considerations on Supporting Discontinuous Coverage	NEC

- R2-2310182  RRC release procedure in discontinuous coverage	Qualcomm Incorporated

- R2-2310808  RRC Release in discontinuous coverage	Interdigital, Inc.

- R2-2310826  Remaining issues of discontinuous coverage	ZTE Corporation, Sanechips

- R2-2310919  Enhancements to discontinuous coverage 	Samsung Electronics Iberia SA

- R2-2311013  Discussion on discontinuous coverage Enhancements	Nokia, Nokia Shanghai Bell

- R2-2311232  Measurement information to assist cell search after a coverage gap	Ericsson

RAN2#124, November 13th – November 17th, 2023, Chicago, USA

Submitted TDocs to AI 7.6.1

- R2-2311716	LS on Rel-18 RAN1 UE features list for LTE after RAN1#114bis (R1-2310634; contact: NTT DOCOMO, AT&T)	RAN1

- R2-2311891	Introduction of IoT NTN enhancements	Huawei, HiSilicon

- R2-2311892	Report of [Post123bis][302][IoT-NTN Enh] 36.331 running CR (Huawei)	Huawei, HiSilicon

- R2-2312116	Stage-3 running CR for TS 36.321 for Rel-18 IoT-NTN	MediaTek Inc.

- R2-2312281	Introduction of Rel-18 IoT NTN UE capabilities	Qualcomm Incorporated

- R2-2312282	Rapporteur input to open issues on the introduction of Rel-18 IoT NTN UE capabilities	Qualcomm Incorporated

- R2-2313301	Introduction of IoT NTN enhancements	Ericsson

- R2-2313304	Stage 2 open issues	Ericsson

- R2-2313320	Introduction of IoT-NTN Enhancements	Nokia Solutions & Networks (I)

- R2-2313321	Report of  [Post123bis][304][IoT-NTN Enh] 36.304 running CR (Nokia)	Nokia Solutions & Networks (I)

Submitted TDocs to AI 7.6.2.1

- R2-2311838	Remaining Issues on HARQ Enhancement for IoT NTN	vivo

- R2-2311958	Discussion on HARQ enhancement for IoT NTN	OPPO

- R2-2312114	Remaining Issues on HARQ Enhancements in IoT-NTN	MediaTek Inc.

- R2-2312244	Remaining issues of HARQ enhancements	ZTE Corporation, Sanechips

- R2-2312283	Open issues on HARQ enhancements	Qualcomm Incorporated

- R2-2312700	Remaining issues on HARQ enhancements for IoT NTN	Nokia, Nokia Shanghai Bell

- R2-2312714	Remaining issues on HARQ enhancement	Huawei, Turkcell, HiSilicon

- R2-2312722	Discussion on HARQ enhancement open issues	Xiaomi

- R2-2313300	R18 IoT NTN HARQ enhancements	Ericsson

- R2-2313317	Discussion on HARQ enhancements in IoT NTN	CATT

Submitted TDocs to AI 7.6.2.2

- R2-2311839	Remaining Issues on GNSS Operation for IoT NTN	vivo

- R2-2311962	Discussion on GNSS operation for IoT NTN	OPPO

- R2-2311963	DRAFT LS on GNSS validity duration	OPPO

- R2-2312046	Leftover issues on the GNSS operation enhancements	Google Inc.

- R2-2312054	Discussion on GNSS operation enhancements	CATT

- R2-2312115	Remaining GNSS Enhancement Issues in IoT-NTN	MediaTek Inc.

- R2-2312246	Remaining issues of GNSS enhancements	ZTE Corporation, Sanechips

- R2-2312286	Open issues on GNSS fix in RRC_CONNECTED	Qualcomm Incorporated

- R2-2312353	Leftover issues in improved GNSS operation	Apple

- R2-2312458	Views on timer handling during GNSS measurement gap	Lenovo

- R2-2312608	GNSS operation enhancement	NEC

- R2-2312673	Discussion on GNSS enhancement for IoT-NTN	CMCC

- R2-2312701	Remaining issues on GNSS operation enhancement for IoT NTN	Nokia, Nokia Shanghai Bell

- R2-2312715	Remaining issues on GNSS measurement	Huawei, Turkcell, HiSilicon

- R2-2312721	Discussion on GNSS operation enhancement open issues	Xiaomi

- R2-2312879	GNSS acquisition and reporting for IoT NTN	Interdigital, Inc.

- R2-2313010	GNSS measurement procedures in connected mode	Samsung R&D Institute UK

- R2-2313299	R18 IoT NTN GNSS operation enhancements	Ericsson

Submitted TDocs to AI 7.6.3.1

- R2-2311959	Discussion on mobility enhancement for IoT NTN	OPPO

- R2-2312055	Discussion on leftover issues of mobility enhancements	CATT

- R2-2312247	Remaining issues of mobility enhancements	ZTE Corporation, Sanechips

- R2-2312285	Open issues on measurement and Mobility enhancements	Qualcomm Incorporated

- R2-2312355	Neighbour cell measurements before RLF for eMTC-NTN	Apple

- R2-2312764	Discussion on the remaining issues for the mobility enhancements	Xiaomi

- R2-2312860	Further analysis on open issues for IoT-NTN Mobility Enhancements	Nokia, Nokia Shanghai Bell

- R2-2312880	Fast RLF and re-establishment in the discontinuous coverage scenario	Interdigital, Inc.

- R2-2313011	Enhancements for neighbour cell measurements	Samsung R&D Institute UK

- R2-2313078	Remaining issues on mobility enhancements	Huawei, HiSilicon, Turkcell

- R2-2313228	Neighbour cell measurements in IoT NTN	Ericsson

- R2-2313229	Discussion on triggering RA for RRC connection re-establishment in IoT NTN	Ericsson

Submitted TDocs to AI 7.6.3.2

- R2-2311840	Discussion on CHO Enhancement for IoT NTN	vivo

- R2-2312354	Leftover issues for mobility enhancement in IoT NTN	Apple

- R2-2312459	Views on providing NB-IoT UE location information	Lenovo

- R2-2312878	CHO enhancement for earth-moving cells	Interdigital, Inc.

- R2-2313012	On other mobility enhancements for IoT NTN	Samsung R&D Institute UK

Submitted TDocs to AI 7.6.4

- R2-2311841	Discussion on Discontinuous Coverage	vivo

- R2-2312048	Leftover issues on the discontinuous coverage	Google Inc.

- R2-2312056	Discussion on open issues for discontinuous coverage	CATT

- R2-2312199	Considerations on Supporting Discontinuous Coverage	NEC

- R2-2312248	Paging window alignment in discontinuous coverage	ZTE Corporation, Sanechips

- R2-2312284	UE Autonomous release in discontinuous coverage	Qualcomm Incorporated

- R2-2312460	Views on some remaining issues for discontinuous coverage	Lenovo

- R2-2312631	Discussion on enhancement to discontinuous coverage for IoT NTN	Transsion Holdings

- R2-2312716	Remaining issues on discontinuous coverage	Huawei, Turkcell, HiSilicon

- R2-2312723	Discussion on Discontinuous coverage open issues	Xiaomi

- R2-2312861	Discussion on remaining issues discontinuous coverage Enhancements	Nokia, Nokia Shanghai Bell

- R2-2312881	RRC Release in discontinuous coverage	Interdigital, Inc.

- R2-2313296	Enhancements to Discontinuous Coverage	SHARP Corporation

- R2-2313397	Enhancements to discontinuous coverage	Samsung

## 4.3	RAN3

RAN3#121, Toulouse, France, 21st - 25th August 2023

- R3-234059, Further discussion on discontinuous coverage issue for IoT NTN, ZTE

- R3-234200, (TP for BL CR IoT NTN TS36.423) Correction for Time based CHO, CATT, Ericsson, Nokia, Nokia Shanghai Bell, ZTE, Huawei

- R3-234219, (TP to BL CRs, 36.300, 36.423) Alignment with NR NTN, Huawei, Ericsson

- R3-234017, (TP for BL CR 36.300 and 36.413) on support X2-CHO and S1-HO with time-based trigger condition, Nokia, Nokia Shanghai Bell

- R3-234105, Time-Based HO and IoT NTN - Stage 2 Impacts, Ericsson, CATT, ESA, Huawei

- R3-234106, Time-Based HO for IoT NTN - S1AP Impacts, Ericsson, Huawei, CATT, ESA

- R3-234107, Time Margin for CHO in IoT NTN - X2AP Impact, Ericsson, Inmarsat, ESA

## 4.4	RAN4

RAN4#108-bis, October 9th – October 13th, 2023, Xiamen, China

RRM

- R4-2317395 WF on R18 IoT NTN RRM requirements Moderator (MediaTek)

- R4-2317223 Topic summary for [108bis][231] IoT_NTN_enh Moderator (MediaTek inc.)

- R4-2315176 Discussion on RRM core requirements for IOT NTN enhancement	CMCC

- R4-2315253 Discussion on RRM requirements for IoT NTN enhancement MediaTek inc.

- R4-2315631 Discussion on RRM requirements for IoT NTN enhancement Huawei, HiSilicon

- R4-2316261 Discussion on mobility requirements for IoT NTN enhancements Nokia, Nokia Shanghai Bell

- R4-2316351 Discussions on RRM requirements for IoT NTN enhancements Ericsson

- R4-2315254 Introduction of location based cell re-selection requirement for  IoT NTN enhancement for UE category NB-IoT MediaTek inc.

- R4-2315632 Draft CR on RRM impact of GNSS re-acquisition for NB-IoT	 Huawei, HiSilicon

- R4-2316066 draftCR on IDLE mode requirements for eMTC over NTN Huawei, HiSilicon

- R4-2316262 DraftCR on NB-IoT  Neighbor Cell Measurements in RRC_Connected (36.133)	Nokia, Nokia Shanghai Bell

- R4-2316263 DraftCR on eMTC Connected Mode Measurements (36.133) Nokia, Nokia Shanghai Bell

RAN4#109, November 13th – November 17th, 2023, Chicago, USA

RRM

- R4-2321646 Big CR to TS 38.133 on RRM requirements for IoT NTN enhancements MediaTek

- R4-2321577 WF on R18 IoT NTN RRM requirements Moderator (MediaTek)

- R4-2321339 Ad-hoc minutes on IoT_NTN_enh WI MediaTek

- R4-2318075 Work Plan on RRM performance part for IoT NTN enhancements MediaTek

- R4-2318191 Topic summary for [109][235] IoT_NTN_enh Moderator (MediaTek)

- R4-2318073 Discussion on RRM core requirements for IoT NTN enhancement MediaTek inc.

- R4-2318074 CR on cell re-selection requirement for IoT NTN enhancement for UE category NB-IoT MediaTek inc.

- R4-2318912 Discussion on RRM core requirements for IOT NTN enhancement	CMCC

- R4-2318913 Draft CR to TS 36.133: Conditional HO for Cat-M1 for IOT-NTN	CMCC

- R4-2319352 Discussion on RRM requirements for IoT NTN enhancement	Huawei, HiSilicon

- R4-2319353 Draft CR on RRM impact of GNSS re-acquisition for NB-IoT	Huawei, HiSilicon

- R4-2320015 draftCR on IDLE mode requirements for eMTC over NTN	Huawei, HiSilicon

- R4-2320140 Discussions on RRM requirements for IoT NTN enhancements	Ericsson

- R4-2320141 IoT NTN RRM requirements during GNSS reacquisition	Ericsson

- R4-2320142 IoT NTN RRC re-establishment requirements during discontinuous coverage Ericsson

- R4-2320741 Discussion on open issues for IoT NTN enh	Nokia, Nokia Shanghai Bell

- R4-2320742 DraftCR to 36.133 on Connected Mode Mobility for IoT NTN	Nokia, Nokia Shanghai Bell

- R4-2320743 DraftCR to 36.133 on Connected Mode Mobility for Emtc NTN	Nokia, Nokia Shanghai Bell

- R4-2318075 Work Plan on RRM performance part for IoT NTN enhancements	MediaTek inc.

- R4-2318076 Discussion on RRM performance requirements for IoT NTN enhancement	MediaTek inc.

- R4-2319354 Discussion on performance requirements for IoT NTN enhancement	Huawei, HiSilicon

- R4-2320143 Discussions on RRM performance requirements for IoT NTN enhancements	Ericsson

- R4-2320744 Performance Considerations for IoT NTN enhancements	Nokia, Nokia Shanghai Bell

Demod

- R4-2321145 WF on [109][332] IoT_NTN_Demod Moderator (MediaTek)

- R4-2318224 Topic summary for [109][332] IoT_NTN_Demod Moderator (MediaTek)

- R4-2318232 Discussion on IoT NTN	Nokia, Nokia Shanghai Bell

- R4-2318666 Workplan on demodulation requirements for IoT-NTN enhancement	MediaTek inc.

- R4-2318667 Discussion on UE requirements for IoT-NTN enhancement	MediaTek inc.

- R4-2318734 Discussion on the performance requirements for IoT NTN enhancements	Qualcomm India Pvt Ltd

- R4-2319749 Discussion on demodulation requirements for IoT-NTN enhancements	Ericsson

- R4-2319847 View on SAN demodulation requirement for Rel-18 IoT over NTN	Samsung

- R4-2320229 Discussion on demodulation performance requirements for IoT NTN enhancement	Huawei,HiSilicon

# 5	Others

END