3GPP TSG RAN Meeting #102	RP-23xxxx

Edinburgh, Scotland, December 11-15, 2023	(revision of RP-223276)

- Source:	Samsung (Moderator)

- Title:	WID Update: MIMO Evolution for Downlink and Uplink

- Document for:	Approval

Agenda Item:	9.3.1.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: MIMO Evolution for Downlink and Uplink

## Acronym: NR_MIMO_evo_DL_UL

## Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release: Rel-18

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan. NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X |  |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| NR_eMIMO | RAN1 | 800085 | Enhancements on MIMO for NR |
| NR_FeMIMO | RAN1 | 860040 | Further enhancements on MIMO for NR |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should just include the feature WI data (In case the feature covers Core and 	Perf. part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

## 3	Justification

MIMO is one of the key technologies in NR systems and is successful in commercial deployment. In Rel-15/16/17, MIMO features were investigated and specified for both FDD and TDD systems, of which major parts were for downlink MIMO operation. In Rel-18, it is important to identify and specify necessary enhancements for uplink MIMO, while necessary enhancements on downlink MIMO that facilitate the use of large antenna array, not only for FR1 but also for FR2, would still need to be introduced to fulfil the request for evolution of NR deployments. This comprises the following areas of enhancement.

First, significant loss of performance for a UE at high/medium speed has been observed in commercial deployments especially in multi-user MIMO (MU-MIMO) scenarios. As the performance loss is partly caused by outdated CSI, enhancements on CSI acquisition to alleviate such loss can be beneficial. Second, the unified TCI framework was introduced in Rel-17 which facilitates streamlined multi-beam operation targeting FR2. As Rel-17 focuses on single-TRP use cases, extension of unified TCI framework that focuses on multi-TRP use cases is beneficial. Third, due to the increasing need for multiplexing capacity of downlink and uplink demodulation reference signal (DMRS) from various use cases, there is a need for increasing the number of orthogonal ports for DMRS. Fourth, features for facilitating multi-TRP deployments have been introduced in Rel-16/17 focusing on non-coherent joint transmission (NC-JT). As coherent joint transmission (CJT) improves coverage and average throughput in commercial deployments with high-performance backhaul and synchronization, enhancement on CSI acquisition for FDD and TDD, targeting FR1, can be beneficial in expanding the utility of multi-TRP deployments. Fifth, as advanced UEs (e.g. CPE, FWA, vehicle, industrial devices) become more relevant, introducing necessary enhancements to support for 8 antenna ports as well as 4 and more layers for UL transmission can offer the needed improvement for UL coverage and average throughput. Sixth, with the introduction of features for UL panel selection in Rel-17, advanced UEs (e.g. CPE, FWA, vehicle, industrial devices) can benefit from higher UL coverage and average throughput with simultaneous UL multi-panel transmission. Finally, some further enhancement to facilitate UL multi-TRP deployments via two timing advances (TAs) and enhanced UL power control can offer additional UL performance improvement.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The detailed objectives are as follows:

RAN1:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows:

  - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis

  - UE reporting of time-domain channel properties measured via CSI-RS for tracking

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM,

  - Striving for a common design between DL and UL DMRS

  - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

    - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

  - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

- Study, and if justified, specify the following

  - Two TAs for UL multi-DCI for multi-TRP operation

  - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

Check RAN1 workload situation at RAN#98 (Dec 2022).

RAN2:

Specify higher layer support of the enhancements listed above.

RAN4:

Specify necessary core requirements for the enhancements listed above.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Specify necessary performance requirements for the enhancements listed above.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.211 | NR; Physical channels and modulation | RAN#101 | Core Part |
| 38.212 | NR; Multiplexing and channel coding | RAN#101 | Core Part |
| 38.213 | NR; Physical layer procedures for control | RAN#101 | Core Part |
| 38.214 | NR; Physical layer procedures for data | RAN#101 | Core Part |
| 38.215 | NR; Physical layer measurements | RAN#101 | Core Part |
| 38.300 | NR; NR and NG-RAN Overall description; Stage-2 | RAN#102 | Core Part |
| 38.306 | NR; User Equipment (UE) radio access capabilities | RAN#102 | Core Part |
| 38.321 | NR; Medium Access Control (MAC) protocol specification | RAN#102 | Core Part |
| 38.331 | NR; Radio Resource Control (RRC); Protocol specification | RAN#102 | Core Part |
| 38.101-1 | NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone | RAN#102 | Core Part |
| 38.101-2 | NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone | RAN#102 | Core Part |
| 38.133 | NR; Requirements for support of radio resource management | RAN#102 | Core Part |
| 38.133 | NR; Requirements for support of radio resource management | RAN#104 | Performance Part |
| 38.101-4 | NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements | RAN#104 | Performance Part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

Primary: 	Eko Onggosanusi, Samsung, eko.o@samsung.com

Secondary:	Yubo Yang, Huawei, yangyubo1@huawei.com

Riki Okawa, NTT DOCOMO, riki.ookawa.rp@nttdocomo.com

## 7	Work item leadership

RAN1

Secondary responsible Working Group(s): RAN2, RAN4

## 8	Aspects that involve other WGs

{Specify all the other WG(s) to be involved and, if specific, their task. E.g.: "SA2, SA3, SA5. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet".}

NOTE:	For RAN WIs: Section 8 applies only to WGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

Samsung

AT&T

BJTU

CableLabs

CAICT

CATT

CEWiT

China Telecom

China Unicom

CMCC

Continental Automotive GmbH

DENSO CORPORATION

Deutsche Telekom

Ericsson

ETRI

Fraunhofer HHI

Fraunhofer IIS

Fujitsu

Futurewei

H3C

HiSilicon

Huawei

Intel

InterDigital Inc.

ITRI

KDDI

KT Corp.

Lenovo

LG Electronics

LG Uplus

MediaTek Inc.

Motorola Mobility

NEC

Nokia

Nokia Shanghai Bell

NTT DOCOMO

OPPO

Orange

Qualcomm

Sanechips

SHARP

SIA

SK Telecom

SoftBank

Sony

Spark

Spreadtrum

TCL Communication Ltd

Telecom Italia

Telia Company

Telstra

UESTC

Verizon

vivo

Xiaomi

Yonsei University

Vodafone

ZTE

Telefonica