3GPP TSG RAN Meeting #102		                         RP-23xxxx

Edinburgh, Scotland, December 11-15, 2023

Source: 			ZTE Corporation (rapporteur)

Title: 	WI summary: Mobile Terminated-Small Data Transmission (MT-SDT) for NR

Agenda item:	9.3.2.2

Document for: 	Information

# Introduction and justification for the work item

In Rel-17, the work item for small data enhancements (NR_SmallData_INACTIVE) enabled transmission of small signalling and/or data packets whilst the UE remains in RRC_INACTIVE state. These Rel-17 enhancements supported only the Mobile Originated Small Data Transmissions (MO-SDT). In Rel-18 the Mobile Terminated Small Data Transmissions are supported with the completion of the MT-SDT WI. For DL, MT-SDT (i.e. DL-triggered small data) allows similar benefits as MO-SDT i.e. 1) reducing signalling overhead and UE power consumption by avoiding unnecessary transitions to RRC_CONNECTED and reducing latency by allowing fast transmission of (small and infrequent) packets, e.g. for positioning.

# Summary of the SDT feature

For a UE in RRC_INACTIVE state, MT-SDT is initiated by the network with an indication to the UE in the paging message when DL data awaits transmission for radio bearers configured for SDT; based on the indication, the UE initiates the MT-SDT only if the DL RSRP is above a configured threshold. When MT-SDT is initiated by the UE, a resume cause indicating MT-SDT is included in the RRCResumeRequest/RRCResumeRequest1 message. It is possible for the network to enable MO-SDT or MT-SDT or both MO-SDT and MT-SDT in a cell. MT-SDT procedure can be initiated with either a transmission over RACH or over Type 1 Configured Grant (CG) resources (configured via dedicated signalling in RRCRelease). If MT-SDT procedure is initiated over RACH, only the RACH resources not configured for SDT can be used by the UE.

# References

| Tdoc | Spec | CR# | Title |
| --- | --- | --- | --- |
| R3-237876 | 37.483 | 0054 | Introduction of MT-SDT |
| R2-2313426 | 38.300 | 0711 | Introduction of MT-SDT in Stage-2 |
| R2-2312091 | 38.331 | 4194 | Introduction of MT-SDT |
| R3-237878 | 38.401 | 0284 | Introduction on MT-SDT |
| R3-237875 | 38.420 | 0034 | Introduction on MT-SDT |
| R3-237880 | 38.423 | 1010 | (CR to TS 38.423) Introduction of MT-SDT |
| R3-237874 | 38.473 | 1140 | Introduction on MT-SDT |
| R2-231xxxx | 38.321 | 1699 | Introduction of MT-SDT to MAC spec |