3GPP TSG RAN2#127 Meeting                                                              	            R2-24xxxxx

Maastricht, Netherlands, Aug 19th – 23th, 2024

Title:	LS on CSI-RS/SRS for spatial relation in RRC_INACTIVE

Response to:

Release:	Release 18

Work Item:	          NR_pos_enh2-Core

Source:	RAN2

To:	RAN1

Cc:

Contact Person:

#### Name:	Yu Pan

Tel. Number:

E-mail Address:	pan.yu24@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 has agreed to reuse the SP Positioning SRS Activation/Deactivation MAC CE (TS38.321-i20, section 6.1.3.36) for activating/deactivating SRS transmission in RRC_INACTIVE state for both, semi-persistent SRS for positioning without validity area (Rel-17) and semi-persistent SRS for positioning with validity area (Rel-18). RAN2 has also specified a new Aggregated SP Positioning SRS Activation/Deactivation MAC CE (TS38.321-i20, section 6.1.3.83) in Rel-18 for activating/deactivating aggregated semi-persistent SRS for positioning in both, RRC_CONNECTED and RRC_INACTIVE state.

Those two MAC CEs can both contain spatial relation field to indicate specific spatial relation for specific SRS resource (or specific aggregated SRS resource). Specifically, spatial relation for resource IDi with NZP-CSI-RS and SRS contains serving cell ID and BWP ID where the NZP-CSI-RS or the SRS is configured, see below:

TS38.321-i20 #### 6.1.3.36 SP Positioning SRS Activation/Deactivation MAC CE Figure 6.1.3.36-2: Spatial Relation for Resource IDi with NZP CSI-RS Figure 6.1.3.36-4: Spatial Relation for Resource IDi with SRS

In the current specification, a UE stores the RRC configurations in RRC_CONNECTED into the UE Inactive AS Context when transferring from RRC_CONNECTED to RRC_INACTIVE, and UE in RRC_INACTIVE state only works on single cell and single BWP. So it causes confusion that whether the NZP-CSI-RS or SSB configured in RRC_CONNECTED can be used as spatial relation RS in RRC_INACTIVE or not.

Therefore, RAN2 would like to invite RAN1 to answer the following questions:

Q1: When activating semi-persistent SRS for positioning in RRC_INACTIVE, whether the indication for spatial relation for resource Idi with SRS and NZP-CSI-RS can be based on the NZP-CSI-RS or SRS configuration which are configured in RRC_CONNECTED?

Q2: Whether the above answer is applicable for SP-SRS without validity area activation in RRC_INACTIVE, SP-SRS with validity area activation in RRC_INACTIVE and aggregated SP-SRS activation in RRC_INACTIVE?

2. Actions:

To RAN1

ACTION: RAN2 respectfully asks RAN1 to answer the above questions and provide feedback.

3. Date of Next TSG-WG2 Meetings:

TSG RAN WG2 Meeting #127-bis    	14th October– 18th October, 2024		Hefei, China

TSG RAN WG2 Meeting #128     	18th November– 22th November, 2024	           Orlando, USA