3GPP TSG-RAN WG2 Meeting #129		R2-2500xxxx

Athens, Greece, 17th – 21st Feb. 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1204 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE capability on ncd-SSB-BWP-Wor-r18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Qualcomm Incorporated, Guangdong Genius |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BWP_wor-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 38.306, the current field description for ncd-SSB-BWP-Wor-r18 only mentions the Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured). For PSCell case, the current wording may not be clear enough on whether it includes the case for CD-SSB-less PSCell, e.g. when the UE is configured only with the initial BWP which contains NCD-SSB, and there is no UE-specific BWP or other NCD-SSB; or when NCD-SSB is associated with both initial BWP and UE specific BWP, and UE’s active BWP is the initial BWP. In this way, the current specification may be mis-interpreted as ncd-SSB-BWP-Wor-r18 is not applicable for the case that PSCell’s initial BWP or the active BWP is associated with NCD-SSB. According to RAN4 specifcation, UE will not measure NCD-SSB unless UE supports ncd-SSB-BWP-Wor-r18 in this case. Thus, it is clear that ncd-SSB-BWP-Wor-r18 is also applicable for the case that PSCell’s initial BWP or the active BWP is associated with NCD-SSB. In order to avoid any mis-interpretation on the current field decription for ncd-SSB-BWP-Wor-r18, it is better to change the corresponding clarifcation for PSCell in the field description for ncd-SSB-BWP-Wor-r18 as: For PSCell (if configured), bandwidth of UE-specific RRC configured BWP needs not include bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB indicated by absoluteFrequencySSB (either CD-SSB or NCD-SSB). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the clarifcation in the field description for ncd-SSB-BWP-Wor-r18 as: For PSCell (if configured), bandwidth of UE-specific RRC configured BWP needs not include bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB indicated by absoluteFrequencySSB (either CD-SSB or NCD-SSB). Impact analysis Impacted 5G architecture options: MR-DC Impacted functionality ncd-SSB-BWP-Wor Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. - If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether ncd-SSB-BWP-Wor-r18 is applicable for the case that PSCell’s initial BWP or the active BWP is associated with NCD-SSB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

4.2.7.2	BandNR parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ------------------------------Omit the unrelated part------------------------------------- |  |  |  |  |
| ncd-SSB-BWP-Wor-r18 Indicates whether the UE supports RLM/BM/BFD and gapless L3 intra-frequency measurements based on NCD-SSB within active BWP. For Pcell, bandwidth of UE-specific RRC configured BWP needs not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB. For PSCell (if configured), bandwidth of UE-specific RRC configured BWP needs not include bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB indicated by absoluteFrequencySSB (either CD-SSB or NCD-SSB). NCD-SSB within the active DL BWP can be used as the QCL source for other reference signal. UE performs L3 intra-frequency measurements without gaps based on NCD-SSB, where the NCD-SSB is within the active DL BWP. NOTE: This feature applies only to PCell and PSCell (if configured). It is not applicable to RedCap or eRedCap UEs. | Band | No | N/A | N/A |
| ------------------------------Omit the unrelated part------------------------------------- |  |  |  |  |

End of change