3GPP TSG-RAN2 Meeting #128	R2-2411211

# Orlando, US, 18 – 22 Nov, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1903 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Capability on measurement gap enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2 |  |  |  |  |  | Date: |  |  | 2024-11-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1) In RAN4 feature list R4-2410748/R2-2406232, the LTE feature group “Simultaneous reception of EUTRAN data and NR SSB with different numerology” (FG x-z) was added. However, this has not been captured yet in the specification. 2) In the existing capability interRAT-NeedForInterruptionNR-r18, the wording “The UE shall only include this field if it indicates measurement gap is not required in the corresponding interRAT-NeedForGapsNR-r16 field” may lead to the wrong interpretation that UE shall always include the field if it indicates measurement gap is not required in the corresponding interRAT-NeedForGapsNR-r16 field, and based on RAN4 discussion the capability is optionally reported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Capture the LTE UE feature “Simultaneous reception of EUTRAN data and NR SSB with different numerology” in the specification. 2) Update the description of interRAT-NeedForInterruptionNR-r18 as “The UE includes this field only if it indicates measurement gap is not required in the corresponding interRAT-NeedForGapsNR-r16 field”. Impact analysis Impacted functionality: SSB-based inter-RAT measurements on NR Inter-operability: If the UE is implemented according to this CR but the network is not, or vice versa, the consequences if not approved remain. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1) The NW will always apply scheduling restriction, even if the UE supports simultaneous reception, which may result in performance degradation. 2) The capability interRAT-NeedForInterruptionNR-r18 remains ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.53, 4.3.6.xx (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.331 CR 5082 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

4.3.6	Measurement parameters

<unrelated parts omitted>

4.3.6.53	interRAT-NeedForInterruptionNR-r18

This field defines for each supported E-UTRA band or band combination whether interruption is required to perform SSB based inter-RAT measurements without gap on each supported NR band. Value no-gap-with-interruption indicates measurement gap is not needed but interruption is needed, and value no-gap-no-interruption indicates neither measurement gap nor interruption is needed. The UE includes this field only if it indicates measurement gap is not required in the corresponding interRAT-NeedForGapsNR-r16 field.

4.3.6.xx	simultaneousRxDataSSB-DiffNumerology-FR1-r18

This field defines whether the UE supports concurrent SSB-based inter-RAT measurement on NR FR1 cell and PDCCH or PDSCH reception from the serving cell with a different numerology. The UE includes this field only if it indicates support of interRAT-NeedForInterruptionNR-r18 for at least one target band in at least one band combination. This field applies only if interRAT-NeedForInterruptionNR-r18 is reported for the target band in the band combination.

End of Change