3GPP TSG-RAN2 Meeting #128	R2-24xxxxx

Orlando, U.S.A, 18th – 22nd November 2024

Title:	DRAFT LS on co-existence of SL-CA and SL PRS transmission/reception

Release:	Rel-18

Work Item:	NR_pos_enh2-Core

Source:	vivo (to be TSG RAN2)

To:	TSG RAN1

Cc:

Contact Person:

Name:	LI Yuan

E-mail Address:	yuanli@vivo.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		-

1. Overall Description:

RAN2 notices RAN1 has discussed on co-existence of SL-CA and SL PRS transmission/reception and come to the following agreements.

Working assumption@RAN1#116 meeting In NR Rel-18, in a band (pre)configured with SL CA, SL PRS transmission /reception can be supported: - In a shared SL PRS resource pool in a single SL carrier. - Tx power control follows the rule defined for SL CA in NR Rel-18 - In a dedicated SL PRS resource pool in a single SL carrier when the slots (pre)configured for the dedicated SL PRS resource pool do not collide with the slots (pre)configured for any other resource pool or S-SSB resource(s) in other carriers. - FFS: new UE capability(ies) are defined for this combination of features Note: whether this combination of features is supported in Rel-18 requires a conclusion on whether to introduce new UE capability(ies). No specification work until the FFS is resolved. Agreement@RAN1#116bis meeting For a band configured with SL CA, confirm the related working assumption from RAN1 #116 with the introduction of the following new UE capabilities: - One UE capability for SL PRS transmission for a band configured with SL CA - One UE capability for SL PRS reception for a band configured with SL CA - Note: there will not be two separate FG components for shared RP and dedicated RP

Regarding the restriction of SL-PRS transmission/reception in a dedicated SL-PRS resource pool in a band (pre-)configured with SL CA as highlighted above, RAN2 discussed potential specification impact but has not reached a consensus. The following understandings are potential directions to capture:

- Understanding 1: NW restriction to guarantee that Dedicated SL-PRS resource pool is not configured in the slots colliding with slots (pre-)configured for any other resource pool(s) or S-SSB resource(s) of other SL carriers.

- Understanding 2: UE behaviour to select a dedicated SL-PRS resource pool which is configured in the slot that does not collide with any slot (pre-)configured for any other resource pool(s) or S-SSB resource(s) in other SL carriers.

2. Actions:

To RAN1

ACTION: RAN2 kindly asks RAN1 to feedback on which understanding fits the original intention of RAN1 agreement.

3. Date of Next TSG-RAN2 Meetings:

TSG RAN2 Meeting #129		17 - 21 February 2024		Athens, Greece

TSG RAN2 Meeting #129bis	7 - 11 April 2024				Wuhan, China