3GPP TSG-RAN WG2 Meeting #129bis	R2-250xxxx

Wuhan, China, April 7 – 11, 2025

Agenda item:	8.5.1

Source: 	InterDigital (Rapporteur)

Title: 	[POST129][103][NES] 38.321 CR (InterDigital)

Document for:	Discussion and Decision

# Introduction

This document collects the comments received during the following email discussion on the MAC CR for the R19 NES work item:

- [POST129][103][NES] (IDC)

Scope: Capture all agreements in 38.321 running CR and identify stage 3 open issues.

Intended outcome: Endorsed 38.321 running CR in R2-2501463 (including editor’s notes for stage 3 open issues).

Deadline: Long email discussion

## Contact information:

| Company | Delegate Name | Email |
| --- | --- | --- |
| OPPO | Qianxi Lu | qianxi.lu@oppo.com |
| Xiaomi | Shukun Wang | Wangshukun3@xiaomi.com |
| vivo | Jianhui Li | jianhui.li@vivo.com |
| Ericsson | Helka-Liina Määttänen | Helka-liina.maattanen@ericsson.com |
| Fujitsu | Katsunari Uemura | u-katsunari@fujitsu.com |
| Qualcomm | Qing Li | qinli@qti.qualcomm.com |

# Discussion on TS 38.321 running CR

Companies can provide comments and suggestions to the uploaded running CR in this table. Please do not add changes, suggestions, or comments directly to the draft CR document.

| Company + Issue Number (e.g., ID001) | Issue | Comments and proposed changes |
| --- | --- | --- |
| OPPO001 | 1> if the Random Access Resources for SIB1 request have been explicitly provided by RRC: 2> if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available: 3> select an SSB with SS-RSRP above rsrp-ThresholdSSB. | [OPPO] Based on R1 conclusion, this one should be a separate para in the WUS configuration? If so, good to have some description in the field def part in 5.1.1 to diff [Rapp]: Added the SIB1 part to “rsrp- SIB1ThresholdSSB” in v1, in 5.1.1 and 5.1.2. |
| OPPO002 | 1> if ra-AssociationPeriodIndexSib1 and sib1-RequestPeriod are configured: 2> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB in the association period given by ra-AssociationPeriodIndexSib1 in the sib1-RequestPeriod permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] corresponding to the selected SSB). | [OPPO] Similar comment as above (especially for ra-ssb-OccasionMaskIndex which may be confusing considering the existing parameter with the same name) [Rapp]: Added the Sib1 part to “ra-ssb-OccasionMaskIndexSib1”in v1 to distinguish the configuration, similar to the other parameters. For other SI request, these parameters are not mentioned in 5.1.1, thought they are configured and used in 5.1.2. Same structure is followed for SIB1 request, however the parameter name is differentiated with the “Sib1” addition, as highlighted. My suggestion is to allow some time to check and we can add it indeed afterwards if needed. Just want to ensure the spec is compatible with the other SI syntax. [ER Helka-Liina] We can definitely update the RRC parameter names if it creates confusion. Could you point out in the RRC email discussion, e.g. as IDC response all your suggestions. The namings will likely change until ASN1 review which means final sync can be done after it. [Apple] We have some sympathy on the issue raised by Ericsson. We understand generally the RRC IE name used in 38.321 is changed after change of 38.331, esp. after ASN.1 review. Suggest to add EN in section 5.1.1 and 5.1.2 that the name of parameters may be updated in accordance with running RRC CR. [Rapp]: Added an editor’s note in v2 to mention OD-SIB1 configuration parameter names may be updated in accordance with RRC names. |
| OPPO003 | - ra-PreambleStartIndex: the starting index of Random Access Preamble(s) for on-demand SI request; - ra-SIB1PreambleStartIndex: the starting index of Random Access Preamble(s) for SIB1 request; | [OPPO] editorial: would it be better to align the wording by adding ‘on-demand’ to SIB1 request case as well? [Rapp]: Added in v1. |
| Xiaomi001 | All changes in section 5 due to on-demand SIB1. | For changes in section 5 due to on-demand SIB1, RAN2 did not reach a common understanding or consensus about the RACH configuration parameters for on-demand SIB1, it is too early to capture these changes in section 5. [Samsung]: RAN1 has already agreed these parameters. So do not see any reason to not capture. [Rapp]: these parameters are captured in the RRC running CR, and also come from R1 agreement (refer to R1-2501645) [Apple] Same view as Samsung and IDC. The parameters have been captured in L1 excel from RAN1 in R1-2501645. Not sure what is RAN2 concern. |
| Samsung 001 | 1> if the Random Access procedure was initiated for SIB1 request (as specified in TS 38.331 [5]); and 1> if ra-AssociationPeriodIndexSib1 and sib1-RequestPeriod are configured: 2> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB in the association period given by ra-AssociationPeriodIndexSib1 in the sib1-RequestPeriod permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] corresponding to the selected SSB). | ‘else’ is missing 1> else if the Random Access procedure was initiated for SIB1 request (as specified in TS 38.331 [5]); and [Rapp]: Added in v1. |
| Samsung 002 | Preamble backoff during SIB1 request procedure | It is not clear whether UE should apply preamble backoff during SIB1 request procedure. Since only Msg1 based SIB1 request is supported and RACH resources are dedicatedly configured there seems no need to perform backoff. Maybe we can just set PREAMBLE_BACKOFF to 0 during SIB1 request procedure. [Rapp]: I can add an editor’s note if needed. But I understand that if the NW doesn’t indicate any backoff in the RAR, the following text is applied and 0 backoff is applied: 1> else if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded: 2> if the Random Access Response contains a MAC subPDU with Backoff Indicator: 3> set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI. 2> else: 3> set the PREAMBLE_BACKOFF to 0 ms. [ER-Helka-Liina] Suggest to have edito’r note for the backoff so companies can check. We also think now it is not needed. [Apple] We agree with IDC that the current text can already work (i.e. rely on NW implementation). It seems no need to have a new text to fix backoff to 0 for OD-SIB1. But Ok to have an EN for company check. [Samsung]: In RAR MAC PDU design, same MAC PDU can include RARs for different random access preamble transmission from different UEs. So, we cannot assume that backoff subheader is not included in RAR MAC PDU including response for SIB1 request. It can be there but UE ignores this for SIB1 request as UE is using dedicated preamble. So we are ok to add editors note for now. [Rapp]: Added an editor’s note in v2 “FFS whether any changes are required for handling preamble backoff during SIB1 request procedure”. |
| vivo 001 |  | Editorial suggestion 1 it should be Rel-19 in the cover page. [Rapp]: Fixed in v1. Editorial suggestion 2 - ra-SIB1PreambleStartIndex: the starting index of Random Access Preamble(s) for SIB1 request; add ‘on-demand’ before SIB1 to align with OD-OSI description. [Rapp]: Added in v1. |
| Apple 001 | In Cover page, the following two items are empty and need to fill: - Clauses affected: - Affected other core spec: at least 38.331, 38.300 | [Rapp]: Fixed in v2. |
| FJ 001 | 1) Section 5.1.1: - ra-ResponseWindow: the time window to monitor RA response(s) (SpCell only); 2) Section 5.1.3: 2> else: 3> increment PREAMBLE_TRANSMISSION_COUNTER by 1; 3> if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1: 4> if the Random Access Preamble is transmitted on the SpCell: 5> indicate a Random Access problem to upper layers; 5> if this Random Access procedure was triggered for SI request or SIB1 request: 6> consider the Random Access procedure unsuccessfully completed. 3) section 5.1.4: 2> monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running. … 2> if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1: 3> if the Random Access Preamble is transmitted on the SpCell: 4> indicate a Random Access problem to upper layers; 4> if this Random Access procedure was triggered for SI request or SIB1 request: 5> consider the Random Access procedure unsuccessfully completed. | When the UE is camping on Cell A and request OD-SIB1 of an NES cell, the UE’s SpCell is Cell A but not the NES cell. In this case, UL WUS (preamble) is transmitted on the NES cell and Random access response is received on the NES cell, but not on the UE’s SpCell. It is suggested to change “SpCell” to “SpCell or a cell supporting OD-SIB1”. |
| FJ002 | RAN1 has agreed shared RO, i.e., The dedicated WUS resource shares the same PRACH resource pool with PRACH resource for other usages. When an NES UE camping on Cell A is requesting OD-SIB1 from an NES cell, the NES UE receives SIB1 request ACK in Msg2 transmitted by the NES cell. The agreed UL WUS configuration does not include preamble configuration for SI request in the NES cell, so the UE might not know the configured preambles of SI request in NES cell. In case of shared RO among WUS and other RA preambles, Msg2 transmitted by the NES cell may include SIB1 request ACK for the NES UE and SI request ACK and/or RAR for other UEs in the NES cell. If the SIB1 request ACK for the NES UE is after an SI request ACK (1-byte long) or RAR (1+7=8 byte long) for other UEs in Msg2 (Random access response MAC PDU), the NES UE might not know the start position of its SIB1 request ACK in the MAC PDU. | RAN2 is suggested to discuss this issue. |
| QC01 | ### 5.1.2 Random Access Resource selection … 1> else if the Random Access procedure was initiated for SIB1 request (as specified in TS 38.331 [5]); and 1> if the Random Access Resources for SIB1 request have been explicitly provided by RRC: 2> if at least one of the SSBs with SS-RSRP above rsrp- SIB1ThresholdSSB is available: 3> select an SSB with SS-RSRP above rsrp- SIB1ThresholdSSB. …. 1> else if the Random Access procedure was initiated for SIB1 request (as specified in TS 38.331 [5]); and 1> if ra-AssociationPeriodIndexSib1 and sib1-RequestPeriod are configured: 2> determine the next available PRACH occasion …. Comment: remove “else”, since OD-SI and OD-SIB1 are not mutually excluding with each other. |  |

# Conclusion

TBD

# Annex A: R2 agreements affecting TS 38.321

Change is introduced to support the following objectives:
-	On-demand SSB for SCell operation

-	On-demand SIB1 for idle/inactive UEs

-	Adaptation of common channels/signals

The following color code is used to categorize agreements interms of if/how they have been captured. It will be removed when the CR is finalized.

Fully implemented
Already specified in MAC
Not/partially implemented but additional agreements/FFSs needed before conclusion
Doesn’t impact MAC spec

RAN2 Agreements

On-demand SSB for SCell operation

RAN2#127

RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B

RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

R2-2407414: Proposal 3: RAN2 WG to discuss the issue of false measurement report triggering due to no SSB transmission when on-demand SSB is deactivated.

- RAN2 does not handle this issue in OD-SSB

RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

RAN2#127bis

No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

RAN2#129

RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

On-demand SIB1 for idle/inactive UEs

RAN2#125bis

At least RAN2 starts scenario 1a. Other scenarios are not excluded.

- Scenario 1a: Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell. with below potential RAN2 impacts:

  - Add WUS configuration in SIB of cell A.

  - Cell reselection from cell A to NES cell, including trigger condition and cell barring changes.

  - Whether allow camping, paging and SIB update in NES cell.

  - Cell reselection from NES cell to cell A or normal cell.

Contents of UL WUS

- RAN2 assumes that RACH procedure is reused for UE to request on-demand SIB1.

- UL WUS configuration includes at least below information:

  - RACH configuration

- A UE needs to know a UL WUS configuration to request SIB1 of which cell.

On-demand SIB1 acquisition procedure

-	Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS:T whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).

RAN2#126

Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism).

How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

RAN2 to wait for RAN1’s progress whether to support scenario 3.

RAN2#127

No consensus for RAN1 case 3 in RAN2.

We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

RAN2#127bis

We will inherit all agreements made during SI phase to WI phase.

A NES cell can include neighbouring NES cell’s WUS configuration.

NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

RAN2#128

Let’s wait for more RAN1 progress on Kssb discussion.

NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

Wait one cycle of RAN discussion (i.e. to see whether WID is updated or not to handle RRC connected UEs)

Working assumption: UL-WUS configuration in RRC release is not supported.

RAN2#128

There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

The UE follows the legacy validity principle of stored SIB.

For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

SIB-x can be cell specific or area specific, as legacy.

Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

Adaptation of common channels/signals

RAN2#125bis

From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

RAN2#127

Option-a) is about the bundling of PF for R19 NES UEs.

R2 observe that the option-a) and option-b) can be designed to configure the PO:s at same time position.

RAN2#127bis

Select option-b as baseline for R19 NES paging enhancement.

R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

Legacy UE is not barred.

Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

RAN2#128

Introduce value for Ns=8. FFS on 16.

Introduce value for N= T/32. FFS on T/64, T/128, T/256.

RAN2#129

For N, values smaller than T/32 are not supported.

The maximum possible value for Ns is 8.

Introduce a separate PEI configuration.

Paging adaptations are configured semi-statically and updated via system information update notification.

A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

RAN2 starts CBRA for RACH adaptation.

RAN2 starts 4-step RACH adaptation.

From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

From R2 perspective, RACH adaptation is not modelled as RA feature(s).

From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

Will follow legacy mechanism regarding how to select RACH resource.

# Annex B: R1 agreements affecting TS 38.321

On-demand SSB for SCell operation

RAN1#119

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

RAN1#118

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

On-demand SIB1

RAN1#119

At least the contents of RAR in response to SI request are included in RAR in response to UL WUS

There is no RAN1 consensus to support UL WUS repetition in R19.

The mapping rule between ra-PreambleStartIndex for OD-SIB1 and SSB follows the mapping rule between ra-PreambleStartIndex for OSI request and SSB as in legacy specification.

RAN1#118bis

No further optimization in RAN1 on power control and power ramp-up procedure for UL WUS in R19.

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration:

- rsrp-ThresholdSSB

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon

It is up to gNB to configure whether RACH occasions for UL WUS are shared or separated from the RACH occasions for other usages.

RAN1#117

For further study of on-demand SIB1 in idle/inactive mode, on the spatial relationships among PDCCH/PDSCH of on-demand SIB1, SSB, and UL WUS, as UL WUS is using dedicated PRACH resource, it is assumed that spatial relationships among PDCCH/PDSCH of on-demand SIB1, SSB and UL WUS can follow legacy mechanism.

Adaptation of common signals and channels

RAN1#120

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping

  - Note: the overlapped RO for legacy resource is not impacted

  - FFS: Clarification on configuration of legacy ROs

For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH.

- FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s)

- FFS: Details on how to reuse existing bit for the 1-bit indication

For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration

- FFS: Validity time duration for availability is configured by higher layer signaling or predefined

- FFS: Location of the reference point defined in the specification

- FFS: Value/granularity of the validity time duration.

- FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources

RAN1#119

At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH.

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16]

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

    - Alt 2-6: Network configured time period

RAN1#118bis

For adaptation of PRACH in time-domain, support both of the following

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources

- FFS: Additional details

RAN1#118

Extend the RAN1#117 agreement on SSB-RO mapping rule for additional PRACH resources to Case 1

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

RAN1#117 Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

For SSB-RO mapping rule for additional PRACH resources for Case 2.

- Extend the RAN1#117 and RAN1#118 agreements on SSB-RO mapping

RAN1#116bis

For adaptation of PRACH in time-domain, support at least the following:

- Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

  - Configuration of additional PRACH resources is provided by semi-static signalling

    - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs

  - FFS: adaptation mechanism for additional PRACH resources

  - Note: No change to the existing PRACH configuration tables in 38.211

Support adaptation mechanisms of PRACH in time-domain for following:

- UE in idle/inactive mode

- UE in connected mode