3GPP TSG-RAN2 Meeting #129	R2-25xxxxx

Athens, Greece, 17-21 February 2025

- Title:	Discussion on MCSt

- Source:	ZTE Corporation, Sanechips

- Agenda Item:	7.6

- Document for:	Discussion, Decision

# Introduction

In this paper, we discuss following email discussion:

- [POST129][119][SL] (ZTE)

Scope: Check the relation between resource (re)selection approach a/b and {MCSt + a resource pool with/without PSFCH}, prepare and discuss stage 2 CR.

Intended outcome: 38.300 CR in R2-2501488 to be agreed.

Deadline: Short email discussion

# Approach1&2

Below definitions for approach1&2 defined in R1-2304257 are for your information:

Approach 1: “best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (, remaining PDB, and ) - R16/17 behavior. - Step 2: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure - R16/17 behavior. - Step 3: Higher layer selects a set of resources either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior) to achieve MCSt. - Step 4: Repeat Step 1-3 for different TB if required. Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (, remaining PDB, and ) + “number of slots for MCSt” which could be derived based on CAPC of the logical channel/TB or other means. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior). - Step 4: Repeat Step 1-3 for different TB if required.

# Discussion

## Resource pool with PSFCH

According to the below RAN2#129 agreement, Rapporteur believe we do not need to re-discuss whether MCSt is supported on resource pool with PSFCH resource:

RAN2 123bis： 5. For a resource pool configured with PSFCH resource, UE can NOT select consecutive slots (i.e., MCSt) for transmissions of a single TB. RAN2 129: RAN2 keep the conclusion of allowing MCSt for multiple TBs in resource pool with PSFCH, and not pursue optimization for the case of PFSCH between two selected consecutive slots..

### Question1

- Question1: Regarding resource pool with PSFCH, do you agree to capture following statement in 38.300?

A UE using mode 2 resource allocation with single slot candidate resources and operating in a pool of resources with PSFCH configured supports resource selection for multiple TBs MCSt.

| Company | Agree or disagree | Comments |
| --- | --- | --- |
| ZTE | Agree |  |
| OPPO | Some rewording | A UE using mode 2 resource allocation with single slot candidate resourcesand operating in a pool of resources with PSFCH configured supports resource selection for multiple TBs MCSt with single slot candidate resources. Otherwise, readers may wonder what is the case for “A UE using mode 2 resource allocation with multiple slot candidate resources” |
| LG | Agree with OPPO’s rewording |  |

## Resource pool without PSFCH

For resource pool without PSFCH, following cases can be discussed. Case1a and Case1b are approach2 defined in R1-2304257. Case2b and case2c is two different implementation ways of approach1. RAN1 does not mention case2a, but rapporteur think current MAC spec cannot prevent case2a due to the random resource selection.

| Multipe slots sensing: Take N=3 as an example, N=3 is indicated to PHY layer | Case1a(Approach2) | single TB | UE use the multiple slots candidate resources by filling the same TB. |
| --- | --- | --- | --- |
|  | Case1b(Approach2) | multi-TBs | UE use the multiple slots candidate resources by filling the same TB. And based on MAC implementation, two consecutive “multi slots candidate resource” are selected for different TBs. |
| Single slots sensing: N = 1 is indicated to PHY layer or no value N is indicated to PHY layer. | Case2a | single TB | Based on MAC implementation, the single slot candidate resources for same TB are consecutive. |
|  | Case2b(Approach1) | multi-TBs | Based on MAC implementation, the single slot candidate resources for different TBs are consecutive. |
|  | Case2c(Approach1) | multi-TBs | Based on MAC implementation, the single slot candidate resources for different TBs and multiple TBs are consecutive. |

Table I: candidate cases for MCSt on pool without PSFCH

### Question2a

- Question2a: Regarding resource pool without PSFCH, do you agree below case1a is supported and following statement is correct:

A UE using mode 2 resource allocation with multiple slots candidate resources and operating in a pool of resources without PSFCH configured supports resource selection for single TBs MCSt.

| Company | Agree or disagree | Comments |
| --- | --- | --- |
| ZTE | Agree |  |
| OPPO | Agree yet wording can rely on Q3 decision |  |
| LG | Agree |  |

### Question2b

- Question2b: Regarding resource pool without PSFCH, do you agree below case1b is supported and following statement is correct:

A UE using mode 2 resource allocation with multiple slots candidate resources and operating in a pool of resources without PSFCH configured supports resource selection for multiple TBs MCSt.

| Company | Agree or disagree | Comments |
| --- | --- | --- |
| ZTE | Agree |  |
| OPPO | Agree yet wording can rely on Q3 decision |  |
| LG | Agree |  |

### Question2c

- Question2c: Regarding resource pool without PSFCH, do you agree below case2a is supported and following statement is correct:

A UE using mode 2 resource allocation with single slots candidate resources and operating in a pool of resources without PSFCH configured supports resource selection for single TBs MCSt.

| Company | Agree or disagree | Comments |
| --- | --- | --- |
| ZTE | Agree |  |
| OPPO |  | It seems neither approach-1 nor approach-2? Yet since MAC perform selection randomly, there is no need to prevent this case though for pool without PSFCH. Open to hear the view from others. |
| LG | Disagree | RAN2 has never agreed on the MCSt configuration of the same TB based on single slot candidate resources. |

### Question2d

- Question2d: Regarding resource pool without PSFCH, do you agree below case2b/c are supported and following statement is correct:

A UE using mode 2 resource allocation with single slots candidate resources and operating in a pool of resources without PSFCH configured supports resource selection for multiple TBs MCSt.

| Company | Agree or disagree | Comments |
| --- | --- | --- |
| ZTE | Agree |  |
| OPPO |  | Similar view as for Q2c. |
| LG | Disagree | Similar view as for Q2c |

### Question3

- Question3: if the case1a/b and case2a/b/c are supported, which one of below options is preferred to be adopted in 38.300?

Option1:

A UE using mode 2 resource allocation with multiple/single slot candidate resources and operating in a pool of resources without PSFCH configured supports resource selection for multiple/single TB multiple consecutive slot transmission (MCSt)

Option2:

A UE using mode 2 resource allocation operating in a pool of resources without PSFCH configured supports resource selection for multiple consecutive slot transmission (MCSt)

| Company | option | Comments |
| --- | --- | --- |
| ZTE | 1 or 2 |  |
| OPPO | 2 |  |
| LG | 2 |  |

# Conclusion