3GPP TS 38.321 V18.4.0 (2024-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

Medium Access Control (MAC) protocol specification

(Release 18)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Bluetooth® is a Trade Mark of the Bluetooth SIG registered for the benefit of its members

Contents

Foreword	9

1	Scope	10

2	References	10

3	Definitions, symbols and abbreviations	11

3.1	Definitions	11

3.2	Abbreviations	13

4	General	15

4.1	Introduction	15

4.2	MAC architecture	15

4.2.1	General	15

4.2.2	MAC Entities	15

4.3	Services	17

4.3.1	Services provided to upper layers	17

4.3.2	Services expected from physical layer	17

4.4	Functions	18

4.5	Channel structure	18

4.5.1	General	18

4.5.2	Transport Channels	18

4.5.3	Logical Channels	19

4.5.4	Mapping of Transport Channels to Logical Channels	19

4.5.4.1	General	19

4.5.4.2	Uplink mapping	19

4.5.4.3	Downlink mapping	19

4.5.4.4	Sidelink mapping	20

5	MAC procedures	20

5.1	Random Access procedure	20

5.1.1	Random Access procedure initialization	20

5.1.1a	Initialization of variables specific to Random Access type	25

5.1.1b	Selection of the set of Random Access resources for the Random Access procedure	28

5.1.1c	Availability of the set of Random Access resources	34

5.1.1d	Selection of the set of Random Access resources based on feature prioritization	35

5.1.1e	Selection of Msg1 repetition for SI request	35

5.1.2	Random Access Resource selection	35

5.1.2a	Random Access Resource selection for 2-step RA type	39

5.1.3	Random Access Preamble transmission	41

5.1.3a	MSGA transmission	42

5.1.4	Random Access Response reception	44

5.1.4a	MSGB reception and contention resolution for 2-step RA type	47

5.1.5	Contention Resolution	50

5.1.6	Completion of the Random Access procedure	53

5.2	Maintenance of Uplink Time Alignment	53

5.2a	Maintenance of UL Synchronization	57

5.3	DL-SCH data transfer	58

5.3.1	DL Assignment reception	58

5.3.2	HARQ operation	60

5.3.2.1	HARQ Entity	60

5.3.2.2	HARQ process	61

5.3.3	Disassembly and demultiplexing	62

5.4	UL-SCH data transfer	63

5.4.1	UL Grant reception	63

5.4.2	HARQ operation	68

5.4.2.1	HARQ Entity	68

5.4.2.2	HARQ process	72

5.4.3	Multiplexing and assembly	74

5.4.3.1	Logical Channel Prioritization	74

5.4.3.1.1	General	74

5.4.3.1.2	Selection of logical channels	75

5.4.3.1.3	Allocation of resources	75

5.4.3.2	Multiplexing of MAC Control Elements and MAC SDUs	77

5.4.4	Scheduling Request	77

5.4.5	Buffer Status Reporting	82

5.4.6	Power Headroom Reporting	86

5.4.7	Pre-emptive Buffer Status Reporting	93

5.4.8	Timing Advance Reporting	93

5.4.9	Delay status reporting	94

5.5	PCH reception	95

5.6	BCH reception	95

5.7	Discontinuous Reception (DRX)	95

5.7a	Discontinuous Reception (DRX) for MBS Broadcast	104

5.7b	Discontinuous Reception (DRX) for MBS Multicast	104

5.8	Transmission and reception without dynamic scheduling	108

5.8.1	Downlink	108

5.8.1a	Downlink for Multicast	109

5.8.2	Uplink	109

5.8.3	Sidelink	114

5.9	Activation/Deactivation of SCells	116

5.10	Activation/Deactivation of PDCP duplication	118

5.11	MAC reconfiguration	119

5.12	MAC Reset	119

5.12a	Void	121

5.13	Handling of unknown, unforeseen and erroneous protocol data	121

5.14	Handling of measurement gaps	121

5.15	Bandwidth Part (BWP) operation	122

5.15.1	Downlink and Uplink	122

5.15.2	Sidelink	126

5.16	SUL operation	128

5.17	Beam Failure Detection and Recovery procedure	128

5.18	Handling of MAC CEs	132

5.18.1	General	132

5.18.2	Activation/Deactivation of Semi-persistent CSI-RS/CSI-IM resource set	133

5.18.3	Aperiodic CSI Trigger State Subselection	133

5.18.4	Activation/Deactivation of UE-specific PDSCH TCI state	133

5.18.5	Indication of TCI state for UE-specific PDCCH	134

5.18.6	Activation/Deactivation of Semi-persistent CSI reporting on PUCCH	134

5.18.7	Activation/Deactivation of Semi-persistent SRS and Indication of spatial relation of SP/AP SRS	134

5.18.8	Activation/Deactivation of spatial relation of PUCCH resource	135

5.18.9	Activation/Deactivation of semi-persistent ZP CSI-RS resource set	135

5.18.10	Recommended Bit Rate	136

5.18.11	Void	136

5.18.12	Void	136

5.18.13	Void	136

5.18.14	Update of Pathloss Reference RS of SRS	136

5.18.15	Update of Pathloss Reference RS of PUSCH	136

5.18.16	Indication of spatial relation of SRS resource for a Serving Cell set	137

5.18.17	Activation/Deactivation of Semi-Persistent Positioning SRS	137

5.18.18	Timing offset adjustments for IAB	137

5.18.19	Guard symbols for IAB	137

5.18.20	Positioning Measurement Gap Activation/Deactivation Command	138

5.18.21	PPW Activation/Deactivation Command	139

5.18.22	Update of PUCCH Power Control Set for multiple TRP PUCCH repetition	139

5.18.23	Unified TCI States Activation/Deactivation MAC CE	139

5.18.24	Update of Differential Koffset	140

5.18.25	BFD-RS Indication MAC CE	140

5.18.26	Restricted and recommended beam indication for IAB	140

5.18.27	DL TX power adjustment for IAB	140

5.18.28	UL PSD range adjustment for IAB	141

5.18.29	Timing case indication for IAB	141

5.18.30	Case-6 Timing Request	142

5.18.31	Backhaul Link Beam Indication for NCR	142

5.18.32	Access Link Beam Indication for NCR	142

5.18.33	Enhanced Unified TCI States Activation/Deactivation MAC CE	142

5.18.34	Activation/deactivation of PSI-based SDU discard	143

5.18.35	LTM Cell Switch Command	143

5.18.36	Candidate Cell TCI States Activation/Deactivation	143

5.18.37	Activation/Deactivation of Aggregated Semi-Persistent Positioning SRS	144

5.19	Data inactivity monitoring	144

5.20	Void	144

5.21	LBT operation	144

5.21.1	General	144

5.21.2	LBT failure detection and recovery procedure	144

5.22	SL-SCH Data transfer and SL-PRS transmission	146

5.22.1	SL-SCH Data and SL-PRS transmission	146

5.22.1.1	SL Grant reception and SCI transmission	146

5.22.1.2	TX resource (re-)selection check	168

5.22.1.2a	Re-evaluation and Pre-emption	169

5.22.1.2b	Re-selection for using a received resource conflict indication	171

5.22.1.2c	Resource re-selection from SL LBT Failure indication	172

5.22.1.3	Sidelink HARQ operation and SL-PRS transmission	173

5.22.1.3.1	Sidelink HARQ Entity	173

5.22.1.3.1a	Sidelink process not associated with Dedicated SL-PRS resource pool	176

5.22.1.3.2	PSFCH reception	177

5.22.1.3.3	HARQ-based Sidelink RLF detection	178

5.22.1.3.4	Processing of sidelink grant on Dedicated SL-PRS resource pool	179

5.22.1.3.5	Sidelink process associated with Dedicated SL-PRS resource pool	180

5.22.1.4	Multiplexing and assembly	180

5.22.1.4.0	General	180

5.22.1.4.1	Logical channel prioritization	181

5.22.1.4.1.1	General	181

5.22.1.4.1.2	Selection of logical channels and SL-PRS	181

5.22.1.4.1.3	Allocation of sidelink resources	184

5.22.1.4.2	Multiplexing of MAC Control Elements and MAC SDUs	185

5.22.1.5	Scheduling Request	185

5.22.1.6	Buffer Status Reporting	187

5.22.1.7	CSI Reporting	189

5.22.1.8	Void	190

5.22.1.9	IUC-Request transmission	190

5.22.1.10	IUC-Information Reporting	190

5.22.1.10.1	General	190

5.22.1.10.2	Reception of IUC-Information Reporting	191

5.22.1.11	TX carrier (re-)selection	191

5.22.1.12	SL-PRS Resource Request	193

5.22.2	SL-SCH Data and SL-PRS reception	193

5.22.2.1	SCI reception	193

5.22.2.2	Sidelink HARQ operation and SL-PRS reception on Shared SL-PRS resource pool	194

5.22.2.2.1	Sidelink HARQ Entity	194

5.22.2.2.2	Sidelink process	195

5.22.2.3	Disassembly and demultiplexing	196

5.22.2.4	SL-PRS reception on Dedicated SL-PRS resource pool	197

5.23	SL-BCH data transfer	197

5.23.1	SL-BCH data transmission	197

5.23.2	SL-BCH data reception	197

5.24	Handling of PRS Processing Window	197

5.25	Positioning Measurement Gap Activation/Deactivation Request	198

5.26	Positioning SRS transmission in RRC_INACTIVE	198

5.26.1	General	198

5.26.2	TA validation for SRS transmission in RRC_INACTIVE	199

5.27	Small Data Transmission	199

5.27.1	General	199

5.27.2	TA Validation for CG-SDT	201

5.28	Sidelink Discontinuous Reception (DRX)	202

5.28.1	General	202

5.28.2	Behaviour of UE receiving SL-SCH Data	202

5.28.3	Behaviour of UE transmitting SL-SCH Data	206

5.29	Activation/Deactivation of SCG	207

5.30	Handling of FR2 UL gap	208

5.31	Sidelink LBT operation	208

5.31.1	General	208

5.31.2	Sidelink LBT failure detection and recovery procedure	208

5.32	SRS for positioning Tx frequency hopping	209

5.33	RACH-less initial UL transmission	210

5.34	Cell-Level Energy Saving	210

5.34.1	General	210

5.34.2	Cell Discontinuous Transmission	211

5.34.3	Cell Discontinuous Reception	212

6	Protocol Data Units, formats and parameters	213

6.1	Protocol Data Units	213

6.1.1	General	213

6.1.2	MAC PDU (DL-SCH and UL-SCH except transparent MAC and Random Access Response)	213

6.1.3	MAC Control Elements (CEs)	216

6.1.3.1	Buffer Status Report MAC CEs	216

6.1.3.2	C-RNTI MAC CE	225

6.1.3.3	UE Contention Resolution Identity MAC CE	225

6.1.3.4	Timing Advance Command MAC CE	225

6.1.3.4a	Absolute Timing Advance Command MAC CE	226

6.1.3.5	DRX Command MAC CE	226

6.1.3.6	Long DRX Command MAC CE	226

6.1.3.7	Configured Grant Confirmation MAC CE	226

6.1.3.8	Single Entry PHR MAC CE	226

6.1.3.9	Multiple Entry PHR MAC CE	228

6.1.3.10	SCell Activation/Deactivation MAC CEs	231

6.1.3.11	Duplication Activation/Deactivation MAC CE	232

6.1.3.12	SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE	232

6.1.3.13	Aperiodic CSI Trigger State Subselection MAC CE	233

6.1.3.14	TCI States Activation/Deactivation for UE-specific PDSCH MAC CE	234

6.1.3.15	TCI State Indication for UE-specific PDCCH MAC CE	235

6.1.3.16	SP CSI reporting on PUCCH Activation/Deactivation MAC CE	235

6.1.3.17	SP SRS Activation/Deactivation MAC CE	236

6.1.3.18	PUCCH spatial relation Activation/Deactivation MAC CE	237

6.1.3.19	SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE	238

6.1.3.20	Recommended bit rate MAC CE	238

6.1.3.21	Timing Delta MAC CE	239

6.1.3.22	Guard Symbols MAC CEs	240

6.1.3.23	BFR MAC CEs	241

6.1.3.24	Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE	242

6.1.3.25	Enhanced PUCCH Spatial Relation Activation/Deactivation MAC CE	243

6.1.3.26	Enhanced SP/AP SRS Spatial Relation Indication MAC CE	244

6.1.3.27	SRS Pathloss Reference RS Update MAC CE	245

6.1.3.28	PUSCH Pathloss Reference RS Update MAC CE	246

6.1.3.29	Serving Cell Set based SRS Spatial Relation Indication MAC CE	246

6.1.3.30	LBT failure MAC CEs	248

6.1.3.31	Multiple Entry Configured Grant Confirmation MAC CE	249

6.1.3.32	Duplication RLC Activation/Deactivation MAC CE	249

6.1.3.33	Sidelink Buffer Status Report MAC CEs	249

6.1.3.34	Sidelink Configured Grant Confirmation MAC CE	250

6.1.3.35	Sidelink CSI Reporting MAC CE	251

6.1.3.36	SP Positioning SRS Activation/Deactivation MAC CE	251

6.1.3.37	Guard Symbols MAC CEs for Case-6 and Case-7 timing modes	254

6.1.3.38	Case-7 Timing advance offset MAC CE	255

6.1.3.39	Case-6 Timing Request MAC CE	255

6.1.3.40	Positioning Measurement Gap Activation/Deactivation Request MAC CE	255

6.1.3.41	Positioning Measurement Gap Activation/Deactivation Command MAC CE	256

6.1.3.42	PPW Activation/Deactivation Command MAC CE	256

6.1.3.43	Enhanced BFR MAC CEs	257

6.1.3.44	Enhanced TCI States Indication for UE-specific PDCCH MAC CE	260

6.1.3.45	PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE	261

6.1.3.46	PUCCH Power Control Set Update for multiple TRP PUCCH repetition MAC CE	262

6.1.3.47	Unified TCI States Activation/Deactivation MAC CE	263

6.1.3.48	Enhanced Single Entry PHR MAC CE	264

6.1.3.49	Enhanced Multiple Entry PHR MAC CE	265

6.1.3.50	Enhanced Single Entry PHR for multiple TRP MAC CE	270

6.1.3.51	Enhanced Multiple Entry PHR for multiple TRP MAC CE	270

6.1.3.52	Sidelink DRX Command MAC CE	273

6.1.3.53	Sidelink Inter-UE Coordination Information MAC CE	273

6.1.3.54	Sidelink Inter-UE Coordination Request MAC CE	276

6.1.3.55	Enhanced SCell Activation/Deactivation MAC CEs	278

6.1.3.56	Timing Advance Report MAC CE	279

6.1.3.57	Differential Koffset MAC CE	279

6.1.3.58	BFD-RS Indication MAC CE	279

6.1.3.59	SP/AP SRS TCI State Indication MAC CE	280

6.1.3.60	Serving Cell Set based SRS TCI State Indication MAC CE	281

6.1.3.61	Child IAB-DU Restricted Beam Indication MAC CE	282

6.1.3.62	IAB-MT Recommended Beam Indication MAC CE	286

6.1.3.63	DL TX Power Adjustment and Desired DL TX Power Adjustment MAC CEs	289

6.1.3.64	Desired IAB-MT PSD range MAC CE	290

6.1.3.65	Timing Case Indication MAC CE	292

6.1.3.66	NCR Downlink Backhaul Link Beam Indication MAC CE	293

6.1.3.67	NCR Uplink Backhaul Link Beam Indication MAC CE	293

6.1.3.68	NCR Access Link Beam Indication MAC CE	294

6.1.3.69	SL LBT failure MAC CEs	294

6.1.3.70	Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States	294

6.1.3.71	Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States	295

6.1.3.72	Delay Status Report MAC CE	296

6.1.3.73	PSI-Based SDU Discard Activation/Deactivation MAC CE	297

6.1.3.74	SL-PRS Resource Request MAC CE	298

6.1.3.75	LTM Cell Switch Command MAC CE	298

6.1.3.76	Candidate Cell TCI States Activation/Deactivation MAC CE	300

6.1.3.77	Cross-RRH TCI State Indication for UE-specific PDCCH MAC CE	301

6.1.3.78	Single Entry PHR with assumed PUSCH MAC CE	302

6.1.3.79	Multiple Entry PHR with assumed PUSCH MAC CE	303

6.1.3.80	Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE	306

6.1.3.81	Enhanced Single Entry PHR for multiple TRP STx2P MAC CE	307

6.1.3.82	Enhanced Multiple Entry PHR for multiple TRP STx2P MAC CE	308

6.1.3.83	Aggregated SP Positioning SRS Activation/Deactivation MAC CE	311

6.1.4	MAC PDU (transparent MAC)	313

6.1.5	MAC PDU (Random Access Response)	313

6.1.5a	MAC PDU (MSGB)	314

6.1.6	MAC PDU (SL-SCH)	315

6.2	Formats and parameters	316

6.2.1	MAC subheader for DL-SCH and UL-SCH	316

6.2.2	MAC subheader for Random Access Response	321

6.2.2a	MAC subheader for MSGB	321

6.2.3	MAC payload for Random Access Response	322

6.2.3a	MAC payload for MSGB	323

6.2.4	MAC subheader for SL-SCH	325

7	Variables and constants	326

7.1	RNTI values	326

7.2	Backoff Parameter values	329

7.3	DELTA_PREAMBLE values	329

7.4	PRACH Mask Index values	330

Annex A (informative): Change history	331

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

A2X communication: A communication to support A2X services leveraging PC5 reference points, as defined in TS 23.256 [31]. A2X services are realized by various types of A2X applications, e.g., BRID or DAA.

Air to Ground network: An NG-RAN consisting of ground-based gNBs, which provide cell towers that send signals up to an aircraft's antenna(s) of onboard ATG terminal, with typical vertical altitude of around 10,000 m and take-off/landing altitudes down to 3000 m.

BWP for SRS for positioning Tx frequency hopping: For SRS for positioning Tx frequency hopping, separate BWP configuration outside BWP configuration for data transmission.

Dedicated SL-PRS resource pool: A sidelink resource pool which can be used for the transmission of SL-PRS and cannot be used for the transmission of PSSCH.

Dormant BWP: The dormant BWP is one of downlink BWPs configured by the network via dedicated RRC signaling. In the dormant BWP, the UE stop monitoring PDCCH on/for the SCell, but continues performing CSI measurements, Automatic Gain Control (AGC) and beam management, if configured.

DRX group: A group of Serving Cells that is configured by RRC and that have the same DRX Active Time.

eRedCap UE: A UE with enhanced reduced capabilities as specified in clause 4.2.22.1 of TS 38.306 [25].

HARQ information: HARQ information for DL-SCH, for UL-SCH, or for SL-SCH transmissions consists of New Data Indicator (NDI), Transport Block Size (TBS), Redundancy Version (RV), and HARQ process ID.

IAB-donor: gNB that provides network access to UEs via a network of backhaul and access links.

IAB-node: RAN node that supports NR access links to UEs and NR backhaul links to parent nodes and child nodes.

Listen Before Talk: A procedure according to which transmissions are not performed if the channel is identified as being occupied, see TS 37.213 [18].

LTM candidate cell: A candidate cell configured for LTM as defined in TS 38.331 [5].

Msg3: Message transmitted on UL-SCH containing a C-RNTI MAC CE or CCCH SDU, submitted from upper layer and associated with the UE Contention Resolution Identity, as part of a Random Access procedure.

Multi-path: Mode of operation of a UE in RRC_CONNECTED configured with one direct path on which the UE connects to gNB using NR Uu, and one indirect path on which the UE connects to the same gNB via another UE using PC5 unicast link or non-3GPP connection (N3C).

Multi-PUSCH configured grant: A configured grant configuration configured with nrOfSlotsInCG-Period (see TS 38.331 [5]). It includes multiple consecutive configured uplink grants within a single periodicity.

N3C indirect path: In Multi-path, the indirect path using Non-3GPP Connection between remote UE and relay UE.

NCR-Fwd: NCR-node function, which performs amplifying-and-forwarding of UL/DL RF signals between gNB and UE. The behavior of the NCR-Fwd is controlled according to the side control information received by the NCR-MT from a gNB.

NCR-MT: NCR-node entity which communicates with a gNB via a control link to receive side control information. The control link is based on NR Uu interface.

NCR-node: RAN node comprising NCR-MT and NCR-Fwd.

Non-terrestrial network: An NG-RAN consisting of gNBs, which provide non-terrestrial NR access to UEs by means of an NTN payload embarked on an airborne or space-borne NTN vehicle and an NTN Gateway.

NR backhaul link: NR link used for backhauling between an IAB-node and an IAB-donor, and between IAB-nodes in case of a multi-hop backhauling.

NR sidelink communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [19] and ProSe communication (including ProSe non-Relay, UE-to-Network Relay and UE-to-UE Relay communication (including ProSe UE-to-UE Relay communication with integrated discovery)) as defined in TS 23.304 [26], between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink discovery: AS functionality enabling ProSe non-Relay discovery, ProSe UE-to-Network Relay discovery and ProSe UE-to-UE Relay discovery for Proximity based Services as defined in TS 23.304 [26], between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink transmission: Any NR Sidelink-based transmission, including transmission for NR sidelink discovery, transmission for NR sidelink communication, transmission for Ranging/Sidelink Positioning, and transmission for A2X communication.

PDCCH occasion: A time duration (i.e. one or a consecutive number of symbols) during which the MAC entity is configured to monitor the PDCCH.

Positioning SRS Bandwidth Aggregation: Transmission of positioning SRS on multiple carriers in RRC_CONNECTED and RRC_INACTIVE where the positioning SRS resources are linked in RRC configuration as defined in TS 38.331 [5].

PRS Processing Window: A time window during which UE may perform PRS measurement inside the active DL BWP with the same numerology as the active DL BWP without measurement gap.

Ranging/Sidelink Positioning: AS functionality enabling ranging-based services and sidelink positioning as specified in TS 23.586 [30].

RB set: A RB set refers to a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum as defined in TS 37.213 [18].

RedCap UE: A UE with reduced capabilities as specified in clause 4.2.21.1 in TS 38.306 [25].

Serving Cell: A PCell, a PSCell, or an SCell in TS 38.331 [5].

Shared SL-PRS resource pool: A sidelink resource pool which can be used for the transmission of both SL-PRS and PSSCH.

Sidelink transmission information: Sidelink transmission information included in an SCI for an SL-SCH transmission or SL-PRS transmission with or without SL-SCH transmission on Shared SL-PRS resource pool as specified in clause 8.3 and 8.4 of TS 38.212 [9] consists of Sidelink HARQ information including NDI, RV, Sidelink process ID, HARQ feedback enabled/disabled indicator, Sidelink identification information including cast type indicator, Source Layer-1 ID and Destination Layer-1 ID, and Sidelink other information including CSI request, SL-PRS request, SL-PRS resource ID, a priority, a communication range requirement and Zone ID and COT sharing information.

SL-PRS delay budget: Delay budget before which the SL-PRS is expected to be transmitted by the Tx UE.

SL-PRS transmission information on Dedicated SL-PRS resource pool: SL-PRS transmission information on Dedicated SL-PRS resource pool is included in an SCI for an SL-PRS transmission on Dedicated SL-PRS resource pool, as specified in TS 38.212 [9], consisting of

-	SL-PRS identification information, including cast type indicator, source ID and destination ID;

-	SL-PRS transmission other information, including SL-PRS priority, SL-PRS request, SL-PRS resource ID and resource reservation period.

SRS for positioning Tx frequency hopping: Transmit frequency hopping of positioning SRS in RRC_INACTIVE and RRC_CONNECTED.

SRS positioning validity area: An area consisting of a list of cells within which the corresponding positioning SRS configuration is considered as valid.

Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG depending on if the MAC entity is associated to the MCG or the SCG, respectively. Otherwise the term Special Cell refers to the PCell. A Special Cell supports PUCCH transmission and contention-based Random Access, and is always activated.

Timing Advance Group: A group of Serving Cells that is configured by RRC and that, for the cells with a UL configured, using the same timing reference cell and the same Timing Advance value. A Timing Advance Group containing the SpCell of a MAC entity is referred to as Primary Timing Advance Group (PTAG), whereas the term Secondary Timing Advance Group (STAG) refers to other TAGs.

UE-gNB RTT: For non-terrestrial networks, the sum of the UE's Timing Advance value (see TS 38.211 [8] clause 4.3.1) and kmac.

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [20], between nearby UEs, using E-UTRA technology but not traversing any network node.

NOTE 1:	A timer is running once it is started, until it is stopped or until it expires; otherwise it is not running. A timer can be started if it is not running or restarted if it is running. A Timer is always started or restarted from its initial value. The duration of a timer is not updated until it is stopped or expires (e.g. due to BWP switching). When the MAC entity applies zero value for a timer, the timer shall be started and immediately expire unless explicitly stated otherwise.

NOTE 2:	In this version of the specification, the SRS in the procedural description includes Positioning SRS except for the Positioning SRS for transmission in RRC_INACTIVE as in clause 5.26. Positioning SRS except for the Positioning SRS for transmission in RRC_INACTIVE is treated the same as SRS by the UE unless explicitly stated otherwise.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A2X	Aircraft-to-Everything

AP	Aperiodic

BFR	Beam Failure Recovery

BRID	Broadcast Remote Identification

BSR	Buffer Status Report

BWP	Bandwidth Part

CE	Control Element

CG	Cell Group

CG-SDT	Configured Grant-based SDT

CI-RNTI	Cancellation Indication RNTI

CSI	Channel State Information

CSI-IM	CSI Interference Measurement

CSI-RS	CSI Reference Signal

CS-RNTI	Configured Scheduling RNTI

DAA	Detect And Avoid

DAPS	Dual Active Protocol Stack

DCP	DCI with CRC scrambled by PS-RNTI

DL-PRS	DownLink-Positioning Reference Signal

DSR	Delay Status Report

DTX	Discontinuous Transmission

G-CS-RNTI	Group Configured Scheduling RNTI

G-RNTI	Group RNTI

IAB	Integrated Access and Backhaul

INT-RNTI	Interruption RNTI

LBT	Listen Before Talk

LCG	Logical Channel Group

LCP	Logical Channel Prioritization

LTM	L1/L2 Triggered Mobility

MBS	Multicast/Broadcast Services

MCCH	MBS Control Channel

MCCH-RNTI	MBS Control Channel RNTI

MCG	Master Cell Group

MO-SDT	Mobile Originated SDT

MPE	Maximum Permissible Exposure

MTCH	MBS Traffic Channel

MT-SDT	Mobile Terminated SDT

N3C	Non-3GPP Connection

NCD-SSB	Non Cell Defining SSB

NCR	Network-Controlled Repeater

NSAG	Network Slice AS Group

NUL	Normal Uplink

NZP CSI-RS	Non-Zero Power CSI-RS

PDB	Packet Delay Budget

PEI-RNTI	Paging Early Indication RNTI

PHR	Power Headroom Report

PQI	PC5 QoS Identifier

PS-RNTI	Power Saving RNTI

PSI	PDU Set Importance

PTAG	Primary Timing Advance Group

PTM	Point to Multipoint

PTP	Point to Point

QCL	Quasi-colocation

PPW	PRS Processing Window

PRS	Positioning Reference Signal

RA-SDT	Random Access-based SDT

RRH	Remote Radio Head

RS	Reference Signal

SCG	Secondary Cell Group

SDT	Small Data Transmission

SFI-RNTI	Slot Format Indication RNTI

SI	System Information

SL-PRS-CS-RNTI	SL-PRS-Configured Scheduling-RNTI

SL-PRS-RNTI	SL-PRS-RNTI

SL-CS-RNTI	Sidelink-Configured Scheduling-RNTI

SL-PRS	Sidelink-PRS

SL-RNTI	Sidelink-RNTI

SpCell	Special Cell

SP	Semi-Persistent

SP-CSI-RNTI	Semi-Persistent CSI RNTI

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRI	SRS Resource Indicator

SS	Synchronization Signals

SSB	Synchronization Signal Block

STAG	Secondary Timing Advance Group

STx2P	Simultaneous Transmission with 2 Panels

SUL	Supplementary Uplink

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TPC-SRS-RNTI	Transmit Power Control-Sounding Reference Signal-RNTI

TRIV	Time Resource Indicator Value

TRP	Transmit/Receive Point

TRS	CSI-RS for tracking

U2N	UE-to-Network

U2U	UE-to-UE

UCI	Uplink Control Information

UTO-UCI	Unused Transmission Occasion - UCI

UTW	Uplink Time Window

V2X	Vehicle-to-Everything

ZP CSI-RS	Zero Power CSI-RS

5	MAC procedures

5.4	UL-SCH data transfer

5.4.9	Delay status reporting

The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time for a certain DSR reporting threshold, which is the smallest remaining value of the running PDCP discardTimers among PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU as specified in clause 7.3 in TS 38.323 [4], and the total amount of UL data for a certain DSR reporting threshold for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively.

RRC controls the DSR procedure by configuring the following parameter:

-	remainingTimeThreshold (per LCG): the DSR triggering threshold on remaining time for triggering a DSR for a logical channel within an LCG. Also serves as the DSR reporting threshold on remaining time for the content of DSR when the field dsr-ReportingThresList is not configured.

-	dsr-ReportingThresList (per LCG): the DSR reporting threshold for determining the content of DSR.

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG:

1>	if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and

1>	if there is no DSR pending for the logical channel:

2>	trigger a DSR for the logical channel.

If there is at least one DSR pending, the MAC entity shall:

1>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization:

2>	instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE as specified in clause 6.1.3.72.

1>	else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR:

2>	trigger a Scheduling Request.

NOTE 1:	The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same critieria specified in clause 5.4.5.

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR.

A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs.

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

NOTE 2:	It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader.