3GPP TSG-RAN WG2 Meeting #129bis      											                     R2-250xxxx

Wuhan, China, April 7th – 11th, 2025

Agenda item:						8.9.3

Source:								MediaTek Inc.

Title:		Report of [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4 (Mediatek)

Document for:	 	Discussion and decision

# Introduction

This is the report of the following offline discussion from RAN2#129:

- [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4 (Mediatek)

Scope: discuss CB-msg3 resource configuration parameters and CB-msg4 monitoring window and RNTI design

Intended outcome: summary of the email discussion

- Deadline: long

The deadline for long email discussion is March 21st 10:00 UTC.

# Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| MediaTek (Rapp) | Felix Tsai | chun-fan.tsai@mediatek.com |
| Nokia | Ping Yuan | Ping.1.yuan@nokia-sbell.com |

# Discussion

## CB-Msg3 resource configuration parameters

Regarding the CB-Msg3 resource configuration, the following agreements have been reached during the RAN2#129 meeting.

RAN2 #129 Agreements:

1.	RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects)

-	Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN)

-	Frequency domain resources for (N)PUSCH occasions

-	repetition number

-	(N)PDCCH resource

-	MCS

2.	For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels.

3.	The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT.

4.	The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels.

5.	CB EDT Config has three RSRP thresholds for eMTC for the four CE levels.

6.	As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config.

7.	RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature

Working assumption:

- For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today

TP for CB-Msg3 resource parameters for both eMTC and NB-IoT are provided in next two sessions. The TP is based on current agreements and mainly reuse from PRACH and PUR configuration parameters. In the TP, the IEs that are introduced based on RAN2 agreement or work assumption are marked with green; the IEs copied from PUR configuration are marked with yellow; the IEs copied from PRACH configuration are marked with cyan. The current PRACH and PUR parameters are also copied for reference.

### eMTC

#### –	PUR-Config

The IE PUR-Config is used to specify the PUR configuration.

PUR-Config information element

-- ASN1START

PUR-Config-r16 ::=		SEQUENCE {

pur-ConfigID-r16				PUR-ConfigID-r16			OPTIONAL,	-- Need OR

pur-ImplicitReleaseAfter-r16	ENUMERATED {n2, n4, n8, spare}	OPTIONAL,	-- Need OR

pur-StartTimeParameters-r16		SEQUENCE {

periodicityAndOffset-r16		PUR-PeriodicityAndOffset-r16,

startSFN-r16					INTEGER (0..1023),

startSubFrame-r16				INTEGER (0..9),

hsfn-LSB-Info-r16				BIT STRING (SIZE(1))

}		OPTIONAL,	--Need ON

pur-NumOccasions-r16			ENUMERATED {one, infinite},

pur-RNTI-r16					C-RNTI						OPTIONAL,	-- Need ON

pur-TimeAlignmentTimer-r16		INTEGER (1..8)				OPTIONAL,	-- Need OR

pur-RSRP-ChangeThreshold-r16	SetupRelease {PUR-RSRP-ChangeThreshold-r16} OPTIONAL,	-- Need ON

pur-ResponseWindowTimer-r16		ENUMERATED {sf240, sf480, sf960, sf1920, sf3840, sf5760, sf7680, sf10240}		OPTIONAL,	-- Need ON

pur-MPDCCH-Config-r16			PUR-MPDCCH-Config-r16		OPTIONAL,	-- Need ON

pur-PDSCH-FreqHopping-r16		BOOLEAN,

pur-PUCCH-Config-r16			PUR-PUCCH-Config-r16		OPTIONAL,	-- Need ON

pur-PUSCH-Config-r16			PUR-PUSCH-Config-r16		OPTIONAL,	-- Need ON

...,

[[	pur-PDSCH-maxTBS-r17		BOOLEAN						OPTIONAL	-- Need ON

]]

}

PUR-MPDCCH-Config-r16 ::=		SEQUENCE {

mpdcch-FreqHopping-r16			BOOLEAN,

mpdcch-Narrowband-r16			INTEGER (1..maxAvailNarrowBands-r13),

mpdcch-PRB-PairsConfig-r16		SEQUENCE{

numberPRB-Pairs-r16				ENUMERATED {n2, n4, n6, spare1},

resourceBlockAssignment-r16		BIT STRING (SIZE(4))

},

mpdcch-NumRepetition-r16		ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256},

mpdcch-StartSF-UESS-r16			CHOICE {

fdd								ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8, v10},

tdd							ENUMERATED {v1, v2, v4, v5, v8, v10, v20, spare1}

},

mpdcch-Offset-PUR-SS-r16	ENUMERATED {zero, oneEighth, oneQuarter,

threeEighth, oneHalf, fiveEighth,

threeQuarter, sevenEighth}

}

PUR-PUCCH-Config-r16 ::=			SEQUENCE {

n1PUCCH-AN-r16						INTEGER (0..2047)			OPTIONAL,	-- Need ON

pucch-NumRepetitionCE-Format1-r16	ENUMERATED {n1, n2, n4, n8}	OPTIONAL	-- Need ON

}

PUR-PUSCH-Config-r16 ::=		SEQUENCE {

pur-GrantInfo-r16				CHOICE {

ce-ModeA						SEQUENCE {

numRUs-r16						BIT STRING (SIZE(2)),

prb-AllocationInfo-r16			BIT STRING (SIZE(10)),

mcs-r16							BIT STRING (SIZE(4)),

numRepetitions-r16				BIT STRING (SIZE(3))

},

ce-ModeB						SEQUENCE {

subPRB-Allocation-r16			BOOLEAN,

numRUs-r16						BOOLEAN,

prb-AllocationInfo-r16			BIT STRING (SIZE(8)),

mcs-r16							BIT STRING (SIZE(4)),

numRepetitions-r16				BIT STRING (SIZE(3))

}

}	OPTIONAL,	-- Need ON

pur-PUSCH-FreqHopping-r16		BOOLEAN,

p0-UE-PUSCH-r16					INTEGER (-8..7),

alpha-r16						Alpha-r12,

pusch-CyclicShift-r16			ENUMERATED {n0, n6},

pusch-NB-MaxTBS-r16				BOOLEAN,

locationCE-ModeB-r16			INTEGER (0..5)	OPTIONAL -- Cond SubPRB

}

PUR-RSRP-ChangeThreshold-r16 ::= SEQUENCE {

increaseThresh-r16				RSRP-ChangeThresh-r16,

decreaseThresh-r16				RSRP-ChangeThresh-r16	OPTIONAL		--Need OP

}

RSRP-ChangeThresh-r16 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

-- ASN1STOP

#### –	PRACH-Config

The IE PRACH-ConfigSIB and IE PRACH-Config are used to specify the PRACH configuration in the system information and in the mobility control information, respectively.

PRACH-Config information elements

-- ASN1START

PRACH-ConfigSIB ::=				SEQUENCE {

rootSequenceIndex					INTEGER (0..837),

prach-ConfigInfo					PRACH-ConfigInfo

}

PRACH-ConfigSIB-v1310 ::=			SEQUENCE {

rsrp-ThresholdsPrachInfoList-r13		RSRP-ThresholdsPrachInfoList-r13,

mpdcch-startSF-CSS-RA-r13			CHOICE {

fdd-r13								ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8,

v10},

tdd-r13								ENUMERATED {v1, v2, v4, v5, v8, v10, v20, spare}

}																		OPTIONAL,	-- Cond MP

prach-HoppingOffset-r13				INTEGER (0..94)						OPTIONAL,	-- Need OR

prach-ParametersListCE-r13			PRACH-ParametersListCE-r13

}

PRACH-ConfigSIB-v1530 ::=			SEQUENCE {

edt-PRACH-ParametersListCE-r15		SEQUENCE (SIZE(1..maxCE-Level-r13)) OF EDT-PRACH-ParametersCE-r15

}

PRACH-Config ::=					SEQUENCE {

rootSequenceIndex					INTEGER (0..837),

prach-ConfigInfo					PRACH-ConfigInfo					OPTIONAL	-- Need ON

}

PRACH-Config-v1310 ::=				SEQUENCE {

rsrp-ThresholdsPrachInfoList-r13		RSRP-ThresholdsPrachInfoList-r13		OPTIONAL,	-- Cond MP

mpdcch-startSF-CSS-RA-r13			CHOICE {

fdd-r13								ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8,

v10},

tdd-r13								ENUMERATED {v1, v2, v4, v5, v8, v10, v20, spare}

}																		OPTIONAL,	-- Cond MP

prach-HoppingOffset-r13				INTEGER (0..94)						OPTIONAL,	-- Need OR

prach-ParametersListCE-r13			PRACH-ParametersListCE-r13			OPTIONAL,	-- Cond MP

initial-CE-level-r13					INTEGER (0..3)		OPTIONAL	-- Need OR

}

PRACH-Config-v1430 ::=				SEQUENCE {

rootSequenceIndexHighSpeed-r14				INTEGER (0..837),

zeroCorrelationZoneConfigHighSpeed-r14		INTEGER (0..12),

prach-ConfigIndexHighSpeed-r14				INTEGER (0..63),

prach-FreqOffsetHighSpeed-r14				INTEGER (0..94)

}

PRACH-ConfigSCell-r10 ::=				SEQUENCE {

prach-ConfigIndex-r10					INTEGER (0..63)

}

PRACH-ConfigInfo ::=				SEQUENCE {

prach-ConfigIndex					INTEGER (0..63),

highSpeedFlag						BOOLEAN,

zeroCorrelationZoneConfig			INTEGER (0..15),

prach-FreqOffset					INTEGER (0..94)

}

PRACH-ParametersListCE-r13 ::=	SEQUENCE (SIZE(1..maxCE-Level-r13)) OF PRACH-ParametersCE-r13

PRACH-ParametersCE-r13 ::=			SEQUENCE {

prach-ConfigIndex-r13					INTEGER (0..63),

prach-FreqOffset-r13						INTEGER (0..94),

prach-StartingSubframe-r13				ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128,

sf256}				OPTIONAL,	-- Need OP

maxNumPreambleAttemptCE-r13

ENUMERATED {n3, n4, n5, n6, n7, n8, n10}	OPTIONAL,	-- Need OP

numRepetitionPerPreambleAttempt-r13		ENUMERATED {n1,n2,n4,n8,n16,n32,n64,n128},

mpdcch-NarrowbandsToMonitor-r13			SEQUENCE (SIZE(1..2)) OF

INTEGER (1..maxAvailNarrowBands-r13),

mpdcch-NumRepetition-RA-r13				ENUMERATED {r1, r2, r4, r8, r16,

r32, r64, r128, r256},

prach-HoppingConfig-r13					ENUMERATED {on,off}

}

EDT-PRACH-ParametersCE-r15 ::=		SEQUENCE {

edt-PRACH-ParametersCE-r15		SEQUENCE {

prach-ConfigIndex-r15				INTEGER (0..63),

prach-FreqOffset-r15					INTEGER (0..94),

prach-StartingSubframe-r15			ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128, sf256}			OPTIONAL,	-- Need OP

mpdcch-NarrowbandsToMonitor-r15		SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)

}	OPTIONAL -- Need OR

}

RSRP-ThresholdsPrachInfoList-r13 ::= SEQUENCE (SIZE(1..3)) OF RSRP-Range

PRACH-TxDuration-r17::=			SEQUENCE {

prach-TxDuration-r17				ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128}

}

-- ASN1STOP

Here is the TP for eMTC CB-Msg3 configuration parameters.

#### –	CB-Msg3-ConfigSIB

The IE CB-Msg3-Config is used to specify the CB-Msg3 configuration.

CB-Msg3-ConfigSIB information element

-- ASN1START

CB-Msg3-ConfigSIB-r19 ::=			SEQUENCE {

cb-Msg3-MinRSRP-Threshold-r19		NRSRP-Range							OPTIONAL,	--Need OR

cb-Msg3-RSRP-ThresholdList-r19      CB-Msg3-RSRP-ThresholdList-r19		OPTIONAL,	--Need OP

cb-Msg3-ParametersList-r19			CB-Msg3-ParametersList-r19,

cb-Msg3-ResponseWindowTimer-r19		ENUMERATED {FFS},

}

CB-Msg3-ParametersList-r19 ::=		SEQUENCE (SIZE (1.. maxCB-Msg3-Resources-r19)) OF

CB-Msg3-Parameters-r19

CB-Msg3-Parameters-r19 ::=			SEQUENCE {

cb-Msg3-NumReplicas					INTEGER(1..4),

cb-Msg3-DSATransmissionWindow-r19		SEQUENCE {

startSFN-19								INTEGER (0.. 1023),

windowSize-19							ENUMERATED {FFS},

windowPeriodicity-r19					ENUMERATED {FFS}

}																		OPTIONAL,	--Need OP

cb-Msg3-StartTimeParameters-r19		SEQUENCE {

pusch-periodicity-r19				ENUMERATED {FFS},

pusch-startTime-r19					INTEGER (0..1023),

pusch-startSubframe-r19				INTEGER (0..9)

}

cb-Msg3-MPDCCH-Config-r19			CB-Msg3-MPDCCH-Config-r19,

cb-Msg3-PUCCH-Config-r19			CB-Msg3-PUCCH-Config-r19,

cb-Msg3-PUSCH-Config-r19			CB-Msg3-PUSCH-Config-r19,

cb-Msg3-PDSCH-Config-r19			CB-Msg3-PDSCH-Config-r19,

cb-Msg3-TBS-r19						ENUMERATED {FFS},

...

}

CB-MSG3-MPDCCH-Config-r19 ::=		SEQUENCE {

mpdcch-FreqHopping-r19			BOOLEAN,

mpdcch-Narrowband-r19			INTEGER (1..maxAvailNarrowBands-r13),

mpdcch-PRB-PairsConfig-r19		SEQUENCE{

numberPRB-Pairs-r19				ENUMERATED {n2, n4, n6, spare1},

resourceBlockAssignment-r19		BIT STRING (SIZE(4))

},

mpdcch-NumRepetition-r19		ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256},

mpdcch-StartSF-UESS-r19			ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8, v10},

mpdcch-Offset-SS-r19			ENUMERATED {zero, oneEighth, oneQuarter,

threeEighth, oneHalf, fiveEighth,

threeQuarter, sevenEighth}

}

CB-Msg3-PUCCH-Config-r19 ::=		SEQUENCE {

n1PUCCH-AN-r19						INTEGER (0..2047),			OPTIONAL,	-- Need ON

pucch-NumRepetitionCE-Format1-r19	ENUMERATED {n1, n2, n4, n8} OPTIONAL	-- Need ON

}

CB-Msg3-PUSCH-Config-r19 ::=		SEQUENCE {

numRUs-r19							BIT STRING (SIZE(2)),

prb-AllocationInfo-r19				BIT STRING (SIZE(10)),

mcs-r19								BIT STRING (SIZE(4)),

numRepetitions-r19					BIT STRING (SIZE(3))

PUSCH-FreqHopping-r19				BOOLEAN,

p0-UE-PUSCH-r19						INTEGER (-8..7),

alpha-r19							Alpha-r12

}

CB-Msg3-PDSCH-Config-r19 ::=		SEQUENCE {

cb-Msg3-PDSCH-FreqHopping-r19		BOOLEAN,

cb-Msg3-PDSCH-maxTBS-r19			BOOLEAN

}

CB-Msg3-RSRP-ThresholdList-r19 ::=	 SEQUENCE (SIZE (1..3)) OF RSRP-Range

maxCB-Msg3-Resources-r19	INTEGER ::=	4	-- Maximum number of CB-Msg3 resources

-- ASN1STOP

Notes

- CB-Msg3-ConfigSIB: This IE is assumed to be in SIB2.

- cb-Msg3-MinRSRP-Threshold-r19: It is assumed as an optional IE. If it is absent, no minimal threshold is used.

- cb-Msg3-RSRP-ThresholdList-r19: It is assumed as an optional IE. If this IE is absent, the thresholds list of PRACH is used.

- cb-Msg3-DSATransmissionWindow-r19: It is assumed as an optional IE. If the number of the replicas is one, the DSA transmission window is not needed.

  - startSFN-19 in cb-Msg3-DSATransmissionWindow-r19: Although the term H-SFN offset is used in the agreement, the rapporteur believes it should be a SFN offset during the H-SFN duration.

  - hsfn-LSB-Info-r16 in PUR-Config: This IE is not used as the periodicity of CB-Msg3 resource is assumed shorted than a H-SFN duration (i.e., 10.24s).

- CB-MSG3-MPDCCH-Config-r19:

  - mpdcch-StartSF-UESS-r19: The configuration for TDD is not used, as only FDD eMTC is supported for IoT NTN.

  - Other parameters in are copied from IE PUR-MPDCCH-Config-r16.

- CB-Msg3-PUSCH-Config-r19:

  - According to the RAN1 reply LS R1-2407548 on TA validation, only CE mode A is confirmed. Whether the CE mode B is supported is still FFS. Therefore, only the PUSCH configuration for only CE mode A is introduced for now.

  - pusch-CyclicShift-r16: This parameter is not used because this parameter is specified for PUR only.

  - pusch-NB-MaxTBS-r16: This parameter is not used as a maximum TBS is provided.

  - locationCE-ModeB-r16: This parameter is not used as support of CE mode B is FFS.

- CB-Msg3-PDSCH-Config-r19 CB-Msg3-PUCCH-Config-r19

  - Two fields copied from PUR-PUCCH-Config-r16. Note it is assumed that the fields are mandatory as there is no dedicated configuration from connected mode in CB-MSG3 procedure.

- cb-Msg3-TBS-r19: Whether the maximum TBS is CE level specific is FFS. Rapporteur assumes it is CE level specific.

Companies are invited to provide comments on the above TP including but not limited to below discussion points.

- Which SIB should be used for IE CB-Msg3-ConfigSIB ?

- Any parameter is missing in the TP and why this should be added?

- Any parameter should be removed from the TP?

- Most value range of the parameters are copied from PUR parameters, any further change needed?

Q1: Any comments on the TP of CB-Msg3-ConfigSIB?

| Company | Comments |
| --- | --- |
| Nokia | On the TP itself, we have below comments: - For the IE cb-Msg3-MinRSRP-Threshold-r19, for modelling simplicity, it can be merged into the list of cb-Msg3-RSRP-ThresholdList-r19 (e.g., as the entry in the list to determine the most robust CE level). i.e., the number of elements in the list indicates the number of supported CE level. The value of each element is the RSRP threshold for the corresponding CE level. - For the “NRSRP-Range”, it should be “RSRP-Range” since the IE is for eMTC instead of NB-IoT. - For the “CB-Msg3-ParametersList-r19 ::= SEQUENCE (SIZE (1.. maxCB-Msg3-Resources-r19))”, should it be (1…maxCE-Level)? We understand the CB-Msg3-Parameters-r19 is a per-CE level configuration. - For the “cb-Msg3-NumReplicas INTEGER(1..4)”, in principle it can be an optional parameter. If absent, it then means 1 (stand-alone case). - For the MPDCCH search space for CB-Msg4, we think it should be checked and confirmed by RAN1. - For the field description on “cb-Msg3-DSATransmissionWindow-r19:... If the number of the replicas is one, the DSA transmission window is not needed”, the wording “not needed” should be “not applied”. Additional parameters: - The parameters for CB-Msg3 fallback to 4-step RACH/EDT can be considered. Furthermore, the power ramping step parameters can be considered if the UE transmits CB-Msg3 multiple times. |

### NB-IoT part

#### –	PUR-Config-NB

The IE PUR-Config-NB is used to specify PUR configuration.

PUR-Config-NB information element

-- ASN1START

PUR-Config-NB-r16	::=				SEQUENCE {

pur-ConfigID-r16					PUR-ConfigID-NB-r16				OPTIONAL,	--Need OR

pur-TimeAlignmentTimer-r16			INTEGER (1..8)				OPTIONAL,	--Need OR

pur-NRSRP-ChangeThreshold-r16		SetupRelease {PUR-NRSRP-ChangeThreshold-NB-r16}

OPTIONAL,	--Need ON

pur-ImplicitReleaseAfter-r16		ENUMERATED {n2, n4, n8, spare}	OPTIONAL,	--Need OR

pur-RNTI-r16						C-RNTI							OPTIONAL,	--Need ON

pur-ResponseWindowTimer-r16			ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}

OPTIONAL,	--Need ON

pur-StartTimeParameters-r16			SEQUENCE {

periodicityAndOffset-r16			PUR-PeriodicityAndOffset-NB-r16,

startSFN-r16						INTEGER (0..1023),

startSubframe-r16					INTEGER (0..9),

hsfn-LSB-Info-r16					BIT STRING (SIZE(1))

}																	OPTIONAL,	--Need ON

pur-NumOccasions-r16				ENUMERATED {one, infinite},

pur-PhysicalConfig-r16				SEQUENCE {

carrierConfig-r16					CarrierConfigDedicated-NB-r13,

npusch-NumRUsIndex-r16				INTEGER (0..7),

npusch-NumRepetitionsIndex-r16		INTEGER (0..7),

npusch-SubCarrierSetIndex-r16		CHOICE {

khz15								INTEGER (0..18),

khz3dot75							INTEGER (0..47)

},

npusch-MCS-r16						CHOICE {

singleTone							INTEGER (0..10),

multiTone							INTEGER (0..13)

},

p0-UE-NPUSCH-r16					INTEGER (-8..7),

alpha-r16							ENUMERATED {al0, al04, al05, al06,

al07, al08, al09, al1},

npusch-CyclicShift-r16				ENUMERATED {n0, n6},

npdcch-Config-r16					NPDCCH-ConfigDedicated-NB-r13

}	OPTIONAL,	-- Need ON

...,

[[

pur-PhysicalConfig-v1650			SEQUENCE {

ack-NACK-NumRepetitions-r16		ACK-NACK-NumRepetitions-NB-r13

}														OPTIONAL	--Need ON

]],

[[

pur-PhysicalConfig-v1700			SEQUENCE {

pur-UL-16QAM-Config-r17		SetupRelease {PUR-UL-16QAM-Config-NB-r17} OPTIONAL,	-- Need ON

pur-DL-16QAM-Config-r17		SetupRelease {NPDSCH-16QAM-Config-NB-r17} OPTIONAL	-- Need ON

}				OPTIONAL	-- Need ON

]]

}

PUR-NRSRP-ChangeThreshold-NB-r16 ::=	SEQUENCE {

increaseThresh-r16						NRSRP-ChangeThresh-NB-r16,

decreaseThresh-r16						NRSRP-ChangeThresh-NB-r16	OPTIONAL	--Need OP

}

PUR-UL-16QAM-Config-NB-r17 ::= SEQUENCE {

uplinkPowerControlDedicated-r17		UplinkPowerControlDedicated-NB-v1700

}

NRSRP-ChangeThresh-NB-r16 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

-- ASN1STOP

#### –	NPRACH-ConfigSIB-NB

The IE NPRACH-ConfigSIB-NB is used to specify the NPRACH configuration for the anchor and non-anchor carriers.

NPRACH-ConfigSIB-NB information elements

-- ASN1START

NPRACH-ConfigSIB-NB-r13 ::=			SEQUENCE {

nprach-CP-Length-r13				ENUMERATED {us66dot7, us266dot7},

rsrp-ThresholdsPrachInfoList-r13	RSRP-ThresholdsNPRACH-InfoList-NB-r13	OPTIONAL,	-- Need OR

nprach-ParametersList-r13		NPRACH-ParametersList-NB-r13

}

NPRACH-ConfigSIB-NB-v1330 ::=		SEQUENCE {

nprach-ParametersList-v1330			NPRACH-ParametersList-NB-v1330

}

NPRACH-ConfigSIB-NB-v1450 ::=		SEQUENCE {

maxNumPreambleAttemptCE-r14			ENUMERATED {n3, n4, n5, n6, n7, n8, n10, spare1}

}

NPRACH-ConfigSIB-NB-v1530 ::=		SEQUENCE {

tdd-Parameters-r15					SEQUENCE {

nprach-PreambleFormat-r15			ENUMERATED {

fmt0, fmt1, fmt2, fmt0-a, fmt1-a},

dummy								ENUMERATED {

n1, n2, n4, n8, n16, n32, n64, n128,

n256, n512, n1024},

nprach-ParametersListTDD-r15		NPRACH-ParametersListTDD-NB-r15

}	OPTIONAL,		-- Cond TDD

fmt2-Parameters-r15					SEQUENCE {

nprach-ParametersListFmt2-r15		NPRACH-ParametersListFmt2-NB-r15 OPTIONAL,	-- Need OR

nprach-ParametersListFmt2EDT-r15	NPRACH-ParametersListFmt2-NB-r15 OPTIONAL	-- Cond EDT2

}	OPTIONAL,		-- Need OR

edt-Parameters-r15					SEQUENCE {

edt-SmallTBS-Subset-r15				ENUMERATED {true}				OPTIONAL,	-- Need OR

edt-TBS-InfoList-r15				EDT-TBS-InfoList-NB-r15,

nprach-ParametersListEDT-r15		NPRACH-ParametersList-NB-r14	OPTIONAL	-- Need OR

}	OPTIONAL		-- Cond EDT1

}

NPRACH-ConfigSIB-NB-v1550 ::=		SEQUENCE {

tdd-Parameters-v1550				SEQUENCE {

nprach-ParametersListTDD-v1550		NPRACH-ParametersListTDD-NB-v1550

}

}

NPRACH-ParametersList-NB-r13 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r13

NPRACH-ParametersList-NB-v1330 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-v1330

NPRACH-Parameters-NB-r13::=			SEQUENCE {

nprach-Periodicity-r13					ENUMERATED {ms40, ms80, ms160, ms240,

ms320, ms640, ms1280, ms2560},

nprach-StartTime-r13					ENUMERATED {ms8, ms16, ms32, ms64,

ms128, ms256, ms512, ms1024},

nprach-SubcarrierOffset-r13				ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1},

nprach-NumSubcarriers-r13				ENUMERATED {n12, n24, n36, n48},

nprach-SubcarrierMSG3-RangeStart-r13	ENUMERATED {zero, oneThird, twoThird, one},

maxNumPreambleAttemptCE-r13				ENUMERATED {n3, n4, n5, n6, n7, n8, n10, spare1},

numRepetitionsPerPreambleAttempt-r13	ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128},

npdcch-NumRepetitions-RA-r13			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1},

npdcch-StartSF-CSS-RA-r13				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64},

npdcch-Offset-RA-r13					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

}

NPRACH-Parameters-NB-v1330 ::=		SEQUENCE {

nprach-NumCBRA-StartSubcarriers-r13		ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24,

n32, n34, n35, n36, n40, n44, n46, n48}

}

NPRACH-ParametersList-NB-r14 ::=		SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF

NPRACH-Parameters-NB-r14

NPRACH-Parameters-NB-r14 ::=			SEQUENCE {

nprach-Parameters-r14					SEQUENCE {

nprach-Periodicity-r14					ENUMERATED {ms40, ms80, ms160, ms240,

ms320, ms640, ms1280, ms2560}

OPTIONAL,	-- NEED OP

nprach-StartTime-r14					ENUMERATED {ms8, ms16, ms32, ms64,

ms128, ms256, ms512, ms1024}

OPTIONAL,	-- NEED OP

nprach-SubcarrierOffset-r14				ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1}

OPTIONAL,	-- NEED OP

nprach-NumSubcarriers-r14				ENUMERATED {n12, n24, n36, n48}

OPTIONAL,	-- NEED OP

nprach-SubcarrierMSG3-RangeStart-r14	ENUMERATED {zero, oneThird, twoThird, one}

OPTIONAL,	-- NEED OP

npdcch-NumRepetitions-RA-r14			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

npdcch-StartSF-CSS-RA-r14				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}

OPTIONAL,	-- NEED OP

npdcch-Offset-RA-r14					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

OPTIONAL,	-- NEED OP

nprach-NumCBRA-StartSubcarriers-r14		ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24,

n32, n34, n35, n36, n40, n44, n46, n48}

OPTIONAL,	-- NEED OP

npdcch-CarrierIndex-r14					INTEGER (1..maxNonAnchorCarriers-NB-r14)

OPTIONAL,	-- Need OP

...

}	OPTIONAL	-- Need OR

}

NPRACH-ParametersListTDD-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF

NPRACH-ParametersTDD-NB-r15

NPRACH-ParametersTDD-NB-r15 ::=		SEQUENCE {

nprach-Parameters-r15					SEQUENCE {

nprach-Periodicity-r15					ENUMERATED {ms80, ms160, ms320, ms640,

ms1280, ms2560, ms5120, ms10240}

OPTIONAL,	-- NEED OP

nprach-StartTime-r15					ENUMERATED {ms10, ms20, ms40, ms80,

ms160, ms320, ms640, ms1280,

ms2560, ms5120, spare6, spare5,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

nprach-SubcarrierOffset-r15				ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1}

OPTIONAL,	-- NEED OP

nprach-NumSubcarriers-r15				ENUMERATED {n12, n24, n36, n48}

OPTIONAL,	-- NEED OP

nprach-SubcarrierMSG3-RangeStart-r15	ENUMERATED {zero, oneThird, twoThird, one}

OPTIONAL,	-- NEED OP

npdcch-NumRepetitions-RA-r15			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

npdcch-StartSF-CSS-RA-r15				ENUMERATED {v4, v8, v16, v32, v48, v64, v96, v128}

OPTIONAL,	-- NEED OP

npdcch-Offset-RA-r15					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

OPTIONAL,	-- NEED OP

nprach-NumCBRA-StartSubcarriers-r15		ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24,

n32, n34, n35, n36, n40, n44, n46, n48}

OPTIONAL,	-- NEED OP

...

}	OPTIONAL	-- Need OR

}

NPRACH-ParametersListTDD-NB-v1550 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF

NPRACH-ParametersTDD-NB-v1550

NPRACH-ParametersTDD-NB-v1550 ::=	SEQUENCE {

maxNumPreambleAttemptCE-v1550			ENUMERATED {n3, n4, n5, n6, n7, n8, n10, spare1},

numRepetitionsPerPreambleAttempt-v1550	ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128,

n256, n512, n1024}

}

NPRACH-ParametersListFmt2-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ParametersFmt2-NB-r15

NPRACH-ParametersFmt2-NB-r15 ::=		SEQUENCE {

nprach-Parameters-r15					SEQUENCE {

nprach-Periodicity-r15					ENUMERATED {ms40, ms80, ms160, ms320,

ms640, ms1280, ms2560, ms5120}

OPTIONAL,	-- NEED OP

nprach-StartTime-r15					ENUMERATED {ms8, ms16, ms32, ms64,

ms128, ms256, ms512, ms1024}

OPTIONAL,	-- NEED OP

nprach-SubcarrierOffset-r15				ENUMERATED {n0, n36, n72, n108, n6, n54, n102, n42,

n78, n90, n12, n24, n48, n84, n60, n18}

OPTIONAL,	-- NEED OP

nprach-NumSubcarriers-r15				ENUMERATED {n36, n72, n108, n144}

OPTIONAL,	-- NEED OP

nprach-SubcarrierMSG3-RangeStart-r15	ENUMERATED {zero, oneThird, twoThird, one}

OPTIONAL,	-- NEED OP

npdcch-NumRepetitions-RA-r15			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

OPTIONAL,	-- NEED OP

npdcch-StartSF-CSS-RA-r15				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}

OPTIONAL,	-- NEED OP

npdcch-Offset-RA-r15					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

OPTIONAL,	-- NEED OP

nprach-NumCBRA-StartSubcarriers-r15		ENUMERATED {

n24, n30, n33, n36, n60, n66, n69, n72,

n96, n102, n105, n108, n120, n132, n138, n144}

OPTIONAL,	-- NEED OP

npdcch-CarrierIndex-r15					INTEGER (1..maxNonAnchorCarriers-NB-r14)

OPTIONAL,	-- Need OP

...

}	OPTIONAL	-- Need OR

}

NPRACH-TxDurationFmt01-NB-r17 ::=	SEQUENCE {

nprach-TxDurationFmt01-r17		ENUMERATED {n2, n4, n8, n16, n32, n64}

}

NPRACH-TxDurationFmt2-NB-r17 ::=	SEQUENCE {

nprach-TxDurationFmt2-r17		ENUMERATED {n1, n2, n4, n8, n16}

}

RSRP-ThresholdsNPRACH-InfoList-NB-r13 ::= SEQUENCE (SIZE(1..2)) OF RSRP-Range

EDT-TBS-InfoList-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF EDT-TBS-NB-r15

EDT-TBS-NB-r15 ::=	SEQUENCE {

edt-SmallTBS-Enabled-r15		BOOLEAN,

edt-TBS-r15						ENUMERATED {b328, b408, b504, b584, b680, b808, b936, b1000}

}

-- ASN1STOP

Here is the TP for NB-IoT CB-Msg3 configuration parameters.

#### –	CB-Msg3-ConfigSIB-NB

The IE CB-Msg3-ConfigSIB-NB is used to specify CB-Msg3 configuration.

CB-Msg3-ConfigSIB-NB information element

-- ASN1START

CB-Msg3-ConfigSIB-NB-r19 ::=			SEQUENCE {

cb-Msg3-MinRSRP-Threshold-r19			NRSRP-Range-NB-r14					OPTIONAL,	--Need OR

cb-Msg3-RSRP-ThresholdList-r19      	CB-Msg3-RSRP-ThresholdList-NB-r19	OPTIONAL,	--Need OP

cb-Msg3-ParametersList-r19				CB-Msg3-ParametersList-NB-r19,

cb-Msg3-ResponseWindowTimer-r19			ENUMERATED {FFS}

}

CB-Msg3-ParametersList-NB-r19 ::=			SEQUENCE (SIZE (1.. maxCB-Msg3-Resources-NB-r19)) OF

CB-Msg3-Parameters-NB-r19

CB-Msg3-Parameters-NB-r19 ::=			SEQUENCE {

cb-Msg3-NumReplicas						INTEGER(1..4),

cb-Msg3-DSATransmissionWindow-r19		SEQUENCE {

startSFN-19								INTEGER (0.. 1023),

windowSize-19							ENUMERATED {FFS},

windowPeriodicity-r19					ENUMERATED {FFS}

}																		OPTIONAL,	--Need OP

cb-Msg3-StartTimeParameters-r19		SEQUENCE {

npusch-periodicity-r19				ENUMERATED {FFS},

npusch-startTime-r19				INTEGER (0..1023),

npusch-startSubframe-r19			INTEGER (0..9)

}

cb-Msg3-PhysicalConfig-r19 ::=		SEQUENCE {

npusch-NumRUsIndex-r19				INTEGER (0..7),

npusch-NumRepetitionsIndex-r19		INTEGER (0..7),

npusch-SubCarrierIndex-r19			INTEGER (0..47),

npusch-MCS-r19						INTEGER (0..10),

p0-UE-NPUSCH-r19					INTEGER (-8..7),

alpha-r19							ENUMERATED {al0, al04, al05, al06,

al07, al08, al09, al1},

npdcch-NumRepetitions-r19			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1},

npdcch-StartSF-CSS-r19				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64},

npdcch-Offset-r19					ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

npdcch-CarrierIndex-r19				INTEGER (1..maxNonAnchorCarriers-NB-r14)

OPTIONAL	-- Need OP

}

cb-Msg3-TBS-NB-r19						ENUMERATED {FFS},

...

}

CB-Msg3-RSRP-ThresholdList-NB-r19 ::=	SEQUENCE (SIZE(1..2)) OF RSRP-Range

maxCB-Msg3-Resources-NB-r19	INTEGER ::=	3	-- Maximum number of CB-Msg3 resources for NB-IoT

-- ASN1STOP

Notes

- CB-Msg3-ConfigSIB-NB:This IE is used in the SIB2-NB for anchor carrier and SIB22-NB for non-anchor carrier.

- cb-Msg3-MinRSRP-Threshold-r19: It is assumed as an optional IE. If it is absent, no minimal threshold is used.

- CB-Msg3-RSRP-ThresholdList-NB-r19: It is assumed as an optional IE. If this IE is absent, the thresholds list of PRACH is used.

- cb-Msg3-DSATransmissionWindow-r19: It is assumed as an optional IE. If the number of the replicas is one, the DSA transmission window is not needed.

  - startSFN-19 in cb-Msg3-DSATransmissionWindow-r19: Although the term H-SFN offset is used in the agreement, the rapporteur believes it should be a SFN offset during the H-SFN duration.

  - hsfn-LSB-Info-r16 in PUR-Config-NB: This IE is not used as the periodicity of CB-Msg3 resource is assumed shorted than a H-SFN duration (i.e., 10.24s).

- cb-Msg3-PhysicalConfig-r19

  - npusch-SubCarrierIndex-r19 / npusch-MCS-r19: According to the RAN1 reply LS R1-2407548 on TA validation, only 3.75kHz SCS is confirmed. Whether the 15kHz SCS NPUSCH is supported is still FFS. Therefore, only parameters for 3.75kHz SCS is introduced for now.

  - p0-UE-NPUSCH-r19, alpha-r19: The parameters for NPUSCH transmission power are assumed as CE level specific.

  - npusch-CyclicShift-r16 in PUR-Config-NB: This IE is not used because this parameter is specified for PUR only.

  - npdcch-NumRepetitions-r19/npdcch-StartSF-CSS-r19/npdcch-Offset-r19: 3 parameters copied from NPDCCH-ConfigDedicated-NB-r13. Note hear it should be CSS instead of USS.

  - npdcch-CarrierIndex-r19: The non-anchor carrier index for monitoring Msg4. If this IE is absent, anchor carrier is assumed to be used.

  - ack-NACK-NumRepetitions-r16 in PUR-Config-NB: This IE is not used because the same IE in Msg4 can be used.

- cb-Msg3-TBS-NB-r19: Whether the maximum TBS is CE level specific is FFS. Rapporteur assumes it is CE level specific.

Companies are invited to provide comments on the above TP including but not limited to below discussion points.

- Which SIB should be used for IE CB-Msg3-ConfigSIB ?

- Any parameter is missing in the TP and why this should be added?

- Any parameter should be removed from the TP?

- Most value range of the parameters are copied from PUR parameters, any further change needed?

Q2: Any comments on the TP of CB-Msg3-ConfigSIB-NB?

| Company | Comments |
| --- | --- |
| Nokia | Same comments as Q1 (except the one for NRSRP-Range). Furthermore, for NB-IoT, we think it is too early to conclude “the periodicity of CB-Msg3 resource is assumed shorted than a H-SFN duration (i.e., 10.24s).” E.g., when considering the largest number of repetitions, number of resources units (RU), and number of slots in RU, the maximum transmission time for one replica could be 10.24 s for 15kHz SCS and 40.96 s for 3.75kHz SCS. |

## Msg4 monitoring window and RNTI

Regarding the Msg4 monitoring window and RNTI, the following agreements have been reached:

RAN2#127bis agreement:

- The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

RAN2#128 agreement:

1.	For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

2.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

During the online discussion on CB-Msg3 in RAN2#129, it was found that the DSA transmission window, the Msg4 monitoring window, and the RNTI for Msg4 are closely related. An offline discussion was initiated to narrow down the combinations of these three aspects.

[AT129][306][R19 IoT NTN] TX and RX window for CB-msg3 (Mediatek)

Scope: discuss details of transmission window (e.g. sliding or fixed), monitoring window and whether 1 or multiple RNTIs should be considered for CB-msg3/DSA

Intended outcome: summary of the offline discussion

Deadline for companies' feedback:  Thursday 2025-02-20 20:00

Deadline for rapporteur's summary (in R2-2501420):  Friday 2025-02-21 08:00

As a result of the offline discussion [1], two proposals were submitted as follows:

Proposal 1: For CB-Msg3 DSA transmission window design, RAN2 to discuss below two options:

- Option 1

  - Transmission window is started at the first replica that the UE randomly select from CB-MSG3 occasions. Window length is configured by network via SIB.

- Option 2

  - Transmission window is configured by network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity.

  - The UE first selects a nearest DSA transmission window and then randomly select K replicas inside the window.

Proposal 2: For CB-Msg3 DSA monitor window and RNTI design, RAN2 to discuss below two options:

- Option 1

  - RNTI is calculated for each Replica respectively according to the selected resource (same as SA).

  - After the end of all repetition of CB-Msg3 PUSCH transmission of each replica, UE starts the corresponding monitor window, taking UE-eNB RTT into account.

  - The UE has to monitor multiple RNTIs in multiple monitor windows.

- Option 2

  - A single RNTI is used for all replicas and it is calculated based on the location of the selected DSA transmission window.

  - A single monitor window is used for all replicas.

Regarding proposal 1 (i.e., the DSA transmission window), the following working assumption have been reached.

RAN2#129 Working assumption:

- For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today

The UE first selects the next DSA transmission window and then randomly select K replicas inside the window.

P2 was briefly discussed, but no conclusion was reached. We will continue our discussion on the Msg4 monitoring window and RNTI in this offline.

Although option 2 is adopted for the DSA transmission window, companies have varied views on the Msg4 monitoring window and RNTI. Some companies mentioned that even with a fixed DSA transmission window, the Msg4 monitoring window can still be multiple. The RNTI design would also be impacted accordingly. Therefore, the rapporteur would like to decouple options on the monitoring window and RNTI.

Companies are invited to provide their views on the following questions:

Q3: Should there be one Msg4 monitoring window or multiple windows for DSA transmission?

| Company | Multiple or Single | Comments |
| --- | --- | --- |
| Nokia | Single for Half-Duplex UEs. Multiple for Full-Duplex eMTC UE. | In last RAN2 meeting, it was agreed that RAN2 assumes that a pointer solution is not needed in Rel-19. This means the NW is not aware of the Msg3 transmission occasions within the Msg3 transmission window. - For half-duplex UE, there may have Msg3 and Msg4 collision if the NW schedules the Msg4 within the Msg3 transmission window. Therefore, a single Msg4 monitoring window could be started at least after the end of Msg3 transmission window hence the NW schedules Msg4 within the Msg4 monitoring window to avoid the UL/DL collision. The drawback of single Msg4 monitoring window is that, the NW has no means to respond Msg4 to UE as early as possible even if NW has decoded a Msg3 successfully. This is not only a waste of UE’s power consumption but also a waste of NW resource (which will increase the Msg3 collision rate as well). - For full-duplex UE, there is no Msg3 and Msg4 window collision issue at all. Therefore, once the eNB successfully decodes one of the multiple replicas, it may respond as early as possible (i.e., without waiting for the reception of the remaining replica(s)). The UE could stop transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity. The early termination of Msg3 replicas transmission would benefit both UE and NW. In this case, a Msg4 reception window should be started after the first Msg3 replica (plus RTT) to receive the possible Msg4 response as early as possible. On how to (re)start the Msg4 reception window, there may have two options: - In one way (Option#1), the UE may apply the window with quite long duration to cover the whole period to receive possible Msg4 reception responding to the first or subsequent Msg3 replicas. However, this may cause UE and NW misalignment because NW may miss the first Msg3 replica reception hence UE and NW have different understanding on when the window is started and the exact window length. To avoid the NW and UE misalignment, the Msg4 reception window should be restarted after each Msg3 replica. - In the other way (Option#2), the UE may start an individual Msg4 reception window for each of the replica. (i.e., multiple Msg4 reception windows). From specification implementation point of view, Option#2 seems easy for window modelling, and it anyway needs to be specified for SA with single replica. Therefore, we prefer the Option#2 for Full Duplex UE. Note: PUSCH repetition early termination is a legacy feature which has been supported in legacy eMTC Full Duplex system (see mpdcch-UL-HARQ-ACK-FeedbackConfig). |

Q4: What should be the start point of Msg4 monitoring window(s) for DSA transmission?

| Company | Comments |
| --- | --- |
| Nokia | - For half-duplex, since the NW is not aware the Msg3 replica occasions, to avoid UL/DL collision, the single Msg4 monitoring window starts at the end of the Msg3 transmission window plus UE-eNB RTT. - For full-duplex, the UE starts individual Msg4 reception window for each of the replica in the subframe containing the last PUSCH repetition of the replica plus UE-eNB RTT. |

Q5: Should there be one RNTI or multiple RNTIs for DSA transmission?

| Company | Multiple or Single | Comments |
| --- | --- | --- |
| Nokia | Single | Due to up to 4 RNTIs can be derived in DSA for different replica occasions, it may burden UE to monitor 4 RNTIs for Msg4 reception at the same time. |

Q6: How is the RNTI derived for DSA transmission?

| Company | Comments |
| --- | --- |
| Nokia | For the RNTI used for Msg4 monitor, the most straight-forward method is that the UE includes the RNTI information in the Msg3 content. We acknowlege the 16 bits C-RNTI may overhead Msg3 a lot. The alternative is to derive the C-RNTI based on the NW-configured/fixed Msg3 transmission window. We think RAN2 can further discuss the details on how to reduce the C-RNTI overhead or how to derive the C-RNTI from the window. |

# Summary

# References

[1] R2-2501420 Report of [AT129][306][R19 IoT NTN] TX and RX window for CB-msg3 Mediatek