# 3GPP TSG RAN WG2 Meeting #130      	                                                 R2-250xxxx
Malta, MT, 19th to 23rd May 2025

Agenda item:	8.5.1

Source:	Ericsson

Title:	Comments to 38.331 CR for NES

Document for:	Discussion and Decision

# 1	Introduction

This is a summary document on collection of comments to TS 38.331 CR during below running CR discussion:

[POST129b][111][NES] (Ericsson)

Scope: Update RRC running CR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (RRC running CR can keep editor’s notes for readability).

Intended outcome: Updated RRC running CR and essential RRC open issue list.

Deadline: Long email discussion

DL for the email discussion is 2nd May, please provide your comments early so there is time to resolve when needed. Last comments to take into account should be uploaded by 23:59 UTC 1st May. Later comments are taken into account by best effort.

# 2	Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Ericsson | Helka-Liina Määttänen | Helka-liina.maattanen@ericsson.com |
| Apple | Peng Cheng | Pcheng24@apple.com |
| OPPO | Qianxi Lu | qianxi.lu@oppo.com |

# 3	Open issue list

5.2.2.3.1

Editor’s note:

FFS whether to capture the UE first should acquire a valid SIB1 (e.g. via SIB1 request) for camping on an OD-SIB1 NES cell.

FFS RAN1 discussion on e.g. ssb-SubcarrierOffset

5.2.2.3.3x

Editor’s note:

FFS how does UE check is SIB1 is already provided.

FFS: if there is need to emphasize it is normal uplink

FFS reference for where are the details on how UE is obtaining SIB1, possibly RAN1 specification

5.2.2.4.2x

Editors notes: FFS depending SIBxx/UL WUS validity discussion details

SIBxx

Editor’s note: Only parameters in R1-2501645 that are in own rows are implemented and not all listed e.g. in cell 17P or 21P.

FFS to group some parameters under subIEs like frequencyInfoUL

FFS to separate IE OD-SIB1  as own IE, for review purposes it is here now.

FFS: value for maxCells, maxSIB1-Message, maxPCI

FFS: optionality of the parameters as there was no input on this

FFS: if list of cells is ARFCN&PCI or only PCI

DownlinkConfigCommonSIB

Editor’s note:

FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions. FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations.

FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

UE-RadioPagingInfo

Editor’s note: FFS details

si-BroadcastStatus

FFS: how to capture that a CONNECTED MODE UE supporting OD-SIB1 who is in a cell that does not broadcast SIB1, understands that the stored SIB1 is the latest SIB1.

| Company | Detailed comments on FFSs | Rapporteur response |
| --- | --- | --- |
| Apple | We think the following ENs can be removed: FFS whether to capture the UE first should acquire a valid SIB1 (e.g. via SIB1 request) for camping on an OD-SIB1 NES cell. [Apple] We are a bit confused by this FFS. It seems that Section 5.2.2.3.3x has captured the procedure that the UE acquires a SIB1 via SIB1 request. FFS how does UE check is SIB1 is already provided. [Apple] RAN1#121b has agreed it is up to UE implementation when K_SSB>=24 for FR1 or K_SSB>=12 for FR2: Agreement If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=12 for FR2, select the following: - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission For the other case (i.e. when K_SSB<24 for FR1 or K_SSB<12 for FR2), it is legacy UE behaviour on monitoring CD-SSB and no need of new specification. Thus, we think it is sufficient to add the following in Section 5.2.2.3.3x, and remove EN. “NOTE: It is up to UE implementation on how to check SIB1 is being broadcasted.” FFS: if there is need to emphasize it is normal uplink [Apple] As it is same as legacy text in 38,331, we suggest: - Keep the current text in running RRC CR and remove EN. - Whether to support SUL in OD-SIB1 is a separate discussion, which can be company contribution driven. FFS: how to capture that a CONNECTED MODE UE supporting OD-SIB1 who is in a cell that does not broadcast SIB1, understands that the stored SIB1 is the latest SIB1. [Apple] See A004 |  |

# 4	RRC CR

| Company | Detailed comments RRC CR | Rapporteur response |
| --- | --- | --- |
| Apple A001 | Where: 5.2.2.1 General UE requirements: and SIBxx (if UE is supporting OD-SIB1). The in RRC_CONNECTED shall ensure having a valid version of SIBxx (if UE is supporting OD-SIB1) Issue: - Some typos in above text. - We understand OD-SIB1 UE in all RRC state has this requirement (rather than only RRC_CONNECTED). Suggested change: and SIBxx (if UE is supporting OD-SIB1). The UE supporting OD-SIB1 in RRC_CONNECTED shall ensure having a valid version of SIBxx. Nokia: Seems legit proposal to us |  |
| A002 | Where: ##### 5.2.2.2.2 SI change indication and PWS notification UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion exept those for paging adaptation at least once per modification period if the UE is provided with common search space ETWS or CMAS capable UEs in RRC_CONNECTED shall monitor for indication about PWS notification in any paging occasion exept those only for paging adaptation at least once every defaultPagingCycle if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging. Issue: We think the above highlighted text is not clear because paging adaptation is a NES technique rather than indicating some specific paging occasion. Suggested change: We think it is sufficient to add the following simple clarification text in section 5.2.2.2.2 or in 38.300: “Paging adaptation is not supported for the UE in RRC_CONNECTED.” Nokia: Stage-2 is not full requirement so not sure if that would be enough. If there is no nclear requirement for UE then NW cannot utilize this. So maybe we should in fact refer to actual IE to make it clear e.g. “ except the ones configured in pgingAdaptation-NS and pagingAdaptationNAndPagingFrameOffset” if possible one could consider adding a IE pagingAdapation-r19 which contain above two listed parameters to simplify the wording. |  |
| A003 | Where: Section 6.3.1 sib1-rsrp-ThresholdSSB L1-RSRP threshold used for determining whether a candidate beam may be used by the UE to attempt to transmit OD-SIB1 request, see TS XXXXX Issue: It is sufficient to refer to 38.321. Suggested change: sib1-rsrp-ThresholdSSB L1-RSRP threshold used for determining whether a candidate beam may be used by the UE to attempt to transmit OD-SIB1 request, see TS 38.321 [3]. XXXXX |  |
| A004 | Where: Section 6.3.2 si-BroadcastStatus Indicates if the SI message is being broadcasted or not. Change of si-BroadcastStatus should not result in system information change notifications in Short Message transmitted with P-RNTI over DCI (see clause 6.5). The value of the indication is valid until the end of the BCCH modification period when set to broadcasting. When SIB19 is scheduled in an NTN cell, the si-BroadcastStatus for the mapped SIB19 is set to broadcasting. When SIB22 is scheduled in an ATG cell, the si-broadcastStatus for the mapped SIB22 is set to broadcasting. FFS: how to capture that a CONNECTED MODE UE supporting OD-SIB1 who is in a cell that does not broadcast SIB1, understands that the stored SIB1 is the latest SIB1. Issue: According to RAN2#129b agreement, we think it is sufficient to capture that “The UE supporting OD-SIB1 in RRC_CONNECTED regards the stored SIB1 is the latest SIB1”. - UE understands that the stored SIB1 is the latest SIB1. Suggested change: Indicates if the SI message is being broadcasted or not. Change of si-BroadcastStatus should not result in system information change notifications in Short Message transmitted with P-RNTI over DCI (see clause 6.5). The value of the indication is valid until the end of the BCCH modification period when set to broadcasting. When SIB19 is scheduled in an NTN cell, the si-BroadcastStatus for the mapped SIB19 is set to broadcasting. When SIB22 is scheduled in an ATG cell, the si-broadcastStatus for the mapped SIB22 is set to broadcasting. FFS: how to capture that a CONNECTED MODE UE supporting OD-SIB1 who is in a cell that does not broadcast SIB1, understands that the stored SIB1 is the latest SIB1. The UE supporting OD-SIB1 in RRC_CONNECTED regards the stored SIB1 is the latest SIB1. Nokia: Maybe we need nothing for this. What else can UE do than consider latest one valid? So likely we don’t need to capture anything on this. I would be fine to just remove FFS. |  |
| OPPO001 | In 5.2.2.1, The in RRC_CONNECTED shall ensure having a valid version of SIBxx (if UE is supporting OD-SIB1) [OPPO] The sentence is not completed. But even if adding UE here, the intention seems not aligned with 129b conclusion, where it is to rely on *NW* to ensure the validity rather than UE, for RRC_CONNECTED state. => NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1. Nokia: we don’t write NW specification but UE specification in stage-3. Only thing we need to define is UE behaviour. If we don’t capture anything then it is clear that NW needs to update SIB1 to UEs. No need to capture anything. |  |
| OPPO002 | In 5.2.2.3.3x For the deletion of “immediately” [OPPO] we share the concern from RRC Rapp, since the word is used in other places, so the deletion may cause the misunderstanding that there is a difference between SIB1 acquisition and other cases. Nokia: After further thinking we are OK either way – For some reason we use word immediately for SIB acquisition. Not sure why though. So maybe better to keep immediately for now to aligne with legacy text. |  |
| OPPO003 | In the FD below totalNumberOfRA-Preambles Total number of preambles used for contention based and contention free 4-step or 2-step random access in the RACH resources defined in RACH-ConfigCommon, excluding preambles used for other purposes (e.g. for SI request). If the field is absent, all 64 preambles are available for RA. [OPPO] can we extend the “(e.g. for SI request)”, to cover SIB1 acquisition here. [OPPO] is it really possible for the value here to use 64 “ If the field is absent, all 64 preambles are available for RA.”, which means no preamble left for SIB1 acquisition? |  |
| OPPO004 | In the condition below FR2-Only This field is mandatory present for an FR2 carrier frequency. It is absent otherwise and UE releases any configured value . [OPPO] If the field is either mandatory present, or always absent, why there is a case that a value was configured but now absent? And if there is, is the intention to say it is need-R? |  |
| OPPO005 | In the FD below pagingAdaptationPEI-Config The PEI related configuration for paging adaptation. The UE supporting paging adapdation ignores field pei-Config, if configured. [OPPO] here “if configured”, is to say the new PEI configuration is configured, but not the legacy pei-Config is configured (?), if so, good to clarify to avoid misunderstanding. |  |
| Nokia001 | Editorial – several places exept=>except |  |
| Nokia002 | 5.2.2.3.3x – In 38.304 there is also failure if UE cannot acquire during the SI windows. Should we remove it from 38.304 as I guess it is covered by last two bullets in this section i.e. general failure to acquire SIB1. That seems to work to us. |  |
| Nokia003 | Field description of odsib1-cellReselectionPriority, odsib1-cellReselectionSubPriority Maybe align with excluded cell list to clarify these are applicable only for UE supporting OD-SIB1 |  |
| Nokia004 | Field description of pagingAdaptation parameters. For PEI parameter do we need to highlight UE supporting both PEI and OD-SIB1? And similarly for NS/N/frameoffset clarify UE supportin OD-SIB1 usese these if configured? Would it make sense to have all parameter in one IE pagingAdapation-r19? |  |

# 5	Conclusion