3GPP TSG-RAN WG2 Meeting #129bis						[Draft] R2-250xxxx
Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.5.4

Source:	OPPO

Title:	Summary of [122]

Document for:	Discussion, Decision

- Introduction

This is to discuss the offline as follows.

[POST129b][122][NES] (OPPO)

Scope: Discuss and make conclusions on proposal 1 in R2-2501817.

Intended outcome: Discussion summary.

Deadline: Long email discussion.

- Discussion

In this section, the defined scope of the post email discussion is further expanded to dig into details.

In the current RACH framework, CFRA could be initiated by a PDCCH order, by the MAC entity itself, or by RRC signalling. For different CFRA cases, their CFRA resources may be configured differently, i.e.,

1)	CFRA for additional PCI initiated by PDCCH order: the CFRA resources are configured by rach-configGeneric from additionalRACH-perPCI-ToAddModList-r18, which is mandatory present.

For this case, there is a single mandatory rach-configGeneric to refer to, so if one wants to enable the usage of RACH adaptation for this case, network can simply set rach-configGeneric to be the additional RACH resource.

And since it is based on PDCCH order, network can ensure the PDCCH order is sent during the period when the additional RACH is activated, so no need to concern the case when the additional RACH resources are (de)activated.

Q1: For CFRA for additional PCI initiated by PDCCH order, what is your preference

- R2 does not consider the support of RACH adaptation in this case;

- R2 assume the support of RACH adaptation in this case can be done by network implementation without spec impact;

- R2 aims at supporting RACH adaptation in this case, and spec impact is foreseen (if this option is selected, please indicate what the spec impact is)

| Company | Option (A/B/C) | Comment |
| --- | --- | --- |
| OPPO | A or B |  |
| Xiaomi | B |  |
| Samsung | A | Same as legacy behaviour. No new enhancement is needed for CFRA. |
| Nokia | A | The framework with DCI dynamic activation wouldn’t work since the UE does not know non-serving cell status. |
| Lenovo | A or B |  |

2)	CFRA for L3 HO initiated by RRC signalling (via RRCReconfiguration with ReconfigurationWithSync): the CFRA resources are configured by rach-configGeneric from the rach-ConfigDedicated (if provided, otherwise from rach-configCommon) of target cell.

3)	CFRA for LTM cell switch initiated by LTM Cell Switch Command MAC CE: the CFRA resources are configured by rach-configGeneric from the rach-ConfigDedicated (if provided, otherwise from rach-configCommon) of each LTM candidate cell.

4)	CFRA for LTM early sync initiated by PDCCH order: the CFRA resources are configured by rach-configGeneric from EarlyUL-SyncConfig of each LTM candidate cell, which is mandatory present.

For the case 2) and 3), there is a single optional rach-configGeneric to refer to, so if one wants to enable the usage of RACH adaptation for this case, network can simply set rach-configGeneric to be the additional RACH resource.

While for case 4), the CFRA resources are configured by rach-configGeneric from EarlyUL-SyncConfig of each LTM candidate cell, which is mandatory present. And thus also network can simply set rach-configGeneric to be the additional RACH resource.

And for the LTM cases (for both early UL sync and CSC MAC-CE), due to the uncertainty of the RACH timing, inter-node signaling is necessary (and thus R3 impact) in order for source node to indicate the additional RACH resource, when it is available at target node side.

Q2a: For CFRA for L3 HO initiated by RRC signalling (via RRCReconfiguration with ReconfigurationWithSync), what is your preference

- R2 does not consider the support of RACH adaptation in this case;

- R2 assume the support of RACH adaptation in this case can be done by network implementation without spec impact;

- R2 aims at supporting RACH adaptation in this case, and spec impact is foreseen (if this option is selected, please indicate what the spec impact is)

| Company | Option (A/B/C) | Comment |
| --- | --- | --- |
| OPPO | A or B |  |
| Xiaomi | B |  |
| Samsung | A | Same as legacy behaviour. No new enhancement is needed for CFRA. |
| Nokia | A or B | For HO case, if configured in HO command then it is assumed to be usable for the RA procedure to target cell. Overlapped resource validation defined for normal case CBRA can be applied without further impact. [Rapp] After offline with Nokia, the intention of “Overlapped resource validation” is to say that if a same rach-ConfigurGeneric contains configuration for two RACH resources, e.g., legacy and additional RACH, the selection between the two follows the behavior for CBRA case. While Rapp understand there are another assumption that a single rach-ConfigurGeneric would contain a single RACH resource, either legacy or the additional one. So there might be different interpretation/view in the two directions. |
| Lenovo | A or B | Our interpretation to use rach-ConfigurGeneric is like Rapp i.e., the network can include additional resource in rach-ConfigurGeneric if it wants to. |

Q2b: For CFRA for LTM cell switch initiated by LTM Cell Switch Command MAC CE, what is your preference

- R2 does not consider the support of RACH adaptation in this case;

- R2 assume the support of RACH adaptation in this case can be done by network implementation without spec impact;

- R2 aims at supporting RACH adaptation in this case, and spec impact is foreseen (if this option is selected, please indicate what the spec impact is)

| Company | Option (A/B/C) | Comment |
| --- | --- | --- |
| OPPO | A or B | If companies have a concern on the sync between source and target node regarding the additional RACH (de)activation status, we are surely fine to limit to option-A. |
| Xiaomi | B | For the A/D sync issue, if additional PRACH is provided as CFRA resource, the target cell should ensure the additional PRACH is activated. Even if the additional PRACH becomes deactivated afterwards, the target cell can update the CFRA resource or release the target cell from the candidate cell for LTM if needed based on the existing procedure. |
| Samsung | A | Same as legacy behaviour. No new enhancement is needed for CFRA. |
| Nokia | A | Considering the difference from L3 HO and pre-configuration of multiple candidate cells, thus no T304 timer for LTM like for L3 HO with which NW knows for how long it needs assume those resources might be used. |
| Lenovo | A |  |

Q2c: For CFRA for LTM early sync initiated by PDCCH order, what is your preference

- R2 does not consider the support of RACH adaptation in this case;

- R2 assume the support of RACH adaptation in this case can be done by network implementation without spec impact;

- R2 aims at supporting RACH adaptation in this case, and spec impact is foreseen (if this option is selected, please indicate what the spec impact is)

| Company | Option (A/B/C) | Comment |
| --- | --- | --- |
| OPPO | A or B | If companies have a concern on the sync between source and target node regarding the additional RACH (de)activation status, we are surely fine to limit to option-A. |
| Xiaomi | B | Similar comment as above |
| Samsung | A | Same as legacy behaviour. No new enhancement is needed for CFRA. |
| Nokia | A | Same as above. |
| Lenovo | A |  |

5)	CFRA for BFR initiated by MAC entity itself: the CFRA resources are configured by rach-configGeneric from BeamFailureRecoveryConfig.

For the case here, there is a single optional rach-configGeneric to refer to, so if one wants to enable the usage of RACH adaptation for this case, network can simply set rach-configGeneric to be the additional RACH resource.

The concern here yet is since network cannot know when the BFR is initiated, while the rach-configGeneric is provided statically, there might be a case where the additional RACH (provided via rach-configGeneric) is deactivated, yet the UE initiated the BFR procedure. So from some companies perspective, spec impact is foreseen to handle this case. While there is also company(ies) thinking that when additional RACH is configured to BFR, there is no need to be further dependent on the additional RACH (de)activation status as indicated in DCI 1_0 with P-RNTI.

Q3: For CFRA for BFR initiated by MAC entity itself, what is your preference

- R2 does not consider the support of RACH adaptation in this case;

- R2 assume the support of RACH adaptation in this case can be done by network implementation without spec impact;

- R2 aims at supporting RACH adaptation in this case, and spec impact is foreseen (if this option is selected, please indicate what the spec impact is)

| Company | Option (A/B/C) | Comment |
| --- | --- | --- |
| OPPO | A or B | For B, our understanding is that when additional RACH is configured to BFR via rach-configGeneric, no dependency on the additional RACH (de)activation status as indicated by P-RNTI based DCI, and thus the UE behavior is still the same as in legacy. |
| Xiaomi | B | We think additional PRACH config is configured via SIB1 but should be activated by DCI as agreed by RAN1. It cannot be assumed as always activated after being configured. But we think one NW implementation solution is if NW configures additional PRACH for BFR, NW should always ensure the additional PRACH as activated. Or if the NW can not always ensure the additional PRACH as activated, NW does not configure the additional PRACH for BFR. Generally, we see no spec impact, we can leave this to NW implementation. |
| Samsung | - | No new enhancement is needed if rach-configGeneric is configured in beam failure recovery configuration. If rach-configGeneric is not configured in beam failure recovery configuration, UE will apply rach-configcommon in active UL BWP. If this rach-configcommon includes additional ROs, UE will use or not use them based on whether they are activated or not according to DCI addressed to P-RNTI. This is same as any other CBRA procedure. |
| Nokia | B | Additional resource can be configured as CFRA resource for BFR and only usable if activated as the UE knows the activation status of the additional resource. Activation/deactivation follows current DCI mechanism without additional impact. [Rapp] After offline with Nokia, the intention is to say that if a same rach-ConfigurGeneric contains configuration for two RACH resources, e.g., legacy and additional RACH, the UE is to select resource - Either from both RACH resources, if DCI indicates that the additional RACH is available - Or from legacy RACH resource only, if DCI indicates that the additional RACH is not available Similar to Q2a, while Rapp understand there are another assumption that a single rach-ConfigurGeneric would contain a single RACH resource, either legacy or the additional one. So there might be different interpretation/view in the two directions. |
| Lenovo | B | This is the “source” side case and like SS indicated, DCI based activation can be used to control if additional PRACH resources will be used in the source side including in the BFR scenario. |

During online/offline, some company(ies) raised the issue to further check the applicability of normal PDCCH order, i.e., CFRA only, or CBRA as well.

6. RACH Adaptation for CFRA

R2-2501817	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: R2 confirms time-domain RACH adaptation is supported for CFRA initiated by normal PDCCH order, but not for other CFRA cases. Send LS to R1 if any concern.

[Nokia]: Any technical reason not to apply PDCCH order based CFRA RACH adaptation to other use case? [OPPO]: For example, BFR, RACH configuration is presented as mandatory configuration. And if needed, NW can put additional RACH RO into this configuration. [Samsung, Apple]: Agree with OPPO proposal. And note RAN1 decided that additional RO is only applicable to initial BWP. [Huawei]: Understand RAN1 introduced PDCCH order can be applicable to CBRA also. [Apple]: Understand there is no restriction that PDCCH order is applied to CBRA. [Ericsson]: Not sure if new mechanism for additional RO cannot be used for other cases. [Nokia]: Understand the additional RO activation/deactivation still can be applicable e.g. into BFR case. [OPPO]: Understand without additional RO activation/deactivation, if network wants, the network just includes additional RO into BFR configuration, then the UE uses it unless it is reconfigured or released. [Nokia]: Can we try to agree no additional mechanism is needed for BFR case in order to use additional RO. [Samsung]: For any kind of HO mechanisms, RACH generic is optional and if network wants to use additional RACH RO, network can configure the additional RACH RO to RACH generic configuration. Dynamic activation/deactivation is based on the serving cell’s short message. It seems clear no need of dynamic activation/deactivation to the non-serving cell(s). [Spreadtrum]: Wonder how it works if multiple initial BWPs are configured. [OPPO]: Propose to have post email discussion on this issue.

Q4: For the RAN1 agreed 1-bit indication in DCI 1_0 for C-RNTI, i.e., PDCCH order, what is your view for the applicability?

- CFRA only

- CFRA and CBRA.

| Company | Option (A/B/C) | Comment |
| --- | --- | --- |
| OPPO | B | Based on our internal check with R1 colleagues. |
| Xiaomi | B | Same understanding from our RAN1 |
| Samsung | B |  |
| Nokia | B |  |
| Lenovo | B |  |

- Conclusion

Based on the offline, we reached the following WF