3GPP TSG-RAN WG2 Meeting #129bis		R2-250xxxx

Wuhan, China, 7th – 11th April 2025

Title:	[Draft] LS on LP-WUS subgrouping progress

Response to:	-

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	NEC [to be RAN2]

To:	RAN3

Cc: 	SA2, SA3

- Contact Person:

- Name:	                Rao Shi

E-mail Address:	 shi_rao@nec.cn

- 1. Overall Description:

RAN2 has discussed LP-WUS subgrouping and made the following agreements:

LP-WUS UE-ID based subgrouping formula (RAN2#129):

- For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is related to 5G-S-TMSI,

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

UE-ID calculation (RAN2#129bis):

- Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID=5G-S-TMSI mod X.

- X is based on 32 subgrouping number. Details can be discussed in the running CR.

LP-WUS with eDRX case (RAN2#129bis):

- LP-WUS is supported with eDRX, FFS on exact impact if any

.

For more clarification, as for the design on specific value of X, there may or may not be multiple values of X considering different case (e.g., differentiation on eDRX and/or PEI). But it is clear that the maximum value of X is 1048576 (i.e., 20 bits).

RAN2 assumes that the above information is necessary for RAN3 specification work (e.g., providing additional bits of the UE identity index value) and would like to request RAN3 to take them into account.

- 2. Actions:

- To RAN WG3:

- RAN2 respectfully requests RAN3 to take the above information into account during the future work, and provide feedback, if any.

- 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #130		19th May – 23th May 2025					St. Julian’s, Malta

TSG RAN WG2 Meeting #131		25th Aug – 29th Aug 2025					Bangalore, India