3GPP TSG-RAN WG2 Meeting #130	R2-25xxxxx

Malta, MT, 19 – 23 May 2025

Agenda Item:	8.13.3

Source:	Rapporteur(ZTE)

Title:	Report of [Post129bis][412][Relay] FFS issues on system information

Document for: Discussion and Decision

# Introduction

This contribution gives the discussion summary of following post email discussion.

- [Post129bis][412][Relay] FFS issues on system information (ZTE)

Scope: Discuss the identified FFS issues on system information (proposals flagged for treatment and not treated in RAN2#129bis) and converge where possible.

Intended outcome: Report to next meeting

Deadline: Long

## Contact information

| Company | Name (Email) |
| --- | --- |
| OPPO | lengbingxue@oppo.com |
| CATT | xuhao@catt.cn |
| LG | seoyoung.back@lge.com |
| vivo | liangjing@vivo.com |
| Huawei, HiSilicon | jagdeep.singh6@huawei.com |
| Samsung | ww1016.wang@samsung.com |
| ZTE | du.weiqiang2@zte.com.cn |

# FFS issue list

In [1][2], following FFS issues are identified:

The intermediate UE, when it receives SI from a parent relay, forwards the SI message only to the child UEs which requested the SI (i.e., the intermediate UE keeps track of the required SI for each child node).  FFS on the need for including a “remote UE ID” in the request.

The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list.  FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain the SI required by it or requested by the remote UE by requesting SI from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink).  FFS if it can also receive it directly from SIB broadcast by the last relay UE’s serving cell on Uu (when the intermediate relay UE is in coverage of the same cell that serves the last relay UE).

The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay):

•	upon reception of new/changed required SI received from a remote UE/child relay UE (where the concerned SI was not previously requested)

•	FFS when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay.

The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node:

a)	Upon reception of SI received from intermediate relay or last relay containing SI requested by a child node (intermediate relay or remote UE)

b)	Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE)

c)	Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE)

d)	Upon reception of SIB1 received from a parent relay (i.e., this case may correspond to SIB1 update detected by the last relay, or unsolicited SIB1 forwarding by the last relay)

e)	Upon receiving updated SIB1 from the network (as in Rel17)

f)	Upon unsolicited SIB1 forwarding to a connected child node (intermediate relay UE or remote UE)

FFS if b), c), and e) are limited to RRC_CONNECTED relay UE or apply in all states.  FFS whether to consolidate some of the conditions in stage 3 specification text.

# Discussion

Except the “remote UE ID” FFS issue, rapporteur think the FFS issues have dependence with each other, so the FFS issues are organized as below:

We first check whether IDLE/INACTIVE intermediate relay UE is allowed to obtain the SIB requested by child UE via Uu interface directly in clause 1.2. If it is allowed, check whether IDLE/INACTIVE intermediate relay UE can respond the SIB request upon SIB is obtained via Uu interface directly or unsolicited SIB1 forwarding in clause 1.3. After that, whether SIB request in parent link is initiated always is discussed in clause 1.4. At last, clarify the UE behaviour of when there is a change in the ability on SIB reception over Uu interface in clause 1.5.

Additionally, rapporteur believe the specification change may influence the companies’ view, e.g. companies may select the option that has no or less specification change, therefore we also check companies view on the possible specification change.

## FFS issue-1

The first FFS is:

The intermediate UE, when it receives SI from a parent relay, forwards the SI message only to the child UEs which requested the SI (i.e., the intermediate UE keeps track of the required SI for each child node).  FFS on the need for including a “remote UE ID” in the request.

Rapporteur think the motivation of including the remote UE ID in SIB request/delivery message is to allow intermediate relay UE to not keep track of the required SI for each child node, instead, relying on remote UE ID included in the RemoteUEInformationSidelink and  UuMessageTransferSidelink to do the routing.

In legacy, RemoteUEInformationSidelink is transmitted via SL-SRB3 which is a unicast signaling, i.e. it is transmitted per PC5 RRC connection. The relay UE will associate the RemoteUEInformationSidelink to corresponding PC5 RRC connection identified by Remote UE’s L2 ID, store store [remote UE L2 ID, requested SIB list], respond the request according to the stored information.

For multi-hop relay, after receiving the RemoteUEInformationSidelink from child UE, the candidate intermediate UE behaviour can be:

- Option1: intermediate relay UE associate the RemoteUEInformationSidelink to corresponding PC5 RRC connection identified by child UE’s L2 ID, and store [child UE L2 ID, requested SIB list]. And forward the RemoteUEInformationSidelink to parent UE. After receiving the UuMessageTransferSidelink from parent UE, the intermediate relay UE take out the SIBs requested by child UE according to the the stored information and forward the requested SIBs to child UE.

- Option2: RemoteUEInformationSidelink and UuMessageTransferSidelink include the remote UE ID who request the corresponding SIB.  Upon receiving the RemoteUEInformationSidelink, intermediate relay UE does not store the [remote UE L2 ID, requested SIB list],  just forward the RemoteUEInformationSidelink to parent UE. After receiving the UuMessageTransferSidelink from parent UE, intermediate relay UE forward the SIB information to concerned child UE identified by remote UE ID included in UuMessageTransferSidelink, and may also need to encapsulate remote UE ID in UuMessageTransferSidelink.

Option1 has been supported per RAN2 agreement. Regarding option2, rapporteur think many details need to be further clarified and studied, e.g. who (remote UE or first intermediate relay UE) will put the “remote UE ID” into RemoteUEInformationSidelink? Does first intermediate relay UE need to change remote UE’s L2 ID in SIB request to it’s own L2 ID, since second intermediate relay can not recognize the remote UE’s L2 ID.

Question 1:	Do you agree remote UE ID need to be included in the SIB request or SIB delivery, i.e. RemoteUEInformationSidelink and UuMessageTransferSidelink?

If agree, please clarify the detailed remote/intermediate/last relay UE’s behaviour.

| Company | Agree) or Disagree) | Comments |
| --- | --- | --- |
| OPPO | Disagree | With the agreed tree-topology, there is nothing different compared to single-hop U2N Relay case. So no need for the additional enhancement. |
| NEC | Agree | Putting the “remote UE ID” into message e.g. RemoteUEInformationSidelink can help the Intermediate Relay node to route the message along with the transmission path from the source node to the destination node. |
| Ericsson | disagree | As Rapp described option 1 is already sufficient. |
| CATT | Disagree | Fail to see the need to further discuss option2. In other word, is there any issue identified with option1 for multi-hop U2N relay? Same view as OPPO and Ericsson. |
| Sharp | Disagree | Internal routing table is enough (no enhancement of message is needed). We shared the details in R2-2502379. |
| LG | Disagree | We think option 1 is enough. |
| vivo | Disagree | We don’t see the real motivation for the enhancement and share the view that option-1 can work well. |
| Lenovo | Disagree | Option 1 is sufficient. |
| Huawei, HiSilicon | Disagree | We don’t think Option 2 is needed. Option 1 is sufficient. |
| Samsung | Disagree | Option 1 is enough. |
| ZTE | Disagree |  |

## FFS issue-2

It’s FFS that:

When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain the SI required by it or requested by the remote UE by requesting SI from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink).  FFS if it can also receive it directly from SIB broadcast by the last relay UE’s serving cell on Uu (when the intermediate relay UE is in coverage of the same cell that serves the last relay UE).

In current 38.300, it is clarified that:

The in-coverage L2 U2N Remote UE is allowed to acquire any necessary SIB(s) over Uu interface irrespective of its PC5 connection to L2 U2N Relay UE. The L2 U2N Remote UE can also receive the system information from the L2 U2N Relay UE after PC5 connection establishment with L2 U2N Relay UE.

In current 38.331, it is specified that:

When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE shall: 1> if the UE has SIB request information to provide (e.g. the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the L2 U2N Relay UE before): 2> include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s);

Rapporteur think based on above description, IC remote UE in IDLE/INACTIVE state is allowed to acquire SIB via either Uu interface or U2N relay UE or both. And there is no text to describe whether IC remote UE needs to identify the SIB received from Uu interface and relay UE belongs to same cell, it’s totally up to remote UE’s implementation.

Regarding IC intermediate relay UE in IDLE/INACTIVE,  for it’s own concerned SIB, legacy principle can be re-used, i.e. allow it to acquire SIB via either Uu interface or U2N relay UE, which is left to UE implementation.

Question 2.1:	Do you agree that, in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire SIB broadcast by the last relay UE’s serving cell via Uu interface directly for it’s own concerned SIB as legacy. Possible specification change can be:

The in-coverage L2 U2N Remote UE/intermediate relay UE is allowed to acquire any necessary SIB(s) over Uu interface irrespective of its PC5 connection to L2 U2N Relay UE.

| Company | Agree) or Disagree) | Comments |
| --- | --- | --- |
| OPPO | Agree |  |
| NEC | Agree |  |
| Ericsson | agree |  |
| CATT | Agree with the intention, but | In Q2.1, the object we discussed is for in-coverage intermediate relay UE(IDLE/INACTIVE)’s own concerned SIB. But in the possible specification change listed, the wording “is allowed to acquire any necessary SIB(s)” is not match well with what we discussed. |
| Sharp | Agree |  |
| LG | Agree |  |
| vivo | Agree | It is fine as it is left to UE implementation as legacy. |
| Lenovo | Agree |  |
| Huawei, HiSilicon | Agree | In Rel-17, the remote UE is able to receive the SI on the Uu based on UE implementation if the Uu is available. Similarly, the intermediate relay UE can act like a remote UE, i.e., receiving the SI form the parent UE as well as the Uu broadcasted SI if the Uu is available. |
| Samsung | Agree |  |
| ZTE | Agree |  |

Another issue is whether IC intermediate relay UE is allowed to acquire SIB required by child UE via Uu interface directly. This issue is related to UE behaviour after reception of RemoteUEInformationSidelink including the sl-RequestedSIB-List. If it is allowed, according to current specification, the intermediate relay UE may need to perform acquisition of corresponding SIB. Rapporteur think this depends on whether the SIB requested by remote UE is also the concerned SIB of intermediate relay UE.

Question 2.2:	For the SIB requested by child UE, if it is also the concerned SIB of intermediate relay UE, if in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly, for specification impact, which option is preferred:

- Option1: No specification impact, leave it to UE implementation or it has been supported by current specification since the SIB requested by remote UE is also the concerned SIB of intermediate relay UE.

- Option2: Clarified in 300, possible wording can be :“For SIB requested by child UE, if it is also the concerned SIB of intermediate relay UE, in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly”

- Option3: Specified in 331, possible specification change can be, green part is common specification change of option3 in Q2.2&2.4:

The L2 U2N Relay UE or intermediate relay UE shall: ..... 1> if the RemoteUEInformationSidelink includes the sl-RequestedSIB-List: 2> if the sl-RequestedSIB-List is set to setup: 3> if the L2 U2N Relay UE has not stored a valid version of SIB(s) indicated in sl-RequestedSIB-List , and if the intermediate relay UE is in coverage of the last relay UE’s serving cell, and if the SIB indicated in sl-RequestedSIB-List is also the concerned SIB of intermediate relay UE: 4> perform acquisition of the system information indicated in sl-RequestedSIB-List in accordance with 5.2.2;

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option1 | No further specification impact |
| NEC | Option 2 |  |
| Ericsson | Option 1 | Agree with OPPO, if the Request SIB is also concerning the intermediate relay UE itself, we can assume it is sufficient to reuse the remote UE behavior in Rel-17, it seems unnecessary to introduce any spec change. |
| CATT | Option1 |  |
| Sharp | Option 1 | Option 2 is ok if it’s majority opinion. |
| LG | Option 1 | We think it’s an intermediate UE implementation. No spec impact is expected. |
| vivo | Option 2 or option 3 | We understand the condition-1: In coverage, condition-2: same cell of last relay, are necessary anyway. We slightly prefer to capture that, either in 300 or 331. |
| Lenovo | Option 2 or Option 3 | We can make it clear in specification. |
| Huawei, HiSilicon | Option 1 | No further spec impact is foreseen. |
| Samsung | Option 1 |  |
| ZTE | Option1 |  |

Question 2.3:	For the SIB requested by child UE, if it is also the concerned SIB of intermediate relay UE, do you agree that in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly.

| Company | Agree) or Disagree) | Comments |
| --- | --- | --- |
| OPPO | No additional specification impact to support this | If Option-1 in Q2.2 can be agreed, we are OK to follow majority view if companies think this can already be supported with no specification impact. |
| NEC | Yes |  |
| Ericsson | agree |  |
| CATT | Agree | In case it is also the concerned SIB of intermediate relay UE for the SIB requested by child UE, to follow what we discussed in Q2.1, we think the currentspecification has already been captured.Hence, we support this function.. |
| Sharp | agree | Same view with OPPO |
| LG | agree | We already answered as option-1 in Q2.2. |
| vivo | Agree |  |
| Lenovo | Agree |  |
| Huawei, HiSilicon | Agree |  |
| Samsung | Agree |  |
| ZTE | Agree |  |

Question 2.4:	For the SIB requested by child UE, if it is NOT the concerned SIB of intermediate relay UE, if in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly, for specification impact, which option is preferred:

- Option1: No specification impact, leave it to UE implementation. Do not mandate the intermediate relay UE to perform acquisition of SI which is not the concerned SI of it self.

- Option2: Clarified in 300, possible wording can be: “For SIB requested by child UE, if it is not the concerned SIB of intermediate relay UE, in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly”

- Option3: Specified in 331, possible specification change can be, green part is common specification change of option3 in Q2.2&2.4:

The L2 U2N Relay UE or intermediate relay UE shall: ..... 1> if the RemoteUEInformationSidelink includes the sl-RequestedSIB-List: 2> if the sl-RequestedSIB-List is set to setup: 3> if the L2 U2N Relay UE has not stored a valid version of SIB(s) indicated in sl-RequestedSIB-List if the intermediate relay UE is in coverage of the last relay UE’s serving cell and if the SIB indicated in sl-RequestedSIB-List is not the concerned SIB of intermediate relay UE: 4> perform acquisition of the system information indicated in sl-RequestedSIB-List in accordance with 5.2.2;

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option1 | No further specification impact |
| NEC | Option 2 |  |
| Ericsson | Option 2 | If the requested SIB by child UE is not concerning the intermediate relay UE itself, it is beneficial/clearer to have some spec change, we prefer to have it in 38.300. |
| CATT | See comment | We do not support the function raised in question 2.4, hence we don’t think we need to further discuss the potentional specificaiton impact. |
| Sharp | Option 1 | If option 1 of Q2.2 is majority view, it is natural to go to option 1 |
| LG | Option 1 | No spec impact. We don’t think there is need to differentiate whether the request SIB from child UE is the concerned SIB of the intermediate Relay UE or not. |
| vivo | Option 2 or option 3 | Can be combined with Q2.2 using a single sentence. |
| Lenovo | Option 2 or Option 3 | We should make it clear in specification. |
| Huawei, HiSilicon | Option 1 | No further spec impacts foreseen |
| Samsung | Option 2 | Some clarifications in Stage 2 would be beneficial. The wording “is allowed to acquire” can be changed to “may” considering it is implementation issue. |
| ZTE | Option1 |  |

Question 2.5:	For the SIB requested by child UE, if it is NOT the concerned SIB of intermediate relay UE, do you agree that in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly.

| Company | Yes) or No) | Comments |
| --- | --- | --- |
| OPPO | No additional specification impact to support this | If Option-1 in Q2.4 can be agreed, we are OK to follow majority view if companies think this can already be supported with no specification impact. |
| NEC | Yes |  |
| Ericsson | Yes |  |
| CATT | Disagree | This belongs to optimization. |
| Sharp | Yes | Same view with OPPO |
| LG | Yes | Same view with OPPO |
| vivo | Yes |  |
| Lenovo | Yes | No reason to delay the SI acquisition. |
| Huawei, HiSilicon | Yes | It is upto UE intermediate relay UE’s implementation whether to monitor Uu when in coverage of the last relay UEs serving cell. This is the legacy behavior of the remote UE and there is no spec impact for this aspect. If any clarification is considered necessary we can further clarify this is 38.300 to say that that intermediate relay UE is allowed to monitor Uu interface and acquire such SIB by UE implementation. |
| Samsung | Yes |  |
| ZTE | Yes |  |

## FFS issue-3

It’s FFS that:

The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node:

a)	Upon reception of SI received from intermediate relay or last relay containing SI requested by a child node (intermediate relay or remote UE)

b)	Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE)

c)	Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE)

d)	Upon reception of SIB1 received from a parent relay (i.e., this case may correspond to SIB1 update detected by the last relay, or unsolicited SIB1 forwarding by the last relay)

e)	Upon receiving updated SIB1 from the network (as in Rel17)

f)	Upon unsolicited SIB1 forwarding to a connected child node (intermediate relay UE or remote UE)

FFS if b), c), and e) are limited to RRC_CONNECTED relay UE or apply in all states.  FFS whether to consolidate some of the conditions in stage 3 specification text.

Rapporteur think the later FFS issue should be discussed in running CR, we will not spend time on it. And the prior FFS issue is related to FFS issue-2. As long as the IDLE/INACTIVE intermediate relay UE is allowed to obtains the SIB requested by remote UE via Uu interface directly, then it is exactly b), c), and e) are applied in all RRC states.

Question 3:	Do you agree below b), c), and e) are applied also in IDLE/INACTIVE states for intermediate relay UE.

The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node:

a)	Upon reception of SI received from intermediate relay or last relay containing SI requested by a child node (intermediate relay or remote UE)

b)	Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE)

c)	Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE)

d)	Upon reception of SIB1 received from a parent relay (i.e., this case may correspond to SIB1 update detected by the last relay, or unsolicited SIB1 forwarding by the last relay)

e)	Upon receiving updated SIB1 from the network (as in Rel17)

f)	Upon unsolicited SIB1 forwarding to a connected child node (intermediate relay UE or remote UE)

| Company | Agree) or Disagree) | Comments |
| --- | --- | --- |
| OPPO | No additional specification impact to support this | We are OK to follow majority view if companies think this can already be supported with no specification impact. |
| NEC | Yes |  |
| Ericsson | Yes |  |
| CATT | See comments | Same view as OPPO. |
| Sharp | No additional specification impact |  |
| LG | No | Same view as OPPO. |
| vivo | Yes |  |
| Lenovo | Yes | If intermediate relay UE in idle/inactive state is allowed to perform SI acquisition, b,c,e should be supported. |
| Huawei, HiSilicon | No additional specification impact to support this | Same view as Oppo |
| Samsung | Yes with no spec. impact. |  |
| ZTE | Yes | Do not need to clarify the SIB is obtained via Uu interface directly or Parent UE in current specification. |

## FFS issue-4

It's FFS that:

The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list.  FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

Regarding this FFS issue, rapporteur think it is related to IDLE/INACTIVE state intermediate relay UE, since CONNECTED intermediate relay UE can use dedicated signaling to obtain the request SIB, forward the request to parent node is exactly not needed.

- Question 4.1:	Do you agree “FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.” is related to IDLE/INACTIVE state intermediate relay UE, not CONNECTED intermediate relay UE.

| Company | Agree) or Disagree) | Comments |
| --- | --- | --- |
| OPPO | No additional specification impact to support this | We are OK to follow majority view if companies think this can already be supported with no specification impact. |
| Ericsson | Agree |  |
| CATT | Agree |  |
| Sharp | Disagree | Even if the intermediate relay UE is in CONNECTED, it is reasonable to respond available SIBs directly from signaling perspective. |
| LG | Agree | We think the intermediate Relay UE can respond directly without forwarding if the intermediate Relay UE has a valid SIB. |
| vivo | Agree | We can follow majority view if we only focus on IDLE/INACTIVE |
| Lenovo | Agree |  |
| Huawei, HiSilicon | Agree | It is straightforward that the intermediate relay UE can respond directly if the intermediate relay UE also request this SI from the parent and already received the SI form the parent. No spec impacts are foreseen |
| Samsung | Agree |  |
| ZTE | Agree |  |

Regarding the FFS issue, without forwarding the request to the parent node, IDLE/INACTIVE intermediate relay UE can get the SIB via Uu interface directly or via unsolicited SIB1 forwarding. Therefore, it is suggested to discuss the issue for these two cases separately.

Additionally, the FFS issue can be separated into following two stage:

Stage1: whether intermediate relay UE can respond child UE’s SIB request if UE has the requested SI, and if the SIB has not been included in SIB request in parent link.

Stage2: whether SIB request in parent link is needed after determination of stage1.

Regarding stage1, “intermediate relay UE can respond child UE’s SIB1 request if UE has the requested SIB1 via unsolicited SIB1 forwarding” has been supported per below RAN2 agreement:

The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node:

a)	Upon reception of SI received from intermediate relay or last relay containing SI requested by a child node (intermediate relay or remote UE)

d)	Upon reception of SIB1 received from a parent relay (i.e., this case may correspond to SIB1 update detected by the last relay, or unsolicited SIB1 forwarding by the last relay)

Whether intermediate relay UE can respond child UE’s SIB request if UE has the requested SI obtained via Uu interface directly will be discussed in FFS issue-3.

With regard to Stage2, if the answer of stage1 is NO, then initiate SIB request in parent link is always needed.

Otherwise if the answer of stage1 is Yes(i.e. intermediate relay UE CAN respond child UE’s SIB1 request if UE has the requested SIB1), then the following issue is that whether SIB request in parent link is still needed:

Option1: SIB request in parent link is still needed in this case, i.e. the intermediate relay UE CAN respond directly AND forwarding the request to the parent node if it has the requested SI. That is, upon receiving the child UE’s SIB request, always forwarding the request to parent, irrespective of whether intermediate relay UE can or can not respond it directly(has or not has the requested SIB).

Option2: SIB request in parent link is not needed needed in this case, the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

Option3: leave it to UE implementation to to determine whether forwarding the SIB request to parent UE, in case the intermediate relay UE CAN respond directly.

Regarding option1 and option2, there are two ways to implement them, one is capture it as a separate bullet in clause 5.8.9.8.2, as clarified in option1a and option2a in Q4.2, another one is capture it is as an example in clause 5.8.9.8.2, as clarified in option1b and option2b in Q4.2. Please check it.

Let’s first check companies view on the delta part of specification of these two options.

- Question 4.2: For the SIB requested by child node, if the SIB has not been included in SIB request in parent link, if intermediate relay UE can obtain the requested SIB and respond the SIB request of child node, do you agree following possible specification change, detailed wording can be refined in running CR:

| Option1a | ##### 5.8.9.8.2 Actions related to transmission of RemoteUEInformationSidelink message When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE shall: 1> if the UE has SIB request information to provide (e.g. the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the L2 U2N Relay UE before), or 1> if the RemoteUEInformationSidelink is received from child node : 2> include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s); |
| --- | --- |
| Option1b | ##### 5.8.9.8.2 Actions related to transmission of RemoteUEInformationSidelink message When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE shall: 1> if the UE has SIB request information to provide (e.g. the RemoteUEInformationSidelink is received from child node, or the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the L2 U2N Relay UE before): 2> include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s); |
| Option2a | ##### 5.8.9.8.2 Actions related to transmission of RemoteUEInformationSidelink message When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE shall: 1> if the UE has SIB request information to provide (e.g. the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the L2 U2N Relay UE before), or 1> if the RemoteUEInformationSidelink is received from child node and UE has no valid version of SIB indicated in RemoteUEInformationSidelink : 2> include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s) that UE has no valid version of SIB; |
| Option2b | ##### 5.8.9.8.2 Actions related to transmission of RemoteUEInformationSidelink message When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE shall: 1> if the UE has SIB request information to provide (e.g. the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 or in accordance with clause 5.8.9.8.3, and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the L2 U2N Relay UE before): 2> include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s); |
| Option3 | No specification change, Use “if the UE has SIB request information to provide” to cover option1 and/or option2. |

| Company | Agree) or Disagree) | Comments |
| --- | --- | --- |
| OPPO | Option 2b or Option 3 are both fine for us | We are not sure about the Option-1/2 here, we understand the RemoteUEInformationSidelink transmission behavior at the intermediate relay UE is same the remote UE, i.e., triggered by SIB request change, the only difference is the SIB request change may be caused by SIB request from the child node in accordance with clause 5.8.9.8.3. Based on thie understanding, seems the following simple change is sufficient? ##### 5.8.9.8.2 Actions related to transmission of RemoteUEInformationSidelink message When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE shall: 1> if the UE has SIB request information to provide (e.g. the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 in accordance with clause 5.8.9.8.3, and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the L2 U2N Relay UE before): 2> include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s); Update of view: we are also fine with no change, i.e., option-3 |
| NEC | Option 2a |  |
| Ericsson | Option 2a | “in accordance with clause 5.8.9.83” is unclear to be referred here, better to spell out the exact details. |
| CATT | Option 3 | In case the intermediate has the valid SIB (which is not included in the request SIB list), this case is not totally new, that is to say, the currnet spec impact had already covered it. |
| Sharp | 2b or 3 | We assume that the intermediate relay UE doesn’t need to send a request to the parent UE if the intermediate relay UE already requested the SIB to the parent UE since the SIB should be available at the intermediate relay UE. It should be based on whether the SIB is already requested to the parent UE, NOT based on whether the valid version is available. For option 2a, some updates are needed: 1> if the RemoteUEInformationSidelink is received from child node and the UE has no valid version of the SIB(s) indicated in the RemoteUEInformationSidelink : 2> include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s); sl-RequesttedSIB-List is not ToAddModList. Thus, the intermediate relay UE should request merged required SIBs list. |
| LG | Option 3 |  |
| vivo | Option 3 | We prefer to adopt the simplest one, and we think it is enough. |
| Lenovo | See comments. (disagree O3) | Specification change is needed. The details can be discussed at stage 3. In general, if the intermediate relay UE can receive SIB directly, the SIB request is not forwarded to parent node. |
| Huawei, HiSilicon | Option 3 | This can be covered in the current specs. |
| Samsung | Option 3 |  |
| ZTE | Option3 |  |

If option1 is adopted, assuming that intermediate relay UE receives the SIB requested by child UE via Uu interface and then receives the same SIB from parent UE, intermediate relay UE may double-trigger the SIB delivery message for same SIB.

- Question 4.3: Do you agree “the intermediate relay UE can respond directly AND forward the request to the parent node if it has the requested SI” may cause the double-trigger of SIB delivery message(“UuMessageTransferSidelink ”)?

| Company | Agree) or Disagree) | Comments |
| --- | --- | --- |
| OPPO | See above reply |  |
| Lenovo |  | If the intermediate relay UE can respond directly, it is obvious not to forward the request. Why do we design a system with low efficiency? |

- Question 4.4: If the answer of Q4.3 is yes, do you agree it can be left to UE implementation to solve the double-trigger?

| Company | Agree) or Disagree) | Comments |
| --- | --- | --- |
| OPPO | See above reply |  |

Then, let’s check companies view on option1 and option2.

- Question 4.5: For the SIB requested by child node, if the SIB has not been included in SIB request in parent link, if intermediate relay UE obtain the requested SIB via Uu interface and CAN respond SIB request of child UE, do you agree SIB request for this SIB in parent link is still needed?

| Company | Agree) or Disagree) or Leave to UE implementation | Comments |
| --- | --- | --- |
| OPPO | See above reply |  |
| Ericsson | disagree |  |
| CATT | See comments | As we replied in Q2.5, we think the intermediate relay UE obtain the requested SIB via Uu interface (for the SIB requested by child node and the SIB has not been included in SIB request in parent link) belongs to optimaztion, we don’t support this funcation. |
| LG | disagree |  |
| vivo | Disagree | In this case it can just respond to child node. |
| Lenovo | Disagree (= no need) |  |
| Huawei, HiSilicon | See Comment | Obtaining SIBs through the parent relay UE is the primary mechanism for the intermediate relay UE. Obtaining SIB through Uu is an additional Implementation based mechanism when the intermediate relay UE is in the cell coverage. If the intermediate relay UE has the requested SI it can respond to the child UE. |
| ZTE | Agree | Our view is that we do not mandate intermediate relay UE to acquire the SIB requested by child UE, therefore the SIB request in parent link should always be supported in current specification. |

- Question 4.6: For the SIB1 requested by child node, if the SIB1 has not been included in SIB request in parent link, if intermediate relay UE obtain the requested SIB1 via unsolicited SIB1 forwarding and CAN respond SIB request of child UE, do you agree SIB request for this SIB1 in parent link is still needed?

| Company | Agree) or Disagree) or Leave to UE implementation | Comments |
| --- | --- | --- |
| OPPO | See above reply |  |
| Ericsson | disagree |  |
| CATT | See comments | Simialr as legacy, we can leave it to intermediate relay UE implementation. |
| LG | disagree | As legacy, it can be up to intermediate relay UE implementation. |
| vivo | Disagree | In this case it can just respond to child node. |
| Lenovo | Disagree (= no need) |  |
| Huawei, HiSilicon | Disagree | As legacy, it can be up left to intermediate relay UE implementation. |
| ZTE | Agree | Our view is that we do not mandate intermediate relay UE to acquire the SIB requested by child UE, therefore the SIB request in parent link should always be supported in current specification. Initiation of unsolicited SIB1 forwarding is left to UE implementation, if SIB request for SIB1 in parent link is not initiated, parent UE may not always initiate unsolicited SIB1 forwarding. |

## FFS issue-5

It’s FFS that:

The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay):

•	upon reception of new/changed required SI received from a remote UE/child relay UE (where the concerned SI was not previously requested)

•	FFS when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay.

Rapporteur think the answer of this FFS issue depends on the previous discussion and it’s about IDLE/INACTIVE intermediate relay UE behaviour. If SIB request in parent link is always initiated when intermediate relay UE respond the child UE’s SIB request upon requested SIB is obtained via Uu interface, we do not need to discuss this FFS issue.

Additionally, in legacy, remote UE can obtain the SIB via either Uu interface or relay UE or both, “when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay” is left to UE implementation.

- Question 5.1: Do you agree that this FFS issue is valid only if the intermediate relay UE can receive SIB broadcast on last relay UE’s serving cell, and can respond child UE’s SIB request without initiating a SIB request in parent link when it has the requested SIB. And it’s about IDLE/INACTIVE intermediate relay UE behaviour.

| Company | Agree) or Disagree) or Leave to UE implementation | Comments |
| --- | --- | --- |
| OPPO | No need to further diff of cases and specify “ability of the intermediate UE to receive SIB broadcast on Uu” | Based on the reply to Q 4.2, we understand the current spec on the triggering on SIB request information is quite general, i.e., if there is a Sib request to provide, this can cover all the cases. So no need to further diff the case, and also no need for further specify the “ability of the intermediate UE to receive SIB broadcast on Uu” |
| CATT | See comments | As what we discussed in the previous questions, the raised conditions are not supported from our point of view, hence, we fail to say it is valid. |
| Sharp | No need spec impact for this |  |
| LG | Leave it UE implementation. |  |
| vivo | See comments | The case is valid when the intermediate UE was previously performing the SI acquisition procedure over Uu interface (for IC to OOC), or when the intermediate UE has sent SI request to its parent relay before and determined to perform the SI acquisition procedure over Uu interface afterwards ( for OOC to IC). |
| Lenovo | Need. | moving in/out of coverage will impact it. For example, whether to transfer SIB request to parent. |
| Huawei, HiSilicon | No need to further distinguish this case. No spec impact for this. | SI forwarding based on the PC5-RRC should always be supported as a baseline, and receiving SIB broadcast on Uu can be a supplemental method, but should not impact the SI forwarding on PC5. Besides, it is the relay UE implementation whether to monitor on Uu. |
| Samsung | UE implementation |  |
| ZTE | Agree |  |

- Question 5.2: Do you agree if we support “when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay”, the possible stage3 specification change can be shown in below, which is highlighted in yellow, green part is a common specification change regard less we support or not support this UE behaviour, and please clarify any other specification impact, if any:

| Option1 | ##### 5.8.9.8.2 Actions related to transmission of RemoteUEInformationSidelink message When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE shall: 1> if the UE has SIB request information to provide (e.g. the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the L2 U2N Relay UE before), or 1> if the RemoteUEInformationSidelink is received from child node and UE is not able to receive the SIB indicated in RemoteUEInformationSidelink and broadcast on last relay UE’s serving cell(e.g. moving out of last relay UE’s serving cell’s coverage): 2> include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s) that the UE is not able to receive; When entering RRC_CONNECTED, if L2 U2N remote UE had sent sl-RequestedSIB-List, sl-RequestedPosSIB-List, and/or sl-PagingInfo-RemoteUE, the L2 U2N Remote UE shall: 1> set the sl-RequestedSIB-List to the value release if requested before; 1> set the sl-RequestedPosSIB-List to the value release if requested before; 1> set the sl-PagingInfo-RemoteUE to the value release if sent before; 1> submit the RemoteUEInformationSidelink message to lower layers for transmission; If UE is able to receive the SIB broadcast on last relay UE’s serving cell(e.g. moving in of last relay UE’s serving cell’s coverage), and if L2 intermediate relay UE had sent sl-RequestedSIB-List, the L2 intermediate relay UE shall: 1> set the sl-RequestedSIB-List to the value release if requested before; 1> submit the RemoteUEInformationSidelink message to lower layers for transmission; |
| --- | --- |
| Option2 | ##### 5.8.9.8.2 Actions related to transmission of RemoteUEInformationSidelink message When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE shall: 1> if the UE has SIB request information to provide (e.g. if the RemoteUEInformationSidelink is received from child node and UE is not able to receive the SIB indicated in RemoteUEInformationSidelink and broadcast on last relay UE’s serving cell(e.g. moving out of last relay UE’s serving cell’s coverage),the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the L2 U2N Relay UE before): 2> include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s); |
| Opiton3 | No specification change, Use “if the UE has SIB request information to provide” to cover this case. |

| Company | Agree) or Disagree) | Comments |
| --- | --- | --- |
| OPPO | No need to further diff of cases and specify “ability of the intermediate UE to receive SIB broadcast on Uu” | See reply in Q4.2 and Q5.1 |
| Ericsson | Option 3 seems sufficient to cover this case. |  |
| LG | Option 3 |  |
| vivo | Option 3 |  |
| Lenovo | See comments | How to capture can be discussed at stage 3. |
| Huawei, HiSilicon | Option 3 |  |
| Samsung | Option 3 |  |
| ZTE | Option3 |  |

- Question 5.3: Do you Agree“when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay”.

| Company | Agree) or Disagree) or Leave to UE implementation | Comments |
| --- | --- | --- |
| OPPO | Disagree | See reply in Q4.2 and Q5.1 |
| Ericsson | disagree |  |
| CATT | Disagree |  |
| Sharp | disagree |  |
| LG | Disagree |  |
| vivo | Agree with comments | Not sure the real intention of this ‘agree/disagree’. As commented in Q 5.2, seems (some) companies are ok with this proposal but just try to minimize the spec impact. Our understanding is same, this behaviour itself is OK, but we need to consider to minimize the spec impact. |
| Lenovo | Agree |  |
| Huawei, HiSilicon | Disagree |  |
| Samsung | Disagree |  |
| ZTE | Disagree |  |

# Conclusion

In this contribution, we have following proposals:

# Reference

[1] R2-2502194	Control plane and SRAP for multi-hop relay	Nokia	discussion	NR_SL_relay_multihop

[2]R2-2501853	Discussion on control plane procedures for multi-hop SL Relay	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop

# Agreements on SI：

## RAN2 128 meeting:

Agreement:

In multi-hop, the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s).  FFS how to perform the forwarding and whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE).

## RAN2 129 meeting:

Agreements:

For system information acquisition in multi-hop, the remote UE:

o	When RRC_CONNECTED, uses end-to-end RRC signaling to obtain its system information directly from its connected cell.

o	When RRC_IDLE/RRC_INACTIVE, can request SI using PC5-RRC signaling (e.g., RemoteUEInformationSidelink message)

o	When in RRC_IDLE/RRC_INACTIVE, receives the required SI from PC5-RRC signaling (e.g., UuMessageTransferSidelink)

For system information acquisition by the remote UE in multi-hop, the same triggers as Rel17 are supported for sending the PC5-RRC message (e.g., RemoteUEInformationSidelink) namely:

o	when there is a change in the required SI while in RRC_IDLE/RRC_INACTIVE, or when entering RRC_IDLE/RRC_INACTIVE

o	when it entering RRC_CONNECTED, a PC5-RRC message (e.g., RemoteUEInformationSidelink) is sent to cancel a previously sent required SI

The last relay UE in multihop can forward SI (e.g., in a UuMessageTransferSidelink) to an intermediate Relay upon:

•	acquisition of the SIB(s) requested (in a hop by hop manner) by a connected child node (intermediate node and/or remote UE, but the last relay UE is not required to determine which node originated the request)

•	reception of updates of any SIBs requested by a remote UE or another a child relay UE (in a hop-by-hop manner), including SIB1

•	deciding to perform unsolicited SIB1 forwarding

An intermediate relay UE that is RRC_CONNECTED may use end-to-end RRC signaling to obtain its system information directly from its connected cell (i.e., behaving as a remote UE).

The intermediate UE, when it receives SI from a parent relay, forwards the SI message only to the child UEs which requested the SI (i.e., the intermediate UE keeps track of the required SI for each child node).  FFS on the need for including a “remote UE ID” in the request.

The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list.  FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

Re-use RemoteUEInformationSidelink as the PC5-RRC message transmitted by the remote UE or by the intermediate relay UE to the parent node (intermediate relay or last relay) to request the required SI.

The PC5-RRC message transmitted by the last relay UE or by the intermediate relay UE that provides the SI to a child UE contains at least containers with SIB1 and other system information requested by the child UE.

Re-use UuMessageTransferSidelink as the PC5-RRC message transmitted by the Last relay or by the intermediate relay UE that provides SI to the child UE.

Agreements:

When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain the SI required by it or requested by the remote UE by requesting SI from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink).  FFS if it can also receive it directly from SIB broadcast by the last relay UE’s serving cell on Uu (when the intermediate relay UE is in coverage of the same cell that serves the last relay UE).

The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay), following legacy remote UE behaviour:

•	when there is a change in the SI required by the intermediate UE for its own use (as a remote UE)

•	when the intermediate UE enters RRC_IDLE/RRC_INACTIVE

•	when the intermediate UE enters RRC_CONNECTED (to cancel a previously sent SI request; if a child node requested the SI, the intermediate UE acquires it by dedicated signalling)

The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay):

•	upon reception of new/changed required SI received from a remote UE/child relay UE (where the concerned SI was not previously requested)

•	FFS when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay.

The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node:

a)	Upon reception of SI received from intermediate relay or last relay containing SI requested by a child node (intermediate relay or remote UE)

b)	Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE)

c)	Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE)

d)	Upon reception of SIB1 received from a parent relay (i.e., this case may correspond to SIB1 update detected by the last relay, or unsolicited SIB1 forwarding by the last relay)

e)	Upon receiving updated SIB1 from the network (as in Rel17)

f)	Upon unsolicited SIB1 forwarding to a connected child node (intermediate relay UE or remote UE)

FFS if b), c), and e) are limited to RRC_CONNECTED relay UE or apply in all states.  FFS whether to consolidate some of the conditions in stage 3 specification text.