3GPP TSG-RAN WG2 Meeting #131bis	R2-25xxxxx

Prague, Czech Republic, October 13-17, 2025

Agenda Item:	8.1.1

Source:	Qualcomm Incorporated (Rapporteur)

Title:	Remaining LPP open issues for feature "AI/ML for NR air interface"

Document for:	Discussion

# 1.	Introduction

This document summarizes the remaining open issues related to LPP for the correction phase. It is based on R2-2505702 [2], the agreements reached during RAN2#131 [4], the latest available RAN1 feature list [5], and the most recent RAN1 higher layer parameters list [6].

## References

R2-2504130, "LPP open issues for feature "AI/ML for NR air interface", Qualcomm Incorporated (Rapporteur). RAN2#130.

R2-2505702, "LPP open issues for feature "AI/ML for NR air interface", Qualcomm Incorporated (Rapporteur). RAN2#131.

R2-2506584, CR to 37.355: Introduction of AI/ML Positioning Accuracy Enhancements, Qualcomm Incorporated (Rapporteur). RAN2#131.

Chairnotes RAN2#131, RAN2 Chair (InterDigital).

R1-2506426, "LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#122".

R1-2506626, "LS on Rel-19 higher layers parameters list Post RAN1#122".

# 2.	Remaining Open issues for specification 37.355 (LPP)

The following Table is taken from Annex A in R2-2505702 [2], with updates after RAN2#131 shown with change bars.

## Summary of LPP Open Issues and Status

Please see R2-2505702 [2] for further details on each issue.

| Issue |  |  |  |  | RAN2#130 Agreement |  | Status after RAN2#130 |  | Action after RAN2#130 | RAN2#131 Updates | Action after RAN2#131 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LPP#1 | Applicability of dl-PRS-ResourcePrioritySubset | The field dl-PRS-ResourcePrioritySubset in IE NR-DL-PRS-Info should be ignored for NR AI/ML positioning. Remove corresponding 'Editor's Note' from the running CR. | Closed | Updated in _v02 of running CR | - | - |
| LPP#2 |  | Applicability of IE NR-DL-PRS-ProcessingCapability |  |  | Regarding the applicability of IE NR-DL-PRS-ProcessingCapability to NR AI/ML positioning Case 1, wait for further RAN1 input and keep the current "Editor's Note" in the running CR for now. |  | Open |  | Wait for final RAN1 feature list | The IE NR-DL-PRS-ProcessingCapability-r16 is currently included in NR-DL-AIML-ProvideCapabilities-r19. Per latest RAN1 feature list in R1-2506627 (LS in R1-2506426) most (but not all) fields are applicable to Case 1. | TP is provided in Annex A. Company input is requested in section 3.1 below. |  |
| LPP#3 |  | Applicability of NR-DL-PRS-QCL-ProcessingCapability |  |  | Regarding the applicability of IE NR-DL-PRS-QCL-ProcessingCapability to NR AI/ML positioning Case 1, wait for further RAN1 input and keep the current "Editor's Note" in the running CR for now. |  | Open |  | Wait for final RAN1 feature list | The IE NR-DL-PRS-QCL-ProcessingCapability-r16 is included in NR-DL-AIML-ProvideCapabilities-r19. Per latest RAN1 feature list in R1-2506627 (LS in R1-2506426), the existing fields are all applicable to Case 1: 58-2-5, 58-2-6. | Closed |  |
| LPP#4 |  | Applicability of NR-DL-PRS-ResourcesCapability |  |  | Regarding the applicability of IE NR-DL-PRS-ResourcesCapability to NR AI/ML positioning Case 1, wait for further RAN1 input and keep the current "Editor's Note" in the running CR for now. |  | Open |  | Wait for final RAN1 feature list | The IE NR-DL-PRS-ResourcesCapability-r16 is included in NR-DL-AIML-ProvideCapabilities-r19. Per latest RAN1 feature list in R1-2506627 (LS in R1-2506426), the existing fields are all applicable to Case 1: 58-2-3, 58-2-3a, 58-2-3b. | Closed |  |
| LPP#5 | Applicability of NR-On-Demand-DL-PRS-Configurations-Selected-IndexList | The IE NR-On-Demand-DL-PRS-Configurations-Selected-IndexList is also applicable to NR AI/ML positioning Case 1. The corresponding Editor's Notes in clause 6.4.3, 6.5.10.1, and 6.5.11.1 can be removed. | Closed | Updated in _v02 of running CR | - | - |
| LPP#6 | Applicability of NR-PRU-DL-Info | [LPP-6] The IE NR-PRU-DL-Info is also applicable to NR AI/ML positioning Case 1. The corresponding Editor's Notes in clause 6.4.3 can be removed. FFS if more PRUs are needed to be included for training purposes | Closed. The FFS is captured as new issue #6a. | Updated in _v02 of running CR | - | - |
| LPP#6a (new) |  | Number of PRUs in NR-PRU-DL-Info |  |  | The IE NR-PRU-DL-Info is also applicable to NR AI/ML positioning Case 1. The corresponding Editor's Notes in clause 6.4.3 can be removed. FFS if more PRUs are needed to be included for training purposes |  | Open (only the FFS part) |  | Company contributions (if any) | RAN2#131 Agreement: Do not introduce a request for additional PRUs (e.g., a number of PRUs) in the Request Assistance Data message. | Closed |  |
| LPP#7 | Applicability of NR-SelectedDL-PRS-IndexList | NR-SelectedDL-PRS-IndexList is applicable to AI/ML positioning Case 1. | Closed | Updated in _v02 of running CR | - | - |
| LPP#8 |  | Details of IE NR-AI-ML-PositioningProvideAssistanceData |  |  | The IE NR-AI-ML-PositioningProvideAssistanceData contains (at least) all assistance data elements from UE-based DL-TDOA as starting point. This will be revised when additional RAN1 input is available. The current Editor's Note is kept for now. [RAN1#121] For AI/ML based positioning Case 1, regarding info #7 in the assistance information from legacy UE-based DL-TDOA, it can be provided as in legacy UE-based DL-TDOA or implicitly. |  | Open |  | Wait for further RAN1 input (if any) | RAN1 parameter list in R1-2506622: LMF can provide to UE the assistance information (optional) from legacy UE-based DL-TDOA: Info #1 ~ Info #6, Info #8 ~ Info #15 in Table 8.12.2.1.0-1 in 38.305 v18.3.0. LMF can provide to UE the assistance information (optional) from legacy UE-based DL-TDOA: Info #7 in Table 8.12.2.1.0-1 in 38.305 v18.3.0. The associated ID (optional) provides implicit indication of Info #7. For given TRP(s), the same associated ID implies that geographical coordinates of the TRP(s) can be understood as consistent by the UE. The associated ID is not expected to provide the real value of Info #7 (i.e., geographical coordinates of the TRP(s) are not disclosed). An associated ID is configured per-cell (e.g., NCGI-r15). All the above is supported in current LPP. | Closed |  |
| LPP#9 |  | Details of IE NR-AI-ML-PositioningRequestAssistanceData |  |  | The IE NR-AI-ML-PositioningRequestAssistanceData contains (at least) all assistance data elements from UE-based DL-TDOA as starting point. This will be revised when additional RAN1 input is available. The current Editor's Note is kept for now. [RAN1#121] For AI/ML based positioning Case 1, regarding info #7 in the assistance information from legacy UE-based DL-TDOA, it can be provided as in legacy UE-based DL-TDOA or implicitly. |  | Open |  | Wait for further RAN1 input (if any) | IE NR-DL-AIML-RequestAssistanceData matches the IE NR-DL-AIML-ProvideAssistanceData | Closed |  |
| LPP#10 |  | Details of IE NR-AI-ML-PositioningProvideLocationInformation |  |  | The IE NR-AI-ML-PositioningProvideLocationInformation contains (at least) the time stamp for the location coordinates (which are reported in CommonIEsProvideLocationInformation). This will be revised when additional RAN1 input is available. The current Editor's Note is kept for now. |  | Open |  | Wait for further RAN1 input (if any) | No additional RAN1 input has been received (e.g., in parameter list) | Closed |  |
| LPP#10a |  | Applicability of "batch reporting" for AI/ML positioning. |  |  |  |  | Open |  | Company contributions (if any) | RAN2#131 Agreement: "Batch reporting", i.e., reporting of up to 32 location results in a single report as supported for the current NR positioning methods, is also applicable to "NR AI/ML Positioning Case 1". Completed in current LPP. | Closed |  |
| LPP#11 |  | Details of IE NR-AI-ML-PositioningRequestLocationInformation |  |  | The IE NR-AI-ML-PositioningRequestLocationInformation contains (at least) the AssistanceAvailability flag. Additional details/information can be discussed via company contributions. |  | Open |  | Company contributions (if any) | RAN2#131 Agreement: Keep NR-DL-AIML-RequestLocationInformation, excluding UE-assisted measurement parameters, and retain only UE-based and common parameters (e.g., nr-AssistanceAvailability). Completed in current LPP | Closed |  |
| LPP#12 |  | Details of IE NR-AI-ML-PositioningProvideCapabilities |  |  | The IE NR-AI-ML-PositioningProvideCapabilities contains (at least) all capabilities from UE-based DL-TDOA as starting point, except the capability related to DL-PRS processing (see #LPP-2/3/4). This will be revised when additional RAN1 input is available. The current Editor's Note is kept for now. |  | Open |  | Related to #2/3/4 | DL-PRS (and other RAN1) capabilities have been added Completed in current LPP. | Closed |  |
| LPP#13 | Location server error causes | Reuse the existing NR-DL-TDOA-LocationServerErrorCauses structure for AI/ML positioning Case 1, and do not introduce additional error causes in NR-DL-AI-ML-LocationServerErrorCauses. | Closed | No additional LPP impacts. | - | - |
| LPP#14 |  | Target device error causes |  |  | Introduce ‘DL AIML positioning not available’ as new target device error cause for AI/ML positioning case 1, to indicate UE cannot perform positioning method (e.g. model not available and performance monitoring outcome not available). |  | Closed |  | Updated in _v02 of running CR. | RAN2#131 Agreement: We do not introduce new error cause for the target device error causes. | Closed |  |
| LPP#15 | Applicability of Positioning Integrity to AI/ML positioning | (LPP-15) positioning Integrity is supported for AI/ML positioning Case 1 | Closed | No additional LPP impacts. | - | - |
| LPP#16 | Signalling of Monitoring Outcome | No new LPP message is introduced for performance monitoring purposes | Closed | No additional LPP impacts. | - | - |
| LPP#17 | Signalling of "ground-truth label" information | (LPP-17): A target UE can obtain the "ground-truth label" information via existing MO-LR procedures. No additional RAN2 specification impacts are foreseen | Closed | No additional LPP impacts. | - | - |
| LPP#18 (new) |  | Consistency between training and inference |  |  | To ensure the consistency between training and inference, the UE should be able to request assistance data associated with a specific group of TRPs. FFS the request associated information in on demand prs request. |  | Open |  | Company contributions (if any) | RAN2#131 Agreement: Introduce list of global cell information (i.e., NCGIs, or PCIs with ARFCN) and TRP ID, as the request associated information to ensure consistency between training and inference. The UE asks specific TRPs for PRS transmission with on-demand PRS configuration, i.e., within NR-On-Demand-DL-PRS-Request. Completed in current LPP. | Closed |  |
| LPP#19 (new) |  | Applicability of BM related agreements |  |  |  |  | Open |  | Company contributions (if any) | RAN2#131 Agreement: Similar to BM, UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. If nw side additional conditions are not provided then we follow BM conclusion. No stage 3 impacts. Similar to AI PHY, when applicability changes the UE should report this to the LMF and only what changed. For now capture this at least in stage 2. Check offline if and how this would be implemented in stage 3. | Closed (captured in Stage 2) |  |
| LPP#20 (new) |  | LPP Impacts related to Case 3a/3b |  |  |  |  | Open |  | Company contributions (if any) | RAN2#131 Agreement: Case 3a and Case 3b can be supported without new impact to LPP. | Closed |  |
| LPP#21 (new) |  | "Associated ID" for TRP Location Coordinates (IE TRP-ImplicitLocationInfo-r19) |  |  |  |  | Open |  | Wait for further RAN1 input and/or company contributions(if any) | RAN2#131 Agreement: Wait for RAN1 for LPP-21. Take what RAN1 gives us and we implemented. Can compile an LS for next meeting if we have questions. | LS has been sent to RAN1 (R2-2506658). Wait for RAN1 response. |  |
| LPP#21a (new) |  | "Associated ID" for TRP Location Coordinates (IE TRP-ImplicitLocationInfo-r19) |  |  |  |  | Open |  |  | The RAN1 parameter list in R1-2506622 includes the following: "The associated ID (optional) provides implicit indication of Info #7. For given TRP(s), the same associated ID implies that geographical coordinates of the TRP(s) can be understood as consistent by the UE. The associated ID is not expected to provide the real value of Info #7 (i.e., geographical coordinates of the TRP(s) are not disclosed). An associated ID is configured per-cell (e.g., NCGI-r15). Note: Info #7 can be provided explicitly (as in legacy UE-based DL-TDOA) or implicitly by Associated ID. Note: RAN1 has no consensus whether the IE for assistance data Info#7 and AssociatedID-TRP-LocationInfo can be simultaneously enabled. It is up to RAN2 to decide whether the IE for assistance data Info#7 and AssociatedID-TRP-LocationInfo can be simultaneously enabled." LPP [3] has currently the following Editor's Note: It is FFS whether the fields nr-TRP-LocationInfo and nr-TRP-LocationInfo-Implicit can both be present in IE NR-PositionCalculationAssistance. |  | TP is provided in Annex B. Company input is requested in section 3.2 below. |

# 3.	Discussion

## 3.1	DL-PRS Capabilities

It is proposed to replace the IE NR-DL-PRS-ProcessingCapability in IE NR-DL-AIML-ProvideCapabilities with an AI/ML positioning specific version (NR-DL-AIML-PRS-ProcessingCapability-r19) because of the following reasons:

- Per latest available RAN1 feature list (R1-2506627) not all fields in the IE NR-DL-PRS-ProcessingCapability are applicable to DL AI/ML positioning.

- For some features, if the capability indication is not provided for DL AI/ML positioning, the "legacy" capability in IE NR-DL-PRS-ProcessingCapability apply.

- One feature (58-2-11) is currently not supported in any LPP capability IE.

A TP based on R1-2506627 is provided in Annex A.

Companies are requested to provide feedback, corrections, or any other comments on the TP1 in Annex A.

| Company | Comments | Rapporteur Comments |
| --- | --- | --- |
| ZTE | Agree to have AI/ML-dedicated NR-DL-PRS-ProcessingCapability, and have shared NR-DL-PRS-QCL-ProcessingCapability and NR-DL-PRS-ResourcesCapability. For 58-2-4, the R1-2506627 has a note saying that ‘if the UE does not indicate this capability for a band or band combination, the UE does not support PRS processing in this band or band combination.’ Does this note mean if UE does not report this capability in any of DL-TDOA/DL-AoD/AI/ML pos method, then UE does not support PRS processing in this band or band combination? Should this note be captured in the corresponding field description NR-DL-AIML-PRS-ProcessingCapabilityElement-r19? | Regarding the NOTE in 58-2-4, such NOTEs are also in the Rel-16 capabilities but not explicitly captured in the LPP. If (any) capability is not provided for a band, the UE cannot support the corresponding capability on this band (in this case, PRS processing). I think this should be obvious and probably the reason why it was also not explicitly captured in the Rel-16 capabilities. |
| Ericsson | We agree with rapporteur reasoning that we can have a new IE NR-DL-AIML-PRS-ProcessingCapability-r19 |  |
| Nokia | - Clause 6.4.3, in the description for IE NR-DL-PRS-ProcessingCapability, stating here that the NR DL AI/ML positioning is an exception, is confusing because of the NOTE 2 under the field description for nr-dl-aiml-prs-ProcessingCapability which says that some fields from this IE are applicable to AIML Case 1 also. BTW, The NOTE 2 under field description for nr-dl-aiml-prs-ProcessingCapability is in yellow, same as in the UE features list for 58-2-4. Yellow highlight in feature list means it is FFS isn’t it? - Rename the field supportedActivatedPRS-ProcessingWindow to “multipleActivatedPRS-ProcessingWindows” and update the field description as: “Indicates support for more than one activated PRS processing windows and the number of activated PRS processing windows supported” |  |

## 3.2	"Associated ID" for TRP Location Coordinates

Per Conclusion from RAN1#122 (R1-2506557), which is also captured in the parameter list [6] (R1-2506626), no consensus was reached in RAN1 "whether the IE for assistance data Info#7 and AssociatedID-TRP-LocationInfo can be simultaneously enabled.":

Conclusion: RAN1 has no consensus whether the IE for assistance data Info#7 and AssociatedID-TRP-LocationInfo can be simultaneously enabled. It is up to RAN2 to decide whether the IE for assistance data Info#7 and AssociatedID-TRP-LocationInfo can be simultaneously enabled.

In current LPP, there is the following Editor's Note in clause 6.4.3:

Editor's Note: It is FFS whether the fields nr-TRP-LocationInfo and nr-TRP-LocationInfo-Implicit can both be present in IE NR-PositionCalculationAssistance.

It is proposed to delete the Editor's Note, since this can be left to implementation/deployment (like it is the case for all LPP assistance data and for all LPP positioning methods).

A TP is provided in Annex B.

Companies are requested to provide feedback on the TP2 in Annex B.

If companies believe this network behaviour must be explicitly specified, please provide appropriate justification. For example, would the feature fail or behave unexpectedly if both the UE and the network support explicit and implicit Info#7, and the UE requests both while the network provides both?

| Company | Comments | Rapporteur Comments |
| --- | --- | --- |
| ZTE | Agree with Rapp that there is no need to have description/classification in spec. can be left to NW implementation, i.e., even if NW provides both, NW will ensure the consistency |  |
| Ericsson | Yes, it should be up to NW implementation |  |
| Nokia | When BOTH UE and LMF support explicit and implicit TRP location, then the NW truly has implementation choice to meet the deployment needs of the operator. But what if the UE supports only one option, either explicit or implicit TRP location since these are independent capability bits in the capabilities message (see nr-PosCalcAssistanceSupport-r19)? Then the NW has to implement the same option as supported by the UE or return an error if the UE requests the TRP location option that is not supported by the NW. It would be really unfortunate if both the UE and NW supports AI/ML positioning Case 1 but it is not possible to be used in a deployment only because the UE and NW chose different TRP location signalling options. Can it then be clarified that if UE supports AI/ML positioning Case 1 the UE shall support both explicit and implicit Info#7 OR the UE can only request the explicit or implicit info#7 depending on what the LMF provided the UE in the initial ProvideAssistanceData message? |  |

## 3.3	Additional Corrections/Comments/Issues

If there are any additional comments, new issues or corrections required to LPP, please provide a description (and possible resolution) in the Table below.

NOTE:	When LPP version 19.0.0 is available, the ASN.1 review process will start as endorsed in R2-2505781 [4]. Therefore, corrections could also be proposed later as part of the ASN.1 review.

| Company | Any Other Issues and possible resolution | Rapporteur Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Based on the previous RAN2 discussion, the common understanding is that an Associated ID is configured per Cell. However, the current granularity (i.e. per TRP) leads to signalling redundancy, i.e. the same TRP under the same NCGI (cell) carries the same "NCGI+nr-AIML-AssociatedID". Therefore, we propose to change the granularity from TRP to Cell, e.g. change the nrMaxTRPsPerFreq-r16 in the original NR-TRP-LocationInfo-Implicit to nrMaxCellsForAIMLPositioning in the proposal, and the definition is provided in TP3 of Annex C. | This is related to Issue#21 where we already sent an LS to RAN1 with questions. We should address this comment when we receive the RAN1 response LS. I assume there may be more changes required to this IE. In addition, we should not add additional FFS or [ ] for the correction phase. |
| Ericsson | We have sympathy to Huawei concern above, but the approach is not correct to solve it. We agree that this can be rather a RIL. |  |
| ZTE2 | In nr-TRP-RequestList IE, the field description should say that this IE is only used for AI/ML positioning. Other positioning method does not need to request TRP information. nr-TRP-RequestList This field specifies a list of TRPs for which the on-demand DL-PRS configuration information is requested for AI/ML positioning and comprises the following subfields:. |  |
| Nokia | We agree with the rapporteur to wait for the response LS from RAN1 before considering other changes related to associated ID. |  |

# Annex A: TP1

### 6.4.3	Common NR Positioning Information Elements

[…]

#### –	NR-DL-PRS-ProcessingCapability

The IE NR-DL-PRS-ProcessingCapability defines the common DL-PRS Processing capability. In the case of capabilities for multiple NR positioning methods (except to NR DL AI/ML positioning) are provided, the IE NR-DL-PRS-ProcessingCapability applies across the NR positioning methods (except to NR DL AI/ML positioning) and the target device shall indicate the same values for the capabilities in IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities.

The PRS-ProcessingCapabilityPerBand is defined for a single positioning frequency layer on a certain band (i.e., a target device supporting multiple positioning frequency layers is expected to process one frequency layer at a time).

-- ASN1START

NR-DL-PRS-ProcessingCapability-r16 ::= SEQUENCE {

prs-ProcessingCapabilityBandList-r16	SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

PRS-ProcessingCapabilityPerBand-r16,

maxSupportedFreqLayers-r16				INTEGER (1..4),

simulLTE-NR-PRS-r16						ENUMERATED { supported }		OPTIONAL,

...,

[[

dummy									ENUMERATED { m1, m2, ... }		OPTIONAL

]]

}

PRS-ProcessingCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16				FreqBandIndicatorNR-r16,

supportedBandwidthPRS-r16			CHOICE {

fr1										ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100},

fr2										ENUMERATED {mhz50, mhz100, mhz200, mhz400},

...

},

dl-PRS-BufferType-r16				ENUMERATED {type1, type2, ...},

durationOfPRS-Processing-r16		SEQUENCE {

durationOfPRS-ProcessingSymbols-r16	ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

durationOfPRS-ProcessingSymbolsInEveryTms-r16

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160,n320, n640, n1280},

...

},

maxNumOfDL-PRS-ResProcessedPerSlot-r16	SEQUENCE {

scs15-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs30-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs60-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs120-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

...,

[[

scs15-v1690								ENUMERATED {n6, n12}					OPTIONAL,

scs30-v1690								ENUMERATED {n6, n12}					OPTIONAL,

scs60-v1690								ENUMERATED {n6, n12}					OPTIONAL,

scs120-v1690							ENUMERATED {n6, n12}					OPTIONAL

]]

},

...,

[[

supportedDL-PRS-ProcessingSamples-RRC-CONNECTED-r17	ENUMERATED { supported }		OPTIONAL,

prs-ProcessingWindowType1A-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingWindowType1B-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingWindowType2-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingCapabilityOutsideMGinPPW-r17

SEQUENCE (SIZE(1..3)) OF

PRS-ProcessingCapabilityOutsideMGinPPWperType-r17

OPTIONAL,

dl-PRS-BufferType-RRC-Inactive-r17		ENUMERATED { type1, type2, ... }			OPTIONAL,

durationOfPRS-Processing-RRC-Inactive-r17	SEQUENCE {

durationOfPRS-ProcessingSymbols-r17			ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

durationOfPRS-ProcessingSymbolsInEveryTms-r17

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160,n320, n640, n1280},

...

}																					OPTIONAL,

maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17	SEQUENCE {

scs15-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs30-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs60-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs120-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

...

}																					OPTIONAL,

supportedLowerRxBeamSweepingFactor-FR2-r17	ENUMERATED { n1, n2, n4, n6 }			OPTIONAL

]],

[[

supportedDL-PRS-ProcessingSamples-RRC-Inactive-r17	ENUMERATED { supported }		OPTIONAL

]],

[[

prs-MeasurementWithoutMG-r17				ENUMERATED {cp, symbolDot25, symbolDot5,

slotDot5}					OPTIONAL

]],

[[

maxNumOfOneSymbolPRS-ResProcessedPerSlot-RRC-Inactive-r18	SEQUENCE {

scs15-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs30-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs60-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs120-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

...

}																					OPTIONAL,

maxNumOfOneSymbolPRS-ResProcessedPerSlot-RRC-Connected-r18	SEQUENCE {

scs15-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs30-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs60-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs120-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

...

}																					OPTIONAL,

ppw-maxNumOfOneSymbolPRS-ResProcessedPerSlot-r18	SEQUENCE {

scs15-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs30-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs60-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs120-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

...

}																					OPTIONAL,

prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected-r18

PRS-BWA-TwoContiguousIntrabandInMG-r18		OPTIONAL,

prs-BWA-ThreeContiguousIntrabandInMG-RRC-Connected-r18

PRS-BWA-ThreeContiguousIntrabandInMG-r18	OPTIONAL,

prs-BWA-TwoContiguousIntraband-RRC-IdleAndInactive-r18

PRS-BWA-TwoContiguousIntrabandInMG-r18		OPTIONAL,

prs-BWA-ThreeContiguousIntraband-RRC-IdleAndInactive-r18

PRS-BWA-ThreeContiguousIntrabandInMG-r18	OPTIONAL,

reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-Connected-r18		ENUMERATED { supported }

OPTIONAL,

reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive-r18

ENUMERATED { supported }					OPTIONAL,

dl-PRS-MeasurementWithRxFH-RRC-Inactive-r18		ENUMERATED { supported }			OPTIONAL,

dl-PRS-MeasurementWithRxFH-RRC-Idle-r18			ENUMERATED { supported }			OPTIONAL,

reducedNumOfSampleForMeasurementWithFH-RRC-Connected-r18	ENUMERATED { supported }

OPTIONAL,

reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive-r18	ENUMERATED { supported }

OPTIONAL,

supportOfPRS-BWA-WithTwoPFL-Combination-r18			ENUMERATED { supported }		OPTIONAL,

dl-PRS-MeasurementWithRxFH-RRC-Connected-r18	DL-PRS-MeasurementWithRxFH-RRC-Connected-r18																						OPTIONAL

]]

}

PRS-ProcessingCapabilityOutsideMGinPPWperType-r17 ::= SEQUENCE {

prsProcessingType-r17						ENUMERATED { type1A, type1B, type2 },

ppw-dl-PRS-BufferType-r17					ENUMERATED { type1, type2, ... },

ppw-durationOfPRS-Processing1-r17			SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN-r17

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms4,

ms6, ms8, ms12, ms16, ms20, ms25, ms30, ms32, ms35,

ms40, ms45, ms50 },

ppw-durationOfPRS-ProcessingSymbolsT-r17

ENUMERATED { ms1, ms2, ms4, ms8, ms16, ms20, ms30, ms40, ms80,

ms160, ms320, ms640, ms1280 }

}																					OPTIONAL,

ppw-durationOfPRS-Processing2-r17			SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN2-r17

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms3, ms4, ms5,

ms6, ms8, ms12 },

ppw-durationOfPRS-ProcessingSymbolsT2-r17

ENUMERATED { ms4, ms5, ms6, ms8 }

}																					OPTIONAL,

ppw-maxNumOfDL-PRS-ResProcessedPerSlot-r17	SEQUENCE {

scs15-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs30-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs60-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs120-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

...

},

...,

[[

ppw-maxNumOfDL-Bandwidth-r17		CHOICE {

fr1										ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100},

fr2										ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}																					OPTIONAL

]]

}

PRS-BWA-TwoContiguousIntrabandInMG-r18 ::= SEQUENCE {

maximumOfTwoAggregatedDL-PRS-Bandwidth-FR1-r18	ENUMERATED {mhz10, mhz20, mhz40, mhz50,

mhz80, mhz100, mhz160, mhz200}

OPTIONAL,

maximumOfTwoAggregatedDL-PRS-Bandwidth-FR2-r18	ENUMERATED {mhz100, mhz200, mhz400, mhz800}

OPTIONAL,

maximumOfDL-PRS-BandwidthPerPFL-FR1-r18			ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100}	OPTIONAL,

maximumOfDL-PRS-BandwidthPerPFL-FR2-r18			ENUMERATED {mhz50, mhz100, mhz200, mhz400}

OPTIONAL,

dl-PRS-BufferTypeOfBWA-r18						ENUMERATED {type1, type2},

prs-durationOfTwoPRS-BWA-Processing-r18			SEQUENCE {

prs-durationOfTwoPRS-BWA-ProcessingSymbolsN-r18

ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2, ms4, ms6, ms8, ms12,

ms16, ms20, ms25, ms30, ms32, ms35, ms40, ms45, ms50},

prs-durationOfTwoPRS-BWA-ProcessingSymbolsT-r18

ENUMERATED {ms8, ms16, ms20, ms30, ms40, ms80, ms160, ms320, ms640, ms1280}

}																					OPTIONAL,

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r18	SEQUENCE {

scs15-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs30-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs60-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL

},

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r18	SEQUENCE {

scs60-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs120-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL

}

}

PRS-BWA-ThreeContiguousIntrabandInMG-r18 ::= SEQUENCE {

maximumOfThreeAggregatedDL-PRS-Bandwidth-FR1-r18

ENUMERATED {mhz15, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz120,

mhz140, mhz150, mhz180, mhz200, mhz240, mhz300}		OPTIONAL,

maximumOfThreeAggregatedDL-PRS-Bandwidth-FR2-r18

ENUMERATED {mhz150, mhz200, mhz300, mhz400, mhz600, mhz800, mhz1000,

mhz1200}											OPTIONAL,

maximumOfDL-PRS-BandwidthPerPFL-FR1-r18

ENUMERATED {mhz5, mhz10, mhz20, mhz40, mhz50, mhz80, mhz100}	OPTIONAL,

maximumOfDL-PRS-BandwidthPerPFL-FR2-r18

ENUMERATED {mhz50, mhz100, mhz200, mhz400}						OPTIONAL,

dl-PRS-BufferTypeOfBWA-r18							ENUMERATED {type1, type2},

prs-durationOfThreePRS-BWA-Processing-r18			SEQUENCE {

prs-durationOfThreePRS-BWA-ProcessingSymbolsN-r18

ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2, ms4, ms6, ms8, ms12,

ms16, ms20, ms25, ms30, ms32, ms35, ms40, ms45, ms50},

prs-durationOfThreePRS-BWA-ProcessingSymbolsT-r18

ENUMERATED {ms8, ms16, ms20, ms30, ms40, ms80, ms160,

ms320, ms640, ms1280}

}																					OPTIONAL,

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r18	SEQUENCE {

scs15-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs30-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs60-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL

},

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r18	SEQUENCE {

scs60-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs120-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL

}

}

DL-PRS-MeasurementWithRxFH-RRC-Connected-r18 ::=SEQUENCE {

maximumPRS-BandwidthAcrossAllHopsFR1-r18	ENUMERATED {mhz40, mhz50, mhz80, mhz100}

OPTIONAL,

maximumPRS-BandwidthAcrossAllHopsFR2-r18	ENUMERATED {mhz100, mhz200, mhz400}		OPTIONAL,

maximumFH-Hops-r18							ENUMERATED {n2, n3, n4, n5, n6}			OPTIONAL,

processingDuration-r18	SEQUENCE {

processingPRS-SymbolsDurationN3-r18		ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2,

ms4, ms6, ms8, ms12,ms16, ms20, ms25,

ms30, ms32, ms35, ms40, ms45, ms50},

processingDurationT3-r18				ENUMERATED {ms8, ms16, ms20, ms30, ms40, ms80,

ms160, ms320, ms640, ms1280}

}																					OPTIONAL,

rf-RxRetuneTimeFR1-r18						ENUMERATED {n70,n140,n210}				OPTIONAL,

rf-RxRetuneTimeFR2-r18						ENUMERATED {n35,n70,n140}				OPTIONAL,

numOfOverlappingPRB-r18						ENUMERATED {n0,n1,n2,n4}				OPTIONAL,

...

}

-- ASN1STOP

NR-DL-PRS-ProcessingCapability field descriptions

maxSupportedFreqLayers Indicates the maximum number of positioning frequency layers supported by UE.

simulLTE-NR-PRS Indicates whether the UE supports parallel processing of LTE PRS and NR DL-PRS.

dummy This field is not used in the specification. If received it shall be ignored by the receiver.

supportedBandwidthPRS Indicates the maximum number of DL-PRS bandwidth in MHz, which is supported and reported by UE.

dl-PRS-BufferType Indicates DL-PRS buffering capability. Value type1 indicates sub-slot/symbol level buffering and value type2 indicates slot level buffering.

durationOfPRS-Processing Indicates the duration N of DL-PRS symbols in units of ms a UE can process every T ms assuming maximum DL-PRS bandwidth provided in supportedBandwidthPRS and comprises the following subfields: - durationOfPRS-ProcessingSymbols: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - durationOfPRS-ProcessingSymbolsInEveryTms: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. See NOTE 9.

maxNumOfDL-PRS-ResProcessedPerSlot Indicates the maximum number of DL-PRS Resources that UE can process in a slot. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands.

supportedDL-PRS-ProcessingSamples-RRC-CONNECTED Indicates the UE capability for support of measurements based on measuring M=1 or M=2 (instances) of a DL-PRS Resource Set. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 1: This feature is supported for both UE-assisted and UE based positioning.

prs-ProcessingWindowType1A Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 1A refers to the determination of prioritization between DL-PRS and other DL signals/channels in all OFDM symbols within the DL-PRS Processing Window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR. Enumerated value indicates supported priority handing options of DL-PRS: - option1: Support of "st1" and "st3" defined in clause 5.1.6.5 of TS 38.214 [45]. - option2: Support of "st1", "st2", and "st3" defined in clause 5.1.6.5 of TS 38.214 [45]. - option3: Support of "st1" only defined in clause 5.1.6.5 of TS 38.214 [45]. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 2: Within a DL-PRS processing window, UE measurement is inside the active DL BWP with DL-PRS having the same numerology as the active DL BWP. NOTE 2a: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS.

prs-ProcessingWindowType1B Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 1B refers to the determination of prioritization between DL-PRS and other DL signals/channels in all OFDM symbols within the DL-PRS processing window. The DL signals/channels from a certain band are affected. Enumerated value indicates supported priority handing options of DL-PRS (see prs-ProcessingWindowType1A). The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 3: Within a DL-PRS processing window, UE measurement is inside the active DL BWP with DL-PRS having the same numerology as the active DL BWP. NOTE 3a: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS.

prs-ProcessingWindowType2 Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 2 refers to the determination of prioritization between DL-PRS and other DL signals/channels only in DL-PRS symbols within the DL-PRS processing window. Enumerated value indicates supported priority handing options of DL-PRS (see prs-ProcessingWindowType1A). The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 4: Within a DL-PRS processing window, UE measurement is inside the active DL BWP with DL-PRS having the same numerology as the active DL BWP. NOTE 4a: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS.

prs-ProcessingCapabilityOutsideMGinPPW Indicates the DL-PRS Processing Capability outside MG of each of the supported PPW Type in the case the UE supports multiple PPW Types in a band and comprises the following subfields: - prsProcessingType: Indicates the DL-PRS Processing Window Type for which the prs-ProcessingCapabilityOutsideMGinPPW are provided. - ppw-dl-PRS-BufferType: Indicates DL-PRS buffering capability. Value 'type1' indicates sub-slot/symbol level buffering and value 'type2' indicates slot level buffering. - ppw-durationOfPRS-Processing1: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum DL-PRS bandwidth provided in ppw-maxNumOfDL-Bandwidth and comprises the following subfields: - ppw-durationOfPRS-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - ppw-durationOfPRS-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. - ppw-durationOfPRS-Processing2: Indicates the duration of DL-PRS symbols N2 in units of ms a UE can process inT2 ms assuming maximum DL-PRS bandwidth provided in ppw-maxNumOfDL-Bandwidth and comprises the following subfields: - ppw-durationOfPRS-ProcessingSymbolsN2: This field specifies the values for N2. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12 ms. - ppw-durationOfPRS-ProcessingSymbolsT2: This field specifies the values for T2. Enumerated values indicate 4, 5, 6, 8 ms. - ppw-maxNumOfDL-PRS-ResProcessedPerSlot: Indicates the maximum number of DL-PRS resources that UE can process in a slot. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands. - ppw-maxNumOfDL-Bandwidth: Indicates the maximum number of DL-PRS bandwidth in MHz, which is supported and reported by UE for DL-PRS measurement outside MG within the PPW. The UE can include this field only if the UE supports one of prs-ProcessingWindowType1A, prs-ProcessingWindowType1B and prs-ProcessingWindowType2. Otherwise, the UE does not include this field. NOTE 5: A UE that supports one of prs-ProcessingWindowType1A, prs-ProcessingWindowType1B or prs-ProcessingWindowType2 shall always include the prs-ProcessingCapabilityOutsideMGinPPW. NOTE 6: The (N, T) UE capability in ppw-durationOfPRS-Processing1 is interpreted as in NOTE 9, and the UE is expected to receive the DL-PRS within the DL-PRS processing window but the processing of the received DL-PRS may be outside a DL-PRS processing window. NOTE 7: The (N2, T2) UE capability in ppw-durationOfPRS-Processing2 is interpreted such that the UE is capable of measuring up to N2 ms DL-PRS within a PPW and is capable of completing the DL-PRS processing within the PPW, e.g., if the time duration from the last symbol of the measured DL-PRS Resource(s) inside the PPW to the end of PPW is not smaller than T2 ms. NOTE 8: A UE which supports prs-ProcessingCapabilityOutsideMGinPPW shall support either ppw-durationOfPRS-Processing1 or ppw-durationOfPRS-Processing2, but not both for each supported type in a band.

dl-PRS-BufferType-RRC-Inactive Indicates DL-PRS buffering capability in RRC_INACTIVE state. Value 'type1' indicates sub-slot/symbol level buffering and value 'type2' indicates slot level buffering.

durationOfPRS-Processing-RRC-Inactive Indicates the duration N of DL-PRS symbols in units of ms a UE can process every T ms in RRC_INACTIVE state assuming maximum DL-PRS bandwidth provided in supportedBandwidthPRS and comprises the following subfields: - durationOfPRS-ProcessingSymbols: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - durationOfPRS-ProcessingSymbolsInEveryTms: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. See NOTE 9.

maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive Indicates the maximum number of DL-PRS Resources a UE can process in a slot in RRC_INACTIVE state. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands.

supportedLowerRxBeamSweepingFactor-FR2 Indicates support of the lower Rx beam sweeping factor than 8 for FR2. Enumerated value indicates the number of Rx beam sweeping factors supported.

supportedDL-PRS-ProcessingSamples-RRC-Inactive Indicates the UE capability for support of reduced number of samples for DL-PRS measurement in RRC_INACTIVE state. The UE can include this field only if the UE supports prs-ProcessingRRC-Inactive defined in TS 38.331 [35]. Otherwise, the UE does not include this field.

maxNumOfOneSymbolPRS-ResProcessedPerSlot-RRC-Inactive Indicates the maximum number of single-symbol DL-PRS Resources that UE can process in a slot in RRC_INACTIVE. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands. A UE which supports maxNumOfOneSymbolPRS-ResProcessedPerSlot-RRC-Inactive shall support single-symbol DL-PRS with the comb sizes from {2,4,6,12}. The UE can include this field only if the UE supports one of dl-PRS-BufferType-RRC-Inactive, durationOfPRS-Processing-RRC-Inactive, and maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive. Otherwise, the UE does not include this field.

maxNumOfOneSymbolPRS-ResProcessedPerSlot-RRC-Connected Indicates the maximum number of single-symbol DL-PRS Resources that UE can process in a slot inside a measurement gap in RRC_CONNECTED. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands. A UE which supports maxNumOfOneSymbolPRS-ResProcessedPerSlot-RRC-Connected shall support single-symbol DL-PRS with the comb sizes from {2,4,6,12}. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field.

ppw-maxNumOfOneSymbolPRS-ResProcessedPerSlot Indicates the maximum number of single-symbol DL-PRS Resources that UE can process in a slot outside a measurement gap in RRC_CONNECTED. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands. A UE which supports ppw-maxNumOfOneSymbolPRS-ResProcessedPerSlot shall support single-symbol DL-PRS with the comb sizes from {2,4,6,12}. The UE can include this field only if the UE supports prs-ProcessingCapabilityOutsideMGinPPW. Otherwise, the UE does not include this field.

prs-MeasurementWithoutMG Indicates the UE capability for support of Rx timing difference between the serving cell and non-serving cell for DL-PRS measurement within a PPW. Value 'cp' indicates one CP length, value 'symbolDot25' indicates 0.25 symbol length, value 'symbolDot5' indicates 0.5 symbol length and value 'slotDot5' indicates 0.5 slot length. The UE can include this field only if the UE supports one of prs-ProcessingWindowType1A, prs-ProcessingWindowType1B and prs-ProcessingWindowType2. Otherwise, the UE does not include this field.

prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected Indicates the UE capability for support of DL-PRS processing capabilities for aggregated DL-PRS processing of 2 PFLs in intra-band contiguous within a MG for RRC_CONNECTED state and and comprises the following subfields: - maximumOfTwoAggregatedDL-PRS-Bandwidth-FR1: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR1, which is supported and reported by UE. - maximumOfTwoAggregatedDL-PRS-Bandwidth-FR2: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR2, which is supported and reported by UE. - maximumOfDL-PRS-BandwidthPerPFL-FR1: Indicates the maximum DL-PRS bandwidth in MHz for FR1, per PFL. - maximumOfDL-PRS-BandwidthPerPFL-FR2: Indicates the maximum DL-PRS bandwidth in MHz for FR2, per PFL. - dl-PRS-BufferTypeOfBWA: Indicates the DL-PRS buffering capability. - prs-durationOfTwoPRS-BWA-Processing: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL-PRS bandwidth in MHz, which is supported and reported by UE. - prs-durationOfTwoPRS-BWA-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - prs-durationOfTwoPRS-BWA-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 2560 ms. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR1. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR2. The UE can include this field only if the UE supports supportedBandwidthPRS, dl-PRS-BufferType, durationOfPRS-Processing and maxNumOfDL-PRS-ResProcessedPerSlot. Otherwise, the UE does not include this field. NOTE 10: dl-PRS-BufferTypeOfBWA follows buffering capability type reported in dl-PRS-BufferType. NOTE 11: The value N should be equal or smaller than the value N reported by durationOfPRS-ProcessingSymbols, or this value T should be equal or larger than the value T reported by durationOfPRS-ProcessingSymbolsInEveryTms. NOTE 12: Each two linked DL-PRS Resources are counted as 1 resource. NOTE 13: maxNumOfAggregatedDL-PRS-ResourcePerSlot should be equal or smaller than the value reported by maxNumOfDL-PRS-ResProcessedPerSlot. NOTE 14: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL)/measurement gap repetition period (MGRP) of no more than 30%.

prs-BWA-ThreeContiguousIntrabandInMG-RRC-Connected Indicates the UE capability for support of DL-PRS processing capabilities for aggregated DL-PRS processing of 3 PFLs in intra-band contiguous within a MG for RRC_CONNECTED state and comprises the following subfields: - maximumOfThreeAggregatedDL-PRS-Bandwidth-FR1: Indicates the maximum aggregated DL-PRS bandwidth in MHz of for FR1, which is supported and reported by UE. - maximumOfThreeAggregatedDL-PRS-Bandwidth-FR2: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR2, which is supported and reported by UE. - maximumOfDL-PRS-BandwidthPerPFL-FR1: Indicates the maximum DL-PRS bandwidth in MHz for FR1, per PFL - maximumOfDL-PRS-BandwidthPerPFL-FR2: Indicates the maximum DL-PRS bandwidth in MHz for FR2, per PFL - dl-PRS-BufferTypeOfBWA: Indicates the DL-PRS buffering capability. - prs-durationOfThreePRS-BWA-Processing: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL-PRS bandwidth in MHz, which is supported and reported by UE. - prs-durationOfThreePRS-BWA-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - prs-durationOfThreePRS-BWA-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 3840 ms. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR1. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR2. The UE can include this field only if the UE supports prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field. NOTE15: dl-PRS-BufferTypeOfBWA follows buffering capability type reported in dl-PRS-BufferType. NOTE16: The value N should be equal or smaller than the value N reported by durationOfPRS-ProcessingSymbols, or this value T should be equal or larger than the value T reported by durationOfPRS-ProcessingSymbolsInEveryTms. NOTE17: Each three linked DL-PRS Resources are counted as 1 resource. NOTE18: maxNumOfAggregatedDL-PRS-ResourcePerSlot should be equal or smaller than the value reported by maxNumOfDL-PRS-ResProcessedPerSlot. NOTE19: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL)/measurement gap repetition period (MGRP) of no more than 30%.

prs-BWA-TwoContiguousIntraband-RRC-IdleAndInactive Indicates the UE capability for support of DL-PRS processing capabilities for aggregated DL-PRS processing of 2 PFLs in intra-band contiguous for RRC_INACTIVE and RRC_IDLE state. The UE can include this field only if the UE supports dl-PRS-BufferType-RRC-Inactive, durationOfPRS-Processing-RRC-Inactive and maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maximumOfTwoAggregatedDL-PRS-Bandwidth-FR1: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR1, which is supported and reported by UE. - maximumOfTwoAggregatedDL-PRS-Bandwidth-FR2: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR2, which is supported and reported by UE. - maximumOfDL-PRS-BandwidthPerPFL-FR1: Indicates the maximum DL-PRS bandwidth in MHz for FR1, per PFL. - maximumOfDL-PRS-BandwidthPerPFL-FR2: Indicates the maximum DL-PRS bandwidth in MHz for FR2, per PFL. - dl-PRS-BufferTypeOfBWA: Indicates the DL-PRS buffering capability. - prs-durationOfTwoPRS-BWA-Processing: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL-PRS bandwidth in MHz, which is supported and reported by UE. - prs-durationOfTwoPRS-BWA-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - prs-durationOfTwoPRS-BWA-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 2560 ms. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR1. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR2. NOTE 20: dl-PRS-BufferTypeOfBWA follows buffering capability type reported in dl-PRS-BufferType. NOTE 21: The value N should be equal or smaller than the value N reported by durationOfPRS-ProcessingSymbols, or this value T should be equal or larger than the value T reported by durationOfPRS-ProcessingSymbolsInEveryTms. NOTE 22: Each two linked DL-PRS resources are counted as 1 resource. NOTE 23: maxNumOfAggregatedDL-PRS-ResourcePerSlot should be equal or smaller than the value reported by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive.

prs-BWA-ThreeContiguousIntraband-RRC-IdleAndInactive Indicates the UE capability for support of DL-PRS processing capabilities for aggregated DL-PRS processing of 3 PFLs in intra-band contiguous for RRC_INACTIVE and RRC_IDLE state. The UE can include this field only if the UE supports prs-BWA-TwoContiguousIntraband-RRC-IdleAndInactive. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maximumOfThreeAggregatedDL-PRS-Bandwidth-FR1: Indicates the maximum aggregated DL-PRS bandwidth in MHz of for FR1, which is supported and reported by UE. - maximumOfThreeAggregatedDL-PRS-Bandwidth-FR2: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR2, which is supported and reported by UE. - maximumOfDL-PRS-BandwidthPerPFL-FR1: Indicates the maximum DL-PRS bandwidth in MHz for FR1, per PFL - maximumOfDL-PRS-BandwidthPerPFL-FR2: Indicates the maximum DL-PRS bandwidth in MHz for FR2, per PFL - dl-PRS-BufferTypeOfBWA: Indicates the DL-PRS buffering capability. - prs-durationOfThreePRS-BWA-Processing: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL-PRS bandwidth in MHz, which is supported and reported by UE. - prs-durationOfThreePRS-BWA-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - prs-durationOfThreePRS-BWA-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 3840 ms. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR1. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR2. NOTE 24: dl-PRS-BufferTypeOfBWA follows buffering capability type reported in dl-PRS-BufferType. NOTE 25: The value N should be equal or smaller than the value N reported by durationOfPRS-ProcessingSymbols, or this value T should be equal or larger than the value T reported by durationOfPRS-ProcessingSymbolsInEveryTms. NOTE 26: Each two linked DL-PRS resources are counted as 1 resource. NOTE 27: maxNumOfAggregatedDL-PRS-ResourcePerSlot should be equal or smaller than the value reported by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive.

reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-Connected Indicates whether the UE supports reduced number of samples in positioning measurements with DL-PRS bandwidth aggregation for RRC_CONNECTED. The UE can include this field only if the UE indicates the capability of maximum aggregated DL-PRS bandwidth for the supported FR1 or FR2 bands by using maximumOfTwoAggregatedDL-PRS-Bandwidth-FR1 or maximumOfTwoAggregatedDL-PRS-Bandwidth-FR2 of prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field.

reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive Indicates whether the UE supports reduced number of samples in positioning measurements with DL-PRS bandwidth aggregation for RRC_IDLE and RRC_INACTIVE. The UE can include this field only if the UE indicates the capability of maximum aggregated DL-PRS bandwidth for the supported FR1 or FR2 bands by using maximumOfTwoAggregatedDL-PRS-Bandwidth-FR1 or maximumOfTwoAggregatedDL-PRS-Bandwidth-FR2 of prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field.

dl-PRS-MeasurementWithRxFH-RRC-Inactive Indicates the UE capability for support of DL-PRS measurement with Rx frequency hopping in RRC_INACTIVE for RedCap UEs. The UE can include this field only if the UE supports dl-PRS-MeasurementWithRxFH-RRC-Connected and prs-ProcessingRRC-Inactive defined in TS 38.331 [35]. Otherwise, the UE does not include this field.

dl-PRS-MeasurementWithRxFH-RRC-Idle Indicates the UE capability for support of DL-PRS measurement with Rx frequency hopping in RRC_IDLE for RedCap UEs. The UE can include this field only if the UE supports dl-PRS-MeasurementWithRxFH-RRC-Connected. Otherwise, the UE does not include this field.

reducedNumOfSampleForMeasurementWithFH-RRC-Connected Indicates whether the UE supports reduced number of samples for DL-PRS based positioning measurements with frequency hopping for RRC_CONNECTED. The UE can include this field only if the UE supports supportOfRedCap or supportOfERedCap defined in TS 38.331 [35], supportedDL-PRS-ProcessingSamples-RRC-CONNECTED and dl-PRS-MeasurementWithRxFH-RRC-Connected. Otherwise, the UE does not include this field.

reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive Indicates whether the UE supports reduced number of samples for DL-PRS based positioning measurements with frequency hopping for RRC_IDLE and RRC_INACTIVE. The UE can include this field only if the UE supports supportOfRedCap or supportOfERedCap defined in TS 38.331 [35], supportedDL-PRS-ProcessingSamples-RRC-CONNECTED and dl-PRS-MeasurementWithRxFH-RRC-Connected. Otherwise, the UE does not include this field.

supportOfPRS-BWA-WithTwoPFL-Combination Indicates whether the UE supports DL-PRS bandwidth aggregation with two PFL combinations. The UE can include this field only if the UE supports prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field.

dl-PRS-MeasurementWithRxFH-RRC-Connected Indicates the UE capability for DL-PRS measurement with Rx frequency hopping within a MG and measurement reporting in RRC_CONNECTED for RedCap UEs. The UE can include this field only if the UE supports supportedBandwidthPRS, dl-PRS-BufferType, durationOfPRS-Processing, maxNumOfDL-PRS-ResProcessedPerSlot and one of supportOfRedCap and supportOfERedCap defined in TS 38.331 [35]. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maximumPRS-BandwidthAcrossAllHopsFR1: Indicates the maximum DL-PRS bandwidth across all hops in MHz for FR1, which is supported and reported by UE. - maximumPRS-BandwidthAcrossAllHopsFR2: Indicates the maximum DL-PRS bandwidth across all hops in MHz for FR2, which is supported and reported by UE. - maximumFH-Hops: Indicates the maximum number of hops, which is supported and reported by UE. - processingDuration: Indicates the duration of DL-PRS symbols N3 in units of ms a UE can process every T3 ms. - processingPRS-SymbolsDurationN3: This field specifies the values for N3. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - processingDurationT3: This field specifies the values for T3. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280ms. - rf-RxRetuneTimeFR1: Indicates the RF Rx retune times between consecutive hops for FR1. Enumerated values indicate 70, 140, 210µs. - rf-RxRetuneTimeFR2: Indicates the RF Rx retune times between consecutive hops for FR2. Enumerated values indicate 35, 70, 140µs. - numOfOverlappingPRB: Indicates the overlapping PRB(s) between adjacent hops. Enumerated values indicate 0,1,2,4 PRBs. NOTE 28: The maximum DL-PRS bandwidth per hop follows supportedBandwidthPRS. NOTE 29: DL-PRS buffering capability follows dl-PRS-BufferType.

NOTE 9: When the target device provides the durationOfPRS-Processing capability (N, T) for any P(≥T) time window defined in TS 38.214 [45] clause 5.1.6.5, the target device should be capable of processing all DL-PRS Resources within P, if - N≥K where K is defined in the TS 38.214 [45] clause 5.1.6.5, and - the number of DL-PRS Resources in each slot does not exceed the maxNumOfDL-PRS-ResProcessedPerSlot, and - the configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) is as specified in TS 38.133 [46].

#### –	NR-DL-PRS-QCL-ProcessingCapability

The IE NR-DL-PRS-QCL-ProcessingCapability defines the common DL-PRS QCL Processing capability. The target device can include this IE only if the target device supports NR-DL-PRS-ProcessingCapability or NR-DL-AIML-PRS-ProcessingCapability. Otherwise, the target device does not include this IE.

In the case of capabilities for multiple NR positioning methods (except for NR DL AI/ML positioning) are provided, the IE NR-DL-PRS-QCL-ProcessingCapability applies across the NR positioning methods (except to NR DL AI/ML positioning) and the target device shall indicate the same values for the capabilities in IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities.

-- ASN1START

NR-DL-PRS-QCL-ProcessingCapability-r16 ::= SEQUENCE {

dl-PRS-QCL-ProcessingCapabilityBandList-r16			SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

DL-PRS-QCL-ProcessingCapabilityPerBand-r16,

...

}

DL-PRS-QCL-ProcessingCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16						FreqBandIndicatorNR-r16,

ssb-FromNeighCellAsQCL-r16					ENUMERATED { supported}	OPTIONAL,

prs-FromServNeighCellAsQCL-r16				ENUMERATED { supported} OPTIONAL,

...

}

-- ASN1STOP

NR-DL-PRS-QCL-ProcessingCapability field descriptions

ssb-FromNeighCellAsQCL Indicates the support of SSB from neighbour cell as QCL source of a DL-PRS. UE supporting this feature also support reusing SSB measurement from RRM for receiving DL-PRS. Note: It refers to Type-C for FR1 and Type-C & Type-D support for FR2.

prs-FromServNeighCellAsQCL Indicates the support of DL-PRS from serving/neighbour cell as QCL source of a DL-PRS. Note 1: It refers to Type-D support for FR2. Note 2: A DL-PRS from a PRS-only TP is treated as DL-PRS from a non-serving cell.

[…]

#### –	NR-DL-PRS-ResourcesCapability

The IE NR-DL-PRS-ResourcesCapability defines the DL-PRS Resources capability for each NR positioning method. The target device can include this IE only if the target device supports NR-DL-PRS-ProcessingCapability or NR-DL-AIML-PRS-ProcessingCapability. Otherwise, the target device does not include this IE.

-- ASN1START

NR-DL-PRS-ResourcesCapability-r16 ::= SEQUENCE {

maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer-r16

INTEGER (1..2),

maxNrOfTRP-AcrossFreqs-r16					ENUMERATED { n4, n6, n12, n16, n32,

n64, n128, n256, ..., n24-v1690},

maxNrOfPosLayer-r16							INTEGER (1..4),

dl-PRS-ResourcesCapabilityBandList-r16		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

DL-PRS-ResourcesCapabilityPerBand-r16,

dl-PRS-ResourcesBandCombinationList-r16		DL-PRS-ResourcesBandCombinationList-r16,

...

}

DL-PRS-ResourcesCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16						FreqBandIndicatorNR-r16,

maxNrOfDL-PRS-ResourcesPerResourceSet-r16	ENUMERATED { n1, n2, n4, n8, n16, n32, n64, ...},

maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer-r16

ENUMERATED { n6, n24, n32, n64, n96, n128,

n256, n512, n1024, ...},

...

}

DL-PRS-ResourcesBandCombinationList-r16 ::=	SEQUENCE (SIZE (1..maxBandComb-r16)) OF

DL-PRS-ResourcesBandCombination-r16

DL-PRS-ResourcesBandCombination-r16 ::=	SEQUENCE {

bandList-r16							SEQUENCE (SIZE (1..maxSimultaneousBands-r16)) OF

FreqBandIndicatorNR-r16,

maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet-r16

CHOICE {

fr1-Only-r16							ENUMERATED {n6, n24, n64, n128, n192,

n256, n512, n1024, n2048},

fr2-Only-r16							ENUMERATED {n24, n64, n96, n128, n192,

n256, n512, n1024, n2048},

fr1-FR2Mix-r16							SEQUENCE {

fr1-r16									ENUMERATED {n6, n24, n64, n96, n128,

n192, n256, n512, n1024, n2048},

fr2-r16									ENUMERATED {n24, n64, n96, n128, n192,

n256, n512, n1024, n2048},

...

},

...

},

...

}

-- ASN1STOP

NR-DL-PRS-ResourcesCapability field descriptions

maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer Indicates the maximum number of DL-PRS Resource Sets per TRP per positioning frequency layer supported by UE.

maxNrOfTRP-AcrossFreqs Indicates the maximum number of TRPs across all positioning frequency layers.

maxNrOfPosLayer Indicates the maximum number of supported positioning frequency layers.

dl-PRS-ResourcesBandCombinationList Provides the capabilities of DL-PRS Resources for the indicated band combination in bandList. This field is provided for all band combinations for which the target device supports DL-PRS.

maxNrOfDL-PRS-ResourcesPerResourceSet Indicates the maximum number of DL-PRS Resources per DL-PRS Resource Set. Value 16, 32, 64 are only applicable to FR2 bands. Value 1 is not applicable for DL-AoD.

maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer Indicates the maximum number of DL-PRS Resources per positioning frequency layer. Value 6 is only applicable to FR1 bands.

maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet Indicates the maximum number of DL-PRS Resources supported by UE across all frequency layers, TRPs and DL-PRS Resource Sets. fr1-Only: This is applicable for FR1 only band combinations; fr2-Only: This is applicable for FR2 only band combinations; fr1-FR2Mix: This is applicable for band combinations containing FR1 and FR2 bands. fr1 means for FR1 in FR1/FR2 mixed operation, and fr2 means for FR2 in FR1/FR2 mixed operation.

[…]

#### 6.5.13.6	NR DL AI/ML Positioning Capability Information

#### –	NR-DL-AIML-ProvideCapabilities

The IE NR-DL-AIML-ProvideCapabilities is used by the target device to indicate its capability to support NR DL AI/ML positioning and to provide its NR DL AI/ML positioning capabilities to the location server.

-- ASN1START

NR-DL-AIML-ProvideCapabilities-r19 ::= SEQUENCE {

locationCoordinateTypes-r19					LocationCoordinateTypes					OPTIONAL,

periodicalReporting-r19						ENUMERATED { supported }				OPTIONAL,

periodicReportingIntervalMsSupport-r19		PeriodicReportingIntervalMsSupport-r18	OPTIONAL,

ten-ms-unit-ResponseTime-r19				ENUMERATED { supported }				OPTIONAL,

scheduledLocationRequestSupported-r19		ScheduledLocationTimeSupport-r17		OPTIONAL,

nr-PosCalcAssistanceSupport-r19				BIT STRING {

trpLocSup			(0),

beamInfoSup			(1),

rtdInfoSup			(2),

trpTEG-InfoSup		(3),

nr-IntegritySup 		(4),

pruInfoSup			(5),

trpLoc-ImplicitSup	(6)

} (SIZE (1..8))							OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r19		SEQUENCE {

type		LOS-NLOS-IndicatorType2-r17,

granularity	LOS-NLOS-IndicatorGranularity2-r17,

...

}										OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r19		BIT STRING {

eAoD		(0),

eAoA		(1)

} (SIZE (1..8))							OPTIONAL,

nr-DL-AIML-On-Demand-DL-PRS-Support-r19		NR-On-Demand-DL-PRS-Support-r17			OPTIONAL,

nr-DL-AIML-On-Demand-DL-PRS-ForBWA-Support-r19

ENUMERATED { supported }				OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r19		SEQUENCE {

area-validity	INTEGER (1..maxNrOfAreas-r17)			OPTIONAL,

...

}														OPTIONAL,

multiLocationEstimateInSameMeasReport-r19	ENUMERATED { supported }				OPTIONAL,

nr-IntegrityAssistanceSupport-r19			BIT STRING {

serviceParametersSup		(0),

serviceAlertSup				(1),

riskParametersSup			(2),

integrityParaTRP-LocSup		(3),

integrityParaBeamInfoSup	(4),

integrityParaRTD-InfoSup	(5)

} (SIZE (1..8))							OPTIONAL,

nr-DL-AIML-CapabilityPerBandList-r19		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

NR-DL-AIML-CapabilityPerBand-r19	OPTIONAL,	nr-DL-AIML-PRS-Capability-r19				NR-DL-PRS-ResourcesCapability-r16,

nr-DL-AIML-QCL-ProcessingCapability-r19		NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-AIML-PRS-ProcessingCapability-r19		NR-DL-AIML-PRS-ProcessingCapability-r19,

...

}

NR-DL-AIML-CapabilityPerBand-r19 ::= SEQUENCE {

freqBandIndicatorNR-r19						FreqBandIndicatorNR-r16,

simul-DL-AIML-and-DL-TDOA-r19				ENUMERATED { supported}					OPTIONAL,

simul-DL-AIML-and-DL-AoD-r19				ENUMERATED { supported}					OPTIONAL,

supportOfDL-PRS-BWA-RRC-Connected-r19		ENUMERATED { supported }				OPTIONAL,

...

}

-- ASN1STOP

NR-DL-AIML-ProvideCapabilities field descriptions

locationCoordinateTypes This field indicates the geographical location coordinate types that a target device supports for DL AI/ML positioning. TRUE indicates that a location coordinate type is supported and FALSE that it is not.

periodicalReporting This field, if present, indicates that the target device supports periodicalReporting. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation from the location server.

periodicReportingIntervalMsSupport This field, if present, indicates that the target device supports the reportingIntervalMs in IE PeriodicalReportingCriteriaExt in IE CommonIEsRequestLocationInformation from the location server and specifies the minimum millisecond periodic reporting interval supported.

ten-ms-unit-ResponseTime This field, if present, indicates that the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation.

scheduledLocationRequestSupported This field, if present, indicates that the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time.

nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-DL-PRS-TRP-TEG-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 4 together with bit 0 indicates whether the fields nr-IntegrityTRP-LocationBounds, nr-IntegrityDL-PRS-ResourceSetARP-LocationBounds, nr-IntegrityDL-PRS-ResourceARP-LocationBounds in IE NR-TRP-LocationInfo are supported or not; bit 4 together with bit 1 indicates whether the field nr-IntegrityBeamInfoBounds in IE NR-DL-PRS-BeamInfo is supported or not; bit 4 together with the bit 2 indicates whether the field nr-IntegrityRTD-InfoBounds in IE NR-RTD-Info is supported or not; - bit 5 indicates whether the field nr-PRU-DL-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 6 indicates whether the field nr-TRP-LocationInfo-Implicit in IE NR-PositionCalculationAssistance is supported or not.

nr-los-nlos-AssistanceDataSupport This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedLOS-NLOS-Assistance in IE NR-PositionCalculationAssistance: - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in LOS-NLOS-Indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance. - granularity indicates whether the target device supports the granularity for nr-los-nlos-indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance of 'per-trp', 'per-resource', or both.

nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData.

nr-DL-AIML-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-DL-AIML-On-Demand-DL-PRS-ForBWA-Support This field, if present, indicates that the target device supports on-demand DL-PRS request for bandwidth aggregation.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports.

multiLocationEstimateInSameMeasReport This field, if present, indicates that the target device supports multiple location estimate instances in a single measurement report.

nr-IntegrityAssistanceSupport This field indicates the Integrity Assistance Data supported. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-IntegrityServiceParameters in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-IntegrityServiceAlert in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-IntegrityRiskParameters in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-IntegrityParametersTRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 4 indicates whether the field nr-IntegrityParametersDL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 5 indicates whether the field nr-IntegrityParametersRTD-Info in IE NR-PositionCalculationAssistance is supported or not.

simul-DL-AIML-and-DL-TDOA This field, if present, indicates that the target device supports simultaneous operation of NR DL AI/ML and NR DL-TDOA positioning. The target device can include this field only if the target device supports UE-based NR DL-TDOA. Otherwise, the target device does not include this field.

simul-DL-AIML-and-DL-AoD This field, if present, indicates that the target device supports simultaneous operation of NR DL AI/ML and NR DL-AoD positioning. The target device can include this field only if the target device supports UE-based NR DL-AoD. Otherwise, the target device does not include this field.

supportOfDL-PRS-BWA-RRC-Connected Indicates whether the target device supports DL-PRS bandwidth aggregation in RRC_CONNECTED for NR DL AI/ML positioning. The target device can include this field only if the target device supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the target device does not include this field.

nr-DL-AIML-PRS-Capability This field indicates the DL-PRS Resources capability supported by the target device.

nr-DL-AIML-QCL-ProcessingCapability This field indicates DL-PRS QCL Processing capability supported by the target device.

nr-DL-AIML-PRS-ProcessingCapability This field indicates the DL-PRS Processing capability supported by the target device.

#### 6.5.13.6a	NR DL AI/ML Positioning Capability Information Elements

#### –	NR-DL-AIML-PRS-ProcessingCapability

The IE NR-DL-AIML-PRS-ProcessingCapability defines the DL-PRS Processing capability for NR DL AI/ML Positioning.

-- ASN1START

NR-DL-AIML-PRS-ProcessingCapability-r19 ::= SEQUENCE {

nr-dl-aiml-prs-ProcessingCapabilityBandList-r19	SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

NR-DL-AIML-PRS-ProcessingCapabilityPerBand-r19,

supportedActivatedPRS-ProcessingWindow-r19		ENUMERATED {n2, n3, n4}				OPTIONAL,

...

}

NR-DL-AIML-PRS-ProcessingCapabilityPerBand-r19 ::= SEQUENCE {

freqBandIndicatorNR-r19					FreqBandIndicatorNR-r16,

nr-dl-aiml-prs-ProcessingCapability-r19	NR-DL-AIML-PRS-ProcessingCapabilityElement-r19

OPTIONAL,

prs-ProcessingWindowType1A-r19			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingWindowType1B-r19			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingWindowType2-r19			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingCapabilityOutsideMGinPPW-r19

SEQUENCE (SIZE(1..3)) OF

PRS-ProcessingCapabilityOutsideMGinPPWperType-r19

OPTIONAL,

prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected-r19

PRS-BWA-TwoContiguousIntrabandInMG-r19		OPTIONAL,

prs-BWA-ThreeContiguousIntrabandInMG-RRC-Connected-r19

PRS-BWA-ThreeContiguousIntrabandInMG-r19	OPTIONAL,

supportOfPRS-BWA-WithTwoPFL-Combination-r19

ENUMERATED { supported }					OPTIONAL,

...

}

NR-DL-AIML-PRS-ProcessingCapabilityElement-r19 ::= SEQUENCE {

supportedBandwidthPRS-r19			CHOICE {

fr1										ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100},

fr2										ENUMERATED {mhz50, mhz100, mhz200, mhz400},

...

},

dl-PRS-BufferType-r19				ENUMERATED {type1, type2, ...},

durationOfPRS-Processing-r19		SEQUENCE {

durationOfPRS-ProcessingSymbols-r19	ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

durationOfPRS-ProcessingSymbolsInEveryTms-r19

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160, n320, n640, n1280},

...

},

maxNumOfDL-PRS-ResProcessedPerSlot-r19

SEQUENCE {

scs15-r19							ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs30-r19							ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs60-r19							ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs120-r19							ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

...

},

...

}

PRS-ProcessingCapabilityOutsideMGinPPWperType-r19 ::= SEQUENCE {

prsProcessingType-r19			ENUMERATED { type1A, type1B, type2 },

ppw-dl-PRS-BufferType-r19		ENUMERATED { type1, type2, ... },

ppw-durationOfPRS-Processing1-r19

SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN-r19

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms4,

ms6, ms8, ms12, ms16, ms20, ms25, ms30, ms32,

ms35, ms40, ms45, ms50 },

ppw-durationOfPRS-ProcessingSymbolsT-r19

ENUMERATED { ms1, ms2, ms4, ms8, ms16, ms20, ms30, ms40,

ms80, ms160, ms320, ms640, ms1280 },

...

}																				OPTIONAL,

ppw-durationOfPRS-Processing2-r19

SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN2-r19

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms3, ms4,

ms5, ms6, ms8, ms12 },

ppw-durationOfPRS-ProcessingSymbolsT2-r19

ENUMERATED { ms4, ms5, ms6, ms8 },

...

}																				OPTIONAL,

ppw-maxNumOfDL-PRS-ResProcessedPerSlot-r19

SEQUENCE {

scs15-r19						ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64 }						OPTIONAL,

scs30-r19						ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64 }						OPTIONAL,

scs60-r19						ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64 }						OPTIONAL,

scs120-r19						ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64 }						OPTIONAL,

...

},

ppw-maxNumOfDL-Bandwidth-r19	CHOICE {

fr1								ENUMERATED {mhz5, mhz10, mhz20, mhz40, mhz50, mhz80,mhz100},

fr2								ENUMERATED {mhz50, mhz100, mhz200, mhz400},

...

}																				OPTIONAL,

...

}

PRS-BWA-TwoContiguousIntrabandInMG-r19 ::= SEQUENCE {

maximumOfTwoAggregatedDL-PRS-Bandwidth-FR1-r19	ENUMERATED {mhz10, mhz20, mhz40, mhz50,

mhz80, mhz100, mhz160, mhz200}

OPTIONAL,

maximumOfTwoAggregatedDL-PRS-Bandwidth-FR2-r19	ENUMERATED {mhz100, mhz200, mhz400, mhz800}

OPTIONAL,

maximumOfDL-PRS-BandwidthPerPFL-FR1-r19			ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100}	OPTIONAL,

maximumOfDL-PRS-BandwidthPerPFL-FR2-r19			ENUMERATED {mhz50, mhz100, mhz200, mhz400}

OPTIONAL,

dl-PRS-BufferTypeOfBWA-r19						ENUMERATED {type1, type2},

prs-durationOfTwoPRS-BWA-Processing-r19			SEQUENCE {

prs-durationOfTwoPRS-BWA-ProcessingSymbolsN-r19

ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2, ms4, ms6, ms8, ms12,

ms16, ms20, ms25, ms30, ms32, ms35, ms40, ms45, ms50},

prs-durationOfTwoPRS-BWA-ProcessingSymbolsT-r19

ENUMERATED {ms8, ms16, ms20, ms30, ms40, ms80, ms160, ms320, ms640, ms1280},

...

}																					OPTIONAL,

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r19	SEQUENCE {

scs15-r19						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs30-r19						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs60-r19						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

...

},

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r19	SEQUENCE {

scs60-r19						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs120-r19						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

...

},

...

}

PRS-BWA-ThreeContiguousIntrabandInMG-r19 ::= SEQUENCE {

maximumOfThreeAggregatedDL-PRS-Bandwidth-FR1-r19

ENUMERATED {mhz15, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz120,

mhz140, mhz150, mhz180, mhz200, mhz240, mhz300}		OPTIONAL,

maximumOfThreeAggregatedDL-PRS-Bandwidth-FR2-r19

ENUMERATED {mhz150, mhz200, mhz300, mhz400, mhz600, mhz800, mhz1000,

mhz1200}											OPTIONAL,

maximumOfDL-PRS-BandwidthPerPFL-FR1-r19

ENUMERATED {mhz5, mhz10, mhz20, mhz40, mhz50, mhz80, mhz100}	OPTIONAL,

maximumOfDL-PRS-BandwidthPerPFL-FR2-r19

ENUMERATED {mhz50, mhz100, mhz200, mhz400}						OPTIONAL,

dl-PRS-BufferTypeOfBWA-r19

ENUMERATED {type1, type2},

prs-durationOfThreePRS-BWA-Processing-r19			SEQUENCE {

prs-durationOfThreePRS-BWA-ProcessingSymbolsN-r19

ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2, ms4, ms6, ms8, ms12,

ms16, ms20, ms25, ms30, ms32, ms35, ms40, ms45, ms50},

prs-durationOfThreePRS-BWA-ProcessingSymbolsT-r19

ENUMERATED {ms8, ms16, ms20, ms30, ms40, ms80, ms160,

ms320, ms640, ms1280},

...

}																					OPTIONAL,

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r19	SEQUENCE {

scs15-r19						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs30-r19						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs60-r19						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

...

},

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r19	SEQUENCE {

scs60-r19						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs120-r19						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

...

},

...

}

-- ASN1STOP

NR-DL-AIML-PRS-ProcessingCapability field descriptions

supportedActivatedPRS-ProcessingWindow Indicates the number of supported PRS Processing windows.

nr-dl-aiml-prs-ProcessingCapability Indicates the DL-PRS processing capability for NR DL AI/ML positioning and comprises the following subfields: - supportedBandwidthPRS: Indicates the maximum number of DL-PRS bandwidth in MHz, which is supported and reported by UE. - dl-PRS-BufferType: Indicates DL-PRS buffering capability. Value type1 indicates sub-slot/symbol level buffering and value type2 indicates slot level buffering. - durationOfPRS-Processing: Indicates the duration N of DL-PRS symbols in units of ms a UE can process every T ms assuming maximum DL-PRS bandwidth provided in supportedBandwidthPRS and comprises the following subfields: - durationOfPRS-ProcessingSymbols: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - durationOfPRS-ProcessingSymbolsInEveryTms: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. - maxNumOfDL-PRS-ResProcessedPerSlot: Indicates the maximum number of DL-PRS Resources that UE can process in a slot. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands. NOTE 1: When the target device provides the durationOfPRS-Processing capability (N, T) for any P(≥T) time window defined in TS 38.214 [45] clause 5.1.6.5, the target device should be capable of processing all DL-PRS Resources within P, if - N≥K where K is defined in the TS 38.214 [45] clause 5.1.6.5, and - the number of DL-PRS Resources in each slot does not exceed the maxNumOfDL-PRS-ResProcessedPerSlot, and - the configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) is as specified in TS 38.133 [46]. NOTE 2: If this group of fields is not included, but the IE NR-DL-PRS-ProcessingCapability is included in the ProvideCapabilities message body, the corresponding fields in IE NR-DL-PRS-ProcessingCapability (supportedBandwidthPRS-r16, dl-PRS-BufferType-r16, durationOfPRS-Processing-r16, maxNumOfDL-PRS-ResProcessedPerSlot-r16) are also applicable to NR DL AI/ML positioning.

prs-ProcessingWindowType1A Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 1A refers to the determination of prioritization between DL-PRS and other DL signals/channels in all OFDM symbols within the DL-PRS Processing Window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR. Enumerated value indicates supported priority handing options of DL-PRS: - option1: Support of "st1" and "st3" defined in clause 5.1.6.5 of TS 38.214 [45]. - option2: Support of "st1", "st2", and "st3" defined in clause 5.1.6.5 of TS 38.214 [45]. - option3: Support of "st1" only defined in clause 5.1.6.5 of TS 38.214 [45]. The UE can include this field only if the UE supports nr-dl-aiml-prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 3: Within a DL-PRS processing window, UE measurement is inside the active DL BWP with DL-PRS having the same numerology as the active DL BWP. NOTE 3a: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS.

prs-ProcessingWindowType1B Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 1B refers to the determination of prioritization between DL-PRS and other DL signals/channels in all OFDM symbols within the DL-PRS processing window. The DL signals/channels from a certain band are affected. Enumerated value indicates supported priority handing options of DL-PRS (see prs-ProcessingWindowType1A). The UE can include this field only if the UE supports nr-dl-aiml-prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 4: Within a DL-PRS processing window, UE measurement is inside the active DL BWP with DL-PRS having the same numerology as the active DL BWP. NOTE 4a: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS.

prs-ProcessingWindowType2 Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 2 refers to the determination of prioritization between DL-PRS and other DL signals/channels only in DL-PRS symbols within the DL-PRS processing window. Enumerated value indicates supported priority handing options of DL-PRS (see prs-ProcessingWindowType1A). The UE can include this field only if the UE supports nr-dl-aiml-prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 5: Within a DL-PRS processing window, UE measurement is inside the active DL BWP with DL-PRS having the same numerology as the active DL BWP. NOTE 5a: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS.

prs-ProcessingCapabilityOutsideMGinPPW Indicates the DL-PRS Processing Capability outside MG of each of the supported PPW Type in the case the UE supports multiple PPW Types in a band and comprises the following subfields: - prsProcessingType: Indicates the DL-PRS Processing Window Type for which the prs-ProcessingCapabilityOutsideMGinPPW are provided. - ppw-dl-PRS-BufferType: Indicates DL-PRS buffering capability. Value 'type1' indicates sub-slot/symbol level buffering and value 'type2' indicates slot level buffering. - ppw-durationOfPRS-Processing1: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum DL-PRS bandwidth provided in ppw-maxNumOfDL-Bandwidth and comprises the following subfields: - ppw-durationOfPRS-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - ppw-durationOfPRS-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. - ppw-durationOfPRS-Processing2: Indicates the duration of DL-PRS symbols N2 in units of ms a UE can process inT2 ms assuming maximum DL-PRS bandwidth provided in ppw-maxNumOfDL-Bandwidth and comprises the following subfields: - ppw-durationOfPRS-ProcessingSymbolsN2: This field specifies the values for N2. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12 ms. - ppw-durationOfPRS-ProcessingSymbolsT2: This field specifies the values for T2. Enumerated values indicate 4, 5, 6, 8 ms. - ppw-maxNumOfDL-PRS-ResProcessedPerSlot: Indicates the maximum number of DL-PRS resources that UE can process in a slot. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands. - ppw-maxNumOfDL-Bandwidth: Indicates the maximum number of DL-PRS bandwidth in MHz, which is supported and reported by UE for DL-PRS measurement outside MG within the PPW. The UE can include this field only if the UE supports one of prs-ProcessingWindowType1A, prs-ProcessingWindowType1B and prs-ProcessingWindowType2. Otherwise, the UE does not include this field. NOTE 6: A UE that supports one of prs-ProcessingWindowType1A, prs-ProcessingWindowType1B or prs-ProcessingWindowType2 shall always include the prs-ProcessingCapabilityOutsideMGinPPW. NOTE 7: The (N, T) UE capability in ppw-durationOfPRS-Processing1 is interpreted as in NOTE 1, and the UE is expected to receive the DL-PRS within the DL-PRS processing window but the processing of the received DL-PRS may be outside a DL-PRS processing window. NOTE 8: The (N2, T2) UE capability in ppw-durationOfPRS-Processing2 is interpreted such that the UE is capable of measuring up to N2 ms DL-PRS within a PPW and is capable of completing the DL-PRS processing within the PPW, e.g., if the time duration from the last symbol of the measured DL-PRS Resource(s) inside the PPW to the end of PPW is not smaller than T2 ms. NOTE 9: A UE which supports prs-ProcessingCapabilityOutsideMGinPPW shall support either ppw-durationOfPRS-Processing1 or ppw-durationOfPRS-Processing2, but not both for each supported type in a band. NOTE 10: If this group of fields is not included, but the IE NR-DL-PRS-ProcessingCapability is included in the ProvideCapabilities message body, the corresponding fields in IE NR-DL-PRS-ProcessingCapability (prs-ProcessingCapabilityOutsideMGinPPW-r17) are also applicable to NR DL AI/ML positioning.

prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected Indicates the UE capability for support of DL-PRS processing capabilities for aggregated DL-PRS processing of 2 PFLs in intra-band contiguous within a MG for RRC_CONNECTED state and and comprises the following subfields: - maximumOfTwoAggregatedDL-PRS-Bandwidth-FR1: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR1, which is supported and reported by UE. - maximumOfTwoAggregatedDL-PRS-Bandwidth-FR2: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR2, which is supported and reported by UE. - maximumOfDL-PRS-BandwidthPerPFL-FR1: Indicates the maximum DL-PRS bandwidth in MHz for FR1, per PFL. - maximumOfDL-PRS-BandwidthPerPFL-FR2: Indicates the maximum DL-PRS bandwidth in MHz for FR2, per PFL. - dl-PRS-BufferTypeOfBWA: Indicates the DL-PRS buffering capability. - prs-durationOfTwoPRS-BWA-Processing: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL-PRS bandwidth in MHz, which is supported and reported by UE. - prs-durationOfTwoPRS-BWA-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - prs-durationOfTwoPRS-BWA-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 2560 ms. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR1. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR2. The UE can include this field only if the UE supports supportedBandwidthPRS, dl-PRS-BufferType, durationOfPRS-Processing and maxNumOfDL-PRS-ResProcessedPerSlot. Otherwise, the UE does not include this field. NOTE 11: dl-PRS-BufferTypeOfBWA follows buffering capability type reported in dl-PRS-BufferType. NOTE 12: The value N should be equal or smaller than the value N reported by durationOfPRS-ProcessingSymbols, or this value T should be equal or larger than the value T reported by durationOfPRS-ProcessingSymbolsInEveryTms. NOTE 13: Each two linked DL-PRS Resources are counted as 1 resource. NOTE 14: maxNumOfAggregatedDL-PRS-ResourcePerSlot should be equal or smaller than the value reported by maxNumOfDL-PRS-ResProcessedPerSlot. NOTE 15: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL)/measurement gap repetition period (MGRP) of no more than 30%. NOTE 16: If this group of fields is not included, but the IE NR-DL-PRS-ProcessingCapability is included in the ProvideCapabilities message body, the corresponding fields in IE NR-DL-PRS-ProcessingCapability (prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected-r18) are also applicable to NR DL AI/ML positioning.

prs-BWA-ThreeContiguousIntrabandInMG-RRC-Connected Indicates the UE capability for support of DL-PRS processing capabilities for aggregated DL-PRS processing of 3 PFLs in intra-band contiguous within a MG for RRC_CONNECTED state and comprises the following subfields: - maximumOfThreeAggregatedDL-PRS-Bandwidth-FR1: Indicates the maximum aggregated DL-PRS bandwidth in MHz of for FR1, which is supported and reported by UE. - maximumOfThreeAggregatedDL-PRS-Bandwidth-FR2: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR2, which is supported and reported by UE. - maximumOfDL-PRS-BandwidthPerPFL-FR1: Indicates the maximum DL-PRS bandwidth in MHz for FR1, per PFL - maximumOfDL-PRS-BandwidthPerPFL-FR2: Indicates the maximum DL-PRS bandwidth in MHz for FR2, per PFL - dl-PRS-BufferTypeOfBWA: Indicates the DL-PRS buffering capability. - prs-durationOfThreePRS-BWA-Processing: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL-PRS bandwidth in MHz, which is supported and reported by UE. - prs-durationOfThreePRS-BWA-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - prs-durationOfThreePRS-BWA-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 3840 ms. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR1. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR2. The UE can include this field only if the UE supports prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field. NOTE17: dl-PRS-BufferTypeOfBWA follows buffering capability type reported in dl-PRS-BufferType. NOTE18: The value N should be equal or smaller than the value N reported by durationOfPRS-ProcessingSymbols, or this value T should be equal or larger than the value T reported by durationOfPRS-ProcessingSymbolsInEveryTms. NOTE19: Each three linked DL-PRS Resources are counted as 1 resource. NOTE20: maxNumOfAggregatedDL-PRS-ResourcePerSlot should be equal or smaller than the value reported by maxNumOfDL-PRS-ResProcessedPerSlot. NOTE21: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL)/measurement gap repetition period (MGRP) of no more than 30%. NOTE 22: If this group of fields is not included, but the IE NR-DL-PRS-ProcessingCapability is included in the ProvideCapabilities message body, the corresponding fields in IE NR-DL-PRS-ProcessingCapability (prs-BWA-ThreeContiguousIntrabandInMG-RRC-Connected-r18) are also applicable to NR DL AI/ML positioning.

supportOfPRS-BWA-WithTwoPFL-Combination Indicates whether the UE supports DL-PRS bandwidth aggregation with two PFL combinations. The UE can include this field only if the UE supports prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field.

# Annex B: TP2

### 6.4.3	Common NR Positioning Information Elements

[…]

#### –	NR-PositionCalculationAssistance

The IE NR-PositionCalculationAssistance is used by the location server to provide assistance data including integrity information to enable UE-based downlink positioning.

-- ASN1START

NR-PositionCalculationAssistance-r16 ::= SEQUENCE {

nr-TRP-LocationInfo-r16			NR-TRP-LocationInfo-r16				OPTIONAL,	-- Need ON

nr-DL-PRS-BeamInfo-r16			NR-DL-PRS-BeamInfo-r16				OPTIONAL,	-- Need ON

nr-RTD-Info-r16					NR-RTD-Info-r16						OPTIONAL,	-- Need ON

...,

[[

nr-TRP-BeamAntennaInfo-r17		NR-TRP-BeamAntennaInfo-r17			OPTIONAL,	-- Need ON

nr-DL-PRS-Expected-LOS-NLOS-Assistance-r17

NR-DL-PRS-ExpectedLOS-NLOS-Assistance-r17

OPTIONAL,	-- Need ON

nr-DL-PRS-TRP-TEG-Info-r17		NR-DL-PRS-TRP-TEG-Info-r17			OPTIONAL	-- Need ON

]],

[[

nr-IntegrityServiceParameters-r18	NR-IntegrityServiceParameters-r18	OPTIONAL,	-- Need OR

nr-IntegrityServiceAlert-r18		NR-IntegrityServiceAlert-r18		OPTIONAL,	-- Need OR

nr-IntegrityRiskParameters-r18		NR-IntegrityRiskParameters-r18		OPTIONAL,	-- Need OR

nr-IntegrityParametersTRP-LocationInfo-r18																								NR-IntegrityParametersTRP-LocationInfo-r18

OPTIONAL, -- Cond Integrity1

nr-IntegrityParametersDL-PRS-BeamInfo-r18

NR-IntegrityParametersDL-PRS-BeamInfo-r18

OPTIONAL, -- Cond Integrity2

nr-IntegrityParametersRTD-Info-r18																										NR-IntegrityParametersRTD-Info-r18

OPTIONAL, -- Cond Integrity3

nr-IntegrityParametersTRP-BeamAntennaInfo-r18																							NR-IntegrityParametersTRP-BeamAntennaInfo-r18

OPTIONAL, -- Cond Integrity4

nr-PRU-DL-Info-r18					NR-PRU-DL-Info-r18				OPTIONAL	-- Need ON

]],

[[

nr-TRP-LocationInfo-Implicit-r19	NR-TRP-LocationInfo-Implicit-r19	OPTIONAL	-- Need ON

]]

}

NR-IntegrityParametersTRP-LocationInfo-r18 ::= SEQUENCE {

trp-ErrorCorrelationTime-r18					INTEGER(0..255)		OPTIONAL, -- Need ON

dl-PRS-ResourceSetARP-ErrorCorrelationTime-r18	INTEGER(0..255)		OPTIONAL, -- Need ON

dl-PRS-ResourceARP-ErrorCorrelationTime-r18		INTEGER(0..255)		OPTIONAL, -- Need ON

...

}

NR-IntegrityParametersDL-PRS-BeamInfo-r18 ::= SEQUENCE {

dl-PRS-BeamInfoErrorCorrelationTime-r18		INTEGER (0..255),

...

}

NR-IntegrityParametersRTD-Info-r18 ::= SEQUENCE {

rtd-ErrorCorrelationTime-r18		INTEGER (0..255),

...

}

NR-IntegrityParametersTRP-BeamAntennaInfo-r18 ::= SEQUENCE {

trp-BeamAntennaInfoErrorCorrelationTime-r18		INTEGER (0..255),

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Integrity1 | The field is optionally present, need OR, if nr-TRP-LocationInfo is present and nr-IntegrityTRP-LocationBounds is present in IE NR-TRP-LocationInfo; otherwise it is not present. |
| Integrity2 | The field is optionally present, need OR, if nr-DL-PRS-BeamInfo is present and nr-IntegrityBeamInfoBounds is present in IE NR-DL-PRS-BeamInfo; otherwise it is not present. |
| Integrity3 | The field is optionally present, need OR, if nr-RTD-Info is present and nr-IntegrityRTD-InfoBounds is present in IE NR-RTD-Info; otherwise it is not present. |
| Integrity4 | The field is optionally present, need OR, if nr-TRP-BeamAntennaInfo is present and nr-IntegrityBeamPowerBounds is present in IE NR-TRP-BeamAntennaInfo; otherwise it is not present. |

NR-PositionCalculationAssistance field descriptions

nr-TRP-LocationInfo This field provides the location coordinates of the TRPs and location coordinates of antenna reference points for DL-PRS Resource Set(s) and DL-PRS Resources of the TRPs.

nr-DL-PRS-BeamInfo This field provides the spatial directions of DL-PRS Resources for TRPs.

nr-RTD-Info This field provides the time synchronization information between the reference TRP and neighbour TRPs.

nr-TRP-BeamAntennaInfo This field provides the relative DL-PRS Resource power between DL-PRS Resources per angle per TRP.

nr-DL-PRS-ExpectedLOS-NLOS-Assistance This field provides the expected likelihood of a LOS propagation path from a TRP to the target device. The information is provided per TRP or per DL-PRS Resource.

nr-DL-PRS-TRP-TEG-Info This field provides the TRP Tx TEG ID associated with the transmission of each DL-PRS Resource of the TRP.

nr-IntegrityServiceParameters This field specifies the range of Integrity Risk (IR) for which the integrity assistance data are valid.

nr-IntegrityServiceAlert This field indicates whether the corresponding assistance data can be used for integrity related applications.

trp-ErrorCorrelationTime This field specifies the TRP Error Correlation Time which is the upper bound of the correlation time of the TRP error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

dl-PRS-ResourceSetARP-ErrorCorrelationTime This field, if present, specifies the DL-PRS Resource Set ARP Error Correlation Time which is the upper bound of the correlation time of the DL-PRS Resource Set ARP error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

dl-PRS-ResourceARP-ErrorCorrelationTime This field, if present, specifies the DL-PRS Resource ARP Error Correlation Time which is the upper bound of the correlation time of the DL-PRS Resource ARP error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

rtd-ErrorCorrelationTime This field specifies the inter-TRP synchronization error Correlation Time which is the upper bound of the correlation time of the inter-TRP synchronization error. The correlation time is calculated using: t=10i, 1≤&i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, 234<i [s] Where i is the value given by rtdErrorCorrelationTime. Range is 1-28,200 s.

dl-PRS-BeamInfoErrorCorrelationTime This field specifies the Beam Boresight Direction Angle Error Correlation Time which is the upper bound of the correlation time of the DL-PRS Resource angle error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

trp-BeamAntennaInfoErrorCorrelationTime This field specifies the Mean Beam Power Error Correlation Time which is the upper bound of the correlation time of the mean beam power error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

nr-PRU-DL-Info This field provides the measurements reported by a PRU to the target UE.

nr-TRP-LocationInfo-Implicit This field provides implicit information on location coordinates of the TRPs. NOTE: This field is only applicable to NR DL AI/ML positioning.

# Annex C: TP3

### 6.4.3	Common NR Positioning Information Elements

#### –	NR-TRP-LocationInfo-Implicit

The IE NR-TRP-LocationInfo-Implicit provides information to enable a target device to determine whether the coordinates of TRPs are consistent between training and inference phases for NR DL AI/ML positioning.

-- ASN1START

NR-TRP-LocationInfo-Implicit-r19 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-TRP-LocationInfo-Implicit-PerFreqLayer-r19

NR-TRP-LocationInfo-Implicit-PerFreqLayer-r19 ::= SEQUENCE {

cell-LocationInfo-Implicit-List-r19	SEQUENCE (SIZE (1.. nrMaxCellsPerFreq-r19)) OF

TRP-LocationInfo-Implicit-Element-r19,

...

}

TRP-LocationInfo-Implicit-Element-r19 ::= SEQUENCE {

nr-PhysCellID-r19				NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r19				NCGI-r15					OPTIONAL,	-- Need ON

nr-ARFCN-r19					ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

nr-AIML-AssociatedID-r19		INTEGER (0..255),

...

}

-- ASN1STOP

NR-TRP-LocationInfo-Implicit field descriptions

nr-PhysCellID This field specifies the physical cell identity of the cell.

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR.

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

nr-AIML-AssociatedID This field provides an identity associated with the coordinates of the TRP(s) belonging to the indicated cell. The value of the nr-AIML-AssociatedID is changed if/when the coordinates of the TRP(s) is changed. NOTE: The target device is not expected to receive different values of nr-AIML-AssociatedID for TRPs belonging to the same cell.

## 6.6	Multiplicity and type constraint values

#### –	Multiplicity and type constraint definitions

-- ASN1START

maxEARFCN					INTEGER ::= 65535	-- Maximum value of EUTRA carrier frequency

maxEARFCN-Plus1				INTEGER ::= 65536	-- Lowest value extended EARFCN range

maxEARFCN2					INTEGER ::= 262143	-- Highest value extended EARFCN range

maxMBS-r14					INTEGER ::= 64

maxWLAN-AP-r13				INTEGER ::= 64

maxKnownAPs-r14				INTEGER ::= 2048

maxVisibleAPs-r14			INTEGER ::= 32

maxWLAN-AP-r14				INTEGER ::= 128

maxWLAN-DataSets-r14		INTEGER ::= 8

maxBT-Beacon-r13			INTEGER ::= 32

maxBT-BeaconAntElt-r18		INTEGER ::= 74

maxBT-BeaconAD-r18			INTEGER ::= 64

nrMaxBands-r16							INTEGER ::= 1024	-- Maximum number of supported bands in

-- UE capability.

nrMaxCellsPerFreq-r19					INTEGER ::= [64]	-- Max Cells per frequency

nrMaxFreqLayers-r16						INTEGER ::= 4		-- Max freq layers

nrMaxFreqLayers-1-r16					INTEGER ::= 3

nrMaxNumDL-PRS-ResourcesPerSet-1-r16	INTEGER ::= 63

nrMaxNumDL-PRS-ResourceSetsPerTRP-1-r16	INTEGER ::= 7

nrMaxResourceIDs-r16					INTEGER ::= 64		-- Max Resource IDs

nrMaxResourceOffsetValue-1-r16			INTEGER ::= 511

nrMaxResourcesPerSet-r16				INTEGER ::= 64		-- Maximum resources for one set

nrMaxSetsPerTrpPerFreqLayer-r16			INTEGER ::= 2		-- Maximum resource sets for one TRP

nrMaxSetsPerTrpPerFreqLayer-1-r16		INTEGER ::= 1

nrMaxTRPs-r16							INTEGER ::= 256		-- Max TRPs per UE

nrMaxTRPsPerFreq-r16					INTEGER ::= 64		-- Max TRPs per freq layers

nrMaxTRPsPerFreq-1-r16					INTEGER ::= 63

maxSimultaneousBands-r16				INTEGER ::= 4		-- Maximum number of simultaneously

-- measured bands

maxBandComb-r16							INTEGER ::= 1024

nrMaxConfiguredBands-r16				INTEGER ::= 16

maxNumOfRxTEGs-r17						INTEGER ::= 32

maxNumOfRxTEGs-1-r17					INTEGER ::= 31

maxNumOfTxTEGs-1-r17					INTEGER ::= 7

maxTxTEG-Sets-r17						INTEGER ::= 256		-- Maximum applicable number is 64

maxNumOfRxTxTEGs-1-r17					INTEGER ::= 255

maxNumOfTRP-TxTEGs-1-r17				INTEGER ::= 7

maxNumOfSRS-PosResources-r17			INTEGER ::= 64

maxNumOfSRS-PosResources-1-r17			INTEGER ::= 63

maxNumResourcesPerAngle-r17				INTEGER ::= 24

maxNumPrioResources-r17					INTEGER ::= 24

maxAddMeasTDOA-r17						INTEGER ::= 31

maxAddMeasAoD-r17						INTEGER ::= 23

maxAddMeasRTT-r17						INTEGER ::= 31

maxOD-DL-PRS-Configs-r17				INTEGER ::= 8

maxCellIDsPerArea-r17					INTEGER ::= 256

maxNrOfAreas-r17						INTEGER ::= 16

maxMeasInstances-r17					INTEGER ::= 32

nrMaxNumPRS-BandWidthAggregation-r18	INTEGER ::= 256		-- Max number of DL-PRS bandwidth

-- aggregation configurations that a

-- location server can provide to a UE

nrNumOfSamples-r18						INTEGER ::= 4		-- NSample of RSCP/RSCPD

nrNumOfSamples-1-r18					INTEGER ::= 3

-- ASN1STOP

#### –	End of LPP-PDU-Definitions

-- ASN1START

END

-- ASN1STOP