3GPP TSG-RAN WG2 Meeting #131bis	R2-250xxxx

# Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		8.x.x

Source:	Xiaomi

Title:	Open issues of Rel-19 AIML air UE capabilities

Document for:		Discussion and Decision

# Introduction

As part of email discussion “[POST131][043][AI PHY] UE capabilities (Xiaomi)”, this document is to collect open issues on Rel-19 AIML air UE capabilities.

Phase 1: Companies are invited to provide input on open issue list no later than 19th Sep.

Phase 2: Rapporteur will further provide the resolution to each open issue afterwards. Companies further input on the open issue list and resolution are expected before 1st Oct.

| Company | Name | Email |
| --- | --- | --- |

# Discussion

During previous discussion, rapporteur observes there’s only one open issue left:

Open issue UECap-1: whether UE can support other memory sizes and indicate to network via optional capability signaling

Issue Description:

This was discussed during the email discussion, and there are 5 companies think that UE can optionally indicate other AS layer memory sizes to network due to the following reasons:

-	network flexibility for higher buffer threshold configuration

-	common AS layer buffer might need larger memory size to support other use cases as well

4/9 companies don’t agree to indicate other memory size to network due to the following reasons:

-	for Rel-19 data collection, no need for additional memory

-	more memory size is up to UE implementation

-	network can retrieve data based on UE buffer full indication

Status in running CR:

Not captured

Based on offline and online discussion, the supporting of additional memory sizes mainly motivated by supporting more flexible buffer-threshold based configuration. It seems reasonable to consider dataThresholdAvailabiltyIndication-r19 as the prerequisite of this new capability (if introduced). If dataThresholdAvailabiltyIndication-r19 is not supported, UE does not need to support additional memory size.

Therefore, rapporteur proposes with the following way-forward, where the candidate values consider memory size supported in qoe-AdditionalMemoryMeasReport-r18 as baseline (except introducing a smaller value of 64kB):

Proposal 1: Define aiml-AdditionalMemoryReport-r19 to indicate the additional AS layer memory size the UE support for logged measurement of network-side data collection in addition to 64kB.

| aiml-AdditionalMemoryReport-r19 Indicates whether the UE supports the additional AS layer memory size the UE support for logged measurement of network-side data collection in addition to 64kB. Value kB64 means the UE supports at least 64 kilobytes for this purpose, and so on. A UE supporting this feature shall also indicate support of dataThresholdAvailabiltyIndication-r19. | UE | No | No | No |
| --- | --- | --- | --- | --- |

AIML-Parameters information element

-- ASN1START

-- TAG-AIML-PARAMETERS-START

AIML-Parameters-r19 ::=            SEQUENCE {

applicabilityReportingCSI-r19                  ENUMERATED {supported}                                      OPTIONAL,

applicabilityReportingOther-r19                ENUMERATED {supported}                                      OPTIONAL,

loggedDataCollection-r19                       ENUMERATED {supported}                                      OPTIONAL,

eventBasedLoggedDataCollection-r19             ENUMERATED {supported}                                      OPTIONAL,

dataThresholdAvailabilityIndication-r19        ENUMERATED {supported}                                      OPTIONAL,

aiml-AdditionalMemoryReport-r19                ENUMERATED {kB64, kB128, kB256, kB512, kB1024}

OPTIONAL

}

-- TAG-AIML-PARAMETERS-STOP

-- ASN1STOP

Q1: Do companies agree with above proposal and the relative TPs for this new introduced capability? Comment to the TP is also welcomed.

| Company | Yes/No | Comment |
| --- | --- | --- |

Besides the above open issue, companies are invited to describe any identified open issues in the table below.

| Company | Description of open issues and potential resolution | Rapporteur comment |
| --- | --- | --- |

# Conclusion

In this contribution, we collect open issues of Rel-19 AIML air UE capabilities. Based on above discussion, following open issues are identified: